Chesapeake Bay National Estuarine Research Reserve in Virginia (CBV)

NERR Nutrient Metadata

January-December, 2007

Latest Update:  November 15, 2011

I.  Data Set and Research Descriptors

a)  Reserve Contact

Kenneth A. Moore, Ph.D.

Research Coordinator

1208 Greate Road

Virginia Institute of Marine Science

Gloucester Point, Virginia  23062

###### Phone: 804-684-7119

e-mail: moore@vims.edu

b)  Laboratory Contact

Carol Pollard

1208 Greate Road

VIMS Analytical Service Center for Nutrients

Virginia Institute of Marine Science

Gloucester Point, Virginia  23062

Phone: 804-684-7213

e-mail: pollard@vims.edu

c)  Other Contacts and Programs

Eduardo Miles

1208 Greate Road

Virginia Institute of Marine Science

Gloucester Point, Virginia  23062

Phone: 804-684-7836

e-mail: emiles@vims.edu

Joy Austin

1208 Greate Road

Virginia Institute of Marine Science

Gloucester Point, Virginia  23062

Phone: 804-684-7307

e-mail: justjoy@vims.edu

William G. Reay, Ph.D.

1208 Greate Road

Virginia Institute of Marine Science

Gloucester Point, Virginia  23062

###### Phone: 804-684-7119

e-mail: wreay@vims.edu

2)  Research objectives

- Monthly Grab

Monthly grab samples are collected to quantify the spatial and temporal variability of selected nutrients and plant pigments in the water column along a salinity gradient within the York River estuary system.

- Diel Sampling Program

On a monthly basis, samples are collected at Taskinas Creek, a small tributary of the York River, every two and one-half hours throughout a complete tidal cycle in order to quantify the short-term temporal variability of selected nutrients and plant pigments in the water column.

3) Research methods

- Monthly Grab Sampling Program

## Monthly grab samples were taken at six stations within the York River estuary.  Samples were taken at the four principal CBNERRVA datasonde stations (Goodwin Islands, Clay Bank, Taskinas Creek and Sweet Hall Marsh and two other stations which were the York River Bridge, located at Gloucester Point, and Catlett Islands.  All grab samples were taken on the same day between +3 hrs before or after slack low-water.  No distinction was made between neap and spring tide conditions.  Efforts were made to allow for an antecedent dry period of 72 hours prior to sampling.  Replicate (N=2) samples were collected by hand at an approximate depth of 10 cm.  At the time of sample collection, water temperature, salinity and dissolved oxygen was measured with a YSI Model 85 meter.  All samples were collected in amber, wide-mouth, nalgene sample bottles that were previously acid washed (10% HCl), rinsed (3x) with distilled deionized water, dried and followed by rinsing (3x) of ambient water prior to collection of the sample.  Samples were immediately placed on ice, in the dark and returned to the laboratory.  Once in the laboratory, samples were shaken and processed for nutrient and plant pigment analysis.

- Diel Sampling Program

Diel grab samples were taken at the Taskinas Creek long-term datasonde station.  Samples were collected over a lunar day (24hr:48min) time period at 2.5 hour intervals using an ISCO auto-sampler.  Collection of samples began at predicted slack low water.  Samples were collected at a fixed depth (0.5m) from the bottom which reflected the water mass sampled by the datasonde.  No distinction was made between neap and spring tide conditions.  Efforts were made to allow for an antecedent dry period of 72 hours prior to sampling.  Samples were collected in 1000 ml bottles that were previously acid washed (10% HCl), rinsed (3x) with distilled-deionized water and dried.  Prior to sample collection, the ISCO sampler including all sample tubing was rinsed with ambient water.  Samples were collected and stored inside a ISCO sampler that contained ice.  Once in the laboratory, samples were shaken and processed for nutrient and plant pigments.

4)  Site location and character

a)  Goodwin Islands (Lat 37.21739; Long 76.3954)

The Goodwin Islands component of CBNERRVA is located on the southern side of the mouth of the York River. The station is located approximately 400 meters from shore, with an average water depth on the order of 1 meter.  MHW depth at the sample location is approximately 1.70 meters. Goodwin Islands are a 315 ha (777 acre) archipelago of salt-marsh islands surrounded by inter-tidal flats and extensive beds of submerged aquatic vegetation dominated by eelgrass (Zostera marina) and Widgeon grass (Ruppia maritime). Water circulation patterns around the island are influenced by York River discharge and wind patterns of the Chesapeake Bay. Tides at the Goodwin Islands are semi-diurnal and display an average range of 0.7 m (range: 0.4 - 1.1 m). Mean seasonal water temperature values ranged from 13.7-15.6 °C for spring (March-May), 25.7-27.2 °C for summer (June-August), 18.0-19.2 °C for fall (September-November), and 4.7-8.2 °C for winter (January-February, and December).  Mean seasonal salinity values ranged from 13.9-23.0 psu for spring, 17.2-23.0 psu for summer, 16.5-24.0 for fall, and 15.9-23.3 psu for winter.  Summary water quality statistics were derived from 15-minute interval data for the years 1998-2004. The data logger probes are located 0.5 m above the sandy substrate bottom. Potential activities that could impact the site included, light recreational and commercial boating activity.

b)  York River Bridge (Lat 37.24489; Long 76.50522)

The York River Bridge station is located in the mid-channel approximately 9 km from the mouth of the York River estuary between Gloucester Point and Yorktown, Virginia.  The sampling station is located within the polyhaline region with a mean depth of 21 meters and a tidal range of 0.75 meters.  Mean salinity at this site is 17.4 ppt and the salinity range is 8.2 to 25.1 ppt (taken from the 2002-2005 monthly nutrient data).  Depth at the sample location ranges from 13.7 to 22.9 meters.  The bottom type is a mixture of sand and fine sediments and there are no known pollutants.

c)  Catlett Islands (Lat 37.30000; Long 76.55000)

The Catlett Islands are located approximately 20 km from the mouth and on the North side of the York River in Gloucester County, Virginia. Timberneck Creek flows into the York River on the eastern side of the Catlett Islands and Cedarbush Creek enters the river on the western side. Poplar Creek bisects the two large areas of the Catlett Islands.  Catlett Islands encompass 280 ha (690acres) and displays a ridge-and-swale geomorphology. The islands consist of multiple parallel ridges of forested wetland hammocks, forested upland hammocks, emergent wetlands and tidal creeks surrounded by shallow subtidal areas that once supported beds of submerged aquatic vegetation.  Water quality ranges listed here are based on weekly volunteer citizen water quality monitoring has been ongoing since the incorporation of the site in 1990. Surface water temperatures range from 5.4 °C to 27.4 °C. Salinity is indicative of mesohaline conditions, ranging from 14-18 ppt in the fall to 8.2-12 ppt in the spring. Dissolved oxygen concentration ranges from 4.2 to 14.0 mg/L.  Tidal range is on the order of 0.75 meters and depth at the sample location is from 1.0 to 1.5 meters.  The bottom type is a mixture of sand and fine sediments and there are no known pollutants.

d) Claybank (Lat 37.34855; Long 76.61123)

The Claybank station is loaded within a shallow (<2m) littoral area approximately 300-400 meters wide along the mesohaline portion of the York River estuary.  The site is approximately 26 km upriver from the mouth of the estuary.  The shoreline consists of a narrow fringe of salt marsh with some areas armored with bulkhead or stone. Tidal range is on the order of 0.85 meters and depth at MHW is approximately 2.25 meters.  This station is located along the north shoreline of the estuary in an area that historically (prior to 1972) supported submersed aquatic vegetation.  The sampling station is influenced by a secondary turbidity maximum that moves back and forth in a region of about 20-40 km from the mouth of the York River estuary.  Substrate within the region varies from fine sediments to sand.  The site is exposed to strong winds from the northwest and re-suspension of sediment during storm events can be high. There is no fresh water input at this site. Seasonal water quality conditions described here are from 2001 through 2003 (Spring: March-May; Summer: June-August; Fall: September-November; Winter: December-February).  Mean seasonal water temperature ranged between 14.0-16.2 for spring, 26.1-27.7 for summer, 17.5-19.4 for fall, and 4.9-8.0 °C in winter.  Mean seasonal salinity ranged between 15.7-20.3 for spring, 16.5-21.3 for summer, 13.2-21.6 for fall, and 14.3-20.0 ppt in winter.

e)  Taskinas Creek (Lat 37.41497; Long 76.71441)

Taskinas Creek Reserve, component of CBNERRVA, encompasses 397 ha (980 acres) and is located within the boundaries of York River State Park near the town of Croaker, Virginia.  The small sub-estuary of the York River is located on the southern side of the river, approximately 37 km up river from the mouth of the York River.  The Taskinas Creek watershed is representative of an inner coastal plain, rural watershed within the southern Chesapeake Bay system. The watershed is dominated by forested and agricultural land uses with an increasing residential land use component. The non-tidal portion of Taskinas Creek contains feeder streams that drain oak-hickory forests, maple-gum-ash swamps and freshwater marshes. Freshwater mixed wetlands are found in the upstream reaches of Taskinas Creek.  The creek is roughly 2 meters deep and 20 meters wide towards the lower end of the creek where substrate is dominated by fine sediment.  MHW depth at the sample location is approximately 2.0 meters and mean tide range is 0.85 meters.  Mean seasonal water temperature values ranged from 15.2-19.0 °C for spring, 26.8-28.2 °C for summer, 15.7-18.3 °C for fall, and 3.6-9.0 °C for winter.  Located within the meso-polyhaline region of the York River estuary, mean seasonal salinity values ranged from 4.0-14.0 psu for spring, 7.0-18.2 psu for summer, 6.9-17.0 for fall, and 5.8-15.3 psu for winter.  Summary water quality statistics were derived from 15-minute interval data for the years 1998-2004. Potential activities that could impact the site include residential development, selective hardwood logging, and light recreational boating activity.  Wildlife populations have been shown to influence microbiological water quality within the watershed.

f)  Sweet Hall Marsh (Lat 37.57138; Long 76.88424)

Sweet Hall Marsh is the most downriver extensive tidal freshwater marsh located in the Pamunkey River, one of two major tributaries of the York River.  The marsh is located approximately 77 km upriver from the mouth of the York River estuary. The reserve is 353 ha (871 acres) in area and includes 331 ha (818 acres) of emergent fresh-water marsh, 14 ha (35 acres) of permanently flooded broad-leaved forested wetlands and approximately 4 ha (9 acres) of scrub-shrub. The marsh community is classified as freshwater mixed.  Mean tidal range at Sweet Hall Marsh is on the order of 0.9 meters and MHW depth at the sample location is approximately 1.5 meters.  The Pamunkey River, which surrounds Sweet Hall Marsh, can reach depths up to 15 meters.  Substrate within the littoral zone and channel is dominated by fine sediment. Mean seasonal water temperature values ranged from 14.7-16.7 °C for spring, 26.7-27.9 °C for summer, 18.6-19.1 °C for fall, and 4.7-6.3 °C for winter.  Located within the oligohaline, lower freshwater reaches of the Pamunkey River, mean seasonal salinity values ranged from 0.1-3.4 psu for spring, 0.1-8.4 psu for summer, 0.3-8.4 psu for fall, and 0.1-3.2 psu for winter.  Summary water quality statistics were derived from 15-minute interval data for the years 2002-2004.   Potential activities that could impact the site include minor municipal point source discharges above and below river of Sweet Hall Marsh, a major industrial discharge site (pulp mill) in the town of West Point and significant groundwater withdrawal near the confluence of the Pamunkey and York Rivers.

5) Coded variable definitions

cbvcbnut = Chesapeake Bay Virginia Claybank nutrients

monthly grab sample program = 1

cbvcinut = Chesapeake Bay Virginia Catlett Island nutrients

monthly grab sample program = 1

cbvginut = Chesapeake Bay Virginia Goodwin Island nutrients

monthly grab sample program = 1

cbvshnut = Chesapeake Bay Virginia Sweet Hall Marsh nutrients

monthly grab sample program = 1

cbvtcnut = Chesapeake Bay Virginia Taskinas Creek nutrients

monthly grab sample program = 1

diel grab sample program = 2

cbvybnut = Chesapeake Bay Virginia York River Bridge nutrients

monthly grab sample program = 1

6) Data collection period

a) Monthly Grab Sample Program (Monitoring Program 1)

| Station Code | Start Date | Start Time |
| --- | --- | --- |
| GI | 1/11/2007 | 7:44 |
| GI | 2/13/2007 | 9:42 |
| GI | 3/13/2007 | 8:37 |
| GI | 4/10/2007 | 8:05 |
| GI | 5/11/2007 | 7:30 |
| GI | 6/07/2007 | 6:36 |
| GI | 7/09/2007 | 8:50 |
| GI | 8/06/2007 | 7:30 |
| GI | 9/05/2007 | 7:59 |
| GI | 10/22/2007 | 8:58 |
| GI | 11/19/2007 | 8:55 |
| GI | 12/18/2007 | 9:39 |
| Station Code | Start Date | Start Time |
| YB | 1/11/2007 | 8:12 |
| YB | 2/13/2007 | 10:28 |
| YB | 3/13/2007 | 9:25 |
| YB | 4/10/2007 | 8:39 |
| YB | 5/11/2007 | 7:51 |
| YB | 6/07/2007 | 6:58 |
| YB | 7/09/2007 | 9:17 |
| YB | 8/06/2007 | 7:55 |
| YB | 9/05/2007 | 8:28 |
| YB | 10/22/2007 | 9:24 |
| YB | 11/19/2007 | 9:35 |
| YB | 12/18/2007 | 9:58 |
| Station Code | Start Date | Start Time |
| CI | 1/11/2007 | 8:25 |
| CI | 2/13/2007 | 11:03 |
| CI | 3/13/2007 | 10:13 |
| CI | 4/10/2007 | 9:10 |
| CI | 5/11/2007 | 8:03 |
| CI | 6/07/2007 | 7:11 |
| CI | 7/09/2007 | 9:31 |
| CI | 8/06/2007 | 8:03 |
| CI | 9/05/2007 | 8:46 |
| CI | 10/22/2007 | 9:37 |
| CI | 11/19/2007 | 10:16 |
| CI | 12/18/2007 | 10:48 |

| Station Code | Start Date | Start Time |
| --- | --- | --- |
| CB | 1/11/2007 | 8:39 |
| CB | 2/13/2007 | 11:25 |
| CB | 3/13/2007 | 10:32 |
| CB | 4/10/2007 | 9:30 |
| CB | 5/11/2007 | 8:18 |
| CB | 6/07/2007 | 7:25 |
| CB | 7/09/2007 | 10:05 |
| CB | 8/06/2007 | 8:20 |
| CB | 9/05/2007 | 9:05 |
| CB | 10/22/2007 | 9:49 |
| CB | 11/19/2007 | 10:32 |
| CB | 12/18/2007 | 11:06 |
| Station Code | Start Date | Start Time |
| TC | 1/11/2007 | 9:07 |
| TC | 2/13/2007 | 10:47 |
| TC | 3/13/2007 | 9:39 |
| TC | 4/10/2007 | 9:16 |
| TC | 5/11/2007 | 11:08 |
| TC | 6/07/2007 | 8:45 |
| TC | 7/09/2007 | 9:59 |
| TC | 8/06/2007 | 10:22 |
| TC | 9/05/2007 | 9:44 |
| TC | 10/22/2007 | 10:18 |
| TC | 11/19/2007 | 10:05 |
| TC | 12/18/2007 | 9:22 |
| Station Code | Start Date | Start Time |
| SH | 1/11/2007 | 10:05 |
| SH | 2/13/2007 | 13:51 |
| SH | 3/13/2007 | 12:28 |
| SH | 4/10/2007 | 11:34 |
| SH | 5/11/2007 | 11:58 |
| SH | 6/07/2007 | 11:48 |
| SH | 7/09/2007 | 12:22 |
| SH | 8/06/2007 | 11:44 |
| SH | 9/05/2007 | 11:00 |
| SH | 10/22/2007 | 14:00 |
| SH | 11/19/2007 | 12:58 |
| SH | 12/18/2007 | 12:16 |

b)  Diel Sample Program (Monitoring Program 2)

| Station Code | Start Date | Start Time | End Date | End Time |
| --- | --- | --- | --- | --- |
| TC | 1/11/2007 | 10:21 | 1/12/2007 | 11:21 |
| TC | 2/13/2007 | 13:36 | 2/14/2007 | 14:36 |
| TC | 3/13/2007 | 12:55 | 3/14/2007 | 13:55 |
| TC | 4/10/2007 | 11:15 | 4/11/2007 | 12:15 |
| TC | 5/09/2007 | 10:45 | 5/10/2007 | 11:45 |
| TC | 6/07/2007 | 10:20 | 6/08/2007 | 11:20 |
| TC | 7/09/2007 | 12:35 | 7/10/2007 | 13:35 |
| TC | 8/06/2007 | 11:12 | 8/07/2007 | 12:12 |
| TC | 9/05/2007 | 12:04 | 9/06/2007 | 13:04 |
| TC | 10/22/2007 | 14:11 | 10/23/2007 | 15:11 |
| TC | 11/19/2007 | 11:42 | 11/20/2007 | 12:42 |
| TC | 12/18/2007 | 11:20 | 12/19/2007 | 12:20 |

Note:  Time is coded based on a 2400 hour clock and is referenced to Eastern Standard Time (EST).

7) Associated researchers and projects

Additional water quality monitoring programs within the York River system include:

a)  USEPA Chesapeake Bay Mainstem and Tributary Monitoring Program.  Since 1984, biweekly to monthly water quality sampling at a series of sites located along the mid-river channel has been conducted as part of the Chesapeake Bay Program (www.chesapeakebay.net).  Station ID’s: York River proper (WE4.2, LE4.3, LE4.2, LE4.1, RET4.3), the Pamunkey River (RET4.1, TF4.2) and Mattaponi River (RET4.2 and TF4.4).

b)  VIMS Shoal Survey.  Since 1984, biweekly to monthly water quality sampling at a series of sites located along the shoal areas of the lower York River estuary has been conducted by the Biological Sciences Department at the Virginia Institute of Marine Science.  Station ID’s include: Guinea Marsh, Goodwin Island, VIMS, Yorktown, Mumfort Islands, Catlett Islands and Clay Bank.

c)  Alliance for the Chesapeake Bay Volunteer Monitoring Program.  Physical and chemical (limited nutrients) data are collected by a volunteer network along the York River system (www. Acb-online.org).  Station ID’s include: Thorofare Creek, Wormley Creek, Blackwell Landing, Pamunkey Trail, Timberneck Creek, Yorktown Naval Weapons Station, Gloucester Point, West Point and Croaker Landing.  Note: Some stations may be inactive.

d)  VIMS Juvenile Abundance Monitoring Survey.  As part of their Juvenile Abundance Monitoring Surveys, water quality and hydrographic data has been collected since 1968 along a series of sites in the York River estuary (includes the Mattaponi and Pamunkey River systems) by the Fisheries Science Department (www.fisheries.vims.edu/research.html at the Virginia Institute of Marine Science.  Surveys include the VIMS Trawl Survey, the Striped Bass Seine Survey and the Juvenile Shad/River Herring Pushnet Survey.

e)  Virginia Department of Health.  The Virginia Department of Health, Division of Shellfish Sanitation’s (www.vdh.state.va.us/shellfish) Seawater Sampling Program collects microbial and general water quality and hydrographic data along a series of sites in the York River estuary (includes lower portions of the Mattaponi and Pamunkey River systems).

f)  ) USEPA Chesapeake Bay Shallow Water Monitoring Program.  Since May 2003, CBNERRVA has maintained additional continuous (15 minute) fixed water monitoring stations within the York, Piankatank,  James River, Rappahannock River, and Potomac River estuary systems using YSI 6600 EDS Datasondes.  Measurements for this program include: temperature, specific conductivity, dissolved oxygen, pH, turbidity, insitu fluorescence, and depth.  York River stations are located at Gloucester Point and White House (Pamunkey River).  Piankatank River stations are located at Burton’s Point, Bland’s Wharf, and Dragon Run.  James River stations are located at Wythe Point, James River Country Club, 4H Club, Chickahominy Haven, Rice Center, Appomattox, and Osborne Landing.  Rappahannock River stations are located at Hicks Landing, Kendale Farms, Bowler’s Wharf, Christ Church, and Corrotoman River.  Potomac River stations are located Potomac Creek, Colonial Beach, Yeocomico River, and Nomini Bay.  An additional surface water quality mapping program, which monitors the above stated parameters, at sub-surface depths of approximately 0.25 m along continuous cruise tracts, occurs on a monthly basis in the York River estuary (see www.VECOS.org).  This sub-surface continuous sampling of water quality has been conducted since May 2003 on the York River until present, and for the Pamunkey and Mattaponi Rivers from May 2003 through October 2005.

g)  Chesapeake Bay National Estuarine Research Reserve (CBV) Water Quality Monitoring Program.  The Chesapeake Bay Virginia NERR maintains a long-term water quality monitoring stations at Goodwin Island, Claybank, Sweet Hall Marsh, and Taskinas Creek within the York River estuary.  Goodwin Island station was established in 1997, Claybank in 2001, Sweet Hall Marsh in 2000, and Taskinas Creek in 1997.  Using YSI 6600 EDS Datasondes, measurements for this program include: temperature, specific conductivity, dissolved oxygen, pH, turbidity, insitu fluorescence, and depth.

h)  Chesapeake Bay National Estuarine Research Reserve (CBV) Meteorological Monitoring Program.  Since 2001, CBNERR-VA has maintained a meteorological monitoring station located at Taskinas Creek within the York River estuary system.  Measurements for this program include:  Air Temperature (degrees C), Relative Humidity (%), Barometric  Pressure (mb), Wind speed (m/s),Wind Direction (degrees), PAR (mmol/m^2), and Precipitation (mm) data.

8) Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format.

II. Physical Structure Descriptors

9) Entry verification

Monthly nutrient and Chlorophyll a data files, in excel format, are sent to CBNERRVA by the VIMS Analytical Service Center, Carol Pollard, for Nutrients.  Files consist of sampling station ID, date and time and parameter values expressed in unit concentrations.  The Laboratory Supervisor, Joy Austin, verifies all parameter values in the excel file through cross comparison with laboratory data sheets.  In order to identify any obvious discrepancies in field sample replicates, replicates were plotted against each other and visually inspected.

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and automatically flags and codes values below MDL; calculates parameters chosen by the user and automatically flags for component values below MDL and negative values; allows the user to apply QAQC flags and codes to the data; graphs selected parameters for review; append files; and export the resulting data files to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

10) Parameter titles and variable names by category

Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisks “*”.

Data Category	Parameter				Variable Name    Units of Measure

Phosphorus:		Total Dissolved Phosphorus		TDP		    mg/L as P

*Orthophosphate, Filtered		PO4F		    mg/L as P

Dissolved Organic Phosphorus		DOP		    mg/L as P

Nitrogen:		Total Dissolved Nitrogen		TDN		    mg/L as N

*Nitrite + Nitrate, Filtered		NO23F		    mg/L as N

*Nitrite, Filtered				NO2F		    mg/L as N

*Nitrate, Filtered			NO3F		    mg/L as N

*Ammonium, Filtered			NH4F		    mg/L as N

Dissolved Inorganic Nitrogen		DIN		    mg/L as N

Dissolved Organic Nitrogen		DON		    mg/L as N

Other Lab Parameters:

*Chlorophyll a				CHLA_N	    g/L

Phaeophytin				PHEA		    g/L

Field Parameters:

Dissolved Oxygen			DO_N		    mg/L

%Dissolved Oxygen Saturation		DO_S_N	    %

Salinity					SALT_N	    ppt

Water Temperature			WTEM_N	    C

Notes:

1.  Time is coded based on a 2400 clock and is referenced to Standard Time.

2.  Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

11) Measured or calculated laboratory parameters – This section lists all measured and calculated variables.  Only list those parameters that are collected and reported.  See the “Nutrient and Chlorophyll Monitoring Program and Database Design” Version 1.2 (January 2008) document for a full list of directly measured and computed variables.

- Parameters measured directly

Nitrogen species:		NH4, NO2, NO23, TDN

Phosphorus species:		PO4F, TDP

Other:			CHLA, PHEA

- Calculated parameters

DOP				TDP-PO4F

NO3				NO23-NO2

DIN				NO23+NH4

DON				TDN-DIN

12) Limits of detection

Method Detection Limits (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect, have been established by the VIMS Nutrient Analytical Laboratory.  The MDL is determined as 3 times the standard deviation of a minimum of 7 replicates of a single low concentration sample.  These values are reviewed and revised periodically.

| Parameter | Start Date | End Date | MDL |
| --- | --- | --- | --- |
| CHLA_N | 01/01/07 | 12/31/07 | 0.50 |
| NH4F | 01/01/07 | 12/31/07 | 0.0054 |
| NO23F | 01/01/07 | 12/31/07 | 0.0010 |
| PO4F | 01/01/07 | 12/31/07 | 0.0015 |
| NO2F | 01/01/07 | 12/31/07 | 0.0002 |
| TDN | 01/01/07 | 12/31/07 | 0.0084 |
| TDP | 01/01/07 | 12/31/07 | 0.0030 |
| PHEA | 01/01/07 | 12/31/07 | 0.50 |

13) Laboratory methods

a)  Parameter: NH4F

i) Method Summary:  Alkaline phenol and hypochlorite react with ammonia to form indophenol blue that is proportional to the ammonia concentration. The blue color formed is intensified with sodium nitroprusside. Reaction is heat catalyzed at 37oC and is measured colorimetrically at 660 nm. Instrumentation:  SKALAR San-Plus continuous flow autoanalyzer.

ii)  Method Reference(s):

U.S. EPA.  1974.  Methods for Chemical Analysis of Water and Wastes, pp. 168-174.

Standard Methods for the Examination of Water and Wastewater, 14th edition. p 410. Method 418A and 418B (1975).

Annual Book of ASTM Standards, Part 31. "Water", Standard 1426-74, Method A, p 237 (1976).

EPA 600/R-97/072 Method 349.0. Determination of Ammonia in Estuarine and Coastal Waters by Gas Segmented Continuous Flow Colorimetric Analysis.  In: Methods for the  Determination of Chemical Substances in Marine and Estuarine Environmental Matrices - 2nd Edition. National Exposure Research Laboratory, Office of Research and Development U.S. EPA, Cincinnati, Ohio 45268.

iii)  Preservation Method:  Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis.  Maximum holding time is 28 days.

b)  Parameter: NO23F

i) Method Summary:  Nitrate is reduced to nitrite by a copper/cadmium reductor column. The nitrite ion then reacts with sulfanilimide to form a diazo compound. This compound then couples with n-1-napthylenediamine dihydrochloride to form a reddish/purple azo dye and is read colorimetrical at 540 nm.  Nitrate concentration is obtained by subtracting the corresponding nitrite value from the NO3- + NO2- concentration.  The color development chemistry is the same as that used in Nitrite.  Instrumentation:  SKALAR San-Plus continuous flow autoanalyzer.

ii)  Method Reference(s):

EPA 600/R-97/072 Method 353.4. Determination of Nitrate and Nitrite in Estuarine and Coastal Waters by Gas Segmented Flow Colorimetric Analysis.  In: Methods for the  Determination of Chemical Substances in Marine and Estuarine Environmental Matrices - 2nd Edition.  National Exposure Research Laboratory, Office of Research and Development  U.S. EPA, Cincinnati, Ohio 45268.

U.S. EPA. 1974 Methods for Chemical Analysis of Water and Wastes, pp. 207 -212.

Wood, E.D., F.A.G. Armstrong and F.A. Richards. 1967. Determination of nitrate in seawater by cadmium-copper reduction to nitrite. J. Mar. Biol. Assoc. U.K. 47: 23.

Grasshoff, K., M. Ehrhardt and K. Kremling. 1983. Methods of Seawater Analysis.  Verlag  Chemie, Federal Republic of Germany. 419 pp.

iii)  Preservation Method:  Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis.  Maximum holding time is 28 days.

c)  Parameter: NO2F

i) Method Summary:  An adaptation of the diazotization method. Under acidic conditions, nitrite ion reacts with sulfanilimide to yield a diazo compound that couples with N-1-napthylethylenediamine dihydrochloride to form a soluble dye that is measured colorimetrically at 540nm.  Instrumentation: SKALAR San-Plus continuous flow autoanalyzer.

ii)  Method Reference(s):

EPA 600/R-97/072 Method 353.4. Determination of Nitrate and Nitrite in Estuarine and Coastal Waters by Gas Segmented Flow Colorimetric Analysis.  In: Methods for the  Determination of Chemical Substances in Marine and Estuarine Environmental Matrices - 2nd Edition.  National Exposure Research Laboratory, Office of Research and Development  U.S. EPA, Cincinnati, Ohio 45268.

U.S. EPA. 1974 Methods for Chemical Analysis of Water and Wastes, pp. 207 -212.

Grasshoff, K., M. Ehrhardt and K. Kremling. 1983. Methods of Seawater Analysis.  Verlag  Chemie, Federal Republic of Germany. 419 pp.

iii)  Preservation Method:  Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis.  Maximum holding time is 28 days.

d)  Parameter: TDN

- Method Summary:  The sample is autoclaved in the presence of alkaline potassium persulfate.  Following digestion, the sample is then buffered and analyzed for nitrate.  It should be noted that this is an adaption of D’Elia’s method of 1977.

ii)  Method Reference(s):

D’Elia, C.F., P.A. Steudler, and N. Corwin. 1977.  Determination of Total Nitrogen in Aqueous Samples using Persulfate Digestion.  Limnology and Oceanography 22: 760-764.

EPA 600/R-97/072 Method 353.4. Determination of Nitrate and Nitrite in Estuarine and Coastal Waters by Gas Segmented Flow Colorimetric Analysis.  In: Methods for the  Determination of Chemical Substances in Marine and Estuarine Environmental Matrices - 2nd Edition.  National Exposure Research Laboratory, Office of Research and Development, U.S. EPA, Cincinnati, Ohio 45268.

iii)  Preservation Method:  Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis.  Maximum holding time is 28 days.

e)  Parameter: PO4F

i)  Method Summary:  Ammonium molybdate and antimony potassium tartrate react in a sulfuric acid environment to form an antimony-phospho-molybdo complex, which is reduced to a blue colored complex by ascorbic acid.  Reaction is heat catalizyed at 40C and measured colorimetrically at 880nm. Instrumentation: SKALAR San-Plus continuous flow autoanalyzer.

ii)  Method Reference(s):

SKALAR Method: O-Phosphate/Total Phosphate Catnr.  503-365.1 Issue 042993/MH/93-Demo1.

EPA 600/R-97/072 Method 365.5. Determination of Orthophosphate in Estuarine and Coastal Waters by Automated Colorimetric Analysis.  In: Methods for the  Determination of Chemical Substances in Marine and Estuarine Environmental Matrices - 2nd Edition.  National Exposure Research Laboratory, Office of Research and Development . U.S. EPA, Cincinnati, Ohio 45268.

Murphy, J. and J.P. Riley. 1962. A modified single solution method for the determination of phosphate in natural waters. Analytica Chim. Acta 27 : 31-36.

iii)  Preservation Method:  Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis.  Maximum holding time is 28 days.

f)  Parameter: TDP

i)  Method Summary:  The sample is autoclaved in the presence of alkaline potassium persulfate.  Following digestion, the sample is then buffered and analyzed for orthophosphate.  It should be noted that this is an adaptation of D’Elia’s method of 1977.

ii)  Method Reference(s):

EPA 600/R-97/072 Method 365.5 Determination of Orthophosphate in Estuarine and Coastal Waters by Automated Colorimetric Analysis.  In: Methods for the  Determination of Chemical Substances in Marine and Estuarine Environmental Matrices - 2nd Edition.  National Exposure Research Laboratory, Office of Research and Development . U.S. EPA, Cincinnati, Ohio 45268.

D’Elia, C.F., P.A. Steudler, and N. Corwin. 1977.  Determination of Total Nitrogen in Aqueous Samples using Persulfate Digestion.  Limnology and Oceanography 22: 760-764.

iii) Preservation Method:  Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis.  Maximum holding time is 28 days.

g)  Parameter: CHLA_N

i) Method Summary:  The method used requires filtering a known quantity of water through a glass fiber filter (4.7 cm GF/F).  This filter is later ground with a tissue grinder made of teflon/glass.  Approximately 1-3 ml of 90% acetone is added to the filter before grinding.  Acetone is also used to wash the filter into 17 x 150 test tube with tight fitting cap.  The sample is steeped at least 2 hours and not exceeding 24 hours at 4oC, in the dark. The samples are centrifuged and read on fluorometer.  If the samples cannot be read within that time period, storage in the freezer at -20oC for a few days is acceptable.  Instrumentation: Turner Designs TD-700 fluorometer.

iii)  Method Reference(s):

Strickland, J.D.H., and Parson, T.R. 1972. A Practical Handbook of Seawater Analysis.  Fish. Res. Bd. Canada 167:310.

TD-700 Laboratory Fluorometer Operating Manual.  Version 1.8.  July 7, 1999.  Turner Designs, 845 West Maude Avenue, Sunnyvale, CA 94086.

EPA /600/ R-97/072 -  Method 445.0.  In Vitro Determination of Chlorophyll a and  Pheophytin a in Marine and Freshwater Algae by Fluorescence.   Methods for the Determination of Chemical Substances in Marine and Estuarine Environmental Matrices   Revision 1.2.  September 1997.

Using the Turner Designs Model 10 Analog, The 10AU Digital, Or the TD-700 Fluorometer with EPA Method 445.0.  January 19, 1999.  Turner Designs, 845 West Maude Avenue, Sunnyvale, CA 94086.

iii) Preservation Method:  Samples are immediately filtered with a 0.7 m glass fiber filter upon return to the laboratory.  The filter is drawn dry, folded, sealed in an aluminum foil packet and stored at –20 C until analysis.  Maximum holding time is 28 days.

h)  Parameter: PHEA

i) Method Summary:  The method used requires filtering a known quantity of water through a glass fiber filter (4.7 cm GF/F).  This filter is later ground with a tissue grinder made of teflon/glass.  Approximately 1-3 ml of 90% acetone is added to the filter before grinding.  Acetone is also used to wash the filter into 17 x 150 test tube with tight fitting cap.  The sample is steeped at least 2 hours and not exceeding 24 hours at 4oC, in the dark. The samples are centrifuged and read on fluorometer.  For pheophytin measurements, the sample is acidified and read again.  If the samples cannot be read within that time period, storage in the freezer at -20oC for a few days is acceptable.  Instrumentation: Turner Designs TD-700 fluorometer.

ii)  Method Reference(s):

Strickland, J.D.H., and Parson, T.R. 1972. A Practical Handbook of Seawater Analysis.  Fish. Res. Bd. Canada 167:310.

TD-700 Laboratory Fluorometer Operating Manual.  Version 1.8.  July 7, 1999.  Turner Designs, 845 West Maude Avenue, Sunnyvale, CA 94086.

EPA /600/ R-97/072 -  Method 445.0.  In Vitro Determination of Chlorophyll a and  Pheophytin a in Marine and Freshwater Algae by Fluoresence.   Methods for the Determination of Chemical Substances in Marine and Estuarine Environmental Matrices   Revision 1.2.  September 1997.

Using the Turner Designs Model 10 Analog, The 10AU Digital, Or the TD-700 Fluorometer with EPA Method 445.0.  January 19, 1999.  Turner Designs, 845 West Maude Avenue, Sunnyvale, CA 94086.

iii)  Preservation Method:  Samples are immediately filtered with a 0.7 m glass fiber filter upon return to the laboratory.  The filter is drawn dry, folded, sealed in an aluminum foil packet and stored at –20 C until analysis.  Maximum holding time is 28 days.

14)  Field and Laboratory QAQC programs

  - Precision

i)  Field variability – CBNERRVA collects two successive grab samples for the monthly grab sample program.

ii)  Laboratory variability – The VIMS Analytical Service Center for Nutrients analyzes a laboratory duplicate once for every ten samples.

iii)  Inter-organizational splits – None

  - Accuracy

i)  Sample spikes.  The VIMS Analytical Service Center for Nutrients analyzes a matrix spike once for every ten samples.

ii)  Standard reference material analysis.  CBNERRVA has participated in the 2004, 2005, 2006, and 2007 NOAA/NERR Interlaboratory Comparison Studies.  VIMS Analytical Service Center results have all fallen within acceptable control limits.  For details, contact the Reserve Research Coordinator.

    - Cross calibration exercises - None

15) QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range*

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

*The -4 Outside Low Sensor Range flag was added to the 2007 dataset in August of 2011.  See the Other Remarks section for more details.

16)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data rejected due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Value calculated with a value that is below the MDL

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N	from the north

NNE	from the north northeast

NE	from the northeast

ENE	from the east northeast

E	from the east

ESE	from the east southeast

SE	from the southeast

SSE	from the south southeast

S	from the south

SSW	from the south southwest

SW	from the southwest

WSW	from the west southwest

W	from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17)  Other remarks/notes

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded with -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

*The 2007 dataset was updated on August of 2011 to include the -4 Outside Low Sensor Range flag.  The 2007 data published prior to that time used the -3 Rejected data flag with the SBL and SCB QAQC codes to indicate that data were below the minimum detection limit.  These flag code combinations were all replaced with the -4 SBL or SCB update as mandated by the Data Management Committee.

- Precipitation data for the sampling date and three days prior to the monthly grab and diel sampling programs.  Data are presented for two meteorological stations, locations are the Sweet Hall Marsh reserve and the Taskinas Creek reserve.  Data units are in mm.

| Type of Sampling | Previous Dates | Sweet Hall | Taskinas |
| --- | --- | --- | --- |
| Monthly Grab | 01/08/2007 | 6.6 | 5.5 |
| Monthly Grab | 01/09/2007 | 0.0 | 0.0 |
| Monthly Grab | 01/10/2007 | 0.0 | 0.0 |
| Monthly Grab | 01/11/2007 | 0.0 | 0.0 |
| Diel | 01/09/2007 | 0.0 | 0.0 |
| Diel | 01/10/2007 | 0.0 | 0.0 |
| Diel | 01/11/2007 | 0.0 | 0.0 |
| Diel | 01/12/2007 | 0.0 | 0.0 |
| Monthly Grab | 02/10/2007 | 0.0 | 0.0 |
| Monthly Grab | 02/11/2007 | 0.0 | 0.0 |
| Monthly Grab | 02/12/2007 | 0.0 | 0.0 |
| Monthly Grab | 02/13/2007 | 5.6 | 2.1 |
| Diel | 02/11/2007 | 0.0 | 0.0 |
| Diel | 02/12/2007 | 0.0 | 0.0 |
| Diel | 02/13/2007 | 5.6 | 2.1 |
| Diel | 02/14/2007 | 26.2 | 20.1 |
| Monthly Grab | 03/10/2007 | 0.0 | 0.0 |
| Monthly Grab | 03/11/2007 | 0.0 | 0.0 |
| Monthly Grab | 03/12/2007 | 0.0 | 0.0 |
| Monthly Grab | 03/13/2007 | 0.5 | 0.5 |
| Diel | 03/11/2007 | 0.0 | 0.0 |
| Diel | 03/12/2007 | 0.0 | 0.0 |
| Diel | 03/13/2007 | 0.5 | 0.5 |
| Diel | 03/14/2007 | 0.0 | 0.0 |
| Monthly Grab | 04/07/2007 | 8.9 | 9.3 |
| Monthly Grab | 04/08/2007 | 0.0 | 0.0 |
| Monthly Grab | 04/09/2007 | 0.0 | 0.0 |
| Monthly Grab | 04/10/2007 | 0.0 | 0.0 |
| Diel | 04/08/2007 | 0.0 | 0.0 |
| Diel | 04/09/2007 | 0.0 | 0.0 |
| Diel | 04/10/2007 | 0.0 | 0.0 |
| Diel | 04/11/2007 | 0.0 | 0.0 |
| Monthly Grab | 05/08/2007 | 0.0 | 0.3 |
| Monthly Grab | 05/09/2007 | 0.8 | 0.8 |
| Monthly Grab | 05/10/2007 | 0.0 | 0.0 |
| Monthly Grab | 05/11/2007 | 0.0 | 0.0 |
| Diel | 05/07/2007 | 0.0 | 0.0 |
| Diel | 05/08/2007 | 0.0 | 0.3 |
| Diel | 05/09/2007 | 0.8 | 0.8 |
| Diel | 05/10/2007 | 0.0 | 0.0 |
| Monthly Grab | 06/04/2007 | 0.3 | 1.6 |
| Monthly Grab | 06/05/2007 | 0.3 | 1.1 |
| Monthly Grab | 06/06/2007 | 3.3 | 4.7 |
| Monthly Grab | 06/07/2007 | 0.5 | 0.5 |
| Diel | 06/05/2007 | 0.3 | 1.1 |
| Diel | 06/06/2007 | 3.3 | 4.7 |
| Diel | 06/07/2007 | 0.5 | 0.5 |
| Diel | 06/08/2007 | 4.2 | 6.3 |
| Monthly Grab | 07/06/2007 | 0.0 | 0.0 |
| Monthly Grab | 07/07/2007 | 0.0 | 0.0 |
| Monthly Grab | 07/08/2007 | 0.0 | 0.0 |
| Monthly Grab | 07/09/2007 | 0.0 | 0.0 |
| Diel | 07/07/2007 | 0.0 | 0.0 |
| Diel | 07/08/2007 | 0.0 | 0.0 |
| Diel | 07/09/2007 | 0.0 | 0.0 |
| Diel | 07/10/2007 | 0.0 | 0.0 |
| Monthly Grab | 08/03/2007 | 0.0 | 23.3 |
| Monthly Grab | 08/04/2007 | 0.0 | 0.0 |
| Monthly Grab | 08/05/2007 | 6.9 | 9.3 |
| Monthly Grab | 08/06/2007 | 0.0 | 0.0 |
| Diel | 08/04/2007 | 0.0 | 0.0 |
| Diel | 08/05/2007 | 6.9 | 9.3 |
| Diel | 08/06/2007 | 0.0 | 0.0 |
| Diel | 08/07/2007 | 0.0 | 0.0 |
| Monthly Grab | 09/02/2007 | 0.0 | 0.0 |
| Monthly Grab | 09/03/2007 | 0.0 | 0.0 |
| Monthly Grab | 09/04/2007 | 0.0 | 0.0 |
| Monthly Grab | 09/05/2007 | 0.0 | 0.0 |
| Diel | 09/03/2007 | 0.0 | 0.0 |
| Diel | 09/04/2007 | 0.0 | 0.0 |
| Diel | 09/05/2007 | 0.0 | 0.0 |
| Diel | 09/06/2007 | 0.0 | 0.0 |
| Monthly Grab | 10/19/2007 | 0.0 | 0.6 |
| Monthly Grab | 10/20/2007 | 0.0 | 0.0 |
| Monthly Grab | 10/21/2007 | 0.0 | 0.0 |
| Monthly Grab | 10/22/2007 | 0.3 | 0.5 |
| Diel | 10/20/2007 | 0.0 | 0..0 |
| Diel | 10/21/2007 | 0.0 | 0.0 |
| Diel | 10/22/2007 | 0.3 | 0.5 |
| Diel | 10/23/2007 | 0.0 | 0.0 |
| Monthly Grab | 11/16/2007 | 0.0 | 0.0 |
| Monthly Grab | 11/17/2007 | 0.0 | 0.0 |
| Monthly Grab | 11/18/2007 | 0.0 | 0.0 |
| Monthly Grab | 11/19/2007 | 1.0 | 0.5 |
| Diel | 11/17/2007 | 0.0 | 0.0 |
| Diel | 11/18/2007 | 0.0 | 0.0 |
| Diel | 11/19/2007 | 1.0 | 0.5 |
| Diel | 11/20/2007 | 0.0 | 0.0 |
| Monthly Grab | 12/15/2007 | 9.4 | 14.5 |
| Monthly Grab | 12/16/2007 | 10.2 | 14.6 |
| Monthly Grab | 12/17/2007 | 0.0 | 0.0 |
| Monthly Grab | 12/18/2007 | 0.8 | 0.0 |
| Diel | 12/16/2007 | 10.2 | 14.6 |
| Diel | 12/17/2007 | 0.0 | 0.0 |
| Diel | 12/18/2007 | 0.8 | 0.0 |
| Diel | 12/19/2007 | 0.8 | 0.0 |

b)  For the following dates and times, reported data is below the lowest standard concentration for PO4.  PO4 and calculated DOP data suspect, data retained.

Claybank:

04/10/2007  09:30:00

04/10/2007  09:31:00

05/11/2007  08:18:00

05/11/2007  08:19:00

Catlett Island:

04/10/2007 09:10:00

04/10/2007 09:11:00

05/11/2007 08:03:00

Goodwin:

04/10/2007 08:05:00

04/10/2007 08:06:00

Taskinas:

04/11/2007 02:15:00

04/11/2007 04:45:00

05/09/2007 13:15:00

05/09/2007 15:45:00

05/09/2007 20:45:00

05/10/2007 04:15:00

York Bridge:

05/11/2007 07:51:00

05/11/2007 07:52:00

c)  For the following dates and times, reported data is below the lowest standard concentration for TDP.  TDP and calculated DOP data suspect, data retained.

Claybank:

04/10/2007 09:30:00

04/10/2007 09:31:00

05/11/2007 08:18:00

05/11/2007 08:19:00

Catlett Island:

04/10/2007 09:11:00

05/11/2007 08:03:00

05/11/2007 08:04:00

Goodwin:

04/10/2007 08:05:00

04/10/2007 08:06:00

05/11/2007 07:30:00

05/11/2007 07:31:00

Taskinas:

05/09/2007 15:45:00

05/09/2007 18:15:00

05/09/2007 20:45:00

05/10/2007 01:45:00

05/10/2007 04:15:00

05/10/2007 06:45:00

York Bridge:

04/10/2007 08:39:00

04/10/2007 08:40:00

05/11/2007 07:51:00

05/11/2007 07:52:00

d)  For the following dates and times, reported data is below the lowest standard concentration for NH4.  NH4 and calculated DIN data suspect, data retained.

Sweet Hall:

05/11/2007 11:58:00

e)  For the following dates and times, reported data is below the lowest standard concentration for NO23. NO23 and calculated DIN data suspect, data retained.

Goodwin:

05/11/2007 07:30:00

05/11/2007 07:31:00

f)  For the following dates and times, reported data is below the lowest standard concentration for CHLA. Data suspect, data retained.

Goodwin:

05/11/2007 07:30:00

05/11/2007 07:31:00

g)  For the following dates and times, reported data is below the lowest standard concentration for PHEA. Data suspect, data retained.

Claybank:

05/11/2007 08:19:00

Catlett Island:

05/11/2007 08:03:00

05/11/2007 08:04:00

Goodwin:

05/11/2007 07:30:00

Sweet Hall:

05/11/2007 11:58:00

05/11/2007 11:59:00

Taskinas:

05/09/2007 10:45:00

05/09/2007 23:15:00

h)  For the following dates and times, reported data is below the lowest standard concentration for TDN. TDN and calculated DON data suspect, data retained.

Claybank:

04/10/2007 09:30:00

04/10/2007 09:31:00

Catlett Island:

04/10/2007 09:10:00

04/10/2007 09:11:00

Goodwin:

04/10/2007 08:05:00

04/10/2007 08:06:00

06/07/2007 06:36:00

06/07/2007 06:37:00

Taskinas:

03/13/2007 12:55:00

03/13/2007 15:25:00

03/13/2007 17:55:00

03/13/2007 20:25:00

03/13/2007 22:55:00

03/14/2007 01:25:00

03/14/2007 03:55:00

03/14/2007 06:25:00

03/14/2007 08:55:00

03/14/2007 11:25:00

03/14/2007 13:55:00

04/10/2007 09:16:00

04/10/2007 09:17:00

04/10/2007 13:45:00

04/10/2007 16:15:00

04/10/2007 18:45:00

04/10/2007 21:15:00

04/10/2007 23:45:00

04/11/2007 02:15:00

04/11/2007 04:45:00

04/11/2007 07:15:00

04/11/2007 09:45:00

04/11/2007 12:15:00

York Bridge:

04/10/2007 08:39:00

04/10/2007 08:40:00