Delaware (DEL) NERR Water Quality Metadata

January- December 2012

Latest Revision: September 22, 2015

I. Data Set & Research Descriptors

1.  Principal investigator(s) & contact persons:

Robert Scarborough, Environmental Scientist (P.I.), email: Bob.Scarborough@state.de.us

Michael G. Mensinger, Environmental Scientist, email: Mike.Mensinger@state.de.us

Delaware Coastal Programs/National Estuarine Research Reserve Program

Dept. of Natural Resources & Environmental Control

Division of Soil & Water Conservation

818 Kitts Hummock Road

Dover, Delaware 19901

Phone: (302) 739-3436

2.  Entry Verification:

Deployment data are uploaded from the YSI data logger to a Personal Computer (IBM compatible).  Files are exported from EcoWatch in a comma-delimited format (.CDF) and uploaded to the CDMO where they undergo automated primary QAQC; automated depth/level corrections for changes in barometric pressure (cDepth or cLevel parameters); and become part of the CDMO’s online provisional database.  All pre- and post-deployment data are removed from the file prior to upload.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve for secondary QAQC where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, remove any overlapping deployment data, append files, and export the resulting data file for upload to the CDMO.  Upload after secondary QAQC results in ingestion into the database as provisional plus data, recalculation of cDepth or cLevel parameters, and finally tertiary QAQC by the CDMO and assimilation into the CDMO’s authoritative online database.  Where deployment overlap occurs between files, the data produced by the newly calibrated sonde is generally accepted as being the most accurate.  For more information on QAQC flags and codes, see Sections 11 and 12.

Michael G. Mensinger is responsible for the acquisition, editing, and archiving of the data. Data archival takes place on both the NERRS Centralized Data Management Office (CDMO) and State of Delaware servers.

3.  Research Objectives:

The principal objective of this study is to record long-term water quality data for Delaware Bay in order to observe any physical changes or trends in water quality over time. The four sites chosen for monitoring will assist in understanding the impacts of both urban and agricultural impacts on the respective watersheds.  Scotton Landing, Lebanon Landing, and Division Street are located within the St. Jones River watershed, while the Blackbird Landing component is located in the Blackbird Creek watershed. The St. Jones River is impacted by urbanization upstream and extensive agricultural activity throughout the basin. The Blackbird Creek is one of the least impacted riverine systems in Delaware, although development pressures are currently increasing. Measurements are taken every 15 minutes over two week collecting periods at the Scotton Landing (SL), Blackbird Landing (BL), Lebanon Landing (LL), and Division Street (DS) sites.

4.  Research Methods:

The YSI data sonde monitoring program commenced in July 1995 at Scotton Landing (St. Jones River) and Blackbird Landing (Blackbird Creek).  In June 1996 Penrose Branch (St. Jones River Basin) was added to the monitoring program. The Penrose Branch site was later discontinued on December 26, 2001. In January 2002 the Lebanon Landing and Division Street sites were added to the monitoring program.  Prior to deployment, a 4" diameter schedule 80 PVC pipe was mounted to a piling at Scotton Landing and a bridge at Blackbird Landing using stainless steel brackets and bolts.  These methods were also used to install a pipe to a piling at the Lebanon Landing dock and the pumping station bank wall at Division Street. Each YSI 6600 data sonde is lowered into the PVC pipe containing eight (8) two-inch diameter holes cut out at the bottom, affording the probes direct contact with the water column.  A large stainless steel bolt was placed at the bottom end of the PVC pipe which maintained each YSI 6600 data sonde at each site exactly one foot (30 cm) above the river bottom.

Every 15 minutes over a two-week period, measurements of specific conductivity, salinity, percent saturation, dissolved oxygen, temperature, pH, and water level are recorded.  Approximately seven days after the deployment of each YSI 6600, the sonde and sensors undergo an in-situ cleaning with a Kimwipes (sometimes dipped in methanol depending on fouling severity). Two-week sampling intervals were selected to minimize the effects of bio-fouling. At the end of each two-week interval, the deployed YSI 6600 data sonde is brought back to the office for downloading, cleaning, and recalibration. A second YSI 6600 data sonde is immediately deployed for the next two week interval when the previous unit is retrieved. This method alleviates the potential for lost data during calibration and data download times. Calibrations performed include: dissolved oxygen using 100% air saturated water, specific conductivity using Oakton 12,880 uS standard, turbidity using YSI 6073G Turbidity Standard at 126 NTU and distilled water at 0 NTU, pH using Fisher pH 4.00, 7.00 and 10.00 buffers, and depth.  These procedures are carried out according to the methods described in the YSI Model 6600 Operating and Service Manual and NERR Water Quality SOP. Data files undergo automated QA/QC through the CDMO and are then futher QA/QC checked using Excel macros provided by the CDMO which assist in graphing and reviewing the data.

Depth probe calibration procedures changed during 2007 and involved the inclusion of a depth offset based on data acquired from RTK unit work at each site in order to achieve vertical control. This calibration change greatly augments the depth data set by referencing it to a known vertical datum, in this case NAVD88. Depth data changed as a result of this work on the following dates/times at the following stations: Scotton Landing (10/16/2007, 09:45:00 EST), Lebanon Landing (10/16/2007, 14:00:00 EST), Division Street (10/16/2007, 13:30:00 EST), and Blackbird Landing (11/01/2007, 10:45:00 EST). All depth data collected in 2012 is considered “water level” data as opposed to the historic “depth data” collected before late 2007.

A Sutron Sat-Link2 transmitter was installed at the Scotton Landing station on 11/16/05 and transmits data to the NOAA GOES satellite, NESDIS ID #3B00E488. A second Sutron Sat-Link2 transmitter was installed at the Lebanon Landing station on 11/06/06 and previously transmitted data to the NOAA GOES satellite, NESDIS ID #3B03C56A. The Lebanon Landing telemetry station was destroyed by acts of vandalism in 2009, which resulted in its decommission as a telemetry site.  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

5.  Site location and character:

The Delaware National Estuarine Research Reserve is comprised of two component sites, the St. Jones River and Blackbird Creek components.  Both components are located along the Delaware Bay Coast.   The St. Jones River Component is located in central Kent County Delaware, east of the state capital city, Dover.  The Blackbird Creek component is located in the unincorporated area of Southern New Castle County.  There are four sampling sites, three located in the St. Jones component and one in the Blackbird Creek component.

1) Scotton Landing – is located in the Lower St. Jones River at the Scotton Landing Public Fishing Pier, slightly upstream of Delaware Route 113.  The river is 22.3 km long (mainstream linear dimension), has an average depth of 4m MHW and the width is 50 m. At the sampling site, the depth is 3.2 m MHW and the width is 40 m. The sediment is clayey silt with no bottom vegetation. The St. Jones watershed drainage area is 228.1 km2 (22810 ha) and Scotton Landing’s drainage area is 196.2 km2 (19620 ha). The site is influenced by freshwater runoff from the relatively urbanized area upstream. Pollutants in the area include PCB's, PAH's, and pesticides.

Salinity ranges from 1- 30 ppt.

Tidal Range: Spring Mean (m) – 1.26

Neap Mean (m) – 1.13

Position:  Latitude     39 degree 05'  05.9160"  N

Longitude  75 degree 27'  38.1049"  W

2) Blackbird Landing - is located in the upper Blackbird Creek at Blackbird Landing Road.  The creek is 25.8 km long (mainstream linear dimension), has an average depth of 3 m MHW, and an average width of 90 m. At the sampling site, the depth is 1.8 m MHW and width is 110 m. The sediment is silty clay with no bottom vegetation. The Blackbird watershed drainage area is 90.6 km2 (9060 ha) and Blackbird Landing’s drainage area is 48.3 km2 (4830 ha). The site is influenced by freshwater runoff from unimpacted forested areas intermixed with agricultural land uses and a small amount of low-density development.  There is very little pollutant presence in the area.

Salinity ranges from 0-9 ppt.

Tidal Range: Spring Mean (m) – 1.12

Neap Mean (m) – 1.13

Position:  Latitude     39 degree 23'  19.5196"  N

Longitude  75 degree 38'  09.5882"  W

3) Lebanon Landing- is located in the mid portion of the St. Jones River at the Lebanon Landing Public Fishing Pier, farther upstream from the Scotton Landing monitoring site. The St. Jones River is 22.3 km long (mainstream linear dimension), has an average depth of 4m MHW and the width is 50 m. At the sampling site, the depth is 3.0 m MHW and the width is 28 m. The sediment is clayey silt with no bottom vegetation. The St. Jones watershed drainage area is 228.1 km2 (22810 ha) and Lebanon Landing’s drainage area is 171.6 km2 (17160 ha). The site is influenced by freshwater runoff from the relatively urbanized area upstream. Pollutants in the area include PCB's, PAH's, and pesticides.

Salinity ranges from 0 to 28 ppt.

Tidal Range: Spring Mean (m) – 0.855

Neap Mean (m) –  0.671

Position: Latitude      39 degree 06'  51.8"  N

Longitude   75 degree 29'  57.1"  W

4) Division Street- is located in the upper portion of the St. Jones River near the USGS station on Division Street. The site is influenced by runoff from the urbanized surroundings. The St. Jones River is 22.3 km long (mainstream linear dimension), has an average depth of 4m MHW and the width is 50 m. At the sampling site, the depth is 0.6m MHW and the width is 26 m. The sediment is clayey silt with no bottom vegetation. The site is fresh water and is influenced by urban freshwater runoff. The St. Jones watershed drainage area is 228.1 km2 (22810 ha) and Division Street’s drainage area is 81.2 km2 (8120 ha). Pollutants in the area include PCB's, PAH's, and pesticides.

Salinity Range: Fresh water (0.1 ppt.)

Tidal Range: Not Applicable, freshwater

Position: Latitude      39 degree 09'  49.4"  N

Longitude   75 degree 31'  08.7"  W

6.  Data collection period:

Data collection began for these sites on the following dates:

a) Scotton Landing: July 1, 1995

b) Blackbird Landing: August 1, 1995

c) Lebanon Landing: January 01, 2002

d) Division Street: January 31, 2002

All sampling is ongoing. The deployment/retrieval dates and times for 2012 are as follows:

Scotton Landing

BEGAN			ENDED

12/20/2011, 10:15:00		01/17/2012, 14:00:00

01/17/2012, 14:15:00		02/16/2012, 13:00:00

02/16/2012, 13:15:00		03/06/2012, 09:15:00

03/06/2012, 09:30:00		03/22/2012, 12:45:00

03/22/2012, 13:00:00		04/05/2012, 07:45:00

04/05/2012, 08:00:00		04/12/2012, 01:15:00

04/16/2012, 12:30:00		04/30/2012, 04:00:00

05/09/2012, 13:45:00		05/31/2012, 08:15:00

05/31/2012, 08:30:00		06/14/2012, 12:30:00

06/14/2012, 12:45:00		06/26/2012, 12:00:00

06/26/2012, 12:15:00		07/10/2012, 12:45:00

07/10/2012, 13:00:00		07/24/2012, 08:00:00

07/24/2012, 08:15:00		08/16/2012, 09:30:00

08/16/2012, 09:45:00		09/05/2012, 14:15:00

09/05/2012, 14:30:00		09/19/2012, 07:30:00

09/19/2012, 07:45:00		10/04/2012, 14:15:00

10/04/2012, 14:30:00		10/26/2012, 11:00:00

10/26/2012, 11:15:00		11/19/2012, 14:15:00

11/19/2012, 14:30:00		12/12/2012, 13:45:00

12/12/2012, 14:00:00		01/03/2012, 09:00:00

Lebanon Landing

BEGAN			ENDED

12/20/2011, 10:30:00		01/18/2012, 13:45:00

01/18/2012, 14:00:00		02/14/2012, 12:30:00

02/14/2012, 12:45:00		03/06/2012, 09:30:00

03/06/2012, 09:45:00		03/22/2012, 13:00:00

03/22/2012, 13:15:00		04/05/2012, 08:00:00

04/05/2012, 08:15:00		04/16/2012, 12:30:00

04/16/2012, 13:00:00		05/10/2012, 08:15:00

05/10/2012, 08:30:00		05/31/2012, 08:30:00

05/31/2012, 08:45:00		06/14/2012, 12:45:00

06/14/2012, 13:00:00		06/26/2012, 11:30:00

06/26/2012, 11:45:00		07/10/2012, 13:00:00

07/10/2012, 13:15:00		07/24/2012, 08:45:00

07/24/2012, 09:00:00		08/16/2012, 14:15:00

08/16/2012, 14:30:00		09/05/2012, 13:45:00

09/05/2012, 14:00:00		09/19/2012, 08:00:00

09/19/2012, 08:15:00		10/04/2012, 14:30:00

10/04/2012, 14:45:00		10/18/2012, 10:15:00

10/18/2012, 10:30:00		10/26/2012, 11:30:00

10/26/2012, 11:45:00		11/19/2012, 14:30:00

11/19/2012, 14:45:00		12/12/2012, 14:00:00

12/12/2012, 14:15:00		01/03/2012, 09:15:00

Division Street

BEGAN			ENDED

12/29/2011, 09:30:00		01/17/2012, 09:15:00

01/17/2012, 09:30:00		02/14/2012, 12:15:00

02/14/2012, 12:30:00		03/06/2012, 10:00:00

03/06/2012, 10:15:00		03/22/2012, 13:30:00

03/22/2012, 13:45:00		04/05/2012, 08:30:00

04/05/2012, 08:45:00		04/16/2012, 13:00:00

04/16/2012, 13:15:00		05/10/2012, 07:45:00

05/10/2012, 08:00:00		05/31/2012, 08:45:00

05/31/2012, 09:00:00		06/15/2012, 07:45:00

06/15/2012, 08:00:00		06/26/2012, 13:00:00

06/26/2012, 13:15:00		07/10/2012, 13:30:00

07/10/2012, 13:45:00		07/24/2012, 09:15:00

07/24/2012, 09:30:00		08/16/2012, 10:00:00

08/16/2012, 10:15:00		09/06/2012, 11:45:00

09/06/2012, 12:00:00		09/18/2012, 13:00:00

09/18/2012, 13:15:00		10/04/2012, 15:00:00

10/04/2012, 15:15:00		10/18/2012, 10:30:00

10/18/2012, 10:45:00		10/26/2012, 12:00:00

10/26/2012, 12:15:00		11/15/2012, 16:15:00

11/19/2012, 15:15:00		12/12/2012, 14:15:00

12/12/2012, 14:30:00		01/03/2012, 09:30:00

Blackbird Landing

BEGAN			ENDED

12/16/2011, 11:15:00		01/06/2012, 09:45:00

01/06/2012, 10:00:00		01/20/2012, 10:15:00

01/20/2012, 10:30:00		02/17/2012, 13:15:00

02/17/2012, 13:30:00		03/09/2012, 11:15:00

03/09/2012, 11:30:00		03/23/2012, 09:30:00

03/23/2012, 09:45:00		04/03/2012, 07:30:00

04/03/2012, 07:45:00		04/20/2012, 08:30:00

04/20/2012, 08:45:00		05/11/2012, 08:45:00

05/11/2012, 09:00:00		05/25/2012, 09:30:00

05/25/2012, 09:45:00		06/08/2012, 09:45:00

06/08/2012, 10:00:00		06/22/2012, 09:45:00

06/22/2012, 10:00:00		07/06/2012, 08:30:00

07/06/2012, 08:45:00		07/20/2012, 08:45:00

07/20/2012, 09:00:00		08/03/2012, 08:30:00

08/03/2012, 08:45:00		08/17/2012, 08:45:00

08/17/2012, 09:00:00		08/31/2012, 09:30:00

08/31/2012, 09:45:00		09/14/2012, 09:15:00

09/14/2012, 09:30:00		10/01/2012, 09:30:00

10/01/2012, 09:45:00		10/12/2012, 07:45:00

10/12/2012, 08:00:00		10/26/2012, 07:45:00

10/26/2012, 08:00:00		11/09/2012, 09:30:00

11/09/2012, 09:45:00		12/07/2012, 10:15:00

12/07/2012, 10:30:00		12/28/2012, 09:30:00

12/28/2012, 09:45:00		01/18/2013, 09:30:00

7. Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR water quality data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format.

8.  Associated researchers and projects:

An extensive stormwater sampling and hydrodynamic modeling project is underway in the upper reaches of the St. Jones River Basin.  This project includes flow weighted stormwater sampling to quantify pollutant loads from each of the major land uses found in the Basin. An additional stormwater sampling program is underway to analyze the impact of agricultural BMP's at sites in the Blackbird Creek Watershed, St. Jones River Watershed. A joint effort underway through NOAA's Center for Coastal Ecosystem Health and the DNERR characterization studies will initiate detailed monthly water quality sampling at three sites in each river over a two year period and will quantify the phytoplankton community with collections from the same sites.   This project was scheduled to be initiated in February 1995.(See Section I.1. for research contacts.)

The NERR SWMP Nutrient Monitoring Project has five stations located at the DNERR. The objective of this monitoring program is to provide baseline information on inorganic nutrient and Chla water quality status in the Delaware NERR while also contributing to baseline information nationally. Diel and grab samples are collected monthly at each of the five sites.

The NERR SWMP Meteorological Monitoring Project has one station located at the DNERR. The principal objective is to record long-term meteorological data for the St. Jones component of the Delaware National Estuarine Research Reserve in order to observe any environmental changes or trends over time. The data are also used for specific research studies relating to atmospheric deposition of nutrients and pesticides, and nutrient runoff influences from encroaching urbanization on estuarine systems. The meteorological data also serves a supporting role for the SWMP water quality and nutrient data sets since meteorological conditions directly impact these projects.

II.  Physical Structure Descriptors

9.  Variable sequence, range of measurements, units, resolution, and accuracy: YSI 6600/6600EDS data sonde

The Delaware NERR utilized YSI 6600 EDS sondes at all four stations during 2011. Due to the use of ROX optical DO probes, the EDS brushes were never installed or used on the sondes.

Parameter: Temperature

Units: Celsius (C)

Sensor Type: Thermistor

Model#: 6560

Range: -5 to 45 C

Accuracy: +/-0.15

Resolution: 0.01 C

Parameter: Conductivity

Units: milli-Siemens per cm (mS/cm)

Sensor Type: 4-electrode cell with autoranging

Model#: 6560

Range: 0 to 100 mS/cm

Accuracy: +/--0.5% of reading + 0.001 mS/cm

Resolution: 0.001 mS/cm to 0.1 mS/cm (range dependant)

Parameter: Salinity

Units: parts per thousand (ppt)

Sensor Type: Calculated from conductivity and temperature

Range: 0 to 70 ppt

Accuracy: +/-1.0% of reading or 0.1 ppt, whichever is greater

Resolution: 0.01 ppt

Sensor Type: Optical probe w/ mechanical cleaning

Model#: 6150 ROX

Range: 0 to 500% air saturation

Accuracy:

0-200% air saturation: +/-1% of the reading or 1% air saturation, whichever is greater 200-500% air saturation: +/- 15% or reading

Resolution: 0.1% air saturation

Units: milligrams/Liter (mg/L)

Sensor Type: Optical probe w/ mechanical cleaning

Model#: 6150 ROX

Range: 0 to 50 mg/L

Accuracy:

0-20 mg/L: +/-0.1 mg/l or 1% of the reading, whichever is greater

20 to 50 mg/L: +/-15% of the reading

Resolution: 0.01 mg/L

Parameter: Non-vented Level - Shallow (Depth)

Units: feet or meters (ft or m)

Sensor Type: Stainless steel strain gauge

Range: 0 to 30 ft (9.1 m)

Accuracy: +/- 0.06 ft (0.018 m)

Resolution: 0.001 ft (0.001 m)

Parameter: pH - EDS flat probe

Units: pH units

Sensor Type: Glass combination electrode

Range: 0 to 14 units

Accuracy: +/- 0.2 units

Resolution: 0.01 units

Parameter: Turbidity

Units: nephelometric turbidity units (NTU)

Sensor Type: Optical, 90 degree scatter, with mechanical cleaning

Model#: 6136

Range: 0 to 1000 NTU

Accuracy: ± 2% of reading or 0.3 NTU (whichever is greater)

Resolution: 0.1 NTU

Parameter: Date

Units: month, day, year

Range: 1-12, 1-31, 00-99 (month, day, year)

Accuracy: N/A

Resolution: 1 month, 1 day, 1 year

Parameter: Time

Units: hour, minute, second

Range: 0-24, 0-60, 0-60 (hour, minute, second)

Accuracy: N/A

Resolution: 1 hour, 1 minute, 1 second

Dissolved Oxygen Qualifier (Rapid Pulse / Clark type sensor):

The reliability of dissolved oxygen (DO) data collected with the rapid pulse / Clark type sensor after 96 hours post-deployment for non-EDS (Extended Deployment System) data sondes may be problematic due to fouling which forms on the DO probe membrane during some deployments (Wenner et al. 2001).  Some Reserves utilize the YSI 6600 EDS data sondes, which increase DO accuracy and longevity by reducing the environmental effects of fouling.  Optical DO probes have further improved data reliability.  The user is therefore advised to consult the metadata for sensor type information and to exercise caution when utilizing rapid pulse / Clark type sensor DO data beyond the initial 96-hour time period.  Potential drift is not always problematic for some uses of the data, i.e. periodicity analysis.   It should also be noted that the amount of fouling is very site specific and that not all data are affected.  If there are concerns about fouling impacts on DO data beyond any information documented in the metadata and/or QAQC flags/codes, please contact the Research Coordinator at the specific NERR site regarding site and seasonal variation in fouling of the DO sensor.

Depth Qualifier:

The NERR System-Wide Monitoring Program utilizes YSI data sondes that can be equipped with either vented or non-vented depth/level sensors.  Readings for both vented and non-vented sensors are automatically compensated for water density change due to variations in temperature and salinity; but for all non-vented depth measurements, changes in atmospheric pressure between calibrations appear as changes in water depth.  The error is equal to approximately 1.03 cm for every 1 millibar change in atmospheric pressure, and is eliminated for vented sensors because they are vented to the atmosphere throughout the deployment time interval.

Beginning in 2006, NERR SWMP standard calibration protocol calls for all non-vented depth sensors to read 0 meters at a (local) barometric pressure of 1013.25 mb (760 mm/hg).  To achieve this, each site calibrates their depth sensor with a depth offset number, which is calculated using the actual atmospheric pressure at the time of calibration and the equation provided in the SWMP calibration sheet or digital calibration log.  This offset procedure standardizes each depth calibration for the entire NERR System.  If accurate atmospheric pressure data are available, non-vented sensor depth measurements at any NERR can be corrected.

In 2010, the CDMO began automatically correcting depth/level data for changes in barometric pressure as measured by the Reserve’s associated meteorological station during data ingestion.  These corrected depth/level data are reported as cDepth and cLevel, and are assigned QAQC flags and codes based on QAQC protocols.  Please see sections 11 and 12 for QAQC flag and code definitions.

Salinity Units Qualifier:

In 2013, EXO sondes were approved for SWMP use and began to be utilized by Reserves. While the 6600 series sondes report salinity in parts per thousand (ppt) units, the EXO sondes report practical salinity units (psu). These units are essentially the same and for SWMP purposes are understood to be equivalent, however psu is considered the more appropriate designation. Moving forward the NERR System will assign psu salinity units for all data regardless of sonde type.

Turbidity Qualifier:

In 2013, EXO sondes were approved for SWMP use and began to be utilized by Reserves. While the 6600 series sondes report turbidity in nephelometric turbidity units (NTU), the EXO sondes use formazin nephelometric units (FNU). These units are essentially the same but indicate a difference in sensor methodology, for SWMP purposes they will be considered equivalent.  Moving forward, the NERR System will use FNU/NTU as the designated units for all turbidity data regardless of sonde type. If turbidity units and sensor methodology are of concern, please see the Sensor Specifications portion of the metadata.

10. Coded variable indicator and variable code definitions:

Sampling Station:	Sampling Site Code:	Station Code

Scotton Landing	SL			delslwq

Lebanon Landing	LL			delllwq

Division Street	DS			deldswq

Blackbird Landing	BL			delblwq

11.  QAQC flag definitions:

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is missing and above or below sensor range.  All remaining data are then flagged 0, passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Calculated data: non-vented depth/level sensor correction for changes in barometric pressure

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12.  QAQC code definitions:

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the deployment or YSI datasonde, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below) can be applied to the entire record in the F_Record column.

General Errors

GIC	No instrument deployed due to ice

GIM	Instrument malfunction

GIT	Instrument recording error; recovered telemetry data

GMC 	No instrument deployed due to maintenance/calibration

GNF	Deployment tube clogged / no flow

GOW	Out of water event

GPF	Power failure / low battery

GQR	Data rejected due to QA/QC checks

GSM	See metadata

Corrected Depth/Level Data Codes

GCC	Calculated with data that were corrected during QA/QC

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GCS	Calculated value suspect due to questionable data

GCU 	Calculated value could not be determined due to unavailable 				data

Sensor Errors

SBO	Blocked optic

SCF	Conductivity sensor failure

SCS	Chlorophyll spike

SDF	Depth port frozen

SDG	Suspect due to sensor diagnostics

SDO	DO suspect

SDP	DO membrane puncture

SIC	Incorrect calibration / contaminated standard

SNV	Negative value

SOW	Sensor out of water

SPC	Post calibration out of range

SQR	Data rejected due to QAQC checks

SSD	Sensor drift

SSM	Sensor malfunction

SSR	Sensor removed / not deployed

STF	Catastrophic temperature sensor failure

STS	Turbidity spike

SWM	Wiper malfunction / loss

Comments

CAB*	Algal bloom

CAF	Acceptable calibration/accuracy error of sensor

CAP	Depth sensor in water, affected by atmospheric pressure

CBF	Biofouling

CCU	Cause unknown

CDA*	DO hypoxia (<3 mg/L)

CDB*	Disturbed bottom

CDF	Data appear to fit conditions

CFK*	Fish kill

CIP*	Surface ice present at sample station

CLT*	Low tide

CMC*	In field maintenance/cleaning

CMD*	Mud in probe guard

CND	New deployment begins

CRE*	Significant rain event

CSM*	See metadata

CTS	Turbidity spike

CVT*	Possible vandalism/tampering

CWD*	Data collected at wrong depth

CWE*	Significant weather event

13. Post deployment information:

Retrieval Readings: this sub-section contains the field readings collected upon retrieval of the sonde. Post-calibration Readings: this sub-section contains sonde values in the lab taken once a deployment period ends. The readings take place while the probes are still dirty and have not yet been recalibrated.

Key to abbreviations:

* N/A: reading for parameter is not available-explanations usually found on field data sheets, but are not all included in this document for length reasons.

SCOTTON LANDING 2012 RETRIEVAL VALUES

Site	Date	H20 Temp(C)  Sp. Cond(ms) Salinity(ppt) DO (%sat) DO (mg/l) Tide

SL	01/17/2012	3.2	1.40	0.7	98.8	13.16	Slack Low

SL	02/16/2012	5.3	5.80	3.1	103.5	13.03	Ebb

SL	03/06/2012	6.3	18.87	11.1	89.1	10.21	Flood

SL	03/22/2012	17.6	17.03	10.1	70.7	6.36	Ebb

SL	04/05/2012	12.1	22.85	13.9	81.9	8.07	Flood

SL	04/16/2012	17.4	12.20	7.0	127.2	11.69	Ebb

SL	05/09/2012	18.4	31.95	20.0	63.1	5.27	Flood

SL	05/31/2012	25.0	32.35	20.2	52.6	3.87	Ebb

SL	06/14/2012	24.2	24.23	15.0	48.3	3.69	Ebb

SL	06/26/2012	25.2	23.55	14.2	57.8	4.39	Flood

SL	07/10/2012	28.8	33.03	19.1	44.6	3.11	Flood

SL	07/24/2012	26.4	23.44	14.2	32.0	2.40	Ebb

SL	08/16/2012	26.4	34.92	21.9	41.9	2.98	Flood

SL	09/05/2012	27.1	34.37	21.5	44.2	3.14	Slack High

SL	09/19/2012	21.8	17.95	10.6	52.8	4.35	Ebb

SL	10/04/2012	22.6	39.25	25.0	6.88	5.14	Ebb

SL	10/26/2012	17.0	34.19	21.5	64.0	5.42	Ebb

SL	11/19/2012	7.4	24.94	15.1	95.4	10.44	Slack High

SL	12/12/2012	9.4	16.20	9.5	82.6	8.91	Ebb

SL	01/03/2013	3.3	0.17	0.8	79.3	10.64	Slack Low

LEBANON LANDING 2012 RETRIEVAL VALUES

Site	Date	H20 Temp(C)  Sp. Cond(ms) Salinity(ppt) DO (%sat) DO (mg/l) Tide

LL	01/18/2012	6.4	0.27	0.1	93.8	11.60	Slack Low

LL	02/14/2012	3.8	0.48	0.2	93.2	12.25	Flood

LL	03/06/2012	7.2	0.73	0.4	77.1	9.30	Flood

LL	03/22/2012	19.5	0.32	1.8	75.2	6.38	Ebb

LL	04/05/2012	14.3	8.07	4.5	95.5	9.50	Flood

LL	04/16/2012	21.2	2.85	1.5	187.4	16.55	Ebb

LL	05/10/2012	18.9	10.42	5.9	63.2	5.65	Ebb

LL	05/31/2012	25.8	22.29	13.4	33.6	2.56	Ebb

LL	06/14/2012	25.1	14.51	8.4	125.2	9.86	Ebb

LL	06/26/2012	25.3	11.28	6.4	82.0	6.43	Flood

LL	07/10/2012	29.5	23.06	12.6	93.2	6.56	Flood

LL	07/24/2012	27.2	11.78	6.4	86.5	6.71	Ebb

LL	08/16/2012	29.2	21.07	12.5	130.9	9.35	Ebb

LL	09/05/2012	27.4	15.87	9.2	45.8	3.42	Flood

LL	09/19/2012	21.3	8.53	4.8	53.4	5.48	Ebb

LL	10/04/2012	22.7	31.15	19.4	40.7	3.12	Slack High

LL	10/18/2012	15.0	20.32	12.2	44.8	4.21	Flood

LL	10/26/2012	17.1	29.99	18.6	51.6	4.44	Ebb

LL	11/18/2012	7.5	15.15	8.8	78.8	8.92	Slack High

LL	12/12/2012	9.9	7.26	4.0	73.2	8.11	Ebb

LL	01/03/2012	2.4	0.41	0.2	NA	NA	Slack Low

DIVISION STREET 2012 RETRIEVAL VALUES

Site	Date	H20 Temp(C)  Sp. Cond(ms) Salinity(ppt) DO (%sat) DO (mg/l) Tide

DS	01/17/2012	NA	NA	0.1	NA	NA 	NA

DS	02/14/2012	6.0	0.17	0.1	104.8	13.42	NA

DS	03/06/2012	7.6	0.12	0.1	91.0	10.89	NA

DS	03/22/2012	19.3	0.16	0.1	117.2	10.79	NA

DS	04/05/2012	14.1	0.18	0.1	99.0	10.19	NA

DS	04/16/2012	20.9	0.18	0.1	104.0	9.30	NA

DS	05/10/2012	19.7	0.16	0.1	83.0	7.73	NA

DS	05/31/2012	25.1	0.19	0.1	53.9	4.42	NA

DS	06/15/2012	22.6	0.19	0.1	45.7	3.96	NA

DS	06/26/2012	25.1	0.21	0.1	90.2	7.55	NA

DS	07/10/2012	30.1	0.23	0.1	99.2	7.51	NA

DS	07/24/2012	24.7	0.25	0.1	30.8	2.54	NA

DS	08/16/2012	26.0	0.24	0.1	42.7	3.50	NA

DS	09/06/2012	26.8	0.23	0.1	79.9	6.38	NA

DS	09/18/2012	22.6	0.22	0.1	49.0	4.28	NA

DS	10/04/2012	22.8	2.62	0.1	58.6	5.05	NA

DS	10/18/2012	15.1	0.34	0.1	41.3	4.20	NA

DS	10/26/2012	17.6	0.40	0.2	28.1	2.66	NA

DS	11/19/2012	9.3	1.70	0.1	87.2	10.10	NA

DS	12/12/2012	9.4	0.18	0.1	103.2	11.93	NA

DS	01/03/2012	3.0	0.11	0.1	NA	NA	NA

BLACKBIRD LANDING 2012 RETRIEVAL VALUES

Site	Date	H20 Temp(C)  Sp. Cond(ms) Salinity(ppt) DO (%sat) DO (mg/l) Tide

BL	01/06/2012	0.9	NA	0.2	89.7	12.81	Flood

BL	01/20/2012	2.4	0.99	0.5	93.1	12.69	Slack High

BL	02/17/2012	6.9	0.72	0.3	100.2	12.15	Ebb

BL	03/09/2012	12.0	0.34	0.2	92.7	9.98	Flood

BL	03/23/2012	18.2	0.62	0.3	91.4	8.63	Flood

BL	04/03/2012	12.2	2.04	1.0	104.0	11.09	Slack High

BL	04/20/2012	18.3	3.14	1.6	83.0	7.73	Flood

BL	05/11/2012	18.6	3.68	1.9	115.0	10.64	Ebb

BL	05/25/2012	24.6	4.46	2.4	69.9	5.76	Ebb

BL	06/08/2012	NA	NA	NA	NA	NA	Ebb

BL	06/22/2012	30.1	5.07	2.7	60.1	4.45	Flood

BL	07/06/2012	30.4	6.12	3.3	53.1	3.93	Ebb

BL	07/20/2012	28.6	8.12	4.5	42.7	3.23	Flood

BL	08/03/2012	27.8	10.16	5.7	53.7	3.97	Ebb

BL	08/17/2012	27.4	7.26	4.0	54.5	4.23	Flood

BL	08/31/2012	26.8	9.90	5.6	66.4	5.18	Slack High

BL 	09/14/2012	23.0	7.27	4.0	66.2	5.55	Flood

BL	10/01/2012	18.6	8.17	4.6	75.8	6.84	Flood

BL	10/12/2012	14.6	8.57	4.8	72.3	7.12	Flood

BL	10/26/2012	16.9	9.26	5.2	75.0	7.04	Flood

BL	11/09/2012	5.8	1.34	0.7	92.1	11.54	Ebb

BL	12/07/2012	6.8	4.79	2.6	106.0	12.86	Ebb

BL	12/28/2012	4.2	0.26	0.1	86.3	11.25	Flood

BL	01/18/2012	4.9	0.32	0.1	88.6	11.36	Ebb

b) End of Deployment Post-calibration Readings in Standard Solution:

SCOTTON LANDING 2012 POST CALIBRATION VALUES

Site   	Retrieval Date   DO%   Sp Cond.   	pH    	pH    	pH   Turb   Turb   Depth

(air sat) (ms/cm)

Std: 	(100%)		(12.88)	(7)	(10) 	(4) (0 NTU)( 123 NTU)

SL	01/17/2012	98.9, 99.4	12.99	6.96	10.07	4.01	0.1	129.3	-1.304

SL	02/16/2012	101.2, 101.3	13.02	6.91	9.75	4.15	-0.5	121.6	-1.262

SL	03/06/2012	102.2, 102.2	12.73	7.20	10.34	4.21	0.0	122.7	-1.183

SL	03/22/2012	100.0, 100.5	12.49	7.08	10.15	4.13	-0.3	125.3	-1.190

SL	04/05/2012	99.2, 99.3	13.00	7.07	10.07	4.04	0.2	124.5	-1.338

SL	04/16/2012	100.4, 100.4	13.28	6.93	9.92	4.00	-0.7	126.2	-1.238

SL	05/09/2012	NA, NA	13.08	6.93	9.96	4.07	2.2	125.9	-1.392

SL	05/31/2012	98.0, 97.6	12.81	7.01	9.93	4.19	-0.1	125.6	-1.309

SL	06/14/2012	101.9, 102.9	13.17	6.91	10.05	4.44	0.5	126.8	-1.190

SL	06/26/2012	100.9, 101.0	12.64	6.92	9.99	3.96	0.2	123.8	-1.310

SL	07/10/2012	99.2, 99.2	13.05	6.94	10.04	4.73	0.6	140.0	-1.242

SL	07/24/2012	101.1, 101.2	12.47	7.14	10.19	4.24	-0.2	125.9	-1.285

SL	08/16/2012	98.5, 98.1	13.06	6.90	9.89	3.98	-0.2	121.5	-1.301

SL	09/05/2012	99.1, 99.4	12.97	6.87	10.01	4.12	0.4	123.1	-1.138

SL	09/19/2012	98.1, 97.7	13.14	6.97	10.07	4.04	-0.4	129.6	-1.258

SL	10/04/2012	99.5, 99.5	12.96	7.07	10.18	4.15	0.0	124.4	-1.253

SL	10/26/2012	99.1, 99.2	13.16	6.78	9.80	3.87	-0.2	131.4	-1.437

SL	11/19/2012	102.0, 102.3	12.69	6.78	9.86	3.80	0.9	128.0	-1.234

SL	12/12/2012	102.0, 101.7	12.92	6.79	9.69	3.77	1.0	123.4	-1.134

SL	01/03/2012	100.4, 100.4	13.15	6.70	9.64	3.75	0.1	125.3	-1.244

LEBANON LANDING 2012 POST CALIBRATION VALUES

Site   	Retrieval Date   DO%   Sp Cond.   	pH    	pH    	pH   Turb   Turb   Depth

(air sat) (ms/cm)

Std: 	(100%)		(12.88)	(7)	(10) 	(4) (0 NTU)( 123 NTU)

LL	01/18/2012	100.5, 100.6	13.25	6.86	9.85	4.09	-0.6	124.2	-2.169
LL	02/14/2012	100.7, 100.6	12.77	6.98	10.07	4.09	-0.2	124.5	-2.120

LL	03/06/2012	101.6, 101.7	11.61	7.02	10.04	4.11	-0.2	127.0	-2.089

LL	03/22/2012	100.9, 100.8	12.67	7.00	9.98	4.06	-0.1	124.4	-2.078

LL	04/05/2012	99.6, 99.7	12.90	7.01	10.08	3.96	-0.1	125.3	0.352

LL	04/16/2012	102.4, 102.3	13.23	7.01	10.09	3.96	0.0	133.5	-2.140

LL	05/10/2012	96.8, 97.3	12.94	7.09	10.05	4.03	0.2	125.0	-2.299

LL	05/31/2012	97.5, 97.3	13.09	7.06	10.09	4.05	-0.1	124.9	-2.209

LL	06/14/2012	100.5, 100.4	12.78	7.02	10.15	4.19	0.8	125.0	-2.085

LL	06/26/2012	99.1, 99.3	12.86	7.02	10.07	4.26	0.0	130.3	-2.210

LL	07/10/2012	100.0, 99.6	12.74	7.14	10.15	4.77	0.9	124.7	-2.140

LL	07/24/2012	101.3, 101.3	12.39	7.03	9.59	5.55	-0.1	124.3	-2.186

LL	08/16/2012	98.0, 98.2	12.79	7.10	9.92	4.97	-0.3	121.2	-2.204

LL	09/05/2012	98.1, 98.7	13.08	7.10	10.14	4.24	-0.1	127.1	-2.224

LL	09/19/2012	96.1, 96.6	12.85	6.82	9.73	4.18	-0.1	127.8	-2.157

LL	10/04/2012	99.5, 99.5	12.69	6.98	10.02	4.00	0.1	124.9	-2.140

LL	10/18/2012	99.2, 99.4	12.98	6.95	9.93	3.95	-1.5	133.0	-2.203

LL	10/26/2012	98.5, 98.6	12.99	6.95	10.00	4.23	0.8	124.0	-2.352

LL	11/18/2012	100.8, 101.0	13.03	6.92	9.84	4.02	1.4	134.4	-2.133

LL	12/12/2012	101.7, 101.7	12.94	6.96	10.06	3.94	0.1	125.5	-2.038

LL	01/03/2012	99.4, 99.5	13.29	6.92	9.81	3.82	0.8	124.5	-2.141

DIVISION STREET 2012 POST CALIBRATION VALUES

Site   	Retrieval Date   DO%   Sp Cond.   	pH    	pH    	pH   Turb   Turb   Depth

(air sat) (ms/cm)

Std: 	(100%)		(12.88)	(7)	(10) 	(4) (0 NTU)( 123 NTU)

DS	01/17/2012	99.8, 99.8	13.34	6.98	10.01	4.02	0.1	124.5	0.326

DS	02/14/2012	100.9, 101.0	12.72	7.03	10.03	4.05	0.1	126.2	0.482

DS	03/06/2012	103.7, 103.9	11.31	7.09	10.05	4.13	0.0	123.1	0.520

DS	03/22/2012	100.4, 100.5	12.84	7.14	10.18	4.15	0.1	126.1	0.523

DS	04/05/2012	99.5, 99.5	12.82	6.99	10.03	3.93	0.1	124.4	-2.240

DS	04/16/2012	99.6, 99.9	13.57	6.87	9.73	3.78	0.2	129.6	0.483

DS	05/10/2012	99.3, 99.4	13.05	7.07	10.13	4.11	0.1	124.1	0.296

DS	05/31/2012	96.7, 96.6	12.76	6.96	10.05	3.92	-0.2	124.5	0.389

DS	06/15/2012	99.7, 99.7	12.80	7.08	10.13	4.25	0.5	125.2	0.501

DS	06/26/2012	79.9, 79.2	9.795	6.88	9.79	3.93	-0.2	137.3	0.388

DS	07/10/2012	99.1, 98.7	12.61	7.06	10.12	4.57	1.2	126.6	0.452

DS	07/24/2012	100.6, 100.8	12.14	6.90	9.97	4.00	0.3	123.8	0.407

DS	08/16/2012	95.2, 93.8	12.14	7.05	10.01	4.12	-0.3	122.9	0.398

DS	09/06/2012	97.2, 97.3	12.32	7.14	10.16	4.35	1.0	125.6	0.361

DS	09/18/2012	97.4, 97.0	12.89	7.08	10.08	4.11	-1.0	128.4	0.437

DS	10/04/2012	99.5, 99.5	6.739	NA	NA	NA	NA	NA	0.430

DS	10/18/2012	97.8, 97.2	13.01	7.01	9.96	3.98	0.0	132.5	0.391

DS	10/26/2012	98.2, 98.1	12.91	7.15	10.16	4.02	1.1	125.7	0.240

DS	11/19/2012	101.4, 101.2	12.89	7.03	10.06	4.02	0.1	127.9	0.457

DS	12/12/2012	101.1, 101.0	12.89	7.03	10.06	4.14	0.6	125.3	0.551

DS	01/03/2012	99.5, 99.7	12.85	7.01	10.08	4.06	1.1	129.1	0.459

BLACKBIRD LANDING 2012 POST CALIBRATION VALUES

Site   	Retrieval Date   DO%   Sp Cond.   	pH    	pH    	pH   Turb   Turb   Depth

(air sat) (ms/cm)

Std: 	(100%)		(12.88)	(7)	(10) 	(4) (0 NTU)( 123 NTU)

BL	01/06/2012	99.3, 99.2	13.18	7.05	9.97	4.08	0.2	127.1	-1.206

BL	01/20/2012	101.3, 101.3	12.89	6.97	9.93	4.02	0.0	126.4	-1.098

BL	02/17/2012	100.6, 100.6	12.80	7.05	10.07	4.01	-0.1	125.9	-1.142

BL	03/09/2012	100.9, 100.9	12.76	6.99	10.04	3.99	0.0	126.1	-1.120

BL	03/23/2012	100.2, 100.3	12.43	6.96	9.90	4.03	-0.1	123.6	-1.134

BL	04/03/2012	99.9, 99.9	13.01	7.08	10.12	4.08	-0.1	124.4	-1.158

BL	04/20/2012	101.5, 101.4	12.48	7.03	10.03	4.04	0.0	128.3	-1.146

BL	05/11/2012	100.1, 100.0	12.60	7.06	10.11	4.13	-0.4	105.6	-1.122

BL	05/25/2012	100.0, 100.1	12.92	7.10	10.10	4.05	0.0	121.1	-1.093

BL	06/08/2012	99.8, 99.8	12.94	7.09	10.06	4.05	0.0	123.2	-1.137

BL	06/22/2012	98.1, 97.9	12.78	7.03	10.05	3.98	-0.2	122.8	-1.209

BL	07/06/2012	98.1, 98.3	12.86	7.12	10.08	4.16	-0.1	127.7	-1.146

BL	07/20/2012	101.1, 101.1	12.76	7.11	10.10	4.15	0.0	125.1	-1.159

BL	08/03/2012	99.0, 99.2	12.85	7.13	10.07	4.18	0.1	125.5	-1.128

BL	08/17/2012	97.7, 97.8	12.81	7.08	10.10	4.09	-0.1	123.8	-1.184

BL	08/31/2012	100.5, 100.5	12.89	7.07	10.05	4.04	0.1	123.3	-1.118

BL	09/14/2012	100.7, 100.7	12.85	7.09	10.05	4.14	0.1	122.6	-1.074

BL	10/01/2012	99.5, 99.4	13.09	7.04	10.05	4.08	0.0	129.0	NA

BL	10/12/2012	100.7, 100.7	12.88	7.07	10.09	4.08	0.0	136.1	-1.090

BL	10/26/2012	99.2, 99.2	12.78	6.93	9.95	3.97	0.4	125.8	-1.232

BL	11/09/2012	101.0, 100.9	12.95	6.96	9.96	4.04	0.4	125.5	-1.106

BL	12/07/2012	NA, NA	NA	NA	NA	NA	NA	NA	NA

BL	12/28/2012	100.6, 100.6	12.93	6.95	10.04	3.96	0.2	125.1	-1.135

14. Other remarks/notes:

Data are missing due to equipment or associated specific probes not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

a) Sonde deployment tubes were cleaned on August 08, 2012 at all four sites.

b) “CSM” (See Metadata) Code Explanations:

Lebanon Landing:

- Water level data from 03/22/2012 (13:15 EST) to 04/05/2012 (08:00 EST) were post-corrected by 2.595 m to account for the incorrect calibration (sonde was accidentally calibrated using Division Street’s water level station calibration offset of 0.378 m as opposed  to the correct Lebanon Landing offset of  -2.217m).

- pH data from 11/19/2012 (14:45 EST) to 12/12/2012 (14:00 EST) marked as suspect due to a depressed trend when compared to the deployment prior to and afterwards that cannot be explained by rainfall.

- From 02/14/2012 (12:45 EST) to 03/06/2012 (09:30 EST) SpCond and salinity data are marked 0 SDG CSM. The conductivity cell constant (4.45527) for this deployment was slightly out of the accepted range of 4.6 to 5.45, however the data appears valid.

Division Street:

- Water level data from 03/22/2012 (13:45 EST) to 04/05/2012 (08:30 EST) were post-corrected by 2.595 m to account for the incorrect calibration (sonde was accidentally calibrated using Lebanon Landing’s  water level station calibration offset of -2.217  m as opposed  to the correct Lebanon Landing offset of  0.378 m).

- From 02/14/2012 (12:30 EST) to 03/06/2012 (10:00 EST) SpCond and salinity data is marked 0 SDG CSM. SpCond and salinity values during this timeframe appear valid however the cell constant for the deployment was slightly lower at 4.3 than the accepted range of 4.6 – 5.45.

Blackbird Landing

- From 01/20/2012 (10:30:00 EST) to 02/17/2012 (13:15:00 EST) The dissolved oxygen trend (both % saturation and concentration) appear slightly lower compared to the deployments that precede and follow it; data marked suspect as a result.

c) Hurricane Sandy hit Delaware on 10/28/2012 with a combined 3-day precipitation total of 216.9 mm. Heavy precipitation, winds, and storm surge affected most parameters in some way at all four monitoring stations (Scotton Landing, Lebanon Landing, Division Stree, and Blackbird Landing).

d) Major rain/storm events (near or exceeding 25.4 mm (1 inch) of rainfall) during 2012 took place on the following dates (data originates from the Delaware NERR St. Jones meteorological station):

February 29 - March 01, 2012	(46.7 mm)

April 22 – 23, 2012			(52.9 mm)

July 20, 2012				(59.2 mm)

August 17 – 18, 2012			(25.9 mm)

September 07, 2012			(24.6 mm)

October 28 - 30, 2012 		(216.9 mm)

December 07 – 08, 2012 		(42.7 mm)

December 20 – 21, 2012 		(33.0 mm)

December 26 – 27, 2012 		(50.6 mm)