Elkhorn Slough (ELK) National Estuarine Research Reserve Meteorological Metadata

January 2008-December 2008

Latest Update:  June 20, 2017

I. Data Set & Research Descriptors

- Principal investigator(s) & contact persons:

Contact Persons:

Dr. Kerstin Wasson, Research Coordinator, research@elkhornslough.org

John Haskins, Water Monitoring Scientist, john@elkhornslough.org

Address:

Elkhorn Slough NERR

1700 Elkhorn Rd

Watsonville, CA 95076

Phone: 831-728-2822

Homepage: http://www.elkhornslough.org

- Entry verification:

The meteorological information is sampled every 5 seconds from each instrument on the weather station and stored on a Campbell Scientific CR1000 datalogger.  Data are uploaded from the CR1000 from LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database. During primary QAQC, data are flagged if they are missing or out of sensor range. The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro. The macro inserts station codes, created metadata worksheets for flagged data, and graphs the data for review. It allow the user to apply QAQC flags and coded to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database. For more information on QAQC flags ad QAQC codes, see Sections 11 and 12.

All data files are backed up on an external hard drive. SWMP Technician John Haskins and Research Assistant Brent Hughes error checked and compiled the 2008 weather data.

- Research objectives:

The principal objectives are to record meteorological information for the Elkhorn Slough NERR’s site that can be used 1) as a vital reference of atmospheric data for various research projects at the reserve -- an integral part of our general NERR mission is to provide a platform for estuarine research, 2) to give meteorological context (atmospheric-forcing) for our quarter-hourly SWMP water quality data, and other long-term environmental monitoring programs at the Reserve (including nutrients and shoreline change), 3) to observe and characterize important events, such as storms, heat and cold waves, droughts and heavy rainfalls, and 4) to detect trends and characterize climate variability over the long-term.

- Research methods:

The Campbell Scientific weather station samples every 5 seconds continuously throughout the year.  Sensors on the weather station are inspected monthly for damage or debris. The rain gauge tends to collect debris and is cleaned out every month, particularly before and after major storms events.  Sensors are removed and calibrated on an annual or biannual basis depending on the particular sensor. Also, once a month on download day, we use a handheld Kestrel 4000, to run a comparative set of observations as a general check on the Campbell station sensors.

Campbell Scientific data telemetry equipment was installed at Caspian Weather station to transmit to the NOAA GOES satellite, NESDIS ID #3B0155FC. (Where 3B0155FC is the GOES ID for that particular station.) The transmissions are scheduled hourly and contain four (4) datasets reflecting the fifteen min data sampling interval. Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above. The “real-time” telemetry data becomes part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database. Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Wind Direction Standard Deviation (degrees) (Recorded as a standard parameter at ELK beginning 11/18/2008), Battery Voltage (volts)

Maximum, Minimum, and their times from 5-second data:

Air Temperature (°C), (these data are not available in the dataset, but are available from ELK NERR)

Maximum and times from 5-second data:

Wind Speed, (m/s) (Recorded as a standard parameter at ELK beginning 1/17/2008)

Totals:

Precipitation (mm) and PAR (millimoles/m2) and Cumulative Precipitation (mm) (Recorded as a standard parameter beginning 6/23/2008)

- Site location and character:

The Elkhorn Slough NERR is located on the central California coast at the head of the Monterey Bay. It connects with the Pacific Ocean near Moss Landing, California and harbors the largest tract of tidal salt marsh in California outside of San Francisco Bay. There is one weather station on the reserve that is located approximately one kilometer south of the South Marsh water quality site (3649’05.00”N, 12144’21.83”W). The sensors are located on a 10 ft tower placed according to descriptions outlined in the CDMO Manual V 5.0 and are located approximately 200 feet above the marsh on a bluff in the uplands of the marsh. The wind and PAR sensors are located at a height of approximately 12 ft, the temp/RH sensor is are located at approximately 6 ft, and the barometric pressure sensor is located in the enclosure at approximately 5 ft.   The tipping bucket is located to the west of the tower on the ground, approximately 20 ft away.  The top of the tipping bucket is 20 inches off of the ground.   There are no shadow or wind obstructions from buildings or trees in the immediate area surrounding the location of any of the sensors.

- Data collection period:

Weather data has been collected since September 18, 1998. The current weather station has been operational since this date. Data was collected for the entire year in 2008 from 01/01/2008 00:00 thru 12/31/2008 23:45.

- Distribution:

According to the Ocean and Coastal Resource Management Data Dissemination Policy for the NERRS System-wide Monitoring Program,

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR weather data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see section 1. Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see the CDMO Staff link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/. Data are available in comma separated format.

- Associated researchers and projects:

Rain data from the weather station are shared with the Naval Post Graduate School who in turn organize data from many stations in the county. These weather data are also combined with water quality (pH, salinity, DO, depth, turbidity and temperature) and nutrient data (ammonia, phosphate, nitrate and chlorophyll) for analysis and correlations with rain and other weather events.

II. Physical Structure Descriptors

- Sensor specifications:

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #:  HMP45C Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Date of Last calibration: Sept 29, 2008

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #:  HMP45C Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Date of Last calibration: Sept 29, 2008

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Date of Last calibration:  November 29, 2007

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 03101-5 Anemometer

Range:  0-60 m/s (130 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 2%

Date of last calibration: November 4, 2008

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Model #: R.M. Young 03301-5 Wind Vane

Range:  360° mechanical, 355° electrical (5° open)

Accuracy: +/- 5%

Date of last calibration: November 4, 2008

Parameter: Photosynthetically Active Radiation -PAR

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI-COR Quantum Sensor LI190SB

Light spectrum waveband:  400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity: 0 to 100%

Sensitivity:  typically 5 µA per 1000 µmoles s-1 m-2

Date of last calibration:  December 3, 2007

Multiplier: 2.33

Dates in use:	01/01/2008 00:00 thru 12/31/2008 23:45.

Parameter: Precipitation

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to +/- 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Date of Last calibration: October 1, 2007

The CR1000 has two MB Flash EEPROM that are used to store the Operating System. Another 128 K Flash is used to store configuration settings. A minimum of 2 MB SRAM is (4 MB optional) is available for program storage (16K), operating system use, and data storage. Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Dates in use:  July 18, 2006-present

- Coded variable definitions:

Sampling station:		Sampling site code:		Station code:

Caspian Weather		CW				elkcwmet

11) QAQC flag definitions:

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range or missing.  All remaining data are then flagged with a 0, as passing initial QAQC checks.  During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0	Passed Initial QAQC Checks

1	Suspect Data

2	Open - reserved for later flag

3	Open - reserved for later flag

4	Historical Data:  Pre-Auto QAQC

5	Corrected Data

12) QAQC code definitions:

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the deployment or the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SIC	Incorrect Calibration Constant, Multiplier or Offset

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR	Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CCU	Cause Unknown

CDF	Data Appear to Fit Conditions

CRE*	Significant Rain Event

CSM*	See Metadata

CVT*	Possible Vandalism/Tampering

13) Other remarks/notes:

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

All Reserves were required to align their wind sensors to True North by April 1, 2008.  The exact date for ELK NERR’s sensor alignment was not recorded; however, it is believed to have been aligned on March 10, 2008 since monthly maintenance was performed on that date.  The time that the sensor was adjusted was also not recorded.  The monthly MET log has 9:45 as the time for monthly maintenance.  Wind direction data for March 10, 2008 at 9:45 were coded as SMT CSM but not rejected since the exact time of alignment was not recorded.

Cumulative precipitation data are recorded from 00:00 to 23:59 with the daily total recorded at the midnight mark (00:00). The midnight CumPrcp value is actually the total from the previous day.

During 2017 the CDMO discovered an incorrect line in the CR1000 programming.

If RHumidity>100 And RHumidity<108 Then RHumidity=100.  A decision was made by the DMC during 2006 to discontinue correcting >100 RH values to 100. This change was never made in our program and has remained in each updated version until it was removed during 2017.  By correcting all values >100 during data collection we may have missed erroneous values that could have indicated a problem with the RH sensor.  CSM coding was added to all RH data from 2007 until the programming change in 2017.

PAR:

Negative PAR data have been observed during the night; small negative values are within range of the sensor and are due to normal errors in the sensor and the CR10X Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15-minute interval. These data have been retained.

A multiplier value of 0.85 was initially used for the PAR sensor and later identified as incorrect. The multiplier was corrected by dividing the PAR data by the incorrect multiplier then multiplying it by the correct multiplier (2.33). This was done for the entire year.

Throughout the year ELK experienced elevated nighttime PAR values.  These values were low and below the maximum signal noise error for the sensor.  These elevated values may be caused from moisture seeping into the sensor.

Following the program upload on 4/23/2008 at 08:45. The PAR multiplier was not inserted and PAR values of 0.0 were recorded.  The multiplier was inserted at 15:15.  All PAR data from 09:00 thru 15:15 were rejected.

RH:

Relative Humidity data greater than 100% have been observed; these data are within range of the sensor accuracy of +/-3%. These data have been retained.  See note above about RH data coding and CR1000 programming errors.

The ATemp/RH sensor was switched out on 10/13/2008 at 14:45.  Data for that timestamp were rejected.  After the sensor switch there was a noticeable difference in RH values, most likely due to replacing the sensor with a freshly calibrated one.