Elkhorn Slough (ELK) NERR Meteorological Metadata

January 2016-Dec 2016

Latest Update:  31 Jan 2017

Note: This is a provisional metadata document; it has not been authenticated as of its download date.  Contents of this document are subject to change throughout the QAQC process and it should not be considered a final record of data documentation until that process is complete.  Contact the CDMO (cdmosupport@belle.baruch.sc.edu) or Reserve with any additional questions.

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons

Contact Persons:

Dr. Kerstin Wasson, Research Coordinator, research@elkhornslough.org

John Haskins, Water Monitoring Scientist, john@elkhornslough.org

Address:

Elkhorn Slough NERR

1700 Elkhorn Rd

Watsonville, CA 95076

Phone: 831-728-2822

Homepage: http://www.elkhornslough.org

2)  Entry verification

The meteorological information is sampled every 5 seconds from each instrument on the weather station and stored on a Campbell Scientific CR1000 datalogger.  Data are uploaded from the CR1000 from LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database. During primary QAQC, data are flagged if they are missing or out of sensor range. The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro. The macro inserts station codes, created metadata worksheets for flagged data, and graphs the data for review. It allow the user to apply QAQC flags and coded to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database. For more information on QAQC flags ad QAQC codes, see Sections 11 and 12.

All data files are backed up on an external hard drive. SWMP Technician John Haskins and Research Assistant Rikke Priesler error checked and compiled the 2016 weather data.

3)  Research objectives

The principal objectives are to record meteorological information for the Elkhorn Slough NERR’s site that can be used 1) as a vital reference of atmospheric data for various research projects at the reserve -- an integral part of our general NERR mission is to provide a platform for estuarine research, 2) to give meteorological context (atmospheric-forcing) for our quarter-hourly SWMP water quality data, and other long-term environmental monitoring programs at the Reserve (including nutrients and shoreline change), 3) to observe and characterize important events, such as storms, heat and cold waves, droughts and heavy rainfalls, and 4) to detect trends and characterize climate variability over the long-term.

4)  Research methods

The Campbell Scientific weather station samples every 5 seconds continuously throughout the year.  Sensors on the weather station are inspected monthly for damage or debris. The rain gauge tends to collect debris and is cleaned out every month, particularly before and after major storms events.  Sensors are removed and calibrated on an annual or biannual basis depending on the particular sensor. Also, once a month on download day, we use a handheld Kestrel 4000, to run a comparative set of observations as a general check on the Campbell station sensors.

Campbell Scientific data telemetry equipment was installed at Caspian Weather station to transmit to the NOAA GOES satellite, NESDIS ID #3B0155FC. (Where 3B0155FC is the GOES ID for that particular station.) The transmissions are scheduled hourly and contain four (4) datasets reflecting the fifteen min data sampling interval. Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above. The “real-time” telemetry data becomes part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database. Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts)

Maximum, Minimum, and their times from 5-second data:

Air Temperature (°C), Wind Speed, (m/s), Wind Direction Standard Deviation (degrees) from 5-second data

Totals:  Precipitation (mm) and PAR (millimoles/m2)

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- yearly recalibration

- Rain Gauge- yearly recalibration

- Wind Speed/Direction- yearly or every 2 years (depending on the sensor)

- Barometric Pressure- every 2 years recalibration

- PAR- every 2 years recalibration

- CR1000-every 5 years (required beginning 2014, one year initial grace period)

-Total Solar Radiation (this is an optional sensor, only include if you are collecting TSR data)-

Eppley 8-48 - every 5 years, but annual recalibration encouraged

LI-COR LI-200S - every 2 years, with recalibrations performed during the spring and summer

5)  Site location and character

The Elkhorn Slough NERR is located on the central California coast at the head of the Monterey Bay. It connects with the Pacific Ocean near Moss Landing, California and harbors the largest tract of tidal salt marsh in California outside of San Francisco Bay. There is one weather station on the reserve that is located approximately one kilometer south of the South Marsh water quality site (3649’05.00”N, 12144’21.83”W). The sensors are located on a 10 ft tower placed according to descriptions outlined in the CDMO Manual V 5.0 and are located approximately 100 feet above the marsh on a grassy low shrub bluff in the uplands of the marsh. The wind and PAR sensors are located at a height of approximately 12 ft, the temp/RH sensor is are located at approximately 6 ft, and the barometric pressure sensor is located at approximately 5 ft.   The tipping bucket is located to the west of the tower on the ground, approximately 20 ft away.   There are no shadow or wind obstructions from buildings or trees in the immediate area surrounding the location of any of the sensors.

6)  Data collection period

Weather data has been collected since September 18, 1998. The current weather station has been operational since this date. Data was collected from 12/01/15 14:30 thru 03/02/2015 10:45.

Here are the start and end times of the raw files

12/29/15 13:15 – 02/22/16 10:45

02/22/16 11:00 – 03/21/16 14:15

7)  Distribution – This section will address data ownership and data liability with the following excerpt from the Ocean and Coastal Resource Management Data Dissemination Policy for the NERRS System-wide Monitoring Program in the metadata.

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2012.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

8)  Associated researchers and projects

These weather data are also combined with water quality (pH, salinity, DO, depth, turbidity and temperature) and nutrient data (ammonia, phosphate, nitrate and chlorophyll) for analysis and correlations with rain and other weather events.

II.  Physical Structure Descriptors

9)  Sensor specifications – Include parameter description, units, sensor type, model #, (operating temperature), range of measurement, accuracy, (temperature dependence), (sensitivity), (stability), serial numbers, multipliers, date of last calibration, and dates in use for each sensor (include at a minimum the sensors installed for the year covered by this metadata document) and CR1000 description.  Ensure that sensor information is accurate for your weather station.

Parameter: Temperature

Units: Celsius

Sensor type: PT100 RTD, IEC 751 1/3 Class B, with calibrated signal conditioning

Model #:  HC2-S3 Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.1 °C @ 23°C

Serial Number: 20053048

Date of Last calibration: February 22, 2016

Parameter: Relative Humidity

Units: Percent

Sensor type: ROTRONIC® Hygromer IN-1

Model #: HC2-S3 Temperature and Relative Humidity Probe

Range: 0-100% non-condensing

Accuracy at 23°C:  +/- 0.8% RH with standard configuration settings

Temperature dependence of RH measurement +/- 3% (-40 to 60C)

Serial Number: 20053048

Date of Last calibration: February 22, 2016

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Serial Number: P4830023

Date of Last calibration:  November 02, 2015

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 03101-5 Anemometer

Range:  0-60 m/s (130 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 2%

Date of last calibration: November 20, 2015

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Model #: R.M. Young 03301-5 Wind Vane

Parameter: Photosynthetically Active Radiation (PAR)

Units:  mmoles m-2 (total flux)

Sensor type:  anodized aluminum with cast acrylic diffuser

Model #SQ110 Apogee Quantum Sensor

Light spectrum waveband:  410 to 655 nm

Temperature dependence: 0.06+/-0.06% per °C

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 70°C; Humidity 0 to 100%

Cosine Response: 45° zenith angle: +/- 2%; 75° zenith angle: +/- 5%

Sensitivity:  0.2mV per µmol s-1 m-2

Serial number, Q42860

Date of last calibration: Mar 13, 2015

Parameter: Precipitation (specify if heated rain gauge)

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Date of Last calibration: October 26, 2015

Another 128 K Flash is used to store configuration settings. A minimum of 2 MB SRAM is (4 MB optional) is available for program storage (16K), operating system use, and data storage. Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Date installed:  July 18, 2006-present

Date CR1000 Calibrated: April 02, 2014

CR1000 Firmware Version (s):  Version(s) and dates changed

CR1000 Program Version(s):  elkcwmet_5.6_031614

10)  Coded variable definitions - List the sampling station, sampling site code, and station code used in the data.

Sampling station:		Sampling site code:	Station code:

Carriage House		CH			wqbchmet

11)  QAQC flag definitions – This section details the automated primary and secondary QAQC flag definitions.  Include the following excerpt.

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions – This section details the secondary QAQC Code definitions used in combination with the QAQC flags above.   Include the following excerpt.

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument malfunction

GIT	Instrument recording error, recovered telemetry data

GMC	No instrument deployed due to maintenance/calibration

GMT 	Instrument maintenance

GPD	Power down

GPF	Power failure / Low battery

GPR	Program reload

GQR	Data rejected due to QA/QC checks

GSM	See metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC		Incorrect calibration constant, multiplier or offset

SIW	Incorrect wiring

SMT	Sensor maintenance

SNV	Negative value

SOC	Out of calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor drift

SSN	Not a number / unknown value

SSM	Sensor malfunction

SSR		Sensor removed

Comments

CAF	Acceptable calibration/accuracy error of sensor

CCU	Cause unknown

CDF	Data appear to fit conditions

CML	Snow melt from previous snowfall event

CRE*	Significant rain event

CSM*	See metadata

CVT*	Possible vandalism/tampering

CWE*	Significant weather event

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

Some minimum temperature readings of -39. These started in march and continued through December. Don't seem to effect the overall temp significantly but seems it would bring it down for the 15 min reading that it occurred.

After wind sensor was changed on May 17 14:00 (uploaded the file at 10:45 but then swapped out the instrument at 14:00) wind direction and speed look suspect afterwards. The bearings were locked up on the sensor since it was upside down for some unknown time. Wind direction and speed data was deleted from 05/07/2016 at 15:00 through 05/17/2016 10:30. Swapped the housing holding the bearings from the other wind vane to this one so as not to have to re wire the old vane. Then reset the base to point N and S on 05/17/2016 at 14:00.

Wind direction is suspect since this new instrument was deployed 05/17/16. The mode for the period from May through early July was nearly 60 degrees less this year (218) than last year (271).

Forgot to incorporate PAR multiplier when putting in new program Aug 18 so no PAR data from 18 Aug 13:45 through 07 Sept 12:00.

Aug 18 2016 replace wind sensor with new 5106 calibrated 7/28/16.

Jan 05 2017 for monthly maintenance and found tipping bucket filled to brim with water. Today has very high rain at 10:45 but this was due to draining of the bucket. Missed rain events in Dec last known was Jan 3 and 4. As well as Dec 8, 11, 15, 24

Moss Landing Marine Labs weather station is the closest approximately 5 km recorded a total of 5.23 inches of rain during this time period. The Moss Landing Marine Laboratory station also recorded rain on Nov 26 and 27 of 1.14 and .45 inches respectively. I assume the bucked was clogged during this time too as there was lots of daily readings indicating a very slow leak of water for months of Nov and Dec. Potentially the bucket was clogged and slowly dripping since Oct 15 2016. All rain data from Oct 15 2016 through Jan 5 2017 has been marked rejected. Data from MLML can be retrieved at http://pubdata.mlml.calstate.edu/weather/index.html.