Grand Bay (GND) NERR Nutrient Metadata

January - December 2007

Last Updated:  November 15, 2011

I.  Data Set and Research Descriptors

1)  Principal Investigator(s) and Contact Persons:

Address:		Grand Bay National Estuarine Research Reserve

6005 Bayou Heron Road

Moss Point, MS  39562

Phone:  (228) 475-7047

Fax:      (228) 475-8097

Contact Persons:	Dr. Mark Woodrey, Research Coordinator

E-mail:  mark.woodrey@dmr.ms.gov

(228) 475-7047, ext. 107

Christine Walters, Research Technician

E-mail:  christine.walters@dmr.ms.gov

(228) 475-7047, ext. 119

2)  Research objectives:

a)	Monthly Grab

Monthly grab samples were collected to quantify the spatial variability of important nutrients in the water column between sites representing the local salinity gradient.

b)	Diel Sampling Program

Once per month, samples were collected every 125 minutes through a tidal cycle to quantify the temporal variability of important nutrients and sediment loading in the water column as a function of tidal forcing.

3)  Research methods:

a)	Monthly Grab Sampling Program

Monthly grab samples were taken at five stations within the Grand Bay estuary.  Replicate samples (n=2) were taken in sequence at each of the Grand Bay datasonde stations; Bayou Heron (BH), Point Aux Chenes (PC), Bayou Cumbest (BC), and Bangs Lake (BL) and at a spill site in Bangs Lake, called Bangs Lake North (BN).  Replicate surface samples were also taken at the Bayou Heron site due to stratification at this site (surface sample replicates are designated in Section 6 Data Collection Period).  All other grab samples were collected at sonde depth, 0.25 meters from the bottom.  Bayou Heron and Point Aux Chenes required the use of a Wildco Horizontal Beta Bottle to obtain these grab samples.  Attempts were made to collect grab samples within two hours of slack low tide.  Rainfall conditions prior to grab sampling were not considered.  When possible, grab samples were obtained in conjunction with diel samples which are taken via an ISCO 3700 portable sampler (See 3b).  Samples were collected in 500ml Nalgene HDPE (high-density polyethylene) wide-mouthed amber bottles.  Sample bottles were rinsed with 10% HCL and then with distilled water and allowed to air dry in the laboratory.  Sample bottles were also rinsed with sample water at the sample site prior to sample collection.  Samples were immediately placed on ice, stored in a dark cooler, and returned to the laboratory. Once in the laboratory samples were processed for nutrient and Chla analysis.

b)	Diel Sampling Program

Twelve diel samples are collected each month at Bangs Lake using an ISCO 3700 portable auto-sampler .25m from the bottom.  The sampler was programmed to take a 500ml collection every 125 minutes throughout a complete tidal cycle (lunar day).  No distinction between neap and spring tide was considered.  Prior to collection, 500 ml semi-transparent ISCO sample containers were rinsed with 10% HCL and then with distilled water and allowed to air dry in the laboratory.  During collection, samples are stored on ice in these containers within the body of the sampler.  The sampler was programmed to flush the collection line prior to collecting each sample. As soon as possible after the final collection time has ended, the samples were returned to the laboratory for nutrient and Chla analysis.

4)  Site Location and Character

Thousands of years ago, the Escatawpa River built a sizable delta in southeast Mississippi.  The deterioration of this delta began when the Escatawpa River changed its course hundreds of years ago.  In 1999, the retrograding delta became home to the 24th National Estuarine Research Reserve, the Grand Bay NERR.

The Grand Bay NERR is part of the Grand Bay coastal streams watershed which consists of three primary sub-watersheds that provide much of the freshwater inputs into the system. The Bayou Heron site, located in the upper reaches of the Bayou Heron sub-watershed, monitors water quality for a semi-pristine area with no development and serves as a reference site for the reserve. The Bayou Cumbest site monitors water quality for the Bayou Cumbest sub-watershed which is a moderately impacted area with some residential housing development and non-point source pollution issues related to failing septic tanks (i.e., elevated levels of fecal coliforms).  Located towards the southern end of the Bangs Lake sub-watershed, the Bangs Lake site monitors water quality for a second impacted area with minimal residential development.  Although this sub-watershed has less development than the Bayou Cumbest sub-watershed, a recent study found extremely elevated levels of fecal coliforms.  Subsequent investigations of the Bangs Lake area showed a small drainage ditch running directly into the northern end of the sub-watershed from several residential housing units with failing septic systems.  This site called Bangs Lake North (BN) was chosen for diel monitoring after highly acidic runoff breached a containment system owned and operated by a nearby fertilizer manufacturer in early April 2005.  An estimated 18.5 million gallons of contaminated runoff was released, with an unknown amount entering Bangs Lake.  The fifth water quality monitoring site is located in Point Aux Chenes Bay, near the southern boundary of the Reserve.  This site provides baseline data of the relative influence of marine inputs and tidal influence from the East Mississippi Sound.

The five water quality monitoring sites within the Grand Bay NERR have a tidal range of approximately 2 feet.  Additional site specific characteristics are as follows:

a)	Bayou Heron (BH):  [30° 25.068’ N, 88° 24.324’W].   The Bayou Heron site provides water quality data for the Bayou Heron sub-watershed.  The water depth range is from .16 m to 1.69 m and the salinity range is from 0 ppt to 25.3 ppt.  Salinity tends to decrease rapidly when rain events occur at the site or within the watershed.  Because the sensor is located .25 m from the bottom and there are no major sources of freshwater into this bayou, it has been concluded that some source of freshwater exist close to the bottom of the bayou.  Hydrology studies are being designed to further understand this occurrence.  Also, two sets of grab samples are taken, one just below the surface and the second at sonde depth, to determine if stratification is occurring at this site as well.  This site tends to become hypoxic during warmer months (March – October).  The bottom habitat is soft sediments.

b)	Point Aux Chenes (PC):  [30° 20.916’N, 88° 25.112’W].   The Point Aux Chenes site is the most southern water quality site within the boundaries of Grand Bay NERR.  It is located in Point Aux Chenes Bay and is highly influenced by the Mississippi Sound.   The water depth range is from .3 m to 1.6 m and the salinity range is from 17.2 ppt to 32.1 ppt.  The bottom habitat is soft sediments.

c)	Bayou Cumbest (BC):  [30° 23.016’N, 88° 26.184’W].   The Bayou Cumbest site monitors water quality for the Bayou Cumbest sub-watershed.  The water depth range is from 0 m to 1.10 m and the salinity range is from 0.5 ppt to 27.8 ppt.  The site is located in soft sediments with unconsolidated oyster shell reefs.

- Bangs Lake (BL):  [30° 21.426’N, 88° 27.774’W].  The Bangs Lake site monitors the                           water quality of the Bangs Lake sub-watershed.  The water depth range is from 0 m to 1.37 m and the salinity range is from 9.6 ppt to 28.0 ppt. The site is located on a soft sediment bottom.

- Bangs Lake North (BN):  [30o 22.017’N, 88o 28.260’W].  The Bangs Lake North site monitors the water quality of the north end of Bangs Lake.  This site was added in order to monitor the effects of millions of gallons of highly acidic runoff entering the lake during an extremely heavy rain event in April 2005.  The runoff escaped a breached containment system owned and operated by a nearby fertilizer manufacturer northwest of Bangs Lake.  This site is very similar to Bangs Lake (BL) in water depth, salinity range and bottom habitat.

5)  Coded variable definitions

a)	Station codes (column ‘A’ of nutrient data report):

gndbhnut = Grand Bay NERR, site Bayou Heron nutrient data

gndpcnut = Grand Bay NERR, site Point Aux Chenes nutrient data

gndbcnut = Grand Bay NERR, site Bayou Cumbest nutrient data

gndblnut = Grand Bay NERR, site Bangs Lake nutrient data

gndbnnut= Grand Bay NERR, site Bangs Lake North nutrient data

b)	Monitoring program (column “C” of nutrient data report):

1 = Monthly grab sample

2 = Diel grab sample

c)	Nutrient parameter comment code columns (denoted with a ‘F_’ and found in columns immediately proceeding reported data variable. Refer to section 10 for parameter titles and variable names by data category.)

6)  Data collection period

The first nutrient samples were collected at all sites in March 2005.  Individual collection dates and times for both the monthly grab program and diel program are reported in column B of the nutrient database.

Monthly Grab Samples

| Station Code | DateTimeStamp |
| --- | --- |
| *gndbhnut | 1/9/2007 11:55 |
| *gndbhnut | 1/9/2007 11:56 |
| gndbhnut | 1/9/2007 12:00 |
| gndbhnut | 1/9/2007 12:01 |
| gndpcnut | 1/9/2007 09:50 |
| gndpcnut | 1/9/2007 09:51 |
| gndbcnut | 1/9/2007 11:05 |
| gndbcnut | 1/9/2007 11:06 |
| gndblnut | 1/9/2007 10:35 |
| gndblnut | 1/9/2007 10:36 |
| gndbnnut | 1/9/2007 10:43 |
| gndbnnut | 1/9/2007 10:44 |
| *gndbhnut | 2/13/2007 11:35 |
| *gndbhnut | 2/13/2007 11:36 |
| gndbhnut | 2/13/2007 11:30 |
| gndbhnut | 2/13/2007 11:31 |
| gndpcnut | 2/13/2007 09:55 |
| gndpcnut | 2/13/2007 09:56 |
| gndbcnut | 2/13/2007 10:42 |
| gndbcnut | 2/13/2007 10:43 |
| gndblnut | 2/13/2007 10:20 |
| gndblnut | 2/13/2007 10:21 |
| gndbnnut | 2/13/2007 10:28 |
| gndbnnut | 2/13/2007 10:29 |
| *gndbhnut | 3/13/2007 06:35 |
| *gndbhnut | 3/13/2007 06:36 |
| gndbhnut | 3/13/2007 06:30 |
| gndbhnut | 3/13/2007 06:31 |
| gndpcnut | 3/13/2007 06:55 |
| gndpcnut | 3/13/2007 06:56 |
| gndbcnut | 3/13/2007 07:25 |
| gndbcnut | 3/13/2007 07:26 |
| gndblnut | 3/13/2007 07:00 |
| gndblnut | 3/13/2007 07:01 |
| gndbnnut | 3/13/2007 07:05 |
| gndbnnut | 3/13/2007 07:06 |
| *gndbhnut | 4/11/2007 04:03 |
| *gndbhnut | 4/11/2007 04:04 |
| gndbhnut | 4/11/2007 03:58 |
| gndbhnut | 4/11/2007 03:59 |
| gndpcnut | 4/11/2007 04:15 |
| gndpcnut | 4/11/2007 04:16 |
| gndbcnut | 4/11/2007 05:00 |
| gndbcnut | 4/11/2007 05:01 |
| gndblnut | 4/11/2007 05:38 |
| gndblnut | 4/11/2007 05:39 |
| gndbnnut | 4/11/2007 05:44 |
| gndbnnut | 4/11/2007 05:45 |
| *gndbhnut | 5/15/2007 18:20 |
| *gndbhnut | 5/15/2007 18:21 |
| gndbhnut | 5/15/2007 18:25 |
| gndbhnut | 5/15/2007 18:26 |
| gndpcnut | 5/15/2007 18:05 |
| gndpcnut | 5/15/2007 18:06 |
| gndbcnut | 5/15/2007 17:25 |
| gndbcnut | 5/15/2007 17:26 |
| gndblnut | 5/15/2007 17:40 |
| gndblnut | 5/15/2007 17:41 |
| gndbnnut | 5/15/2007 17:50 |
| gndbnnut | 5/15/2007 17:51 |
| *gndbhnut | 6/11/2007 14:20 |
| *gndbhnut | 6/11/2007 14:21 |
| gndbhnut | 6/11/2007 14:25 |
| gndbhnut | 6/11/2007 14:26 |
| gndpcnut | 6/11/2007 14:41 |
| gndpcnut | 6/11/2007 14:42 |
| gndbcnut | 6/11/2007 15:20 |
| gndbcnut | 6/11/2007 15:21 |
| gndblnut | 6/11/2007 15:00 |
| gndblnut | 6/11/2007 15:01 |
| gndbnnut | 6/11/2007 15:10 |
| gndbnnut | 6/11/2007 15:11 |
| *gndbhnut | 7/10/2007 16:02 |
| *gndbhnut | 7/10/2007 16:01 |
| gndbhnut | 7/10/2007 16:05 |
| gndbhnut | 7/10/2007 16:06 |
| gndpcnut | 7/10/2007 16:22 |
| gndpcnut | 7/10/2007 16:23 |
| gndbcnut | 7/10/2007 17:06 |
| gndbcnut | 7/10/2007 17:07 |
| gndblnut | 7/10/2007 16:42 |
| gndblnut | 7/10/2007 16:43 |
| gndbnnut | 7/10/2007 16:52 |
| gndbnnut | 7/10/2007 16:53 |
| *gndbhnut | 8/7/2007 17:00 |
| *gndbhnut | 8/7/2007 17:01 |
| gndbhnut | 8/7/2007 17:05 |
| gndbhnut | 8/7/2007 17:06 |
| gndpcnut | 8/7/2007 17:35 |
| gndpcnut | 8/7/2007 17:36 |
| gndbcnut | 8/7/2007 17:55 |
| gndbcnut | 8/7/2007 17:56 |
| gndblnut | 8/7/2007 18:25 |
| gndblnut | 8/7/2007 18:26 |
| gndbnnut | 8/7/2007 18:35 |
| gndbnnut | 8/7/2007 18:36 |
| *gndbhnut | 9/4/2007 14:30 |
| *gndbhnut | 9/4/2007 14:31 |
| gndbhnut | 9/4/2007 14:35 |
| gndbhnut | 9/4/2007 14:36 |
| gndpcnut | 9/4/2007 13:20 |
| gndpcnut | 9/4/2007 13:21 |
| gndbcnut | 9/4/2007 13:55 |
| gndbcnut | 9/4/2007 13:56 |
| gndblnut | 9/4/2007 13:35 |
| gndblnut | 9/4/2007 13:36 |
| gndbnnut | 9/4/2007 13:40 |
| gndbnnut | 9/4/2007 13:41 |
| *gndbhnut | 10/2/2007 13:45 |
| *gndbhnut | 10/2/2007 13:46 |
| gndbhnut | 10/2/2007 13:50 |
| gndbhnut | 10/2/2007 13:51 |
| gndpcnut | 10/2/2007 14:10 |
| gndpcnut | 10/2/2007 14:11 |
| gndbcnut | 10/2/2007 14:50 |
| gndbcnut | 10/2/2007 14:51 |
| gndblnut | 10/2/2007 14:22 |
| gndblnut | 10/2/2007 14:23 |
| gndbnnut | 10/2/2007 14:35 |
| gndbnnut | 10/2/2007 14:36 |
| *gndbhnut | 11/13/2007 09:40 |
| *gndbhnut | 11/13/2007 09:41 |
| gndbhnut | 11/13/2007 09:45 |
| gndbhnut | 11/13/2007 09:46 |
| gndpcnut | 11/13/2007 10:10 |
| gndpcnut | 11/13/2007 10:11 |
| gndbcnut | 11/13/2007 11:20 |
| gndbcnut | 11/13/2007 11:21 |
| gndblnut | 11/13/2007 10:35 |
| gndblnut | 11/13/2007 10:36 |
| gndbnnut | 11/13/2007 10:44 |
| gndbnnut | 11/13/2007 10:45 |
| *gndbhnut | 12/11/2007 08:55 |
| *gndbhnut | 12/11/2007 08:56 |
| gndbhnut | 12/11/2007 09:00 |
| gndbhnut | 12/11/2007 09:01 |
| gndpcnut | 12/11/2007 10:05 |
| gndpcnut | 12/11/2007 10:06 |
| gndbcnut | 12/11/2007 09:25 |
| gndbcnut | 12/11/2007 09:26 |
| gndblnut | 12/11/2007 09:40 |
| gndblnut | 12/11/2007 09:41 |
| gndbnnut | 12/11/2007 09:50 |
| gndbnnut | 12/11/2007 09:51 |

* Surface samples.

Diel Samples

| Station Code | DateTimeStamp |
| --- | --- |
| gndblnut | 1/9/2007 10:40 |
| gndblnut | 1/9/2007 12:45 |
| gndblnut | 1/9/2007 14:50 |
| gndblnut | 1/9/2007 16:55 |
| gndblnut | 1/9/2007 19:00 |
| gndblnut | 1/9/2007 21:05 |
| gndblnut | 1/9/2007 23:10 |
| gndblnut | 1/10/2007 01:15 |
| gndblnut | 1/10/2007 03:20 |
| gndblnut | 1/10/2007 05:25 |
| gndblnut | 1/10/2007 07:30 |
| gndblnut | 1/10/2007 09:35 |
| gndblnut | 2/12/2007 03:51 |
| gndblnut | 2/12/2007 05:56 |
| gndblnut | 2/12/2007 08:01 |
| gndblnut | 2/12/2007 10:06 |
| gndblnut | 2/12/2007 12:11 |
| gndblnut | 2/12/2007 14:16 |
| gndblnut | 2/12/2007 16:21 |
| gndblnut | 2/12/2007 18:26 |
| gndblnut | 2/12/2007 20:31 |
| gndblnut | 2/12/2007 22:36 |
| gndblnut | 2/13/2007 00:41 |
| gndblnut | 2/13/2007 02:46 |
| gndblnut | 3/12/2007 03:13 |
| gndblnut | 3/12/2007 05:18 |
| gndblnut | 3/12/2007 07:23 |
| gndblnut | 3/12/2007 09:28 |
| gndblnut | 3/12/2007 11:33 |
| gndblnut | 3/12/2007 13:38 |
| gndblnut | 3/12/2007 15:43 |
| gndblnut | 3/12/2007 17:48 |
| gndblnut | 3/12/2007 19:53 |
| gndblnut | 3/12/2007 21:58 |
| gndblnut | 3/13/2007 00:03 |
| gndblnut | 3/13/2007 02:08 |
| gndblnut | 4/10/2007 01:20 |
| gndblnut | 4/10/2007 03:25 |
| gndblnut | 4/10/2007 05:30 |
| gndblnut | 4/10/2007 07:35 |
| gndblnut | 4/10/2007 09:40 |
| gndblnut | 4/10/2007 11:45 |
| gndblnut | 4/10/2007 13:50 |
| gndblnut | 4/10/2007 15:55 |
| gndblnut | 4/10/2007 18:00 |
| gndblnut | 4/10/2007 20:05 |
| gndblnut | 4/10/2007 22:10 |
| gndblnut | 4/11/2007 00:15 |
| gndblnut | 5/14/2007 17:17 |
| gndblnut | 5/14/2007 19:22 |
| gndblnut | 5/14/2007 21:27 |
| gndblnut | 5/14/2007 23:32 |
| gndblnut | 5/15/2007 01:37 |
| gndblnut | 5/15/2007 03:42 |
| gndblnut | 5/15/2007 05:47 |
| gndblnut | 5/15/2007 07:52 |
| gndblnut | 5/15/2007 09:57 |
| gndblnut | 5/15/2007 12:02 |
| gndblnut | 5/15/2007 14:07 |
| gndblnut | 5/15/2007 16:12 |
| gndblnut | 6/10/2007 16:07 |
| gndblnut | 6/10/2007 18:12 |
| gndblnut | 6/10/2007 20:17 |
| gndblnut | 6/10/2007 22:22 |
| gndblnut | 6/11/2007 00:27 |
| gndblnut | 6/11/2007 02:32 |
| gndblnut | 6/11/2007 04:37 |
| gndblnut | 6/11/2007 06:42 |
| gndblnut | 6/11/2007 08:47 |
| gndblnut | 6/11/2007 10:52 |
| gndblnut | 6/11/2007 12:57 |
| gndblnut | 6/11/2007 15:02 |
| gndblnut | 7/9/2007 16:03 |
| gndblnut | 7/9/2007 18:08 |
| gndblnut | 7/9/2007 20:13 |
| gndblnut | 7/9/2007 22:18 |
| gndblnut | 7/10/2007 00:23 |
| gndblnut | 7/10/2007 02:28 |
| gndblnut | 7/10/2007 04:33 |
| gndblnut | 7/10/2007 06:38 |
| gndblnut | 7/10/2007 08:43 |
| gndblnut | 7/10/2007 10:48 |
| gndblnut | 7/10/2007 12:53 |
| gndblnut | 7/10/2007 14:58 |
| gndblnut | 8/6/2007 14:55 |
| gndblnut | 8/6/2007 17:00 |
| gndblnut | 8/6/2007 19:05 |
| gndblnut | 8/6/2007 21:10 |
| gndblnut | 8/6/2007 23:15 |
| gndblnut | 8/7/2007 01:20 |
| gndblnut | 8/7/2007 03:25 |
| gndblnut | 8/7/2007 05:30 |
| gndblnut | 8/7/2007 07:35 |
| gndblnut | 8/7/2007 09:40 |
| gndblnut | 8/7/2007 11:45 |
| gndblnut | 8/7/2007 13:50 |
| gndblnut | 9/3/2007 13:23 |
| gndblnut | 9/3/2007 15:28 |
| gndblnut | 9/3/2007 17:33 |
| gndblnut | 9/3/2007 19:38 |
| gndblnut | 9/3/2007 21:43 |
| gndblnut | 9/3/2007 23:48 |
| gndblnut | 9/4/2007 01:53 |
| gndblnut | 9/4/2007 03:58 |
| gndblnut | 9/4/2007 06:03 |
| gndblnut | 9/4/2007 08:08 |
| gndblnut | 9/4/2007 10:13 |
| gndblnut | 9/4/2007 12:18 |
| gndblnut | 10/1/2007 11:39 |
| gndblnut | 10/1/2007 13:44 |
| gndblnut | 10/1/2007 15:49 |
| gndblnut | 10/1/2007 17:54 |
| gndblnut | 10/1/2007 19:59 |
| gndblnut | 10/1/2007 22:04 |
| gndblnut | 10/2/2007 00:09 |
| gndblnut | 10/2/2007 02:14 |
| gndblnut | 10/2/2007 04:19 |
| gndblnut | 10/2/2007 06:24 |
| gndblnut | 10/2/2007 08:29 |
| gndblnut | 10/2/2007 10:34 |
| gndblnut | 11/12/2007 08:45 |
| gndblnut | 11/12/2007 10:50 |
| gndblnut | 11/12/2007 12:55 |
| gndblnut | 11/12/2007 15:00 |
| gndblnut | 11/12/2007 17:05 |
| gndblnut | 11/12/2007 19:10 |
| gndblnut | 11/12/2007 21:15 |
| gndblnut | 11/12/2007 23:20 |
| gndblnut | 11/13/2007 01:25 |
| gndblnut | 11/13/2007 03:30 |
| gndblnut | 11/13/2007 05:35 |
| gndblnut | 11/13/2007 07:40 |
| gndblnut | 12/10/2007 07:53 |
| gndblnut | 12/10/2007 09:58 |
| gndblnut | 12/10/2007 12:03 |
| gndblnut | 12/10/2007 14:08 |
| gndblnut | 12/10/2007 16:13 |
| gndblnut | 12/10/2007 18:18 |
| gndblnut | 12/10/2007 20:23 |
| gndblnut | 12/10/2007 22:28 |
| gndblnut | 12/11/2007 00:33 |
| gndblnut | 12/11/2007 02:38 |
| gndblnut | 12/11/2007 04:43 |
| gndblnut | 12/11/2007 06:48 |

7)  Associated researchers and projects

Additional SWMP datasets collected at Grand Bay NERR include meteorological and water quality data.  These data can be obtained from the Research Coordinator or the CDMO website at http://cdmo.baruch.sc.edu.

- The use of remote sensing and molecular detection to predict the risk of infection by Vibrio parahaemolyticus – Adrienne Russell-Flowers (Ph.D. Candidate) and Dr. Jay Grimes (Professor) - Gulf Coast Research Laboratory, University of Southern Mississippi

- Quantification of dissolved organic nitrogen (DON) concentration in the Grand Bay NERR – Dr. Kevin Dillon (Professor) – Gulf Coast Research Laboratory, University of Southern Mississippi and Christine Walters, Grand bay National Estuarine Research Reserve.

- Linkages between tidal creek ecosystems and landscape and demographic attributes of their watersheds – Dr. Fred Holland (Director) and others – Hollings Marine Laboratory, NOAA, Gretchen Grammer (Research Technician) - Grand Bay National Estuarine Research Reserve), and Dr. Mark Woodrey (Research Coordinator/Research Biologist), Coastal Research and Extension Center, Mississippi State University and Grand Bay National Estuarine Research Reserve

8)  Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR water quality data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Section 1. Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu.  Data are available in text tab-delimited format.

II. Physical Structure Descriptors:

9)  Entry verification

Samples were collected and analysis performed in accordance with Weeks Bay National Estuarine Research Reserve’s Standard Operating Procedures for Water Chemistry.  Analysis data was recorded in both a laboratory log book and electronically in spreadsheet form. This data was then transferred in general formatting into the comprehensive Excel form employed by the NERR system for yearly reporting purposes. Data was checked twice for transfer accuracy.  Christine Walters was responsible for these tasks.

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and automatically flags and codes values below MDL; calculates parameters chosen by the user and automatically flags for component values below MDL and negative values; allows the user to apply QAQC flags and codes to the data; graphs selected parameters for review; append files; and export the resulting data files to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  Christine Walters was responsible for these tasks.

10)  Parameter titles and variable names by data category

Laboratory parameters

Data Category		Parameter				Variable	Unit

Phosphorous		Total Dissolved Phosphorous		TDP		mg/L as P				Orthophosphate, filtered		PO4F		mg/L as P				Dissolved Organic Phosphate		DOP		mg/L as P

Nitrogen		Nitrite + Nitrate, filtered		NO23F	mg/L as N

Nitrite, filtered				NO2F		mg/L as N				Nitrate, filtered			NO3F		mg/L as N				Ammonium, filtered			NH4F		mg/L as N				Dissolved Inorganic Nitrogen		DIN		mg/L as N

Plant pigments		Chlorophyll a				CHLA N	ug/L

Pheaophytin				PHEA		ug/L

11)  Measured and Calculated Laboratory Parameters

- Directly measured variables

Nitrogen species: NO23F, NO2F, NH4F

Phosphorus species: PO4F, TDP

Other: CHLA N, PHEA (See Section 13 Lab Methods)

b)	Calculated variables

Nitrogen species: NO3F = NO23F – NO2F, DIN = NO23F + NH4F

Phosphorus species: DOP = TDP – PO4F

12)  Limits of detection

Method Detection Limit (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect, has been established by the Weeks Bay laboratory technicians for each parameter.  The MDL is determined as 3 times the standard deviation of a minimum of 7 replicates of a single low concentration sample.

| Parameter | Start Date | End Date | MDL |
| --- | --- | --- | --- |
| CHLA_N | 1/1/2007 | 12/31/2007 | .05 |
| NH4F | 1/1/2007 | 12/31/2007 | .01 |
| NO23F | 1/1/2007 | 12/31/2007 | .01 |
| NO2F | 1/1/2007 | 12/31/2007 | .01 |
| PHEA | 1/1/2007 | 12/31/2007 | .05 |
| PO4F | 1/1/2007 | 12/31/2007 | .01 |
| TDP | 1/1/2007 | 12/31/2007 | .01 |

13)  Laboratory Methods

# Parameter:  Chlorophyll a and Pheophytin (January through April)

Method References:

Weeks Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 10-18. 10200 H. Chlorophyll - Spectrophotometric Determination of Chlorophyll.

Method Descriptor:

Instrumentation:  Spectrophotometer (Spectronic Genesys 5).

The method used requires filtering a known quantity of water through a glass fiber filter (4.7 cm GF/F).  This filter is stored in 10mLs of 90% acetone/10% saturated magnesium carbonate solution in the freezer at -200C for at least one week for extraction.  The samples are centrifuged, transferred into cuvettes, and absorbance determined at 664 nm (using an absorbance of 750nm to correct for background absorbance) Samples are then acidified to convert all chlorophyll a to pheophytin and absorbance determined at 665nm.

Preservation Method:

A known quantity of water is filtered through a glass fiber filter (4.7 cm GF/F).  This filter is stored in 10mLs of 90% acetone/10% saturated magnesium carbonate solution in the freezer at -20oC until analysis or for at least one week to allow for extraction of pigments.

b)	Parameter:  Chlorophyll a (May through December)

Method References:

Weeks Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 10-18. 10200 H. Chlorophyll - Fluorometric Determination of Chlorophyll.

EPA Method 445.0 In Vitro Determination of Chlorophyll a by Fluorescence revision 1.2 pp. 22.

Method Descriptor:

Instrumentation:  Fluorometer (Turner Designs Trilogy)

The method used requires filtering a known quantity of water through a glass fiber filter (4.7 cm GF/F).  This filter is stored dry in a freezer at -20°C until extraction.  In preparation for extraction, the filter is placed in a 15 mL centrifuge tube with 10mLs of DMSO/aqueous acetone solution.  The tube is then placed in a dark freezer for a minimum of 1 hour for extraction.  After extraction is complete, the tube is removed from the freezer and stored in a dark room for 30 minutes to allow for temperature equilibration.  Three milliliters of the sample is then removed from the tube and placed in a 1.0 cm glass (or methacrylate) fluorometer cell.  Fluorescence is read at excitation = 485 nm and emission = 685 nm (note: emission filter must be accurate to within 10 nm).   Chlorophyll a concentration of the sample is determined by comparison with a standard curve of known chlorophyll a concentrations.  The Turner Designs Trilogy performs this determination automatically against a standard curve with known concentrations that has been programmed into its memory.

Preservation Method:

A known quantity of water is filtered through a glass fiber filter (4.7 cm GF/F).  This filter is stored dry in a freezer at -20°C until extraction.

c)	Parameter:  Ammonia

# Method References:

Weeks Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 4-108. 4500-NH3  F.  Phenate Method.

Method Descriptor:

Instrumentation: Spectrophotometer (Spectronic Genesys 5).

An intensely blue compound, indophenol, is formed by the reaction of ammonia, hypochlorite, and phenol catalyzed by sodium nitroprusside.  The indophenol blue is proportional to the ammonia concentration.  The color develops at room temperature (22 to 27oC) in subdued light after 1 hour and is stable for 24 hours.  Absorbance is measured with a spectrophotometer at 640 nm.

Preservation Method:

Sample is filtered as soon as possible after collection.  Ammonia analysis is begun as soon after filtering as possible.  If necessary, samples can be held in a refrigerator at 4oC for a short period of time until analysis.

- Parameter:  Nitrite

# Method References:

Weeks Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 4-112. 4500-NO2-  B.  Colorimetric Method.

Method Descriptor:

Instrumentation: Spectrophotometer (Spectronic Genesys 5).

Nitrite is determined through formation of a reddish purple dye produced at pH 2.0 to 2.5 by coupling diazotized sulfanilamide with N-(1-naphthyl)-ethylenediamine dihydrochloride (NED dihydrochloride).  The color system obeys Beer’s law up to 180 g N/L with a 1-cm light path at 543 nm.

Preservation Method:

Sample is filtered as soon as possible after collection.  Nitrite analysis is begun as soon after filtering as possible.  If necessary, samples can be held in a refrigerator at 4oC for a short period of time until analysis.

- Parameter:  Nitrite + Nitrate

# Method References:

Weeks Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 4-117. 4500-NO3-  E.  Cadmium Reduction Method.

Method Descriptor:

Instrumentation: Spectrophotometer (Spectronic Genesys 5).

Nitrate is reduced almost quantitatively to nitrite in the presence of cadmium (Cd).  This method uses commercially available Cd granules treated with copper sulfate and packed in a glass column.  The nitrite produced thus is determined by diazotizing with sulfanilamide and coupling with N-(1-naphthyl)-ethylenediamine dihydrochloride to form a highly colored azo dye that is measured colorimetrically.

Preservation Method:

Sample is filtered as soon as possible after collection.  Analysis is begun as soon after filtering as possible.  If necessary, samples can be held in a refrigerator at 4oC for a short period of time until analysis.

- Parameter:  Dissolved Reactive Phosphorus

# Method References:

Weeks Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 4-146. 4500-P  E.  Ascorbic Acid Method.

Method Descriptor:

Instrumentation: Spectrophotometer (Spectronic Genesys 5).

Ammonium molybdate and potassium antimonyl tartrate react in acid medium with orthophosphate to form a heteropoly acid – phosphomolybdic acid – that is reduced to intensely colored molybdenum blue by ascorbic acid.  Measure absorbance of each sample at 880 nm.

Preservation Method:

Sample is filtered as soon as possible after collection.  Analysis is begun as soon after filtering as possible.  If necessary, samples can be held in a refrigerator at 4oC for a short period of time until analysis.

# g)	Parameter:  Total Dissolved Phosphorus

# Method References:

Weeks Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 4-142. 4500-P  B.  5. Persulfate Digestion Method. Then method 4500-P E. Ascorbic Acid Method (see above).

Method Descriptor:

Instrumentation: Spectrophotometer (Spectronic Genesys 5).

Di, tri, poly and organic phosphates are oxidized to mono-phosphates using K2S2O8 (potassium persulphate) and heat. Mono-phosphates are then determined using the Ascorbic Acid Method outlined above (see Dissolved Reactive Phosphate).

Preservation Method:

Sample is filtered as soon as possible after collection.  Analysis is begun as soon after filtering as possible.  If necessary, samples can be held in a refrigerator at 4oC for a short period of time until analysis.

14)  Field and Laboratory QA/QC programs

  - Precisions:

- Field Variability – True field replicates are taken at each site during grab sampling.  The one replicate is a successive grab.  Sample XXXXXX-G1 is taken and then sampler emptied.  The grab sampler is deployed once again to acquire XXXXXX-G2.

ii)   Laboratory variability – none

iii)  Inter-organizational splits – Samples were not split or analyzed by     	 two different labs.

  - Accuracy:

- Sample spikes – information unavailable

- Standard reference material analysis – information unavailable

- Cross calibration exercises – GND NERR did not participate in cross calibration exercises.

15)  QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range*

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

*The -4 Outside Low Sensor Range flag was added to the 2007 dataset in August of 2011.  See the Other Remarks section for more details.

16)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data rejected due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Value calculated with a value that is below the MDL

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N	from the north

NNE	from the north northeast

NE	from the northeast

ENE	from the east northeast

E	from the east

ESE	from the east southeast

SE	from the southeast

SSE	from the south southeast

S	from the south

SSW	from the south southwest

SW	from the southwest

WSW	from the west southwest

W	from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17)  Other remarks/notes

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded with -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

*The 2007 dataset was updated on August of 2011 to include the -4 Outside Low Sensor Range flag.  The 2007 data published prior to that time used the -3 Rejected data flag with the SBL and SCB QAQC codes to indicate that data were below the minimum detection limit.  These flag code combinations were all replaced with the -4 SBL or SCB update as mandated by the Data Management Committee.

It is possible for interested researchers to correlate meteorological and abiotic water quality parameters with diel and grab sampling times by referencing the 2007 Grand Bay NERR meteorological and water quality data available via the Central Data Management Office or contacting the Grand Bay NERR Research Coordinator.

Chlorophyll a samples from January through April were analyzed using spectrophometric determination of chlorophyll a.  This method determines both chlorophyll a and pheophytin.  Chlorophyll a samples from May through December were analyzed using fluorometric determination of chlorophyll a.  This method determines only chlorophyll a.  Therefore, as of May 2007 samples were no longer analyzed for pheophytin.

July chlorophyll a values are missing.  These samples were filtered and stored; however, the filters were misplaced prior to analysis.

Total dissolved phosphorus values are missing for January.  Two separate techniques of digestion (boiling and baking) were attempted.  Neither set of values were reported, as they did not agree with one another and there was not enough samples left to rerun the analysis.

Ammonia values for February are missing.  Samples were contaminated.

The total dissolved phosphorus value for 4/10/2007 gndblnut @ 2210 is contaminated.

The ammonia value for 5/15/2007 gndbnnut @ 1750 is contaminated.