Grand Bay (GND) NERR Nutrient Metadata

January - December 2015

Last Updated:  September 23, 2016

I.  Data Set and Research Descriptors

1)  Principal Investigator(s) and Contact Persons:

Address:		Grand Bay National Estuarine Research Reserve

6005 Bayou Heron Road

Moss Point, MS  39562

Phone:  (228) 475-7047

Fax:      (228) 475-8097

Contact Persons:	Dr. Mark Woodrey, Research Coordinator

E-mail:  mark.woodrey@msstate.edu

(228) 475-7047, ext. 107

Kim Cressman, Research Technician

E-mail:  kimberly.cressman@dmr.ms.gov

(228) 475-7047, ext. 119

2)  Research objectives:

a)	Monthly Grab Sampling Program

Monthly grab samples were collected to quantify the spatial variability of important nutrients in the water column between sites representing the local salinity gradient.

b)	Diel Sampling Program

Once per month, samples were collected every 125 minutes through a tidal cycle to quantify the temporal variability of important nutrients and sediment loading in the water column as a function of tidal forcing.

3)  Research methods:

a)	Monthly Grab Sampling Program

Monthly grab samples were taken at six stations within the Grand Bay estuary.  Samples were taken at the four Grand Bay data sonde (primary SWMP) stations: Bayou Heron (BH), Point Aux Chenes (PC), Bayou Cumbest (BC), and Bangs Lake (BL).  Samples were also taken at two secondary SWMP sites: the surface of Bayou Heron (HS), due to stratification, and at a spill site in Bangs Lake called Bangs Lake North (BN).  Attempts were made to collect grab samples within two hours of slack low tide.  Rainfall conditions prior to grab sampling were not considered.  When possible, grab samples were obtained in conjunction with diel samples (See 3b).  At the Heron Surface and Point Aux Chenes stations, grab samples were collected 0.25 meters below the surface to avoid surface scum.  The Point Aux Chenes station is well-mixed and it was determined that sampling at sonde depth was unnecessary.  All other grab samples were collected at sonde depth, 0.25 meters from the bottom.  Bayou Heron required the use of a Wildco Horizontal Beta Bottle to obtain the samples.  Samples were collected in 500ml Nalgene HDPE (high-density polyethylene) wide-mouthed amber bottles.  Prior to sampling, sample bottles were rinsed with 10% HCL and then with distilled water and allowed to air dry in the laboratory.  Sample bottles were also rinsed three times with sample water at the sample site prior to sample collection.  Samples were immediately placed on ice, stored in a dark cooler, and returned to the laboratory. Once in the laboratory samples were processed for nutrient and Chla analysis.

b)	Diel Sampling Program

Twelve diel samples were collected each month at Bangs Lake using an ISCO 6712 portable auto-sampler 0.25m from the bottom.  The sampler was programmed to take a 500ml collection every 125 minutes.  No distinction between neap and spring tide was considered.  Prior to collection, 500 ml semi-transparent ISCO sample containers were rinsed with 10% HCL and then with distilled water and allowed to air dry in the laboratory.  During collection, samples were stored on ice in these containers within the body of the sampler.  The sampler was programmed to flush the collection line prior to collecting each sample. As soon as possible after the final collection time ended, the samples were returned to the laboratory for nutrient and Chla analysis.

4)  Site Location and Character

The Grand Bay Delta was created by sediments from both the Pascagoula and Escatawpa Rivers several thousand to hundreds of years ago.  Soon after the delta was created, the Escatawpa became a major tributary to the Pascagoula River when its flow was captured by the larger river system.  With the Escatawpa’s freshwater inflow shunted to the Pascagoula River, sediment discharge to Grand Bay Delta and the delta’s further growth was terminated. The meandering delta channels of the Escatawpa became exclusively tidal water courses a few hundred years ago, after cessation of river flow (Otvos 2007).  In 1999, the retrograding delta became home to the 24th National Estuarine Research Reserve, the Grand Bay NERR.

The Grand Bay NERR is part of the Coastal Stream Basin watershed, which consists of three primary sub-watersheds that provide much of the freshwater inputs into the system: Bayou Heron, Bayou Cumbest, and Bangs Lake.

The water quality monitoring sites within the Grand Bay NERR have a tidal range of approximately 0.5 m.  Additional site specific characteristics are as follows:

a) Bayou Heron (BH) and Bayou Heron Surface (HS):  [30.4178 N, 88.4054 W]

From the beginning of the nutrient sampling program in 2005 until early 2016, both surface and bottom samples from Bayou Heron were labeled gndbhnut in the data. Metadata records differentiated between the depths. Because there was not a straightforward way to differentiate between them in the data files themselves, the surface site was re-named Bayou Heron Surface, gndhsnut. It is considered a secondary SWMP station.  The bottom site is at sonde depth (0.25 meters above the sediment) and will retain primary SWMP status and the name gndbhnut. Diel samples collected at this site have always been taken at sonde depth.  Only surface samples will be affected by the change. We are in the process of re-naming the surface samples in older data files; please read all metadata carefully. Moving forward, surface samples will be gndhsnut.

The Bayou Heron site, located in the middle reaches of the Bayou Heron sub-watershed, monitors water quality for a semi-pristine area with little development and serves as a reference site for the reserve.  Freshwater input is derived from several sources including networks of hydric drains, sheet flow from pine flatwoods/savannas, and groundwater.  Hydric drains import large amounts of surface water from the sub-watershed, including water originating north of Hwy 90 and Interstate 10.  Franklin Creek, located northeast of the reserve, flows WNW into the Escatawpa River.  Franklin Creek drains a large portion of agricultural land on the Grand Bay plateau and periodically crests into the Bayou Heron sub-watershed during high water events via a small network of hydric drains.  These drains eventually deposit water into numerous tidal creeks that enter Bayou Heron.  Little is known about the quantity and quality of water flowing through these drains.

- Depth range: 0.16 – 1.69 m

- Salinity range in 2015:  0.0 – 29.8 ppt

- Average Salinity in 2015: 17.0 (± 8.6) ppt

- Typical salinity range: 5.9 – 27.6 ppt

The above typical salinity range represents 90% of the data points collected from 2005-2011 (n=217,996)

- Median salinity from 2005-2011: 20.9 ppt

- Bottom habitat: soft sediments

Due to water column stratification and poor mixing, this site can become naturally hypoxic during warmer months (March – October).

b) Point Aux Chenes (PC):  [30.3486 N, 88.4185 W].

This is the most southern water quality site within the boundaries of Grand Bay NERR.  It is located in Point Aux Chenes Bay, which is which is directly influenced by the Mississippi Sound, receiving inputs from waters to the east, including Mobile Bay.  This site is also potentially influenced, to an unknown degree, by Bangs Lake and Bayou Cumbest outflows.  This station was established in 2005 to replace the Crooked Bayou water quality monitoring station.  There are no known direct pollutant sources into Point Aux Chenes Bay, although a large industrial complex is located along the western boundary of the Reserve. This site provides baseline data on the relative influence of marine inputs and tidal influence from the East Mississippi Sound.

- Depth range: 0.3 – 1.6 m

- Salinity range in 2015:  10.3 – 31.9 ppt

- Average Salinity in 2015:  23.4 (± 5.6) ppt

- Typical salinity range: 12.4 – 29.7 ppt

The above typical salinity range represents 90% of the data points collected from 2005-2011 (n=195,836)

- Median salinity from 2005-2011: 24.2

- Bottom habitat: soft sediments

c) Bayou Cumbest (BC):  [30.3836 N, 88.4364 W].

The Bayou Cumbest site monitors water quality for the Bayou Cumbest sub-watershed, which is a moderately impacted area with some residential housing development and non-point source pollution issues related to failing septic tanks (i.e., elevated levels of fecal coliforms; LaSalle 1997).  A substantial canal, called the Nine Mile Canal, was built in the late 1930’s and connects the Escatawpa River to Bayou Cumbest upstream from the water quality station.

- Depth range: 0.0 – 1.10 m

- Salinity range in 2015: 0.2 – 31.3 ppt

- Average Salinity in 2015: 15.8 (± 7.9) ppt

- Typical salinity range: 3.8 – 28.4 ppt

The above typical salinity range represents 90% of the data points collected from 2005-2011 (n=211,045)

- Median salinity from 2005-2011: 19.1

- Bottom habitat: soft sediments with fringing oyster shell reefs

d) Bangs Lake (BL):  [30.3571 N, 88.4629 W].

The Bangs Lake site is located towards the southern end of the Bangs Lake sub-watershed, an area with minimal residential development.  Adjacent parcels include Chevron USA oil and gas refinery and the Mississippi Phosphates industrial facility.  Both sites are surrounded by containment levees constructed to direct any contaminant spills towards Bayou Casotte, a heavily industrialized and impacted area to the west of these facilities.  However, a spill from a gypsum pile at the phosphate facility was discharged into Bangs Lake in 2005 and had substantial negative impacts.  A man-made drainage ditch runs into the north part of Bangs Lake and is believed to drain a residential area, which may have failing septic tanks (LaSalle 1997).  The ditch is also adjacent to the Jackson County Industrial Water Plant.  Bangs Lake has been impacted by high fecal coliform counts and a fecal coliform TMDL was developed for the Bayou Cumbest and Bangs Lake watersheds in 2000 (MSU-CREC 2000).

- Depth range: 0.0 – 1.37 m

- Salinity range in 2015:  6.3 – 31.6 ppt

- Average Salinity in 2015: 21.1 (± 5.7) ppt

- Typical salinity range: 13.1 – 29.6 ppt

The above typical salinity range represents 90% of the data points collected from 2005-2011 (n=201,833)

- Median salinity from 2005-2011: 23.1

- Bottom habitat: soft sediments

e)   Bangs Lake North (BN):  [30.3669 N, 88.471 W].

The Bangs Lake North site monitors the water quality of the north end of Bangs Lake.  This site was added in order to monitor the effects of millions of gallons of highly acidic runoff that breached a nearby fertilizer manufacturer’s containment system and entered the lake during an extremely heavy rain event in April 2005.  This site is very similar to Bangs Lake (BL) in water depth, salinity range and bottom habitat.

Sources cited within this section:

LaSalle, M.W. (1997). Water Quality Monitoring of Shellfish Growing Waters and Residential Rock-Reed Wastewater Treatment Systems at Bayou Cumbest, Mississippi.  Final Report to the Gulf of Mexico Program.  58 pp.

Mississippi State University Coastal Research and Extension Center (MSU-CREC). (2000). Fecal Coliform TMDL for Bayou Cumbest/Bangs Lake Watershed, Coastal Streams Basin, Jackson County, MS.  Prepared for the Mississippi Department of Environmental Quality. Approved Final Version May 5, 2000.

Otvos, E.G. (2007). Geological Framework and Evolution History. Pages 22-46 in Grand Bay National Estuarine Research Reserve: An Ecological Characterization (Peterson, M.S., G.L. Waggy and M.S. Woodrey, editors).  Grand Bay National Estuarine Research Reserve, Moss Point, Mississippi.

5)  Coded variable definitions

a)	Station codes (column ‘A’ of nutrient data report):

gndbhnut = Grand Bay NERR, site Bayou Heron (sonde depth) nutrient data

gndhsnut = Grand Bay NERR, site Bayou Heron Surface nutrient data

gndpcnut = Grand Bay NERR, site Point Aux Chenes nutrient data

gndbcnut = Grand Bay NERR, site Bayou Cumbest nutrient data

gndblnut = Grand Bay NERR, site Bangs Lake nutrient data

gndbnnut= Grand Bay NERR, site Bangs Lake North nutrient data

b)	Monitoring program (column “C” of nutrient data report):

1 = Monthly grab sample

2 = Diel grab sample

c)	Nutrient parameter comment code columns (denoted with a ‘F_’ and found in columns immediately following reported data variable. Refer to section 10 for parameter titles and variable names by data category.)

6)  Data collection period

The first nutrient samples were collected at all sites in March 2005.  Individual collection dates and times for both the monthly grab program and diel program are reported in column B of the nutrient database.

Monthly Grab Samples

| Station Code | Date | Rep 1 Time | Rep 2 Time |
| --- | --- | --- | --- |
| gndhsnut | 1/21/15 | 9:44 | 9:45 |
| gndhsnut | 2/19/15 | 9:04 | 9:05 |
| gndhsnut | 3/19/15 | 7:31 | 7:32 |
| gndhsnut | 4/21/15 | 9:48 | 9:49 |
| gndhsnut | 5/11/15 | 8:19 | 8:20 |
| gndhsnut | 6/11/15 | 14:43 | 14:44 |
| gndhsnut | 7/9/15 | 15:31 | 15:32 |
| gndhsnut | 8/21/15 | 12:41 | 12:42 |
| gndhsnut | 9/21/15 | 15:30 | 15:31 |
| gndhsnut | 10/16/15 | 9:43 | 9:44 |
| gndhsnut | 11/12/15 | 10:41 | 10:42 |
| gndhsnut | 12/16/15 | 10:32 | 10:33 |
| gndbhnut | 1/21/15 | 9:49 | 9:50 |
| gndbhnut | 2/19/15 | 9:09 | 9:10 |
| gndbhnut | 3/19/15 | 7:36 | 7:37 |
| gndbhnut | 4/21/15 | 9:53 | 9:54 |
| gndbhnut | 5/11/15 | 8:24 | 8:25 |
| gndbhnut | 6/11/15 | 14:48 | 14:49 |
| gndbhnut | 7/9/15 | 15:36 | 15:37 |
| gndbhnut | 8/21/15 | 12:46 | 12:47 |
| gndbhnut | 9/21/15 | 15:35 | 15:36 |
| gndbhnut | 10/16/15 | 9:48 | 9:49 |
| gndbhnut | 11/12/15 | 10:46 | 10:47 |
| gndbhnut | 12/16/15 | 10:37 | 10:38 |
| gndpcnut | 1/21/15 | 10:14 | 10:15 |
| gndpcnut | 2/20/15 | 10:40 | 10:41 |
| gndpcnut | 3/19/15 | 8:05 | 8:06 |
| gndpcnut | 4/21/15 | 10:26 | 10:27 |
| gndpcnut | 5/11/15 | 5:51 | 5:52 |
| gndpcnut | 6/11/15 | 15:20 | 15:21 |
| gndpcnut | 7/9/15 | 13:44 | 13:45 |
| gndpcnut | 8/21/15 | 10:20 | 10:21 |
| gndpcnut | 9/21/15 | 12:51 | 12:52 |
| gndpcnut | 10/16/15 | 7:33 | 7:34 |
| gndpcnut | 11/12/15 | 8:39 | 8:40 |
| gndpcnut | 12/16/15 | 8:15 | 8:16 |
| gndbcnut | 1/21/15 | 11:32 | 11:33 |
| gndbcnut | 2/20/15 | 9:15 | 9:16 |
| gndbcnut | 3/19/15 | 9:16 | 9:17 |
| gndbcnut | 4/21/15 | 10:51 | 10:52 |
| gndbcnut | 5/11/15 | 6:22 | 6:23 |
| gndbcnut | 6/11/15 | 15:42 | 15:43 |
| gndbcnut | 7/9/15 | 14:05 | 14:06 |
| gndbcnut | 8/21/15 | 11:44 | 11:45 |
| gndbcnut | 9/21/15 | 14:25 | 14:26 |
| gndbcnut | 10/16/15 | 8:48 | 8:49 |
| gndbcnut | 11/12/15 | 9:08 | 9:09 |
| gndbcnut | 12/16/15 | 9:48 | 9:49 |
| gndblnut | 1/21/15 | 10:35 | 10:36 |
| gndblnut | 2/20/15 | 9:33 | 9:34 |
| gndblnut | 3/19/15 | 8:58 | 8:59 |
| gndblnut | 4/21/15 | 11:23 | 11:24 |
| gndblnut | 5/11/15 | 7:02 | 7:03 |
| gndblnut | 6/11/15 | 16:02 | 16:03 |
| gndblnut | 7/9/15 | 14:38 | 14:39 |
| gndblnut | 8/21/15 | 11:10 | 11:11 |
| gndblnut | 9/21/15 | 13:40 | 13:41 |
| gndblnut | 10/16/15 | 8:14 | 8:15 |
| gndblnut | 11/12/15 | 9:41 | 9:42 |
| gndblnut | 12/16/15 | 9:08 | 9:09 |
| gndbnnut | 1/21/15 | 10:47 | 10:48 |
| gndbnnut | 2/20/15 | 9:53 | 9:54 |
| gndbnnut | 3/19/15 | 8:32 | 8:33 |
| gndbnnut | 4/21/15 | 11:12 | 11:13 |
| gndbnnut | 5/11/15 | 6:51 | 6:52 |
| gndbnnut | 6/11/15 | 16:10 | 16:11 |
| gndbnnut | 7/9/15 | 14:25 | 14:26 |
| gndbnnut | 8/21/15 | 10:54 | 10:55 |
| gndbnnut | 9/21/15 | 13:16 | 13:17 |
| gndbnnut | 10/16/15 | 8:04 | 8:05 |
| gndbnnut | 11/12/15 | 9:32 | 9:33 |
| gndbnnut | 12/16/15 | 8:54 | 8:56 |

Monthly Diel Samples

| Station | Start Date | Start Time | End Date | End Time |
| --- | --- | --- | --- | --- |
| gndblnut | 1/11/2015 | 8:27 | 1/12/2015 | 7:22 |
| gndblnut | 2/25/2015 | 13:50 | 2/26/2015 | 12:45 |  |
| gndblnut | 3/18/2015 | 5:29 | 3/19/2015 | 4:24 |  |
| gndblnut | 4/20/2015 | 10:49 | 4/21/2015 | 9:44 |  |
| gndblnut | 5/9/2015 | 23:39 | 5/10/2015 | 22:34 |  |
| gndblnut | 6/10/2015 | 18:56 | 6/11/2015 | 17:51 |  |
| gndblnut | 7/8/2015 | 15:21 | 7/9/2015 | 14:16 |
| gndblnut | 8/4/2015 | 19:54 | 8/5/2015 | 12:24 |  |
| gndblnut | 9/20/2015 | 13:07 | 9/21/2015 | 12:02 |  |
| gndblnut | 10/15/2015 | 7:55 | 10/16/2015 | 6:50 |  |
| gndblnut | 11/11/2015 | 6:35 | 11/12/2015 | 5:30 |  |
| gndblnut | 12/15/2015 | 9:50 | 12/16/2015 | 8:45 |  |

7)  Associated researchers and projects

Several research and monitoring projects, in addition to educational workshops and outreach events, are currently using nutrient data from the Grand Bay NERR.  In addition to nutrient data, the NERR SWMP program also generates meteorological and water quality data sets that are available at www.nerrsdata.org.

Research and Monitoring

- Water Quality in Bangs Lake: effects of recurrent phosphate spills to a coastal estuary – Dr. Kevin Dillon (Gulf Coast Research Laboratory, University of Southern Mississippi), Dr. Mark Woodrey (Grand Bay NERR/Mississippi State University), Kim Cressman (Grand Bay NERR), Larissa Graham (Grand Bay NERR), Dr. Ruth Carmichael (Dauphin Island Sea Lab), Dr. Jane Caffrey (University of West Florida)

- Understanding long-term changes by linking monthly chlorophyll measurements to high-frequency water quality data – Kim Cressman (Grand Bay NERR), Dr. Jane Caffrey (University of West Florida), Dr. Marcus Beck (USEPA Gulf Breeze Laboratory), Cher Griffin (Grand Bay NERR)

- Macroinvertebrates in Grand Bay NERR – Dr. Ayesha Gray, Dr. Mark Woodrey, Will Weathersby, Ron Cole, Kim Cressman, Cher Griffin (Grand Bay NERR)

- Use of SPATT Bags to detect Harmful Algal Blooms – Shareena Cannonier, Dr. Michael Abezinge (ECSC, Florida A&M University)

Education, Outreach, and Training

- SWMPrats.net – interactive widgets for data viewing – Dr. Marcus Beck (USEPA Gulf Breeze Lab) and Dr. Todd O’Brien (NOAA)

- Grand Bay NERR Interpretive Center display includes real-time weather data

- Grandbaynerr.org displays real-time weather data on the front page

- National Estuaries Day

- Teacher workshops

8)  Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: www.nerrsdata.org; accessed 12 October 2012.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma separated version format.

II. Physical Structure Descriptors:

9)  Entry verification

Samples were collected and analysis performed in accordance with Grand Bay National Estuarine Research Reserve’s Standard Operating Procedures for Water Chemistry.  Analysis data was recorded in both a laboratory log book and electronically in spreadsheet form. This data was then transferred in general formatting into the comprehensive Excel form employed by the NERR system for yearly reporting purposes. Data was checked twice for transfer accuracy.  Kim Cressman was responsible for these tasks.

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; adds chosen parameters and facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and then automatically flags/codes measured values below MDL and inserts the MDL; calculates parameters chosen by the user and automatically flags/codes for component values below MDL, negative calculated values, and missing data; allows the user to apply QAQC flags and codes to the data; produces summary statistics; graphs selected parameters for review; and exports the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  Kim Cressman was responsible for these tasks.

10)  Parameter titles and variable names by data category

Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisk “*”.

Laboratory parameters

Data Category		Parameter				Variable	Unit

Phosphorus		*Orthophosphate, filtered		PO4F		mg/L as P

Nitrogen		*Nitrite + Nitrate, filtered		NO23F		mg/L as N

*Ammonium, filtered			NH4F		mg/L as N				Dissolved Inorganic Nitrogen		DIN		mg/L as N

Plant pigments	*Chlorophyll a				CHLA N	g/L

Notes:

1.  Time is coded based on a 2400 clock and is referenced to Standard Time.

2.  Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.  NO2 has been shown to be a minor component in the Grand Bay NERR system by data from 2005-2011. Through July 2012, NO2 was measured when NO23 was above the detection limit. The majority of NO2 samples measured during this time period were below detection. In August 2012, Grand Bay NERR completely stopped measuring NO2 as a separate parameter and began measuring NO23 exclusively.

11)  Measured and Calculated Laboratory Parameters

- Parameters measured directly

Nitrogen species: NO23F, NH4F

Phosphorus species: PO4F

Other: CHLA_N

b)	Calculated parameters

Nitrogen species: 	DIN = NO23F + NH4F

12)  Limits of detection

Method Detection Limit (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect, has been established by the Grand Bay laboratory technicians for each parameter.  The MDL is determined as 3 times the standard deviation of a minimum of 7 replicates of a single low concentration sample.  These values are reviewed and revised periodically.

| Parameter | Start Date | End Date | MDL |
| --- | --- | --- | --- |
| CHLA_N | 01/01/15 | 12/31/15 | 0.01 |
| NH4F | 01/01/15 | 12/31/15 | 0.002 |
| NO23F | 01/01/15 | 02/28/15 | 0.005 |
|  | 03/01/15 | 12/31/15 | 0.007 |
| PO4F | 01/01/15 | 12/31/15 | 0.002 |

13)  Laboratory Methods

a)	Parameter:  Chlorophyll a (CHLA_N)

Method References:

Grand Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 10-18. 10200 H. Chlorophyll - Fluorometric Determination of Chlorophyll.

EPA Method 445.0 In Vitro Determination of Chlorophyll a by Fluorescence revision 1.2 pp. 22.

Method Descriptor:

Instrumentation:  Fluorometer (Turner Designs Trilogy)

The method used requires filtering a known quantity of water through a glass fiber filter (4.7 cm GF/F).  This filter is stored dry in a freezer at -20°C until extraction.  In preparation for extraction, the filter is placed in a 15 mL centrifuge tube with 10mL of 90% aqueous acetone.  The tube is then placed in a dark refrigerator for 18-24 hours.  After extraction is complete, the tube is removed from the freezer and stored in a dark room for 30 minutes to allow for temperature equilibration.  Three milliliters of the sample is then removed from the tube and placed in a 1.0 cm glass (or methacrylate) fluorometer cell.  Fluorescence is read at excitation = 485 nm and emission = 685 nm (note: emission filter must be accurate to within 10 nm).   Chlorophyll a concentration of the sample is determined by comparison with a standard curve of known chlorophyll a concentrations.  The Turner Designs Trilogy performs this determination automatically against a standard curve with known concentrations that has been programmed into its memory.

Preservation Method:

A known quantity of water is filtered through a glass fiber filter (4.7 cm GF/F).  This filter is stored dry in a freezer at -20°C until extraction.

b)	Parameter:  Ammonia (NH4F)

# Method References:

Grand Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 4-108. 4500-NH3  F.  Phenate Method.

Method Descriptor:

Instrumentation: Spectrophotometer (Shimadzu UV-2550).

An intensely blue compound, indophenol, is formed by the reaction of ammonia, hypochlorite, and phenol catalyzed by sodium nitroprusside.  The indophenol blue is proportional to the ammonia concentration.  The color develops at room temperature (22 to 27oC) in subdued light after 1 hour and is stable for 24 hours.  Absorbance is measured with a spectrophotometer at 640 nm.

Preservation Method:

Sample is filtered as soon as possible after collection.  Ammonia analysis is begun as soon after filtering as possible.  If necessary, samples can be held in a refrigerator at 4oC for a short period of time until analysis.

c)	Parameter:  Nitrite + Nitrate (NO23F)

NOTE: NO2 is a minor component of NO23 in the Grand Bay NERR system, as evidenced by data from 2005-2011. Through July 2012, NO2 was measured when NO23 was above the detection limit and was otherwise assumed to be below detection. The majority of NO2 samples measured during this time period were below detection. In August 2012, Grand Bay NERR completely stopped measuring NO2 as a separate parameter and began measuring NO23 exclusively.

# Method References:

Grand Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 4-117. 4500-NO3-  E.  Cadmium Reduction Method.

Method Descriptor:

Instrumentation: Spectrophotometer (Shimadzu UV-2550).

Nitrate is reduced almost quantitatively to nitrite in the presence of cadmium (Cd).  This method uses commercially available Cd granules treated with copper sulfate and packed in a glass column.  The nitrite produced thus is determined by diazotizing with sulfanilamide and coupling with N-(1-naphthyl)-ethylenediamine dihydrochloride to form a highly colored azo dye that is measured colorimetrically.

Preservation Method:

Sample is filtered as soon as possible after collection.  Analysis is begun as soon after filtering as possible.  If necessary, samples can be held in a refrigerator at 4oC for a short period of time until analysis.

e)	Parameter:  Dissolved Reactive Phosphorus (PO4F)

# Method References:

Grand Bay National Estuarine Research Reserve - SOP

Standard Methods for the Examination of Water and Wastewater, 20th edition. p 4-146. 4500-P  E.  Ascorbic Acid Method.

Method Descriptor:

Instrumentation: Spectrophotometer (Shimadzu UV-2550).

Ammonium molybdate and potassium antimonyl tartrate react in acid medium with orthophosphate to form a heteropoly acid – phosphomolybdic acid – that is reduced to intensely colored molybdenum blue by ascorbic acid.  Measure absorbance of each sample at 880 nm.

Preservation Method:

Sample is filtered as soon as possible after collection.  Analysis is begun as soon after filtering as possible.  If necessary, samples can be held in a refrigerator at 4oC for a short period of time until analysis.

14)  Field and Laboratory QA/QC programs

  - Precisions:

- Field Variability – True field replicates are taken at each site during grab sampling.  The one replicate is a successive grab.  Sample XXXXXX-G1 is taken and then sampler emptied.  The grab sampler is deployed once again to acquire XXXXXX-G2.

ii)   Laboratory variability – Duplicates and spikes are analyzed with each monthly batch of samples.

iii)  Inter-organizational splits – Samples were not split or analyzed by     	 different labs.

  - Accuracy:

- Sample spikes – Spikes are performed monthly. 0.5 mL of 1 mg/L standard is added to a replicate sample, which should increase the concentration of the analyte by ~0.02 mg/L.

- Standard reference material analysis – QC standards are ordered from North Central Labs for NO23, NH4, and PO4 annually.

- Cross calibration exercises – GND NERR did not participate in cross calibration exercises in 2015.

15)  QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

16)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data suspect due to QA/QC checks

GSM	See metadata

Sensor errors

SBL		Value below minimum limit of method detection

SCB	Calculated value could not be determined due to a below MDL component

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS		Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS		Lab analysis from unpreserved sample

Cloud cover

CCL		clear (0-10%)

CSP		scattered to partly cloudy (10-50%)

CPB		partly to broken (50-90%)

COC	overcast (>90%)

CFY		foggy

CHY	hazy

CCC		cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR		light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE		ebb tide

TSF		flood tide

TSH	high tide

TSL		low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N		from the north

NNE	from the north northeast

NE		from the northeast

ENE	from the east northeast

E		from the east

ESE		from the east southeast

SE		from the southeast

SSE		from the south southeast

S		from the south

SSW	from the south southwest

SW		from the southwest

WSW	from the west southwest

W		from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17)  Other remarks/notes

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

February: Sampling occurred over multiple days due to weather issues.

March:

3/18/15 20:04 and 3/19/15 2:19 – Diel samples not processed due to likely contamination from presence of dead fish in bottles.

April:

4/20/15 10:49 – 4/21/15 9:44 – Diel samples collected from HIGH tide to HIGH tide this month (normally low to low)

4/20/15 21:14 and 23:19 – Diel samples not collected; bottles did not fill due to low tide

June:

6/11/15 17:51 – last diel sample not collected due to time constraints.

August:

8/4/15 12:41 – 8/5/15 12:24: Diel samples collected every 90 minutes due to a shorter-than-normal tidal cycle.

8/5/2015 12:24 – last diel sample not collected due to impending storms.

September:

NO23 and DIN: All rejected due to suspected glassware contamination. The standard curve and other laboratory QC checks for NO23 were not acceptable, so all NO23 data points were rejected. Because NO23 is part of the calculation for DIN, DIN was also rejected. Many NO23 data points were below detection, and DIN was not calculated for those data points before these parameters were rejected, which is why some DIN points are missing.

October:

10/15/2015 18:20 – chlorophyll a sample not analyzed; lid of extraction tube was cracked and acetone could have evaporated, skewing the result.