Great Bay Research Reserve (GRB) NERR Meteorological Metadata

January 1 – December 31, 2011

Latest Update:  July 13, 2017

# I. Data Set and Research Descriptors

1) Principal investigators and contact persons

Jaimie Wolf, Research Technician

email: jaimie.wolf@unh.edu

Jackson Estuarine Laboratory

85 Adams Point Road

Durham, NH 03824

Phone: 603-862-5122

Thomas K. Gregory, Research Scientist

email: tom.gregory@unh.edu

Jackson Estuarine Laboratory

85 Adams Point Road

Durham, NH 03824

Ocean Process Analysis Lab

University of New Hampshire

Phone: 603-862-5136

Jonathan Pennock, Ph.D., Professor of Natural Resources,

Director of the Jackson Estuarine Laboratory and the

UNH Marine Program

email: Jonathan.Pennock@unh.edu

Chase Ocean Eng. Room 102

U.N.H.

Durham, NH 03824

Phone: (603) 862-2921

Paul E. Stacey

Research Coordinator

Great Bay National Estuarine Research Reserve

New Hampshire Fish & Game Department

225 Main Street, Durham, NH 03824

Phone (603) 868-1095

Email: mailto:Paul.Stacey@wildlife.nh.gov

2)  Entry verification

Data are uploaded from the CR1000 data logger to a personal computer (IBM compatible).  Files are exported from Loggernet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range. The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

The data were downloaded to the PC once a month. Files are stored on the PC as well as backed up onto an external hard drive. Jaimie Wolf was responsible for data management and QA/QC for the 2011 Meteorological data set.

## 3) Research objectives

The principal objective is to record long-term meteorological data at the Great Bay Estuary in order to aid with the interpretation of changes in water quality, as well as to identify and catalog extreme weather events, and to maintain an historical database of weather conditions at the Great Bay Reserve.

4) Research methods

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees),  Battery Voltage (volts)

Maximum and Minimum Air Temperature (°C) and their times from 5-second data (these data are available from the Reserve)

Maximum Wind Speed (m/s) and time from 5-second data

Wind Direction Standard Deviation (degrees)

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm)

A Campbell TX312 GOES satellite transmitter is used for real-time access to the weather data. A monthly download interval was chosen to coincide with the required monthly physical inspection of the weather station and probes. Sensors are removed and sent back to Campbell Scientific for calibration according to NERR CDMO protocols. During monthly QA/QC checks, a Kestrel 4000 handheld meter is used for rough data accuracy comparisons.

Campbell Scientific data telemetry equipment was installed at the GRB Greenland (grbglmet) station on 07/13/06 and transmits data to the NOAA GOES satellite, NESDIS ID #3B023714. The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The "real-time" telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO's authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu/.

5) Site location and character

The GRBNERR weather station is located in an open cornfield in Greenland, NH, on the eastern shore of Great Bay. The GPS coordinates for the location are: 43°03'31.56" N, 70°49'49.38" W.

The approximate distance from the weather station location to the NERR sampling sites are as follows:

Great Bay sonde: 1 mile

Squamscott River sonde: 2 miles

Lamprey River sonde: 3 miles

Oyster River sonde: 5 miles

The weather station is located approximately 0.25 mile from the water’s edge of Great Bay. The CR1000 logger and all probes except for the rain gauge are mounted on a collapsible 20 ft (6.1m) tower. The rain gauge is mounted on a separate post nearby. The tower location is approximately 10ft (3m) above mean high water. The surrounding area is open field that is planted with corn in the summer.

The temperature and RH probe is mounted approximately 5ft (1.5m) off the ground. The wind speed and wind direction probes are atop the tower approximately 20ft (6.1m) off the ground. The PAR probe is also approximately 20ft (6.1m) off the ground, mounted on the southernmost location to avoid potential shading from other instrumentation. The BP sensor is mounted in the CR1000 enclosure at a height of 5.9ft (1.8m).

The location lacks AC power, and as a result GRB purchased a Texas Electronics precipitation gauge with snow adapter that uses antifreeze to measure frozen precipitation in winter. This unit was connected November 18th, 2005. The gauge is mounted on a pole approximately 5ft (1.5m) off the ground, 20ft (6.1m) from the main tower.

6) Data collection period

Weather data has been collected at the Great Bay site since April 1997.

This metadata document covers the period of time from January 1, 2011 00:00 through December 31, 2011 23:45.

7) Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR weather data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/. Data are available in comma-separated format.

8) Associated researchers and projects

In addition to this meteorological monitoring dataset, GRB NERR also monitors water quality and nutrient data as part of SWMP.  These data are available from the Research Coordinator or online at http://cdmo.baruch.sc.edu/.

The weather data are used by a number of University and State agency scientists conducting biological, physical and chemical projects in the estuarine and near shore environments.  The data are used by the SWMP program to assist in the interpretation of short and long-term changes in water column conditions.  Some general examples include 1) using rainfall data to understand changes in salinity, 2) using wind speed and direction to understand changes in turbidity and 3) using PAR data to understand changes in fluorescence and chlorophyll a concentrations.

Examples of other projects and researchers that use the data are:

Microbial source tracking and microbial pollution source identification studies by Dr. Steven Jones at UNH (rainfall data)

Sediment resuspension and transport studies by Dr. Larry Ward at UNH (wind speed and direction)

Eel grass modeling by Dr. Fred Short at UNH (light data)

Nutrient and chlorophyll data are used by Mr. Phil Trowbridge, the coastal scientist for the Piscataqua Region Estuaries Partnership (PREP part of USEPA's NEP) and TMDL coordinator for the NH Department of Environmental Services for PREP's State of the Estuaries Reports and for listing impaired waters for the state's 305B report to EPA.

Mr. Chris Nash, director on the NH Shellfish Program for NH's Department of Environmental Services uses rainfall data to enact shellfish bed closures.

Commercial lobster fishermen and recreational striped bass fishermen use water temperature and salinity data.

II. Physical Structure Descriptors

9) Sensor specifications

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #:  HMP45C Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Calibrations:

s/n Z2220075 cal date 5/26/2004

s/n Z2440015 cal date 6/11/2004

s/n Z2220075 cal date 2/13/2006

s/n Z2440015 cal date 1/10/2008 (installed 4/30/2008)

s/n Z2220075 cal date 1/28/10 (installed 3/11/10)

s/n Z2440015 cal date 12/29/10 (installed 1/17/11)

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #:  Vaisala RH45C Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Calibrations:

s/n Z2220075 cal date 5/26/2004

s/n Z2440015 cal date 6/11/2004

s/n Z2220075 cal date 2/13/2006

s/n Z2440015 cal date 1/10/2008 (installed 4/30/2008)

s/n Z2220075 cal date 1/28/10 (installed 3/11/10)

s/n Z2440015 cal date 12/29/10 (installed 1/17/11)

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range: Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Calibrations:

s/n Z2130023 cal date 2/21/06

s/n P5240003 cal date 1/18/08 (installed 4/30/08)

s/n Z2130023 cal date 2/8/10 (installed 3/11/10)

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Calibrations:

s/n WM60353 cal date 3/3/04 NEW (installed 4/30/08)

s/n WM87987 cal date 7/24/08 (installed 2/10/11)

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Calibrations:

s/n WM60353 cal date 3/3/04 NEW (installed 4/30/08)

s/n WM87987 cal date 7/24/08 (installed 2/10/11)

Parameter: Photosynthetically Active Radiation (PAR)

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SZ

Light spectrum waveband:  400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity: 0 to 100%

Sensitivity:  typically 5 µA per 1000 µmoles s-1 m-2

Calibrations:

s/n Q21685 cal date 5/21/01

s/n Q33231 cal date 4/12/04

s/n Q21685 cal date 3/2/06 (installed 4/30/08)

s/n Q33231 cal date 2/10/10 (installed 3/11/10)

Parameter: Precipitation /with antifreeze snowfall adapter

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525WS (rain gauge) CS705 (precip adapter)

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to +/- 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -2.5% from 1 to 2 in./hr; +0, -3.5% from 2 to 3 in./hr

Calibrations:

9/9/09

10/2/10

12/1/11

The CR1000 has two MB of Flash EEPROM that is used to store the operating system.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM (4MB optional) is available for program storage (16K), operating system use, and data storage. Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

The CR1000 was installed July 13, 2006.

10) Coded variable definitions

Sampling Station:	Sampling site code:	Station code:

Greenland			GL				grbglmet

11) QAQC flag definitions – This section details the automated primary and secondary QAQC flag definitions.  Include the following excerpt.

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12) QAQC code definitions – This section details the secondary QAQC Code definitions used in combination with the QAQC flags above.

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment (marked with an * below) codes can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SDG       Suspect due to sensor diagnostics

SIC	Incorrect Calibration Constant, Multiplier or Offset

SIW	Incorrect Wiring

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor Drift

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR	Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CDF	Data Appear to Fit Conditions

CML	Snow melt from previous snowfall event

CRE*	Significant Rain Event

CSM*	See Metadata

CCU	Cause Unknown

CVT*	Possible Vandalism/Tampering

CWE*	Significant weather event

13) Other Remarks/Notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform. Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings. If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.

If data appeared aberrant or unseasonable, they were compared to two local outside sources.

Barometric pressure data were corroborated with data collected by the National Data Buoy Center located at the Isles of Shoals, NH.

http://www.ndbc.noaa.gov/station_page.php?station=IOSN3

Temperature, relative humidity and precipitation data were compared to data collected by the weather station at Morse Hall, University of New Hampshire.  http://www.weather.unh.edu

Remarks on (CSM) coded data:

During 2017 the CDMO discovered an incorrect line in the CR1000 programming.  If RHumidity>100 And RHumidity<108 Then RHumidity=100.  A decision was made by the DMC during 2006 to discontinue correcting >100 RH values to 100. This change was never made in our program and has remained in each updated version until it was removed during 2017.  By correcting all values >100 during data collection we may have missed erroneous values that could have indicated a problem with the RH sensor.  CSM coding was added to all RH data from 2007 until the programming change in 2017.

The ATemp/RH sensor was swapped on 1/17/11.  Data at 1415 and 1430 were rejected.

The tower was down on 2/10/11 1120 to 1230 in order to swap the wind sensor.  All data recorded during this period should be disregarded.

Precipitation data 2/10/11 1300– 3/4/11 0745:  Inspection of the weather station on March 4 revealed that the snowfall adapter had failed. Since we have no means of knowing exactly when it happened, any precipitation data recorded during this period should be compared to another source for accuracy. These data are considered suspect.

Precipitation data is incorrect for the period 3/4/11 0800 to 1630 and cumulative precipitation data is incorrect for 3/4/11 0800 to 3/5/11 0000 due to sensor maintenance.  These data were rejected.

Temperature data is incorrect from 12/12/11 1415 to 12/15/11 1030 due to sensor servicing.  These data were rejected.

Total precipitation data were rejected from 12/12/11 1415 to 12/19/11 1345.  Cumulative precipitation data were rejected 12/12/1415 to 12/20/11 0000.  The precipitation gauge was removed for maintenance.

PAR Flagging and Coding:

PAR data for 2011 are not flagged and coded for sensor drift, other than CSM.  Once the calibration certificates for the next sensor swap become available, the data will be reviewed and flagged and coded for sensor drift if necessary.

Elevated nighttime values are flagged as <1>.

Elevated nighttime PAR values:  Small and positive PAR values are sometimes recorded at night.  These are times of the day during which we can expect that solar radiation is not the cause of these values.  Times for nautical twilight in Portsmouth, NH were selected to define nighttime.  Nautical twilight is defined when the sun is 12 degrees below the horizon. At the beginning or end of nautical twilight, under good atmospheric conditions and in the absence of other illumination, general outlines of ground objects may be distinguishable, but detailed outdoor operations are not possible, and the horizon is indistinct. These times are available here:  http://www.sunrisesunset.com/USA/New_Hampshire.asp

It’s unclear whether these readings are the result of instrument malfunction, possibly caused by moisture, or some other source of light like artificial lighting or moonlight.  These values are flagged as suspect since, while they don’t represent solar radiation, their presence does not necessarily indicate a problem with the PAR sensor or logging system.

Hurricane Irene caused a good deal of rain to fall on 8/28/11.  The precipitation data for 8/28/11 are coded as CRE while the F_Record is coded as CWE.