Great Bay Estuary (GRB) NERR Nutrient Metadata

April 2007 through December 2007

Latest Update:  May 28, 2014

# I. Data Set and Research Descriptors

- Principal Investigator(s) and Contact Persons

  - Principal Investigator

Jonathan Pennock

University of New Hampshire

24 Colovos Road

Durham, NH 03824

Phone: 603 862 2921

e-mail: jonathan.pennock@unh.edu

  - Reserve Contact

Kathy Mills

NH Fish & Game Department

225 Main St.

Durham, NH 03824

###### Phone: 603 868 1095

e-mail: kmills@nhfgd.org

  - Laboratory Contact

Thomas K. Gregory, Research Scientist

University of New Hampshire

Jackson Estuarine Laboratory

85 Adams Point Rd.

Durham, NH 03824

Phone: 603-862-5136

e-mail: tom.gregory@unh.edu

d) McDowell Analytical Lab

Jeff Merriam, Lab Manager

Water Quality Analysis Laboratory

University of New Hampshire

215 James Hall

Durham, NH 03824

Phone: 603-862-2341

e-mail: jeff.merriam@unh.edu

- Research Objectives

Nutrient monitoring in GBNERR has been conducted since the late 1980’s with the basic goal of developing and maintaining temporally intensive long-term datasets of physio-chemical parameters of water quality at locations that are representative of the Great Bay Estuarine System. The Great Bay site is relatively unimpacted, while the three tidal river sites (Lamprey, Oyster and Squamscott) have large drainage basins and are impacted by both point (wastewater treatment plants) and nonpoint sources of pollution. In addition to establishing a baseline of water quality,and understanding the spatial and temporal variability of important indicators of estuarine water quality, the data is used by a number of researchers in the analysis of physical and biological processes.

  - Monthly Grab

Monthly grab samples are collected to quantify the horizontal spatial variability of important nutrients in the water column at sites located in the upper and lower estuary representative of the local salinity and habitat gradients.

  - Diel Sampling Program

Once per month, samples are collected every two and one half hours through a 24-hour tidal cycle to quantify the temporal variability of important nutrients in the water column as a function of tidal dynamics.

3) Research Methods

a) Monthly Grab Sampling Program

## Monthly grab samples are collected at the Great Bay buoy, Squamscott River, Oyster River and Lamprey River locations.  All grab samples from these stations were collected on ebbing tides within 2 hours of low tide in the same 24-hour period. Under normal conditions, all samples are taken by boat except Oyster River which is accessed by car. Two replicate 1-liter (N=2) samples are collected at each station, at a depth of one foot from the surface and filled facing the direction of the current (if any current pattern is detected).  Each bottle is opened individually and rinsed three times with estuarine water.  All samples are collected in Nalgene sample bottles that were previously acid washed (10%), rinsed (3x) with distilled-deionized water and dried.  Bottles are capped and stored on ice for the remainder of the sampling cruise. Upon arrival to the laboratory, samples are processed immediately.

Field parameter data including salinity, temperature, dissolved oxygen and dissolved oxygen percent saturation are collected using a YSI Model 85 Handheld Dissolved Oxygen and Conductivity Instrument.  The meter is operated and calibrated according to manufacturer instructions.  Measurements are made at grab sample depth (0.5 m) immediately following grab sample collection.

Light attenuation readings in the water column are taken using a LI-COR LI-193 Spherical Quantum Sensor while incident solar irradiance is measured with a LI-COR LI-190 Quantum Sensor.  Data from both sensors are logged with a LI-COR LI-1400 Datalogger.  Care is taken to conduct individual casts during times of consistent solar irradiance.  Measurements are made at least every 0.25 m in the upper 2 m then less frequently deeper.  Where possible, eight or more readings are taken per cast.  The light attenuation coefficient for PAR (Kd) is obtained by computing the linear regression of sample depth vs. ln (PAR) then taking the absolute value of the inverse of the slope of the regression.  Values for Kd are considered robust if the r2 of the regression is >0.95 and values failing this test are not included in the dataset.  It should be noted that, according to the manufacturer, K0 is a more appropriate designation for the data measured with this spherical sensor.  The Kd designation is used for the purposes of this data set.

b) Diel Sampling Program

A Sigma autosampler was deployed from a fixed wharf at the sonde location in the Lamprey River. This device automatically collects two 850 ml water samples one minute apart every 2 hours and 30 minutes (providing field replicates).  All samples are pumped into polyethylene sample bottles that were previously acid washed (10% HCL), rinsed (3x) with distilled-deionized water and dried.  At the end of the 24 hr period, the 12 samples are kept in the dark and returned to the laboratory for immediate processing.

4) Site Location and Character

The Great Bay is a macro-tidal drowned river valley estuary located in northern New England.  Seven major tributaries contribute runoff to Great Bay draining a total area of roughly 2,400 km2.  Strong tidal and wind driven currents drive circulation patterns, vertical mixing, and resuspension of sediments, which in turn affect primary productivity.  The estuary contains five major habitat types in the form of mud flat, eelgrass meadows, salt marsh, channel bottom, and rocky shore.  Four permanent stations are monitored as a part of the water quality and nutrient long-term monitoring programs.  They are located in mid Great Bay, and in the Lamprey, Oyster and Squamscott Rivers.

Site #1 Great Bay (GB)

Location: Central area of Great Bay proper.

Coordinates: 43○ 04' 20" N latitude and 70○ 52' 10" W longitude.

Salinity range: 5-32 ppt (seasonally); 0-5ppt from high to low tide.

Temperature range: -1○C to 24○C (seasonally); 0-3○C(from high to low tide

Depth: 6.5 meters at MLW

Tidal height: 2.7 meters

Bottom type: Mud and rock channel bottom

Tidal velocity: maximum 50 cm/sec

Watersheds: Squamscott, Lamprey and Winnicut Rivers plus smaller streams. High tide influence from Little Bay and associated rivers

Pollutant influence: clean reference site

Data sonde deployment (GRB WQ monitoring data set): The sonde was mounted on a communications buoy at 1 meter below the surface. The sonde data  was telemetered to the lab base station via spread spectrum radio. As a back-up measure, the sonde continued to log in unattended mode to avoid potential data loss due to communications failure.

Site #2 Squamscott River (SQ)

Location: Mid channel of the Squamscott River at the Boston and Maine Railroad bridge, Stratham, NH.

Coordinates: 43○ 02' 30" N latitude and 70○ 55' 20" W longitude

Salinity range: 0-30ppt (seasonally); 5-20 ppt from high to low tide

Temperature range: -1○C to 27○C (seasonally);difference of 0-5○ from high to low tide

Depth: 3.5 meters at MLW

Tidal height of 2.7 m

Bottom type: Mud/oyster channel bottom

Tidal velocity: maximum 50 cm/sec

Watersheds: Exeter River, adjacent marshes

Pollutant influence: Urban storm water, agriculture, two municipal wastewater treatment plants, residential septic systems

Data sonde deployment (GRB WQ monitoring data set): The sonde was deployed in a perforated PVC cylinder attached to a piling at a fixed 0.5m height above the bottom and connected to a YSI 6200 DCP equipped with a cellular modem transmitting to the lab base station via internet. The sonde logged in unattended mode for data redundancy.

Site #3 Lamprey River (LR)

Location: West bank of the tidal portion of the Lamprey River, approximately 300 m downstream of the dam at Route 108 in Newmarket, NH

Coordinates: 43○ 04' 48" N latitude and 70○ 56' 04" W longitude

Salinity range: 0 to 27 (seasonally); difference of up to 15 ppt between high and low tides

Temperature range: -1○C to 27○ (seasonally); difference of 0-5○C between high and low tides

Depth: 3.5 m

Tidal height 2.7 m

Bottom type: mud/rock

Tidal velocity: maximum 40 cm/sec

Watershed: Lamprey River

Pollutant influence:  Urban stormwater, adjacent marina, upstream and downstream wastewater treatment plants, upstream agriculture

Data sonde deployment (GRB WQ monitoring data set): The sonde was deployed in a perforated PVC cylinder attached to a piling at a fixed height of 0.5 m above the bottom . The sonde was telemetered via internet using a RASSL satellite transmitter until support for this communications system was withdrawn. The sonde logged in unattended mode to ensure data acquisition at all times.

Site #4 Oyster River (OR)

Location:  in the center channel of the tidal portion of the Oyster River, approximately 300 m downstream of the head of tide dam adjacent to Jackson’s Landing in Durham, NH.

Coordinates: 43○ 08’ 2” N latitude and 70○ 54’ 40” W longitude (43.134 N, 70.911 W in Decimal Degrees)

Salinity range: 0 to 32 (seasonally); difference of up to 15 ppt between high and low tides

Temperature range: -1○C to 27○C (seasonally); difference of up to 5○C between high and low tides

Depth: 0.3 m at mlw, 3 m at highest high tides

Tidal height 2.7 m (maximum)

Bottom type: mud

Tidal velocity: maximum 40 cm/sec

Watershed: Oyster River

Pollutant influence:  urban stormwater, mooring field and crew dock, downstream wastewater treatment plant, upstream agriculture, residential on-site sewage disposal

Data sonde deployment (GRB WQ monitoring data set): Deployed vertically in a PVC tube attached to the stem of a mushroom anchor, approx. 0.5 meters above bottom.  The Oyster River sonde was telemetered by a Sutron GOES satellite transmitter and logged simultaneously in unattended mode.

5) Code Variable Definitions

GRB = Great Bay Estuary NERR

GB = Great Bay Site

LR = Lamprey River Site

SQ = Squamscott River Railroad Trestle Site

OR = Oyster River Site

NUT = Nutrient Sampling Program

1 = Monthly Grab Sample Program

2 = Diel Grab Sample Program

For example, as sample code of ‘GRBLRNUT’ indicates that the sample was collect in the Great Bay NERR at the Lamprey River Site and is being reported as part of the Nutrient Sampling Program Data Report.  A Monitoring Program Code of ‘2’ indicates that the sample was collected as part of the Diel Grab Sample Program.

6) Data Collection Period

Sampling in the Great Bay Estuary is typically limited to the months of April through November as a result of ice formation during the winter months. We were unable to extend the monthly sampling into December in 2007 due to weather.  For 2007, the first water samples for the SWMP nutrient monitoring program were collected April 25, 2007 and the last were collected on November 27, 2007.

Grab Sample Collection:

| Station | Date | Rep 1 | Rep 2 |
| --- | --- | --- | --- |
| grbgbnut | 4/25/2007 | 12:24 | 12:25 |
| grbgbnut | 5/14/2007 | 15:15 | 15:16 |
| grbgbnut | 6/26/2007 | 14:44 | 14:45 |
| grbgbnut | 7/25/2007 | 14:29 | 14:30 |
| grbgbnut | 8/29/2007 | 7:17 | 7:18 |
| grbgbnut | 9/17/2007 | 9:52 | 9:53 |
| grbgbnut | 10/23/2007 | 14:33 | 14:34 |
| grbgbnut | 11/27/2007 | 8:16 | 8:17 |

| Station | Date | Rep 1 | Rep 2 |
| --- | --- | --- | --- |
| grblrnut | 4/25/2007 | 11:50 | 11:51 |
| grblrnut | 5/14/2007 | 13:44 | 13:45 |
| grblrnut | 6/26/2007 | 13:55 | 13:56 |
| grblrnut | 7/25/2007 | 13:08 | 13:09 |
| grblrnut | 8/29/2007 | 6:35 | 6:36 |
| grblrnut | 9/17/2007 | 8:52 | 8:53 |
| grblrnut | 10/23/2007 | 13:38 | 13:39 |
| grblrnut | 11/27/2007 | 6:57 | 6:58 |

| Station | Date | Rep 1 | Rep 2 |
| --- | --- | --- | --- |
| grbornut | 4/25/2007 | 11:22 | 11:23 |
| grbornut | 5/14/2007 | 13:22 | 13:23 |
| grbornut | 6/26/2007 | 14:07 | 14:08 |
| grbornut | 7/25/2007 | 12:50 | 12:51 |
| grbornut | 8/29/2007 | 6:02 | 6:03 |
| grbornut | 9/17/2007 | 8:01 | 8:02 |
| grbornut | 10/23/2007 | 13:52 | 13:53 |
| grbornut | 11/27/2007 | 6:40 | 6:41 |

| Station | Date | Rep 1 | Rep 2 |
| --- | --- | --- | --- |
| grbsqnut | 5/14/2007 | 14:06 | 14:07 |
| grbsqnut | 6/26/2007 | 14:54 | 14:55 |
| grbsqnut | 7/25/2007 | 14:40 | 14:41 |
| grbsqnut | 8/29/2007 | 7:35 | 7:36 |
| grbsqnut | 9/17/2007 | 10:13 | 10:14 |
| grbsqnut | 10/23/2007 | 13:03 | 13:04 |

Diel Sample Collection

| Station | Start Date | Start Time | End Date | End Time |
| --- | --- | --- | --- | --- |
| grblrnut | 4/30/2007 | 9:00 | 5/1/2007 | 7:30 |
| grblrnut | 5/15/2007 | 9:00 | 5/16/2007 | 7:30 |
| grblrnut | 6/20/2007 | 9:00 | 6/21/2007 | 7:30 |
| grblrnut | 7/30/2007 | 10:30 | 7/31/2006 | 9:00 |
| grblrnut | 8/22/2007 | 10:30 | 8/23/2007 | 9:00 |
| grblrnut | 9/24/2007 | 10:00 | 9/25/2007 | 8:30 |
| grblrnut | 10/9/2007 | 09:00 | 10/10/2007 | 0:00 |
| grblrnut | 11/13/2007 | 10:30 | 11/14/2007 | 9:00 |

7) Associated Researchers and Projects

In addition to the Nutrient data set, long-term SWMP Meteorological and Water Quality monitoring are ongoing at GRB NERR.

The SWMP nutrient data set is one of several water quality monitoring programs in the Great Bay Estuary that include both physical and chemical properties.  More information including over ten years of additional nutrient data can be found at http://www.ciceet.unh.edu/.  A spatially explicit non-exhaustive list and description of additional programs can be found at http://granitweb.sr.unh.edu/GreatBay/.

8) Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR water quality/nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Section 1. Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format, Microsoft Excel spreadsheet format and comma-delimited format.

II. Physical Structure Descriptors

9) Entry Verification

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and automatically flags and codes values below MDL; calculates parameters chosen by the user and automatically flags for component values below MDL and negative values; allows the user to apply QAQC flags and codes to the data; graphs selected parameters for review; append files; and export the resulting data files to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

All field and lab data were collected following protocols described in the project QAPP.  Data were verified by UNH laboratory technicians and/or the Principal Investigator.

10) Parameter Titles and Variable Names by Data Category

The following is a list of water-quality and environmental parameters included in the GRB SWMP Nutrient Database.  Required NOAA/NERRS System-wide Monitoring Program water quality parameters are denoted by an asterisks “*”.

Data Category	Parameter		Variable Name	Units of Measure

    - Phosphorus:

Orthophosphate*			PO4F		mg/l as P

    - Nitrogen:

Nitrite + Nitrate, Filtered*		NO23F		mg/l as N

Nitrite, Filtered*			NO2F		mg/l as N

Nitrate, Filtered*			NO3F		mg/l as N

Ammonium, Filtered*			NH4F		mg/l as N

Dissolved Inorganic Nitrogen		DIN		mg/l as N

Dissolved Organic Nitrogen		DON		mg/L as N

Total Dissolved Nitrogen		TDN		mg/L as N

    - Plant Pigments:

Chlorophyll a				CHLA		g/l

Phaeophytin				PHEA		g/l

    - Other Lab Parameters:

Silicate, filtered			SiO4F		mg/l as SI

Total Suspended Solids		TSS		mg/l

e) Field Parameters:

Water Temperature			 WTEM_N	C

Salinity				 SALT_N	 ppt.

Dissolved Oxygen			 DO_N		 mg/l

Dissolved Oxygen Saturation	 	DO_S_N	 %

Total Water Depth			 TDEP_N	 m

Attenuation of Downwelling Irrad.	 Kd_N		 m-1

(See Section 16 QA/QC Code definitions for the following result codes)

Cloud Cover				 CLOUD

Precipitation				 PRECIP

Tidal Stage				 TIDE

Wave Height				 WAVHGT

Wind Direction			 WINDIR

Wind Speed				 WINSPD

Notes:

1. Time is coded based on a 2400 hour clock and is referenced to Standard Time.

2. Reserves have the option of measuring NO2 and NO3, or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

11) Measured and Calculated Laboratory Parameters

The following parameters are reported in the 2007 Nutrient Data Report for GRB

    - Variables Measured Directly

Nitrogen species:		NH4F, NO2F, NO23F, TDN

Phosphorus species:		PO4F

Other:				 SiO4F, TSS, CHLA, PHEA

b) Computed Variables

NO3:				NO23F-NO2F

DIN:				NO23F+NH4F

DON				TDN-NH4F-NO23F

c) Measured Environmental Variables

Water Temperature, Salinity, Dissolved Oxygen, Dissolved Oxygen Saturation, Total Water Depth, Light Attenuation, Cloud Cover, Precipitation, Tidal Stage, Wave Height, Wind Direction, Wind Speed.

12) Limits of Detection

The following Method Detection Limits (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect, have been established for the UNH laboratory that performed analyses in 2006 (Table 1).  Concentrations below the MDL are rejected in the database.  Samples that fall above the maximum range of the chemistry are diluted so that the analysis falls within the analytical range of the chemistry and these values are multiplied by the dilution factor (e.g. 10 for a 1:10 dilution) and reported in the database.

Table 1.  University of New Hampshire Water Quality Analysis Laboratory Method Detection Limits (MDL) for measured water quality parameters.

| Parameter | Variable |  | Range: mg(N or P)/L |  | MDL mg(N,P or SiO2 )/L |  | Dates in use |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ammonium | NH4F |  | 0 – 0.200 |  | 0.005 |  | 2007 Apr-Dec |  |
| Nitrite | NO2F |  | 0 – 10.00 |  | 0.003 |  | 2007 Apr-Dec |  |
| Nitrate | NO3F |  | 0 – 10.00 |  | 0.005 |  | 2007 Apr-Dec |  |
| Nitrate-Nitrite | NO23F |  | 0 – 10.00 |  | 0.005 |  | 2007 Apr-Dec |  |
| Dissolved Organic Nitrogen | DON |  |  |  | 0.005 |  | 2007 Apr-Dec |  |
| Orthophosphate | PO4F |  | 0 – 0.200 |  | 0.005 |  | 2007 Apr-Dec |  |
| Silicate | SiO4F |  | 0-10.00 |  | 0.01 |  | 2007 Apr-Dec |  |
| Chlorophyll a | chla |  | 0-30.00 |  | 0.06 |  | 2007 Apr-Dec |  |
| Pheaophytin |  | Phea |  | 0-30.00 |  | 0.06 |  | 2007 Apr-Dec |  |

Note:  No MDL values are available for TDN or TSS

13) Laboratory Methods

The University of New Hampshire Water Quality Analysis Laboratory, was responsible for the processing of the 2007 GRB nutrient data; methods for each parameter will be presented below.

    - Parameter:  NH4F

University of New Hampshire

    - Method Reference:  Westco Scientific Instruments, SmartChem Discrete Analyzer.  SmartChem Method #210-200B.  Method is based on USEPA 350.1.

Spectrophotometric and Kinetics Investigation of the Bertholt Reaction for the Determination of Ammonia, Analytical Chemistry, 1977, Vol. 49, #3, P.464-469.

    - Method Descriptor:  This method is based on the Berthholt reaction.  Ammonia reacts in alkaline solution with hypochlorite to form monochloramine which, in the presence of phenol, catalytic amounts of nitroprusside (nitroferricyanide) and excess hypochlorite, gives indophenol blue.  The formation of monochloramine requires a pH between 8 and 11.5.  At higher pH, ammonia may begin to oxidize to nitrate.  At pH greater than 9.6, some precipitation of calcium and magnesium as hydroxides and carbonates occurs in seawater, but these ions may be held in solution by complexing them with EDTA.  The indophenol blue measured at 630 nm is proportional to the original ammonia concentration.

    - Preservation Method:  Sample is filtered through a 0.45 um disposable disk filter and stored at –20oC until analyzed.

  - Parameter:  NO23F, NO3F and NO2F

University of New Hampshire

    - Method Reference:  Westco Scientific Instruments, SmartChem Discrete Analyzer.  SmartChem Method #375-100D.  Method is based on USEPA 353.3.

Armstrong, F.A., Stearns, C.R. and Strickland, J.D.H. (1967) The measurement of upwelling and subsequent biological processes by means of the Technicon AutoAnalyzer and associated equipment. Deep Sea Research 14:381-389.

ii) 	Method Descriptor:  Nitrate is quantitatively reduced to nitrite by passage of the sample through a copperized cadmium column. The nitrite (reduced nitrate plus original nitrite) is then determined by diazotization with sulfanilamide under acidic conditions to form a diazonium ion. The resulting diazonium ion is coupled with N-(1-naphthyl)ethylenediamine dihydrochloride. The resulting pink dye absorbs at 520 nm. The procedure is the same for the nitrite analysis less the cadmium column.  Nitrate concentrations are obtained by subtracting nitrite values, which have been previously analyzed, from the nitrite + nitrate values.

Though the method is written for seawater and brackish water, it is also applicable to non-saline sample matrixes. The method is calibrated using standards prepared in deionized water. Once calibrated, samples of varying salinites (0 to 35 ppt) may be analyzed. The determination of background absorbance is necessary only for samples which have color absorbing at 540 nm. The salt effect is less than 2%.

- Preservation Method:  Sample is filtered through a 0.45 um disposable disk filter and stored at –20oC until analyzed.

c)	Parameter:  PO4F

University of New Hampshire

    - Method Reference:  Westco Scientific Instruments, SmartChem Discrete Analyzer.  SmartChem Method #410-200E.  Method is based on USEPA 365.2. Bernhardt, H. and Wilhelms, A. (1967) The continuous determination of low level iron, soluble phosphate, and total phosphate with the AutoAnalyzer. Technicon Symp. 1:386.

    - Method Descriptor:  Ammonium molybdate and antimony potassium tartrate reacts in an acid medium with phosphate to form an antimony-phospho-molybdate complex. This complex is reduced to an intensely blue-colored complex by ascorbic acid. The color produced is proportional to the phosphate concentration in the sample. Though there is a density difference between seawater and reagent water the bias is less than 2%. Though the method is written for seawater and brackish water it is also applicable to non-saline sample matrixes. The method is calibrated using standards prepared in deionized water. Once calibrated, samples of varying salinities (0 to 35 ppt) may be analyzed. The determination of background absorbance is necessary only for samples, which have color absorbing at 880 nm.

    - Preservation Method:  Sample is filtered through a 0.45 um disposable disk filter and stored at –20oC until analysis.

d)	Parameter:  TDN

i.)    Method Reference: Shimadzu Scientific Instruments Inc., TOC-V with TNM-1 Nitrogen Module.  High Temperature Catalytic Oxidation with chemiluminescent detection.  Merriam, J.L., W.H. McDowell, W.S. Currie, 1996.  A high-temperature catalytic oxidation technique for determining total dissolved nitrogen.  Soil Science Society of America Journal, 60(4) 1050-1055.

ii.)   Method Descriptor:  A precisely measured aliquot of filtered sample is injected and combusted on a catalyst at 720 C.  All fixed N is converted to Nitric Oxide (NO) and then coupled with ozone (O3) producing Nitrogen Dioxide* (NO2*) which is measured chemiluminescently.

iii.) Preservation Method:  Sample is filtered through a 0.45 um disposable disk filter and stored at –20oC until analyzed.

    - Parameter:  SiO4F

University of New Hampshire

      - Method Reference:  Westco Scientific Instruments, SmartChem Discrete Analyzer.  SmartChem Method #440-100C.  Method is based on USEPA 370.1.

      - Method Descriptor:  A sample is mixed with Molybdate ion in acidic solution to form a greenish-yellow color complex proportional to the dissolved silica in the sample.  The color complex is measured spectrophotometrically at 660 nm.

      - Preservation Method:  Sample is filtered through a 0.45 um disposable disk filter and stored at -20oC until analysis.  Just prior to analysis, the sample is thawed and stored refrigerated for 2 days to allow re-dissolution of precipitated SiO2.

    - Parameter:  TSS

University of New Hampshire

i) Method Reference: JEL SOP 1.06; adapted from Standard Methods for the Examination of Water and Wastewater. 2540 B. pp. 2-55-56.

ii) Sample water (100-600ml) is vacuum filtered through a pre-weighed 47mm glass microfiber filter. Place filter in drying oven at 80 degrees C for 24 hours for determination of total suspended solids.

g) Parameters CHLA and PHEA

i) Method Reference: JEL SOP 1.06; adapted from Strickland, J. D. H. and T. R. Parsons. 1968. A Practical Handbook of Seawater Analysis. Fisheries Research Board of Canada, Ottowa. Pp. 185-194.

ii) Sample water (100-600ml) is vacuum filtered through a 47mm glass microfiber filter. 5ml of MgCo3 is added to the last 100ml of sample during filtration. The filter is then frozen until further processing. A holding time of three weeks is allowable. Filters are then ground and  mixed with 12ml of 90% Acetone and set into a centrifuge tube to be refridgerated overnight. The following day, samples are shaken and centrifuged for ten minutes, placed into new tubes and centrifuged for five minutes more. Samples are then placed into a 40 mm cuvette to be run in a Shimadzu UV-1601 UV Visible Spectrophotometer.  Absorption is measured at 750, 665, 645 and 630nm. 0.25ml of 50% HCL is then added to each sample. After 15 minutes, the acidified sample is rerun at the same wavelengths to determine pheaophytin concentration.

14) QA/QC Programs

a) Precision

- Field Variability – GBNERR collects two successive grab samples and two successive diel samples for the determination of water mass variability.

- Laboratory Variability – none

- Inter-organizational splits – none

b) Accuracy

- Sample Spikes – blanks

- Standard Reference Material Analysis – see lab protocols

- Cross Calibration Exercises - none

15) QA/QC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range*

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

*The -4 Outside Low Sensor Range flag was added to the 2007 dataset in August of 2011.  See the Other Remarks section for more details.

16) QA/QC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data rejected due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Value calculated with a value that is below the MDL

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N	from the north

NNE	from the north northeast

NE	from the northeast

ENE	from the east northeast

E	from the east

ESE	from the east southeast

SE	from the southeast

SSE	from the south southeast

S	from the south

SSW	from the south southwest

SW	from the southwest

WSW	from the west southwest

W	from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17) Other Remarks

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded with -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

*The 2007 dataset was updated on August of 2011 to include the -4 Outside Low Sensor Range flag.  The 2007 data published prior to that time used the -3 Rejected data flag with the SBL and SCB QAQC codes to indicate that data were below the minimum detection limit.  These flag code combinations were all replaced with the -4 SBL or SCB update as mandated by the Data Management Committee.

Values noted as above the upper limit of method detection (‘SUL’) are rare as analytical dilutions performed to bring all samples below the upper limit of diction by the methods.

Data denoted as missing were generally lost as a result of our inability to access a station as a result of acute weather events or safety concerns that occurred during a sampling effort (i.e. not an event that caused the rescheduling of an entire trip) or loss of sample resulting from sample handling problems during pre-processing in the laboratory.

Light attenuation (Kd):  It should be noted that, according to the manufacturer, K0 is a more appropriate designation for the data measured with this spherical sensor.  The Kd designation is used for the purposes of this data set.  For more information on methodology, please see the Research Methods section.

Deviations from the sampling plan for 2007

January

- Icing Conditions; no samples collected.

February

- Icing Conditions; no samples collected.

March

- Icing Conditions; no samples collected.

April

- NERR samples collected by vehicle due to the lack of channel markers, no access to SQ site.

- Limited PAR measurements (Light Attenuation) taken as a result of vehicle-based sampling.

September

- CHLA measurements were extremely elevated and therefore rejected for LR at 9/25 1:00 and 1:01.  There doesn’t appear to be a problem with sample collection, unnatural contamination or sample analysis, but the samples likely contained some non-phytoplankton chl.  Perhaps leaf or moss particles.  These numbers are not representative of the phytoplankton in the water.

October

- No PAR taken at OR (low tide); too shallow.

- Some samples missing from diel sampling routine due to autosampler malfunction.

November

- SQ samples missing due to limited site access.

December

- No samples collected due to early onset of icing conditions.