Guana Tolomato Matanzas (GTM) National Estuarine Research Reserve

Meteorological Metadata
January - December 2012

Latest Update:  05/20/2014

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons:

Principal Investigator:

Matt Kimball, Research Coordinator

505 Guana River Road

Ponte Vedra Beach, FL 32082

(904) 823-4500

matthew.kimball@dep.state.fl.us

Other Contact Persons:

Kathryn M. Petrinec, SWMP Manager

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4054

kathryn.petrinec@dep.state.fl.us

Tom Harding, SWMP Research Assistant

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4061

tom.harding@dep.state.fl.us

2)  Entry verification:

The meteorological information were collected and stored on a Campbell Scientific CR1000  datalogger.  The Centralized Data Management Office (CDMO) Data Logger Program (GTMPCMET_V5.5_110912.CR1) controls the sensors, performs all data manipulations, and interfaces the CR1000 with the PConnect and LoggerNet software supplied by Campbell Scientific.

Data are periodically downloaded (at least once per month) to a laptop computer or a m105 Palm Pilot and transferred to a Personal Computer (IBM compatible).  Files are exported from Pconnect (Palm Pilot) or Loggernet (laptop) and the data are saved as raw data files in a comma-delimited format with a gtmpcmetmmddyy.dat naming convention that represents the site name, weather station, and the first day data were recorded into the file (e.g., gtmpcmet011012.dat represents a data file with its first row of data obtained on January 10, 2012).  These data files are uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC MS Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

Flagged and/or anomalous data are evaluated.  Data outside the “normal” range of the instruments are investigated for validity based on field observations, QC checks, graphs, and instrument diagnostics.  Data are rejected if the anomalies are attributed to sensor malfunction.  Any additional information on flagged data is noted in Section 13 Other Remarks/See Metadata (CSM).  Common errors observed are relative humidity greater than 100% and precipitation difference of greater than 5 mm in 15 minutes.  Both raw data files and QAQC files are archived on the CDMO FTP server.  Katie Petrinec and Tom Harding are responsible for these tasks.

3)  Research objectives (Campbell Weather Station)

The principal objective is to record and ultimately integrate meteorological data with water quality data in order to better understand the variability, trends, and relationships of these parameters within the GTMNERR.

4)  Research methods

Campbell Scientific data telemetry equipment was installed at the Pellicer Creek (PC) station on 06/19/2006 and transmits data to the NOAA GOES satellite, NESDIS ID #3B01B60E.    (Where #3B01B60E is the GOES ID for that particular station.)  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts)

Maximum, Minimum, and their times from 5-second data:

Maximum and Minimum Air Temperature (°C) and their times (These data are not available in the dataset, but are available from GTMNERR)

Maximum Wind Speed (m/s) and time

Wind Direction Standard Deviation (degrees)

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm)

The CR1000 internally stores up to 2 MB of programs and data.  A maximum four-week sampling interval (i.e., time between data downloads) was used to avoid losing data in the CR1000 and to provide for regular inspection and maintenance of the station.

At each visit to the station, sensor performance (i.e., temperature, relative humidity, barometric pressure, and wind speed) is checked by comparing weather station values to those of a handheld Kestrel 4000 wind/weather instrument.  All sensors are inspected for damage or debris, and repaired and cleaned accordingly following the download of data.  The surfaces of the LiCor Quantum Sensor and Vaisala Temperature and Relative Humidity Sensor are cleaned at each visit using a paper towel moistened with deionized water.  With the exception of the rain gauge (which is calibrated in-house), all sensors are removed and sent to Campbell Scientific for re-calibration annually or bi-annually.

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- annual recalibration

- Rain Gauge- annual recalibration

- Wind Speed/Direction- annual or every 2 years inspection (depending on the sensor)

- Barometric Pressure- every 2 years recalibration

- PAR- every 2 years

- CR1000-every 5 years (required beginning 2014, one year initial grace period)

5)  Site location and character

The GTMNERR (North section [NW and SE corners]: 30.1632º N, 81.3447º W and 29.9698º N, 81.2488º W; South section: 29.8295º N, 81.3294º W and 29.6017º N, 81.1936º W), located in the Florida Upper East Coast Drainage Basin, includes over 24,281 ha of publicly owned forested uplands, tidal wetlands, estuarine lagoons and offshore seas.  Geographically separated by the greater St. Augustine area, the Reserve is associated with the riverine systems of the Tolomato and Guana River estuaries to the north and the Matanzas River estuary to the south.  The Tolomato River Basin is about 29 km in length with a drainage area of approximately 21,773 ha; it converges with the Matanzas River and Salt Run from the south before flowing into the Atlantic Ocean at the St. Augustine Inlet.  The headwaters of the Guana River originate in the Diego Plains drainage area in Ponte Vedra Beach.  This drainage basin encompasses approximately 3,157 ha.  The Guana River runs parallel to the Tolomato on the seaward side, with the two lagoons joining 11.3 km north of the St. Augustine Inlet.  The Matanzas River estuary is approximately 32 km in length and extends 13 km south of the Matanzas Inlet.  The Matanzas River sub-basin has a drainage area of approximately 41,931 ha and is bounded to the west by the Atlantic Coastal Ridge, which separates it from the lower St. Johns River basin.  Both the St. Augustine and Matanzas Inlets provide oceanic exchange to the system.  The Matanzas Inlet, one of the last “natural” inlets on Florida’s east coast, has been unimproved and is suitable only for small watercraft.  The natural hydrology of the Guana Tolomato Matanzas system has been somewhat altered by water control structures, including dikes, inland wells, drainage ditches and a dam across a portion of the Guana River.  In addition, the Intracoastal Waterway traverses both the Tolomato and Matanzas estuaries.

The climate of northeast Florida is classified as humid subtropical and is characteristic of the Gulf and Atlantic coastal plain of the southeastern United States.  The average annual rainfall is approximately 132 cm per year, with the wet season extending from June through September.  Seasonal variation in temperature within the Reserve follows that of rainfall with a summer period of high temperatures between June and September and a cooler period extending from December through March.  The annual mean air temperature within the Reserve is approximately 21C.

The weather station is located on a 1.8 square meter wooden deck built 1.3 meters above a sand flat in a high marsh region at the mouth of Pellicer Creek (29º 39′ 28.08 N, 81º 13′ 58 W). The height of the deck approximates the maximum height of the patchy mangrove vegetation immediately seaward of the station. With the exception of the rain gauge which is located away from the tower on the SE corner of the platform, all sensors are mounted on a 3 m aluminum tower secured to the wooden deck following the descriptions outlined in the NERR SWMP Campbell Scientific CR10X-2M Based Meteorological Monitoring Station Standard Operating Procedures V 1.0.

Sensor Heights in Meters (m) from Deck (from Ground)

- Temperature and Humidity: 2.0 m (3.3 m)

- LiCor PAR: 3.0 m (4.3 m) Dates: 09/25/2002 10:15 – 10/06/2010 11:00

- LiCor PAR: 2.8 m (4.1 m) Dates: 10/06/2010 15:15 – 12/31/2012 23:45

- Barometric Pressure: 1.0 m (2.3 m)

- Rain Gauge: 0.9 m (2.2 m)

- Wind Monitor: 3.1 m (4.4 m)

Located within the Pellicer Creek Aquatic Preserve (in the southern section of the GTMNERR), the area surrounding the Pellicer Creek weather station is considered fairly pristine.  The site is approximately 4 km SE of the Pellicer Creek water quality and nutrient monitoring station. Pellicer Creek is bordered for much of its length by publicly-owned conservation lands.

6)  Data collection period

The Campbell Scientific Weather Station has been operating at the Pellicer Creek (PC) station since September 25, 2002.  The 2012 data collection period began on January 1, 2012 00:00 and ended on December 31, 2012 23:45.

7)  Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

National Estuarine Research Reserve System (NERRS). 2012.  System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://cdmo.baruch.sc.edu/; accessed 12 October 2012.

NERR water quality data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in comma delimited format.

8)  Associated researchers and projects

The GTMNERR has formed partnerships with other agencies and organizations actively involved in resource protection in the GTMNERR watershed.  Cooperating managers of lands within the NERR include the:  1) National Park Service (NPS), 2) St. Johns River Water Management District (SJRWMD), 3) Department of Environmental Protection (DEP) Division of Recreation and Parks/Florida Park Service (FPS), 4) Florida Fish and Wildlife Conservation Commission (FWC), 5) Florida Division of Forestry (DOF), and 6) Flagler County.

The SJRWMD Northern Coastal Basin Program, established in 1995, covers the watersheds along the Intracoastal Waterway from Ponce Inlet in Volusia County, north through Flagler and St. Johns Counties, to the Guana River marshes at Ponte Vedra in southern Duval County. This program was developed in response to concerns about the impacts of population growth and development on water quality in the Northern Coastal Basin (NCB).  Some of the major research activities conducted within the boundaries of the GTMNERR through the NCB program include: 1) a fisheries monitoring projects conducted by reserve staff and outside agencies (e.g. United States Geological Survey (USGS); 2) hydrodynamic modeling of the estuarine systems; 3) emergent marsh vegetation and oyster habitat mapping; and 4) water quality monitoring.  Many of the water quality monitoring and assessment activities of the NCB program are aimed at pollution source identification and the development of pollution load reduction goals.

Other significant water quality research and monitoring initiatives within the GTMNERR include: 1) studies of nutrient profiles by the laboratory of Dr. Edward Phlips at the University of Florida [This work includes GTMNERR-contracted nutrient analyses (part of SWMP) as well as separately-funded studies], and 2) regular monitoring of fecal coliform levels in shellfish harvesting waters by the Department of Agriculture and Consumer Services.

Complementing the weather data, the GTMNERR also collects abiotic water quality and nutrient data within the Reserve as part of the System-wide Monitoring Program (SWMP).  All SWMP data is available for download through the CDMO.

For further information on research and monitoring activities within the GTMNERR, see the GTMNERR Site Profile at: http://nerrs.noaa.gov/Doc/PDF/Reserve/GTM_SiteProfile.pdf

II.  Physical Structure Descriptors

9)  Sensor specifications

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #: HMP45C Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Sensor 1 Serial Number: X1710109

Date of Calibration: 06/03/2002

Dates of Sensor Use: 09/25/2002 – 03/02/2004

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 05/21/2002

Dates of Sensor Use: 03/02/2004 – 07/27/2004

Sensor 1 Serial Number: X1710109

Date of Calibration: 05/24/2004

Dates of Sensor Use: 07/27/2004 – 06/17/2005

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 06/01/2005

Dates of Sensor Use: 06/17/2005 – 09/20/2006

Sensor 1 Serial Number: T0920010 (replaced SN X1710109)

Date of Last Calibration: 08/03/2006

Dates of Sensor Use: 09/20/2006 – 08/24/2007

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 05/21/2007

Dates of Sensor Use: 08/24/2007 – 09/10/2008

Sensor 1 Serial Number: T0920010

Date of Last Calibration: 06/04/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2009

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 09/15/2009

Dates of Sensor Use: 10/06/2009 – 10/06/2010

Sensor 1 Serial Number: T0920010

Date of Last Calibration: 08/31/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2011

Sensor 2 Serial Number: X2120031

Sensor 2 Date of Last Calibration: 09/23/2011

Sensor 2 Dates of Sensor Use: 10/31/2011 – 10/31/2012

Sensor Retired on 10/31/2012

Parameter: Temperature

Units: Celsius

Sensor type: PT100 RTD, IEC 751 1/3 Class B

Model #:  HC2-S3 Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.1 °C @ 23°C

Sensor 2 Serial Number: 0060837710 (Replaced SN X2120031)

Date of Calibration: 07/24/2012

Dates of Sensor Use: 10/31/2012 – 12/31/2012

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #: HMP45C Temperature and Relative Humidity Probe

Range: 0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Sensor 1 Serial Number: X1710109

Date of Calibration: 06/03/2002

Dates of Sensor Use: 09/25/2002 – 03/02/2004

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 05/21/2002

Dates of Sensor Use: 03/02/2004 – 07/27/2004

Sensor 1 Serial Number: X1710109

Date of Calibration: 05/24/2004

Dates of Sensor Use: 07/27/2004 – 06/17/2005

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 06/01/2005

Dates of Sensor Use: 06/17/2005 – 09/20/2006

Sensor 1 Serial Number: T0920010 (replaced SN X1710109)

Date of Last Calibration: 08/03/2006

Dates of Sensor Use: 09/20/2006 – 08/24/2007

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 05/21/2007

Dates of Sensor Use: 08/24/2007 – 09/10/2008

Sensor 1 Serial Number: T0920010

Date of Last Calibration: 06/04/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2009

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 09/15/2009

Dates of Sensor Use: 10/06/2009 – 10/06/2010

Sensor 1 Serial Number: T0920010

Date of Last Calibration: 08/31/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2011

Sensor 2 Serial Number: X2120031

Sensor 2 Date of Last Calibration: 09/23/2011

Sensor 2 Dates of Sensor Use: 10/31/2011 – 10/31/2012

Sensor 2 Retired on 10/31/2012

Parameter: Relative Humidity

Units: Percent

Sensor type: ROTRONIC® Hygromer IN-1

Model #: HC2-S3 Temperature and Relative Humidity Probe

Range: 0-100% non-condensing

Accuracy at 23°C:  +/- 0.8% RH with standard configuration settings

Sensor 2 Serial Number: 0060837710 (replaced SN X2120031)

Sensor 2 Date of Last Calibration: 07/24/2012

Sensor 2 Dates of Sensor Use: 10/31/2012 – 12/31/2012

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range: Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity: non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Sensor 1 Serial Number: X035001

Date of Calibration: 01/18/2002

Dates of Sensor Use: 09/25/2002 – 07/27/2004

Sensor 2 Serial Number: X0350015

Date of Last Calibration: 01/18/2002

Dates of Sensor Use: 07/27/2004 – 09/20/2006

Sensor 1 Serial Number: X035001

Date of Calibration: 07/11/2006

Dates of Sensor Use: 09/20/2006 – 09/10/2008

Sensor 2 Serial Number: X0350015

Date of Last Calibration: 06/20/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2010

Sensor 1 Serial Number: X035001

Date of Calibration: 09/07/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2012

Sensor 2 Serial Number: X0350015

Date of Last Calibration: 09/25/2012

Dates of Sensor Use: 10/31/2012 – 12/31/2012

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Sensor 1 Serial Number: WM51298

Date of Calibration: 06/06/2002

Dates of Sensor Use: 09/25/2002 – 07/27/2004

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 06/06/2002

Dates of Sensor Use: Back-up Sensor

Sensor 2 Serial Number: WM51295

Date of Calibration: 05/17/2004

Dates of Sensor Use: 07/27/2004 – 09/20/2006

Sensor 1 Serial Number: WM51298

Date of Last Calibration: 08/14/2006

Dates of Sensor Use: 09/20/2006 – 09/10/2008

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 05/05/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2010

Sensor 1 Serial Number: WM51298

Date of Last Calibration: 08/30/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2012

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 09/14/2012

Dates of Sensor Use: 10/31/2012 – 12/31/2012

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Model #: R.M. Young 05103 Wind Monitor

Range: 360° mechanical, 355° electrical (5° open)

Accuracy: +/- 3 degrees

Sensor 1 Serial Number: WM51298

Date of Calibration: 06/06/2002

Dates of Sensor Use: 09/25/2002 – 07/27/2004

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 06/06/2002

Dates of Sensor Use: Back-up Sensor

Sensor 2 Serial Number: WM51295

Date of Calibration: 05/17/2004

Dates of Sensor Use: 07/27/2004 – 09/20/2006

Sensor 1 Serial Number: WM51298

Date of Last Calibration: 08/14/2006

Dates of Sensor Use: 09/20/2006 – 09/10/2008

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 05/05/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2010

Sensor 1 Serial Number: WM51298

Date of Last Calibration: 08/30/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2012

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 09/14/2012

Dates of Sensor Use: 10/31/2012 – 12/31/2012

Parameter: Photosynthetically Active Radiation (PAR)

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SB

Light spectrum waveband: 400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability: <±2% change over 1 yr

Operating Temperature: -40°C to 65°C; Humidity: 0 to 100%

Sensitivity: typically 5 µA per 1000 µmoles s-1 m-2

Sensor 1 Serial Number: Q30811

Date of Calibration: 04/04/2002

Multiplier: 1.189389

Dates of Sensor Use: 09/25/2002 – 07/27/2004

Sensor 2 Serial Number: Q30812

Date of Last Calibration: 04/04/2002

Multiplier: 1.172542

Dates of Sensor Use: Back-up Sensor

Sensor 2 Serial Number: Q30812

Date of Calibration: 05/20/2004

Multiplier: 1.172542

Dates of Sensor Use: 07/27/2004 – 10/31/2006

Sensor 1 Serial Number: Q30811

Date of Last Calibration: 07/13/2006

Multiplier: 1.277492

Dates of Sensor Use: 10/31/2006 – 09/10/2008

Sensor 2 Serial Number: Q30812

Date of Last Calibration: 06/09/2008

Multiplier: 1.22821

Dates of Sensor Use: 09/10/2008 – 10/06/2010

Sensor 1 Serial Number: Q30811

Date of Last Calibration: 08/27/2010

Multiplier: 1.361537

Dates of Sensor Use: 10/06/2010 – 10/04/2012

Retired Sensor on 10/04/2012

Sensor 1 Serial Number: Q47769 (Replaced SN Q30811)

Date of Last Calibration: 05/16/2012

Multiplier: 0.851661

Dates of Sensor Use: 10/04/2012 – 12/31/2012

Parameter: Precipitation (non-heated rain gauge)

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #: TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Sensor 1 Serial Number: 30469-402

Sensor 1 Date of Last Calibration: 04/02/2002

Sensor 1 Date of Last Calibration: 03/02/2004

Sensor 1 Date of Last Calibration: 05/24/2005

Sensor 1 Date of Last Calibration: 09/18/2006

Sensor 1 Date of Last Calibration: 04/24/2007

Sensor 1 Date of Last Calibration: 08/12/2008

Sensor 1 Date of Last Calibration: 10/03/2012

Sensor 1 Dates of Use: 09/25/2002 – 10/06/2009

Sensor 1 Dates of Use: 10/04/2012 – 12/31/2012

Sensor 2 Serial Number: 30230-202

Sensor 2 Date of Last Calibration: 02/19/2002

Sensor 2 Date of Last Calibration: 10/05/2009

Sensor 2 Date of Last Calibration: 10/14/2010

Sensor 2 Date of Last Calibration: 08/30/2011

Sensor 2 Dates of Use: 10/06/2009 – 10/04/2012

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  The CR1000 was installed on June 19, 2006 at the GTMNERR Pellicer Creek Weather Station.

CR1000 Firmware Version (s):  Version(s) and dates changed

CR1000 Program Version (s):  GTMPCMET_5.5_110912

GTMPCMET_V5.5_100412

10) Coded variable definitions

Sampling station:			Sampling site code:		Station code:

Pellicer Creek Weather Station		PC			  gtmpcmet

11) QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by a “F_”).   During primary automated QAQC (performed by the CDMO), “-5,” “-4,” and “-2” flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged with a “0”, as passing initial QAQC checks.   During secondary and tertiary QAQC “1,” “-3,” and “5” flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12) QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below) can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure/Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QAQC Checks

GSM	See Metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC		Incorrect Calibration Constant, Multiplier or Offset

SIW	Incorrect Wiring

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor Drift

SSN	Not a Number/Unknown Value

SSM	Sensor Malfunction

SSR	Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CDF	Data Appear to Fit Conditions

CML	Snow melt from previous snowfall event

CRE*	Significant Rain Event

CSM*	See Metadata

CCU	Cause Unknown

CVT*	Possible Vandalism/Tampering

CWE*	Significant weather event

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Note 1: Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the LiCor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Note 2: Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Note 3: Precipitation values greater than 5mm in a 15 minute time period may occur during some heavy rain events, therefore, these data were not rejected.

Note 4: Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.

Note 5: There appeared to be a slight change in PAR values following the swap to a freshly calibrated sensor (assumed to be accurate) on 10/04/2012.  Unfortunately, the sensor (Q30811) was retired without being post calibrated.  Since % post cal drift for this sensor is unavailable, the data were flagged and coded as though they had exceeded the acceptable drift.   Acceptable drift is +/- 2% for this sensor.  All PAR data prior the sensor swap, from 1/01/2012 to 10/04/2012 are flagged and coded as <1> SSD CSM.  If users are comfortable assuming that drift was linear (in a real world environment it is unlikely to be entirely linear), these data may be ‘corrected’ for assumed linear drift at the user’s discretion using manufacturer’s instructions.

Note 6: PAR sensors on the GTM weather station used to show frequent strings of repeating values. It has been determined that this was caused by the programming of the CR1000 data logger.

old program command: VoltDiff(PAR,1,mV2500,3,True,0,_60Hz,0.897,0)

new program command: VoltDiff(PAR,1,mV25,3,True,0,_60Hz,0.897,0)

The program was updated and sent to the CR1000 on  10/04/2012. The mV25 setting allows measurements to be taken at a finer, more precise scale than mV2500, and has mostly eliminated repeating values from the data set. While values before this program change are not as precise as they could have been, they are not erroneous.  Repeating values are still occurring, but not as frequently.   These data are coded as CSM.

Note 7: Elevated nighttime PAR values occurred throughout the year, even after the sensor swap in 10/04/2012.  Elevated PAR data 0 to 2.214 were flagged as suspect <1> CSM, elevated nighttime values >2.214 were rejected <-3> SSM.  These elevated nighttime PAR values are thought to be a result of moisture intrusion inside the sensor.  Sensor appears to function properly when conditions are dry.  http://www.sunrisesunset.com was used to determine sunrise and sunset times when flagging and coding these data.

- See Metadata [GSM]/(CSM)

January 2012

- Suspect PAR data 01/10 20:15 and 01/10 20:45; attributed to elevated night-time values.  Value is within signal noise error range for sensor.

February 2012

- Suspect PAR data 02/08 00:00; attributed to elevated night-time values.  Value is within signal noise error range for sensor.

- Suspect PAR data 02/16 05:15 attributed to elevated night-time values.  Value is within signal noise error range for sensor.

- Suspect PAR data 02/18 02:45, 03:00, 04:15, 04:30, 04:45, 05:45, and 06:00; attributed to elevated night-time values.  Values are within signal noise error range for sensor.

- Suspect PAR data 02/19 02:00, 02:15, 05:15 and 05:30; attributed to elevated night-time values.  Values are within signal noise error range for sensor.

- Suspect PAR data 02/22 21:45; attributed to elevated night-time values.  Value is within signal noise error range for sensor.

April 2012

- Suspect PAR data 04/22 04:00; attributed to elevated night-time value.  Value is within signal noise error range for sensor.

- Suspect PAR data 04/26 03:30 through 04/26 05:30; attributed to elevated night-time values.  Values are within signal noise error range for sensor.

- Suspect PAR data 04/27 23:15, 23:30; attributed to elevated night-time values.  Values are within signal noise error range for sensor.

- Suspect PAR data 04/28 20:30, 22:45-23:15; attributed to elevated night-time values.  Values are within signal noise error range for sensor.

- Suspect PAR data 04/30 01:45, 04:00; attributed to elevated night-time values.  Values are within signal noise error range for sensor.

May 2012

- Suspect PAR data 05/22 00:45-01:45, 03:00; attributed to elevated night-time value.  Value is within signal noise error range for sensor.

- Suspect PAR data 053/16 20:30-20:45 attributed to elevated night-time values.  Value is within signal noise error range for sensor.

June 2012

- Suspect PAR data 06/09 16:45; attributed to negative PAR value.  Value is within signal noise error range for sensor.

August 2012

- Suspect PAR data 08/02 19:30; attributed to elevated night-time value.  Value is within signal noise error range for sensor.

- Suspect PAR data 08/02 19:45; attributed to negative PAR value.  Value is within signal noise error range for sensor.

- Reject Rainfall data 08/20/2012 08:45; attributed to an accidental bucket tip.   Rainfall daily totals 08/20 08:45 through 08/21 00:00 were edited to measure actual rainfall.

October 2012

- Missing data 10/04 09:15 through 10/04 11:00; attributed to a power down for station maintenance.  Installed new PAR sensor and rain bucket.  PAR sensor had to be mounted slightly lower (4.3m to 4.1m) due to differences between the old and new sensor platforms.  Rain bucket was calibrated in the laboratory and replaced previous rain bucket on the station.  Changed CR1000 program and uploaded it to the station.

- Reject all data 10/04 11:15; attributed to the sensor exchange and program upload.  Sensors were exchanged in the middle of the reading so the 15 minute average is incorrect.

- Missing data 10/31 10:00 through 10/31 10:30; attributed to a power down for station maintenance.  Installed newly calibrated Wind Monitor and Barometric Pressure Sensor.

- Reject all data 10/31 10:45; attributed to the sensor exchange.  Sensors were exchanged the middle of the reading so the 15 minute average is incorrect.

November 2012

- Missing data 11/09 10:15 through 11/09 10:45; attributed to a power down for station maintenance.  Installed new Temperature/Relative Humidity probe.   Changed CR1000 program and uploaded it to the station.

- Suspect PAR data 11/20 19:45 through 11/20 21:15; attributed to elevated night-time values.  Values are within signal noise error range for sensor.

- Suspect PAR data 11/27 05:30 through 11/27 06:00; attributed to elevated night-time values.  Values are within signal noise error range for sensor.

- Daily, Monthly, and Annual Precipitation Totals (from raw data)

Pellicer Creek Weather Station (gtmpcmet)

| Date | Daily Precip. Totals (mm) |
| --- | --- |
| 01/18/2012 | 4.572 |
| 01/19/2012 | 0.254 |
| 01/27/2012 | 0.254 |
| January Monthly Total | 5.080 |
| 02/06/2012 | 0.254 |
| 02/10/2012 | 19.304 |
| 02/11/2012 | 1.270 |
| 02/17/2012 | 0.254 |
| 02/18/2012 | 0.254 |
| 02/19/2012 | 0.762 |
| 02/22/2012 | 2.286 |
| 02/26/2012 | 4.572 |
| 02/27/2012 | 6.35 |
| February Monthly Total | 35.306 |
| 03/04/2012 | 9.652 |
| 03/09/2012 | 11.43 |
| 03/10/2012 | 1.524 |
| 03/11/2012 | 0.254 |
| 03/21/2012 | 0.254 |
| 03/23/2012 | 8.128 |
| 03/24/2012 | 1.778 |
| 03/31/2012 | 28.448 |
| March Monthly Total | 61.468 |
| 04/01/2012 | 0.254 |
| 04/05/2012 | 1.524 |
| 04/21/2012 | 1.524 |
| 04/22/2012 | 9.652 |
| April Monthly Total | 12.954 |
| 05/07/2012 | 1.016 |
| 05/09/2012 | 3.810 |
| 05/14/2012 | 0.762 |
| 05/15/2012 | 16.510 |
| 05/16/2012 | 7.874 |
| 05/17/2012 | 5.080 |
| 05/22/2012 | 4.064 |
| 05/27/2012 | 54.864 |
| 05/28/2012 | 29.718 |
| 05/29/2012 | 1.270 |
| 05/30/2012 | 8.636 |
| 05/31/2012 | 45.212 |
| May Monthly Total | 178.816 |
| 06/01/2012 | 2.286 |
| 06/05/2012 | 19.050 |
| 06/06/2012 | 0.254 |
| 06/07/2012 | 97.790 |
| 06/08/2012 | 11.176 |
| 06/09/2012 | 8.636 |
| 06/10/2012 | 5.588 |
| 06/11/2012 | 2.032 |
| 06/12/2012 | 4.826 |
| 06/14/2012 | 1.016 |
| 06/15/2012 | 2.794 |
| 06/21/2012 | 10.160 |
| 06/24/2012 | 59.690 |
| 06/25/2012 | 21.336 |
| 06/26/2012 | 42.926 |
| 06/27/2012 | 14.732 |
| June Monthly Total | 304.292 |
| 07/06/2012 | 5.334 |
| 07/17/2012 | 0.254 |
| 07/18/2012 | 1.270 |
| 07/19/2012 | 1.778 |
| 07/22/2012 | 1.778 |
| 07/24/2012 | 1.270 |
| 07/28/2012 | 0.508 |
| 07/30/2012 | 15.240 |
| 07/31/2012 | 5.842 |
| July Monthly Total | 33.274 |
| 08/02/2012 | 52.070 |
| 08/04/2012 | 19.050 |
| 08/05/2012 | 5.080 |
| 08/07/2012 | 6.858 |
| 08/08/2012 | 39.878 |
| 08/09/2012 | 3.048 |
| 08/10/2012 | 0.762 |
| 08/11/2012 | 8.636 |
| 08/12/2012 | 1.270 |
| 08/15/2012 | 11.430 |
| 08/17/2012 | 1.524 |
| 08/18/2012 | 10.922 |
| 08/19/2012 | 30.480 |
| 08/20/2012 | 16.764 |
| 08/21/2012 | 16.764 |
| 08/23/2012 | 1.270 |
| 08/26/2012 | 6.096 |
| 08/27/2012 | 32.512 |
| 08/28/2012 | 33.274 |
| 08/29/2012 | 9.398 |
| August Monthly Total | 307.086 |
| 09/07/2012 | 3.048 |
| 09/09/2012 | 7.620 |
| 09/12/2012 | 3.048 |
| 09/14/2012 | 1.016 |
| 09/18/2012 | 4.572 |
| 09/20/2012 | 30.734 |
| 09/21/2012 | 26.924 |
| 09/26/2012 | 10.922 |
| 09/30/2012 | 41.148 |
| September Monthly Total | 129.032 |
| 10/02/2012 | 0.508 |
| 10/03/2012 | 2.794 |
| 10/04/2012 | 3.556 |
| 10/05/2012 | 3.810 |
| 10/06/2012 | 3.302 |
| 10/08/2012 | 6.350 |
| 10/17/2012 | 0.508 |
| October Monthly Total | 20.828 |
| 11/11/2012 | 0.254 |
| 11/14/2012 | 7.620 |
| 11/17/2012 | 1.524 |
| November Monthly Total | 9.398 |
| 12/01/2012 | 3.048 |
| 12/02/2012 | 0.254 |
| 12/05/2012 | 3.302 |
| 12/06/2012 | 0.762 |
| 12/10/2012 | 38.608 |
| 12/11/2012 | 9.144 |
| 12/12/2012 | 6.350 |
| 12/15/2012 | 0.508 |
| 12/16/2012 | 0.254 |
| 12/20/2012 | 3.048 |
| 12/21/2012 | 0.508 |
| 12/26/2012 | 10.160 |
| 12/29/2012 | 16.510 |
| December Monthly Total | 92.456 |
| Annual Total | 1189.99 |