Guana Tolomato Matanzas (GTM) National Estuarine Research Reserve (NERR)

Meteorological Metadata
January – December 2013

Latest Update:  04/15/2014

Note: This is a provisional metadata document; it has not been authenticated as of its download date.  Contents of this document are subject to change throughout the QAQC process and it should not be considered a final record of data document until that process is complete.  Contact the CDMO (cdmosupport@belle.baruch.sc.edu) or the GTMNERR with any additional questions.

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons:

Principal Investigator:

Nikki Dix, Research Coordinator

505 Guana River Road

Ponte Vedra Beach, FL 32082

(904) 823-4500

nikki.dix@dep.state.fl.us

Other Contact Persons:

Matthew Welsh, SWMP Manager

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4061

matthew.welsh@dep.state.fl.us

Kathryn M. Petrinec, SWMP Data Specialist

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4054

kathryn.petrinec@dep.state.fl.us

2)  Entry verification:

The meteorological information were collected and stored on a Campbell Scientific CR1000  datalogger.  The Centralized Data Management Office (CDMO) Data Logger Program (GTMPCMET_V5.5_110912.CR1 or GTMPCMET_V5.5_061413.CR1) controls the sensors, performs all data manipulations, and interfaces the CR1000 with the PConnect or LoggerNet software supplied by Campbell Scientific.

Once a month data are downloaded from the CR1000 using a laptop or tablet personal computer (historically a Palm Pilot m105).  All files are exported from Pconnect (Palm Pilot) or Loggernet (laptop) and the data are saved as raw data files in a comma-delimited format with a gtmpcmetmmddyy.dat naming convention that represents the site name, weather station, and the first day data were recorded into the file (e.g., gtmpcmet011213.dat represents a data file with its first row of data obtained on January 12, 2013).  These data files are uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC MS Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

Flagged and/or anomalous data are evaluated.  Data outside the “normal” range of the instruments are investigated for validity based on field observations, QC checks, graphs, and instrument diagnostics.  Data are rejected if the anomalies are attributed to sensor malfunction.  Any additional information on flagged data is noted in Section 13 Other Remarks/See Metadata (CSM).  Common errors observed are relative humidity greater than 100% and precipitation difference of greater than 5 mm in 15 minutes.  Both raw data files and QAQC files are archived on the CDMO FTP server.  Katie Petrinec and Matthew Welsh are responsible for these tasks.

3)  Research objectives (Campbell Weather Station)

The principal objective is to record and ultimately integrate meteorological data with water quality data in order to better understand the variability, trends, and relationships of these parameters within the GTM NERR.

4)  Research methods

Campbell Scientific data telemetry equipment was installed at the Pellicer Creek (PC) station on 06/19/2006 and transmits data to the NOAA GOES satellite, NESDIS ID #3B01B60E.    (Where #3B01B60E is the GOES ID for that particular station.)  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts)

Maximum, Minimum, and their times from 5-second data:

(These data are not available in the dataset, but are available from GTMNERR)

Maximum and Minimum Air Temperature (°C) and their times

Maximum Wind Speed (m/s) and time

Wind Direction Standard Deviation (degrees)

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm)

The CR1000 internally stores up to 2 MB of programs and data.  A maximum four-week sampling interval (i.e., time between data downloads) was used to avoid losing data in the CR1000 and to provide for regular inspection and maintenance of the station.

At each visit to the station, sensor performance (i.e., temperature, relative humidity, barometric pressure, and wind speed) is checked by comparing weather station values to those of a handheld Kestrel 4000 wind/weather instrument.  All sensors are inspected for damage or debris, and repaired and cleaned accordingly following the download of data.  The surfaces of the LiCor Quantum Sensor and Vaisala Temperature and Relative Humidity Sensor are cleaned at each visit using a paper towel moistened with deionized water.  With the exception of the rain gauge (which is calibrated in-house), all sensors are removed and sent to Campbell Scientific for re-calibration annually or bi-annually.

5)  Site location and character

The GTMNERR (North section [NW and SE corners]: 30.1632º N, 81.3447º W and 29.9698º N, 81.2488º W; South section: 29.8295º N, 81.3294º W and 29.6017º N, 81.1936º W), located in the Florida Upper East Coast Drainage Basin, includes over 24,281 ha of publicly owned forested uplands, tidal wetlands, estuarine lagoons and offshore seas.  Geographically separated by the greater St. Augustine area, the Reserve is associated with the riverine systems of the Tolomato and Guana River estuaries to the north and the Matanzas River estuary to the south.  The Tolomato River Basin is about 29 km in length with a drainage area of approximately 21,773 ha; it converges with the Matanzas River and Salt Run from the south before flowing into the Atlantic Ocean at the St. Augustine Inlet.  The headwaters of the Guana River originate in the Diego Plains drainage area in Ponte Vedra Beach.  This drainage basin encompasses approximately 3,157 ha.  The Guana River runs parallel to the Tolomato on the seaward side, with the two lagoons joining 11.3 km north of the St. Augustine Inlet.  The Matanzas River estuary is approximately 32 km in length and extends 13 km south of the Matanzas Inlet.  The Matanzas River sub-basin has a drainage area of approximately 41,931 ha and is bounded to the west by the Atlantic Coastal Ridge, which separates it from the lower St. Johns River basin.  Both the St. Augustine and Matanzas Inlets provide oceanic exchange to the system.  The Matanzas Inlet, one of the last “natural” inlets on Florida’s east coast, has been unimproved and is suitable only for small watercraft.  The natural hydrology of the Guana Tolomato Matanzas system has been somewhat altered by water control structures, including dikes, inland wells, drainage ditches and a dam across a portion of the Guana River.  In addition, the Intracoastal Waterway traverses both the Tolomato and Matanzas estuaries.

The climate of northeast Florida is classified as humid subtropical and is characteristic of the Gulf and Atlantic coastal plain of the southeastern United States.  The average annual rainfall is approximately 132 cm per year, with the wet season extending from June through September.  Seasonal variation in temperature within the Reserve follows that of rainfall with a summer period of high temperatures between June and September and a cooler period extending from December through March.  The annual mean air temperature within the Reserve is approximately 21C.

The weather station is located on a 1.8 square meter wooden deck built 1.3 meters above a sand flat in a high marsh region at the mouth of Pellicer Creek (29º 39′ 28.08 N, 81º 13′ 58 W). The height of the deck approximates the maximum height of the patchy mangrove vegetation immediately seaward of the station. With the exception of the rain gauge which is located away from the tower on the SE corner of the platform, all sensors are mounted on a 3 m aluminum tower secured to the wooden deck following the descriptions outlined in the NERR SWMP Campbell Scientific CR10X-2M Based Meteorological Monitoring Station Standard Operating Procedures V 1.0.

Sensor Heights in Meters (m) from Deck (from Ground)

- Temperature and Humidity: 2.0 m (3.3 m)

- LiCor PAR: 3.0 m (4.3 m) Dates: 09/25/2002 10:15 – 10/06/2010 11:00

- LiCor PAR: 2.8 m (4.1 m) Dates: 10/06/2010 15:15 – present

- Barometric Pressure: 1.0 m (2.3 m)

- Rain Gauge: 0.9 m (2.2 m)

- Wind Monitor: 3.1 m (4.4 m)

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- annual recalibration

- Rain Gauge- annual recalibration

- Wind Speed/Direction- annual or bi-annual inspection (depending on the sensor)

- Barometric Pressure- bi-annual recalibration

- PAR- bi-annual recalibration

- CR1000-every 5 years (required beginning 2014, one year initial grace period)

Located within the Pellicer Creek Aquatic Preserve (in the southern section of the GTM NERR), the area surrounding the Pellicer Creek weather station is considered fairly pristine.  The site is approximately 4 km SE of the Pellicer Creek water quality and nutrient monitoring station. Pellicer Creek is bordered for much of its length by publicly-owned conservation lands.

6)  Data collection period

The Campbell Scientific Weather Station has been operating at the Pellicer Creek (PC) station since September 25, 2002.  The 2012 data collection period began on January 1, 2013 00:00 and ended on December 31, 2013 23:45.

7)  Distribution

According to the Ocean and Coastal Resource Management Data Dissemination Policy for the NERRS System-Wide Monitoring Program, NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The Principal Investigator (PI) retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.

The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR weather data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in comma separated format.

8)  Associated researchers and projects

The GTMNERR has formed partnerships with other agencies and organizations actively involved in resource protection in the GTMNERR watershed.  Cooperating managers of lands within the NERR include the:  1) National Park Service (NPS), 2) St. Johns River Water Management District (SJRWMD), 3) Department of Environmental Protection (DEP) Division of Recreation and Parks/Florida Park Service (FPS), 4) Florida Fish and Wildlife Conservation Commission (FWC), 5) Florida Division of Forestry (DOF), and 6) Flagler County.

The SJRWMD Northern Coastal Basin Program, established in 1995, covers the watersheds along the Intracoastal Waterway from Ponce Inlet in Volusia County, north through Flagler and St. Johns Counties, to the Guana River marshes at Ponte Vedra in southern Duval County. This program was developed in response to concerns about the impacts of population growth and development on water quality in the Northern Coastal Basin (NCB).  Some of the major research activities conducted within the boundaries of the GTMNERR through the NCB program include: 1) a fisheries monitoring projects conducted by reserve staff and outside agencies (e.g. United States Geological Survey (USGS); 2) hydrodynamic modeling of the estuarine systems; 3) emergent marsh vegetation and oyster habitat mapping; and 4) water quality monitoring.  Many of the water quality monitoring and assessment activities of the NCB program are aimed at pollution source identification and the development of pollution load reduction goals.

Other significant water quality research and monitoring initiatives within the GTM NERR include: 1) studies of nutrient profiles by the laboratory of Dr. Edward Phlips at the University of Florida [This work includes GTM NERR-contracted nutrient analyses (part of SWMP) as well as separately-funded studies], and 2) regular monitoring of fecal coliform levels in shellfish harvesting waters by the Department of Agriculture and Consumer Services.

Complementing the weather data, the GTM NERR also collects abiotic water quality and nutrient data within the Reserve as part of the System-wide Monitoring Program (SWMP).  All SWMP data is available for download through the CDMO.

For further information on research and monitoring activities within the GTM NERR, see the GTMNERR Site Profile at: http://nerrs.noaa.gov/Doc/PDF/Reserve/GTM_SiteProfile.pdf

II.  Physical Structure Descriptors

9)  Sensor specifications

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #: HMP45C Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Sensor 1 Serial Number: X1710109

Date of Calibration: 06/03/2002

Dates of Sensor Use: 09/25/2002 – 03/02/2004

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 05/21/2002

Dates of Sensor Use: 03/02/2004 – 07/27/2004

Sensor 1 Serial Number: X1710109

Date of Calibration: 05/24/2004

Dates of Sensor Use: 07/27/2004 – 06/17/2005

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 06/01/2005

Dates of Sensor Use: 06/17/2005 – 09/20/2006

Sensor 1 Serial Number: T0920010 (replaced SN X1710109)

Date of Last Calibration: 08/03/2006

Dates of Sensor Use: 09/20/2006 – 08/24/2007

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 05/21/2007

Dates of Sensor Use: 08/24/2007 – 09/10/2008

Sensor 1 Serial Number: T0920010

Date of Last Calibration: 06/04/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2009

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 09/15/2009

Dates of Sensor Use: 10/06/2009 – 10/06/2010

Sensor 1 Serial Number: T0920010

Date of Last Calibration: 08/31/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2011

Sensor Retired on 10/31/2011

Sensor 2 Serial Number: X2120031

Sensor 2 Date of Last Calibration: 09/23/2011

Sensor 2 Dates of Sensor Use: 10/31/2011 – 10/31/2012

Sensor Retired on 10/31/2012

Parameter: Temperature

Units: Celsius

Sensor type: PT100 RTD, IEC 751 1/3 Class B

Model #:  HC2-S3 Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.1 °C @ 23°C

Sensor 2 Serial Number: 0060837710 (Replaced SN X2120031)

Date of Calibration: 07/24/2012

Dates of Sensor Use: 10/31/2012 – 10/22/2013

Sensor 1 Serial Number: 006105380 (Replaced SN T0920010)

Date of Calibration: 03/14/2013

Dates of Sensor Use: 10/22/2013 - 12/31/2013

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #: HMP45C Temperature and Relative Humidity Probe

Range: 0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Sensor 1 Serial Number: X1710109

Date of Calibration: 06/03/2002

Dates of Sensor Use: 09/25/2002 – 03/02/2004

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 05/21/2002

Dates of Sensor Use: 03/02/2004 – 07/27/2004

Sensor 1 Serial Number: X1710109

Date of Calibration: 05/24/2004

Dates of Sensor Use: 07/27/2004 – 06/17/2005

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 06/01/2005

Dates of Sensor Use: 06/17/2005 – 09/20/2006

Sensor 1 Serial Number: T0920010 (replaced SN X1710109)

Date of Last Calibration: 08/03/2006

Dates of Sensor Use: 09/20/2006 – 08/24/2007

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 05/21/2007

Dates of Sensor Use: 08/24/2007 – 09/10/2008

Sensor 1 Serial Number: T0920010

Date of Last Calibration: 06/04/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2009

Sensor 2 Serial Number: X2120031

Date of Last Calibration: 09/15/2009

Dates of Sensor Use: 10/06/2009 – 10/06/2010

Sensor 1 Serial Number: T0920010

Date of Last Calibration: 08/31/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2011

Sensor 2 Serial Number: X2120031

Sensor 2 Date of Last Calibration: 09/23/2011

Sensor 2 Dates of Sensor Use: 10/31/2011 – 10/31/2012

Sensor 2 Retired on 10/31/2012

Parameter: Relative Humidity

Units: Percent

Sensor type: ROTRONIC® Hygromer IN-1

Model #: HC2-S3 Temperature and Relative Humidity Probe

Range: 0-100% non-condensing

Accuracy at 23°C:  +/- 0.8% RH with standard configuration settings

Sensor 2 Serial Number: 0060837710 (replaced SN X2120031)

Sensor 2 Date of Last Calibration: 07/24/2012

Sensor 2 Dates of Sensor Use: 10/31/2012 – 06/30/2013

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range: Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity: non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Sensor 1 Serial Number: X035001

Date of Calibration: 01/18/2002

Dates of Sensor Use: 09/25/2002 – 07/27/2004

Sensor 2 Serial Number: X0350015

Date of Last Calibration: 01/18/2002

Dates of Sensor Use: 07/27/2004 – 09/20/2006

Sensor 1 Serial Number: X035001

Date of Calibration: 07/11/2006

Dates of Sensor Use: 09/20/2006 – 09/10/2008

Sensor 2 Serial Number: X0350015

Date of Last Calibration: 06/20/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2010

Sensor 1 Serial Number: X035001

Date of Calibration: 09/07/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2012

Sensor 2 Serial Number: X0350015

Date of Last Calibration: 09/25/2012

Dates of Sensor Use: 10/31/2012 – 12/31/2013

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Sensor 1 Serial Number: WM51298

Date of Calibration: 06/06/2002

Dates of Sensor Use: 09/25/2002 – 07/27/2004

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 06/06/2002

Dates of Sensor Use: Back-up Sensor

Sensor 2 Serial Number: WM51295

Date of Calibration: 05/17/2004

Dates of Sensor Use: 07/27/2004 – 09/20/2006

Sensor 1 Serial Number: WM51298

Date of Last Calibration: 08/14/2006

Dates of Sensor Use: 09/20/2006 – 09/10/2008

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 05/05/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2010

Sensor 1 Serial Number: WM51298

Date of Last Calibration: 08/30/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2012

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 09/14/2012

Dates of Sensor Use: 10/31/2012 – 06/25/2013

Sensor 3 Serial Number: WM126355

Date of Last Calibration: 5/21/2013

Dates of Sensor Use: 06/25/2013 – 12/31/2013

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Model #: R.M. Young 05103 Wind Monitor

Range: 360° mechanical, 355° electrical (5° open)

Accuracy: +/- 3 degrees

Sensor 1 Serial Number: WM51298

Date of Calibration: 06/06/2002

Dates of Sensor Use: 09/25/2002 – 07/27/2004

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 06/06/2002

Dates of Sensor Use: Back-up Sensor

Sensor 2 Serial Number: WM51295

Date of Calibration: 05/17/2004

Dates of Sensor Use: 07/27/2004 – 09/20/2006

Sensor 1 Serial Number: WM51298

Date of Last Calibration: 08/14/2006

Dates of Sensor Use: 09/20/2006 – 09/10/2008

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 05/05/2008

Dates of Sensor Use: 09/10/2008 – 10/06/2010

Sensor 1 Serial Number: WM51298

Date of Last Calibration: 08/30/2010

Dates of Sensor Use: 10/06/2010 – 10/31/2012

Sensor 2 Serial Number: WM51295

Date of Last Calibration: 09/14/2012

Dates of Sensor Use: 10/31/2012 – 06/25/2013

Sensor 3 Serial Number: WM126355

Date of Last Calibration: 5/21/2013

Dates of Sensor Use: 06/25/2013 – 12/31/2013

Parameter: Photosynthetically Active Radiation (PAR)

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SB

Light spectrum waveband: 400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability: <±2% change over 1 yr

Operating Temperature: -40°C to 65°C; Humidity: 0 to 100%

Sensitivity: typically 5 µA per 1000 µmoles s-1 m-2

Sensor 1 Serial Number: Q30811

Date of Calibration: 04/04/2002

Multiplier: 1.189389

Dates of Sensor Use: 09/25/2002 – 07/27/2004

Sensor 2 Serial Number: Q30812

Date of Last Calibration: 04/04/2002

Multiplier: 1.172542

Dates of Sensor Use: Back-up Sensor

Sensor 2 Serial Number: Q30812

Date of Calibration: 05/20/2004

Multiplier: 1.172542

Dates of Sensor Use: 07/27/2004 – 10/31/2006

Sensor 1 Serial Number: Q30811

Date of Last Calibration: 07/13/2006

Multiplier: 1.277492

Dates of Sensor Use: 10/31/2006 – 09/10/2008

Sensor 2 Serial Number: Q30812

Date of Last Calibration: 06/09/2008

Multiplier: 1.22821

Dates of Sensor Use: 09/10/2008 – 10/06/2010

Sensor 1 Serial Number: Q30811

Date of Last Calibration: 08/27/2010

Multiplier: 1.361537

Dates of Sensor Use: 10/06/2010 – 10/04/2012

Retired Sensor on 10/04/2012

Sensor 1 Serial Number: Q47769 (Replaced SN Q30811)

Date of Last Calibration: 05/16/2012

Multiplier: 0.851661

Dates of Sensor Use: 10/04/2012 – 12/31/2013

Parameter: Precipitation (non-heated rain gauge)

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #: TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Sensor 1 Serial Number: 30469-402

Sensor 1 Date of Last Calibration: 04/02/2002

Sensor 1 Date of Last Calibration: 03/02/2004

Sensor 1 Date of Last Calibration: 05/24/2005

Sensor 1 Date of Last Calibration: 09/18/2006

Sensor 1 Date of Last Calibration: 04/24/2007

Sensor 1 Date of Last Calibration: 08/12/2008

Sensor 1 Date of Last Calibration: 10/03/2012

Sensor 1 Dates of Use: 09/25/2002 – 10/06/2009

Sensor 1 Dates of Use: 10/04/2012 – 10/22/2013

Sensor 2 Serial Number: 30230-202

Sensor 2 Date of Last Calibration: 02/19/2002

Sensor 2 Date of Last Calibration: 10/05/2009

Sensor 2 Date of Last Calibration: 10/14/2010

Sensor 2 Date of Last Calibration: 08/30/2011

Sensor 2 Dates of Use: 10/06/2009 – 10/04/2012

Sensor 3 Serial Number: 55024

Sensor 3 Date of Last Calibration: 02/11/2013

Sensor 3 Dates of Use: 10/22/2013 - 12/31/2013

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  The CR1000 was installed on June 19, 2006 at the GTMNERR Pellicer Creek Weather Station.

10) Coded variable definitions

Sampling station:			Sampling site code:		Station code:

Pellicer Creek Weather Station		PC			  gtmpcmet

11) QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		 Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC		Incorrect Calibration Constant, Multiplier or Offset

SIW	Incorrect Wiring

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor Drift

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR		Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CDF	Data Appear to Fit Conditions

CML	Snow melt from previous snowfall event

CRE*	Significant Rain Event

CSM*	See Metadata

CCU	Cause Unknown

CVT*	Possible Vandalism/Tampering

CWE*	Significant weather event

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Note 1: Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the LiCor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Note 2: Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Note 3: Precipitation values greater than 5mm in a 15 minute time period may occur during some heavy rain events, therefore, these data were not rejected.

Note 4: Cumulative precipitation data are recorded from 00:00 to 23:59 with the daily total recorded at the midnight mark (00:00).  The midnight CumPrcp value is actually the total from the previous day.

- See Metadata [GSM]/(CSM)

June 1 – 30, 2013

  - The weather station was powered down on 06/25/2013 10:05 through 06/25/2013 10:55 for installation of new Wind Monitor and new program upload.  The PAR mV values were adjusted in the CR1000 program in an attempt to correct the elevated night-time PAR values.  The new program is named GTMPCMET_V5.5_061413.  Data collected between 06/25/2013 10:15 through 06/25/2013 10:45 are missing due to the power down.  All parameters collected at 11:00 were rejected because 15 minute data were incomplete.

July 1-31, 2013

a. 	The CR1000 program was adjusted back to previous version.  Newer 	program did not fix the elevated night-time PAR values.  The CR1000 	program is now named GTMPCMET_V5.5_110912.

September 1-30, 2013

a. 	Reject precipitation data 09/19/2013 10:45; attributed to an accidental

bucket tip.  Corrected Cumulative Precipitation data to reflect change.

October 1-31, 2013

- The weather station was powered down on 10/22/2013 10:06 through 10/22/2013 10:57 for installation of new Precipitation Gauge and Temperature/Relative Humidity sensors.  Data collected between 10:15 through 10:45 are missing due to the power down.  All parameters collected at 11:00 were rejected because 15 minute data were incomplete.  Tipped bucket inside Precipitation Gauge after installation.  Corrected Cumulative Precipitation data to reflect change.

- Daily, Monthly, and Annual Precipitation Totals (from raw data)

Pellicer Creek Weather Station (gtmpcmet)

| Date | Daily Precip. Totals (mm) |
| --- | --- |
| 01/03/2013 | 1.016 |
| 01/04/2013 | 0.254 |
| 01/06/2013 | 0.254 |
| 01/08/2013 | 0.254 |
| 01/09/2013 | 0.762 |
| 01/10/2013 | 0.254 |
| 01/31/2013 | 1.016 |
| January Monthly Total | 3.810 |
| 02/06/2013 | 2.032 |
| 02/07/2013 | 6.096 |
| 02/13/2013 | 3.048 |
| 02/14/2013 | 4.572 |
| 02/25/2013 | 11.176 |
| 02/26/2013 | 11.430 |
| February Monthly Total | 38.354 |
| 03/12/2013 | 0.508 |
| 03/17/2013 | 1.524 |
| 03/18/2013 | 0.508 |
| 03/20/2013 | 7.874 |
| 03/21/2013 | 0.254 |
| 03/23/2013 | 37.592 |
| 03/24/2013 | 0.508 |
| March Monthly Total | 48.768 |
| 04/01/2013 | 1.270 |
| 04/03/2013 | 13.970 |
| 04/04/2013 | 0.508 |
| 04/05/2013 | 3.048 |
| 04/14/2013 | 53.848 |
| 04/19/2013 | 10.414 |
| 04/20/2013 | 4.572 |
| 04/21/2013 | 23.876 |
| 04/29/2013 | 7.112 |
| 04/30/2013 | 7.366 |
| April Monthly Total | 118.872 |
| 05/02/2013 | 83.312 |
| 05/03/2013 | 122.682 |
| 05/04/2013 | 50.038 |
| 05/05/2013 | 0.254 |
| 05/11/2013 | 4.064 |
| 05/18/2013 | 5.588 |
| 05/19/2013 | 11.684 |
| 05/20/2013 | 1.016 |
| 05/22/2013 | 2.794 |
| May Monthly Total | 281.432 |
| 06/01/2013 | 12.446 |
| 06/03/2013 | 22.352 |
| 06/04/2013 | 10.414 |
| 06/05/2013 | 2.540 |
| 06/06/2013 | 40.640 |
| 06/07/2013 | 0.254 |
| 06/10/2013 | 7.366 |
| 06/14/2013 | 13.208 |
| 06/17/2013 | 3.048 |
| 06/19/2013 | 0.762 |
| 06/20/2013 | 0.508 |
| 06/21/2013 | 7.620 |
| 06/22/2013 | 5.080 |
| 06/27/2013 | 8.636 |
| 06/28/2013 | 6.350 |
| 06/30/2013 | 10.160 |
| June Monthly Total | 151.384 |
| 07/01/2013 | 0.762 |
| 07/02/2013 | 57.150 |
| 07/11/2013 | 16.510 |
| 07/12/2013 | 57.404 |
| 07/13/3013 | 23.622 |
| 07/15/2013 | 0.762 |
| 07/19/2013 | 40.132 |
| 07/20/2013 | 5.588 |
| 07/21/2013 | 17.272 |
| 07/22/2013 | 5.842 |
| 07/23/2013 | 0.762 |
| 07/27/2013 | 2.540 |
| 07/28/2013 | 30.734 |
| 07/31/2013 | 29.718 |
| July Monthly Total | 288.798 |
| 08/01/2013 | 1.778 |
| 08/04/2013 | 6.350 |
| 08/09/2013 | 4.826 |
| 08/14/2013 | 0.762 |
| 08/15/2013 | 36.322 |
| 08/16/2013 | 13.462 |
| 08/17/2013 | 15.494 |
| 08/19/2013 | 1.524 |
| 08/21/2013 | 23.144 |
| 08/22/2013 | 1.270 |
| 08/23/2013 | 2.032 |
| 08/24/2013 | 11.938 |
| 08/25/2013 | 10.160 |
| 08/26/2013 | 2.032 |
| August Monthly Total | 131.064 |
| 09/01/2013 | 12.954 |
| 09/02/2013 | 13.970 |
| 09/03/2013 | 2.540 |
| 09/06/2013 | 0.254 |
| 09/07/2013 | 7.112 |
| 09/10/2013 | 1.016 |
| 09/11/2013 | 19.304 |
| 09/12/2013 | 0.762 |
| 09/16/2012 | 4.318 |
| 09/17/2013 | 0.254 |
| 09/18/2013 | 20.828 |
| 09/22/2013 | 0.508 |
| 09/23/2013 | 5.080 |
| 09/24/2013 | 5.588 |
| 09/25/2013 | 1.524 |
| 09/27/2013 | 1.270 |
| 09/28/2013 | 55.623 |
| 09/29/2013 | 1.016 |
| 09/30/2013 | 1.524 |
| September Monthly Total | 155.488 |
| 10/01/2013 | 2.540 |
| 10/06/2013 | 5.080 |
| 10/07/2013 | 32.258 |
| 10/08/2013 | 1.778 |
| 10/09/2013 | 1.016 |
| 10/14/2013 | 6.604 |
| 10/15/2013 | 7.112 |
| 10/19/2013 | 28.956 |
| 10/20/2013 | 0.254 |
| 10/21/2013 | 12.954 |
| 10/22/2013 | 1.016 |
| 10/31/2013 | 1.016 |
| October Monthly Total | 100.838 |
| 11/02/2013 | 9.652 |
| 11/05/2013 | 10.922 |
| 11/06/2013 | 0.254 |
| 11/15/2013 | 4.064 |
| 11/16/2013 | 0.762 |
| 11/20/2013 | 1.270 |
| 11/26/2013 | 3.556 |
| 11/27/2013 | 6.858 |
| November Monthly Total | 37.338 |
| 12/10/2013 | 1.016 |
| 12/14/2013 | 34.798 |
| 12/15/2013 | 48.541 |
| 12/24/2013 | 0.254 |
| 12/27/2013 | 0.254 |
| 12/28/2013 | 2.032 |
| 12/29/2013 | 3.556 |
| December Monthly Total | 90.451 |
| January – December 2013 Total | 1446.597 |