Guana Tolomato Matanzas (GTM) National Estuarine Research Reserve
Nutrient Metadata Report

January – December 2008

Latest Update: November 5, 2013

I.  Data Set and Research Descriptors

- Principal investigator(s) and contact persons

  - Principal Investigator:

Richard A. Gleeson, Ph.D., Research Coordinator

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4054

rglee@whitney.ufl.edu

rick.gleeson@dep.state.fl.us

  - SWMP Technician:

Kathryn M. Petrinec, B.S., Research Assistant

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4054

kathryn.petrinec@dep.state.fl.us

  - Laboratory Contact:

Edward J. Phlips, Professor

Nikki Dix, QA/QC Officer

7922 N. W. 71st Street

Dept. Fisheries and Aquatic Sciences

University of Florida

Gainesville, FL 32653

(352) 392-9617 Ext. 248

phlips@ufl.edu

- Research Objectives

The System-wide Monitoring Program (SWMP) water quality initiative began within the GTMNERR in 2001.  There are presently four permanent stations at which YSI 6600 EDS datasondes have been deployed for continuously monitoring a suite of selected abiotic parameters at 15 min intervals.  The positions of these stations allow for comparisons between relatively pristine versus more urbanized drainage basins as well as higher versus lower salinity regions of the estuary (see “Station Descriptions” under “Site Location and Character”). Nutrient analyses are performed on water samples collected monthly at each of these sites.  The objective of this effort is to quantify the spatial/temporal variability and trends, both seasonally and as a function of tidal forcing, of selected nutrient parameters within the Reserve and to explore how these are related to concurrently generated sonde and meteorological data.

  - Monthly grab – The principal objective of the monthly grab sampling is to determine spatial and temporal differences in water quality in four ecologically distinct regions of the GTMNERR.

  - Diel sampling program – The principal objective of the diel sampling is to determine the impact of tidal water exchange on water quality.

- Research methods

- Monthly grab sampling program

Monthly grab samples were collected at four stations within the GTMNERR. These samples were taken at the four principal datasonde stations (Pine Island, San Sebastian, Fort Matanzas and Pellicer Creek). All grab samples were obtained on the same day between +3 hrs before slack low-water and slack low-water.  No distinction was made between neap and spring tide conditions.  Efforts were made to allow for an antecedent dry period of 72 hours prior to sampling.  Replicate (N=2) samples were collected using a pole sampler lowered to the approximate depth of the datasonde probes (i.e., 1 meter above the bottom) in closed position and then opened to draw water from the specified depth.  All water samples were collected in nalgene sample bottles that were previously acid washed (19%), rinsed (3x) with distilled-deionized water, dried and followed by rinsing with ambient water prior to collection of the sample. Samples were filtered in the field whenever feasible or placed on ice in the dark and filtered immediately upon returning to the laboratory. Once in the laboratory, samples were shaken and processed for nutrient, Chla and solids analyses.  Turbidity and color were also measured.  At the time of sample collection, water temperature, salinity, dissolved oxygen concentration and pH were measured with Hydrolab Quanta or YSI Model 556 environmental multi-probes calibrated on the day of measurement.  Secchi depth was also recorded.  In addition, PAR (photosynthetically active radiation) light flux measurements were made at the surface and 0.5 m, 1.0 m and 1.5 m depths with LiCor 2 π cosine-corrected quantum light probes. Attenuation of downwelling irradiance was determined from the surface and 1.0 m readings.

- Diel sampling program

Diel sampling was carried out at the Pellicer Creek site.  All water samples were collected automatically at 2.5 hour intervals for a 25 hour (lunar day) period using an ISCO Sampler.  The water was collected from the approximate depth of the datasonde probes (i.e., 1 meter above the bottom).  Diel sampling was generally performed during the same time periods as the monthly grab samples.  The samples were collected into ISCO bottles that were previously acid washed (19%), rinsed (3x) with distilled-deionized water and dried prior to sample collection.  The ISCO unit is equipped with an internal ice bath to keep samples cold.  At the end of the 25-hour period, sample bottles were removed from the ISCO and processed in the same manner as described for the grab samples.  Salinity (SALT_N) associated with these samples was measured in the laboratory using a refractometer.  Turbidity and color were also measured from the samples in the laboratory.

- Site location and character

The GTMNERR (North section [NW and SE corners]: 30.1632º N, 81.3447º W and 29.9698º N, 81.2488º W; South section: 29.8295º N, 81.3294º W and 29.6017º N, 81.1936º W), located in the Florida Upper East Coast Drainage Basin, includes over 60,000 acres (24,281 hectares) of publicly owned forested uplands, tidal wetlands, estuarine lagoons and offshore seas.  Geographically separated by the greater St. Augustine area, the Reserve is associated with the riverine systems of the Tolomato and Guana River estuaries to the north and the Matanzas River estuary to the south.  The Tolomato River Basin is about 18 miles (29 km) in length with a drainage area of approximately 53,802 acres (21,773 hectares); it converges with the Matanzas River and Salt Run from the south before flowing into the Atlantic Ocean at the St. Augustine Inlet.  The headwaters of the Guana River originate in the Diego Plains drainage area in Ponte Vedra Beach.  This drainage basin encompasses approximately 7,800 acres (3,157 hectares).  The Guana River runs parallel to the Tolomato on the seaward side, with the two lagoons joining 7 miles (11.3 km) north of the St. Augustine Inlet.  The Matanzas River estuary is approximately 20 miles (32 km) in length and extends 8 miles (13 km) south of the Matanzas Inlet.  The Matanzas River sub-basin has a drainage area of approximately 103,615 acres (41,931 hectares) and is bounded to the west by the Atlantic Coastal Ridge, which separates it from the lower St. Johns River basin.  Both the St. Augustine and Matanzas Inlets provide oceanic exchange to the system.  The Matanzas Inlet, one of the last “natural” inlets on Florida’s east coast, has been unimproved and is suitable only for small watercraft.  The natural hydrology of the Guana Tolomato Matanzas system has been somewhat altered by water control structures, including dikes, inland wells, drainage ditches and a dam across a portion of the Guana River.  In addition, the Intracoastal Waterway traverses both the Tolomato and Matanzas estuaries.  There are no known significant pollutants within the Reserve.

The climate of northeast Florida is classified as humid subtropical and is characteristic of the Gulf and Atlantic coastal plain of the Southeastern United States.  The average annual rainfall is approximately 52 inches (132 cm) per year, with the wet season extending from June through September.  Seasonal variation in temperature within the Reserve follows that of rainfall with a summer period of high temperatures between June and September and a cooler period extending from December through March.  The annual mean air temperature within the Reserve is approximately 21C.

Station descriptions: The Pine Island (gtmpinut) station is at Channel Marker 25 (30 03.051´N; 81 22.048´W) in the Tolomato River.  This site is located within the Guana River Marsh Aquatic Preserve in the northern section of the GTMNERR.  Channel Marker 25 is adjacent to Pine Island near the mouth of Deep Creek, which provides freshwater drainage from silviculture-dominated uplands in the northwestern portion of the Tolomato River Basin.  The average depth at this site is approximately 3.8 m with a tidal range of about 1.6 m; the bottom type is muddy sand. Salinity ranged from 1.7 to 39.7 ppt during 2008.  The Fort Matanzas (gtmfmnut) site (in the southern section of the GTMNERR) is located at Channel Marker 75 (29 44.222´N; 81 14.757´W) in the Matanzas River.  This site is approximately 4 km north of the Matanzas Inlet and near a shoreline on Anastasia Island that is undergoing residential development.  The average depth at this site is approximately 3.6 m with a tidal range of about 1.4 m; the bottom type is muddy sand.  The salinity ranged from 19.7 to 38.3 ppt during 2008.  Both the FM and PI stations are situated within Class II Shellfish Harvesting Waters.  The San Sebastian (gtmssnut) station is at Channel Marker 1 (29 52.131´N; 81 18.446´W) located at the confluence of the San Sebastian and Matanzas Rivers, approximately 4 km south of the St. Augustine Inlet.  The San Sebastian River drains an urbanized watershed in the western portion of St. Augustine; it is characterized by both residential and commercial development including five marinas.  The average depth at this site is approximately 4.4 m with a tidal range of about 1.7 m; the bottom type is muddy sand.  The salinity range during 2008 was from 20.1 to 37.9 ppt.  The Pellicer Creek (gtmpcnut) station (29 40.024´N; 81 15.444´W) is at the end of a recreational dock in Faver Dykes State Park located within the Pellicer Creek Aquatic Preserve in the southern section of the GTM Reserve.  Pellicer Creek is tidal and bordered for much of its length by publicly-owned conservation lands.  The average depth at this site is approximately 2.3 m with a tidal range of about 0.6 m; the bottom type is muddy sand.  The salinity ranged from 0.1 to 36.0 ppt during 2008.

- Coded variable definitions

| Station Codes |  |  |
| --- | --- | --- |
| gtmfmnut | GTM Reserve nutrient data for Fort Matanzas |  |
| gtmssnut | GTM Reserve nutrient data for San Sebastian |  |
| gtmpinut | GTM Reserve nutrient data for Pine Island |  |
| gtmpcnut | GTM Reserve nutrient data for Pellicer Creek |  |
| Monitoring program codes |  |  |
| 1 = Monthly grab sample program |  | 1 |
| 2 = Diel grab sample program |  | 2 |

- Data collection period

The GTMNERR began collecting monthly grab samples at all four water quality sites in May 2002.  Diel sampling began at the Pellicer Creek site in July 2002.

Diel Data Collection Period:  January – December 2008

| Site | Start Date | Start Time | End Date | End Time |
| --- | --- | --- | --- | --- |
| gtmpcnut | 01/07/08 | 10:30 | 01/08/08 | 11:30 |
| gtmpcnut | 02/04/08 | 09:35 | 02/05/08 | 10:35 |
| gtmpcnut | 03/09/08 | 12:15 | 03/10/08 | 13:15 |
| gtmpcnut | 04/20/08 | 10:30 | 04/21/08 | 11:30 |
| gtmpcnut | 05/19/08 | 11:00 | 05/20/08 | 12:00 |
| gtmpcnut | 06/18/08 | 10:30 | 06/19/08 | 11:30 |
| gtmpcnut | 07/01/08 | 09:07 | 07/02/08 | 10:07 |
| gtmpcnut | 08/25/08 | 09:05 | 08/26/08 | 10:05 |
| gtmpcnut | 09/17/08 | 13:05 | 09/18/08 | 14:05 |
| gtmpcnut | 10/13/08 | 10:15 | 10/14/08 | 11:15 |
| gtmpcnut | 11/10/08 | 09:50 | 11/11/08 | 10:50 |
| gtmpcnut | 12/15/08 | 10:56 | 12/16/08 | 11:56 |

Replicate grab samples were collected at one minute intervals at the times listed below.

Grab Sample Data Collection Period: January – December 2008

| Site | Start Date | Start Time | End Date | End Time |
| --- | --- | --- | --- | --- |
| gtmfmnut | 01/08/08 | 10:56 | 01/08/08 | 10:57 |
| gtmfmnut | 02/05/08 | 11:41 | 02/05/08 | 11:42 |
| gtmfmnut | 03/10/08 | 14:01 | 03/10/08 | 14:02 |
| gtmfmnut | 04/21/08 | 11:33 | 04/21/08 | 11:34 |
| gtmfmnut | 05/20/08 | 12:21 | 05/20/08 | 12:22 |
| gtmfmnut | 06/19/08 | 11:20 | 06/19/08 | 11:21 |
| gtmfmnut | 07/02/08 | 10:41 | 07/02/08 | 10:42 |
| gtmfmnut | 08/18/08 | 11:29 | 08/18/08 | 11:30 |
| gtmfmnut | 09/18/08 | 13:43 | 09/18/08 | 13:44 |
| gtmfmnut | 10/14/08 | 10:48 | 10/14/08 | 10:49 |
| gtmfmnut | 11/11/08 | 10:06 | 11/11/08 | 10:07 |
| gtmfmnut | 12/16/08 | 14:01 | 12/16/08 | 14:02 |
| gtmpcnut | 01/08/08 | 11:51 | 01/08/08 | 11:52 |
| gtmpcnut | 02/05/08 | 12:46 | 02/05/08 | 12:47 |
| gtmpcnut | 03/10/08 | 15:06 | 03/10/08 | 15:07 |
| gtmpcnut | 04/21/08 | 12:37 | 04/21/08 | 12:38 |
| gtmpcnut | 05/20/08 | 14:00 | 05/20/08 | 14:01 |
| gtmpcnut | 6/19/08 | 12:25 | 6/19/08 | 12:26 |
| gtmpcnut | 07/02/08 | 12:06 | 07/02/08 | 12:07 |
| gtmpcnut | 08/18/08 | 10:11 | 08/18/08 | 10:12 |
| gtmpcnut | 09/18/08 | 14:55 | 09/18/08 | 14:56 |
| gtmpcnut | 10/14/08 | 11:34 | 10/14/08 | 11:35 |
| gtmpcnut | 11/11/08 | 11:21 | 11/11/08 | 11:22 |
| gtmpcnut | 12/16/08 | 15:19 | 12/16/08 | 15:20 |
| gtmpinut | 01/08/08 | 09:41 | 01/08/08 | 09:42 |
| gtmpinut | 02/05/08 | 09:51 | 02/05/08 | 09:52 |
| gtmpinut | 03/10/08 | 12:38 | 03/10/08 | 12:39 |
| gtmpinut | 04/21/08 | 10:21 | 04/21/08 | 10:22 |
| gtmpinut | 05/20/08 | 10:28 | 05/20/08 | 10:29 |
| gtmpinut | 06/19/08 | 09:56 | 06/19/08 | 09:57 |
| gtmpinut | 07/02/08 | 08:56 | 07/02/08 | 08:57 |
| gtmpinut | 08/18/08 | 09:41 | 08/18/08 | 09:42 |
| gtmpinut | 09/18/08 | 12:06 | 09/18/08 | 12:07 |
| gtmpinut | 10/14/08 | 09:18 | 10/14/08 | 09:19 |
| gtmpinut | 11/11/08 | 08:21 | 11/11/08 | 08:22 |
| gtmpinut | 12/16/08 | 12:52 | 12/16/08 | 12:53 |
| gtmssnut | 01/08/08 | 08:51 | 01/08/08 | 08:52 |
| gtmssnut | 02/05/08 | 08:51 | 02/05/08 | 08:52 |
| gtmssnut | 03/10/08 | 11:16 | 03/10/08 | 11:17 |
| gtmssnut | 04/21/08 | 09:31 | 04/21/08 | 09:32 |
| gtmssnut | 05/20/08 | 09:13 | 05/20/08 | 09:14 |
| gtmssnut | 06/19/08 | 08:56 | 06/19/08 | 08:57 |
| gtmssnut | 07/02/08 | 07:51 | 07/02/08 | 07:52 |
| gtmssnut | 08/18/08 | 10:43 | 08/18/08 | 10:44 |
| gtmssnut | 09/18/08 | 11:11 | 09/18/08 | 11:12 |
| gtmssnut | 10/14/08 | 08:18 | 10/14/08 | 08:19 |
| gtmssnut | 11/11/08 | 07:20 | 11/11/08 | 07:21 |
| gtmssnut | 12/16/08 | 12:01 | 12/16/08 | 12:02 |

- Associated researchers and projects

Phytoplankton analyses - Aliquots of the monthly grab samples collected for nutrient analyses at the four principal GTMNERR datasonde stations were separately preserved for subsequent plankton analyses.  Phytoplankton was examined using the Utermohl settling method (Utermohl 1958) and an inverted microscope.  Representative digital images of dominant phytoplankton were recorded for each sampling site on a monthly basis and archived to permit subsequent evaluations of blooms for potentially harmful species.  These data are archived at the GTMNERR.

Complementary research - The GTMNERR nutrient monitoring program is part of a broader long-term effort by Dr. Phlips to define the structure and function of coastal marine ecosystems in Florida, including their watersheds.  A central goal of this work is to provide the information needed to help assess the integrity and sustainability of Florida’s coastal marine resources.  Within the lagoonal ecosystems of the east coast of Florida that include the GTMNERR, Dr. Phlips has several ongoing research projects that complement the monitoring program, including:  (1) In 1997 a study of the plankton ecology of the Indian River Lagoon was initiated and continues through the current year.  This project, funded by the St. Johns River Water Management District, includes the description of phytoplankton community structure and physical-chemical characteristics of the water column.  In 1999 aspects of the study (primarily phytoplankton community structure) were extended to include parts of the intra-coastal waterway from Titusville to St. Augustine.  (2) In another complementary study, Dr. Phlips is engaged in a long-term study of the impact of eutrophication on the ecology of the St. Johns River and estuary.  The focus of this work, funded by the St. Johns River Water Management District, is water quality and the plankton community.  The St. Johns study was initiated in 1993 and continues to this date.  (3) Nikki Dix, a graduate student of Dr. Phlips, conducted her master’s research on the effects of the 2004 hurricanes on the salinity regime of Pellicer Creek.  She also received a NERR fellowship to examine nutrient gradients throughout the GTM estuary and relate them to benthic community characteristics.

Complementary monitoring - As part of the System Wide Monitoring Program (SWMP), the GTMNERR collects water quality (i.e., nutrients and water parameters using YSI 6600 EDS datasondes) and weather data.  All SWMP data is available for download through the Centralized Data Management Office (CDMO), http://cdmo.baruch.sc.edu/.

- Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format.

II. Physical Structure Descriptors

- Entry verification

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; adds chosen parameters and facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and then automatically flags/codes measured values below MDL and inserts the MDL ; calculates parameters chosen by the user and automatically flags/codes for component values below MDL, negative calculated values, and missing data; allows the user to apply QAQC flags and codes to the data; produces summary statistics; graphs selected parameters for review; and exports the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

Data entry, verification and chain of custody procedures follow those specified in the Florida Department of Health NELAP QA/QC certification plan.  Data entered into excel spreadsheets by chemical technicians will be verified by the QA/QC Officer (Nikki Dix) and certified by the principal investigator of the laboratory (Edward J. Phlips).  The GTMNERR staff (Richard Gleeson and Kathryn Petrinec) also reviewed the data entries prior to submission to the CDMO.

- Parameter titles and variable names by category

Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisks “*”.

| Data Category | Parameter | Variable Name | Units of Measure |
| --- | --- | --- | --- |
| Phosphorus |  |  |  |
|  | Total Phosphorus | TP | mg/L as P |
|  | Total Dissolved Phosphorus | TDP | mg/L as P |
|  | Particulate Phosphorus | PHOSP | mg/L as P |
|  | *Orthophosphate, Filtered | PO4F | mg/L as P |
| Nitrogen |  |  |  |
|  | Total Nitrogen | TN | mg/L as N |
|  | Total Dissolved Nitrogen | TDN | mg/L as N |
|  | Particulate Nitrogen | PN | mg/L as N |
|  | *Nitrite + Nitrate, Filtered | NO23F | mg/L as N |
|  | *Nitrite, Filtered | NO2F | mg/L as N |
|  | *Nitrate, Filtered | NO3F | mg/L as N |
|  | *Ammonium, Filtered | NH4F | mg/L as N |
|  | Dissolved Inorganic Nitrogen | DIN | mg/L as N |
| Plant Pigments |  |  |  |
|  | Uncorrected Chlorophyll a | UncCHLa_N | µg/L |
|  | *Chlorophyll a | CHLA_N | µg/L |
|  | Pheophytin a | PHEA | µg/L |
| Carbon |  |  |  |
|  | Particulate Organic Carbon | POC | mg/L as C |
| Other Lab Parameters |  |  |  |
|  | Silicate, Filtered | SiO4F | mg/L as SI |
|  | Total Suspended Solids | TSS | mg/L |
|  | Turbidity | TURB_N | NTU |
|  | Water Color | COLOR | PCU |
| Field Parameters |  |  |  |
|  | Salinity | SALT_N | ppt |
|  | Secchi disk depth | SECCHI | m |
|  | Dissolved Oxygen | DO_N | mg/L |
|  | Water Temperature | WTEM_N | degrees Celsius |
|  | pH | PH_N | standard units |
|  | Incident Irradiance | IRR0_N | µmole photons/m2/s |
|  | Irradiance at 1 meter | IRR1_N | µmole photons/m2/s |
|  | Attenuation of Downwelling Irradiance | Kd_N | m-1 |

Notes:

  - Time is coded based on a 2400 clock and is referenced to Standard Time.

  - Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

- Measured or calculated laboratory parameters

  - Parameters measured directly

Nitrogen species:		NH4F, NO2F, NO23F, TN, TDN,

Phosphorus species:		PO4F, TP, TDP

Other:				SiO4F, CHLA_N, PHEA, UncCHLa_N, COLOR, 
                                                              TURB_N,  TSS, POC, IRR0_N, IRR1_N, SALT_N*

*SALT_N is measured in the lab for diel samples, but in the field for grab samples

  - Calculated parameters

NO3				NO23-NO2

DIN				NO23+NH4
	PN				TN-TDN

PHOSP				TP-TDP

Kd_N				1n IRR0_N-1n IRR1_N

- Limits of detection

Method Detection Limits (MDL) are derived from the replicate samples method in APHA (American Public Health Association). 1998.  Standard Methods for the Examination of Water and Wastewater, 20th Edition.  United Book Press, Inc.  Baltimore, Maryland.  MDL will change with the background levels of samples; therefore, there is no constant MDL.

| Parameter | Start Date | End Date | MDL |
| --- | --- | --- | --- |
| CHLA_N | 01/01/2008 | 12/31/2008 | 0.2 |
| NH4F | 01/01/2008 | 12/31/2008 | 0.0117 |
| NO23F | 01/01/2008 | 12/31/2008 | 0.0062 |
| NO2F | 01/01/2008 | 12/31/2008 | 0.0043 |
| PHEA | 01/01/2008 | 12/31/2008 | 0.2 |
| PO4F | 01/01/2008 | 12/31/2008 | 0.002 |
| POC | 01/01/2008 | 12/31/2008 | 0.01 |
| SiO4F | 01/01/2008 | 12/31/2008 | 0.307 |
| TDN | 01/01/2008 | 12/31/2008 | 0.0185 |
| TDP | 01/01/2008 | 12/31/2008 | 0.010 |
| TN | 01/01/2008 | 12/31/2008 | 0.0185 |
| TP | 01/01/2008 | 12/31/2008 | 0.004 |
| TSS | 01/01/2008 | 12/31/2008 | 0.1 |
| UncCHLa_N | 01/01/2008 | 12/31/2008 | 0.2 |

- Laboratory methods

- Parameter:  Ammonium

Method Reference: Strickland & Parsons. 1972. A Practical Handbook of Seawater Analysis: Determination of Ammonia (Oxidation Method). Fisheries Research Board of Canada.

APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM4500-N-I. United Book Press, Inc. Baltimore, Maryland.

As of April 21, 2008, in accordance with the April 7th NELAC inspection and plan of correction, Edward J. Phlips’ Laboratory revised the standard operating procedure for ammonium (NH4; SOP # 3.1). Because the Strickland and Parsons (1972) method used in the SOP converts all NH4 to nitrite (NO2), it was determined necessary to correct for the original NO2 concentration present in each sample. NO2 determined separately (SM 4500 NO3-F) is now subtracted from the sum of NH4 + NO2 present in the original sample.

Method Descriptor: Photometric determination of ammonia in seawater based on the oxidation reaction with hypochlorite in an alkaline medium. Results are read on a Bran-Luebbe autoanalyzer without the cadmium column.  Final ammonium concentrations are corrected for the original nitrite concentrations in the sample.

Preservation Method: Samples are filtered through 0.7 µm pore size glass-fiber filters in the field, stored at 4oC and run within 48 hours.

- Parameter:  Nitrite and Nitrate

Method Reference:  APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM4500-NO3-F.  United Book Press, Inc.  Baltimore, Maryland.

Method Descriptor:  A water sample is passed though a cadmium column where the nitrate is reduced to nitrite, which is then diazotized with sulfanilamide and coupled with N-(1-naphthyl)-ethylenediamine to form a colored azo dye that is measured colorometrically on a Bran-Luebbe autoanalyzer.  The procedure is the same for nitrite analysis less the cadmium column.

Preservation Method:  Samples are filtered through 0.7 µm pore size glass-fiber filters in the field, stored at 4oC and run within 48 hours.

- Parameter:  Total Nitrogen

Method Reference:  APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM4500-N C.  United Book Press, Inc., Baltimore, Maryland.

Method Descriptor:  Potassium persulfate in DI H2O is added to sample which is then autoclaved for 30 minutes at 15 psi and cooled to room temperature.  The digested sample is passed though a cadmium column where the nitrate is reduced to nitrite which is then diazotized with sulfanilamide and coupled with N-(1-naphthyl)-ethylenediamine to form a colored azo dye that is measured colorometrically on a Bran-Luebbe autoanalyzer.

Preservation Method:  Samples are stored at -20oC and run within 28 days.

- Parameter:  Total Dissolved Nitrogen

Method Reference:  APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM4500-N C.  United Book Press, Inc.,Baltimore, Maryland.

Method Descriptor:  Potassium persulfate in DI H2O is added to sample which is then autoclaved for 30 minutes at 15 psi and cooled to room temperature.  The digested sample is passed though a cadmium column where the nitrate is reduced to nitrite which is then diazotized with sulfanilamide and coupled with N-(1-naphthyl)-ethylenediamine to form a colored azo dye that is measured colorometrically on a Bran-Luebbe autoanalyzer.

Preservation Method:  Samples are filtered through 0.7 µm pore size glass-fiber filters in the field, stored at -20oC and run within 28 days.

- Parameter:  Orthophosphate

Method Reference: APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition.  Method SM 4500-P-E (Ascorbic acid method). United Book Press, Inc., Baltimore, Maryland.

Method Descriptor:  Ammonium molybdate and potassium antimony in acid medium react with orthophosphate to form an acid that is reduced to a bright blue by ascorbic acid. Concentrations are measured on a dual-beam scanning spectrophotometer at 882 nm. The curve is read within 30 minutes.

Preservation Method: Samples are filtered through 0.7 µm pore size glass-fiber filters in the field and stored at 4oC.  Samples are run in less than 48 hours.

- Parameter:  Total Phosphorus

Method Reference:  APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM 4500-P E+B5 (Ascorbic acid method with persulfate digestion). United Book Press, Inc., Baltimore, Maryland.

Method Descriptor:  Potassium persulfate in DI H2O is added to sample which is then autoclaved for 30 minutes at 15 psi and cooled to room temperature. Ammonium molybdate and potassium antimony in acid medium are added to sample which reacts with orthophosphate to form an acid that is reduced to a bright blue by ascorbic acid. Concentrations are measured on a dual-beam scanning spectrophotometer at 882 nm. The curve is read within 30 minutes.

Preservation Method: Samples are stored at 4oC and run within 28 days.

- Parameter:  Total Dissolved Phosphorus

Method Reference: APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM 4500-P-E+B5 (Ascorbic acid method with persulfate digestion). United Book Press, Inc., Baltimore, Maryland.

Method Descriptor:  Potassium persulfate in DI H2O is added to sample which is then autoclaved for 30 minutes at 15 psi and cooled to room temperature. Ammonium molybdate and potassium antimony in acid medium are added to sample which reacts with orthophosphate to form an acid that is reduced to a bright blue by ascorbic acid. Concentrations are measured on a dual-beam scanning spectrophotometer at 882 nm. The curve is read within 30 minutes.

Preservation Method: Samples are filtered through 0.7 µm pore size glass-fiber filters in the field, and stored at 4oC and run within 28 days.

- Parameter:  Silicate

Method Reference: APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM 4500-SiO2 C. United Book Press, Inc., Baltimore, Maryland.

Method Descriptor:  HCl followed by ammonium molybdate are added to the sample which is then shaken.  To correct for turbidity and tannin, a background sample of the water is processed similarly, but in the absence of ammonium molybdate.  After 10 minutes, absorbance is measured on a dual-beam scanning spectrophotometer at 820 nm. The results are then subtracted to determine the net amount of silica.

Preservation Method: Samples are stored at 4oC and run within 28 days.

- Parameter:  Chlorophyll a, Uncorrected Chlorophyll a,  Pheophytin a

Method References:  APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM 10200 H.2. United Book Press, Inc., Baltimore, Maryland.

Extraction method for chlorophyll from Sartory, D. P. & Grobbelaar, J. U.  1984.  Hydrobiologia 114, 177-187.

Method Descriptor:  Filters are thawed, placed in test tubes with 90% ethanol and heated in a water bath at 78oC for 5 minutes.  They are subsequently placed in the dark for 24 hours followed by centrifugation to remove particulate material.  Absorbances are read on a dual-beam scanning spectrophotometer according to Standard Methods.  After the initial reading, 0.2N HCl is added to the sample and re-run for pheophytin a determination.  Chlorophyll a (CHLA_N) was determined by correcting chlorophyll for pheophytin content using the method described in Standard Methods.  Uncorrected Chlorophyll a (UncCHLa_N) represents the chlorophyll a concentration, without correction for pheophytin, using a simplified equation based on the extinction coefficient for chlorophyll a in ethanol solvent.

Preservation Method:  Samples are filtered onto 0.7 µm pore size glass-fiber filters in the field, stored in plastic bags in the dark at –20oC, and run within 28 days.

- Parameter:  Color

Method Reference:  APHA (American Public Health Association). 1998.

Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM 2120B H.2. United Book Press, Inc.  Baltimore, Maryland.

Method Descriptor: The absorbance of the filtrate is determined at 465 nm using a dual-beam scanning spectrophotometer.  Color values are measured against a platinum-cobalt standard.

Preservation Method: Samples are filtered through PALL A/E glass-fiber filters (0.7 µm pore size) in the field, stored at 4oC and run within 48 hours.

- Parameter:  Turbidity

Method Reference:  APHA (American Public Health Association). 1998. Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM 2130-B. United Book Press, Inc., Baltimore, Maryland.

Method Descriptor:  NTU values of whole water samples are measured directly in a LaMotte Model 2020 Turbidimeter.

Preservation Method:  Whole water samples are stored at 4oC and run within 48 hours.

- Parameter:  Total Suspended Solids

Method Reference:  APHA (American Public Health Association). 1998.

Standard Methods for the Examination of Water and Wastewater, 20th Edition. Method SM 2540-D. United Book Press, Inc.  Baltimore, Maryland.

Method Descriptor:  Aliqouts of whole water are filtered onto pre-washed and pre-weighed glass-fiber filters (0.7 µm pore size).  The filters are subsequently dried for 24 hours at 104oC, placed in a desiccator until cool and re-weighed.

Preservation Method: Filtration is performed within seven days of collection on whole water samples stored at 4oC.

- Parameter:  Particulate Organic Carbon

Method Descriptor:  Filters are dried at 60oC.  Organic carbon concentration is determined using a coulometer against a dextrose standard.

Preservation Method:  Aliqouts of whole water are filtered onto pre-burned (to eliminate any residual organic carbon) 0.7 µm pore size glass-fiber filters upon returning from the field and kept at –20oC before drying.  Filters are analyzed within 28 days of collection.

- Field and Laboratory QAQC programs

In the field each month, two successive grab samples are collected at each site.  One field blank is included in each monthly collection.

The laboratory QA/QC procedures are as prescribed in the laboratory’s Quality Manual.  The Quality System is approved by the Florida Department of Health according to National Environmental Laboratory Accreditation Conference (NELAC) guidelines and is subject to semiannual proficiency testing with blind samples, as well as site inspections every two years.

When standards are available from another source (and NIST traceable if possible), calibration curves are verified with check standards at least every 20 samples.  To determine the accuracy, each result is divided by the true value and converted to percent recovery.  Method blanks and matrix spikes are also run during each analysis (at least every 20 samples) and percent recovery is calculated.  Each percent recovery result must be within 80-120%.

Sample duplicates are run for each accredited analysis at least every 20 samples.  To determine precision, the relative percent difference (RPD) is calculated (as the difference between duplicate samples, divided by their average, times 100) and the RPD must be ≤ 20%.  If the difference between the sample and the duplicate is below the MDL, the duplicate is acceptable.

Ongoing personnel competency is demonstrated through the QC data generated with each analytical batch and the results of annual audits.  In addition, the analysts each participate in the required semi-annual proficiency testing for analyses they usually perform.  Each analyst that performs each test demonstrates his/her competency through either: 1) acceptable analysis of a blind sample 2) four lab control samples with acceptable precision and accuracy or 3) comparable results for the same sample with another analyst if no blind sample or control samples are available.

- QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

- QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data suspect due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Calculated value could not be determined due to a below MDL component

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N		from the north

NNE	from the north northeast

NE		from the northeast

ENE	from the east northeast

E		from the east

ESE	from the east southeast

SE		from the southeast

SSE		from the south southeast

S		from the south

SSW	from the south southwest

SW		from the southwest

WSW	from the west southwest

W		from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

- Other remarks/notes

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

  - See Metadata (CSM)

Suspect instrument measures for IRR0, IRR1, Kd_N for the dates and locations listed below.

| Site Name | Date/Time |
| --- | --- |
| gtmfmnut | 10/14/2008 10:48 |
| gtmfmnut | 10/14/2008 10:49 |
| gtmssnut | 10/14/2008 08:18 |
| gtmssnut | 10/14/2008 08:19 |
| gtmpinut | 10/14/2008 09:18 |
| gtmpinut | 10/14/2008 09:19 |
| gtmpcnut | 10/14/2008 11:34 |
| gtmpcnut | 10/14/2008 11:35 |

  - Daily, Monthly, and Annual Precipitation Totals

| Date | Daily Precip. Totals (mm) |
| --- | --- |
| 01/08/2008 | 0.762 |
| 01/12/2008 | 0.762 |
| 01/13/2008 | 2.540 |
| 01/14/2008 | 0.254 |
| 01/16/2008 | 4.572 |
| 01/17/2008 | 1.524 |
| 01/19/2008 | 19.558 |
| 01/20/2008 | 0.254 |
| 01/23/2008 | 4.572 |
| 01/26/2008 | 2.540 |
| 01/27/2008 | 2.286 |
| Monthly Total (mm) | 39.624 |
| 02/07/2008 | 1.524 |
| 02/08/2008 | 0.762 |
| 02/12/2008 | 20.320 |
| 02/13/2008 | 0.508 |
| 02/18/2008 | 18.288 |
| 02/21/2008 | 4.318 |
| 02/22/2008 | 0.508 |
| 02/23/2008 | 27.432 |
| 02/26/2008 | 5.334 |
| Monthly Total (mm) | 78.994 |
| 03/04/2008 | 4.318 |
| 03/06/2008 | 2.032 |
| 03/07/2008 | 51.562 |
| 03/08/2008 | 3.302 |
| 03/14/2008 | 1.270 |
| 03/20/2008 | 8.890 |
| 03/31/2008 | 2.286 |
| Monthly Total (mm) | 73.660 |
| 04/03/2008 | 2.286 |
| 04/04/2008 | 1.524 |
| 04/05/2008 | 16.256 |
| 04/06/2008 | 18.034 |
| 04/07/2008 | 0.508 |
| 04/08/2008 | 0.254 |
| 04/09/2008 | 0.508 |
| Monthly Total (mm) | 39.370 |
| 05/18/2008 | 1.524 |
| 05/23/2008 | 15.24 |
| 05/24/2008 | 0.254 |
| 05/27/2008 | 1.524 |
| Monthly Total (mm) | 18.542 |
| 06/11/2008 | 8.382 |
| 06/12/2008 | 0.254 |
| 06/13/2008 | 2.794 |
| 06/15/2008 | 26.416 |
| 06/16/2008 | 7.62 |
| 06/17/2008 | 16.51 |
| 06/18/2008 | 40.132 |
| 06/19/2008 | 11.938 |
| 06/21/2008 | 6.858 |
| 06/23/2008 | 5.08 |
| 06/24/2008 | 0.508 |
| 06/25/2008 | 2.286 |
| Monthly Total (mm) | 128.778 |
| 7/1/2008 | 4.826 |
| 7/8/2008 | 1.27 |
| 7/11/2008 | 1.016 |
| 7/12/2008 | 2.794 |
| 7/14/2008 | 12.954 |
| 7/17/2008 | 18.542 |
| 7/18/2008 | 2.032 |
| 7/19/2008 | 0.508 |
| 7/23/2008 | 12.192 |
| 7/24/2008 | 2.286 |
| 7/25/2008 | 8.382 |
| 7/26/2008 | 23.876 |
| 7/28/2008 | 11.684 |
| 7/29/2008 | 0.254 |
| 7/30/2008 | 5.842 |
| 7/31/2008 | 2.286 |
| Monthly Total (mm) | 110.744 |
| 8/1/2008 | 0.762 |
| 8/3/2008 | 0.254 |
| 8/8/2008 | 2.54 |
| 8/12/2008 | 10.668 |
| 8/13/2008 | 7.112 |
| 8/14/2008 | 3.048 |
| 8/15/2008 | 2.54 |
| 8/17/2008 | 0.254 |
| 8/18/2008 | 0.254 |
| 8/19/2008 | 0.254 |
| 8/20/2008 | 44.45 |
| 8/21/2008 | 33.782 |
| 8/22/2008 | 99.314 |
| 8/23/2008 | 4.826 |
| 8/24/2008 | 10.414 |
| 8/25/2008 | 5.588 |
| 8/28/2008 | 0.762 |
| 8/30/2008 | 3.556 |
| 8/31/2008 | 3.048 |
| Monthly Total (mm) | 233.426 |
| 9/2/2008 | 2.540 |
| 9/3/2008 | 1.778 |
| 9/5/2008 | 12.446 |
| 9/17/2008 | 7.112 |
| 9/18/2008 | 7.366 |
| 9/19/2008 | 15.494 |
| 9/20/2008 | 62.992 |
| 9/21/2008 | 1.016 |
| 9/22/2008 | 10.414 |
| Monthly Total (mm) | 121.158 |
| 10/5/2008 | 2.540 |
| 10/6/2008 | 0.762 |
| 10/8/2008 | 5.334 |
| 10/9/2008 | 25.400 |
| 10/10/2008 | 3.556 |
| 10/12/2008 | 0.254 |
| 10/13/2008 | 0.762 |
| 10/18/2008 | 2.032 |
| 10/24/2008 | 16.510 |
| 10/25/2008 | 4.064 |
| Monthly Total (mm) | 61.214 |
| 11/2/2008 | 12.446 |
| 11/12/2008 | 0.762 |
| 11/14/2008 | 0.508 |
| 11/15/2008 | 0.508 |
| 11/30/2008 | 2.794 |
| Monthly Total (mm) | 17.018 |
| 12/2/2008 | 5.080 |
| 12/6/2008 | 0.508 |
| 12/11/2008 | 10.160 |
| 12/12/2008 | 0.254 |
| 12/17/2008 | 0.254 |
| 12/21/2008 | 0.254 |
| 12/25/2008 | 9.398 |
| Monthly Total (mm) | 25.908 |
| Annual Total (mm) | 948.436 |