Guana Tolomato Matanzas (GTM) National Estuarine Research Reserve
Nutrient Metadata Report

January – December 2013

Latest Update: May 15, 2014

Note: This is a provisional metadata document; it has not been authenticated as of its download date.  Contents of this document are subject to change throughout the QAQC process and it should not be considered a final record of data documentation until that process is complete.  Contact the CDMO (cdmosupport@belle.baruch.sc.edu) or Reserve with any additional questions.

I.  Data Set and Research Descriptors

- Principal investigator(s) and contact persons

  - Principal Investigator:

Nikki Dix, Research Coordinator

505 Guana River Road

Ponte Vedra Beach, FL 32082

(904) 823-4500

nikki.dix@dep.state.fl.us

  - SWMP Technicians:

Kathryn M. Petrinec, SWMP Manager

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4054

kathryn.petrinec@dep.state.fl.us

Matthew Welsh, SWMP Technician

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4061

matthew.welsh@dep.state.fl.us

  - Laboratory Contacts:

Timothy W. Fitzpatrick, Chemistry Section Administrator

Department of Environmental Protection

2600 Blair Stone Road

Tallahassee, FL 32399-2400

(850) 245-8083

Timothy.Fitzpatrick@dep.state.fl.us

David Whiting, Biology Section Administrator

Department of Environmental Protection

2600 Blair Stone Road

Tallahassee, FL 32399-2400

(850) 245-8177

David.D.Whiting@dep.state.fl.us

- Research Objectives

The System-wide Monitoring Program (SWMP) water quality initiative began within the GTM NERR in 2001.  There are presently four permanent stations at which YSI 6600 EDS or YSI EXO2 data sondes have been deployed for continuously monitoring a suite of selected abiotic parameters at 15 minute intervals.  The positions of these stations allow for comparisons between relatively pristine versus more urbanized drainage basins as well as higher versus lower salinity regions of the estuary (see “Station Descriptions” under “Site Location and Character”). Nutrient analyses are performed on water samples collected monthly at each of these sites.  The objective of this effort is to quantify the spatial/temporal variability and trends; both seasonally and as a function of tidal forcing, of selected nutrient parameters within the Reserve and to explore how these are related to concurrently generated data sonde and meteorological data.

  - Monthly grab – The principal objective of the monthly grab sampling is to determine spatial and

temporal differences in water quality in four ecologically distinct regions of the GTM NERR.

  - Diel sampling program – The principal objective of the diel sampling is to determine the impact

of tidal water exchange on water quality.

- Research methods

- Monthly grab sampling program

Monthly grab samples were collected at four stations within the GTM NERR. These samples were taken at the four principal data sonde stations (Pine Island, San Sebastian, Fort Matanzas and Pellicer Creek). All grab samples were obtained during the same ebb tide of each sampling day.  No distinction was made between neap and spring tide conditions.  Efforts were made to allow for an antecedent dry period of 72 hours prior to sampling.  Replicate (N=2) samples were collected using a pole sampler lowered to the approximate depth of the data sonde probes (i.e., 1 meter above the bottom) in closed position and then opened to draw water from the specified depth.  All water samples were collected in new nalgene sample bottles and were rinsed with ambient water prior to collection of the sample. Orthophosphate, Nitrite, Nitrate, Nitrite+Nitrate and Ammonium samples were filtered in the field and placed on ice.  Nitrite+Nitrate, Ammonium, Total Phosphorus, and Total Kjeldahl Nitrogen samples were also acidified to a pH of 2 using Sulfuric Acid.  Whole water was collected for Total phosphorus, Total Suspended Solids, Chlorophyll a, Pheophytin, and Fecal Coliform samples and immediately placed on ice.   Chlorophyll a and Pheophytin samples were collected in a dark nalgene bottle and  were filtered immediately upon returning to the Florida Department of Environmental Protection laboratory in Tallahassee. Once in the laboratory, samples were shaken and processed for nutrients, chlorophyll a and solids analyses.  At the time of sample collection, water temperature, salinity, dissolved oxygen concentration and pH were measured with YSI hand-held that were calibrated prior to measurement.

- Diel sampling program

Diel sampling was carried out at the Pellicer Creek site.  All water samples were collected automatically at 2.5 hour intervals for a 25 hour (lunar day) period using an ISCO Sampler.  The water was collected from the approximate depth of the data sonde probes (i.e., 1 meter above the bottom).  Diel sampling was generally performed during the same time periods as the monthly grab samples.  The samples were collected into ISCO bottles that were previously acid washed (19%), rinsed (3x) with distilled-deionized water and dried prior to sample collection.  The ISCO unit is equipped with an internal ice bath to keep samples cold.  At the end of the 25-hour period, sample bottles were removed from the ISCO and processed in the same manner as described for the grab samples.

- Site location and character

The GTM NERR (North section [NW and SE corners]: 30.1632º N, 81.3447º W and 29.9698º N, 81.2488º W; South section: 29.8295º N, 81.3294º W and 29.6017º N, 81.1936º W), located in the Florida Upper East Coast Drainage Basin, includes over 24,281 ha of publicly owned forested uplands, tidal wetlands, estuarine lagoons and offshore seas.  Geographically separated by the greater St. Augustine area, the Reserve is associated with Tolomato and Guana River estuaries to the north and the Matanzas River estuary to the south.  The Tolomato River Basin is about 29 km in length with a drainage area of approximately 21,773 ha; it converges with the Matanzas River and Salt Run from the south before flowing into the Atlantic Ocean at the St. Augustine Inlet.  The headwaters of the Guana River originate in the Diego Plains drainage area in Ponte Vedra Beach.  This drainage basin encompasses approximately 3,157 ha.  The Guana River runs parallel to the Tolomato on the seaward side, with the two lagoons joining 11.3 km north of the St. Augustine Inlet.  The Matanzas River estuary is approximately 32 km in length and extends 13 km south of the Matanzas Inlet.  The Matanzas River sub-basin has a drainage area of approximately 41,931 ha and is bounded to the west by the Atlantic Coastal Ridge, which separates it from the lower St. Johns River basin.  Both the St. Augustine and Matanzas Inlets provide oceanic exchange to the system.  The Matanzas Inlet, one of the last “natural” inlets on Florida’s east coast, has been unimproved and is suitable only for small watercraft.  The natural hydrology of the Guana Tolomato Matanzas system has been somewhat altered by water control structures, including dikes, inland wells, drainage ditches and a dam across a portion of the Guana River.  In addition, the Intracoastal Waterway traverses both the Tolomato and Matanzas estuaries.

The climate of northeast Florida is classified as humid subtropical and is characteristic of the Gulf and Atlantic coastal plain of the southeastern United States.  The average annual rainfall is approximately 132 cm per year, with the wet season extending from June through September.  Seasonal variation in temperature within the Reserve follows that of rainfall with a summer period of high temperatures between June and September and a cooler period extending from December through March.  The annual mean air temperature within the Reserve is approximately 21C.

Station descriptions

The Pine Island (PI) station is at Channel Marker 25 (30 03.051´N; 81 22.048´W) in the Tolomato River.  This site is located within the Guana River Marsh Aquatic Preserve in the northern section of the GTM NERR.  Channel Marker 25 is adjacent to Pine Island near the mouth of Deep Creek, which provides freshwater drainage from silviculture-dominated uplands in the northwestern portion of the Tolomato River basin.  The average depth at this site is approximately 3.8 m with a tidal range of about 1.6 m; the bottom type is muddy sand. Salinity ranged from 7.2 to 36.2 ppt during 2013. The Fort Matanzas (FM) site (in the southern section of the GTM NERR) is located at Channel Marker 75 (29 44.222´N; 81 14.757´W) in the Matanzas River.  This site is approximately 4 km north of the Matanzas Inlet and near a shoreline on Anastasia Island that is undergoing residential development.  The average depth at this site is approximately 3.6 m with a tidal range of about 1.4 m; the bottom type is muddy sand.  Salinity ranged from 19.1 to 37.9 ppt. during 2013.  Both the FM and PI stations are situated within Class II Shellfish Harvesting Waters.  The San Sebastian (SS) station is at Channel Marker 1 (29 52.131´N; 81 18.446´W) located at the confluence of the San Sebastian and Matanzas Rivers, approximately 4 km south of the St. Augustine Inlet.  The San Sebastian River drains an urbanized watershed in the western portion of St. Augustine; it is characterized by both residential and commercial development including five marinas.  The average depth at this site is approximately 4.4 m with a tidal range of about 1.7 m; the bottom type is muddy sand. Salinity ranged from 13.6 to 37.5 ppt. during the 2013.  The Pellicer Creek (PC) station (29 40.024´N; 81 15.444´W) is at the end of a recreational dock in Faver Dykes State Park located within the Pellicer Creek Aquatic Preserve in the southern section of the GTM Reserve.  Pellicer Creek is tidal and bordered for much of its length by publicly-owned conservation lands.  The average depth at this site is approximately 2.3 m with a tidal range of about 0.6 m; the bottom type is muddy sand. Salinity ranged from 0.1 to 36.2 ppt. during 2013.

- Coded variable definitions

| Station Codes |  |  |
| --- | --- | --- |
| gtmfmnut | GTM Reserve nutrient data for Fort Matanzas |  |
| gtmssnut | GTM Reserve nutrient data for San Sebastian |  |
| gtmpinut | GTM Reserve nutrient data for Pine Island |  |
| gtmpcnut | GTM Reserve nutrient data for Pellicer Creek |  |
| Monitoring program codes |  |  |
| Monthly grab sample program |  | 1 |
| Diel grab sample program |  | 2 |

- Data collection period

The GTM NERR began collecting monthly grab samples at all four water quality sites in May 2002.  Diel sampling began at the Pellicer Creek site in July 2002.

Diel Data Collection Period:  January – December 2013

| Site | Start Date | Start Time | End Date | End Time |
| --- | --- | --- | --- | --- |
| gtmpcnut | 01/08/2013 | 09:30 | 01/09/2013 | 10:30 |
| gtmpcnut | 02/06/2013 | 09:30 | 02/07/2013 | 10:30 |
| gtmpcnut | 03/06/2013 | 09:30 | 03/07/2013 | 10:30 |
| gtmpcnut | 04/22/2013 | 09:30 | 04/23/2013 | 10:30 |
| gtmpcnut | 05/20/2013 | 09:43 | 05/21/2013 | 10:43 |
| gtmpcnut | 06/03/2013 | 09:30 | 06/04/2013 | 10:30 |
| gtmpcnut | 07/17/2013 | 09:30 | 07/18/2013 | 10:30 |
| gtmpcnut | 08/18/2013 | 09:45 | 08/19/2013 | 10:45 |
| gtmpcnut | 09/15/2013 | 09:30 | 09/16/2013 | 10:30 |
| gtmpcnut | 10/01/2013 | 09:30 | 10/02/2013 | 10:30 |
| gtmpcnut | 11/13/2013 | 09:30 | 11/14/2013 | 10:30 |
| gtmpcnut | 12/01/2013 | 09:30 | 12/02/2013 | 10:30 |

Grab Sample Data Collection Period: January – December 2013

| Site | Start Date | Start Time | End Date | End Time |
| --- | --- | --- | --- | --- |
| gtmfmnut | 01/09/2013 | 12:01 | 01/09/2013 | 12:18 |
| gtmfmnut | 02/07/2013 | 10:35 | 02/07/2013 | 10:47 |
| gtmfmnut | 03/07/2013 | 10:41 | 03/07/2013 | 10:49 |
| gtmfmnut | 04/23/2013 | 11:32 | 04/23/2013 | 11:40 |
| gtmfmnut | 05/21/2013 | 10:18 | 05/21/2013 | 10:23 |
| gtmfmnut | 06/04/2013 | 10:20 | 06/04/2013 | 10:29 |
| gtmfmnut | 07/18/2013 | 09:30 | 07/18/2013 | 09:44 |
| gtmfmnut | 08/19/2013 | 10:30 | 08/19/2013 | 10:39 |
| gtmfmnut | 09/16/2013 | 11:56 | 09/16/2013 | 12:05 |
| gtmfmnut | 10/02/2013 | 12:30 | 10/02/2013 | 12:45 |
| gtmfmnut | 11/14/2013 | 11:15 | 11/14/2013 | 11:25 |
| gtmfmnut | 12/02/2013 | 12:30 | 12/02/2013 | 12:35 |
| gtmpcnut | 01/09/2013 | 10:53 | 01/09/2013 | 11:19 |
| gtmpcnut | 02/07/2013 | 10:23 | 02/07/2013 | 10:39 |
| gtmpcnut | 03/07/2013 | 10:07 | 03/07/2013 | 10:30 |
| gtmpcnut | 04/23/2013 | 11:08 | 04/23/2013 | 11:22 |
| gtmpcnut | 05/21/2013 | 10:45 | 05/21/2013 | 10:50 |
| gtmpcnut | 06/04/2013 | 10:43 | 06/04/2013 | 10:48 |
| gtmpcnut | 07/18/2013 | 10:45 | 07/18/2013 | 10:50 |
| gtmpcnut | 08/19/2013 | 10:45 | 08/19/2013 | 10:55 |
| gtmpcnut | 09/16/2013 | 10:29 | 09/16/2013 | 10:52 |
| gtmpcnut | 10/02/2013 | 10:15 | 10/02/2013 | 10:30 |
| gtmpcnut | 11/14/2013 | 09:48 | 11/14/2013 | 10:23 |
| gtmpcnut | 12/02/2013 | 10:00 | 12/02/2013 | 10:14 |
| gtmpinut | 01/09/2013 | 09:15 | 01/09/2013 | 09:23 |
| gtmpinut | 02/07/2013 | 08:01 | 02/07/2013 | 08:12 |
| gtmpinut | 03/07/2013 | 07:56 | 03/07/2013 | 08:15 |
| gtmpinut | 04/23/2013 | 09:16 | 04/23/2013 | 09:24 |
| gtmpinut | 05/21/2013 | 07:48 | 05/21/2013 | 07:58 |
| gtmpinut | 06/04/2013 | 08:03 | 06/04/2013 | 08:17 |
| gtmpinut | 07/18/2013 | 06:50 | 07/18/2013 | 07:03 |
| gtmpinut | 08/19/2013 | 08:10 | 08/19/2013 | 08:23 |
| gtmpinut | 09/16/2013 | 09:40 | 09/16/2013 | 10:05 |
| gtmpinut | 10/02/2013 | 10:15 | 10/02/2013 | 10:28 |
| gtmpinut | 11/14/2013 | 09:15 | 11/14/2013 | 09:25 |
| gtmpinut | 12/02/2013 | 10:40 | 12/02/2013 | 10:46 |
| gtmssnut | 01/09/2013 | 10:50 | 01/09/2013 | 11:03 |
| gtmssnut | 02/07/2013 | 09:30 | 02/07/2013 | 09:40 |
| gtmssnut | 03/07/2013 | 09:43 | 03/07/2013 | 09:50 |
| gtmssnut | 04/23/2013 | 10:30 | 04/23/2013 | 10:35 |
| gtmssnut | 05/21/2013 | 09:10 | 05/21/2013 | 09:22 |
| gtmssnut | 06/04/2013 | 09:08 | 06/04/2013 | 09:28 |
| gtmssnut | 07/18/2013 | 08:15 | 07/18/2013 | 08:26 |
| gtmssnut | 08/19/2013 | 09:30 | 08/19/2013 | 09:38 |
| gtmssnut | 09/16/2013 | 10:57 | 09/16/2013 | 11:10 |
| gtmssnut | 10/02/2013 | 11:30 | 10/02/2013 | 11:45 |
| gtmssnut | 11/14/2013 | 10:15 | 11/14/2013 | 10:30 |
| gtmssnut | 12/02/2013 | 11:40 | 12/02/2013 | 11:41 |

- Associated researchers and projects

Complementary research - The GTM NERR nutrient monitoring program is part of a broader long-term effort to define the structure and function of coastal marine ecosystems in Florida, including their watersheds.  A central goal of this work is to provide the information needed to help assess the integrity and sustainability of Florida’s coastal marine resources.  Within the lagoonal ecosystems of the east coast of Florida that include the GTM NERR, Dr. Edward Phlips of the University of Florida has several ongoing research projects that complement the monitoring program, including a study of the plankton ecology of the Indian River Lagoon funded by the St. Johns River Water Management District and a long-term study of the impact of eutrophication on the ecology of the St. Johns River and estuary, both funded by the St. Johns River Water Management DistrictNikki Dix, a former graduate student of Dr. Phlips, conducted her master’s research on the effects of the 2004 hurricanes on the salinity and nutrient regime of Pellicer Creek.  She also received a NERR fellowship to examine water quality, plankton, and oysters throughout the Matanzas River estuary.  In addition, Asher Williams, former University of North Florida graduate student, and Matthew Kimball, former GTM NERR Research Coordinator, examined water quality patterns in the Nassau River estuary, a relatively natural system north of the GTM Reserve and the St. Johns River.  See the following publications for more information:

Dix, N., E.J. Phlips, and P. Suscy. 2013. Factors controlling phytoplankton biomass in a subtropical coastal lagoon: relative scales of influence. Estuaries and Coasts, 36(5):981-996.

Dix, N.G., E.J. Phlips, and R.A. Gleeson. 2008. Water quality changes in the Guana Tolomato Matanzas National Estuarine Research Reserve, Florida, associated with four tropical storms. Journal of Coastal Research, 55(SI):26-37.

Williams, A.A. and M.E. Kimball. 2013. Evaluation of long-term trends in hydrographic and nutrient

parameters in a southeast US coastal river. Environmental Monitoring and Assessment, 185:10495–10509.

Complementary monitoring - As part of the System Wide Monitoring Program (SWMP), the GTM NERR collects water quality (i.e., using YSI 6600 EDS,V2 2 or EXO2 data sondes) and weather data.  All SWMP data is available for download through the Centralized Data Management Office (CDMO), http://cdmo.baruch.sc.edu/.

- Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The Principal Investigator (PI) retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format.

II. Physical Structure Descriptors

- Entry verification

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; adds chosen parameters and facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and then automatically flags/codes measured values below MDL and inserts the MDL; calculates parameters chosen by the user and automatically flags/codes for component values below MDL, negative calculated values, and missing data; allows the user to apply QAQC flags and codes to the data; produces summary statistics; graphs selected parameters for review; and exports the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

Data entry, verification and chain of custody procedures follow those specified in the Florida Department of Health NELAP QA/QC certification plan.  Nutrient results are sent to the GTM NERR in .pdf format and are entered into excel spreadsheets from the documents received from the Florida Department of Environmental Protection Laboratory (Timothy Fitzpatrick or David Whiting ).  The GTM NERR staff (Kathryn Petrinec) enters and reviews the data entries prior to submission to the CDMO.

- Parameter titles and variable names by category

Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisks “*”.

| Data Category | Parameter | Variable Name | Units of Measure |
| --- | --- | --- | --- |
| Phosphorus |  |  |  |
|  | *Orthophosphate, Filtered | PO4F | mg/L as P |
|  | Total Phosphorus | TP | mg/L as P |
| Nitrogen |  |  |  |
|  | *Nitrite + Nitrate, Filtered | NO23F | mg/L as N |
|  | *Nitrite, Filtered | NO2F | mg/L as N |
|  | *Nitrate, Filtered | NO3F | mg/L as N |
|  | *Ammonium, Filtered | NH4F | mg/L as N |
|  | Total Kjeldahl Nitrogen | TKN | mg/L as N |
|  | Dissolved Inorganic Nitrogen | DIN | mg/L as N |
| Plant Pigments |  |  |  |
|  | Uncorrected Chlorophyll a | UncCHLa_N | µg/L |
|  | *Chlorophyll a | CHLA_N | µg/L |
|  | Pheophytin a | PHEA | µg/L |
| Other Lab Parameters |  |  |  |
|  | Total Suspended Solids | TSS | mg/L |
| Field Parameters |  |  |  |
|  | Salinity | SALT_N | ppt |
|  | Secchi disk depth | SECCHI | m |
|  | Dissolved Oxygen | DO_N | mg/L |
|  | Water Temperature | WTEM_N | degrees Celsius |
|  | pH | PH_N | standard units |

Notes:

  - Time is coded based on a 2400 clock and is referenced to Standard Time.

  - Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

- Measured or calculated laboratory parameters

  - Parameters measured directly

Nitrogen species:		NH4F, NO2F, NO3F, NO23F, TKN

Phosphorus species:		PO4F

Other:				CHLA_N, PHEA, UncCHLa_N, 
                                                              TSS, SALT_N, SECCHI, DO_N, 
                                                              WTEM_N, PH_N

  - Calculated parameters

DIN				NO23+NH4

TON				TKN – NH4

TN				TKN+NO23

- Limits of detection

Florida Department of Environmental Protection – Central Laboratory

Method Detection Limits (MDL) are determined using the method specified in the Federal Register, 40 CFR Part 136 Appendix B, using LCSs prepared near the estimated detection limit as surrogates to estimate methodological noise for censored methods (e.g., chromatographic methods which censor analytical noise) or, for uncensored methods, using actual method blanks to directly measure methodological noise. Where the possibility exists for significant systematic bias from sample preparation and handling or from the analytical determinative step (typically inorganic analyses), bias is taken into account when calculating detection limits.

These were the nominal MDLs in place during 2013; individual samples can have a different MDL due to required dilution or matrix interference.

| Parameter | Start Date | End Date | MDL |
| --- | --- | --- | --- |
| NH4F | 01/01/2013 | 03/31/2013 | 0.010 |
| NH4F | 04/01/2013 | 12/31/2013 | 0.002 |
| TKN | 01/01/2013 | 12/31/2013 | 0.080 |
| NO2F | 01/01/2013 | 12/31/2013 | 0.002 |
| NO23F | 01/01/2013 | 12/31/2013 | 0.004 |
| TP | 01/01/2013 | 12/31/2013 | 0.002 |
| PO4F | 01/01/2013 | 12/31/2013 | 0.004 |
| *TSS | 01/01/2013 | 12/31/2013 | 2 |
| CHLA_N | 01/01/2013 | 12/31/2013 | 0.55 |
| PHEA | 01/01/2013 | 12/31/2013 | 0.4 |
| UncCHLa_N | 01/01/2013 | 12/31/2013 | 0.4 |
| NO3F | 01/01/2013 | 12/31/2013 | 0.004 |

*TSS MDL is 3 if conductivity > 15,000 umhos/cm.

- Laboratory methods

Florida Department of Environmental Protection

- Parameter:  Ammonia (NH3)

Method Reference: U.S. Environmental Protection Agency (EPA), 1993, Determination of Ammonia Nitrogen by Semi-Automated Colorimetry, EPA Method 350.1 Revision 2.0. Cincinnati, OH.

Method Descriptor: Alkaline phenol and hypochlorite react with ammonia to form indophenol blue that is proportional to the ammonia concentration. The blue color is intensified with sodium nitroprusside.

Preservation Method: Samples are filtered through 0.7 µm pore size glass-fiber filters in the field, stored at 4oC and run within 48 hours.

- Parameter:  Nitrite (NO2) and Nitrate (NO3)

Method Reference:  U.S. Environmental Protection Agency (EPA), 1978. Nitrogen, Nitrate-Nitrite (Colorimetric, Automated, Cadmium Reduction), EPA Method 353.2 Ed Rev 1974, 1978.

Method Descriptor:  A filtered sample is passed through a column containing granulated copper-cadmium to reduce nitrate to nitrite. The nitrite (that originally present plus that reduced to nitrate) is determined by diazotizing with sulfanilamide and coupling with N-(1-naphthyl)-ethylenediamine dihydrochloride to form a highly colored azo dye which is measured colorimetrically. Separate, rather than combined nitrate-nitrite, values are readily obtained by carrying out the procedure first with, and then without, the Cu-Cd reduction step.

Preservation Method:  Samples are filtered through 0.7 µm pore size glass-fiber filters in the field, stored at 4oC and run within 48 hours.

- Parameter:  Total Kjeldahl Nitrogen (TKN)

Method Reference: U.S. Environmental Protection Agency (EPA), 1993. Determination of Total Kjeldahl nitrogen by Semi-Automated Colorimetry, EPA Method 351.2 Revision 2.0. Cincinnati, OH.

Method Descriptor: The sample is heated in the presence of sulfuric acid, H2SO4 for two and one half hours. The residue is cooled, diluted to 25 mL and analyzed for ammonia. This digested sample may also be used for phosphorus determination. Total Kjeldahl nitrogen is the sum of free-ammonia and organic nitrogen compounds which are converted to ammonium sulfate (NH4)2SO4, under the conditions of digestion described. Organic Kjeldahl nitrogen is the difference obtained by subtracting the free ammonia value from the total Kjeldahl nitrogen value. Reduced volume versions of this method that use the same reagents and molar ratios are acceptable provided they meet the quality control and performance requirements stated in the method.

Preservation Method: Whole water samples are collected and preserved with H2SO4 to a pH <2, stored for a maximum of 28 days and stored at 4oC.

- Parameter:  Orthophosphate (PO4) (Dissolved)

Method Reference: U.S. Environmental Protection Agency (EPA), 1993. Determination of Phosphorus by Semi-Automated Colorimetry, EPA Method 365.1 Revision 2.0. Cincinnati, OH.

Method Descriptor: A sample is appropriately treated to convert all phosphorus of interest to reactive orthophosphate. Ammonium molybdate and antimony potassium tartrate are added to the treated sample reacting with orthophosphate in an acidic medium to form an antimony-phospho-molybdate complex. This complex is reduced to an intensely blue-colored complex by ascorbic acid. The concentration of the orthophosphate is measured by detecting the absorbance of the complex with a spectrophotometer.

Preservation Method: Samples are filtered through 0.7 µm pore size glass-fiber filters in the field, preserved with H2SO4 to a pH <2, stored at 4oC, and run within 48 hours.

- Parameter:   Total Phosphorus (TP)

Method Reference: U.S. Environmental Protection Agency (EPA), 1993. Determination of Phosphorus by Semi-Automated Colorimetry, EPA Method 365.1 Revision 2.0. Cincinnati, OH.

Method Descriptor: A sample is appropriately treated to convert all phosphorus of interest to reactive orthophosphate. Ammonium molybdate and antimony potassium tartrate are added to the treated sample reacting with orthophosphate in an acidic medium to form an antimony-phospho-molybdate complex. This complex is reduced to an intensely blue-colored complex by ascorbic acid. The concentration of the orthophosphate is measured by detecting the absorbance of the complex with a spectrophotometer.

Preservation Method: Samples are stored for a maximum of 28 days and stored at 4oC.

- Parameter:  Chlorophyll a (CHLA), Functional Chlorophyll a (UncCHLa),  Pheophytin a (PHEA)

Method References:  APHA (American Public Health Association), 1998. Standard Methods for the Examination of Water and Wastewater, (SM 10200H).  20th Edition, Baltimore, Maryland: United Book Press, Inc. APHA (American Public Heath Association), 1998.

Method Descriptor:  Phytoplankton containing chlorophyll a in a measured volume of sample is concentrated by filtration through a glass fiber filter. The photo-pigments are extracted from the phytoplankton by grinding the filter with a tissue grinder and steeping the filter slurry in 90% aqueous acetone solution overnight. The filter slurry is then centrifuged to clarify the solution and then the supernatant is transferred to a glass spectrophotometric cell. For the pheophytin corrected chlorophyll a, the sample’s absorbance is measured at 750 and 664 nm before acidification and 750 and 665 nm after acidification with .1 N HCl. No calibration of the instrument is required. Absorbance values are entered into a set of equations in the computer that utilize the extinction coefficients of the pure pigments in 90% acetone. Concentrations are reported in ug/L.

Preservation Method:  Samples are collected as whole water samples and stored in a dark sampling bottle.  Samples are filtered onto 0.45 µm pore size glass-fiber filters in the laboratory and run immediately upon receiving samples.

- Parameter:  Total Suspended Solids (TSS)

Method Reference: APHA (American Public Health Association), 1999. Standard Methods for the Examination of Water and Wastewater (SM 2540 D-97). Baltimore, Maryland: United Book Press, Inc. APHA (American Public Health Association), 1999.

Method Descriptor:  A well-mixed sample is filtered through a weighed standard glass-fiber filter and the residue retained on the filter is dried to a constant weight at 103 to 105°C. The increase in weight of the filter represents the total suspended solids. If the suspended material clogs the filter and prolongs filtration, it may be necessary to increase the diameter of the filter or decrease the sample volume. To obtain an estimate of total suspended solids, calculate the difference between total dissolved solids and total solids.

Preservation Method: Filtration is performed within 48 hours of collection on whole water samples stored at 4oC.

- Field and Laboratory QAQC programs

In the field each month, two successive grab samples are collected at each site.  One field blank is included in each monthly collection.

The laboratory QA/QC procedures are as prescribed in the Quality Manual for State of Florida Department of Environmental Protection Bureau of Laboratories FDOH Certification Number E31780 (http://www.dep.state.fl.us/labs/docs/lab_qualitymanual.pdf).

The laboratory has an established quality control program for monitoring the performance of test methods conducted under the Quality Manual. The types of quality control (QC) checks and the frequency at which they are performed are listed in the method or test SOPs. A batch of samples consists of 20 or fewer samples that are prepared and/or analyzed in a single run. Saline matrices are batched separately where the test is impacted by high conductivity samples.

The QC objectives are considered when selecting methods and in evaluating the capability of the laboratory to handle sample loads that meet the QC objectives.

- Calibration standards are checked against certified reference materials or other independently prepared standards where available.

- The laboratory is a participant in performance testing studies.

- Replicate analyses are used to evaluate precision. Precision is expressed by the relative percent difference (RPD) to compare duplicate samples/spikes A and B and is based on the formula:

Precision may be determined from duplicate authentic samples, from duplicate LCSs or from

matrix spike duplicates. Where RPDs are calculated based on matrix spike duplicates, A and B represent the raw results of the spiked sample (spike plus the background).

- The accuracy of the test method is assessed in terms of percent recovery for LCSs (fortified blanks) and matrix spikes to evaluate matrix impact.

Quality assurance targets (QATs) for each QC check are defined in terms of relative precision (P) or accuracy (A). Analyte concentrations associated with each QC check are defined as high, mid, or low, depending on what range of the calibration curve the check concentration falls.

A statistical program, written in Borland Delphi and integrated with the Laboratory Information Management System (LIMS), handles the calculations of mean and standard deviation and also calculates warning and control limits for QC elements. QC data are uploaded from electronic QC analysis tools to the LIMS for storage. The statistical program looks at historical data and at a specified frequency, recalculates accuracy and precision acceptance limits for a specific sample matrix, instrument type and QC element. Warning and control limits are calculated when at least seven or more data points are available.

They may be updated when:

- a minimum of at least 7 new data points are available;

- significant changes are made to the instrument or analytical method;

- they have not been updated in the last twelve months.

The user selects at least 7 new, or the most recently generated data points (accuracy, precision or MDL values). The program calculates warning limits for these elements based on approximately two standard deviations from the mean and control limits based on approximately three standard deviations from the mean. In general, the laboratory utilizes method or laboratory defined warning and control limits for reporting data (i.e., statutory control limits). Those statutory limits may be modified utilizing statistical information collected over time. The precision and recovery data are used for the diagnosis of analytical problems. For laboratory parameters, calculated statistical control limits are used as criteria to accept or reject data only if they are more stringent than the listed criteria.

The formulae used for the calculation of standard deviation, mean, upper and lower control and warning limits are shown below. (Reference chapter 6 of "Handbook for Analytical Quality Control in Water and Wastewater Laboratories" - EPA 600/4-79-019, March 1979).

Because levels of statistical confidence vary with sample size, a fixed level of statistical confidence is employed that approximates 2 and 3 standard deviations. Those control limits are based on requirements specified in various EPA methods and in EPA’s ‘Handbook for Analytical Quality Control in Water and Wastewater Laboratories’.

- QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

- QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data suspect due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Calculated value could not be determined due to a below MDL component

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N		from the north

NNE	from the north northeast

NE		from the northeast

ENE	from the east northeast

E		from the east

ESE	from the east southeast

SE		from the southeast

SSE		from the south southeast

S		from the south

SSW	from the south southwest

SW		from the southwest

WSW	from the west southwest

W		from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

- Other remarks/notes

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Sample Holding Times

The FDEP Laboratory follows the EPA preservation and holding times (published in 40 CFR Part 136.3).  NO2, NO3, and PO4 are preserved at 4°C (wet ice) and have holding times of 48 hours.  Samples that are shipped overnight from the field on the day of collection are almost always received in time for analysis without concerns for expirations.  Diel samples are processed immediately upon receiving due to the expiring hold times.  The first diel sample is almost always received beyond its sample hold time for NO2, NO3 and PO4.

  - See Metadata (CSM)

January 2013

FM

- Laboratory indicated that the value reported for TSS sample gtmfmnut1.2 is the mean of two or more determinations.

PC

- Laboratory indicated that the value reported for TSS sample gtmpcnut1.1 is the mean of two or more determinations.

February 2013

SS

- Suspect NH4 for sample gtmssnut1.1; laboratory indicated that there may be more than the usual amount of uncertainty associated with the result.

PC

- Laboratory indicated that the value reported for TSS sample gtmpcnut1.1 is the mean of two or more determinations.

March 2013

PI

- Laboratory indicated that the value reported for TSS sample gtmpinut1.1 is the mean of two or more determinations.

April 2013

PI

- Laboratory indicated that the value reported for CHLA, UncCHLa, and PHEA sample gtmpinut1.1 is the mean of two or more determinations.

- Laboratory indicated that the value reported for TSS sample gtmpinut1.2 is the mean of two or more determinations.

May 2013

FM

- Laboratory indicated that the value reported for CHLA, UncCHLa, PHEA, and TSS sample gtmfmnut1.2 is the mean of two or more determinations.

PC

- Laboratory indicated that the value reported for TSS sample gtmpcnut1.1 is the mean of two or more determinations.

June 2013

PI

- Missing PO4, NO2, and NO3 data for samples, gtmpinut1.1 and gtmpinut1.2 attributed to contamination.

- Laboratory indicated that the value reported for TSS sample gtmpinut1.1 is the mean of two or more determinations.

SS

- Missing PO4, NO2, and NO3 data for sample gtmssnut1.1 attributed to contamination.

PC

- Suspect NO3 for sample gtmpcnut1.1;  laboratory indicated that there may be more than the usual amount of uncertainty associated with the result.

- Laboratory indicated that the value reported for CHLA, UncCHLa, and PHEA sample gtmpcnut1.2 is the mean of two or more determinations.

- Suspect NO2 and NO3 for sample gtmpcnut2.4;  laboratory indicated that there may be more than the usual amount of uncertainty associated with the result.

- Suspect NH4, NO3, and NO23 for sample gtmpcnut2.11;  laboratory indicated that the sample was received unpreserved or improperly preserved and that the data may not be accurate.

July 2013

FM

- Laboratory indicated that the value reported for CHLA, UncCHLa, and PHEA sample gtmfmnut1.2 is the mean of two or more determinations.

PC

- Suspect NO2 for sample gtmpcnut1.1, gtmpcnut1.2,  gtmpcnut2.1,  gtmpcnut2.2,  gtmpcnut2.3,  gtmpcnut2.4,  gtmpcnut2.7,  gtmpcnut2.8,  gtmpcnut2.9,  gtmpcnut2.10, and gtmpcnut2.11;  laboratory indicated that there may be more than the usual amount of uncertainty associated with the result.

- Suspect NO3 for sample gtmpcnut2.5, gtmpcnut2.7, gtmpcnut2.8, and gtmpcnut2.9;  laboratory indicated that there may be more than the usual amount of uncertainty associated with the result.

- Missing CHLA, UncCHLa, and PHEA sample gtmpcnut1.2, gtmpcnut2.5, and gtmpcnut2.9; attributed to sample labels becoming wet and illegible during shipment.

- Missing TSS samples gtmpcnut2.3 and gtmpcnut2.4; attributed to sample labels becoming wet and illegible during shipment.

August 2013

FM

- Laboratory indicated that the value reported for TSS sample gtmfmnut1.2 is the mean of two or more determinations.

PC

- Laboratory indicated that the value reported for TSS sample gtmpcnut1.1 is the mean of two or more determinations.

September 2013

FM

- Laboratory indicated that the value reported for TSS samples gtmfmnut1.1 and gtmfmnut1.2 is the mean of two or more determinations.

October 2013

PC

- Laboratory indicated that the value reported for TSS sample gtmpcnut1.2 is the mean of two or more determinations.

- Suspect NO2 and NO3 for sample gtmpcnut2.5;  laboratory indicated that there may be more than the usual amount of uncertainty associated with the result.

- Laboratory indicated that the value reported for CHLA, and UncCHLa sample gtmpcnut2.8 is the mean of two or more determinations.

November 2013

PC

- Laboratory indicated that the value reported for CHLA, UncCHLa, and PHEA sample gtmpcnut1.1 is the mean of two or more determinations.

December 2013

SS

- Laboratory indicated that the value reported for TSS sample gtmssnut1.2 is the mean of two or more determinations.

  - Daily, Monthly, and Annual Precipitation Totals (from raw data) at Pellicer Creek Weather Station (gtmpcmet).

| Date | Daily Precip. Totals (mm) |
| --- | --- |
| 01/03/2013 | 1.016 |
| 01/04/2013 | 0.254 |
| 01/06/2013 | 0.254 |
| 01/08/2013 | 0.254 |
| 01/09/2013 | 0.762 |
| 01/10/2013 | 0.254 |
| 01/31/2013 | 1.016 |
| January Monthly Total | 3.810 |
| 02/06/2013 | 2.032 |
| 02/07/2013 | 6.096 |
| 02/13/2013 | 3.048 |
| 02/14/2013 | 4.572 |
| 02/25/2013 | 11.176 |
| 02/26/2013 | 11.430 |
| February Monthly Total | 38.354 |
| 03/12/2013 | 0.508 |
| 03/17/2013 | 1.524 |
| 03/18/2013 | 0.508 |
| 03/20/2013 | 7.874 |
| 03/21/2013 | 0.254 |
| 03/23/2013 | 37.592 |
| 03/24/2013 | 0.508 |
| March Monthly Total | 48.768 |
| 04/01/2013 | 1.270 |
| 04/03/2013 | 13.970 |
| 04/04/2013 | 0.508 |
| 04/05/2013 | 3.048 |
| 04/14/2013 | 53.848 |
| 04/19/2013 | 10.414 |
| 04/20/2013 | 4.572 |
| 04/21/2013 | 23.876 |
| 04/29/2013 | 7.112 |
| 04/30/2013 | 7.366 |
| April Monthly Total | 118.872 |
| 05/02/2013 | 83.312 |
| 05/03/2013 | 122.682 |
| 05/04/2013 | 50.038 |
| 05/05/2013 | 0.254 |
| 05/11/2013 | 4.064 |
| 05/18/2013 | 5.588 |
| 05/19/2013 | 11.684 |
| 05/20/2013 | 1.016 |
| 05/22/2013 | 2.794 |
| May Monthly Total | 281.432 |
| 06/01/2013 | 12.446 |
| 06/03/2013 | 22.352 |
| 06/04/2013 | 10.414 |
| 06/05/2013 | 2.540 |
| 06/06/2013 | 40.640 |
| 06/07/2013 | 0.254 |
| 06/10/2013 | 7.366 |
| 06/14/2013 | 13.208 |
| 06/17/2013 | 3.048 |
| 06/19/2013 | 0.762 |
| 06/20/2013 | 0.508 |
| 06/21/2013 | 7.620 |
| 06/22/2013 | 5.080 |
| 06/27/2013 | 8.636 |
| 06/28/2013 | 6.350 |
| 06/30/2013 | 10.160 |
| June Monthly Total | 151.384 |
| 07/01/2013 | 0.762 |
| 07/02/2013 | 57.150 |
| 07/11/2013 | 16.510 |
| 07/12/2013 | 57.404 |
| 07/13/3013 | 23.622 |
| 07/15/2013 | 0.762 |
| 07/19/2013 | 40.132 |
| 07/20/2013 | 5.588 |
| 07/21/2013 | 17.272 |
| 07/22/2013 | 5.842 |
| 07/23/2013 | 0.762 |
| 07/27/2013 | 2.540 |
| 07/28/2013 | 30.734 |
| 07/31/2013 | 29.718 |
| July Monthly Total | 288.798 |
| 08/01/2013 | 1.778 |
| 08/04/2013 | 6.350 |
| 08/09/2013 | 4.826 |
| 08/14/2013 | 0.762 |
| 08/15/2013 | 36.322 |
| 08/16/2013 | 13.462 |
| 08/17/2013 | 15.494 |
| 08/19/2013 | 1.524 |
| 08/21/2013 | 23.144 |
| 08/22/2013 | 1.270 |
| 08/23/2013 | 2.032 |
| 08/24/2013 | 11.938 |
| 08/25/2013 | 10.160 |
| 08/26/2013 | 2.032 |
| August Monthly Total | 131.064 |
| 09/01/2013 | 12.954 |
| 09/02/2013 | 13.970 |
| 09/03/2013 | 2.540 |
| 09/06/2013 | 0.254 |
| 09/07/2013 | 7.112 |
| 09/10/2013 | 1.016 |
| 09/11/2013 | 19.304 |
| 09/12/2013 | 0.762 |
| 09/16/2012 | 4.318 |
| 09/17/2013 | 0.254 |
| 09/18/2013 | 20.828 |
| 09/19/2013 | 0.254 |
| 09/22/2013 | 0.508 |
| 09/23/2013 | 5.080 |
| 09/24/2013 | 5.588 |
| 09/25/2013 | 1.524 |
| 09/27/2013 | 1.270 |
| 09/28/2013 | 55.623 |
| 09/29/2013 | 1.016 |
| 09/30/2013 | 1.524 |
| September Monthly Total | 155.702 |
| 10/01/2013 | 2.540 |
| 10/06/2013 | 5.080 |
| 10/07/2013 | 32.258 |
| 10/08/2013 | 1.778 |
| 10/09/2013 | 1.016 |
| 10/14/2013 | 6.604 |
| 10/15/2013 | 7.112 |
| 10/19/2013 | 28.956 |
| 10/20/2013 | 0.254 |
| 10/21/2013 | 12.954 |
| 10/22/2013 | 1.270 |
| 10/31/2013 | 1.016 |
| October Monthly Total | 100.838 |
| 11/02/2013 | 9.652 |
| 11/05/2013 | 10.922 |
| 11/06/2013 | 0.254 |
| 11/15/2013 | 4.064 |
| 11/16/2013 | 0.762 |
| 11/20/2013 | 1.270 |
| 11/26/2013 | 3.556 |
| 11/27/2013 | 6.858 |
| November Monthly Total | 37.338 |
| 12/10/2013 | 1.016 |
| 12/14/2013 | 34.798 |
| 12/15/2013 | 48.541 |
| 12/18/2013 | 0.508 |
| 12/24/2013 | 0.254 |
| 12/27/2013 | 0.254 |
| 12/28/2013 | 2.032 |
| 12/29/2013 | 3.556 |
| January – December 2013 Total | 1356.360 |