Guana Tolomato Matanzas (GTM) National Estuarine Research Reserve
Nutrient Metadata Report

January – December 2015

Latest Update: June 15, 2016

Note: This is a provisional metadata document; it has not been authenticated as of its download date.  Contents of this document are subject to change throughout the QAQC process and it should not be considered a final record of data documentation until that process is complete.  Contact the CDMO (cdmosupport@belle.baruch.sc.edu) or Reserve with any additional questions.

I.  Data Set and Research Descriptors

- Principal investigator(s) and contact persons

  - Principal Investigator:

Nikki Dix, Research Coordinator

505 Guana River Road

Ponte Vedra Beach, FL 32082

(904) 823-4500

nikki.dix@dep.state.fl.us

  - SWMP Technicians:

Matthew Welsh, SWMP Manager

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4061

matthew.welsh@dep.state.fl.us

Kathryn M. Petrinec, SWMP Data Specialist

9741 Ocean Shore Blvd.

St. Augustine, FL 32080

(904) 461-4054

kathryn.petrinec@dep.state.fl.us

  - Laboratory Contacts:

Timothy W. Fitzpatrick, Chemistry Section Administrator

Department of Environmental Protection

2600 Blair Stone Road

Tallahassee, FL 32399-2400

(850) 245-8083

Timothy.Fitzpatrick@dep.state.fl.us

David Whiting, Biology Section Administrator

Department of Environmental Protection

2600 Blair Stone Road

Tallahassee, FL 32399-2400

(850) 245-8177

David.D.Whiting@dep.state.fl.us

- Research Objectives

The System-wide Monitoring Program (SWMP) water quality initiative began within the GTMNERR in 2001.  There are presently four permanent stations at which YSI 6600 EDS or YSI EXO2 data sondes have been deployed for continuously monitoring a suite of selected abiotic parameters at 15 minute intervals.  The positions of these stations allow for comparisons between relatively pristine versus more urbanized drainage basins as well as higher versus lower salinity regions of the estuary (see “Station Descriptions” under “Site Location and Character”). Nutrient analyses are performed on water samples collected monthly at each of these sites.  The objective of this effort is to quantify the spatial/temporal variability and trends; both seasonally and as a function of tidal forcing, of selected nutrient parameters within the Reserve and to explore how these are related to concurrently generated data sonde and meteorological data.

  - Monthly grab – The principal objective of the monthly grab sampling is to determine spatial and

temporal differences in water quality in four ecologically distinct regions of the GTMNERR.

  - Diel sampling program – The principal objective of the diel sampling is to determine the impact

of tidal water exchange on water quality.

- Research methods

- Monthly grab sampling program

Monthly grab samples were collected at four stations within the GTMNERR. These samples were taken at the four principal data sonde stations (Pine Island, San Sebastian, Fort Matanzas and Pellicer Creek). All grab samples were obtained during the same ebb tide of each sampling day.  No distinction was made between neap and spring tide conditions.  Efforts were made to allow for an antecedent dry period of 72 hours prior to sampling.  Replicate (N=2) samples were collected using a pole sampler lowered to the approximate depth of the data sonde probes (i.e., 1 meter above the bottom) in closed position and then opened to draw water from the specified depth.  All water samples were collected in new nalgene sample bottles and were rinsed with ambient water prior to collection of the sample. Orthophosphate, Nitrite, Nitrite+Nitrate and Ammonium samples were filtered in the field and placed on ice.  Nitrite+Nitrate, Ammonium, Total Phosphorus, and Total Kjeldahl Nitrogen samples were also acidified to a pH of 2 using Sulfuric Acid.  Whole water was collected for Total Phosphorus, Total Suspended Solids, Chlorophyll a, Pheophytin, Fecal Coliform, and Escherichia Coli samples and immediately placed on ice.   Chlorophyll a and Pheophytin samples were collected in a dark nalgene bottle and were filtered immediately upon returning to the Florida Department of Environmental Protection laboratory in Tallahassee. Once in the laboratory, samples were shaken and processed for nutrients, chlorophyll a and solids analyses.  At the time of sample collection, water temperature, salinity, dissolved oxygen concentration and pH were measured with YSI hand-held that was calibrated prior to measurement.  Secchi depth was also recorded.

- Diel sampling program

Diel sampling was carried out at the Pellicer Creek site.  All water samples were collected automatically at 2.5 hour intervals for a 25 hour (lunar day) period using an ISCO Sampler.  The water was collected from the approximate depth of the data sonde probes (i.e., 1 meter above the bottom).  Diel sampling was generally performed during the same time periods as the monthly grab samples.  The samples were collected into ISCO bottles that were previously acid washed (20%), rinsed (3x) with deionized water and dried prior to sample collection.  The ISCO unit is equipped with an internal ice bath to keep samples cold.  At the end of the 25-hour period, sample bottles were removed from the ISCO and processed in the same manner as described for the grab samples.

- Site location and character

The GTMNERR (North section [NW and SE corners]: 30.1632º N, 81.3447º W and 29.9698º N, 81.2488º W; South section: 29.8295º N, 81.3294º W and 29.6017º N, 81.1936º W), located in the Florida Upper East Coast Drainage Basin, includes over 24,281 ha of publicly owned forested uplands, tidal wetlands, estuarine lagoons and offshore seas.  Geographically separated by the greater St. Augustine area, the Reserve is associated with Tolomato and Guana River estuaries to the north and the Matanzas River estuary to the south.  The Tolomato River Basin is about 29 km in length with a drainage area of approximately 21,773 ha; it converges with the Matanzas River and Salt Run from the south before flowing into the Atlantic Ocean at the St. Augustine Inlet.  The headwaters of the Guana River originate in the Diego Plains drainage area in Ponte Vedra Beach.  This drainage basin encompasses approximately 3,157 ha.  The Guana River runs parallel to the Tolomato on the seaward side, with the two lagoons joining 11.3 km north of the St. Augustine Inlet.  The Matanzas River estuary is approximately 32 km in length and extends 13 km south of the Matanzas Inlet.  The Matanzas River sub-basin has a drainage area of approximately 41,931 ha and is bounded to the west by the Atlantic Coastal Ridge, which separates it from the lower St. Johns River basin.  Both the St. Augustine and Matanzas Inlets provide oceanic exchange to the system.  The Matanzas Inlet, one of the last “natural” inlets on Florida’s east coast, has been unimproved and is suitable only for small watercraft.  The natural hydrology of the Guana Tolomato Matanzas system has been somewhat altered by water control structures, including dikes, inland wells, drainage ditches and a dam across a portion of the Guana River.  In addition, the Intracoastal Waterway traverses both the Tolomato and Matanzas estuaries.  There are no known significant pollutants within the Reserve.

The climate of northeast Florida is classified as humid subtropical and is characteristic of the Gulf and Atlantic coastal plain of the southeastern United States.  The average annual rainfall is approximately 132 cm per year, with the wet season extending from June through September.  Seasonal variation in temperature within the Reserve follows that of rainfall with a summer period of high temperatures between June and September and a cooler period extending from December through March.  The annual mean air temperature within the Reserve is approximately 21C.

Station descriptions

The Pine Island (PI) station is at Channel Marker 25 (30° 3' 2.88 N, 81° 22' 3.00 W) in the Tolomato River. This site is located within the Guana River Marsh Aquatic Preserve in the northern section of the GTMNERR. Channel Marker 25 is adjacent to Pine Island near the mouth of Deep Creek, which provides freshwater drainage from silviculture-dominated uplands in the northwestern portion of the Tolomato River basin. The average depth at this site is approximately 3.8 m with a tidal range of about 1.6 m; the bottom type is muddy sand. Salinity ranged from 3.4 to 35.4 psu during 2015.  The Fort Matanzas (FM) site (in the southern section of the GTMNERR) is located at Channel Marker 75 (29° 44' 13.20 N, 81° 14' 45.60 W) in the Matanzas River. This site is approximately 4 km north of the Matanzas Inlet and near a shoreline on Anastasia Island that is undergoing residential development. The average depth at this site is approximately 3.6 m with a tidal range of about 1.4 m; the bottom type is muddy sand. Salinity ranged from 25.7 to 35.9 psu during 2015. Both the FM and PI stations are situated within Class II Shellfish Harvesting Waters.  The San Sebastian (SS) station is at Channel Marker 1 (29° 52' 08 N, 81° 18' 27 W) located at the confluence of the San Sebastian and Matanzas Rivers, approximately 4 km south of the St. Augustine Inlet. The San Sebastian River drains an urbanized watershed in the western portion of St. Augustine; it is characterized by both residential and commercial development including five marinas. The average depth at this site is approximately 4.4 m with a tidal range of about 1.7 m; the bottom type is muddy sand. Salinity ranged from 23.2 to 36.4 psu during the 2015.  The Pellicer Creek (PC) station (29° 39' 28.08 N, 81° 13' 58 W) is at the end of a recreational dock in Faver Dykes State Park located within the Pellicer Creek Aquatic Preserve in the southern section of the GTM Reserve. Pellicer Creek is tidal and bordered for much of its length by publicly-owned conservation lands. The average depth at this site is approximately 2.3 m with a tidal range of about 0.6 m; the bottom type is muddy sand. Salinity ranged from 0.1 to 32.4 psu during 2015.

- Coded variable definitions

| Station Codes |  |  |
| --- | --- | --- |
| gtmfmnut | GTM Reserve nutrient data for Fort Matanzas |  |
| gtmssnut | GTM Reserve nutrient data for San Sebastian |  |
| gtmpinut | GTM Reserve nutrient data for Pine Island |  |
| gtmpcnut | GTM Reserve nutrient data for Pellicer Creek |  |
| Monitoring program codes |  |  |
| Monthly grab sample program |  | 1 |
| Diel grab sample program |  | 2 |

- Data collection period

The GTMNERR began collecting monthly grab samples at all four water quality sites in May 2002.  Diel sampling began at the Pellicer Creek site in July 2002.  The collection periods for 2015 are listed in the following two tables.

Diel Data Collection Period:  January – December 2015

| Site | Start Date | Start Time | End Date | End Time |
| --- | --- | --- | --- | --- |
| gtmpcnut | 01/19/2015 | 09:30 | 01/20/2015 | 10:30 |
| gtmpcnut | 02/16/2015 | 09:30 | 02/17/2015 | 10:30 |
| gtmpcnut | 03/16/2015 | 09:30 | 03/17/2015 | 10:30 |
| gtmpcnut | 04/14/2015 | 09:30 | 04/15/2015 | 10:30 |
| gtmpcnut | 05/13/2015 | 09:30 | 05/14/2015 | 10:30 |
| gtmpcnut | 06/29/2015 | 09:30 | 06/30/2015 | 10:30 |
| gtmpcnut | 07/13/2015 | 09:30 | 07/14/2015 | 10:30 |
| gtmpcnut | 08/10/2015 | 09:30 | 08/11/2015 | 10:30 |
| gtmpcnut | 09/07/2015 | 09:30 | 09/08/2015 | 10:30 |
| gtmpcnut | 10/07/2015 | 09:30 | 10/08/2015 | 10:30 |
| gtmpcnut | 11/08/2015 | 09:30 | 11/09/2015 | 10:30 |
| gtmpcnut | 12/07/2015 | 09:30 | 12/08/2015 | 10:30 |

Grab Sample Data Collection Period: January – December 2015

| Site | Start Date | Start Time | End Date | End Time |
| --- | --- | --- | --- | --- |
| gtmfmnut | 01/20/2015 | 12:39 | 01/20/2015 | 12:42 |
| gtmfmnut | 02/17/2015 | 11:45 | 02/17/2015 | 11:56 |
| gtmfmnut | 03/17/2015 | 07:53 | 03/17/2015 | 08:10 |
| gtmfmnut | 04/15/2015 | 11:50 | 04/15/2015 | 11:53 |
| gtmfmnut | 05/14/2015 | 10:22 | 05/14/2015 | 10:28 |
| gtmfmnut | 06/30/2015 | 12:00 | 06/30/2015 | 12:10 |
| gtmfmnut | 07/14/2015 | 11:30 | 07/14/2015 | 11:45 |
| gtmfmnut | 08/11/2015 | 10:55 | 08/11/2015 | 10:59 |
| gtmfmnut | 09/08/2015 | 09:45 | 09/08/2015 | 09:50 |
| gtmfmnut | 10/08/2015 | 10:15 | 10/08/2015 | 10:20 |
| gtmfmnut | 11/09/2015 | 13:13 | 11/09/2015 | 13:15 |
| gtmfmnut | 12/08/2015 | 11:30 | 12/08/2015 | 11:31 |
| gtmpcnut | 01/20/2015 | 09:47 | 01/20/2015 | 10:09 |
| gtmpcnut | 02/17/2015 | 11:23 | 02/17/2015 | 11:26 |
| gtmpcnut | 03/17/2015 | 09:32 | 03/17/2015 | 09:50 |
| gtmpcnut | 04/15/2015 | 08:53 | 04/15/2015 | 09:00 |
| gtmpcnut | 05/14/2015 | 09:12 | 05/14/2015 | 09:16 |
| gtmpcnut | 06/30/2015 | 09:04 | 06/30/2015 | 09:10 |
| gtmpcnut | 07/14/2015 | 09:03 | 07/14/2015 | 09:05 |
| gtmpcnut | 08/11/2015 | 09:03 | 08/11/2015 | 09:06 |
| gtmpcnut | 09/08/2015 | 08:48 | 09/08/2015 | 09:10 |
| gtmpcnut | 10/08/2015 | 08:46 | 10/08/2015 | 09:00 |
| gtmpcnut | 11/09/2015 | 09:53 | 11/09/2015 | 09:57 |
| gtmpcnut | 12/08/2015 | 09:48 | 12/08/2015 | 09:52 |
| gtmpinut | 01/20/2015 | 10:14 | 01/20/2015 | 10:25 |
| gtmpinut | 02/17/2015 | 09:39 | 02/17/2015 | 09:44 |
| gtmpinut | 03/17/2015 | 10:15 | 03/17/2015 | 10:22 |
| gtmpinut | 04/15/2015 | 08:05 | 04/15/2015 | 08:09 |
| gtmpinut | 05/14/2015 | 07:53 | 05/14/2015 | 07:58 |
| gtmpinut | 06/30/2015 | 09:00 | 06/30/2015 | 09:30 |
| gtmpinut | 07/14/2015 | 09:00 | 07/14/2015 | 09:15 |
| gtmpinut | 08/11/2015 | 08:47 | 08/11/2015 | 08:52 |
| gtmpinut | 09/08/2015 | 07:40 | 09/08/2015 | 07:45 |
| gtmpinut | 10/08/2015 | 08:03 | 10/08/2015 | 08:05 |
| gtmpinut | 11/09/2015 | 10:13 | 11/09/2015 | 10:15 |
| gtmpinut | 12/08/2015 | 08:45 | 12/08/2015 | 08:50 |
| gtmssnut | 01/20/2015 | 11:21 | 01/20/2015 | 11:32 |
| gtmssnut | 02/17/2015 | 09:47 | 02/17/2015 | 09:51 |
| gtmssnut | 03/17/2015 | 09:00 | 03/17/2015 | 09:12 |
| gtmssnut | 04/15/2015 | 09:30 | 04/15/2015 | 09:45 |
| gtmssnut | 05/14/2015 | 09:20 | 05/14/2015 | 09:25 |
| gtmssnut | 06/30/2015 | 10:40 | 06/30/2015 | 10:55 |
| gtmssnut | 07/14/2015 | 10:15 | 07/14/2015 | 10:40 |
| gtmssnut | 08/11/2015 | 09:59 | 08/11/2015 | 10:04 |
| gtmssnut | 09/08/2015 | 08:55 | 09/08/2015 | 09:00 |
| gtmssnut | 10/08/2015 | 09:05 | 10/08/2015 | 09:10 |
| gtmssnut | 11/09/2015 | 11:48 | 11/09/2015 | 11:50 |
| gtmssnut | 12/08/2015 | 10:15 | 12/08/2015 | 10:16 |

- Associated researchers and projects

The GTMNERR has formed partnerships with other agencies and organizations actively involved in resource protection in the GTMNERR watershed.  Cooperating managers of lands within the NERR include the:  1) National Park Service (NPS), 2) St. Johns River Water Management District (SJRWMD), 
3) Department of Environmental Protection (DEP) Division of Recreation and Parks/Florida Park Service (FPS), 4) Florida Fish and Wildlife Conservation Commission (FWC), 5) Florida Division of Forestry (DOF), and 6) Flagler County.

The SJRWMD Northern Coastal Basin Program, established in 1995, covers the watersheds along the Intracoastal Waterway from Ponce Inlet in Volusia County, north through Flagler and St. Johns Counties, to the Guana River marshes at Ponte Vedra in southern Duval County. This program was developed in response to concerns about the impacts of population growth and development on water quality in the Northern Coastal Basin (NCB).  Some of the major research activities conducted within the boundaries of the GTMNERR through the NCB program include hydrodynamic modeling, emergent marsh vegetation and oyster habitat mapping, and water quality monitoring.  Many of the water quality monitoring and assessment activities of the NCB program are aimed at pollution source identification and the development of pollution load reduction goals.

Other significant water quality research and monitoring initiatives within the GTMNERR include: 1) studies of nutrient profiles and plankton ecology by Dr. Edward Phlips at the University of Florida and 2) regular monitoring of fecal coliform levels in shellfish harvesting waters by the Department of Agriculture and Consumer Services.

To complement the water quality data, the GTMNERR also operates a weather station (data available at www.nerrsdata.org) and monitors biological components of the estuary such as emergent vegetation, plankton, and bivalves. Biological data are available upon request.

For further information on the GTMNERR, see the Site Profile at: http://nerrs.noaa.gov/Doc/PDF/Reserve/GTM_SiteProfile.pdf

- Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: www.nerrsdata.org; accessed 12 October 2012.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma separated version format.

II. Physical Structure Descriptors

- Entry verification

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; adds chosen parameters and facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and then automatically flags/codes measured values below MDL and inserts the MDL; calculates parameters chosen by the user and automatically flags/codes for component values below MDL, negative calculated values, and missing data; allows the user to apply QAQC flags and codes to the data; produces summary statistics; graphs selected parameters for review; and exports the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

Data entry, verification and chain of custody procedures follow those specified in the Florida Department of Health NELAP QA/QC certification plan.  Nutrient results are sent to the GTMNERR in .pdf format and are entered into excel spreadsheets from the documents received from the Florida Department of Environmental Protection Central Laboratory (Timothy Fitzpatrick and David Whiting).  The GTMNERR staff (Kathryn Petrinec) enters and reviews the data entries prior to submission to the CDMO.

- Parameter titles and variable names by category

Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisks “*”.

| Data Category | Parameter | Variable Name | Units of Measure |
| --- | --- | --- | --- |
| Phosphorus |  |  |  |
|  | *Orthophosphate, Filtered | PO4F | mg/L as P |
|  | Total Phosphorus | TP | mg/L as P |
| Nitrogen |  |  |  |
|  | *Nitrite + Nitrate, Filtered | NO23F | mg/L as N |
|  | *Nitrite, Filtered | NO2F | mg/L as N |
|  | *Nitrate, Filtered | NO3F | mg/L as N |
|  | *Ammonium, Filtered | NH4F | mg/L as N |
|  | Total Kjeldahl Nitrogen | TKN | mg/L as N |
|  | Dissolved Inorganic Nitrogen | DIN | mg/L as N |
|  | Total Nitrogen | TN | mg/L as N |
| Plant Pigments |  |  |  |
|  | Uncorrected Chlorophyll a | UncCHLa_N | µg/L |
|  | *Chlorophyll a | CHLA_N | µg/L |
|  | Pheophytin a | PHEA | µg/L |
| Other Lab Parameters |  |  |  |
|  | Total Suspended Solids | TSS | mg/L |
|  | Fecal Coliforms | FECCOL | cfu/100 mL |
|  | Escherichia Coli | ECOLI | mpn/100 mL |
| Field Parameters |  |  |  |
|  | Salinity | SALT_N | ppt |
|  | Secchi disk depth | SECCHI | m |
|  | Dissolved Oxygen | DO_N | mg/L |
|  | Water Temperature | WTEM_N | degrees Celsius |
|  | pH | PH_N | standard units |

Notes:

  - Time is coded based on a 2400 clock and is referenced to Standard Time.

  - Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

- Measured or calculated laboratory parameters

  - Parameters measured directly

Nitrogen species:		NH4F, NO2F, NO23F, TKN

Phosphorus species:		PO4F, TP

Other:				CHLA_N, UncCHLa_N, PHEA,
                                                              TSS, WTEM_N, SALT_N, DO_N, PH_N, SECCHI,

FECCOL, ECOLI

  - Calculated parameters

NO3                                            (NO23F*Df) – (NO2F*Df)

Df=Dilution factor

DIN				NO23F + NH4F

TON				TKN – NH4F

TN				TKN + NO23F

- Limits of detection

Florida Department of Environmental Protection – Central Laboratory

Method Detection Limits (MDL) are determined using the method specified in the Federal Register, 40 CFR Part 136 Appendix B, using LCSs prepared near the estimated detection limit as surrogates to estimate methodological noise for censored methods (e.g., chromatographic methods which censor analytical noise) or, for uncensored methods, using actual method blanks to directly measure methodological noise. Where the possibility exists for significant systematic bias from sample preparation and handling or from the analytical determinative step (typically inorganic analyses), bias is taken into account when calculating detection limits.

These were the nominal MDLs in place during 2015; individual samples can have a different MDL due to required dilution or matrix interference.

| Parameter | Start Date | End Date | MDL |
| --- | --- | --- | --- |
| NH4F | 01/01/2015 | 12/31/2015 | 0.002 |
| TKN | 01/01/2015 | 12/31/2015 | 0.080 |
| *NO2F | 01/01/2015 | 12/31/2015 | 0.002 |
| NO23F | 01/01/2015 | 12/31/2015 | 0.004 |
| TP | 01/01/2015 | 12/31/2015 | 0.002 |
| PO4F | 01/01/2015 | 12/31/2015 | 0.004 |
| **TSS | 01/01/2015 | 12/31/2015 | 2 |
| CHLA_N | 01/01/2015 | 12/31/2015 | 0.55 |
| UncCHLa_N | 01/01/2015 | 12/31/2015 | 0.40 |
| PHEA | 01/01/2015 | 12/31/2015 | 0.40 |
| ***FECCOL | 01/01/2015 | 12/31/2015 | 1 |
| ECOLI | 01/01/2015 | 12/31/2015 | 1 |
| NO3F | 01/01/2015 | 12/31/2015 | 0.004 |

* NO2 MDL is 0.003 for some samples in January, April, and September.  See Other Remarks section for more information.

**TSS MDL is 3 if conductivity > 15,000 umhos/cm.

***FECCOL MDL is 2 for some samples in December. See Other Remarks section for more information.

- Laboratory methods

Florida Department of Environmental Protection

- Parameter:  Ammonia (NH3)

Method Reference: U.S. Environmental Protection Agency (EPA), 1993, Determination of Ammonia Nitrogen by Semi-Automated Colorimetry, EPA Method 350.1 Revision 2.0. Cincinnati, OH. and OI Analytical Method 327152 utilizing gas diffusion.

Method Descriptor: The sample pH is raised to a pH of >11. The ammonia molecules generated at this pH pass through a gas diffusion membrane and are absorbed into an alkaline hypochlorite solution to form chloramine. The chloramine reacts with salicylate to form indophenol blue in an amount that is proportional to the ammonia concentration. Sodium nitroferricyanide intensifies the blue color. The absorbance is measured at 660 nm.

Preservation Method:

Samples are filtered through 0.7 µm pore size glass-fiber filters in the field and preserved with H2SO4 to a pH ≤ 2, stored on ice and shipped.  Samples are analyzed within 28 days of collection.

- Parameter:  Nitrite (NO2) and Nitrate-Nitrite (NO23)

Method Reference:  U.S. Environmental Protection Agency (EPA), 1978. Nitrogen, Nitrate-Nitrite (Colorimetric, Automated, Cadmium Reduction), EPA Method 353.2 Ed Rev 1974, 1978.

Method Descriptor:  A filtered sample is passed through a column containing granulated copper-cadmium to reduce nitrate to nitrite. The nitrite (that originally present plus that reduced to nitrate) is determined by diazotizing with sulfanilamide and coupling with N-(1-naphthyl)-ethylenediamine dihydrochloride to form a highly colored azo dye which is measured colorimetrically. Separate, rather than combined nitrate-nitrite, values are readily obtained by carrying out the procedure first with, and then without, the Cu-Cd reduction step.

Preservation Method:  Samples are filtered through 0.7 µm pore size glass-fiber filters in the field and preserved with H2SO4 to a pH ≤ 2, stored on ice and shipped.  Samples are analyzed within 28 days of collection.

- Parameter:  Total Kjeldahl Nitrogen (TKN)

Method Reference: U.S. Environmental Protection Agency (EPA), 1993. Determination of Total Kjeldahl nitrogen by Semi-Automated Colorimetry, EPA Method 351.2 Revision 2.0. Cincinnati, OH.

Method Descriptor: The sample is heated in the presence of sulfuric acid, H2SO4 for two and one half hours. The residue is cooled, diluted to 25 mL and analyzed for ammonia. This digested sample may also be used for phosphorus determination. Total Kjeldahl nitrogen is the sum of free-ammonia and organic nitrogen compounds which are converted to ammonium sulfate (NH4)2SO4, under the conditions of digestion described. Organic Kjeldahl nitrogen is the difference obtained by subtracting the free ammonia value from the total Kjeldahl nitrogen value. Reduced volume versions of this method that use the same reagents and molar ratios are acceptable provided they meet the quality control and performance requirements stated in the method.

Preservation Method: Whole water samples are collected and preserved with H2SO4 to a pH ≤2, stored for a maximum of 28 days and stored at 4oC.

- Parameter:  Orthophosphate (PO4) (Dissolved)

Method Reference: U.S. Environmental Protection Agency (EPA), 1993. Determination of Phosphorus by Semi-Automated Colorimetry, EPA Method 365.1 Revision 2.0. Cincinnati, OH.

Method Descriptor: A sample is appropriately treated to convert all phosphorus of interest to reactive orthophosphate. Ammonium molybdate and antimony potassium tartrate are added to the treated sample reacting with orthophosphate in an acidic medium to form an antimony-phospho-molybdate complex. This complex is reduced to an intensely blue-colored complex by ascorbic acid. The concentration of the orthophosphate is measured by detecting the absorbance of the complex with a spectrophotometer.

Preservation Method: Samples are filtered through 0.7 µm pore size glass-fiber filters in the field, preserved with H2SO4 to a pH ≤2, stored at 4oC, and run within 48 hours.

- Parameter:   Total Phosphorus (TP)

Method Reference: U.S. Environmental Protection Agency (EPA), 1993. Determination of Phosphorus by Semi-Automated Colorimetry, EPA Method 365.1 Revision 2.0. Cincinnati, OH.

Method Descriptor: A sample is appropriately treated to convert all phosphorus of interest to reactive orthophosphate. Ammonium molybdate and antimony potassium tartrate are added to the treated sample reacting with orthophosphate in an acidic medium to form an antimony-phospho-molybdate complex. This complex is reduced to an intensely blue-colored complex by ascorbic acid. The concentration of the orthophosphate is measured by detecting the absorbance of the complex with a spectrophotometer.

Preservation Method: Samples are preserved with H2SO4 to a pH ≤ 2, stored on ice and shipped.  Samples are analyzed within 28 days of collection.

- Parameter:  Chlorophyll a (CHLA), Functional (Uncorrected) Chlorophyll a (UncCHLa),  Pheophytin a (PHEA)

Method References:  APHA (American Public Health Association), 1998. Standard Methods for the Examination of Water and Wastewater, (SM 10200H).  20th Edition, Baltimore, Maryland: United Book Press, Inc. APHA (American Public Health Association), 1998.

Method Descriptor:  Phytoplankton containing chlorophyll a in a measured volume of sample is concentrated by filtration through a glass fiber filter. The photo-pigments are extracted from the phytoplankton by grinding the filter with a tissue grinder and steeping the filter slurry in 90% aqueous acetone solution overnight. The filter slurry is then centrifuged to clarify the solution and then the supernatant is transferred to a glass spectrophotometric cell. For the pheophytin corrected chlorophyll a, the sample’s absorbance is measured at 750 and 664 nm before acidification and 750 and 665 nm after acidification with .1 N HCl. No calibration of the instrument is required. Absorbance values are entered into a set of equations in the computer that utilize the extinction coefficients of the pure pigments in 90% acetone. Concentrations are reported in ug/L.

Preservation Method:  Samples are collected as whole water samples and stored in a dark sampling bottle.  Samples are filtered onto 0.45 µm pore size glass-fiber filters in the laboratory and run immediately upon receiving samples.

- Parameter:  Total Suspended Solids (TSS)

Method Reference: APHA (American Public Health Association), 1999. Standard Methods for the Examination of Water and Wastewater (SM 2540 D-97). Baltimore, Maryland: United Book Press, Inc. APHA (American Public Health Association), 1999.

Method Descriptor:  A well-mixed sample is filtered through a weighed standard glass-fiber filter and the residue retained on the filter is dried to a constant weight at 103 to 105°C. The increase in weight of the filter represents the total suspended solids. If the suspended material clogs the filter and prolongs filtration, it may be necessary to increase the diameter of the filter or decrease the sample volume. To obtain an estimate of total suspended solids, calculate the difference between total dissolved solids and total solids.

Preservation Method: Filtration is performed within 48 hours of collection on whole water samples stored at 4oC.

- Field and Laboratory QAQC programs

In the field each month, two successive grab samples are collected at each site.  One field blank is included in each monthly collection and one duplicate sample.

The laboratory QA/QC procedures are as prescribed in the Quality Manual for State of Florida Department of Environmental Protection Bureau of Laboratories FDOH Certification Number E31780 (http://www.dep.state.fl.us/labs/docs/lab_qualitymanual.pdf).

The laboratory has an established quality control program for monitoring the performance of test methods conducted under the Quality Manual. The types of quality control (QC) checks and the frequency at which they are performed are listed in the method or test SOPs. For every batch of samples (maximum 20 samples) digested and/or analyzed, the following quality control samples are analyzed:

- A Laboratory Fortified Blank (LFB)

- Digestion or Method Blank (MBLK)

- One Matrix Spike

- Matrix Spike Duplicate or Sample Duplicate

Standards are prepared and documented.  Calibration curves are generated at the beginning of each analytical run or at the frequency specified in the method SOP.  The curves are checked against certified or other independently prepared standards during each analytical run.  Control standards are analyzed at least every 20 samples.  A filtered standard and filtered blank are analyzed if any samples are filtered prior to analysis.  The correlation coefficient for a standard curve should be 0.995 or greater and the recovery for each calibrant above the practical quantitation limit (PQL) should be 90 -110%.  These recoveries verify the working calibration range with every calibration.

Any time an instrument takes a replicate reading on a sample or if the sample is analyzed more than one time, the precision needs to be checked.  The relative percent difference (RPD) for two or more data points is measured either by the instrument or analyst.  When the average value of the concentration is above the PQL then the RPD must be less than 20% in order to be acceptable.

- QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP Supported Parameter

0	Data Passed Initial QAQC Checks

1	Suspect Data

4	Historical Data:  Pre-Auto QAQC

5	Corrected Data

- QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data suspect due to QA/QC checks

GSM	See metadata

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Calculated value could not be determined due to a below MDL component

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N		from the north

NNE	from the north northeast

NE		from the northeast

ENE	from the east northeast

E		from the east

ESE	from the east southeast

SE		from the southeast

SSE		from the south southeast

S		from the south

SSW	from the south southwest

SW		from the southwest

WSW	from the west southwest

W		from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

- Other remarks/notes

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note 1: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

Note 2: The following NO2 samples collected at Pellicer Creek station have a MDL of 0.003 due to the method the FDEP Laboratory uses for determining limits.  All of these samples are flagged/coded as out of sensor range low (-4) in the data file.  For additional MDL information see the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.

| January 19, 2015 | January 20, 2015 |
| --- | --- |
| 14:30, 17:00, 19:30 | 00:30, 03:00, 05:30, 08:00 |
| April 14, 2015 | April 15, 2015 |
| 11:00, 13:30, 16:00, 18:30, 21:00, 23:30 | 02:00, 04:30, 07:00 |
| September 7, 2015 | September 8, 2015 |
| 13:30 | 02:00, 08:48, 09:10, 09:30 |

Sample Holding Times

The FDEP Laboratory follows the EPA preservation and holding times (published in 40 CFR Part 136.3).  NO2, NO3, and PO4 are preserved at 4°C (wet ice) and have holding times of 48 hours.  Samples that are shipped overnight from the field on the day of collection are almost always received in time for analysis without concerns for expirations.  Diel samples are processed immediately upon receiving due to the expiring hold times.  The first diel sample is almost always received beyond its sample hold time for NO2, NO3 and PO4.

  - See Metadata (CSM)

January 2015

FM

- Suspect TSS data collected on 01/20/2015 12:42; laboratory indicated that the values reported are the mean of two or more determinations.

PC

- Suspect NO2 data collected on 01/20/2015 09:47; laboratory indicated that the results did not meet established quality control criteria.

- Suspect TSS data collected on 01/20/2015 10:09; laboratory indicated that the values reported are the mean of two or more determinations.

- Suspect CHLA and UncCHLa data collected on 01/20/2015 09:47; laboratory indicated that the values reported are the mean of two or more determinations.

- Suspect TKN data collected on 01/20/2015 10:09; laboratory indicated that the value reported was estimated.

February 2015

PC

- Suspect NO2 data collected on 02/16/2015 14:30; laboratory indicated that the value reported was estimated.

- Suspect CHLA data collected on 02/16/2015 17:00; laboratory indicated that the value reported is the mean of two or more determinations.

March 2015

FM

- Suspect CHLA, UncCHLa, and TSS data collected on 03/17/2015 07:53; laboratory indicated that the values reported are the mean of two or more determinations.

- Suspect PHEA data collected on 03/17/2015 07:53; laboratory indicated that the values reported are the mean of two or more determinations and indicated that the results did not meet established quality control criteria.

PC

- Suspect NO2 data collected on 03/17/2015 07:00; laboratory indicated that the results did not meet established quality control criteria.

April 2015

PI

- Suspect TSS data collected on 04/15/2015 08:05; laboratory indicated that the value reported is the mean of two or more determinations.

PC

- Suspect TSS data collected on 04/15/2015 08:00; laboratory indicated that the value reported is the mean of two or more determinations.

May 2015

SS

- Suspect TSS data collected on 05/14/2015 09:25; laboratory indicated that the value reported is the mean of two or more determinations.

FM

- Suspect TSS data collected on 05/14/2015 10:22; laboratory indicated that the value reported is the mean of two or more determinations.

PC

- Suspect CHLA, UncCHLa, and PHEA data collected on 05/13/2015 08:30; laboratory indicated that the values reported are the mean of two or more determinations and the sample was held beyond normal holding time.

June 2015

PI

- Suspect TSS data collected on 06/30/2015 09:00; laboratory indicated that the value reported is the mean of two or more determinations.

PC

- Suspect CHLA, UncCHLa and PHEA data collected on 06/29/2015 08:30; laboratory indicated that the values reported are the mean of two or more determinations and the sample was held beyond normal hold time.

- Suspect TSS data collected on 06/30/2015 09:10; laboratory indicated that the value reported is the mean of two or more determinations.

July 2015

FM

- Suspect TSS data collected on 07/14/2015 11:45; laboratory indicated that the value reported is the mean of two or more determinations.

PC

- Suspect CHLA, UncCHLa and PHEA data collected on 07/14/2015 09:30; laboratory indicated that the values reported are the mean of two or more determinations.

August 2015

SS

- Suspect TSS data collected on 08/11/2015 10:04; laboratory indicated that the value reported is the mean of two or more determinations.

PC

- Suspect CHLA, UncCHLa and PHEA data collected on 08/11/2015 09:30; laboratory indicated that the values reported are the mean of two or more determinations.

September 2015

PC

- Suspect CHLA, UncCHLa and PHEA data collected on 09/07/2015 13:30, 16:00, 18:30, 21:00, 23:30; laboratory indicated that the values reported are estimated.

- Suspect CHLA, UncCHLa and PHEA data collected on 09/08/2015 02:00, 04:30, 07:00, 08:48, 09:10, and 09:30; laboratory indicated that the values reported are estimated.

- Suspect TSS data collected on 09/08/2015 09:10; laboratory indicated that the values reported are estimated.

October 2015

SS

- Suspect TSS data collected on 10/08/2015 09:05; laboratory indicated that the value reported is the mean of two or more determinations.

PC

- Suspect UncCHLa and PHEA data collected on 10/08/2015 09:30; laboratory indicated that the value reported is the mean of two or more determinations.

November 2015

PC

      - Suspect CHLA, UncCHLa and PHEA data collected on 11/09/2015 08:00; laboratory indicated that the values reported are the mean of two or more determinations and are estimated.

      - Suspect TSS data collected on 11/09/2015 09:57; laboratory indicated that the values reported are the mean of two or more determinations.

December 2015

SS

- Suspect CHLA, UncCHLa and PHEA data collected on 12/08/2015 10:15; laboratory indicated that the values reported are the mean of two or more determinations and are estimated.

  - Daily, Monthly, and Annual Precipitation Totals (from raw data) at Pellicer Creek Weather Station (gtmpcmet).

| Date | Daily Precip. Totals (mm) |
| --- | --- |
| 01/04/2015 | 0.254 |
| 01/11/2015 | 23.622 |
| 01/12/2015 | 62.484 |
| 01/13/2015 | 0.254 |
| 01/15/2015 | 9.398 |
| 01/16/2015 | 1.524 |
| 01/23/2015 | 30.226 |
| 01/24/2015 | 11.938 |
| 01/26/2015 | 3.810 |
| January Monthly Total | 143.510 |
| 02/02/2015 | 1.016 |
| 02/04/2015 | 1.524 |
| 02/05/2015 | 21.590 |
| 02/09/2015 | 1.270 |
| 02/17/2015 | 29.718 |
| 02/18/2015 | 3.556 |
| 02/23/2015 | 2.286 |
| 02/24/2015 | 0.508 |
| 02/26/2015 | 5.080 |
| 02/28/2015 | 5.334 |
| February Monthly Total | 71.628 |
| 03/01/2015 | 1.524 |
| 03/05/2015 | 0.254 |
| 03/06/2015 | 3.302 |
| 03/12/2015 | 15.240 |
| 03/23/2015 | 19.304 |
| 03/26/2015 | 2.032 |
| 03/27/2015 | 3.556 |
| March Monthly Total | 45.212 |
| 04/04/2015 | 29.972 |
| 04/05/2015 | 0.254 |
| 04/11/2015 | 0.254 |
| 04/12/2015 | 7.366 |
| 04/13/2015 | 16.764 |
| 04/14/2015 | 0.254 |
| 04/15/2015 | 55.880 |
| 04/16/2015 | 0.254 |
| 04/19/2015 | 8.636 |
| 04/20/2015 | 19.812 |
| 04/25/2015 | 0.508 |
| 04/27/2015 | 0.762 |
| 04/28/2015 | 0.254 |
| 04/29/2015 | 6.096 |
| April Monthly Total | 147.320 |
| 05/04/2015 | 1.016 |
| 05/05/2015 | 0.254 |
| 05/06/2015 | 1.016 |
| 05/20/2015 | 12.446 |
| 05/25/2015 | 2.794 |
| 05/30/2015 | 0.762 |
| May Monthly Total | 18.288 |
| 06/01/2015 | 0.508 |
| 06/02/2015 | 1.524 |
| 06/03/2015 | 0.254 |
| 06/09/2015 | 6.096 |
| 06/10/2015 | 6.096 |
| 06/11/2015 | 8.636 |
| 06/12/2015 | 0.254 |
| 06/23/2015 | 4.826 |
| 06/24/2015 | 20.320 |
| 06/25/2015 | 5.842 |
| 06/26/2015 | 1.778 |
| 06/28/2015 | 9.652 |
| 06/29/2015 | 0.254 |
| 06/30/2015 | 1.778 |
| June Monthly Total | 146.558 |
| 07/02/2015 | 12.446 |
| 07/05/2015 | 35.306 |
| 07/06/2015 | 0.508 |
| 07/13/2015 | 19.304 |
| 07/14/2015 | 2.540 |
| 07/16/2015 | 18.796 |
| 07/17/2015 | 2.032 |
| 07/18/2015 | 10.160 |
| 07/20/2015 | 16.002 |
| 07/21/2015 | 9.140 |
| 07/22/2015 | 2.286 |
| 07/24/2015 | 21.082 |
| 07/25/2015 | 36.830 |
| 07/26/2015 | 1.016 |
| 07/28/2015 | 0.508 |
| July Monthly Total | 187.960 |
| 08/01/2015 | 6.096 |
| 08/03/2015 | 12.954 |
| 08/07/2015 | 0.508 |
| 08/12/2015 | 12.954 |
| 08/13/2015 | 25.146 |
| 08/15/2015 | 4.318 |
| 08/16/2015 | 2.794 |
| 08/17/2015 | 1.524 |
| 08/18/2015 | 2.032 |
| 08/19/2015 | 21.082 |
| 08/20/2015 | 3.810 |
| 08/21/2015 | 7.112 |
| 08/23/2015 | 42.164 |
| 08/24/2015 | 0.508 |
| 08/26/2015 | 11.176 |
| 08/28/2015 | 0.254 |
| 08/29/2015 | 8.636 |
| 08/30/2015 | 13.970 |
| August Monthly Total | 177.292 |
| 09/02/2015 | 24.638 |
| 09/07/2015 | 7.366 |
| 09/08/2015 | 1.270 |
| 09/12/2015 | 22.860 |
| 09/13/2015 | 6.096 |
| 09/15/2015 | 15.240 |
| 09/16/2015 | 26.162 |
| 09/17/2015 | 3.556 |
| 09/18/2015 | 0.254 |
| 09/19/2015 | 7.366 |
| 09/23/2015 | 0.762 |
| 09/24/2015 | 3.048 |
| 09/28/2015 | 7.112 |
| 09/29/2015 | 0.254 |
| September Monthly Total | 126.492 |
| 10/01/2015 | 4.064 |
| 10/02/2015 | 6.350 |
| 10/06/2015 | 0.508 |
| 10/10/2015 | 0.254 |
| 10/19/2015 | 0.508 |
| 10/21/2015 | 8.128 |
| 10/27/2015 | 2.540 |
| 10/28/2015 | 5.334 |
| October Total | 27.686 |
| 11/04/2015 | 1.016 |
| 11/08/2015 | 1.270 |
| 11/09/2015 | 0.254 |
| 11/10/2015 | 0.254 |
| 11/17/2015 | 0.762 |
| 11/18/2015 | 1.524 |
| 11/19/2015 | 2.540 |
| 11/21/2015 | 4.826 |
| 11/22/2015 | 0.254 |
| 11/25/2015 | 3.556 |
| 11/26/2015 | 3.302 |
| 11/26/2015 | 3.302 |
| 11/29/2015 | 2.032 |
| 11/30/2015 | 0.254 |
| November Monthly Total | 21.844 |
| 12/03/2015 | 0.762 |
| 12/04/2015 | 3.048 |
| 12/05/2015 | 8.890 |
| 12/06/2015 | 0.254 |
| 12/13/2015 | 3.302 |
| 12/14/2015 | 1.016 |
| 12/15/2015 | 1.016 |
| 12/16/2015 | 0.508 |
| 12/18/2015 | 7.112 |
| 12/25/2015 | 0.254 |
| 12/27/2015 | 5.334 |
| 12/28/2015 | 0.254 |
| December Monthly Total | 31.750 |
| January – December Total | 1145.540 |