Kachemak Bay (KAC) NERR Meteorological Metadata

January 2008 – December 2008

Latest update December 2, 2011

I. Data Set and Research Descriptors

- Principal investigator(s) and contact persons

Contact Persons:

Joel Markis, joel.markis@alaska.gov

Angela Doroff angela.doroff@alaska.gov

Oriana Badajos oriana.badajos@alaska.gov

Address:

Kachemak Bay Research Reserve

95 Sterling Hwy, Suite 2

Homer, AK 99603

Phone: (907)-226-4658

Homepage: http://www.kbayrr.org

- Entry verification

Data are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible) which is connected to the CR1000 via a telephone modem.  Files are exported from LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

Joel Markis processed and error checked the data.

- Research objectives

Kachemak Bay is a glacially influenced estuarine fjord with mountains on the southern side and a large bluff on the northern side.  The topography tends to channel the winds along the axis of the Bay.  The topography is also believed to be responsible for a strong curl in the wind field.  The winds measured at the Homer and Seldovia airports are much lower than those observed in the center of the Bay.  By placing the meteorological station at the end of the Homer Spit, which extends to the center of the Bay, we hope to be able to provide a measure of the wind curl.  Our objectives are to 1) develop a better understanding of topographic effects on meteorological variables, 2) provide an atmospheric forcing context for the interpretation of System Wide Monitoring Program water quality data, and 3) detect long-term trends and variability in the local climate.

- Research methods

The Campbell Scientific weather station samples every 5 seconds continuously throughout the year.  These data are used by the CR1000 to produce 15 minute averages of air temperature, relative humidity, barometric pressure, rainfall, wind speed, wind direction, PAR, and downwelling irradiance.  The raw data are stored by the CR1000 and a SM4 storage module.  The data are checked weekly on a PC which is connected directly to the CR1000 via a modem to ensure proper operation.

The meteorological station is serviced monthly and the data are checked against a Kestrel 4000 hand-held meteorological sensor.  We have not found evidence of debris in the rain gauge or birds fouling other sensors.

Campbell Scientific data telemetry equipment was installed at the Homer station on 11/2005 and transmits data to the NOAA GOES satellite, NESDIS ID # 3B00140C MET.  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data becomes part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts)

Maximum, Minimum, and their times from 5-second data:

Air Temperature (°C) (these data are not included in the dataset, but are available from the KAC NERR), Wind Direction Standard Deviation (degrees) from 5-second data

Maximum and time from 5-second data:

Wind Speed (m/s)

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm) (began collecting as a standard parameter on 6/12/2008)

- Site location and character

Kachemak Bay is located approximately 200 kilometers south of Anchorage on the western shore of the Kenai Peninsula.  Kachemak Bay, at 59.6º N and 151.5º W, is a temperate regional fjord with hydrographic conditions unique among the NERR system estuaries.  The tidal range of 8-meters is among the largest in the world, and salinity ranges from near zero at stream mouths to 33.0 PSU at the entrance to the inner Bay.  The bay is 35 kilometers wide at its mouth and approximately 57 kilometers long.   The head of Kachemak Bay is located to the northeast at the Fox River Flats, and the mouth lies to the southwest, along a line between Anchor Point and Point Pogibshi. The 6-kilometer long Homer Spit that extends into the Bay from the northern shoreline splits Kachemak Bay into inner and outer bays. The Kachemak Bay NERR encompasses both the inner and outer bays.  Water flows between the inner and outer Bays through a narrow opening formed between the Spit and the southern shoreline. The Bay has an average depth of 45-meters, and a maximum of 200-meters.  Fresh water introduced primarily by the Fox, Bradley, and Martin Rivers and Sheep Creek at the head of the Bay, flows along the northwest shore of the inner Bay.

The southern shore of the bay is characterized by the presence of steep mountains and deep fjords.  An ice cap remains on the top of the mountain range feeding the several large glaciers that flow into the bay.  The northern shore is characterized by a lower bluff (~300-meters) with a gently-sloping bench below.  The mountains and bluff tend to funnel the winds along the axis of the bay.  They may also be a source of wind curl.  During the summer, the presence of the ice cap to the south and a warm body of land to the north causes strong day breezes to form.

The meteorological station is mounted on top of the Land’s End Resort (59.60091º N 151.40961º W), approximately 150-meters southwest of the Homer water quality monitoring location.   The station is located at the northeast corner of the two-story Land’s End Resort and is approximately 12m above the ground and 19m above mlw.  The sensors are mounted at various heights on a 3-meter mast.  The mast is mounted on a white colored base that sits on a flat, tar-covered roof.  The anemometer is mounted at the top of the mast (4m above the base), the two light sensors are on a south-facing arm (3m above the base), and the temperature and relative humidity sensors are two meters off the base.  The barometric pressure sensor is located 1.5 meters off of the base inside the CR1000 housing and uses an external air vent. The rain gauge is mounted by itself two meters to the south of the station and is approximately 0.5 meters high. The placement of the tower is not optimal for meteorological measurements, but was determined to be the best location to answer the research questions and provide a representative measurement of the wind field near the Homer harbor.

- Data collection period

The CR1000 ran continuously from January 1, 2008 00:00 to December 31, 2008 23:45.

- Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR weather data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in comma separated format.

- Associated researchers and projects

The Kachemak Bay Research Reserve assists Glenn Shaw of the University of Alaska Fairbanks in monitoring atmospheric particles.  KBRR staff change filters weekly and send them to a NOAA lab for analysis.  By combining the particle characteristics and meteorological conditions it may be possible to identify sources of particles, such as Asiatic dust.

Additionally, Kachemak Bay Research Reserve conducts water quality and nutrient monitoring as part of the System-Wide Monitoring Program of which the meteorological data addressed herein are a part.

II. Physical Structure Descriptors

- Sensor specifications

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #:  HMP45ASP Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Date of last calibration: SN Z3810011 04/14/2007

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #:  HMP45ASP Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Date of last calibration: SN Z3810011 04/14/2007

Parameter: Barometric Sensor

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Date of last calibration: SN Z5120018 04/15/2007

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (130 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 2%

Date of last calibration: SN WM65229 04/26/2007

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Model #: R.M. Young 05103 Wind Monitor

Range:  360° mechanical, 355° electrical (5° open)

Accuracy: +/- 5%

Date of last calibration: SN WM65229 01/26/2005

Parameter: LI-COR Quantum Sensor

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SB

Light spectrum waveband:  400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity: 0 to 100%

Sensitivity:  typically 5 µA per 1000 µmoles s-1 m-2

Date of last calibration:  SN Q34395 11/22/2004

Multiplier: 1.3330347

Parameter: Pyranometer

Units: mA per 1000 W m-2

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI200SZ

Light spectrum waveband: 400 to 1100 nm

Temperature dependence: 0.15% per °C

Stability: :  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity: 0 to 100%

Sensitivity:  0.2 kW m-2 mV-1

Date of last calibration:  SN PY43295 05/16/2002

Parameter: Precipitation

Units: millimeters (mm)

Sensor type: Heated Tipping Bucket Rain Gauge

Model #:  MetOne 380

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to +/- 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Date of last calibration: 05/05/2008

Storage Module

Model #: SM4M

Storage capacity: 2 million low-resolution data values

Program storage: stores up to 8 programs with a total capacity of 128 KB

Processor: Hitachi H8S

Operating system: 64 KB, flash memory based, user downloadable

Operating range: Temperature: -35° to +65°C

Baud rates: 9600, 76800

Memory type: user selectable for either ring style (default) or fill and drop.

Power requirements: 5 +/-0.3 VDC @ 100 mA

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Date CR1000 installed: January 12, 2007

- Coded variable definitions

Sampling station: 		Homer, Land’s End Resort

Sampling site code: 		HO

Site code:			kachomet

- QAQC flag definitions – This section details the automated primary and secondary QAQC flag definitions.

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		 Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

- QAQC code definitions – This section details the secondary QAQC Code definitions used in combination with the QAQC flags above.

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SIC	Incorrect Calibration Constant, Multiplier or Offset

SNV	Negative Value

SOC	Out of Calibration

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR	Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CDF	Data Appear to Fit Conditions

CRE	Significant Rain Event

CSM	See Metadata

CVT	Possible Vandalism/Tampering

- Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator, Angela Doroff.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Cumulative precipitation data are recorded from 00:00 to 23:59 with the daily total recorded at the midnight mark (00:00). The midnight CumPrcp value is actually the total from the previous day.

All Reserves were required to align their wind directions sensors to True North by April 1, 2008.  KAC Reserve’s wind direction has been aligned to True North since 2003.

From 1/12/2007- 4/14/2010 PAR data were collected but the wrong multiplier was used.  The data have therefore been rejected.

On 4/21/2008 at 22:15 and 5/10/2008 at 10:00  random data are all missing for unknown reasons.  All other data were collected and appear to be good.

Telemetered data were used to fill in gaps of missing data on a few occasions in September and October.  Those data are coded as GIT.

The Homer MET station fell over in a high wind event on 5/23/08 at 12:45. The instruments continued to function with the exception of wind direction, speed and the PAR sensor, which showed elevated readings due to being out of level with respect to the horizon. The Homer MET station quit working completely on 6/8/08 at 12:15. On 6/10/08 at 13:15, the station was righted and resumed data collection. Wind speed and direction instruments were not fixed until 7/8/08 at 13:15, and the PAR sensor was slightly out of level for the remainder of the year.

Abnormally high precipitation information was noticed during several high wind events. It appears that high winds are capable of tipping the rain gage resulting in inaccurate precipitation data. Efforts were made to cross reference these data with the nearby Homer Airport precipitation information and flag suspect and erroneous data; however, use of this information should be done with caution.