Kachemak Bay (KAC) NERR Meteorological Metadata

January 2012-December 2012

Latest update: 10/29/2015

I. Data Set and Research Descriptors

- Principal investigator(s) and contact persons

Contact Persons:

Oriana Badajos; Fishery Biologist I; oriana.badajos@alaska.gov

Angela Doroff ; Research Coordinator; angela.doroff@alaska.gov

Address:

Kachemak Bay Research Reserve

95 Sterling Hwy, Suite 2

Homer, AK 99603

Phone: (907) 226-4658

Homepage: http://www.adfg.alaska.gov/index.cfm?adfg=kbrr.home

- Entry verification

Data are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from or LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.  Oriana Badajos processed and error checked the data.

- Research objectives

Kachemak Bay is a glacially influenced estuarine fjord with mountains on the southern side and a large bluff on the northern side.  The topography tends to channel the winds along the axis of the Bay.  The topography is also believed to be responsible for a strong curl in the wind field.  The winds measured at the Homer and Seldovia airports are much lower than those observed in the center of the Bay.  By placing the meteorological station at the end of the Homer Spit, which extends to the center of the Bay, we hope to be able to provide a measure of the wind curl.  Our objectives are to 1) develop a better understanding of topographic effects on meteorological variables, 2) provide an atmospheric forcing context for the interpretation of System Wide Monitoring Program water quality data, and 3) detect long-term trends and variability in the local climate.

- Research methods

The Campbell Scientific weather station samples every 5 seconds continuously throughout the year.  These data are used by the CR1000 to produce 15-minute averages of air temperature, relative humidity, barometric pressure, wind speed, wind direction, and total solar radiation.  Precipitation and PAR are cumulative values in a 15-minute sampling interval.  The raw data are stored by the CR1000 and an nl-115 compact flash storage module.  The data are checked weekly on a PC which is connected directly to the CR1000 via a modem to ensure proper operation.

The meteorological station is serviced monthly and the data are checked against a Kestrel 4500 hand-held meteorological sensor.  We have not found evidence of debris in the rain gauge or birds fouling other sensors.  As recommended by the manufacturer, instruments are calibrated annually (Temperature/Humidity; Barometric Pressure; Precipitation Gauge) or biannually (Wind monitor; Pyranometer)

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- yearly recalibration

- Rain Gauge- yearly recalibration

- Wind Speed/Direction- yearly or every 2 years (depending on the sensor)

- Barometric Pressure- every 2 years recalibration

- PAR- every 2 years recalibration

-Total Solar Radiation - , LI-COR recommends every 2 years with recalibrations performed during the spring and summer

Campbell Scientific data telemetry equipment was installed at the Homer station on 11/2005 and transmits data to the NOAA GOES satellite, NESDIS ID # 3B00140C MET.  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts), Total Solar Radiation (W/m²)

Maximum and Minimum, and their times from 5-second data:

Air Temperature (°C) (these data are not in the dataset but are available from the KAC NERR)

Maximum and time from 5-second data:

Wind Speed (m/s)

Wind Direction Standard Deviation (degrees) from 5-second data

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm) (began collecting as a standard parameter on 6/12/2008)

- Site location and character

Kachemak Bay is located approximately 200 kilometers south of Anchorage on the western shore of the Kenai Peninsula.  Kachemak Bay, at 59.6º N and 151.5º W, is a temperate regional fjord with hydrographic conditions unique among the NERR system estuaries.  The tidal range of 8-meters is among the largest in the world, and salinity ranges from near zero at stream mouths to 33.0 PSU at the entrance to the inner Bay.  The bay is 35 kilometers wide at its mouth and approximately 57 kilometers long.   The head of Kachemak Bay is located to the northeast at the Fox River Flats, and the mouth lies to the southwest, along a line between Anchor Point and Point Pogibshi. The 6-kilometer long Homer Spit extends into the Bay from the northern shoreline and splits Kachemak Bay into inner and outer bays. The Kachemak Bay NERR encompasses both the inner and outer bays.  Water flows between the inner and outer Bays through a narrow opening formed between the Spit and the southern shoreline. The Bay has an average depth of 45-meters, and a maximum of 200-meters.  Fresh water introduced primarily by the Fox, Bradley, and Martin Rivers and Sheep Creek at the head of the Bay, flows along the northwest shore of the inner Bay.

The southern shore of the bay is characterized by the presence of steep mountains and deep fjords.  An ice cap remains on the top of the mountain range feeding the several large glaciers that flow into the bay.  The northern shore is characterized by a lower bluff (~300-meters) with a gently-sloping bench below.  The mountains and bluff tend to funnel the winds along the axis of the bay.  They may also be a source of wind curl.  During the summer, the presence of the ice cap to the south and a warm body of land to the north causes strong day breezes to form.

A meteorological station is mounted on top of the Land’s End Resort (59.60093º N 151.40949º W), approximately 150-meters southwest of the Homer water quality monitoring location.   The station is located at the northeast corner of the two-story Land’s End Resort and is approximately 12-meters above the ground and 19-meters above mean low water.  There are no wind blocks and the tower is not shaded or obstructed.  The sensors are mounted at various heights on a 5-meter tower.  The tower is mounted on a white colored base that sits on a flat, tar-covered roof.  The anemometer is mounted at the top of the tower 5-meters above the base; the two light sensors (PAR and TSR) are on a south-facing arm 4-meters above the base; and the temperature and relative humidity sensors are 3-meters off the base.  The barometric pressure sensor is located 1.5 meters off of the base inside the CR1000 housing and uses an external air vent. The rain gauge is mounted by itself two meters to the south of the station and is approximately 0.5 meters high.  The placement of the tower is not optimal for meteorological measurements, but was determined to be the best location to answer the research questions and provide a representative measurement of the wind field near the Homer harbor.

- Data collection period

The CR1000 ran continuously from January 1, 2012 00:00 to December 31, 2012 23:45.

- Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2012.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

- Associated researchers and projects

The Kachemak Bay Research Reserve assists Glenn Shaw of the University of Alaska Fairbanks in monitoring atmospheric particles.  By combining the particle characteristics and meteorological conditions it may be possible to identify sources of particles, such as Asiatic dust or volcanic ash.

Additionally, Kachemak Bay Research Reserve conducts water quality and nutrient monitoring as part of the System-Wide Monitoring Program of which the meteorological data addressed herein are a part.

An additional GOES telemetered meteorological station is maintained in Anchor Point. This station (NESDIS ID # 3B04C75E) is intended to improve weather forecasting and prediction on the Southern Kenai Peninsula as well as act as an aid to mariners in making safe decisions when traveling on the water in Lower Cook Inlet.

II. Physical Structure Descriptors

- Sensor specifications

| Parameter | Units | Sensor Type | Model | Range | Accuracy | Other | Sensor | Deploy Dates | Cal Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | Celsius | Platinum resistance temperature detector (PRT) | HMP45ASP Temperature and Relative Humidity Probe | -40°C to +60°C | ± 0.2 °C @ 20°C |  | Z4020045 | 9/28/11-10/16/12 | 7/29/11 |
|  |  |  |  |  |  |  | Z3810011 | 10/16/12-current as of 12/31/12 | 1/17/12 |
| Relative Humidity | Percent | Vaisala HUMICAP© 180 capacitive relative humidity sensor | HMP45ASP Temperature and Relative Humidity Probe | 0-100% non-condensing | Accuracy at 20°C: +/- 2% RH (0-90%) and +/- 3% (90-100%) | Temperature dependence of RH measurement: +/- 0.05% RH/°C | (See temp probe) |  |  |
| Barometric Sensor | millibars (mb) | Vaisala Barocap © silicon capacitive pressure sensor | CS-105 | Pressure: 600 to 1060 mb; Temperature: -40°C to +60°C; Humidity: non-condensing | ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C | Stability: ± 0.1 mb per year | Z5120018 | 9/28/11-10/16/12 | 8/11/11 |
|  |  |  |  |  |  |  | X3740005 | 10/16/12- current as of 12/31/12 | 9/25/12 |
| Wind direction | degrees | balanced vane, 38 cm turning radius | R.M. Young 05103 Wind Monitor | 360° mechanical, 355° electrical (5° open) | +/- 5% |  | WM84420 | 9/28/2011- current as of 12/31/12 | 7/29/11 |
| Wind Speed | meter per second (m/s) | 18 cm diameter 4-blade helicoids propeller molded of polypropylene | R.M. Young 05103 Wind Monitor | 0-60 m/s (130 mph); gust survival 100 m/s (220 mph) | +/- 2% |  | (See wind dir) |  |  |
| Precipitation | Milli-meters (mm) | Heated Tipping Bucket Rain Gauge | MetOne 380 | Temperature: 0° to +/- 50°C; Humidity: 0 to 100% | +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr | Rainfall per tip: 0.01 inch | B4708 | 8/16/02- current as of 12/31/12 | 9/28/11 |

| Parameter | Units | Sensor Type | Model | Range | Temp Dependence | Sensitivity | Stability | Sensor | Deploy Dates | Cal Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LI-COR Quantum Sensor | mmoles m-2 (total flux) | High stability silicon photovoltaic detector (blue enhanced) | LI190SB | Light spectrum waveband: 400 to 700 nm; Operating Temperature: -40°C to 65°C; Humidity: 0 to 100% | 0.15% per °C | typically 5 µA per 1000 µmoles s-1 m-2 | <±2% change over 1 yr | Q30035 Multiplier= 1.19973 | 4/14/10-10/16/12 | 10/26/09 |
|  |  |  |  |  |  |  |  | Q34395 Multiplier= 1.32028 | 10/16/12- current as of 12/31/12 | 9/19/12 |
| Pyranometer | mA per 1000 W m-2 | High stability silicon photovoltaic detector (blue enhanced) | LI200SZ | Light spectrum waveband: 400 to 1100 nm; Operating Temperature: -40°C to 65°C; Humidity: 0 to 100% | 0.15% per °C | 0.2 kW m-2 mV-1 | <±2% change over 1 yr | PY67826 Multiplier= 18.9654 | 8/20/10- current as of 12/31/12 | 10/28/09 |

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Date CR1000 Installed: 1/12/2007

CR1000 Firmware Version (s):  Version 2.0 as of 7/22/2010

CR1000 Program Version(s):  KACHOMET_7.1_021411, 10/22/2012:  KACHOMET_7.2_101612

The CR1000 is equipped with a nl115 Ethernet and compact flash module which provides two independent capabilities (1) 10baseT Ethernet communication and (2) stores data on a removable compact flash card.

- Coded variable definitions

| Sampling station | Sampling site code | Station code |
| --- | --- | --- |
| Homer (Land’s End Resort) | HO | kachomet |
| Anchor Point (Halibut Campground) | AP | kacapmet |

- QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		 Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

- QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC	Incorrect Calibration Constant, Multiplier or Offset

SIW	Incorrect Wiring

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor Drift

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR		Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CCU	Cause Unknown

CDF	Data Appear to Fit Conditions

CML	Snow melt from previous snowfall event

CRE*	Significant Rain Event

CSM*	See Metadata

CVT*	Possible Vandalism/Tampering

CWE*	Significant weather event

- Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR  values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

Slightly elevated nighttime PAR data are flagged as suspect <1> CSM.  The reason for elevated nighttime readings is unknown, however, there is some concern that moisture intrusion into the PAR sensor may cause elevated nighttime readings (>0).

Slightly elevated nighttime TSR readings are flagged as suspect <1> CSM.  The reasons for the elevated nighttime readings (>0) are unknown.

On 2/27/2012, we discovered that the funnel to the rain gauge had blown away.  All data after this date have been rejected until a new funnel was installed on 3/28/2012.  We compared data to our other rain gauge station and decided to also reject data from 2/1/2012-2/27/2012 as this is when elevated and unlikely rainfall was reported, indicating that the funnel blew away during a period of high winds.  With the funnel gone, the tipping bucket was unprotected during the high winds and it was tipped frequently by the wind between 2/1/2012-3/28/2012.  After the funnel was re-installed, the tipping bucket became jammed and did not tip freely.  This problem was not fixed until 6/29/2012.  All data between 2/1/2012 10:45and 6/29/2012 9:15 (through the midnight time stamp 06/30/2012 00:00 for cumulative precipitation) have been rejected.

7/18/2012 11:45 Total precipitation data were corrected from 0.3 to 0.0.  The gauge was tipped one time for testing, so no precipitation occurred.  Cumulative precipitation data were corrected from 7/18/2012 11:45 through the midnight timestamp at 7/19/2012 00:00.

On 10/17/2012, we deployed new barometric pressure, temperature, and PAR sensors. All data were rejected at 10:00 and 10:30 and were missing at 10:15.  The updated program with the new PAR multiplier was not uploaded until 10/22/12.  All PAR data have been rejected during this time from 10/17/12 10:45 - 10/22/12 10:45.  We also cleaned the precipitation gauge and failed at an attempt to calibrate it, so all recorded total precipitation was corrected to 0.0 during the maintenance period of 10/17/12 11:15-11:45.  Cumulative precipitation data were corrected from 10/17/2012 11:15 through the midnight timestamp at 10/18/2012 00:00.

All data were rejected on 10/22/2012 11:00 due to a program reload to update the PAR multiplier.