Kachemak Bay (KAC) NERR Meteorological Metadata

January-December 2014

Latest update: 09/28/2016

I. Data Set and Research Descriptors

- Principal investigator(s) and contact persons

Contact Persons:

Oriana Badajos; Fishery Biologist I; oriana.badajos@alaska.gov

Steve Baird; Research Analyst II; sjbaird@alaska.edu

Angela Doroff ; Research Coordinator; adoroff@alaska.edu

Address:

Kachemak Bay Research Reserve

2181 Kachemak Dr.

Homer, AK 99603

Tel: 907-235-4799

Fax: 907-235-4794

http://accs.uaa.alaska.edu/kbnerr

- Entry verification

Data are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from or LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12. Oriana Badajos processed and error checked the data (Steve Baird for fourth quarter).

- Research objectives

Kachemak Bay is a glacially influenced estuarine fjord with mountains on the southern side and a large bluff on the northern side. Differences in wind speed and direction between the meteorological station at the Homer airport, and winds experienced in the middle of the Bay and on the Homer Spit have been noted. By placing one meteorological station at the end of the Homer Spit, which extends to the center of the Bay, the original intent was to be able to provide a measure of the wind stress or curl on the sea surface.

The regions around Anchor Point have long been a data gap for weather forecasters, and often represent a transition point between the Kenai/Soldotna inland weather and the Homer maritime weather. By placing an additional meteorological station in Anchor Point, the Reserve aims to improve weather forecasting capabilities on the southern Kenai Peninsula, as well as provide more accurate information to mariners when making decisions about traveling in lower Cook Inlet.

The objectives of the meteorological program are to: 1) develop a better understanding of the topographic effects on meteorological variables in this region; 2) help improve forecasting capabilities on the southern Kenai Peninsula; 3) improve the quantity and quality of information available to mariners when making decisions about traveling on Kachemak Bay and lower Cook Inlet; and 4) detect long-term trends and variability in the local climate.

- Research methods

Each Campbell Scientific weather station samples every 5 seconds continuously throughout the year.  These data are used by the CR1000 to produce 15-minute averages of air temperature, relative humidity, barometric pressure, wind speed, wind direction, and total solar radiation (Total solar radiation is only measured at the Homer Spit weather station).  Precipitation and PAR are cumulative values in a 15-minute sampling interval.  The raw data are stored by the CR1000 and an nl-115 compact flash storage module.  For the Homer Spit station, the data are continuously recorded on a computer which is connected directly to the CR1000 via a modem. The Anchor Point data are downloaded monthly to a laptop connected to the CR1000.

Both meteorological stations are serviced monthly and the data are checked against a Kestrel 4500 hand-held meteorological sensor.  We have not found evidence of debris in the rain gauge or birds fouling other sensors.  As recommended by the manufacturer, instruments are calibrated annually (Temperature/Humidity; Barometric Pressure; Precipitation Gauge) or every 2 years (Wind monitor; light sensors)

Campbell Scientific data telemetry equipment was installed at the Homer station in November 2005 and transmits data to the NOAA GOES satellite, NESDIS ID # 3B00140C.  The Anchor Point station was created in October 2009 and transmits data to the NOAA GOES satellite, NESDIS ID #: 3B04C75E. The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  The CDMO does not authenticate the data from the Anchor Point station since it is not a primary site in the NERRS SWMP.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts), Total Solar Radiation (W/m²) (total solar radiation is measured only at the Homer Spit station)

Maximum, Minimum, and their times from 5-second data:

(these data are not included in the dataset, but are available from the KAC NERR): Air Temperature (°C)

Maximum and time from 5-second data:

Wind Speed (m/s)

Wind Direction Standard Deviation (degrees) from 5-second data

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm) (began collecting as a standard parameter on 6/12/2008)

Recommended calibration frequency for the MET station sensors:

| Sensor | Recommended Calibration Frequency |
| --- | --- |
| Temperature/Humidity | Annual |
| Rain Gauge | Annual |
| Wind Speed/Direction | Every 2 years |
| Barometric Pressure | Every 2 years |
| PAR | Every 2 years |
| Total Solar Radiation | Every 2 years |
| CR1000 | Every 5 years (required beginning 2014) |

- Site location and character

Kachemak Bay is located approximately 200 kilometers south of Anchorage on the western shore of the Kenai Peninsula.  Kachemak Bay, at 59.6º N and 151.5º W, is a temperate regional fjord with hydrographic conditions unique among the NERR system estuaries.  The tidal range of 8-meters is among the largest in the world, and salinity ranges from near zero at stream mouths to 33.0 PSU at the entrance to the inner Bay.  The bay is 35 kilometers wide at its mouth and approximately 57 kilometers long.   The head of Kachemak Bay is located to the northeast at the Fox River Flats, and the mouth lies to the southwest, along a line between Anchor Point and Point Pogibshi. The 6-kilometer long Homer Spit extends into the Bay from the northern shoreline and splits Kachemak Bay into inner and outer bays. The Kachemak Bay NERR encompasses both the inner and outer bays.  Water flows between the inner and outer Bays through a narrow opening formed between the Spit and the southern shoreline. The Bay has an average depth of 45-meters, and a maximum of 200-meters.  Fresh water introduced primarily by the Fox, Bradley, and Martin Rivers and Sheep Creek at the head of the Bay, flows along the northwest shore of the inner Bay.

The southern shore of the bay is characterized by the presence of steep mountains and deep fjords.  An ice cap remains on the top of the mountain range feeding the several large glaciers that flow into the bay.  The northern shore is characterized by a lower bluff (~300-meters) with a gently-sloping bench below.  The mountains and bluff tend to funnel the winds along the axis of the bay.  They may also be a source of wind curl.  During the summer, the presence of the ice cap to the south and a warm body of land to the north causes strong day breezes to form.

Homer Spit:

A meteorological station is mounted on top of the Land’s End Resort (59.60093 º N 151.40949 ºW), approximately 150-meters southwest of the Homer water quality monitoring location.   The station is located at the northeast corner of the two-story Land’s End Resort and is approximately 12-meters above the ground and 19-meters above mean low water.  There are no wind blocks and the tower is not shaded or obstructed.  The sensors are mounted at various heights on a 5-meter tower.  The tower is mounted on a white colored base that sits on a flat, tar-covered roof.  The anemometer is mounted at the top of the tower 5-meters above the base; the two light sensors are on a south-facing arm 4-meters above the base; and the temperature and relative humidity sensors are 3-meters off the base. The barometric pressure sensor is located 1.5 meters off of the base inside the CR1000 housing and uses an external air vent. The rain gauge is mounted by itself two meters to the south of the station and is approximately 0.5 meters high. The placement of the tower is not optimal for meteorological measurements, but was determined to be the best location to answer the research questions and provide a representative measurement of the wind field near the Homer harbor.

Anchor Point:

The Anchor Point meteorological station is located at 59.770260°N 151.867560°W in a large grassy field adjacent to the ocean in the Halibut Campground, Anchor Point. This station is located on the westernmost point of land on the Kenai Peninsula overlooking Cook Inlet and there are no wind blocks and the tower is not shaded or obstructed.  The instruments are deployed on a 10-m (30-ft) tower that sits on the ground approximately 5-m above the mean low water. The anemometer is mounted at the top of the tower 10-m above the ground, the PAR sensor is on a south-facing arm 8-m above the ground, the temperature and relative humidity sensors are 3-m off the ground, and the barometric pressure sensor is located inside the enclosure 2-m above the ground. The rain gauge is deployed on a pole adjacent to the station and sits 1-m above the ground.  The Anchor Point meteorological station is located approximately 23 miles northwest of the Homer water quality monitoring and meteorological stations.

- Data collection period

Homer Spit: The CR1000 ran continuously from January 1, 2014 00:00 to December 31, 2014 23:45.

Anchor Point:  The CR1000 ran continuously from January 7, 2014 13:30 to December 31, 2014 23:45.

2014 raw file start and end time for the Homer Spit station:

| File Start Date and Time | File End Date and Time |
| --- | --- |
| 12/05/2013 14:45 | 01/02/2014 09:15 |
| 01/02/2014 09:30 | 02/07/2014 10:45 |
| 02/07/2014 10:45 | 02/27/2014 14:15 |
| 02/27/2014 14:30 | 03/12/2014 08:30 |
| 03/12/2014 08:45 | 04/18/2014 08:30 |
| 04/18/2014 08:45 | 04/24/2014 08:45 |
| 04/24/2014 09:00 | 05/12/2014 09:15 |
| 05/12/2014 09:30 | 06/10/2014 14:45 |
| 06/10/2014 15:00 | 07/07/2014 13:45 |
| 07/07/2014 14:00 | 07/24/2014 13:00 |
| 07/24/2014 13:15 | 07/30/2014 01:15 |
| 07/30/2014 01:30 | 09/02/2014 14:45 |
| 09/02/2014 15:00 | 10/09/2014 14:45 |
| 10/09/2014 15:00 | 11/06/2014 14:30 |
| 11/06/2014 14:45 | 11/19/2014 09:00 |
| 11/19/2014 09:15 | 12/03/2014 12:30 |
| 12/03/2014 12:45 | 01/26/2015 11:15 |

2014 raw file start and end time for the Anchor Point station:

| File Start Date and Time | File End Date and Time |
| --- | --- |
| 01/07/2014 13:30 | 09/05/2014 12:30 |
| 01/07/2014 13:30 | 10/08/2014 15:00 |
| 01/07/2014 13:30 | 11/06/2014 11:15 |
| 11/06/2014 11:30 | 12/03/2014 14:00 |
| 12/03/2014 14:15 | 01/15/2015 14:15 |

- Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2012.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

- Associated researchers and projects

The Kachemak Bay Research Reserve also conducts water quality and nutrient monitoring as part of the System-Wide Monitoring Program.

II. Physical Structure Descriptors

- Sensor specifications

Homer Spit

| Parameter | Units | Sensor Type | Model | Range | Accuracy | Other | Sensor | Deploy Dates | Cal Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | Celsius | Platinum resistance temperature detector (PRT) | HMP45ASP Temperature and Relative Humidity Probe | -40°C to +60°C | ± 0.2 °C @ 20°C |  | B2640014 | 6/26/13-6/10/14 | 5/22/2013 |
|  |  |  |  |  |  |  | F3630043 | 6/10/14-current | 5/19/2014 |
| Relative Humidity | Percent | Vaisala HUMICAP© 180 capacitive relative humidity sensor | HMP45ASP Temperature and Relative Humidity Probe | 0-100% non-condensing | Accuracy at 20°C: +/- 2% RH (0-90%) and +/- 3% (90-100%) | Temperature dependence of RH measurement: +/- 0.05% RH/°C | (See temp probe) |  |  |
| Barometric Pressure | millibars (mb) | Vaisala Barocap © silicon capacitive pressure sensor | CS-105 | Pressure: 600 to 1060 mb; Temperature: -40°C to +60°C; Humidity: non-condensing | ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C | Stability: ± 0.1 mb per year | Z5120018 | 6/26/13- 6/10/14 | 6/4/2013 |
|  |  |  |  |  |  |  | X3740005 | 6/10/14-current | 5/23/14 |
| Wind direction | degrees | balanced vane, 38 cm turning radius | R.M. Young 05103 Wind Monitor | 360° mechanical, 355° electrical (5° open) | +/- 5% |  | WM73911 | 06/26/13- current | 5/30/2013 |
| Wind Speed | meter per second (m/s) | 18 cm diameter 4-blade helicoids propeller molded of polypropylene | R.M. Young 05103 Wind Monitor | 0-60 m/s (130 mph); gust survival 100 m/s (220 mph) | +/- 2% |  | (See wind dir) |  |  |

Sensor Specifications continued:  Homer Spit

| Parameter | Units | Sensor Type | Model | Range | Temp Dependence | Sensitivity | Stability | Sensor | Deploy Dates | Cal Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Precipitation | Milli-meters (mm) | Heated Tipping Bucket Rain Gauge | MetOne 380 | Temperature: 0° to +/- 50°C; Humidity: 0 to 100% | +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr | Rainfall per tip: 0.01 inch |  | B4708 | 8/16/02- current | 8/28/13 |
|  |  |  |  |  |  |  |  |  |  | 7/9/14 |
| Photosynthetically Active Radiation (PAR) LI-COR Quantum Sensor | mmoles m-2 (total flux) | High stability silicon photovoltaic detector (blue enhanced) | LI190SB | Light spectrum waveband: 400 to 700 nm; Operating Temperature: -40°C to 65°C; Humidity: 0 to 100% | 0.15% per °C | typically 5 µA per 1000 µmoles s-1 m-2 | <±2% change over 1 yr | Q34395 Multiplier= 1.32028 | 10/16/12-6/10/14 | 9/19/12 |
|  |  |  |  |  |  |  |  | Q30035 Multiplier=1.226392 | 6/10/14-current | 5/12/14 |
| Total Solar Radiation Pyranometer | mA per 1000 W m-2 | High stability silicon photovoltaic detector (blue enhanced) | LI200SZ | Light spectrum waveband: 400 to 1100 nm; Operating Temperature: -40°C to 65°C; Humidity: 0 to 100% | 0.15% per °C | 0.2 kW m-2 mV-1 | <±2% change over 1 yr | PY80805 (converted to Li200x; has constant Multiplier=200) | 06/26/13-current | 5/31/13 |

| Parameter | Units | Sensor Type | Model | Range | Accuracy | Other | Sensor | Deploy Dates | Cal Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | Celsius | Platinum resistance temperature detector (PRT) | HMP45ASP Temperature and Relative Humidity Probe | -40°C to +60°C | ± 0.2 °C @ 20°C |  | Z4020045 | 6/27/13-7/09/14 | 5/22/2013 |
| Temperature | Celsius | PT100 RTD, IEC 751 1/3 Class B; Teflon filter | HC2S3-L Temperature and Relative Humidity Probe | -40°C to +60°C | ± 0.1 °C @ 23°C |  | 61222581 | 7/09/14-current | 4/17/2014 |
| Relative Humidity | Percent | Vaisala HUMICAP© 180 capacitive relative humidity sensor | HMP45ASP Temperature and Relative Humidity Probe | 0-100% non-condensing | Accuracy at 20°C: +/- 2% RH (0-90%) and +/- 3% (90-100%) | Temperature dependence of RH measurement: +/- 0.05% RH/°C | (See temp probe) |  |  |
| Relative Humidity | Percent | ROTRONIC Hygromer IN-1 | HC2S3-L Temperature and Relative Humidity Probe | 0-100% non-condensing | Accuracy at 23°C: +/- 0.8% RH with standard configuration |  | (See temp probe) |  |  |
| Barometric Pressure | millibars (mb) | Vaisala Barocap © silicon capacitive pressure sensor | CS-106 | Pressure: 500 to 1100 mb; Temperature: -40°C to +60°C; | ± 0.3 mb @ 20°C; +/- 0.6 mb @ 0°C to 40°C; +/- 1 mb @ -20°C to 45°C; +/- 1.5 mb @ -40°C to 60°C | Stability: ± 0.1 mb per year | E4930045 | 6/27/13- 6/10/14 | 6/4/2013 |
|  |  |  |  |  |  |  | H2260147 | 6/10/14-current | 5/23/14 |

Sensor Specifications continued:  Anchor Point

Sensor Specifications continued:  Anchor Point

| Parameter | Units | Sensor Type | Model | Range | Temp Dependence | Sensitivity | Stability | Sensor | Deploy Dates | Cal Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wind direction | degrees | balanced vane, 38 cm turning radius | R.M. Young 05103 Wind Monitor | 360° mechanical, 355° electrical (5° open) |  | +/- 5% |  | WM84420 | 12/04/13- current | 11/25/2013 |
| Wind Speed | meter per second (m/s) | 18 cm diameter 4-blade helicoids propeller molded of polypropylene | R.M. Young 05103 Wind Monitor | 0-60 m/s (130 mph); gust survival 100 m/s (220 mph) |  | +/- 2% |  | (See wind dir) |  |  |
| Precipitation | Milli-meters (mm) | Heated Tipping Bucket Rain Gauge | MetOne 380 | Temperature: 0° to +/- 50°C; Humidity: 0 to 100% | +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr | Rainfall per tip: 0.01 inch |  | 45950-308 | 10/09/09- current | 9/5/13 |
|  |  |  |  |  |  |  |  |  |  | 7/9/14 |
| Photosynthetically Active Radiation (PAR) LI-COR Quantum Sensor | mmoles m-2 (total flux) | High stability silicon photovoltaic detector (blue enhanced) | LI190SB | Light spectrum waveband: 400 to 700 nm; Operating Temperature: -40°C to 65°C; Humidity: 0 to 100% | 0.15% per °C | typically 5 µA per 1000 µmoles s-1 m-2 | <±2% change over 1 yr | Q29306 Multiplier= 1.29346 | 10/16/12-6/10/14 | 9/18/12 |
|  |  |  |  |  |  |  |  | Q40054 Multiplier=1.33951 | 6/10/14-current | 5/12/14 |

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Homer Spit:

Date CR1000 Installed: 1/12/2007

Date CR1000 Calibrated: 05/02/2014

The CR1000 is equipped with a nl115 Ethernet and compact flash module which provides two independent capabilities (1) 10baseT Ethernet communication and (2) stores data on a removable compact flash card.

CR1000 Firmware Version(s):

CR1000.std.25 uploaded on 3/4/13

CR1000.std.27 uploaded when sent in for calibration. Deployed on 5/7/14

CR1000 Program Version(s):

KACHOMET_V7.2_101612.CR1 uploaded on 10/16/12

KACHOMET_V7.3_062713.CR1 uploaded on 6/27/13 (set PY multiplier)

KACHOMET_V7.4_061014.CR1 uploaded on 6/10/14 (new PAR mult=1.226392)

Anchor Point:

Date CR1000 Installed: 10/9/2009

Date CR1000 Calibrated: 12/14/2013

The CR1000 is equipped with a nl115 Ethernet and compact flash module which provides two independent capabilities (1) 10baseT Ethernet communication and (2) stores data on a removable compact flash card.

CR1000 Firmware Version(s):

CR1000.std.27 uploaded when sent in for calibration. Deployed on 1/7/14

CR1000 Program Version(s):

KACAPMET_V5.9_101612.CR1 uploaded on 10/16/12 (new PAR mult=1.29346)

KACAPMET_V5.10_061014.CR1 uploaded on 6/10/14 (new PAR mult=1.33951)

KACAPMET_V5.11_061814.CR1 uploaded on 7/9/14 (new temp sensor HC2-S3)

- Coded variable definitions

| Sampling station | Sampling site code | Station code |
| --- | --- | --- |
| Homer Spit | HO | kachomet |
| Anchor Point | AP | kacapmet |

11)  QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC		Incorrect Calibration Constant, Multiplier or Offset

SIW	Incorrect Wiring

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor Drift

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR		Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CCU	Cause Unknown

CDF	Data Appear to Fit Conditions

CML	Snow melt from previous snowfall event

CRE*	Significant Rain Event

CSM*	See Metadata

CVT*	Possible Vandalism/Tampering

CWE*	Significant weather event

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

Homer Spit:

Slightly elevated nighttime values occurred during 2014.  These data are flagged as suspect, <1> CSM.

On 4/28/14 12:45, we powered down the weather station and sent the CR1000 in for calibration.  The newly calibrated CR1000 was connected back to the weather station and began recording again on 5/7/14 16:15.

Cumulative precipitation data were rejected on 5/7/14 16:15 - 5/8/14 00:00.  Precipitation was recorded, however, this is not a real precipitation value and more than likely is an accidental tip.  No data were recorded for total precipitation.

On 6/10/14 13:28, we powered down the weather station and deployed new temperature, barometric pressure, and PAR instruments. We uploaded a new program (KACHOMET_V7.4_061014.CR1) with an updated PAR multiplier.  Data are missing 13:30 and 13:45 and  rejected from 13:15 and 14:00.

Cumulative Precipitation data on 6/10/14 14:15 - 6/11/14 00:00 were corrected from 0.0 to 0.3 to reflect precipitation amounts recorded earlier in the day prior to the power down and program reload that reset cumulative values to 0.0.

Data are missing 7/9/14 09:45 and rejected 7/9/14 09:30 and 10:00 due to maintenance to calibrate the precipitation gauge.

Anchor Point:

Anchor Point meteorological data have not gone through tertiary review by the CDMO.

Slightly elevated nighttime values occurred during 2014.  These data are flagged as suspect, <1> CSM.  This includes telemetered data at the end of the year that are flagged as corrected, <5> GIT CSM.

On 12/4/13, we powered down the weather station and sent the CR1000 in for calibration.  The newly calibrated CR1000 was connected back to the weather station on 1/7/14 with data collecting at 13:45.  Data at 13:30 were rejected since more than likely they were not a full 15 minutes of 5-second data.

On 6/10/14 11:15 - 12:00 data were either rejected (11:15 and 12:00) or missing (11:30 – 11:45), we powered down the weather station and deployed new barometric pressure and PAR instruments. We uploaded a new program (KACAPMET_V5.10_061014.CR1) with an updated PAR multiplier.

On 7/9/14 we powered down the weather station and deployed a new temperature probe (different model).  Data were rejected 7/9/1412:00 and 13:15 and missing 12:15 – 13:00. We adjusted the program to account for the new probe and uploaded KACAPMET_V5.11_061814.CR1.  We also calibrated the rain gauge.  As early as 7/25/14, the RH sensor on the new probe began to malfunction and occasionally drop down to 0. Data that dropped were rejected, <-3> SSM CSM.  We determined that the problem was the polyethylene filter can get clogged with salt crystals if the weather station is in a marine environment.  On 10/8/14 15:00, we replaced the polyethylene filter with a Teflon filter to fix the problem.  Even after the polyethylene filter was replaced, it was occasionally clogged and the RH data dropped very low, sometimes to zero.  These data have been rejected.

7/9/14 13:30 to 7/10/14 00:00 cumulative precipitation values after the power down were corrected to reflect actual precipitation that occurred at the site prior to and following the rain gauge calibration.

AP rain gauge has no heater, so precipitation data after the first continuous freeze have been rejected (11/21/14 00:00-12/31/14 23:45).

On 12/12/2014 23:15, the logger/program stopped writing data to the main data table. However, data were still being written to several other tables defined in the program for the logger. Values for air temperature (ATemp), relative humidity (RH), barometric pressure (BP), and photosynthetic-active-radiation (PAR) were recovered from the table that stores data for transmission to the GOES satellite network. These data needed to be divided by either 10 or 100 to shift the decimal point to the proper position, since they are transmitted to the satellite as integers. For some reason, values for wind speed and direction were not recorded properly in this table. Values for wind speed (WSpd) were recovered from a table that stores the values in U.S. English units. The program uses a multiplier of 2.24 to convert wind speed from meters/sec to MPH, so that value was used as a divisor to convert the US units back to meters/sec. The above procedure was used for all data from 12/12/2014 23:15 till the end of the year (12/31/2014 23:45). Data for wind direction and SDWDir were not recovered, and are missing for this time period.