Kachemak Bay (KAC) NERR Nutrient Metadata

January – December, 2007

Last updated: November 15, 2011

# I. Data Set and Research Descriptors

- Principal investigator(s) and contact persons

  - Reserve Contact

Joel Markis

Kachemak Bay Research Reserve

95 Sterling Highway, Suite 2

Homer, AK 99603

Phone: 907-226-4658

e-mail: joel.markis@alaska.gov

  - Laboratory Contact

Edan Badajos

Cook Inlet Keeper

3734 Ben Walters Lane, Suite 201

Homer, AK 99603

Phone: 907-235-4068

e-mail: edan@inletkeeper.org

Carol Pollard

VIMS Analytical Service Center

Rt 1208 Greate Road

Gloucester Point, VA 23062

Phone: 804-684-7213

- Research objectives

Kachemak Bay (KBRR) is a temperate region fjord with hydrographic conditions unique among the NERR system estuaries.  The circulation in the bay is driven primarily by tidal forcing.  Regional circulation is characterized by generally cyclonic ocean currents in the Gulf of Alaska flowing onto the shelf off of Cook Inlet.  Nutrient rich bottom water is up welled and mixed with surface water.  These enriched waters stream into Kachemak Bay, and the inflow tends to stay along the southern shore while water flowing out of the bay stays along the north shore.  These trapped coastal flows separate the bay into two distinct ecosystems.  As the inflowing water proceeds up the bay, fresh water runoff from the surrounding ice fields and watersheds dilute the salinity and increase the sediment load.  The inflowing water, therefore, initially supports a marine system while the northern out flowing water is more estuarine.  The vertical profile is stratified all year with the stratification increasing in strength during the summer.  We are monitoring to quantify the variability of the stratification and the along and across axis water quality gradients of the Bay.

  - Monthly Grab

Monthly grab samples are collected to quantify the vertical and horizontal spatial variability of important nutrients in the water column at sites representing the marine and estuarine endpoints of the local salinity gradient.

  - Diel Sampling Program

Once per month, samples are collected every two hours through a 24-hour tidal cycle to quantify the temporal variability of important nutrients in the water column as a function of tidal forcing.

3) Research methods

## The Cook Inlet Keeper laboratory processed KAC samples from January through September.  VIMS laboratory began processing KAC samples in October.

a) Monthly Grab Sampling Program

## Monthly grab samples are collected at two depths (near-surface and near-bottom) at the Homer and Seldovia datasonde stations.  All grab samples from both stations were taken within a 24-hour period.  To clearly delineate the stratification we attempt to sample near high tide.  The Homer station is accessed year round by vehicle. Because the Seldovia station (25km from Homer) is not connected to the road system, access is via air taxi or skiff and is weather-dependent.  At each station, three replicate (N = 3) samples are collected using a triggered vertical Nisken bottle at a depth of one meter from the surface.  Three replicates are also collected one meter from the bottom.  All samples are transferred to wide-mouth Nalgene sample bottles that were previously acid washed (10% HCL), rinsed (5x) with distilled-deionized water, dried and followed by rinsing (3x) of ambient water prior to collection of the sample.  Samples are immediately shielded from light and returned to the laboratory.  Once in the laboratory, samples are shaken and processed for nutrient and chl-a analysis.

b) Diel Sampling Program

Within the same 24-hour period as our grab sample collection, we deploy an ISCO water sampler from a floating dock in the Homer Harbor located approximately 0.3 miles north of the Homer datasonde station.  .  This device automatically samples 850ml of water every 2hrs 4min from approximately 1-meter depth.  All samples are pumped into polyethylene sample bottles that were previously acid washed (10% HCL), rinsed (5x) with distilled-deionized water and dried.  The 12 samples are kept in the dark and at the end of the 24hr period are returned to the laboratory for immediate processing.

4) Site location and character

The KBRR includes approximately 4,000 km2 of terrestrial and marine habitats, making it the largest reserve in the NERR system.  The bathymetry is characterized by a submerged moraine or sill at the mouth of the Bay and deep trenches and holes extending to almost 200m deep within the Bay. A 6km-long spit extending south from the village of Homer (pop. 5,000) restricts Bay circulation patterns.  The south side of the Bay is mostly deep and rocky and is lined by rugged snow covered mountains, while the north side is mostly shallow with mixed sediment shores.  Seven glaciers flow into Kachemak Bay from the Harding and other ice fields, some of the last remaining ice fields in North America.  The large volume of sediments derived from these glaciers help build and sustain the predominantly sand and gravel beaches surrounding the estuary.  The Fox River Flats, at the head of the bay, is a huge salt marsh complex supporting thousands of migratory birds every spring and fall.  During the summer, glacier melt water contributes approximately 70,000 cubic meters of fresh water each day to the inner Bay.  Another 14 billion cubic meters of cold, nutrient-rich seawater from the Gulf of Alaska flows in and out of the outer Bay, partly driven by an 8.7 meter tidal range that results from the complex geomorphology of the Gulf of Alaska and Cook Inlet.  Fjords such as Kachemak Bay often have a seasonally stratified water column that results when a surface layer of fresh water develops above denser cold seawater.  But one unique feature of this high latitude NERR is that during the 6-month-long winter, when the watershed is frozen, this fresh surface layer disappears and the bay becomes wholly marine in nature.

The Homer water grab site is located on the north side of Kachemak Bay at 59.6028ºN and 151.4081ºW.    One set of grab samples is collected one meter above bottom (“Deep”) and the other is one meter from the surface (“Shallow”).  Data for the Homer site was collected from January through December 2007.  Because of the 8m tidal range, the deep sample is collected from water depths of 8.0 to 17.0m.  The bottom habitat is predominantly sand.  Pollutants in the area are from the excessive boat traffic at the entrance of the Homer harbor, and a nearby fish waste outfall line.  Throughout the year, salinity has ranged from 20.6 to 32.3ppt, the surface site often having significantly lower salinity.  Most of the freshwater during the summer comes from snow and ice melt on the glaciers.  Starting in August rainfall reduces salinity until November when the rain begins to shift to snow and there is very little fresh water input to the bay.

The Seldovia water grab site is located on the south side of Kachemak Bay at 59.4413ºN, 151.7186ºW, approximately 25km west of the Homer site.  As with the Homer site, they are collected from the ferry terminal dock at one meter above the bottom and one meter below the water surface.  Because of the 8m tidal range, the deeper sample is collected from water depths of 5.0 to 13.3m.  The access to Seldovia is limited to boat or air because the site is located off the highway system.  The bottom habitat is predominantly rocky substrate.  Pollutants in the area are minimal.  Throughout the year, salinity has ranged from 26.3 to 32.8ppt at this site. Most of the freshwater during the summer comes from snow and ice melt on the glaciers.  Starting in August rainfall reduces salinity until November when the rain begins to shift to snow and there is very little fresh water input to the Bay.

The diel sampling site is located in the Homer harbor on a marine fuel dock.  The site is located at 59.60308ºN, 151.41815ºW, and has 4m of water at low tide.  Placing the sampling site inside the harbor protects the equipment from winter storms and boat traffic that are often present at the grab site.  The bottom at this site is rocky.  Because the site is on a fuel dock, there is the possibility of hydrocarbon pollutants in the water.  The tidal range, salinity range, and freshwater input are the same as at the Homer grab site.

5) Code variable definitions

Station Codes: Homer Deep (kachdnut), Homer Shallow (kachsnut), Homer Harbor (kachhnut), Seldovia Deep (kacsdnut) and Seldovia Shallow (kacssnut)

Monitoring Programs: Grab Sampling (1), Diel Sampling (2)

6) Data collection period

The first water samples for the SWMP nutrient monitoring program were collected January 17, 2006 at 12:35 and the last were collected on December 07, 2007 at 12:35 For grab samples typically only a starting time was recorded.  It normally takes 20 minutes to collect the six samples—3 “shallow” and 3 “deep”—at both the Homer and Seldovia sites.  We report all times in Alaska Standard Time.

Location	Sample type	Date	Shallow	Deep

Homer		Grab		01/17	12:35		12:45

Seldovia	Grab		01/16	11:05		11:15

Homer		Grab		02/13	11:28		11:32

Seldovia	Grab		02/14	13:05		13:13

Homer		Grab		03/19	13:25		13:35

Seldovia	Grab		03/18	14:03		14:08

Homer		Grab		04/10	10:15		10:23

Seldovia	Grab		04/11	12:40		12:45

Homer		Grab		05/08	08:00		08:15

Seldovia	Grab		05/09	08:25		08:40

Homer		Grab		06/20	09:20		09:30

Seldovia	Grab		06/18	16:40		16:50

Homer		Grab		07/20	10:15		10:25

Seldovia	Grab		07/24	09:45		09:55

Homer		Grab		09/06	11:40		11:50

Seldovia	Grab		09/07	10:27		10:32

Homer		Grab		10/04	11:00		11:10

Seldovia	Grab		10/05	10:20		10:25

Homer		Grab		11/08	11:45		11:55

Seldovia	Grab		11/09	14:45		14:50

Homer		Grab		12/06	11:35		11:45

Seldovia	Grab		12/07	12:25		12:35

Location	Sample type	Date	Start time	End time

Homer		DIEL		01/16	09:45		01/17 	08:29

Homer		DIEL		02/13	11:17		02/14 	10:01

Homer		DIEL		03/20	11:37		03/21	10:21

Homer		DIEL			no ISCO samples

Homer		DIEL		05/08	09:16		05/09	08:00

Homer		DIEL		06/19	08:58		06/20	07:42

Homer		DIEL		07/23	12:00		07/24	10:44

Homer		DIEL			no ISCO samples

Homer		DIEL		09/07	12:15		09/08	10:59

Homer		DIEL		10/04	12:33		10/05	11:17

Homer		DIEL		11/08	11:03		11/09	09:47

Homer		DIEL		12/06	08:27		12/07	07:11

7) Associated researchers and projects

This effort is part of a much larger Coastal Ocean Observing System in Kachemak Bay, Lower Cook Inlet, and the Gulf of Alaska.  See www.kbayrr.org for more information.

As part of the SWMP long-term monitoring program, KAC NERR also monitors Meteorological and Water Quality data which may be correlated with this Nutrient dataset.  These data are available from the Research Coordinator or online at http://cdmo.baruch.sc.edu/.

8) Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format.

II. Physical Structure Descriptors

9) Entry Verification

Cook Inlet Keeper and VIMS Laboratory calculate and report results in on Excel spreadsheets. The data is then transferred to a combined nutrient data sheet covering the year. Data are checked to ensure that values are above the minimum detectable level. Nutrient data are then processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and automatically flags and codes values below MDL; calculates parameters chosen by the user and automatically flags for component values below MDL and negative values; allows the user to apply QAQC flags and codes to the data; graphs selected parameters for review; append files; and export the resulting data files to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database. Judy Hamilton was responsible for this task.

10) Parameter Titles and Variable Names by Data Category

Required NOAA/NERRS System-wide Monitoring Program water quality parameters are denoted by an asterisk (“*”).

Data Category		Parameter		Variable Name	Units of Measure

    - Phosphorus:

*Orthophosphate			PO4F		mg/L as P

    - Nitrogen:

*Nitrite + Nitrate, Filtered		NO23F	mg/L as N

*Nitrite, Filtered			NO2F		mg/L as N

*Nitrate, Filtered			NO3F		mg/L as N

*Ammonium, Filtered			NH4F		mg/L as N

Dissolved Inorganic Nitrogen	DIN		mg/L as N

c) Plant Pigments

*Chlorophyll a			CHLA_N	g/l

Phaeophytin				PHEA		g/l

d) Other Lab Parameters:

Silicate, Filtered			SiO4F		mg/L as SI

Notes:

1.  Time is coded based on a 2400 hour clock and is referenced to Alaska Standard Time (AST).

2.  Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

11) Measured and Calculated Laboratory Parameters –

    - Variables Measured Directly

Nitrogen species:		NO2F, NO23F, NH4F

Phosphorus species:		PO4F

Other:				CHLA, PHEA, SiO4F

b) Computed Variables

NO3:				NO23F - NO2F

DIN:				NO23F + NH4F

12) Limits of Detection

Method Detection Limits (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect, have been established by Cook Inlet Keeper.  Table 1 lists the MDL values for Cook Inlet Keeper (CIK), and these are reviewed and revised periodically.

Table 1.  CIK Method Detection Limits (MDL) for measured water quality parameters.

| Parameter | Variable | Range: μM | MDL mg/L | Dates in use |
| --- | --- | --- | --- | --- |
| Ammonium | NH4F | 0-3 | 0.0058 | 1/1/07-09/28/2007 |
| Nitrite | NO2F | 0-3 | 0.0008 | 1/1/07-09/28/2007 |
| Nitrate + Nitrite | NO23F | 0-25 | 0.0211 | 1/1/07-09/28/2007 |
| Orthophosphate | PO4F | 0-3 |  |  |
|  |  |  | 0.0021 | 1/1/07-09/28/07 |
| Silicate | SiO4F | 0-50 | 0.025 | 1/1/07-09/28/2007 |
| Chlorophyll a, Phaeophtin1 | CHLA, PHEA |  |  | 1/1/07-09/28/2007 |

Notes:

1 Fluorometric analysis performed on Turner model AU10. The MDL is dependent on the amount of water filtered.  The MDL is for 200ml of sample filtered.

Method Detection Limits have also been established by the VIMS Nutrient Analytical Laboratory. The MDL is determined as 3 times the standard deviation of a minimum of 7 replicates of a single low concentration sample. These values are reviewed and revised periodically.

| Parameter | Start Date | End Date | MDL |
| --- | --- | --- | --- |
| PO4F | 9/29/07 | 12/31/07 | 0.0015 |
| NH4F | 9/29/07 | 12/31/07 | 0.0054 |
| NO2F | 9/29/07 | 12/31/07 | 0.0002 |
| NO23F | 9/29/07 | 12/31/07 | 0.0010 |
| CHLA_N | 9/29/07 | 12/31/07 | 0.02 |
| PHEA | 9/29/07 | 12/31/07 | 0.02 |
| SiO4F | 9/29/07 | 12/31/07 | 0.008 |

13) Laboratory Methods

a) Parameter:  NH4

Cook Inlet Keeper:

    - Method Reference:  Slawyk, G. and MacIsaac, J.J. (1972) Comparison of two automated ammonium methods in a region of coastal upwelling. Deep Sea Research 19:521-524.

    - Method Descriptor:  A water sample is treated with phenol and alkaline hypochlorite in the presence of NH3 to form indophenol blue (Berthelot reaction).  Sodium nitroferricyanide is used as a catalyst in the reaction.  Precipitation of Ca and Mg hydroxides is eliminated by the addition of sodium citrate-complexing reagent. The sample stream is passed through a 55oC heating bath, then through a 50 mm flowcell and absorbance is measured at 640 nm.

    - Preservation Method:  Sample is filtered through a 0.45um disposable disk filter and stored at –20oC until analyzed.

VIMS

- Method References:

  - U.S. EPA. 1974. Methods for Chemical Analysis of Water and Wastes. Pp.168-174

  - Standard Methods for the Examination of Water and Wastewater, 14th edition. P. 410. Method 418A and 418B (1975)

  - Annual Book of ASTM Standards, Part 31. “Water”, Standard 1426-74, Method A, p. 237 (1976).

  - EPA 600/R-97/072 Method 349.0. Determination of Ammonia in Estuarine and Coastal Waters by Gas Segmented Continuous Flow Colorimetric Analysis. IN: Methods for the Determination of Chemical Substances in Marine and Estuarine Environmental Matrices – 2nd Edition. National Exposure Research Laboratory, Office of Research and Development U.S. EPA, Cincinnati, Ohio 45268.

- Method Descriptor:  Alkaline phenol and hypochlorite react with ammonia to form indophenol blue that is proportional to the ammonia concentration.  The blue color formed is intensified with sodium nitroprusside.  Reaction is heat catalyzed at 37o C.

- Preservation Method:  The water sample is filtered through a 0.45 m disposable disk filter and stored at –20oC until analyzed.

b) Parameter:  NO23, NO3 and NO2

Cook Inlet Keeper:

    - Method Reference:  Armstrong, F.A., Stearns, C.R. and Strickland, J.D.H. (1967) The measurement of upwelling and subsequent biological processes by means of the Technicon AutoAnalyzer and associated equipment. Deep Sea Research 14:381-389.

    - Method Descriptor:  A water sample is passed through a cadmium column where the nitrate is reduced to nitrite.  This nitrite is then diazotized with sulfanilamide and coupled with N-(1-naphthyl)-ethylenediamine to form an azo dye.  The sample is then passed through a 15mm flowcell and absorbance is measured at 540nm. A 50mm flowcell is required for nitrite (NO2).  The procedure is the same for the nitrite analysis less the cadmium column.  Nitrate concentration equals the (nitrate + nitrite) concentration minus the nitrite concentration.

    - Preservation Method:  Sample is filtered through a 0.45um disposable disk filter and stored at –20oC until analyzed.

VIMS:

- Method References:

  - SKALAR Method: Nitrate+Nitrite/Total Dissolved Nitrogen.  Catnr. 461-353.2 issues 120293/MH/93128060

  - U.S. EPA 1974 Methods for Chemical Analysis of Water and Wastes, pp. 207-212

  - Wood, E.D., F.A.G. Armstrong and F.A. Richards. 1967. Determination of nitrate in seawater by cadmium-copper reduction to nitrite. J. Mar. Biol. Assoc. U.K. 47:23

  - Grasshoff, K., M. Ehrhardt and K. Kremling. 1983. Methods of Seawater Analysis. Verlag Chemie, Federal Republic of Germany. 419 pp.

  - EPA 600/R-97/072 Method 353.4 Determination of Nitrate and Nitrite in Estuarine and Coastal Waters by Gas Segmented Flow Colorimetric Analysis. . IN: Methods for the Determination of Chemical Substances in Marine and Estuarine Environmental Matrices – 2nd Edition. National Exposure Research Laboratory, Office of Research and Development U.S. EPA, Cincinnati, Ohio 45268.

- Method Descriptor:  Nitrate is reduced to nitrite by a copper/cadmium redactor column.  The nitrite ion then reacts with sulfanilamide to form a diazo compound.  This compound then couples with n-1-napthylenediamine dihydrochloride to form a reddish/purple azo dye.  The color development chemistry is the same as that used in Nitrite. Nitrate concentration equals the (nitrate + nitrite) concentration minus the nitrite concentration.

- Preservation Method:  The water sample is filtered through a 0.45 m disposable disk filter and stored at –20oC until analyzed.

c) Parameter:  SiO4F, Si(OH)4

Cook Inlet Keeper:

    - Method Reference:  Armstrong, F.A., Stearns, C.R. and Strickland, J.D.H. (1967) The measurement of upwelling and subsequent biological processes by means of the Technicon AutoAnalyzer and associated equipment. Deep Sea Research 14:381-389.

    - Method Descriptor:  Ammonium molybdate is added to a water sample to produce silicomolybdic acid, which is then reduced to silicomolybdous acid (a blue compound) following the addition of stannous chloride.  The sample is passed through a 15mm flowcell and absorbance is measured at 820nm.

    - Preservation Method:  Sample is filtered through a 0.45um disposable disk filter and stored at –20oC until analyzed.

VIMS:

- Method References:

  - SKALAR Method: Silicate Catnr. 563-052 issue 101899/MH/99208255

  - U.S. EPA. 1974 Methods for Chemical Analysis of Water and Wastes, Method 370.1

  - Grasshoff, K., M. Ehrhardt and K. Kremling. 1983. Methods of Seawater Analysis.  Verlag 	Chemie, Federal Republic of Germany. Pp 374-376.

  - EPA 600/R-97/072 Method 366.0  Determination of Dissolved Silicate in Estuarine and Coastal Waters by Gas Segmented Flow Colorimetric Analysis.  IN: Methods for the  Determination of Chemical Substances in Marine and Estuarine Environmental Matrices - 2nd Edition.  National Exposure Research Laboratory, Office of Research and Development  U.S. EPA, Cincinnati, Ohio 45268.

- Method Descriptor:  This automated procedure for the determination of soluble silicates is based on the reduction of a silicomolybdate in acidic solution to "molybedenum blue" by ascorbic acid.  Oxalic acid is introduced to the sample stream before the addition of ascorbic acid to eliminate interference from phosphates.  The range is 0 - 1.4 mg Si/L.

- Preservation Method:  The water sample is filtered through a 0.45 m disposable disk filter and stored at –20oC until analyzed.

d) Parameter:  PO4F

Cook Inlet Keeper:

    - Method Reference:  Bernhardt, H. and Wilhelms, A. (1967) The continuous determination of low level iron, soluble phosphate, and total phosphate with the AutoAnalyzer. Technicon Symp. 1:386.

    - Method Descriptor:  Ammonium molybdate is added to a water sample to produce phosphomolybdic acid, which is then reduced to phosphomolybdous acid (a blue compound) following the addition of dihydrazine (or hydrazine) sulfate.  The sample is passed through a 50mm flowcell and absorbance is measured at 820nm.

    - Preservation Method:  Sample is filtered through a 0.45um disposable disk filter and stored at –20oC until analysis.

VIMS:

- Method References:

  - SKALAR Method: O-Phosphate/Total Phosphate Catnr. 503-365.1, issue 042993/MH/93-Demol

  - Murphy, J. and J.P. Riley. 1962. A modified single solution method for the determination of phosphate in natural waters. Analytica Chim. Acta 27: 31-36.

  - EPA 600/R-97/072 Method 365.5 Determination of Orthophosphate in Estuarine and Coastal Waters by Gas Segmented Flow Colorimetric Analysis. . IN: Methods for the Determination of Chemical Substances in Marine and Estuarine Environmental Matrices – 2nd Edition. National Exposure Research Laboratory, Office of Research and Development U.S. EPA, Cincinnati, Ohio 45268.

- Method Descriptor:  Ammonium molybdate and antimony potassium tartrate react in a sulfuric acid environment to form an antimony-phospho-molybdo complex, which is reduced to a blue colored complex by ascorbic acid.  Reaction is heat catalyzed at 40oC.

- Preservation Method:  The water sample is filtered through a 0.45 m disposable disk filter and stored at –20oC until analyzed.

e) Parameter:  CHLA, PHEA

    - Method References:  EPA method 445.0UNESCO (1994) Protocols for the joint global ocean flux study (JGOFS) core measurements. pp. 97-100.

    - Method Descriptor:  CHLA is extracted in 10 ml 90% acetone and fluorescence is measured and recorded (Fo).  Several drops (5-7) of 10% are added to convert the CHLA to phaeopigments (PHAE).  The fluorescence is again measured and recorded (Fa).  The concentration (μg/L) of CHLA and PHAE are calculated using the Fo / Fa ratio.

    - Preservation Method:  A known volume of sample is filtered onto a 25mm GF/F filter, folded in half and wrapped in aluminum foil.  Sample is stored at –70oC until analysis.

14)  Field and Lab QA/QC Programs

  - Precision

    - Field Variability – KBRR collects three successive grab samples for the determination of water mass variability.

    - Laboratory Variability – none

    - Inter-organizational splits – none

  - Accuracy

    - Sample Spikes – blanks

    - Standard Reference Material Analysis – see lab protocols

    - Cross Calibration Exercises – none

Cook Inlet Keeper participated in the NERR blind sampling program.  This program provided the lab with a sample with known nutrient concentrations for analysis.  The CIK laboratory also participates in the USGS laboratory-testing program.

15)  QA/QC Programs Flag Definitions:

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range*

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

*The -4 Outside Low Sensor Range flag was added to the 2007 dataset in August of 2011.  See the Other Remarks section for more details.

16)  QA/QC Code Definitions:

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data rejected due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Value calculated with a value that is below the MDL

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N	from the north

NNE	from the north northeast

NE	from the northeast

ENE	from the east northeast

E	from the east

ESE	from the east southeast

SE	from the southeast

SSE	from the south southeast

S	from the south

SSW	from the south southwest

SW	from the southwest

WSW	from the west southwest

W	from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17) Other remarks/notes

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

*The 2007 dataset was updated on August of 2011 to include the -4 Outside Low Sensor Range flag.  The 2007 data published prior to that time used the -3 Rejected data flag with the SBL and SCB QAQC codes to indicate that data were below the minimum detection limit.  These flag code combinations were all replaced with the -4 SBL or SCB update as mandated by the Data Management Committee.

When conducting the chlorophyll-a analysis, we had problems with the test tube holder on the Turner Fluorometer 10-AU getting slightly lifted and stuck in the wrong position.  This caused inaccurate readings by the fluorometer that was not noticed until later during QAQC. Most times these inaccurate readings were obvious and produced negative chlorophyll-a values.  Any negative chlorophyll-a values and corresponding pheophytin results were rejected.   We normally analyze all 24 samples from the same month in a batch.  If a negative value appeared in a batch, all other samples from that month were flagged as <0> CSM because the readings seemed to be accurate but it is possible that a test tube holder in the stuck position might produce a positive result.  The problem with the test tube holder has since been corrected.