Narragansett Bay (NAR) NERR

Meteorological Metadata

January – June 2017

Latest Update:  Tuesday, July 18, 2017

Note:  This is a provisional metadata document; it has not been authenticated as of its download date.  Contents of this document are subject to change throughout the QAQC process and it should not be considered a final record of data documentation until that process is complete.  Contact the CDMO (cdmosupport@belle.baruch.sc.edu) or Reserve with any additional questions.

I.  Data Set and Research Descriptors

1)  Principal Investigator(s) and Contact Persons

Contact Persons:

Dr. Kenneth Raposa, Research Coordinator

E-mail: kenny@nbnerr.org;

(401) 683-7849

Dr. Daisy Durant, Marine Research Specialist II

SWMP Coordinator and Data Manager

E-mail: daisy@nbnerr.org;

(401) 683-7368

Address: 	Narragansett Bay NERR

55 South Reserve Drive

(PO Box 151)

Prudence Island, RI 02872

Phone:	401-683-6780

Fax:	401-682-7366

2)  Entry Verification

Data are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from or LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

Daisy Durant, PhD (Marine Research Specialist II), was responsible for compiling and error checking the January through June 2017 weather data covered in this document.

3)  Research Objectives

The principal objective is to record long-term meteorological data for Narragansett Bay in order to observe any environmental changes or trends over time.  These data are also used to support ongoing water quality and biological monitoring, scientific research, as well as stewardship, training and education activities.

4)  Research Methods

The Campbell Scientific CR1000 data logger collects data every 5 seconds to produce 15-minute averages of those measurements of air temperature, relative humidity, barometric pressure, wind speed and wind direction and photosynthetic active radiation; the data logger collects totals for precipitation, not averages.  The CR1000 program directs how and when the sensors measure and store data.  All data are collected in Eastern Standard Time and stored in the CR1000 data logger.  Monthly trips to the weather station are done to inspect, clean and maintain all sensors and all other weather station related equipment.  During these monthly trips, data are downloaded from the data logger via a RS-232 connection serial cable to a laptop using the Campbell Scientific LoggerNet (v 3.4.1) program.  The data logger is housed in a Campbell Scientific weather resistant enclosure.  A satellite transmitter (TX312), a 12 V sealed rechargeable battery, a regulator (CH-100, which provides built-in temperature compensation to optimize battery performance), and a barometric pressure sensor (vented to the outside of the enclosure) are housed in the enclosure as well.

The 15-minute data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), and Battery Voltage (volts).

Maximum and Minimum Air Temperature (oC) and their times from 5-second data (these data are available from the Reserve).

Maximum Wind Speed (m/s) and time from 5-second data.

Wind Direction Standard Deviation (degrees).

Totals:

Precipitation (mm)

PAR (millimoles/m2)

Cumulative Precipitation (mm)

The recommended calibration frequency for the MET station sensors according to manufacturer are:

Temperature/Humidity	yearly recalibration

Rain Gauge	yearly recalibration

Wind Speed/Direction	yearly or every 2 years (depending on the sensor)

Barometric Pressure	every 2 years recalibration

PAR	every 2 years recalibration

CR1000	every 5 years (required beginning 2014, one year initial grace period)

Ground-truthing is conducted during the monthly trips, and data on air temperature, relative humidity, barometric pressure, and wind speed are collected with a Kestrel 4000 weather meter; wind direction is determined with a handheld compass; and photosynthetic active radiation is measured with a LiCor 1400 equipped with a LI-190SB quantum sensor; total precipitation is checked from the T. F. Green Airport NOAA weather station in Providence, RI; weather data collected from the airport is available online at  http://w2.weather.gov/climate/index.php?wfo=box.  These ground-truthing data are used to compare with the data collected by the weather station on Prudence Island and ensure accuracy of the readings.  Ground-truthing data are recorded on the meteorological monthly log, kept at the Reserve and at CDMO, and available upon request.

In July 31st 2006, the Campbell Scientific data telemetry equipment was installed at the Potter Cove weather station which allowed the transmission of data to the NOAA GOES satellite, NESDIS ID 3B0211F8.  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database. Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

5)  Site Location and Character

The NBNERR is located close to the geographic center of Narragansett Bay in Rhode Island.  It consists of approximately 1802 hectares (4453 acres: 2544 acres of land plus 1909 acres of surrounding estuarine waters, approximately) of diverse estuarine and terrestrial habitats ranging from deep water to salt marshes to forested uplands.  The land holdings include approximately 65% of Prudence Island, most of nearby Patience Island, and all of Hope and Dyer Islands.  The most recent land acquisition was the Eugene Chase Farm property, made official by the end of year 2015.

The Narragansett Bay watershed consists of nine subwatersheds draining an area of approximately 4,836 square km (Pilson, 1985) and numerous and substantial freshwater inputs to the Bay.  Approximately 39% of the watershed lies in Rhode Island and 61% in Massachusetts.  It is referred to as a shallow estuary; however, its water depth varies considerably.  Depth averages approximately 9.0 m throughout the Bay, but it is deeper in the East Passage (approximately 15.2 m) and shallower in the West Passage (approximately 7.5 m).  More information and a detailed description of the Narragansett Bay NERR and the Narragansett Bay watershed can be found in Raposa and Schwartz (2007), available to download at http://nbnerr.org/research-and-monitoring/publications/.

The weather station is located on Prudence Island, approximately 389 m south of Potter Cove (41o 38’ 13.703” N, 71o 20’ 21.790” W, Trimble Geo XT, GeoExplorer 2008 Series).  The Wind Monitor is located at the top of a 10 m high aluminum tower, the temperature and humidity sensor are located at approximately 2.0 m off the ground on the same aluminum tower.  A large wooden platform approximately 2.4 m W x 1.8 m D x 2.1 m H has been situated at the weather station, approximately, since 1991.  It was originally constructed by the U.S. Environmental Protection Agency (EPA) to hold atmospheric deposition equipment, which is no longer in use.

Picture of the weather station on Prudence Island.

Permission secured from the EPA gave access to this platform for weather station equipment.  The Campbell housing unit is situated under the platform and contains the CR1000 data logger and all associated hardware (and telemetry equipment) as well as the barometric pressure unit (approximately 1.8 m off the ground).  On top of the platform railing, we placed the GPS antenna, solar panel, and Yagi antenna.  The PAR meter was also relocated here to make it more accessible for cleaning than the previous location.  It is approximately at 3.68 m off the ground.  The rain gauge is also on the platform railing at approximately 3.46 m off the ground.  All sensors were located in accordance with manufacturer recommendations to avoid the possible influence of shading, wind blocks, etc.

6)  Data Collection Period

Meteorological data has been collected at the weather station on Potter Cove since 1992.  However, it was not until 2001 when the meteorological station was updated and became part of NERR-SWMP.

Data collected during 2017 are included in files downloaded from the weather station from January 01 to June 30 (Table 1).

Table 1.  Date and time of the first and last readings on raw data files downloaded from the CR1000 data logger at the weather station on Prudence Island from January to June 2017.

| First Reading in File |  | Last Reading in File |  |
| --- | --- | --- | --- |
| Date | Time | Date | Time |
| 01/01/17 | 00:00 | 01/01/17 | 15:45 |
| 1 01/01/17 | 16:00 | 01/04/17 | 10:15 |
| 2 01/04/17 | 10:30 | 01/31/17 | 12:45 |
| 1 01/31/17 | 13:00 | 02/02/17 | 14:30 |
| 02/02/17 | 14:45 | 02/03/17 | 11:45 |
| 1 02/03/17 | 12:00 | 02/21/17 | 12:15 |
| 02/21/17 | 12:30 | 02/23/17 | 08:45 |
| 1 02/23/17 | 09:00 | 02/23/17 | 14:45 |
| 02/23/17 | 15:00 | 03/31/17 | 23:45 |
| 04/01/17 | 00:00 | 04/30/17 | 23:45 |
| 05/01/17 | 00:00 | 05/10/17 | 09:45 |
| 3 05/10/17 | 10:00 | 05/10/17 | 12:00 |
| 05/10/17 | 12:15 | 06/30/17 | 23:45 |

Notes

1 No data collected during this period due to power failure at the station.

2 No data collected from Jan 06 09:00 to Jan 09 12:30 due to power failure at the station.

3 Powered down the station for maintenance/troubleshooting, no data collected from Jan 10 10:00 - 12:00.

7)  Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2012.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in comma delimited format.

8)  Associated Researchers and Projects

YSI 6600 V2 (left) and EXO2 (right) data logger used to collect water quality data at the Reserve (pictures from YSI.com).

The NBNERR System-Wide Monitoring Program (SWMP) has four water quality monitoring station around Prudence Island (see map on Section 5, Site Location and Character).  The principal objective of the SWMP program is to record short-term variability and long-term changes in water quality data in order to observe trends or patterns in water quality over time.  Water quality parameters have been collected since 1995 with the establishment of the first water quality monitoring station at Potter Cove.  Other three water quality stations (Nag Creek, T-Wharf Surface and T-Wharf Bottom) were brought online in 2002.  These stations were selected to represent a gradient in habitat types that range from salt marsh (Nag Creek station) to shallow cove (Potter Cove) to open Bay water (T-Wharf Surface and T-Wharf Bottom).  Water temperature, salinity, dissolved oxygen (% saturation, and mg L-1), pH, turbidity, and chlorophyll fluorescence data are collected at each station every 15 minutes using YSI 6600 V2 and EXO2 data loggers (see image at right) that are calibrated and swapped out at each station approximately every two to four weeks.

Complementary to the existing long-term water quality monitoring program, the NERRS implemented a new nutrient and chlorophyll monitoring program in 2002.  The two sub-components of this program include monthly grab sampling at each of the four water quality stations, and diel sampling once a month at one site.  The grab sampling program requires the collection of duplicate water samples every month from each of the four long-term water quality monitoring stations with the purpose of quantify seasonal patterns of nutrient and chlorophyll concentrations in different estuarine habitats (marsh creek, cove, surface open water, bottom open water).  The diel sampling program requires to collect a series of samples from one station over an approximately 24-hour period each month to examine how nutrient and chlorophyll concentrations change over diel and tidal cycles.  Previously (from 2002 to 2010) the diel station was located at T-Wharf.  However, after analyzing the historic data from the site, no significant trends or patterns were found over time.  Therefore, the diel station was moved to Potter Cove in January of 2011 in order to characterize nutrients and chlorophyll from this site.  All collected grab and diel samples are analyzed for concentrations of phosphates (PO4), ammonia (NH4), nitrite (NO2), nitrate (NO3), NO2+NO3, dissolved inorganic nitrogen (DIN), silicates (SiO4) and chlorophyll a.

Meteorological data collected at the NBNERR since 2001 is continuously used to support the aforementioned water quality and nutrient monitoring programs, the biological monitoring efforts at the Reserve, and to assist scientific research and monitoring projects, as well as stewardship, training, and educational activities around Narragansett Bay.

A Physical Oceanographic Real-Time System (PORTS) meteorological station is housed in the NERRS weather station at Potter’s Cove and independently records air temperature, atmospheric pressure, wind speed, and wind direction.  This is one of six PORTS meteorological stations in Narragansett Bay.  The purpose of PORTS is to support safe and cost efficient navigation.  Data are available real-time and the system is managed for quality control.

Bob Marshall of the Prudence Island Groundwater Task Force has been using the NBNERR precipitation data since 2006.  These data are being used in conjunction with groundwater level and stream-flow monitoring efforts to evaluate the status of the groundwater resource on Prudence Island.

Robin Weber, GIS/Natural Resources Specialist from the NBNERR has been conducting biweekly monitoring of tick populations since 2008 during the warmer months, which over time, may inform management of the deer population and indicate the degree of public health risk for tick borne disease.  Sampling success for ticks is influenced by environmental conditions (e.g., relative humidity, air temperature, precipitation), thus, meteorological data available from NBNERR’s weather station is being incorporated in the data analysis for each sampling date and time to remove potential sources of variation in annual tick abundance levels.

II. Physical Structure Descriptors

9)  Sensor Specifications, Operating Range, Accuracy, Date of Last Calibration

Parameter: 	Temperature

Units: 	Celsius

Sensor type: 	PT100 RTD, IEC 751 1/3 Class B, with calibrated signal conditioning

Operating Temperature:  	-40°C to +60°C

Range: 	-40°C to +60°C

Accuracy: 	± 0.1°C at 23°C

Model #:  	HC2-S3

Serial Number:	60835193

Date of calibration:  	02/12/14

Dates of sensor use:	08/12/14 to 09/17/15

Model #:  	HC2-S3

Serial Number:	61180509

Date of calibration:  	03/17/15

Dates of sensor use:	09/17/15 to 09/08/16

Model #:  	HC2-S3

Serial Number:	60835193

Date of calibration:  	02/02/16

Dates of sensor use:	09/08/16 to present

Parameter:	Relative Humidity

Units: 	Percent

Sensor type: 	ROTRONIC® Hygromer IN-1

Range:  	0-100% non-condensing

Accuracy:	± 0.8% RH at 23°C with standard configuration settings

Temperature dependence: 	± 3% (-40 to 60C)

Model #:  	HC2S3 (Temperature and Relative Humidity Probe)

Serial Number:	60835193

Date of calibration:  	02/12/14

Dates of sensor use:	08/12/14 to 09/17/15

Model #:  	HC2S3

Serial Number:	61180509

Date of calibration:  	03/17/15

Dates of sensor use:	09/17/15 to 09/08/16

Model #:  	HC2S3

Serial Number:	60835193

Date of calibration:  	02/02/16

Dates of sensor use:	09/08/16 to present

Parameter: 	Barometric Pressure

Units: 	millibars (mb)

Sensor type: 	Vaisala Barocap © silicon capacitive pressure sensor

Operating Range:	Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C

Humidity:  	non-condensing

Accuracy: 	± 0.5 mb at +20°C, ± 1.5mb at 0°C to 40°C, ± 2.0 mb at -20°C to +45°C,

± 3.0 mb at -40°C to +60°C

Stability: 	± 0.1 mb per year

Model #: 	CS-105

Serial Number:	P4910009

Date of calibration:  	08/09/11

Dates of sensor use:	08/22/12 to 07/02/14

Model #: 	CS-105

Serial Number:	X132022

Date of calibration:  	02/21/14

Dates of sensor use:	07/02/14 to 09/08/16

Model #: 	CS-105

Serial Number:	P4910009

Date of calibration:  	02/29/2016

Dates of sensor use:	09/08/16 to present

Parameter: 	Wind Speed

Units: 	meter per second (m s-1)

Sensor type: 	18 cm diameter 4-blade helicoids propeller molded of polypropylene

Range: 	0-60 m s-1 (0-134 mph); gust survival 100 m s-1 (220 mph)

Accuracy: 	± 0.3 m s-1 (± 0.6 mph)

Model #: 	R.M. Young 05305 L-AQ Wind Monitor

Serial Number:	75311

Date of calibration: 	01/28/13

Dates of sensor use:	07/31/13 to 09/17/15

Model #: 	R.M. Young 05103-5 Wind Monitor

Serial Number:	83868

Date of calibration: 	 03/18/15

Dates of sensor use:	09/17/15 to present

Parameter:	Wind Direction

Units:	degrees

Sensor type: 	balanced vane, 38 cm turning radius

Range: 	0-360° mechanical, 355° electrical (5° open)

Accuracy: 	±3°

Model #: 	R.M. Young 05305 L-AQ Wind Monitor

Serial Number:	75311

Date of calibration: 	01/28/13

Dates of sensor use:	07/31/13 to 09/17/15

Model #: 	R.M. Young 05103-5 Wind Monitor

Serial Number:	83868

Date of calibration: 	 03/18/15

Dates of sensor use:	09/17/15 to present

Parameter: 	Photosynthetic Active Radiation

Units: 	mmoles m-2 (total flux)

Sensor type: 	High stability silicon photovoltaic detector (blue enhanced)

Light spectrum waveband:	400 to 700 nm

Temperature dependence: 	0.15% per °C maximum

Stability:  	<± 2% change over 1 yr. period

Response time:	10µs

Operating range:  	Temperature: -40°C to 65°C; relative humidity: 0 to 100%

Sensitivity:  	typically 5 µA per 1000 µmoles s-1 m-2

Model #: 	LI190SB

Serial Number:	Q31076

Multiplier:	1.488875125065510, changed on 08/22/12

Date of calibration:  	08/16/2011

Dates of sensor use:	08/22/2012 to 07/02/14

Model #: 	LI190SB

Serial Number:	Q22179

Multiplier:	1.635997172996880, changed on 07/02/14

Date of calibration:	02/14/14

Dates of sensor use:	07/02/2014 to 10/13/16

Parameter: 	Photosynthetic Active Radiation

Units: 	mmoles m-2 (total flux)

Sensor type: 	photodiode

Light spectrum waveband:	410 to 655 nm

Temperature dependence: 	0.06% ± 0.06 per °C

Stability:  	<± 2% change over 1 yr.

Operating range:  	Temperature: -40°C to 70°C; relative humidity: 0 to 100%

Cosine Response:	45° zenith angle: ± 2%; 75° zenith angle: ± 5%

Sensitivity:  	0.2mV per µmoles s-1 m-2

Model #: 	SQ110 Apogee Quantum Sensor

Serial Number:	19451

Date of calibration:  	01/11/2015

Dates of sensor use:	10/13/16 to present

Parameter: 	Precipitation

Units: 	millimeters (mm)

Sensor type: 	Tipping bucket/magnetic reed switch

Funnel collector diameter:	6.06 inches (16.4 cm)

Rainfall per tip: 	0.01 inch (0.254 mm)

Operating range: 	Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  	± 1% up to 1 in/hr; +0, -3% from 1 to 2 in/hr; +0, -5% from 2 to3 in/hr

Model #:  	TE525

Serial Number:	24701-899

Date of calibration:  	03/05/14

Dates of sensor use:  	07/02/14 to 09/17/15

Model #:  	TR525I-R3

Serial Number:	31194-902

Date of calibration:  	03/19/15

Dates of sensor use:  	09/17/15 to 10/13/16

Model #:  	TR525I-R3

Serial Number:	31194-902

Date of calibration:  	10/13/16

Dates of sensor use:  	10/13/16 to 01/04/2017

Parameter: 	Precipitation

Units: 	millimeters (mm)

Sensor type: 	Tipping bucket/magnetic reed switch

Funnel collector diameter:	6.36 inches (16.0 cm)

Rainfall per tip: 	0.004 inch (0.1 mm)

Operating range: 	Temperature: -20° to 50°C; Humidity: 0 to 100%

Heated thermostat set point:	10°C ± 3°C

Accuracy:  	± 2% up to 1 in/hr; 3% for up to 2 in/hr

Model #:  	52202

Serial Number:	TB13505

Date of calibration:  	11/03/2016

Dates of sensor use:  	01/04/2017 to present

The CR1000 has 2 MB Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional) is available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

CR1000 Serial Number:	5245

Year of Manufacture: 	2006

Date Installed: 	August 01, 2006

Date Calibrated:  	October 13, 2014

CR1000 Firmware Version (s):	OS31 uploaded on 02/02/17 to present.

OS29 uploaded on 09/08/16 to 02/02/17.

OS27 uploaded on 06/09/2014 to 09/08/16.

CR1000 Program Version(s):

| Program | Changes or notes included in the program |
| --- | --- |
| narpcmet_7.0_010417 | Last modified on January 04, 2017 by Daisy Durant. Included the new RM Young heated rain gauge Model 52202. |
| narpcmet_7.0_101316 | Last modified on October 13, 2016 by Daisy Durant. Included the new PAR sensor, Apogee SQ-110. |
| narpcmet_7.0_090816 | Uploaded the program after updating the OS27 to OS29, and after switching the air temp/relative humidity and barometric pressure sensor with recalibrated ones. |
| narpcmet_7.0_091815 | Last modified on September 18, 2015 by Daisy Durant. No major changes to the program, just added a note regarding the new cable installed on the rain gauge. |
| narpcmet_7.0_112414 | Last modified on November 24, 2014 by Daisy Durant. No major changes to the program, just added a note regarding the calibration of the CR1000. |
| narpcmet_7.0_070214 | Last modified on July 02, 2014 by Daisy Durant to include the specs of a new ATRH bought and installed in August. |
| narpcmet_6.4_091213 | Last modified on Sep 12 2013 by Daisy Durant. Installed HMP45C SN 1294801 (same as X3410026) Temperature & Relative Humidity Sensor. Needed changes in the program because this sensor was initially 7-wire but was switched to 6 at Campbell Scientific. |
| narpcmet_6.4_082113 | Last modified on August 21 2013 by Daisy Durant. Added the details of a different rain gauge to the program. |
| narpcmet_6.3_073113 | Last modified on July 31, 2013 by Daisy Durant. Uploaded new version of the program that includes models, serial numbers, frequency of calibration, and wiring of all sensors currently being used at this station. |
| narpcmet_V6.2_082312 | Last modified on August 23, 2012 by Daisy Durant and Jeff Adams (Campbell Sci). Added PAR Multiplier of 1.48887512506610. Added new AT/RH sensor HC2S3 and comment on wiring. |
| narpcmet_V6.2_082212 | Last modified on August 22, 2012 by Daisy Durant. Added PAR Multiplier of 1.488875125065510. |
| narpcmet_V6.1_082212 | Last modified on August 04, 2010 by Daisy Durant & Jay Poucher. Added PAR Multiplier of 1.582819445. |

10)  Coded Variable Definitions

Sampling station: 		Sampling site code: 		Station code:

Potter Cove			PC				narpcmet

11)  QAQC Flag Definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC, 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC Code Definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

## General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

## Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC	Incorrect Calibration Constant, Multiplier or Offset

SIW	Incorrect Wiring

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor Drift

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR	Sensor Removed

## Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CDF	Data Appear to Fit Conditions

CML	Snow melt from previous snowfall event

CRE*	Significant Rain Event

CSM*	See Metadata

CCU	Cause Unknown

CVT*	Possible Vandalism/Tampering

CWE*	Significant weather event

13)  Other Remarks and Notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Relative Humidity data greater than 100 are within range of the sensor accuracy of ± 3%.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 data logger. The maximum signal noise error for the Apogee sensor is ± 2.214 mmoles m-2 over a 15-minute interval.  These values are automatically flagged and coded as <1> (CAF).

The following are descriptions of different events that happened at the weather station from January to March of 2017, and explanations to the CSM (Comment-See Metadata) code used in the data file.

During 2017, the CDMO discovered an incorrect line in the CR1000 programming.  If RHumidity>100 and RHumidity<108 then RHumidity=100.  A decision was made by the DMC during 2006 to discontinue correcting >100 RH values to 100. This change was not been made in our program and has remained in each updated version up until May 10, 2017 12:15, when the correct program was uploaded.  By correcting all values >100 during data collection we may have missed erroneous values that could have indicated a problem with the RH sensor.  CSM coding has been added to all RH data from 2007 until the program change in 2017.

Elevated nighttime PAR data - We observed slightly elevated nighttime PAR throughout the year.  However, no visible light in the PAR range (400 - 700 nm) before sunrise or after sunset are to be recorded at nighttime by the PAR sensor.  This elevated nighttime PAR might be related, among others, to moisture trapped in the sensor, or electromagnetic noise such as ambient light pollution, electromagnetic radiation, etc.; thus, the data were flagged and coded as 1 CSM.  These small nighttime readings could not be due to moonlight due to the resolution of the PAR sensor and data logger.  To determine sunrise and sunset times for Prudence Island, the www.sunrisesunset.com website was used to flag and coded the data accordingly.

Power problems at the weather station had the station stop collecting data several times from January to March.  Each time we troubleshot the station with help on the phone from experts at CDMO and engineers from Campbell Scientific.  Since the weather station has many components, it was a trail and error situation.  As a result, for the periods of time when the station stopped collecting data, we flagged and coded as -2 GPF CSM.  The data collected immediately before and after a period where the station stopped collecting data were flagged and coded -3 GQR CSM because it is possible that those timestamps are not a full 15 minutes of averaged 5 second data.  As a result,

-3 GQR CSM		-2 GPF CSM			-3 GQR CSM

Before			No data collected		After

Jan 01 15:45		Jan 01 16:00 – Jan 04 10:15	Jan 04 10:30

Jan 06 08:45		Jan 06 09:00 – Jan 09 12:30	Jan 09 12:45

Jan 31 12:45		Jan 31 13:00 – Feb 02 14:30	Feb 02 14:45

Feb 03 11:45		Feb 03 12:00 – Feb 21 12:15	Feb 21 12:30

January 04.  A new tipping bucket rain gauge, RM Young Heated Rain Gauge (Model 52202), was installed during this day.  This heated rain gauge has a thermostat-controlled internal heater that melts snow or other frozen precipitation recording it as liquid precipitation.  After connecting the heated rain gauge to the CR1000, we uploaded the operating system (OS29) and the program (narpcmet_7.0_010417) with the appropriate updates.  As a result,

- 01/04 10:30 data rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

February 02.  Updated the operating system to OS31 and uploaded the program (narpcmet_7.0_010417).  As a result,

- 02/02 14:45 data rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

February 21.  Installed a different data logger, a CR1000M (SN 55704, Calibrated on May 24, 2016) loaned by Campbell Scientific as a troubleshooting option to the collection problem mentioned above.  The CR1000M operating systems (OS31) and our program (narpcmet_7.0_010417) were uploaded.  As a result,

- 02/21 12:30 data rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

February 23 – March 22.  The photosynthetic active radiation sensor was disconnected from the station as a troubleshooting option to the on and off collection of data at the station after this new sensor was installed in October of 2016.  As a result,

- 02/23 08:45 Data rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

- 02/23 09:00 – 14:45 no data collected because the station was powered down.

- 02/23 15:00 the station was powered back up after disconnecting the PAR sensor from the CR1000M.  Data rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

- 02/23 15:15 – 03/22 10:15 PAR data was rejected because the PAR sensor was removed.  However, we did not turn off the sensor in the program, which kept recording numbers.

- 03/20 10:45 the station was power down.  Data rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

- 03/20 11:00 data not collected because the station was power down to remove the PAR sensor jumper cable connected to the data logger.

- 03/20 11:15 the station was power back up.  Data rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

March 22.  The weather station was power down to connect back the PAR sensor.  Engineers at Campbell Scientic suggested installing a new SC12 cable as a troubleshooting option to the problem at the station. If the SC12 had any corrosion, it might be causing the power failures. This activity took less than 15 minutes, as a result:

- 03/22 10:30 the station was power down and back up to install the PAR sensor and new SC12 cable.  Data rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

May 10.  The weather station was power down to switch back our CR1000 (SN 5245).  As a result,

- 05/10 09:45 the station was powered down.  These data were rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

- 05/10 11:45 the station was powered back up, by mistake.  The data were rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

- 05/10 12:15 the station was powered back up and the program was reloaded.  The data were rejected due to QAQC checks.  It is possible that this timestamp is not a full 15 minutes of averaged 5 second data.

Calibration Certificate - RM Young Heated Rain Gauge, Model 52202

Date installed – January 04, 2017

Pilson, M.E.Q. 1985. On the residence time of water in Narragansett Bay. Estuaries 8:2–14.

Narragansett Bay National Estuarine Research Reserve.  2007.  An Ecological Profile of the Narragansett Bay National Estuarine Research Reserve.  K.B. Raposa and M.L. Schwartz (eds.), Rhode Island Sea Grant, Narragansett, R.I. 176pp.