North Inlet – Winyah Bay (NIW) NERR Meteorological Station Metadata

January – December 2008

Latest Update:  July 13, 2011

# I.  Data Set & Research Descriptors

- Principal investigator(s) & contact persons

Address:

Baruch Marine Field Laboratory

University of South Carolina

PO Box 1630, Georgetown, SC 29442

Phone: (843) 546-6219

Baruch Institute home page:  http://inlet.geol.sc.edu/

North Inlet – Winyah Bay NERR home page:  http://www.northinlet.sc.edu/

Contact persons:

Wendy Allen, 	Manager

E-mail: wendy@belle.baruch.sc.edu, ext: 229

Dr. Erik Smith, Research Coordinator

E-mail: erik@belle.baruch.sc.edu, ext: 235

Amy Willman, Research Specialist

E-mail:  willman@belle.baruch.sc.edu, ext. 264

Tracy Buck, Research Specialist

E-mail: tracy@belle.baruch.sc.edu, ext: 259

- Entry verification

Data are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

Amy Willman is responsible for all data management of the Meteorological Data sets.

3)  Research objectives (Campbell Weather Station)

The principal objective is to record long-term meteorological data for North Inlet-Winyah Bay National Estuarine Research Reserve (NIW NERR) in order to observe any environmental changes or trends over time.  The data are also used to compliment other biological, chemical, or physical research conducted throughout the reserve, including YSI data.

- Research methods:

a.  Data Collection Methods

The Campbell Scientific CR1000 datalogger uses 5-second data downloaded from the weather station sensors to calculate 15-minute data.  The CDMO Data Logger Program (NIWOLMET_v3.0_092406) was loaded into the CR1000 on September 26, 2006 and controls the sensors and data collection schedule (section 4b).  The CR1000 then interfaced with the Loggernet software supplied by Campbell Scientific via a short haul modem at Baruch Marine Field Lab (BMFL).  The software is located on a computer in the System Wide Monitoring Lab at BMFL and the data are uploaded (every 5 minutes) via the short haul modem.  The data were saved as a monthly raw data file (SWMP1006.dat) onto a separate hard drive and than backed up onto the BMFL server.  All collected data are then QA/QC’ed monthly as per the entry verification method in Section 2.  Any error messages are further investigated and the data is changed (if necessary), noted as anomalous, or noted but documented as correct data.

Periodically, sensors on the weather station are inspected for damage or debris.  If any are found, it is repaired and/or cleaned.  The rain gauge tends to collect the most debris and is cleaned out weekly.  Sensors are removed and sent back to Campbell Scientific or independent manufacturers for calibrations annually or bi-annually.

b.  Telemetry/Real-time Data:

Campbell Scientific data telemetry equipment was installed at the Oyster Landing station on 07/06/2006 and transmits data to the NOAA GOES satellite, NESDIS ID 3B01F504.  (Where 3B01F504 is the GOES ID for that particular station.)  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data is collected in the following formats for the CR1000:

Averages from 5-second data:

- Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Wind Direction Standard Deviation (degrees)from 5-second data(began collecting as a standard parameter at NIW 12/1/2008) , Battery Voltage (volts), Total Solar Radiation (watts/m2) (TSR data are not included in the dataset but are available from the NIW NERR)

Maximum, Minimum, and their times from 5-second data:

- Air Temperature (°C) (these data are not included in the dataset but are available from the NIW NERR

- Maximum Wind Speed (m/s) (began collecting as a standard parameter at NIW 5/1/2008)

Totals:

- Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm) (began collecting as a standard parameter at NIW 4/1/2008)

c.  Other

LiCor:

From October 19, 1997 (Julian Day 292) through October 21, 2003 (Julian Date 295), all values less than 0 were altered in the raw data to read 0. As of October 21, 2003 to present, LiCor values less than 0 were no longer converted to 0 in the CR1000 programming, because these values may indicate an incorrect multiplier, calibration problems, or a sensor malfunction.  LiCor values were retained because there is a possible maximum signal noise error of +/- 2.214 millimoles/m2 over the 15 minute collection interval.

Relative Humidity:

From October 19, 1997 (Julian Day 292) through May 15, 2003 (Julian Date 135), all Relative Humidity values over 100% were altered in the raw data to read 100%.  As of May 15, 2003 to present, Relative Humidity values over 100% were no longer converted to 100% in the CR1000 programming, since these values may indicate super saturated air, calibration problems, or a sensor malfunction.  The sensitivity of the Temperature/Humidity sensor allows for a 2% to 3% error, therefore, data in this range were retained.

Wind Direction:

The Wind Direction sensor may output slightly negative numbers when the wind direction is in the 5 degree dead band zone (355-0 degrees).  The resulting negation value can be interpreted as 0 degrees.

5)  Site location and character:

The NIW NERR is located on the Southeastern Atlantic coast of the United States in two tidal estuaries, North Inlet and Winyah Bay, near Georgetown, South Carolina.  The North Inlet estuary, located approximately 10 km east of Georgetown, is a bar-built Class C type estuary (Pritchard, 1955).  The North Inlet estuary is composed of numerous winding tidal creeks, and is considered a pristine tidal estuary due to minimal anthropogenic impacts.  The watershed drains a 24.8 km2 area of mostly pine forest and a moderately developed residential watershed to the north.  The Winyah Bay estuary, classified as a Class B type estuary by Pritchard (1955) and composed of waters that originate in the Blue Ridge Mountains of North Carolina, is one of the largest river-estuary ecosystems on the Eastern Seaboard.  It is located 14.4 km south of North Inlet.  Winyah Bay drains the sub-basins of 6 major rivers, which are heavily impacted by agriculture, mining, and industry.  The rivers drain approximately 46,736 km2 of uplands and marshes.

The weather station is located at Oyster Landing (OL) pier.  It consists of a 4.5 meter aluminum tower to which all of the sensors, solar panel, telemetry antennae, and enclosure are attached.  The wind sensor is mounted slightly above the tower at an elevation of 4.6 meters.  The Eppley and Licor sensors are just below at an elevation of  4.0 meters.  These two sensors are mounted on a pole that extends horizontally away from the tower to prevent any shadowing effects from the wind sensor.  The temperature/humidity sensor is at an elevation of 3.7 meters and is contained within a gill radiation shield.  The solar panel is at an elevation of 3.0 meters.  The barometric pressure sensor and all of the hardware are located inside of the enclosure that is attached to the tower at an elevation of 1.2 meters.  All elevations are measured from the platform (ie. the base of the tower).  The height of the Oyster Landing platform is 3.47m above mean sea level.  The tipping bucket rain gauge is located 8.0 meters away from the tower to the NE.  The sensors were all wired to the CR1000 following the protocol in the Campbell Scientific Sensor Manual.

Description of the specific sampling station follows:

Oyster Landing (OL) - (lat.  33:20:58; long.  79:11:34). The OL meteorological monitoring site is considered a fairly pristine and undisturbed area located at the end of the OL pier which is also where one of the NIW NERR water quality station site is located.  The pier stretches into the upper reaches of Crabhaul Creek in the mid western portion of North Inlet.  The sampling site is located approximately 2.8 km from the headwaters of Crabhaul Creek.  The creek directly drains pine forested uplands and wetlands.  Salinity can range from 0 - 32 ppt. and average tidal flux is approximately 1.4m.  The creek has an average depth of ~2 m Mean High Water (MHW) and average width of ~150m MHW.  The bottom mostly is comprised of oyster shell hash with some fine sediment and detritus.

6)  Data Collection period:

Meteorological data has been collected by automated weather stations at the Oyster Landing Pier since 1982.  Data collection using NERR protocols began July 2, 1997.  Long-Term data and metadata from 1982 through 2000 are available online at http://links.baruch.sc.edu/data/.  SWMP Data and metadata for 2001 to 2007 are available online at http://cdmo.baruch.sc.edu/.  Data were collected for 2008 from January 1, 2008 00:00 –December 31, 2008 23:45.

7)  Distribution

According to the Ocean and Coastal Resource Management Data Dissemination Policy for the NERRS System-wide Monitoring Program,

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from the NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance/quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient of third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR weather data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see section 1. Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/ .  Data are available in comma separated format.

8)  Associated researchers and projects

The NERR Water Quality Monitoring Project also has a station located at Oyster Landing.  The principal objective of this study is to record long-term water quality data for North Inlet - Winyah Bay in order to observe any physical changes or trends in water quality over time.  Four sites were chosen; two to represent our pristine site and the other two to represent impacted sites.  The Oyster Landing monitoring site is considered a fairly pristine and undisturbed area located at the end of the Oyster Landing pier, which is also where the NIW NERR Meteorological station site is located.  The Clambank site is located near the center of the Reserve and is considered pristine.  Debidue Creek and Thousand Acre are both located in waterways that receive run-off from heavily developed areas.  Measurements are taken every 30 minutes over roughly two week collecting periods with an YSI 6600 EDS data logger. The parameters measured are specific conductivity, salinity, dissolved oxygen, water temperature, pH, turbidity, and water level.  These parameters are important indicators of habitat quality for numerous estuarine species and to determine health criteria and human uses.

A water chemistry program is associated with the NIW NERR core-monitoring program. Variables sampled include: chlorophyll dissolved organic carbon, nitrate-nitrite, orthophosphate, and ammonia.  (See documentation on the NIW http://www.northinlet.sc.edu/ home page for further details).  Samples for the 20 day program and the monthly grabs are collected at each of the four NERR water-quality monitoring stations.  Thousand Acre is our permanent monitoring station for the NERR monitoring program beginning monitoring in 1994.   Oyster Landing and Clambank Creek were also sampled beginning in 1993. Debidue Creek monitoring began in 1998.  These stations are also included in our NIW core-monitoring program.

a)  Monthly grab samples

Our monthly grab samples were initiated across the NERR platform on February 1, 2001 as a method of providing baseline information on inorganic nutrient and chlorophyll a water quality status within the National Estuarine Research Reserve system.  The North Inlet-Winyah Bay National Estuarine Research Reserve (NIW NERR) began collecting monthly grab samples on April 17, 2002.

b)  Diel Sampling Program (20 day program)

The NIW NERR 20 day water chemistry database is a continuation of the Long-Term Ecological Research (LTER) Daily Water Sample 1978-1993 database whose purpose was to monitor nutrient cycling in the North Inlet Estuary, Georgetown, SC.  The Long-Term Ecological Research Project at North Inlet was part of the national network of LTER sites and was funded by the National Science Foundation from October 1980 through 1993.  The NERR 20 day water chemistry program was initiated in June of 1993 to continue the monitoring of nutrients in the North Inlet Estuary system and also to continue the long-term database on the nutrient concentration dynamics of the North Inlet salt marsh estuarine system.  A larger purpose of this monitoring program is to combine the water chemistry, chlorophyll a, and suspended sediment monitoring data with other existing North Inlet ecological monitoring data to provide ecosystem level information and understanding.  Because the LTER daily water sampling collections were taken every day at 10:00am EST, it was determined that the nutrient data are biased for spring high tides; not all tides levels would be represented.  The new NERR water chemistry sampling protocols (based on a 10 day cycle) sampled all tidal stages over the years.

A long term fish survey is also done at the Oyster Landing site bi-weekly.  Species present, total weight, individual weights and standard lengths of fish that utilize the marsh at high tide are recorded.  In the winter months (November through February), this sampling is done only once monthly.

The National Atmospheric Deposition Program (NADP) (http://nadp.sws.uiuc.edu/) is a nationwide network of precipitation monitoring sites. The network is a cooperative effort between many different groups, including the State Agricultural Experiment Stations, U.S. Geological Survey, U.S. Department of Agriculture, and numerous other governmental and private entities.  The purpose of the network is to collect data on the chemistry of precipitation for monitoring of geographical and temporal long-term trends. The precipitation at each station is collected weekly according to strict clean-handling procedures. It is then sent to the Central Analytical Laboratory where it is analyzed for hydrogen (acidity as pH), sulfate, nitrate, ammonium, chloride, and base cations (such as calcium, magnesium, potassium and sodium).

For a more detailed description of the other 85 projects being conducted at the Baruch Marine Field Lab in the NIW NERR, go to the Baruch website.  To retrieve the list and the project abstracts go to http://www.cas.sc.edu/baruch/researchers.html then go to the Quick Link box and click on Research Projects Lists.

II.  Physical Structure Descriptors:

- Sensor specifications, operating range, accuracy and date of last calibration:

Parameter:  Temperature

Units:  Celsius

Sensor type:  Platinum resistance temperature detector (PRT)

Operating Temperature:  -40 to +60°C

Range: -40 to +60°C

Accuracy: ± 0.2 °C @ 20°C

Model #:  HMP45C Temperature and Relative Humidity Probe

Serial #:  U2520001

Dates in Use:

8/11/99 - 5/17/01

9/18/01 - 5/13/03

2/18/04 - 7/12/04

8/6/04 - 9/2/04

2/15/05 - 4/18/05

6/9/05 - 12/21/05

6/6/07-8/26/08

Date of Last calibration:  04/04/07

Serial #:  A1710069

Dates in Use:

12/21/05 – 6/6/07

8/26/08 – 2/16/09

Date of Last calibration:  7/28/08

Parameter:  Relative Humidity

Units:  Percent

Sensor type:  Vaisala HUMICAP © 180 capacitive relative humidity sensor

Range:  0-100% non-condensing

Accuracy at 20°C:  +/-2% RH (0-90%) and +/-3%(90-100%)

Temperature dependence of RH measurement:  +/- 0.05% RH/°C

Model # HMP45C Temperature and Relative Humidity probe

Serial #:  U2520001

Dates in Use:

8/11/99 - 5/17/01

9/18/01 - 5/13/03

2/18/04 - 7/12/04

8/6/04 - 9/2/04

2/15/05 - 4/18/05

6/9/05 - 12/21/05

6/6/07-8/26/08

Date of Last calibration:  04/04/07

Serial #:  A1710069

Dates in Use:

12/21/05 – 6/6/07

8/26/08 – 2/16/09

Date of Last calibration:  7/28/08

Parameter:  Barometric Sensor

Units:  millibars (mb)

Sensor type:  Vaisala Barocap © silicon capacitive pressure sensor

Operating Range:  Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ±0.5 mb @ 20°C; +/-2mb @ 0°C to 40°C; +/-4mb @ -20°C to 45°C; +/-

6.0mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Model # Viasala CS-105

Serial #:  W1530030 (SN R4230013 replaced under warranty)

Dates in Use:

5/17/01-5/13/03

2/15/05-6/6/07

Date of Last calibration: 12/16/2004

Serial #:  U1120009

Dates in Use:

8/11/99-5/17/01

5/13/03-2/15/05

6/6/07-present

Date of Last calibration:  2/27/07

Parameter:  Wind Speed

Units:  meter per second (m/s)

Sensor type:  18 cm diameter 4-blade helicoids propeller molded of polypropylene

Range: 0-60 m/s (134mph); gust survival 100 m/s (220 mph)

Accuracy:  +/- 0.3 m/s

Model #:  RM Young 05106-5A Marine Wind Monitor

Serial #:  WM65129

Dates in Use:  2/15/05-6/6/07

Date of last calibration:  1/13/05

Parameter:  Wind Direction

Units:  degrees

Sensor type:  balanced vane, 38 cm turning radius

Range:  360° mechanical, 355° electrical (5° open)

Accuracy:  +/- 3 degrees

Model #:  RM Young 05106-5A Marine Wind Monitor

Serial #:  WM65129

Dates in Use:  2/15/05-6/6/07

Date of last calibration:  1/13/05

Parameter Wind Speed

Units:  meter per second (m/s)

Sensor type:  12 cm diameter cup wheel assembly, 40 mm diameter hemispherical cups

Range:  0-50 m/s (112 mph); gust survival 60 m/s (134 mph)

Accuracy:  +/- 0.5 m/s (1.1 mph)

Model #:  Wind Sentry 03001-5

Serial #:  8702

Dates in Use:

7/97-5/17/01

5/13/03-2/15/05

6/6/07-present

Date of last calibration:  4/24/07

Parameter Wind Direction

Units:  degrees

Sensor type:  balance vane, 16 cm turning radius

Range:  360° mechanical, 355° electrical (5° open)

Accuracy:  +/- 5°

Model #:  Wind Sentry 03001-5

Serial #:  8702

Dates in Use:

7/97-5/17/01

5/13/03-2/15/05

6/6/07-present

Date of last calibration:  4/24/07

LI-COR Quantum Sensor (PAR)

Units:  mmoles m-2 (total flux)

Sensor type:  High stability silicon photovoltaic detector (blue enhanced)

Light spectrum waveband:  400 to 700 nm

Temperature dependence:  0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity 0 to 100%

Sensitivity:  typically 8 μA per 1000μmoles s-1 m-2

Model #LI190SZ; Serial #:  Q34301

Dates in Use:			Multiplier:

3/6/06 – 7/5/06		1.66

Date of last calibration:  12/14/04

Model # LI190SB; Serial #:  Q26604

Dates in Use:			Multiplier:

New 7/97

7/97 - 4/99			???

4/99 - 4/19/00			1.27

3/01 - 6/26/01			1.27

6/19/03 - 2/15/05		1.18

7/6/06 - 8/25/06		1.42

6/6/07 - 9/16/08		1.42

Date of last calibration:  4/7/2006

Model # LI190SZ; Serial #:  Q18206

Dates in Use:  			Multiplier:

New 4/00

4/19/00 - 3/01			1.99

6/26/01 - 6/19/03		1.78

2/15/05 - 3/6/06		1.79

8/25/06 – 6/6/07		1.79

9/16/08 – present		1.86

Date of last calibration:  7/21/08

Precipitation

Units:  millimeters (mm)

Sensor type:  Tipping Bucket Rain Gauge

Rainfall per tip:  0.01 inch (.254 mm)

Operating range:  Temperature:  0°C to +/- 50°C; Humidity:  0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Model #:  TE525WS

Dates in Use:  5/17/01 - present

Date of last calibration:  1/31/08

The CR1000 has 2 MB Flash EEPROM that is used to store the Operating System. Another 128 K Flash is used to store configuration settings. A minimum of 2 MB SRAM is (4 MB optional) is available for program storage (16K), operating system use, and data storage. Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Dates CR1000 in use:  07/06/2006 to present

10) Coded variable indicator and definition.

Sampling Station = Oyster Landing

Sampling Site Code = OL

Station code = niwolmet

11) QAQC flag definitions – This section details the automated primary and secondary QAQC flag definitions.

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.  During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0	Passed Initial QAQC Checks

1	Suspect Data

2	 Open – reserved for later flag

3	Open – reserved for later flag

4	Historical Data:  Pre-Auto QAQC

5	Corrected Data

12)  QAQC code definitions – This section details the secondary QAQC Code definitions used in combination with the QAQC flags above.

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SIC	Incorrect Calibration Constant, Multiplier or Offset

SNV	Negative Value

SOC	Out of Calibration

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR	Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CDF	Data Appear to Fit Conditions

CRE	Significant Rain Event

CSM	See Metadata

CVT	Possible Vandalism/Tampering

13) Other remarks/notes:

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR  values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Cumulative precipitation data are recorded from 00:00 to 23:59 with the daily total recorded at the midnight mark (00:00). The midnight CumPrcp value is actually the total from the previous day.

On 3/31/2008 two new parameters were added to the list of SWMP parameters:  Cumulative Precipitation and Total Solar Radiation.  The CumPrcp parameter calculates the total precipitation in a standard 24 hour day in millimeters.  The TotSoRad parameter is a 15 minute average for Total Solar Radiation which is measured by the Eppley Pyranometer sensor and is recorded in watts/m2.  Please contact NIW NERR for 2008 data for these new parameters.

The original program v3.0_092406 was modified and renamed several times throughout the year as specified below:

From January 2008 until September 2008 elevated nighttime PAR readings were recorded.  The reasons are unknown; however, the elevated values may have been caused by an aging sensor.  During this time any nighttime (after sunset and before sunrise) readings >2.214mmols/m2 were rejected.  See below for specific dates and times.

There was a noticeable increase in PAR readings after replacing the PAR sensor (Q26604) with a freshly calibrated one on 9/16/2008 (Q18206).  Sensor Q26604 has since been retired due to problems relating an aging sensor.

The causes of the high humidity readings that are outside of the accuracy range of the HMP45C Temp/Humidity sensor are unknown.  These high readings began showing up in data on 11/13/08 at 5:30.  At our site, it is possible for salt to accumulate on sensors which has an affinity for water.  Several attempts had been made to maintenance the sensor ie. inspecting, rinsing, and drying gill radiation shield as well as sensor components to remove any excess moisture.  These attempts were ultimately unsuccessful.  Our backup sensor was sent out for calibration on 1/29/09 so that we could eventually replace the sensor that is currently installed.

Suspect Data

January 2008

For the following dates and times rainfall data were corrected and changed to 0.  Tips

during these times were caused during maintenance when there was not any actual

rainfall.

Date		Time

1/9/2008	12:00 – 14:00

For the following dates and times rainfall data were corrected and changed to 0.  Tips

were caused during an unsuccessful attempt to calibrate the rain gauge.  There was not

any actual rainfall during these times.

Date		Time

1/11/2008	13:15 – 15:00 (51 tips)

1/15/2008	11:15 – 11:45 (54 tips)

For the following dates rainfall data are considered suspect due to the

rain gauge being out of calibration.  The attempts to calibrate on 1/11/2008 and 	1/15/2008 were unsuccessful (51 tips) so the actual rainfall values should be slightly 	more than recorded by the datalogger.

Date

1/11/2008 – 1/31/2008

For the following dates and times rainfall data were corrected and changed to 0.  Tips

were caused during maintenance (remounted and leveled) and a successful

attempt to calibrate the rain gauge.  There was not any actual rainfall during these times.

Date		Time

1/31/2008	15:15 – 17:00 (2 attempts; 56 tips & 57 tips)

Rejected Data

January 2008

For the following dates and times all wind data were rejected due to the sensor being

realigned to take the magnetic declination of 8º W into account.  All wind data from 	here on out will be recorded in respect to True North.

Date		Time

1/31/2008	13:00 – 15:00

For the following dates and times all PAR data were rejected due to the data being outside of the sensor range of +/- 2.214 mmol/m2 for nighttime readings.

Date		Time

## 1/8/2008	02:45, 03:30-05:15

## 1/9/2008	02:00-07:00

## 1/10/2008	05:00-07:00, 20:00

1/11/2008	20:45, 21:00, 21:30-23:45

1/12/2008	23:15

1/23/2008	02:15-02:30

February 2008

For the following dates and times all PAR data were rejected due to the data being outside of the sensor range of +/- 2.214 mmol/m2 for nighttime readings.

Date		Time

2/1/2008	05:45-06:00

2/24/2008	03:30-06:45, 18:45-23:45

2/25/2008	00:00-06:30, 18:45-23:45

2/26/2008	00:00-06:00

## March 2008

For the following dates and times all PAR data were rejected due to the data being outside of the sensor range of +/- 2.214 mmol/m2 for nighttime readings.

Date		Time

3/1/2008	03:00-3:30, 04:45-05:30

3/3/2008	01:45-06:30

3/4/2008	02:30-06:15

3/6/2008	02:30, 03:45, 05:00, 06:00, 06:15

3/7/2008	03:45-04:15, 20:15-23:45

3/8/2008	00:00-02:15

3/19/2008	21:00, 21:15, 23:30

3/30/2008	21:15-23:30

3/31/2008	02:00, 03:15, 03:30

## April 2008

For the following date and time all data were rejected due to sending a new program

(v5.5_041608) that will now contain a flag column for MaxWSpd.

Date		Time

4/21/2008	8:15

For the following dates and times all PAR data were rejected due to the data being outside of the sensor range of +/- 2.214 mmol/m2 for nighttime readings.

Date		Time

4/2/2008	02:45, 03:00

4/3/2008	05:45, 20:15-23:45

4/4/2008	00:00-01:00, 01:45, 05:00-06:30

4/5/2008	03:00

4/6/2008	04:00-05:30

4/7/2008	04:00-05:45

4/9/2008	19:15-23:45

4/10/2008	00:00-05:30, 19:00-23:45

4/11/2008	00:00-05:30

4/19/2008	05:00, 05:15, 23:00-23:45

4/20/2008	00:00-01:00

4/26/2008	00:45-05:00, 23:30, 23:45

4/27/2008	00:00-05:15, 21:45-23:45

4/28/2008	00:00-05:15

May 2008

For the following dates and times all PAR data were rejected due to the data being outside of the sensor range of +/- 2.214 mmol/m2 for nighttime readings.

Date		Time

5/2/2008	02:15-05:00, 20:00-23:45

5/3/2008	00:00-05:00, 19:45-22:15

5/4/2008	002:15-05:00, 23:15-23:45

5/5/2008	00:00-05:00

5/8/2008	19:30-23:45

5/9/2008	00:00-05:00

5/20/2008	20:15-22:30

5/29/2008	00:30, 00:45, 01:15, 01:45-03:15

5/31/2008	02:30-04:45, 20:30-21:15

June 2008

For the following dates and times all PAR data were rejected due to the data being outside of the sensor range of +/- 2.214 mmol/m2 for nighttime readings.

Date		Time

6/1/2008	01:00-04:45, 20:00, 20:15

6/6/2008	01:45-02:15, 20:00, 20:15, 22:30, 22:45, 23:45

6/7/2008	00:00-04:45, 20:00-23:45

6/8/2008	00:00-04:45

6/9/2008	01:15-04:45, 20:45, 21:00, 22:30-23:45

6/10/2008	00:00-04:45, 19:45-23:15

6/11/2008	02:15-04:45, 22:30-23:45

6/12/2008	00:00-04:45, 19:45-23:45

6/13/2008	00:00-04:45, 19:45-23:45

6/14/2008	00:00-04:45, 19:45-23:45

6/15/2008	00:00-04:45, 19:45-23:45

6/16/2008	00:00-04:45

6/25/2008	20:00-23:45

6/26/2008	00:00-04:45, 23:45

6/27/2008	00:00-04:45, 20:30-23:45

6/28/2008	00:00-04:45, 20:00-23:45

6/29/2008	00:00-04:45

## July 2008

For the following date and time PAR data were rejected due to being outside of the

sensor range of +/- 2.214 mmol/m2

Date		Time

7/11/2008	02:30

For the following dates and times all PAR data were rejected due to the data being outside of the sensor range of +/- 2.214 mmol/m2 for nighttime readings.

Date		Time

## 7/2/2008	00:00-04:45, 22:00-23:45

7/3/2008	00:00-04:45, 20:00-23:45

7/4/2008	00:00-04:45, 20:00-23:45

7/5/2008	00:00-04:45, 20:00-23:45

7/6/2008	00:00-05:00, 20:00-23:45

7/7/2008	00:00-01:00, 02:30-05:00, 20:00-23:45

7/8/2008	00:00-01:30, 20:00-23:45

7/10/2008	20:00-23:45

7/11/2008	00:00-01:00, 01:30-04:00, 04:30-05:00

7/25/2008	03:45-04:00

## August 2008

For the following date and time PAR data were rejected due to being outside of the

sensor range of +/- 2.214 mmol/m2

Date		Time

8/8/2008	01:30

8/8/2008	01:45

For the following dates and times all PAR data were rejected due to the data being outside of the sensor range of +/- 2.214 mmol/m2 for nighttime readings.

Date		Time

8/7/2008	19:45-20:00, 20:30-21:30

8/8/2008	01:30-01:45

8/12/2008	19:30-19:45

8/13/2008	01:00

For the following date & time Temp. and Humidity data were rejected due to removing

the sensor and replacing with a newly calibrated one.

Date		Time

8/26/2008	13:15

### September 2008

For the following dates and times all PAR data were rejected due to the data being outside of the sensor range of +/- 2.214 mmol/m2 for nighttime readings.

Date		Time

9/1/2008	04:45-05:15

For the following dates & times all PAR data were rejected due to removing the sensor 	and replacing with a newly calibrated one.  Some maintenance and troubleshooting was 	also done during this time frame to ensure that the new sensor worked properly.

Date		Time			Date		Time

9/16/2008	12:15       through 	9/17/2008 	14:45

For the following date and time all data were rejected due to sending a new program (5.5_091708) that has a new PAR mV of 7.5 instead of 25.

Date		Time

9/17/2008	13:00

For the following date and time Eppley data were rejected due to forgetting to put the 	multiplier in immediately after sending the new program.

Date		Time

9/17/2008	13:15

For the following data and time all data were rejected due to sending a new program 	(original 5.5_041608).  It was determined that the PAR mV could stay at 25.

Date		Time

9/17/2008	14:45

November 2008

For the following dates and times Humidity data were rejected due to being outside of 	the sensor range of +/- 3%.

Date		Time

11/13/2008	5:30 – 9:30

11/14/2008	7:45

11/29/2008	12:15 – 23:45

11/30/2008	0:00 – 5:15

December 2008

For the following dates and times Humidity data were rejected due to being outside of 	the sensor range of +/- 3%.

Date		Time

12/10/2008	8:15 – 10:00

12/16/2008	5:30 – 12:45

12/16/2008	18:00 – 23:45

12/17/2008	0:00 – 14:15

12/19/2008	2:00 – 8:30

12/25/2008	22:30 – 23:15

12/27/2008	17:15 – 23:45

12/28/2008	0:00 – 8:45

12/29/2008	23:30 – 23:45

12/30/2008	0:00 – 00:15

12/30/2008	2:15 – 4:30

Missing Data

For the following date and time all data were missing due powering down system to

remove the jumper wire from the PAR sensor.

Date		Time

9/17/2008	12:15

For the following date and time all data were missing due to sending a new program (v5.5_121008).

Date		Time

12/10/2008	10:45