North Inlet – Winyah Bay (NIW) NERR Meteorological Station Metadata

January – September, 2015

Latest Update: December 6, 2016

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact

Address:

Baruch Marine Field Laboratory

University of South Carolina

PO Box 1630, Georgetown, SC 29442

Phone: (843) 546-3623

North Inlet – Winyah Bay NERR home page:  http://www.northinlet.sc.edu/

Contact persons:

Dr. Erik Smith, Research Coordinator

E-mail: erik@belle.baruch.sc.edu, (843) 904-9035

Tracy Buck, Research Specialist

E-mail: tracy@belle.baruch.sc.edu, (843) 904-9027

2)  Entry verification

Data are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from or LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

Tracy Buck was responsible for maintenance of the meteorological station for 2015 as well as the associated data management.

3)  Research objectives

The principal objective is to record long-term meteorological data for North Inlet-Winyah Bay National Estuarine Research Reserve (NIW NERR) in order to observe any environmental changes or trends over time.  The data are also used to compliment other biological, chemical, or physical research conducted throughout the reserve, including YSI data.

4)  Research methods

The Campbell Scientific CR1000 datalogger uses 5-second data downloaded from the weather station sensors to calculate 15-minute data.  Data is downloaded bimonthly to monthly directly with a laptop connected to the datalogger via a RS232 connection.  The data are saved as a raw data file onto a separate hard drive and then backed up onto the BMFL server.  All collected data are then QA/QC’ed monthly as per the entry verification method in Section 2.

Sensors on the weather station are inspected for damage or debris on a monthly basis, and real-time sensor readings are compared to an external source as an error check.  Sensors are removed and sent back to Campbell Scientific or independent manufacturers for calibrations annually or bi-annually.

Campbell Scientific data telemetry equipment was installed at the Oyster Landing station on 07/06/2006 and transmits data to the NOAA GOES satellite, NESDIS ID 3B01F504. 3B01F504 is the GOES ID for the Oyster Landing MET station.  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts), Total Solar Radiation (watts/m2)

Maximum and Minimum Air Temperature (°C) and their times from 5-second data (these data are available from the Reserve)

Maximum Wind Speed (m/s) and time from 5-second data

Wind Direction Standard Deviation (degrees)

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm)

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- yearly recalibration

- Rain Gauge- yearly recalibration

- Wind Speed/Direction- yearly or every 2 years (depending on the sensor)

- Barometric Pressure- every 2 years recalibration

- PAR- every 2 years recalibration

- CR1000-every 5 years (required beginning 2014, one year initial grace period)

- Total Solar Radiation - Eppley 8-48 - every 5 years, but annual recalibration encouraged

NOTE:

LiCor:

From October 19, 1997 (Julian Day 292) through October 21, 2003 (Julian Date 295), all values less than 0 were altered in the raw data to read 0. As of October 21, 2003 to present, LiCor values less than 0 were no longer converted to 0 in the CR1000 programming, because these values may indicate an incorrect multiplier, calibration problems, or a sensor malfunction.  LiCor values were retained because there is a possible maximum signal noise error of +/- 2.214 millimoles/m2 over the 15 minute collection interval.

Relative Humidity:

From October 19, 1997 (Julian Day 292) through May 15, 2003 (Julian Date 135), all Relative Humidity values over 100% were altered in the raw data to read 100%.  As of May 15, 2003 to present, Relative Humidity values over 100% were no longer converted to 100% in the CR1000 programming, since these values may indicate super saturated air, calibration problems, or a sensor malfunction.  The sensitivity of the Temperature/Humidity sensor allows for a –2% to 103% error, therefore, data in this range were retained.

Total Solar Radiation:

As of Jan. 1, 2009, Total Solar Radiation data started being ingested by the CDMO upload site as an optional parameter.  The TotSoRad parameter is a 15 minute average for Total Solar Radiation which is measured by the Eppley Pyranometer sensor and is recorded in watts/m2.

5)  Site location and character

The NIW NERR is located on the Southeastern Atlantic coast of the United States in two tidal estuaries, North Inlet and Winyah Bay, near Georgetown, South Carolina.  The North Inlet estuary, located approximately 10 km east of Georgetown, is a bar-built Class C type estuary (Pritchard, 1955).  The North Inlet estuary is composed of numerous winding tidal creeks, and is considered a pristine tidal estuary due to minimal anthropogenic impacts.  The watershed drains a 24.8 km2 area of mostly pine forest and a moderately developed residential watershed to the north.  The Winyah Bay estuary, classified as a Class B type estuary by Pritchard (1955) and composed of waters that originate in the Blue Ridge Mountains of North Carolina, is one of the largest river-estuary ecosystems on the Eastern Seaboard.  It is located 14.4 km south of North Inlet.  Winyah Bay drains the sub-basins of 6 major rivers, which are heavily impacted by agriculture, mining, and industry.  The rivers drain approximately 46,736 km2 of uplands and marshes.

The weather station is located at Oyster Landing (OL) pier.  It consists of a 4.5 meter aluminum tower to which all of the sensors, solar panel, telemetry antennae, and enclosure are attached.  The wind sensor is mounted slightly above the tower at an elevation of 4.6 meters.  The Eppley and Licor sensors are just below the wind sensor at an elevation of 4.0 meters.  These two sensors are mounted on a pole that extends horizontally away from the tower to prevent any shadowing effects from the wind sensor.  The temperature/humidity sensor is at an elevation of 3.7 meters and is contained within a gill radiation shield.  The solar panel is at an elevation of 3.0 meters.  The barometric pressure sensor and all of the hardware are located inside of the enclosure that is attached to the tower at an elevation of 1.2 meters.  All elevations are measured from the platform (ie. the base of the tower).  The height of the Oyster Landing platform is 3.47m above mean sea level.  The tipping bucket rain gauge is located 8.0 meters away from the tower to the NE.  The sensors were all wired to the CR1000 following the protocol in the Campbell Scientific Sensor Manual.

Description of the specific sampling station follows:

Oyster Landing (OL) - (lat. 33:20:58; long. 79:11:34). The OL meteorological monitoring site is considered a fairly pristine and undisturbed area located at the end of the OL pier which is also where one of the NIW NERR water quality station site is located.  The pier stretches into the upper reaches of Crabhaul Creek in the mid western portion of North Inlet.  The sampling site is located approximately 2.8 km from the headwaters of Crabhaul Creek.  The creek directly drains pine forested uplands and wetlands.  Salinity can range from 0 - 32 ppt. and average tidal flux is approximately 1.4m.  The creek has an average depth of ~2 m Mean High Water (MHW) and average width of ~150m MHW.  The bottom mostly is comprised of oyster shell hash with some fine sediment and detritus.

6)  Data collection period

Meteorological data has been collected by automated weather stations at the Oyster Landing Pier since 1982.  Data collection using NERR protocols began July 2, 1997.  Long-Term data and metadata from 1982 through 2000 are available online at http://links.baruch.sc.edu/data/.  SWMP Data and metadata for 2001 to 2015 are available online at http://cdmo.baruch.sc.edu/.

| File Start Date and Time | File End Date and Time |
| --- | --- |
| 01/01/2015 00:00 | 01/05/2015 11:30 |
| 01/05/2015 11:45 | 01/20/2015 14:00 |
| 01/20/2015 14:15 | 02/27/2015 10:30 |
| 02/27/2015 10:45 | 03/26/2015 12:45 |
| 03/26/2015 13:00 | 04/28/2015 10:30 |
| 04/28/2015 10:45 | 05/20/2015 09:45 |
| 05/20/2015 10:00 | 06/26/2015 10:45 |
| 06/26/2015 11:00 | 07/13/2015 10:15 |
| 07/13/2015 10:30 | 07/17/2015 11:45 |
| 07/17/2015 13:00 | 08/06/2015 09:15 |
| 08/06/2016 10:30 | 08/25/2015 09:15 |
| 08/25/2015 09:30 | 09/10/2015 09:30 |
| 09/10/2015 09:45 | 09/23/2015 14:15 |
| 09/23/2015 14:30 | 10/06/2015 14:30 |
| 10/06/2015 14:45 | 10/19/2015 09:30 |
| 10/19/2015 09:45 | 11/10/2015 11:15 |
| 11/10/2015 11:30 | 11/23/2015 16:00 |
| 11/23/2015 16:15 | 12/09/2015 10:45 |
| 12/09/2015 12:15 | 12/21/2015 11:15 |
| 12/21/2015 11:30 | 01/04/2016 10:45 |

7)  Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2012.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

8)  Associated researchers and projects

The NERR Water Quality Monitoring Project also has a station located at Oyster Landing.  The principal objective of this study is to record long-term water quality data for North Inlet - Winyah Bay in order to observe any physical changes or trends in water quality over time.  Four sites were chosen; two to represent our pristine site and the other two to represent impacted sites.  The Oyster Landing monitoring site is considered a fairly pristine and undisturbed area located at the end of the Oyster Landing pier, which is also where the NIW NERR Meteorological station site is located.  The Clambank site is located near the center of the Reserve and is considered pristine.  Debidue Creek and Thousand Acre are both located in waterways that receive run-off from heavily developed areas.  Measurements are taken every 15 minutes over roughly two week collecting periods with an YSI EXO2 data logger. The parameters measured are specific conductivity, salinity, dissolved oxygen, water temperature, pH, turbidity and water level with the addition of fDOM and chlorophyll at Oyster Landing.  These parameters are important indicators of habitat quality for numerous estuarine species and to determine health criteria and human uses.

A water chemistry program is associated with the NIW NERR core-monitoring program. Variables sampled include: chlorophyll dissolved organic carbon, nitrate-nitrite, orthophosphate, and ammonia.  (See documentation on the NIW http://www.northinlet.sc.edu/ home page for further details).  Samples for the 20 day program and the monthly grabs are collected at each of the four NERR water-quality monitoring stations.  Thousand Acre is our permanent monitoring station for the NERR monitoring program beginning monitoring in 1994.   Oyster Landing and Clambank Creek were also sampled beginning in 1993. Debidue Creek monitoring began in 1998.  These stations are also included in our NIW core-monitoring program.

a)  Monthly grab samples

Our monthly grab samples were initiated across the NERR platform on February 1, 2001 as a method of providing baseline information on inorganic nutrient and chlorophyll a water quality status within the National Estuarine Research Reserve system.  The North Inlet-Winyah Bay National Estuarine Research Reserve (NIW NERR) began collecting monthly grab samples on April 17, 2002.

b)  Diel Sampling Program (20 day program)

The NIW NERR 20 day water chemistry database is a continuation of the Long-Term Ecological Research (LTER) Daily Water Sample 1978-1993 database whose purpose was to monitor nutrient cycling in the North Inlet Estuary, Georgetown, SC.  The Long-Term Ecological Research Project at North Inlet was part of the national network of LTER sites and was funded by the National Science Foundation from October 1980 through 1993.  The NERR 20 day water chemistry program was initiated in June of 1993 to continue the monitoring of nutrients in the North Inlet Estuary system and also to continue the long-term database on the nutrient concentration dynamics of the North Inlet salt marsh estuarine system.  A larger purpose of this monitoring program is to combine the water chemistry, chlorophyll a, and suspended sediment monitoring data with other existing North Inlet ecological monitoring data to provide ecosystem level information and understanding.  Because the LTER daily water sampling collections were taken every day at 10:00am EST, it was determined that the nutrient data are biased for spring high tides; not all tides levels would be represented.  The new NERR water chemistry sampling protocols (based on a 10 day cycle) sampled all tidal stages over the years.

II.  Physical Structure Descriptors

9)  Sensor specifications –

Parameter: Temperature

Units: Celsius

Sensor type: PT100 RTD, IEC 751 1/3 Class B, with calibrated signal conditioning

Model #:  HC2-S3 Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.1 °C @ 23°C

Model #:  HC2-S3

Serial #:  61114965

Dates in Use:  				Calibrated

09/10/2013 – 11/04/2014		04/12/2013 (new)

12/09/2015 – current 			11/17/2014

Serial #: 61115408

Dates in Use:				Calibrated

11/04/2014 – 12/09/2015		06/10/2013 (new)

Parameter:  Temperature

Units:  Celsius

Sensor type:  Platinum resistance temperature detector (PRT)

Operating Temperature:  -40 to +60°C

Range: -40 to +60°C

Accuracy: ± 0.2 °C @ 20°C

Model #:  HMP45C Temperature and Relative Humidity Probe

Serial #:  U2520001 (RETIRED)

Dates in Use:				Calibrated

08/11/1999 - 05/17/2001		new 08/1999

09/18/2001 - 05/13/2003		08/21/2001

02/18/2004 - 07/12/2004		12/17/2003

08/06/2004 - 09/02/2004		08/02/2004

02/15/2005 - 04/18/2005		08/02/2004

06/09/2005 - 12/21/2005		08/02/2004

06/06/2007 - 08/26/2008		04/04/2007

02/16/2009 - 02/23/2010		02/02/2009

04/07/2011 - 03/29/2012		01/14/2011

02/05/2013 – 09/10/2013		01/24/2013

Serial #:  A1710069 (RETIRED)

Dates in Use:				Calibrated

12/21/2005 - 06/06/2007		04/26/2005

08/26/2008 - 02/16/2009		07/28/2008

02/23/2010 - 04/07/2011		01/20/2010

03/29/2012 – 02/05/2013		03/14/2012

Parameter: Relative Humidity

Units: Percent

Sensor type: ROTRONIC® Hygromer IN-1

Model #: HC2-S3 Temperature and Relative Humidity Probe

Range: 0-100% non-condensing

Accuracy at 23°C:  +/- 0.8% RH with standard configuration settings

Temperature dependence of RH measurement +/- 3% (-40 to 60C)

Model #:  HC2-S3

Serial #:  61114965

Dates in Use:  				Calibrated

09/10/2013 – 11/04/2014		04/12/2013 (new)

12/09/2015 – current 			11/17/2014

Serial #: 61115408

Dates in Use:				Calibrated

11/04/2014 – 12/09/2015		06/10/2013 (new)

Parameter:  Relative Humidity

Units:  Percent

Sensor type:  Vaisala HUMICAP © 180 capacitive relative humidity sensor

Range:  0-100% non-condensing

Accuracy at 20°C:  +/-2% RH (0-90%) and +/-3% (90-100%)

Temperature dependence of RH measurement:  +/- 0.05% RH/°C

Model # HMP45C Temperature and Relative Humidity probe

Serial #:  U2520001 (RETIRED)

Dates in Use:  				Calibrated

08/11/1999 - 05/17/2001		new 08/1999

09/18/2001 - 05/13/2003		08/21/2001

02/18/2004 - 07/12/2004		12/17/2003

08/06/2004 - 09/02/2004		08/02/2004

02/15/2005 - 04/18/2005		08/02/2004

06/09/2005 - 12/21/2005		08/02/2004

06/06/2007 - 08/26/2008		04/04/2007

02/16/2009 - 02/23/2010		02/02/2009

04/07/2011 - 03/29/2012		01/14/2011

02/05/2013 – 09/10/2013		01/24/2013

Serial #:  A1710069 (RETIRED)

Dates in Use:				Calibrated

12/21/2005 - 06/06/2007		04/26/2005

08/26/2008 - 02/16/2009		07/28/2008

02/23/2010 - 04/07/2011		01/20/2010

03/29/2012 - 02/05/2013		03/14/2012

Parameter:  Barometric Sensor

Units:  millibars (mb)

Sensor type:  Vaisala Barocap © silicon capacitive pressure sensor

Operating Range:  Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ±0.5 mb @ 20°C; +/-2mb @ 0°C to 40°C; +/-4mb @ -20°C to 45°C; +/-

6.0mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Model # Viasala CS-105

Serial #:  U1120009

Dates in Use:				Calibrated

08/11/1999 - 05/17/2001		new 1999

05/13/2003 - 02/15/2005		12/26/2002

06/06/2007 - 06/03/2009		02/27/2007

06/25/2009 - 08/25/2009		02/27/2007

12/02/2011 – 12/19/2013		09/13/2011

12/17/2014 – current			11/18/2014

Serial #:  W1530030 (SN R4230013 replaced under warranty) (RETIRED)

Dates in Use:				Calibrated

05/17/2001 - 05/13/2003		new 2001

02/15/2005 - 06/06/2007		12/16/2004

06/03/2009 - 06/25/2009		04/29/2009

Model # Viasala CS-106

Serial #:  D4930017

Dates in Use:				Calibrated

08/25/2009 - 12/02/2011		new 08/2009

12/19/2013 – 12/17/2014		11/13/2013

Parameter:  Wind Speed

Units:  meter per second (m/s)

Sensor type:  18 cm diameter 4-blade helicoids propeller molded of polypropylene

Range: 0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy:  +/- 0.3 m/s

Model #:  RM Young 05106-5A Marine Wind Monitor

Serial #:  WM65129

Dates in Use:  				Calibrated

02/15/2005 - 06/06/2007		01/13/2005

09/30/2009 - 12/02/2011		09/15/2009

12/19/2013 – 12/09/2015		11/25/2013

Serial #:  WM97356

Dates in Use:				Calibrated

12/02/2011 – 12/19/2013		09/15/2011

12/09/2015 – current			08/28/2015

Parameter:  Wind Direction

Units:  degrees

Sensor type:  balanced vane, 38 cm turning radius

Range:  360° mechanical, 355° electrical (5° open)

Accuracy:  +/- 3 degrees

Model #:  RM Young 05106-5A Marine Wind Monitor

Serial #:  WM65129

Dates in Use:  				Calibrated

02/15/2005 - 06/06/2007		01/13/2005

09/30/2009 - 12/02/2011		09/15/2009

12/19/2013 – 12/09/2015		11/25/2013

Serial #:  WM97356

Dates in Use:  				Calibrated

12/02/2011 – 12/19/2013		09/15/2011

12/09/2015 – current			08/28/2015

Parameter Wind Speed

Units:  meter per second (m/s)

Sensor type:  12 cm diameter cup wheel assembly, 40 mm diameter hemispherical cups

Range:  0-50 m/s (112 mph); gust survival 60 m/s (134 mph)

Accuracy:  +/- 0.5 m/s (1.1 mph)

Model #:  Wind Sentry 03001-5

Serial #:  8702 (RETIRED)

Dates in Use:				Calibrated

07/1997 - 05/17/2001			2001

05/13/2003 - 02/15/2005		03/01/2003

06/06/2007 - 09/30/2009		04/24/2007

Parameter Wind Direction

Units:  degrees

Sensor type:  balance vane, 16 cm turning radius

Range:  360° mechanical, 355° electrical (5° open)

Accuracy:  +/- 5°

Model #:  Wind Sentry 03001-5

Serial #:  8702 (RETIRED)

Dates in Use:				Calibrated

07/1997 - 05/17/2001			2001

05/13/2003 - 02/15/2005		03/01/2003

06/06/2007 - 09/30/2009		04/24/2007

Parameter: Photosynthetically Active Radiation (PAR)

Units:  mmoles m-2 (total flux)

Sensor type:  anodized aluminum with cast acrylic diffuser

Model #SQ110 Apogee Quantum Sensor

Light spectrum waveband:  410 to 655 nm

Temperature dependence: 0.06 +/-0.06% per °C

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 70°C; Humidity 0 to 100%

Cosine Response: 45° zenith angle: +/- 2%; 75° zenith angle: +/- 5%

Sensitivity:  0.2 mV per µmol s-1 m-2

Serial #:  11198

Dates in Use:  			Multiplier:		Calibrated

11/04/2014 – 12/09/2015	0.025			09/25/2013

Serial #:  13334

Dates in Use:  			Multiplier:		Calibrated

08/06/2013 – 11/04/2014	0.025			07/01/2013 (new)

12/09/2015 – current					11/01/2014

Parameter: Photosynthetically Active Radiation (PAR)

Units:  mmoles m-2 (total flux)

Sensor type:  High stability silicon photovoltaic detector (blue enhanced)

Light spectrum waveband:  400 to 700 nm

Temperature dependence:  0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity 0 to 100%

Sensitivity:  typically 8 μA per 1000 μmoles s-1 m-2

LI-COR Quantum Sensor (PAR)

Model #LI190SZ

Serial #:  Q34301 (RETIRED)

Dates in Use:			Multiplier:		Calibrated

03/06/2006 - 07/05/2006	1.66			12/14/2004

Model # LI190SB

Serial #:  Q26604 (RETIRED)

Dates in Use:			Multiplier: 		Calibrated

07/1997 - 04/1999		???			new 07/1997

04/1999 - 04/19/2000		1.27			04/16/1999

03/2001 - 06/26/2001		1.27			04/16/1999

06/19/2003 - 02/15/2005	1.18			02/17/2003

07/06/2006 - 08/25/2006	1.42			04/07/2006

06/06/2007 - 09/16/2008	1.42			04/07/2006

09/30/2009 - 02/23/2010	1.86			07/23/2009

03/29/2010 (for post correct)

Serial # Q41803 (RETIRED)

Dates in Use:			Multiplier:		Calibrated

02/23/2010 - 04/07/2011	1.271605		new 06/2009

03/12/2012 - 03/29/2012	1.527333		09/06/2011

07/05/2012 - 12/04/2012	1.527333		09/06/2011

09/10/2013 – 11/04/2014	1.513372		07/15/2013

Serial # Q41811 (RETIRED)

Dates in Use:			Multiplier:		Calibrated

04/07/2011 - 03/12/2012	1.279466		new 06/25/2009

03/29/2012 - 07/05/2012	1.279466		06/25/2009

12/05/2012 – 09/10/2013	1.499664		07/23/2012

Model # LI190SZ

Serial #:  Q18206 (RETIRED)

Dates in Use:  			Multiplier:		Calibrated

04/19/2000 - 03/2001		1.99			new 04/2000

06/26/2001 - 06/19/2003	1.78			03/06/2001

02/15/2005 - 03/06/2006	1.79			01/11/2005

08/25/2006 - 06/06/2007	1.79			05/02/2006

09/16/2008 - 09/30/2009	1.86			07/21/2008

12/16/2009 (for post correct)

Precipitation

Units:  millimeters (mm)

Sensor type:  Tipping Bucket Rain Gauge

Rainfall per tip:  0.01 inch (.254 mm)

Operating range:  Temperature:  0°C to +/- 50°C; Humidity:  0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Model #:  TE525WS

Dates in Use:  			Calibrated

05/17/2001 – current		11/14/2000

07/10/2002

07/03/2003

03/29/2004

04/18/2005

09/15/2006

01/31/2008

04/17/2009

04/01/2010

11/15/2011

12/04/2012

12/17/2013

12/17/2014

02/27/2015

12/09/2015

Total Solar Radiation

Units:  watts/m2

Sensor type:  Eppley Black and White Pyranometer

Sensitivity:  10 microvolts/watt-meter-2 (approx.)

Impedance:  350 ohms approx.

Temperature dependence:  +/- 1.5% constancy from -20 to +40 °C

Linearity:  +/- 1% from 0 to 1400 watts meter-2

Response time:  3 to 4 seconds (i/e signal)

Cosine response:  +/- 2% from normalization 0-70° zenith angle, +/- 5% 70-80° zenith

angle

Mechanical vibration:  capable of withstanding up to 20g’s

Calibration:  integrating hemisphere (approx. 700 watts/meter ambient temperature +25°C):  calibration reference Eppley standards reproducing the World Radiation Reference

Model #: 8-48

Serial #28078

Dates in Use:			Multiplier:*		Calibrated

08/11/1999 - 05/17/2001	0.1385*			03/19/1997

05/13/2003 - 02/15/2005	0.1402*			12/30/2002

06/06/2007 - 09/30/2009	0.1440*			02/16/2007

12/02/2011 – 08/06/2013	0.1456*			09/01/2011

08/06/2013 – 12/19/2013	101.5228		09/01/2011

12/17/2014 – current		102.0408		11/05/2014

Serial #28070

Dates in Use:			Multiplier:*		Calibrated

05/17/2001 - 05/13/2003	0.1307*			02/07/2011

02/15/2005 - 06/06/2007	0.1365*			12/09/2004

09/30/2009 - 12/11/2011	0.1384*			08/27/2009

12/19/2013 – 12/17/2014	96.8054			11/14/2013

*the multipliers and way in which the 5 second data from the sensor were being used to calculate a 15 minute value were incorrect until a program change on 08/06/2013.  Prior to this date, all total solar radiation data are incorrect.  Please contact the site for the status of corrected total solar radiation data prior to 08/06/2013.

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

CR1000 Serial Number: 5201

Date CR1000 Installed: 07/06/2006 (new)

Date CR1000 Calibrated: 07/29/2015

Dates in Use:

07/06/2006 10:00 – 07/17/2015 12:03

08/06/2015 10:16 - current

CR1000 Firmware Version (s):  CR1000.Std.24

CR1000 Serial Number: 63721

Date CR1000 Installed: 07/17/2015

Date CR1000 Calibrated: 02/10/2014 (new)

Dates in Use:

07/17/2015 12:03 - 08/06/2015 10:16

CR1000 Firmware Version (s):  CR1000.Std.27

CR1000 Program Version(s):

NIWOLMET_V5.5_12192013.CR1

NIWOLMET_V5.5_11042014.CR1

NIWOLMET_V5.5_12172014.CR1

10)  Coded variable definitions

Sampling station:		Sampling site code:	Station code:

Oyster Landing		OL			niwolmet

11)  QAQC flag definitions –

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions –

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC		Incorrect Calibration Constant, Multiplier or Offset

SIW	Incorrect Wiring

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor Drift

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR		Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CCU	Cause Unknown

CDF	Data Appear to Fit Conditions

CML	Snow melt from previous snowfall event

CRE*	Significant Rain Event

CSM*	See Metadata

CVT*	Possible Vandalism/Tampering

CWE*	Significant weather event

13)  Other remarks/notes

- Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

- Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles m-2 over a 15 minute interval.

- Elevated nighttime PAR data has been observed on several days throughout the years.  It is assumed that moisture intrusion into the LI-COR sensor may cause these altered values.  Since the range of error for this sensor is +/- 2.214 moles m-2 timepoints that fall between one half hour before sunrise and one half hour after sunset that have values greater than 2.214 moles m-2 have been rejected due to QA/QC checks and have been coded with CSM comment.  Timepoints that fall between one half hour before sunrise and one half hour after sunset that have values greater than 0 moles m-2 but less than 2.214 moles m-2 are considered suspect and have been coded with CSM comment as well.  Note that sunrise and sunset times are determined by utilizing the Astronomical Applications Department U.S. Naval Observatory website http://aa.usno.navy.mil/data/docs/RS_OneYear.php and using Georgetown, South Carolina as the city of reference.

- Total Solar Radiation night time values of +/-1 are within the expected specs  of the Eppley 8-48 sensor and could be attributed to thermal transitions inside of the sensor, &/or offsets within the CR1000 datalogger or the sensor cable.

- Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

- Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

- Periods of TotPrcp (total precipitation) and CumPrcp (cumulative precipitation) data that are coded <5> [GMT] (CSM) are data that have been corrected due to maintenance or calibration of the rain gauge tripping the sensor and registering precipitation erroneously.

- The NERR Data Management Committee mandated that beginning February 15, 2014, the CR1000 datalogger should be calibrated every 5 years.  They allowed a 1 year grace period for sites to comply, after which, if the datalogger has not been calibrated, all data should be coded as <1> [SOC] (CSM) to indicate that the datalogger is outside of the recommended calibration window. This is not an indication that the data are erroneous or should not be trusted.   Data are flagged and coded as <1> [SOC] (CSM) from 02/16/2015 00:00 - 07/17/2015 11:45.

- Tropical Storm Ana made landfall north of Georgetown in Myrtle Beach, S. C. on 05/09/2015 with strong winds and rain.  Since the effects of TS Ana were mild but noticeable CWE coding is used in the F_Record from 05/06/2015 00:15 - 05/11/2015 00:00.

- A strong low pressure system moved over the area between 01/23/2015 and 01/25/2015. BP data are coded as CRE from 01/23/2015 00:30 - 01/24/2015 20:15.  Wind speed parameters are coded as CRE from 01/23/2015 21:45 - 01/24/2015 18:30.  Precipitation parameters are coded as CRE from 01/23/2015 10:45 - 1/24/2015 09:00 (total precipitation) and through the end of the day, 01/25/2015 00:00, for cumulative precipitation.

- On 07/17/2015 at 12:03 EST the station was powered down and the CR1000 (SN 63721) from another project (SeaGrant Pond calibrated on 02/10/2014) was installed at OL.  The power was back on at the station at 12:45 EST, program loaded and the clock on the CR1000 changed to EST at 12:47 EST.  Data are missing from 07/17/2015 12:00 - 12:45 and are rejected at 13:00.

- On 08/06/2015 at 09:35 EST the station was powered down and the recalibrated OL CR1000 (SN 5201) was reinstalled.  Power back on to the station and the program was loaded at 10:16 EST. Data are missing 08/06/2015 09:30 - 10:15 and are rejected at 10:30.

- In early October of 2015, South Carolina experienced a historic rainfall event that lasted several days and which led to major flooding across large portions of the state.  The impacts of this historic rainfall on water quality within the North Inlet Estuary can be seen in the North Inlet – Winyah Bay NERR’s water quality data from their four monitoring stations, and is accessible via the NERR Centralized Data Management Office at www.nerrsdata.org.  BP, wind parameters, PAR, and TSR are coded as [CWE] and precipitation parameters are coded as [CRE] from 09/30/2015 13:00 - 10/05/2015 19:45 (cumulative precipitation coding is continued to 10/06/2015 00:00).

- 12/09/2015 11:00 - 12:00 data are missing due to a power down to replace the air temperature and RH sensor and PAR sensor.  The wind sensor was also replaced during this power down with maintenance continuing until 12:45.  All data were rejected at 12:15 due to the power down.

- The RH sensor began malfunctioning due to corrosion.  Data were flagged as suspect 12/09/2015 12:30 - 12/31/2015 23:45.