# North Inlet-Winyah Bay National Estuarine Research Reserve (NIW) Nutrient Metadata

January 1, 2010 through December 31, 2010

Latest Update:  April 8, 2013

- Data Set & Research Descriptors

# Principal Investigators and contact persons (all contact personnel are at the North Inlet-Winyah Bay NERR)

    - Reserve Contact

Wendy Allen, Reserve Manager

Email: wendy@belle.baruch.sc.edu

Erik Smith, Ph.D., Research Coordinator

Email:  erik@belle.baruch.sc.edu

North Inlet-Winyah Bay NERR

PO Box 1630

Georgetown, SC 29442

Phone: 843-546-3623

Fax: 843-546-1632

# Laboratory Contacts

Ben Lakish, Research Specialist (water chemist)

Email: ben@belle.baruch.sc.edu

Amy Willman, Research Specialist

Email: willman@belle.baruch.sc.edu

See section 17 Subsection C for a detailed description of additional laboratory personnel and responsibilities during 2009.

### Research Objectives

a) Monthly Grab Samples

Monthly grab samples were initiated across the NERR platform on February 1, 2001 as a method of providing baseline information on inorganic nutrient and chlorophyll a water quality status within the National Estuarine Research Reserve system.  The North Inlet-Winyah Bay National Estuarine Research Reserve (NIW NERR) began collecting monthly grab samples on April 17, 2002.

b) Diel Sampling Program

The NIW NERR 20 day water chemistry database is essentially a continuation of the Long-Term Ecological Research (LTER) Daily Water Sample 1978-1993 database whose purpose was to monitor nutrient cycling in the North Inlet Estuary, Georgetown, SC.  The Long-Term Ecological Research Project at North Inlet was part of the national network of LTER sites and was funded by the National Science Foundation from October 1980 through 1993.  The NERR 20 day water chemistry program was initiated in June of 1993 to continue the monitoring of nutrients in the North Inlet Estuary system and also to continue the long-term database on the nutrient concentration dynamics of the North Inlet salt marsh estuarine system.  A larger purpose of this monitoring program is to combine the water chemistry, chlorophyll a, and suspended sediment monitoring data with other existing North Inlet ecological monitoring data to provide ecosystem- level information and understanding.  Because the LTER daily water sampling collections were taken every day at 10:00am EST, it was determined that the nutrient data are biased for spring high tides; therefore, not all tides levels would be represented.  The NIW NERR water chemistry sampling protocols (based on multiples of a 10 day cycle – see section 3) sample all tidal stages over the years.

# Research Methods

a) Monthly Grab Samples

Two 950-mL samples are collected 2 minutes apart at slack low tide every 20 days.  Samples are collected from 0.5m below the water’s surface.   Sample collection is carried out by an ISCO 3700 automated water sampler programmed to collect two samples at an interval of two minutes after the last diel sample collection time (see below).  These additional samples enable comparisons for analysis of laboratory technique accuracy, sample variation, etc.  All “duplicate” samples are worked up with the rest of the diel samples.   After the final sample is collected, the ISCO samplers are brought back to the lab and the samples stored at 4o C until processed within 48 hours.  Samples are collected at each of the four NERR water-quality monitoring stations: Oyster Landing (OL), Debidue Creek (DC), Clambank Creek (CB), and Thousand Acre Marsh (TA).

b) Diel Sampling Program

Thirteen 950-mL samples are collected every 20 days at 2 hour and 4 minute intervals, for 2 complete tidal cycles (24:48 hours).   Samples are collected from 0.5m below the water’s surface into acid-washed one-Liter ISCO sampling bottles by an ISCO 3700 automated water sampler.  Samples are kept cool by placing a bottle of ice inside the ISCO sampler.  After the final sample is collected, the ISCO samplers are brought back to the lab and the samples are stored at 4oC until processed within 48 hours.  Samples are collected at each of the four NERR water-quality monitoring stations: Oyster Landing (OL), Debidue Creek (DC), Clambank Creek (CB), and Thousand Acre Marsh (TA).

It was determined that a 10 day sampling cycle (or multiples of 10) would represent all tidal stages when run over the years.  A 20-day schedule was selected for the NERR water chemistry project because if a 10-day interval was chosen, the water technician could not process all the sites samples in a timely fashion, and a 30 day schedule did not produce enough data points for the year.  Sampling events always begin and end on predicted slack low tide (SL) and cover two tide cycles, one in the day and one at night.  The timing of SL for the North Inlet sites (DC, CB, OL) is different than the timing of SL for the Winyah Bay site (TA), due to differences in runoff, river flow, and distance from the ocean.  Therefore, sample times for each site do not match exactly within a sample event, but tide stages or levels do.  Sample times are determined based on predicted tide elevations from NOAA tide charts for Charleston, SC.  A study was conducted to determine how much variation there was between North Inlet tides (at OL, DC, and CB) and the predicted times for Charleston.  It was determined that the North Inlet times were not different by more than 5 to 10 minutes.  Estimated times for the Winyah Bay site were attained by comparing the water quality data logger water level measurements between the North Inlet sites and Thousand Acre.  It was determined that slack low for the TA site was one hour and 30 minutes later than the North Inlet sites.  However, there can be a great deal of variability in tide stage at all sites due to wind, rain, and river flow; TA site is the most variable.  Tides can be delayed anywhere from one to two hours from the predicted.

# Site Location and character

The North Inlet-Winyah Bay National Estuarine Research Reserve is located on the southeastern coast of the United States on the Atlantic Ocean in two tidal estuaries, North Inlet and Winyah Bay, near Georgetown, South Carolina.  The North Inlet estuary is composed of numerous winding tidal creeks, and is considered a pristine tidal estuary due to minimal anthropogenic impacts.  The watershed drains a 24.8 km2 area of mostly pine forest and a moderately developed residential watershed to the north.  The Winyah Bay estuary, which originates in the Blue Ridge Mountains of North Carolina, is one of the largest river-estuary ecosystems on the Eastern Seaboard.  It is located 14.4 km south of the mouth of North Inlet Estuary.  Winyah Bay drains the sub-basins of 6 major rivers, which are heavily impacted by agriculture, mining, and industry.  The rivers drain approximately 46,736 km2 of uplands and marshes.  Descriptions of the individual monitoring sites are as follows:

Clambank Creek (CB or niwcbnut): (lat 33:20:02, long 79:11:35) The Clambank Creek monitoring site is located roughly in the center of the NIW NERR.  The site is surrounded by a Spartina marsh and drains associated uplands.  Salinity ranges from 0 to 36.  The creek width and depth at MHW at the sampling site are 40m and 2.4 m respectively.  The average depth of the creek at 	MHW is 2.0m and average tidal flux is approximately 1.4 meters.  The bottom is mostly comprised of oyster shell hash and some fine sediment.  This site is considered pristine and is influenced by its close proximity to the Inlet mouth.

Debidue Creek (DC or niwdcnut): (lat 33:21:37, long 79:10:03) The Debidue Creek monitoring site is located in an ocean-dominated Spartina marsh that was formerly surrounded by pine-dominated uplands.  The site is approximately 1 km south of the Debordieu Colony, a large development built on man-made canals that drain into the northern portion of Debidue Creek, and as a result, may be considered an impacted site.  Salinity can range from 0 to 36 parts per thousand and average tidal flux is approximately 2 meters.  The creek has an average depth of 2.2 m MHW and an average width of 70 m MHW at the sample site.  The bottom is mostly comprised of oyster shell hash with some fine sediment and detritus.

Oyster Landing (OL or niwolnut): (lat. 33:20:58, long. 79:11:20) The Oyster Landing monitoring site is considered a fairly pristine and undisturbed area.  Samples are collected at the end of the Oyster Landing pier from the floating dock, which is adjacent to the location of the NIW NERR weather station.  The pier stretches into the upper reaches of Crab Haul Creek in the mid-western portion of North Inlet and is approximately 2.8 km from the headwaters of Crab Haul Creek.  The creek drains pine forested uplands and wetlands.  Salinity can range from 0 to 36 parts per thousand and average tidal flux is approximately 1.4 m.  The creek has an average depth of ~2 m MHW and an average width of ~150m MHW at the sample site.  The bottom is mostly comprised of oyster shell hash with some fine sediment and detritus.

Thousand Acre (TA or niwtanut): (lat. 33:17:57, long. 79:15:22) The Thousand Acre monitoring site is located in Thousand Acre marsh tidal creek, near the creek mouth.  Prior to 7/19/1999, the sampling location was approximately 30 meters northeast of the west bridge of Thousand Acre marsh.  Because of heavy siltation near the creek bank which caused inaccurate water quality measurements at the original site, beginning on 7/19/1999, the sampling site was moved and samples were collected at the NW corner of the west bridge, about 15 m from the mouth of the creek.  At this new sampling site, creek depth is approximately 2 m MHW and creek width is approximately 10 m.  The creek empties into the northeastern side of the mid-portion of Winyah Bay and drains the Thousand Acre Rice Field and surrounding pine forested uplands and vegetated wetlands. Salinity ranges from 0 to 26 parts per thousand and tidal flux is approximately 1m.  The bottom is mostly composed of fine sediments and detritus.  Georgetown is the homeport for a number of heavy industries, including a steel plant, paper mill, chemical plant, and a coal fired plant. Most of these industrial sites are located on the southern side of Winyah Bay, 5 km upstream from the Thousand Acre site.  A public sewage treatment plant, which discharges into the bay, is also located in Georgetown.

- Code variable definitions

a) Station Code Names

niwcbnut = North Inlet Winyah Bay Clambank Creek nutrients

niwdcnut = North Inlet Winyah Bay Debidue Creek nutrients

niwolnut = North Inlet Winyah Bay Oyster Landing nutrients

niwtanut = North Inlet Winyah Bay Thousand Acre nutrients

b) Monitoring Program

Monthly Grab sample program (1)

Diel grab sample program (2)

- Data collection period

  - Monthly Grab Sampling

| Station Code | Date | Time of rep 1 sample | Time of rep 2 sample |  |  |
| --- | --- | --- | --- | --- | --- |
| niwcbnut | 1/5/2010 | 5:11 | 5:13 |  |  |
| niwcbnut | 1/25/2010 | 9:10 | 9:12 |  |  |
| niwcbnut | 2/14/2010 | 2:10 | 2:12 |  |  |
| niwcbnut | 3/6/2010 | 6:19 | 6:21 |  |  |
| niwcbnut | 3/26/2010 | 10:39 | 10:41 |  |  |
| niwcbnut | 4/15/2010 | 2:38 | 2:40 |  |  |
| niwcbnut | 5/5/2010 | 7:00 | 7:02 |  |  |
| niwcbnut | 5/25/2010 | 11:40 | 11:42 |  |  |
| niwcbnut | 6/14/2010 | 3:18 | 3:20 |  |  |
| niwcbnut | 7/4/2010 | 7:07 | 7:09 |  |  |
| niwcbnut | 7/24/2010 | 12:44 | 12:46 |  |  |
| niwcbnut | 8/13/2010 | 4:08 | 4:10 |  |  |
| niwcbnut | 9/2/2010 | 7:20 | 7:22 |  |  |
| niwcbnut | 9/22/2010 | 1:06 | 1:08 |  |  |
| niwcbnut | 10/12/2010 | 4:51 | 4:53 |  |  |
| niwcbnut | 11/1/2010 | 8:52 | 8:54 |  |  |
| niwcbnut | 11/21/2010 | 1:04 | 1:06 |  |  |
| niwcbnut | 12/11/2010 | 5:34 | 5:36 |  |  |
| niwcbnut | 12/31/2010 | 10:42 | 10:44 | Did not sample | Holiday |
| niwdcnut | 1/5/2010 | 5:11 | 5:13 |  |  |
| niwdcnut | 1/25/2010 | 9:10 | 9:12 |  |  |
| niwdcnut | 2/14/2010 | 2:10 | 2:12 |  |  |
| niwdcnut | 3/6/2010 | 6:19 | 6:21 |  |  |
| niwdcnut | 3/26/2010 | 10:39 | 10:41 |  |  |
| niwdcnut | 4/15/2010 | 2:38 | 2:40 |  |  |
| niwdcnut | 5/5/2010 | 7:00 | 7:02 |  |  |
| niwdcnut | 5/25/2010 | 11:40 | 11:42 |  |  |
| niwdcnut | 6/14/2010 | 3:18 | 3:20 |  |  |
| niwdcnut | 7/4/2010 | 7:07 | 7:09 |  |  |
| niwdcnut | 7/24/2010 | 12:44 | 12:46 |  |  |
| niwdcnut | 8/13/2010 | 4:08 | 4:10 |  |  |
| niwdcnut | 9/2/2010 | 7:20 | 7:22 |  |  |
| niwdcnut | 9/22/2010 | 1:06 | 1:08 |  |  |
| niwdcnut | 10/12/2010 | 4:51 | 4:53 | Sample missed | Sample missed |
| niwdcnut | 11/1/2010 | 8:52 | 8:54 |  |  |
| niwdcnut | 11/21/2010 | 1:04 | 1:06 |  |  |
| niwdcnut | 12/11/2010 | 5:34 | 5:36 |  |  |
| niwdcnut | 12/31/2010 | 10:42 | 10:44 | Did not sample | Holiday |
| niwolnut | 1/5/2010 | 5:11 | 5:13 |  |  |
| niwolnut | 1/25/2010 | 9:10 | 9:12 |  |  |
| niwolnut | 2/14/2010 | 2:10 | 2:12 |  |  |
| niwolnut | 3/6/2010 | 6:19 | 6:21 |  |  |
| niwolnut | 3/26/2010 | 10:39 | 10:41 |  |  |
| niwolnut | 4/15/2010 | 2:38 | 2:40 |  |  |
| niwolnut | 5/5/2010 | 7:00 | 7:02 |  |  |
| niwolnut | 5/25/2010 | 11:40 | 11:42 |  |  |
| niwolnut | 6/14/2010 | 3:18 | 3:20 |  |  |
| niwolnut | 7/4/2010 | 7:07 | 7:09 |  |  |
| niwolnut | 7/24/2010 | 12:44 | 12:46 |  |  |
| niwolnut | 8/13/2010 | 4:08 | 4:10 |  |  |
| niwolnut | 9/2/2010 | 7:20 | 7:22 |  |  |
| niwolnut | 9/22/2010 | 1:06 | 1:08 |  |  |
| niwolnut | 10/12/2010 | 4:51 | 4:53 |  |  |
| niwolnut | 11/1/2010 | 8:52 | 8:54 |  |  |
| niwolnut | 11/21/2010 | 1:04 | 1:06 |  |  |
| niwolnut | 12/11/2010 | 5:34 | 5:36 |  |  |
| niwtanut | 12/31/2010 | 10:42 | 10:44 | Did not sample | Holiday |
| niwtanut | 1/5/2010 | 6:41 | 6:43 |  |  |
| niwtanut | 1/25/2010 | 10:40 | 10:42 |  |  |
| niwtanut | 2/14/2010 | 3:40 | 3:42 |  |  |
| niwtanut | 3/6/2010 | 7:49 | 7:51 |  |  |
| niwtanut | 3/26/2010 | 12:09 | 12:11 |  |  |
| niwtanut | 4/15/2010 | 4:08 | 4:10 |  |  |
| niwtanut | 5/5/2010 | 8:30 | 8:32 |  |  |
| niwtanut | 5/25/2010 | 13:10 | 13:12 |  |  |
| niwtanut | 6/14/2010 | 4:48 | 4:50 |  |  |
| niwtanut | 7/4/2010 | 8:37 | 8:39 |  |  |
| niwtanut | 7/24/2010 | 14:14 | 14:16 |  |  |
| niwtanut | 8/13/2010 | 5:38 | 5:40 |  |  |
| niwtanut | 9/2/2010 | 8:50 | 8:52 |  |  |
| niwtanut | 9/22/2010 | 2:36 | 2:38 |  |  |
| niwtanut | 10/12/2010 | 6:21 | 6:23 |  |  |
| niwtanut | 11/1/2010 | 10:22 | 10:24 |  |  |
| niwtanut | 11/21/2010 | 2:34 | 2:36 |  |  |
| niwtanut | 12/11/2010 | 7:04 | 7:06 |  |  |
| niwtanut | 12/31/2010 | 12:12 | 12:14 | Did not sample | Holiday |

b) Diel Sampling

| Station Code | Start Date/Time | End Date/Time |  |  |
| --- | --- | --- | --- | --- |
| niwcbnut | 1/4/2010 4:21 | 1/5/2010 5:09 |  |  |
| niwcbnut | 1/24/2010 8:20 | 1/25/2010 9:08 |  |  |
| niwcbnut | 2/13/2010 1:20 | 2/14/2010 2:08 |  |  |
| niwcbnut | 3/5/2010 5:29 | 3/6/2010 6:17 |  |  |
| niwcbnut | 3/25/2010 9:49 | 3/26/2010 10:37 |  |  |
| niwcbnut | 4/14/2010 1:48 | 4/15/2010 2:36 |  |  |
| niwcbnut | 5/4/2010 6:10 | 5/5/2010 6:58 |  |  |
| niwcbnut | 5/24/2010 10:50 | 5/25/2010 11:38 |  |  |
| niwcbnut | 6/13/2010 2:28 | 6/14/2010 3:16 |  |  |
| niwcbnut | 7/3/2010 6:17 | 7/4/2010 7:05 |  |  |
| niwcbnut | 7/23/2010 11:54 | 7/24/2010 12:42 |  |  |
| niwcbnut | 8/12/2010 3:18 | 8/13/2010 4:06 |  |  |
| niwcbnut | 9/1/2010 6:30 | 9/2/2010 7:18 |  |  |
| niwcbnut | 9/21/2010 00:16 | 9/22/2010 1:04 |  |  |
| niwcbnut | 10/11/2010 4:01 | 10/12/2010 4:49 |  |  |
| niwcbnut | 10/31/2010 8:02 | 11/1/2010 8:50 |  |  |
| niwcbnut | 11/20/2010 00:14 | 11/21/2010 1:02 |  |  |
| niwcbnut | 12/10/2010 4:44 | 12/11/2010 5:32 |  |  |
| niwcbnut | 12/30/2010 9:52 | 12/31/2010 10:40 | Did not sample | Holiday |
| niwdcnut | 1/4/2010 4:21 | 1/5/2010 5:09 |  |  |
| niwdcnut | 1/24/2010 8:20 | 1/25/2010 9:08 |  |  |
| niwdcnut | 2/13/2010 1:20 | 2/14/2010 2:08 |  |  |
| niwdcnut | 3/5/2010 5:29 | 3/6/2010 6:17 |  |  |
| niwdcnut | 3/25/2010 9:49 | 3/26/2010 10:37 |  |  |
| niwdcnut | 4/14/2010 1:48 | 4/15/2010 2:36 |  |  |
| niwdcnut | 5/4/2010 6:10 | 5/5/2010 6:58 |  |  |
| niwdcnut | 5/24/2010 10:50 | 5/25/2010 11:38 |  |  |
| niwdcnut | 6/13/2010 2:28 | 6/14/2010 3:16 | 2:28-14:52 | Samples missed |
| niwdcnut | 7/3/2010 6:17 | 7/4/2010 7:05 |  |  |
| niwdcnut | 7/23/2010 11:54 | 7/24/2010 12:42 |  |  |
| niwdcnut | 8/12/2010 3:18 | 8/13/2010 4:06 |  |  |  |
| niwdcnut | 9/1/2010 6:30 | 9/2/2010 7:18 |  |  |
| niwdcnut | 9/21/2010 00:16 | 9/22/2010 1:04 |  |  |
| niwdcnut | 10/11/2010 4:01 | 10/12/2010 4:49 | 00:41-4:53 | Samples missed |
| niwdcnut | 10/31/2010 8:02 | 11/1/2010 8:50 |  |  |
| niwdcnut | 11/20/2010 00:14 | 11/21/2010 1:02 |  |  |
| niwdcnut | 12/10/2010 4:44 | 12/11/2010 5:32 |  |  |
| niwdcnut | 12/30/2010 9:52 | 12/31/2010 10:40 | Did not sample | Holiday |
| niwolnut | 1/4/2010 4:21 | 1/5/2010 5:09 |  |  |
| niwolnut | 1/24/2010 8:20 | 1/25/2010 9:08 |  |  |
| niwolnut | 2/13/2010 1:20 | 2/14/2010 2:08 |  |  |
| niwolnut | 3/5/2010 5:29 | 3/6/2010 6:17 |  |  |
| niwolnut | 3/25/2010 9:49 | 3/26/2010 10:37 |  |  |
| niwolnut | 4/14/2010 1:48 | 4/15/2010 2:36 |  |  |
| niwolnut | 5/4/2010 6:10 | 5/5/2010 6:58 |  |  |
| niwolnut | 5/24/2010 10:50 | 5/25/2010 11:38 |  |  |
| niwolnut | 6/13/2010 2:28 | 6/14/2010 3:16 |  |  |
| niwolnut | 7/3/2010 6:17 | 7/4/2010 7:05 |  |  |
| niwolnut | 7/23/2010 11:54 | 7/24/2010 12:42 |  |  |
| niwolnut | 8/12/2010 3:18 | 8/13/2010 4:06 |  |  |
| niwolnut | 9/1/2010 6:30 | 9/2/2010 7:18 |  |  |
| niwolnut | 9/21/2010 00:16 | 9/22/2010 1:04 |  |  |
| niwolnut | 10/11/2010 4:01 | 10/12/2010 4:49 |  |  |
| niwolnut | 10/31/2010 8:02 | 11/1/2010 8:50 |  |  |
| niwolnut | 11/20/2010 00:14 | 11/21/2010 1:02 |  |  |
| niwolnut | 12/10/2010 4:44 | 12/11/2010 5:32 |  |  |
| niwolnut | 12/30/2010 9:52 | 12/31/2010 10:40 | Did not sample | Holiday |
| niwtanut | 1/4/2010 5:51 | 1/5/2010 6:39 |  |  |
| niwtanut | 1/24/2010 9:50 | 1/25/2010 10:38 |  |  |
| niwtanut | 2/13/2010 2:50 | 2/14/2010 3:38 |  |  |
| niwtanut | 3/5/2010 6:59 | 3/6/2010 7:47 |  |  |
| niwtanut | 3/25/2010 11:19 | 3/26/2010 12:07 |  |  |
| niwtanut | 4/14/2010 3:18 | 4/15/2010 4:06 |  |  |
| niwtanut | 5/4/2010 7:40 | 5/5/2010 8:28 |  |  |
| niwtanut | 5/24/2010 12:20 | 5/25/2010 13:08 |  |  |
| niwtanut | 6/13/2010 3:58 | 6/14/2010 4:46 |  |  |
| niwtanut | 7/3/2010 7:47 | 7/4/2010 8:35 |  |  |
| niwtanut | 7/23/2010 13:24 | 7/24/2010 14:12 |  |  |
| niwtanut | 8/12/2010 4:48 | 8/13/2010 5:36 |  |  |
| niwtanut | 9/1/2010 8:00 | 9/2/2010 8:48 |  |  |
| niwtanut | 9/21/2010 1:46 | 9/22/2010 2:34 |  |  |
| niwtanut | 10/11/2010 5:31 | 10/12/2010 6:19 | All samples missed |  |
| niwtanut | 10/31/2010 9:32 | 11/1/2010 10:20 |  |  |
| niwtanut | 11/20/2010 1:44 | 11/21/2010 2:32 |  |  |
| niwtanut | 12/10/2010 6:14 | 12/11/2010 7:02 |  |  |
| niwtanut | 12/30/2010 11:22 | 12/31/2010 12:10 | Did not sample | Holiday |

- Associated researchers and projects

In addition to the SWMP tier I parameters (Ammonium, nitrite +nitrate, nitrite, orthophosphate, chlorophyll and salinity) NIW NERR analyzes for several tier II parameters.  NIW NERR processes the diel and grab samples for total and nonvolatile suspended solids (TSS and NVSS respectively), total nitrogen (TN), total phosphorus (TP) and dissolved organic carbon (DOC).  Samples for TSS and NVSS are collected on pre-burned, pre-weighed Whatman GF/F glass fiber filters.  Filters are dried at 45° C and weighed to obtain TSS; NVSS is measured after combusting the filters at 450° C for four hours.  Samples for TN and TP are analyzed using the NO23 and PO4 methods after the samples are digested with an alkaline persulfate solution.  Samples for DOC are filtered and acidified before analysis; they are analyzed on a Shimadzu TOC-V instrument by combustion and non-dispersive infrared detection.  Data are available from the NIW NERR.

The NIW NERR Water Quality Monitoring Project also has stations located at Oyster Landing, Clambank Creek, Debidue Creek, and Thousand Acre sites.  The principal objective of this study is to record long-term water quality data for North Inlet/Winyah Bay in order to observe any physical changes or trends in water quality over time.  Measurements are currently taken every 15 minutes over roughly two week collecting periods at the Oyster Landing, Clambank Creek, Debidue Creek and Thousand Acre sites.    Historical water quality data from these sites is available online at http://cdmo.baruch.sc.edu/.

The NIW NERR weather station is located at the Oyster Landing site.  Parameters measured include: air temperature, humidity, barometric pressure, solar radiation (total and PAR), wind speed, wind direction, and precipitation.  Historical meteorological data are available from Oyster Landing online at http://cdmo.baruch.sc.edu/.

A long-term fish survey is also done at the Oyster Landing site bi-weekly/monthly depending on the season.  Species present, total weight, individual weights and standard lengths of fish that utilize the marsh at high tide are recorded.

- Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format.

- Physical Structure Descriptors

### Entry Verification

See section 17 Subsection C for a detailed description of additional laboratory personnel and responsibilities during 2009.

## Data recorded on the sediment parameter sheets include:  chemistry #, date, site, event #, sample #, time, salinity, tide stage designation, filter #, filter weight, total weight, and oxidized weight.  Hardcopies of these datasheets are maintained on site in the Baruch Marine Field Lab’s water chemistry laboratory fireproof cabinet.

Information on the samples in each tray of nutrient samples run through the technicon is recorded on a “Run Sheet” by the water chemistry technician.  The technicon has a datalogger that calculates the concentrations, puts them into a temporary text file, and produces an output printout with the same information.  Ben Lakish imported data into an Excel spreadsheet where the appropriate calculations are made.  Anomalous data points and missing data are noted on the original handwritten data sheet for transfer to the Excel file.  Missing data points are denoted with a blank cell.  The MS Excel data file is then verified for typographical errors by Ben Lakish or Amy Willman and formatted and processed to CDMO specifications using the NutrientQAQC macro.

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; adds chosen parameters and facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and then automatically flags/codes measured values below MDL and inserts the MDL ; calculates parameters chosen by the user and automatically flags/codes for component values below MDL, negative calculated values, and missing data; allows the user to apply QAQC flags and codes to the data; produces summary statistics; graphs selected parameters for review; and exports the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

All Baruch Marine Field Lab (BMFL) nutrient chemistry is measured using umole /L (micromoles (N or P) per liter), except for chlorophyll a which is measured in ug/L.  Data for BMFL is reported using these units.  However, for the NERR SWMP nutrient data, these data were converted to mg/L (except chlorophyll a) using the following conversion factors:

1 µmol N=0.01401 mg/L as N

1 µmol P=0.03097 mg/L as P

- Parameter Titles and Variable Names by Data Category

Below are the parameters and variable names reported in the 2009 NIW NERR Nutrient dataset.  Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisks “*”.  Data for additional parameters listed in section 7 are available through the Baruch Marine Field Laboratory’s (BMFL) online database (http://links.baruch.sc.edu/data/) or obtained on request by contacting the Data Manager at datamgr@belle.baruch.sc.edu.   All sample collection times are based on a 24:00-hour clock and are given in Eastern Standard Time.

Data Category		Parameter				Variable Name	Units of Measure

Phosphorus:		*Orthophosphate				PO4F		mg/L as P

Nitrogen:		*Nitrite + Nitrate, Filtered			NO23F		mg/L as N

*Ammonium, Filtered			NH4F		mg/L as N

*Nitrite					NO2F		mg/L as N

*Nitrate					NO3F		mg/L as N

Dissolved Inorganic Nitrogen		DIN		mg/L as N

Other Lab Parameters:	Chlorophyll a				CHLA_N	g/L

Salinity					SALT_N	ppt

Notes:

1.  Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

2.  The following field parameters from each of the four NIW NERR monitoring stations are available through the BMFL database (http://links.baruch.sc.edu/data/) or through the Centralized Data Management Office (CDMO) online Water Quality database at http://cdmo.baruch.sc.edu/.:  Dissolved oxygen, dissolved oxygen saturation, salinity, pH, and water temperature.

##### Measured and Calculated Laboratory Parameters

Listed below are those water chemistry parameters that are determined directly through chemical analysis (parameters measured directly) and those parameters that derived through calculations (parameters calculated) using parameters that are measured directly.

### a) Parameters Measured Directly

Nitrogen species:		NH4, NO2, NO23

Phosphorus species:	PO4

Other:			CHLA

### b) Calculated Parameters

NO3			NO23-NO2

### DIN			NO23 + NH4

- Limits of Detection

Minimum detection limits are determined using seven replicate measurements of a water sample collected from niwolnut.  The method detection limit is estimated as 3.14 times the standard deviation of the replicate measurements.  Below are the method detection limits for the major parameters measured.

### Analytical Parameter		Method Detection Limit		Dates in Use

Ammonium (NH4F)		0.0050 mg/L as N		1/1/2010 – 12/31/2010

Nitrite+Nitrate (NO23F)		0.0016 mg/L as N		1/1/2010 – 12/31/2010

Nitrite (NO2F)			0.0008 mg/L as N		1/1/2010 – 12/31/2010

Orthophosphate (PO4F)		0.0013 mg/L as P			1/1/2010 – 12/31/2010

Chlorophyll a (CHLA_N)		0.08 µg/L			1/1/2010 – 12/31/2010

- Laboratory Methods

All NIW NERR water samples are collected, processed, and analyzed by the Baruch Marine Field Laboratory Water Chemistry Lab.  Descriptions for analysis and reporting of data given below are for the parameters currently submitted by the NIW NERR.  Only those data that are required for submission by the National Estuarine Research Reserve System are included with this report and data submission.  Data for additional parameters can be accessed through the Baruch Marine Field Laboratory’s (BMFL) online database (http://links.baruch.sc.edu/data/) or obtained on request by contacting the Data Manager at datamgr@belle.baruch.sc.edu.

###### General Laboratory Protocol

Before use, all glassware and ISCO sample bottles are scrubbed thoroughly in hot tap water, soaked in a 10% HCL bath for a minimum of 24 hours, rinsed 4 times with deionized water and then dried in a drying oven.  The same ISCO sampler and intake hose is used at the same site each time due to the different set-up required for each site (hose length, etc.).

Stock standards are prepared in 1liter batches, fixed with chloroform and stored at 4 oC.  Working standards for each sampling interval are prepared fresh each time from stock standards.  The water samples are all analyzed at the same range for all sites and seasons with the exception of a higher standard set used for determination of NO23F at the Thousand Acre (TA) site.  The following standard concentrations are used:

Parameter	Concentrations (mg/L) used for standard curve

PO4F		0, 0.0496, 0.0744, 0.0992

NO23F		0, 0.028, 0.042, 0.056

NO2F		0, 0.0056, 0.0112, 0.0168, 0.0224

NH4F		0, 0.028, 0.056, 0.084, 0.112

NO23F (TA site)  0, 0.07, 0.14, 0.28

These concentrations are used to generate the standard curve from which sample concentrations are determined.

Samples are generally processed within 48 hours of collection.  Salinity values are measured with a Mettler SG7 handheld conductivity meter, calibrated prior to use.  Seventy-five to 500 ml of each water sample is filtered through a preweighed, precombusted Whatman GF/F (0.7 micrometer nominal pore size) filter.   Samples are gently shaken before filtering begins; the amount of water filtered is determined by the particulate load of the sample (up to 500 mL during winter, less during summer and after periods of heavy rains).   If necessary, phenol is added to the NH4F samples for preservation.  NH4F, NO2F, NO23F, and PO4F are then stored at 4 0C.   Magnesium carbonate (MgCO3) is added to Chlorophyll a samples before they are frozen.

NO23F, PO4F, NO2F, and NH4F aliquots are analyzed with a Technicon AutoAnalyzer.  Chlorophyll a samples are analyzed using a Turner Designs Trilogy fluorometer.

The Technicon produces output printouts with raw nutrient value concentrations.  With the Technicon model used for this database, there are no peak heights; the technicon converts the peak heights directly into a nutrient concentration.  Results from all other analyses are recorded by hand.

Nitrite-Nitrate (NO23F)

NO23F concentrations are determined on GF/F (0.7 micrometer nominal pore-size) filtered sample water. The basic method is Technicon Method No. 158-71W/B (Technicon Industrial Systems 1979).  Nitrate is reduced to nitrite by a copper-cadmium reductor column.  The nitrite ion then reacts with sulfanilamide under acidic conditions to form a diazo compound, which couples with N-1-napthylethylenediamine dihydrochloride to form a reddish-purple azo dye, the color of which is determined as absorbance at 550 nm wavelength.  For a discussion of the problems associated with EDTA as the buffer choice (Brewer & Riley, 1965) refer to Gilbert & Mlodzinska (1977).

Technicon Industrial Systems. 1979. Technicon Industrial Method No. 158-71W/B; Nitrate-Nitrite in Water

and Seawater. Technicon Industrial Systems; A Division of Technicon Instruments Corporation, Tarrytown, NY 10591.

Brewer, P. G.  and Riley ,J. P. 1965. The automatic determination of nitrate in sea water.  Deep Sea Research and Oceanographic Abstracts, Volume 12, Issue 6, December, Pages 765-772.

Glibert P.M., Mlodzinska, Z., and C.F. D’Elia. 1977.  A semi-automated persulfate oxidation technique for simultaneous total nitrogen and total phosphorus determination in natural water samples.  Woods Hole Oceanographic Institution Contribution Number 3954.  Ocean Industry Program of the Woods Hole Oceanographic Institution, Woods Hole, Massachusetts 02543.

Nitrite (Nitrite NO2F)

NO2 F concentrations are determined on GF/F (0.7 micrometer nominal pore-size) filtered sample water. The basic method is Technicon Method No. 158-71W/B (Technicon Industrial Systems 1979).  The nitrite ion reacts with sulfanilamide under acidic conditions to form a diazo compound.  This compound then couples with N-1-napthylethylenediamine dihydrochloride to form a reddish-purple azo dye, the color of which is determined as absorbance at 550 nm wavelength.  There are very few known interferences at concentrations less than 1000 times that of the nitrite; however, recent addition of strong oxidants or reductants to the samples will readily affect the nitrite concentrations. High alkalinity (600 mgL-1) will give low results due to a shift in pH of the color reaction.

Technicon Industrial Systems. 1976. Technicon Industrial Method No. 161-71W/B; Nitrate-Nitrite in Water

and Seawater. Technicon Industrial Systems; A Division of Technicon Instruments Corporation, Tarrytown, NY 10591.

Orthophosphate (PO4F)

PO4F concentrations are determined on GF/F (0.7 micrometer nominal pore-size) filtered sample water.  The basic method is the Technicon Industrial Method No. 155-71W (1973), which is a modification of the Murphy and Riley (1962) single solution method. The method depends on the formation of a phosphomolybdate blue complex, the color of which is determined as absorbance at 880 nm wavelength.

Technicon Industrial Systems. 1973. Technicon Industrial Method No. 155-71W; Ortho Phosphate in Water

and Seawater. Technicon Industrial Systems; A Division of Technicon Instruments Corporation, Tarrytown, NY 10591.

Murphy, J. and J. P. Riley. 1962. A Modified Single Solution Method for the Determination of Phosphate in

Natural Waters. Anal. Chim. Acta 27:31.

Ammonium (NH4F)

NH4F concentrations are determined on GF/F (0.7 micrometer nominal pore-size) filtered sample water.  The method used is based on that of Grasshoff and Johannsen (1972) and O'Connor and Miloski (1974), with a few modifications. Essentially the method is depends on the Berthelot Reaction.  First, the sample is turned basic by the addition of buffer containing sodium hydroxide; sodium citrate and boric acid are used to prevent precipitation of the hydroxides of calcium and magnesium.  Next, a blue colored compound closely related to indophenol forms when the solution of an ammonium salt is added to sodium phenoxide, followed by the addition of sodium hypochlorite (Glibert and Loder 1977), the color of which is determined as absorbance at 630 nm wavelength.

Glibert, Patricia M. and Theodore C. Loder. 1977. Automated Analysis of Nutrients in Seawater:  A Manual of

Techniques. Publication Number WHOI-77-47. Woods Hole Oceanographic Institution, Woods Hole, Massachusetts 02543.

Grasshoff, K and J. Johannsen.  (1972).  A new sensitive and direct method for the automatic determination of ammonia in seawater.  J. Cons. Int. Explor. Mer 34:516.

O'Connor, B.  and W. Miloski.  (1974).  Ammonia analysis of seawater using a Technicon AutoAnalyzer II.  Suffolk County Department of Environmental Control.  Unpublished manuscript.

Chlorophyll a (CHLA_N)

An appropriate amount of sample (10 or 20 ml) is filtered through a 2.5 cm, (0.7 micrometer, nominal pore size) GF/F filter. The filter is then placed into a scintillation vial with 1 ml of saturated magnesium carbonate (MgCO3) and frozen (-15 0C).  Samples are removed from the freezer, 9 milliliters of 100 percent acetone are added, and then they are shaken and placed in the refrigerator  (1 0C ) for 24 hours.  The samples are removed from the refrigerator, carefully shaken, and then returned to the refrigerator for an additional 24 hours.  After the extraction process is complete, the samples are analyzed with a Turner Designs Trilogy fluorometer.  Analytical range can be adjusted by changing the volume of sample filtered or be diluting the extracted sample with acetone.  Samples are analyzed using a narrow wavelength optical module which eliminates interference by pheophytin.

Turner Designs Inc. Manual.  1981. Fluorometric Facts, Chlorophyll and Phaeophytin.  Ballentin 101,

Mountain View, California.

Greenberg, Arnold E., Connors, Joseph J., Jenkins, David, and Mary Ann H. Franson. 1980. Standard Methods

For the Examination of Water and Waste Water, 15th Edition.  American Public Health Association, 1015 Fifteenth Street NW, Washington, DC 2000.

Glover, L.R. and I. Morris.  1979.  Photosynthetic Carboxylating Enzymes in Marine Phytoplankton. Limnol.

Oceanography 23:80-89.

Arar, Elizabeth J. and Collins, Gary B.  1997.  In Vitro Determination of Chlorophyll a and Pheophytin a in 	Marine and Freshwater Algae by Fluorescence.  EPA Method 445.0  U.S. Environmental Protection 	Agency, Cincinnati, Ohio

14)  Field and Laboratory QAQC programs

Various control measures are taken to monitor the operation of the Technicon AutoAnalyzers.  All standards are made from dried ACS grade chemicals; NO23F, PO4F, and NH4F standards are verified by comparison to nitrate, phosphate and ammonia check standards from the Hach Company (Loveland Colorado).

Additional control measures are used with the Fluorometer.  Chlorophyll a values are determined by standard method of Fluorometric Analysis.  The fluorometer is calibrated by running standards available from Turner Designs or by using chlorophyll a standard purchased from Sigma.

  - Precision:

    - Field Variability – NIW uses an ISCO automated water sampler to collect both the diel and the successively collected monthly grab samples.

    - Laboratory Variability – The Baruch Marine Field Laboratory Water Chemistry Lab analyzes triplicate samples for every OL sample number 10 of the Diel water samples.

    - Inter-organizational splits – None

  - Accuracy:

    - Sample Spikes –None

    - Standard Reference Material Analysis –information not received

    - Cross Calibration Exercises – None

15) QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

16)  QAQC code definitions:

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data suspect due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Calculated value could not be determined due to a below MDL component

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP		scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR		mixed rain and snow

Tide stage

TSE		ebb tide

TSF		flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N		from the north

NNE	from the north northeast

NE		from the northeast

ENE	from the east northeast

E		from the east

ESE		from the east southeast

SE		from the southeast

SSE		from the south southeast

S		from the south

SSW	from the south southwest

SW		from the southwest

WSW	from the west southwest

W		from the west

WNW	from the west northwest

NW		from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17)  Other remarks/notes

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

a) Precipitation during 2009

| 2009 Daily, Monthly, and Annual Precipitation Totals for Oyster Landing Met Station |  |  |
| --- | --- | --- |
| Date Daily Precip Daily Precip Total(mm) Total (in) |  |  |
| 1/1/2010; 0.25; 0.01. 1/8/2010; 0.25; 0.01. 1/16/2010; 4.83; 0.19. 1/17/2010; 14.73; 0.58. 1/21/2010; 13.97; 0.55. 1/24/2010; 0.51; 0.02. 1/25/2010; 22.35; 0.88. 1/30/2010; 19.30; 0.76. January Monthly Total; 76.20; 3.00. 2/2/2010; 24.64; 0.97. 2/5/2010; 52.58; 2.07. 2/6/2010; 0.51; 0.02. 2/9/2010; 13.72; 0.54. 2/12/2010; 1.27; 0.05. 2/13/2010; 9.40; 0.37. 2/15/2010; 1.52; 0.06. 2/22/2010; 3.81; 0.15. 2/23/2010; 0.25; 0.01. 2/24/2010; 3.05; 0.12. 2/25/2010; 0.25; 0.01. February Monthly Total; 111.00; 4.37. 3/2/2010; 19.56; 0.77. 3/3/2010; 0.25; 0.01. 3/11/2010; 23.62; 0.93. 3/12/2010; 0.25; 0.01. 3/13/2010; 1.78; 0.07. 3/17/2010; 5.59; 0.22. 3/21/2010; 1.02; 0.04. 3/22/2010; 2.54; 0.10. 3/23/2010; 0.25; 0.01. 3/29/2010; 53.59; 2.11. 3/30/2010; 0.25; 0.01. March Monthly Total; 108.71; 4.28. 4/8/2010; 11.94; 0.47. 4/9/2010; 6.60; 0.26. 4/20/2010; 0.51; 0.02. 4/21/2010; 4.32; 0.17. 4/25/2010; 7.87; 0.31. 4/27/2010; 0.76; 0.03. April Monthly Total; 32.00; 1.26. 5/4/2010; 30.48; 1.20. 5/17/2010; 2.79; 0.11. 5/18/2010; 14.99; 0.59. 5/22/2010; 0.51; 0.02. 5/23/2010; 4.57; 0.18. 5/24/2010; 1.78; 0.07. 5/25/2010; 6.35; 0.25. 5/26/2010; 0.51; 0.02. 5/27/2010; 1.02; 0.04. 5/29/2010; 27.18; 1.07. May Monthly Total; 90.17; 3.55. 6/1/2010; 38.61; 1.52. 6/2/2010; 2.79; 0.11. 6/3/2010; 0.76; 0.03. 6/4/2010; 6.35; 0.25. 6/13/2010; 4.32; 0.17. 6/17/2010; 1.27; 0.05. 6/20/2010; 13.46; 0.53. 6/25/2010; 1.52; 0.06. 6/27/2010; 37.08; 1.46. 6/28/2010; 17.02; 0.67. 6/29/2010; 0.25; 0.01. 6/30/2010; 5.08; 0.20. June Monthly Total; 128.52; 5.06. 7/1/2010; 34.54; 1.36. 7/9/2010; 8.89; 0.35. 7/10/2010; 23.88; 0.94. 7/12/2010; 0.76; 0.03. 7/13/2010; 6.60; 0.26. 7/14/2010; 7.37; 0.29. 7/15/2010; 64.26; 2.53. 7/17/2010; 4.83; 0.19. 7/20/2010; 1.27; 0.05. 7/21/2010; 3.81; 0.15. 7/22/2010; 0.76; 0.03. 7/26/2010; 2.54; 0.10. 7/27/2010; 25.15; 0.99. 7/28/2010; 0.51; 0.02. 7/29/2010; 9.91; 0.39. July Monthly Total; 195.07; 7.68. 8/1/2010; 23.11; 0.91. 8/6/2010; 38.86; 1.53. 8/13/2010; 21.08; 0.83. 8/14/2010; 8.38; 0.33. 8/19/2010; 5.59; 0.22. 8/20/2010; 0.25; 0.01. 8/22/2010; 16.76; 0.66. 8/23/2010; 6.86; 0.27. 8/24/2010; 16.76; 0.66. 8/27/2010; 4.06; 0.16. 8/28/2010; 0.25; 0.01. August Monthly Total; 141.99; 5.59. 9/6/2010; 3.81; 0.15. 9/11/2010; 0.76; 0.03. 9/12/2010; 11.68; 0.46. 9/26/2010; 35.31; 1.39. 9/27/2010; 133.86; 5.27. 9/28/2010; 27.43; 1.08. 9/29/2010; 55.88; 2.20. 9/30/2010; 12.45; 0.49. September Monthly Total; 281.18; 11.07. 10/14/2010 16:45; 12.19; 0.48. 10/24/2010 23:45; 0.51; 0.02. 10/25/2010 20:00; 27.43; 1.08. 10/28/2010 18:00; 3.30; 0.13. October Monthly Total; 43.43; 1.71. 11/04/2010 18:45; 9.40; 0.37. 11/16/2010 06:30; 0.25; 0.01. 11/26/2010 17:30; 4.57; 0.18. 11/30/2010 17:30; 2.03; 0.08. November Monthly Total; 16.26; 0.64. 2/11/2010 13:15; 14.73; 0.58. 12/12/2010 14:15; 4.32; 0.17. 12/18/2010 21:30; 20.07; 0.79. 12/25/2010 23:45; 4.83; 0.19. 12/26/2010 14:15; 19.56; 0.77. December Monthly Total; 63.50; 2.50. |  |  |

c) Summary of Laboratory Personnel during 2010.

Ben Lakish, Research Specialist: Entered data for almost all samples; verified entry and format of most of the 2010 data.

Amy Willman, Research Specialist: assisted with manual data entry and completed most 	of the data entry verification, formatted data, and edited the metadata document for 2010.

Tracy Buck, Research Specialist:  assisted with manual data entry.

- Summary of missing data during 2010.

On 6/13/2010 2:28 and 14:52 niwdcnut samples missed due to a tangled ISCO tube that did not go up and down with the tide.

On 10/12/2010 4:51 and 4:53 niwolnut samples missed due to distributor arm on ISCO getting jammed (due to overfilling ice jugs which defrosted, overflowed, and caused bottles to float up and hit arm).

On 10/12/2010 00:41 – 4:53 niwdcnut samples missed due to distributor arm on ISCO getting jammed (due to overfilling ice jugs which defrosted, overflowed, and caused bottles to float up and hit arm).

From 10/11/2010 – 10/12/2010 all niwtanut samples missed due to distributor arm on ISCO getting jammed (due to overfilling ice jugs which defrosted, overflowed, and caused bottles to float up and hit arm).

On 11/21/2010 1:04 niwcbnut sample salinity is missing due to not having enough sample left after filtering.

From 12/30/2010 – 12/31/2010 samples from all sites were not collected due to sampling date corresponding with the Christmas holiday break.