Rookery Bay (RKB) National Estuarine Research Reserve Meteorological Metadata

January – December 2014

Latest Update: July 17, 2017

I. Data Set & Research Descriptors

1) Principal investigator(s) & contact persons

Principal investigator:

Kevin Cunniff, Research Coordinator

Kevin.cunniff@dep.state.fl.us

Contact person/weather data manager:

Patrick O’Donnell, Environmental Specialist

patrick.odonnell@dep.state.fl.us

Address:

Rookery Bay NERR

300 Tower Rd.

Naples, FL 34113

Phone: 239-530-5940

Fax: 850-412-0501

2) Entry verification

Patrick O’Donnell, Environmental Specialist, was responsible for sensor calibration, monthly downloads and weather data management and reporting.  The CR1000 program rkbuhmet_V6.0_020811.csi controlled the sensors and data collection schedule until the re-installation of the newly calibrated CR1000 with rkbuhmet_V6.0_070714 on 08/05/2014 at 11:45.  Fifteen minute meteorological data were downloaded from each instrument on the weather station to a Campbell Scientific CR1000 datalogger from 01/01/2014 00:00 until 12/31/2014 23:45.  The CR1000 then interfaced with LoggerNet software supplied by Campbell Scientific. This software was located on a laptop computer to which the data were uploaded approximately every month in a comma-delimited format.  The data were saved as monthly raw data files (.dat) onto a separate hard drive and backed up onto the Rookery Bay NERR server.  The files were uploaded to the CDMO where they underwent automated primary QAQC and became part of the CDMO’s online provisional database.  During primary QAQC, data were flagged if they are missing or out of sensor range.  The edited files were then returned to the Reserve where they were opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserted station codes, created metadata worksheets for flagged data and summary statistics, and graphed the data for review.  It allowed the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

3) Research objectives:

As part of the National Estuarine Research Reserve’s (NERR) System-Wide Monitoring Program (SWMP), the principal objective was to record long-term meteorological data at Rookery Bay in order to observe any environmental changes or trends over time. Secondary objectives included using the meteorological data to help document periodic biological and environmental events such as:  fish kills, red tides, wildfires, prescribed burns, and hurricanes/tropical storms.  It also supported other research conducted at the reserve including providing valuable ancillary data for the reserve's water quality monitoring programs.

4) Research methods:

From January 1, 2014 - December 31, 2014 the weather station (operated by the Campbell Scientific

CR1000 datalogger) collected data every 15 minutes in the following formats:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), and Battery Voltage (volts) based on averages from 5-second data.

Maximum Air Temperature (°C), Minimum Air Temperature (°C), and their times from 5-second data (these data are available from the Reserve).

Maximum Wind Speed (m/s) and its time and Wind Direction Standard Deviation (degrees) from 5-second data.

Precipitation (mm), Cumulative Precipitation (mm), and Photosynthetic Active Radiation (PAR; millimoles/m2) based on totals of 5-second data.

Following the NERR SWMP Campbell Scientific CR1000 Meteorological Monitoring Station Standard Operating Procedure (SOP), every month the weather station was inspected and a field log was completed to document the condition of the weather station.  The CR1000 enclosure was examined, the wiring panel was checked for loose wires, the desiccant packs were replaced, and the barometric pressure vent was inspected for obstructions.  Additionally, each of the sensors, the solar panel, and the cable connections on the weather station were examined for damage or debris.  If any was found, it was repaired and/or cleaned.  To determine the accuracy of the sensors, real-time data from the weather station was recorded in the field log and compared to data from another local weather station.  Sensors were removed and sent back to Campbell Scientific or Li-Cor for recalibration as recommended by the NERR SWMP Campbell Scientific CR1000 Meteorological Monitoring Station SOP.

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- yearly recalibration

- Rain Gauge- yearly recalibration

- Wind Speed/Direction- yearly or every 2 years (depending on the sensor)

- Barometric Pressure- every 2 years recalibration

- PAR- every 2 years recalibration

- CR1000-every 5 years (required beginning 2014, one year initial grace period)

After inspection of the weather station, the data was uploaded from the CR1000 to a laptop.  After submitting the files to CDMO for automated primary QAQC as described in Section 2, the flagged data and graphs were reviewed within one week of the data download.  Data were further flagged and coded based on the field notes and review of the graphs.  There were no other analyses done on the meteorological data.  If erroneous data were found the sensor(s) underwent troubleshooting to determine if there was a problem that warranted removal and repair.

Campbell Scientific data telemetry equipment was installed at this station (rkbuhmet) on 05/13/2006 to transmit data to the NOAA GOES satellite, NESDIS ID #3B01D3E8.   The transmissions were scheduled hourly and contained four (4) datasets reflecting the fifteen minute data sampling intervals. Upon receipt by the CDMO, the data underwent the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data became part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

5) Site location and character:

Rookery Bay is located in the Tropical Rainy climatic group of Koppen (1931), i.e. the area below a west-east line extending from Ft. Myers to Melbourne, and where the mean temperature does not fall below 17.7°C (64°F) in the coolest month of the year.  Owing to the influence of the warm-water Florida Current, the seasonal effects from the Gulf of Mexico Loop Current, and its geographical position at 26° N latitude, the average annual temperature in the Rookery Bay  area is about 24°C (75°F).  Winter temperatures range from -1°C (ca. 30°F) to about 26°C (75°F), with cooler days and nights (10-15°C) in the months of January and February.  Warming trends in April and May are frequently modified by blustery winds from the southwest off the Gulf of Mexico, and by late season cold fronts with northerly breezes.  Summer high temperatures approach 35°C (95°F) or higher on occasion (Thomas 1974).

Rookery Bay and vicinity have an annual rainfall of 50-55 inches (127-140 cm) per year (Thomas 1974).  The heaviest average monthly rainfall, 8-9 inches per month, occurs from June through September.  Lowest average rainfall, 1-2 inches per month, occurs from November through March.  Approximately 66% of the total yearly rainfall occurs between the months of June and October.  Southwest Florida lies in the seasonal tropical weather belt that channels hurricanes toward or along the coast.  Historically, the area has been fortunate in that few severe hurricanes have come ashore.

An 18.2 m tower and the weather station are located a few miles to the northwest of the ELC building on a cleared area with a limestone dirt lot at about 1.8 m above sea level (latitude: 26° 03’00.24' N/ longitude: 81° 42’06.18' W).  The weather station is located approximately 2.5 miles upstream from the water quality site located in lower Henderson Creek.  There is minimal natural vegetation below the tower and weather station.  The weather station is approximately 9.1 m – 60.6 m away from the about 9-18 m high tree line surrounding it.  The CR1000 enclosure housing the GOES transmitter, battery, and the barometric pressure sensor were mounted approximately 1.2 m above the ground on a 2.7 m pole that was set in concrete next to the tower.  The Li-Cor PAR and temperature/relative humidity sensors were mounted on a cross bar about 1.85 m above the ground on the east and west sides of the pole respectively.  The rain gauge was mounted 2.7 m above the ground at the top of the pole.  The WS425 ultrasonic wind sensor was mounted on the north side of the tower at about 18.2 m above the ground (approximately 20 m above sea level) and positioned for true north.  Sensors were wired according to NERR SWMP Campbell Scientific CR1000 Meteorological Monitoring Station SOP.

6) Data collection period:

Weather data have been collected at the Henderson Creek/Shell Island Road site since 1996.  The weather station at the ELC began collecting data on January 7, 2004 at 13:15 and ended on June 12, 2009 at 8:30. The weather station at the ELC was moved to its existing location in October 2009 and began collecting data on 10/28/09 at 12:10.  The data collection for 2014 began on January 1, 2014 at 00:00 and continued through December 31, 2014 at 23:45.

7) Distribution:

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2012.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

8) Associated researchers and projects:

Rookery Bay participates in the NERR SWMP for water quality and has four monitoring locations within the Reserve boundary.  The principal objective of this study is to record long-term water quality data within Rookery Bay in order to observe any physical changes or trends in water quality over time.  The four sites were selected to represent various degrees of watershed hydrologic alteration.  The Henderson Creek water quality monitoring station is located approximately 1-2 miles downstream from this weather station on upper Henderson Creek.  Measurements are taken every 15 minutes including temperature, salinity, conductivity, pH, turbidity, dissolved oxygen and depth.

Additionally Rookery Bay participates in the SWMP for long-term nutrient monitoring.  The parameters included in this project are soluble reactive phosphorus (filtered), ammonium (filtered), nitrite (filtered), nitrate (filtered), nitrite + nitrate (filtered), chlorophyll a. Both water quality and nutrient data generated for the Rookery Bay are being used to analyze restoration targets established for the Picayune Strand Restoration Project (formerly known as Southern Golden Gate Estates) which is a portion of the Comprehensive Everglades Restoration Project (CERP).

A long-term shark population survey is also done in the corresponding SWMP water quality sites monthly.

II. Physical Structure Descriptors

9) Sensor specifications, operating range, accuracy, date of last calibration, dates of deployment:

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #: HMP35C Temperature and Relative Humidity Probe

Operating Temperature:  -40 to 60°C

Range: -40 to 60°C

Accuracy: ± 0.2 °C @ 20°C

Date of last calibration:  	02/13/2013 (serial #666191), 03/27/2014 (serial #2662-01)

Dates of deployment: 	03/05/2013 - 05/01/2014 (serial #666191), 05/01/2014 - current as of 12/31/2014 (serial #2662-01)

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #: HMP35C Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy @ 20°C:  +/-2% RH (0-90%) and +/-3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Uncertainty of calibration: ± 1.2% RH

Date of last calibration:  	02/13/2013 (serial #666191), 03/27/2014 (serial #2662-01)

Dates of deployment: 	03/05/2013 - 05/01/2014 (serial #666191), 05/01/2014 - current as of 12/31/2014 (serial #2662-01)

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105  	Vaisala PTB101B 	serial #Y4810007

Operating Range: Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity: non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Date of Last calibration:  	04/10/2014 (serial #Y4810007)

Dates of deployment: 		08/05/2014 - 09/02/2014 (serial #Y4810007)

Removed from service 	09/02/2014

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-106	Vaisala PTB110	serial # H1230003

Operating Range: Pressure: 500 to 1100 mb; Temperature:  -40°C to +60°C;

Humidity: non-condensing

Accuracy: ± 0.3 mb @ 20°C; +/- 0.6mb@0°C to 40°C; +/- 1.0mb@-20°C to 45°C; +/- 1.5mb@-40°C to 60°C

Stability: ± 0.1 mb per year

Date of Last calibration:  	3/26/2012 (serial #H1230003)

Dates of deployment: 	05/09/2012 -08/05/2014 and 09/02/2014 - 10/29/2014 (serial # H1230003)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-106	Vaisala PTB110	serial #K2330005

Operating Range: Pressure: 500 to 1100 mb; Temperature:  -40°C to +60°C;

Humidity: non-condensing

Accuracy: ± 0.3 mb @ 20°C; +/- 0.6mb@0°C to 40°C; +/- 1.0mb@-20°C to 45°C; +/- 1.5mb@-40°C to 60°C

Stability: ± 0.1 mb per year

Date of Last calibration:  	06/04/2014 (serial # K2330005)

Dates of deployment: 		10/29/2014 - current as of 12/31/2014 (serial # K2330005)

Parameter: Wind Speed

Units: meters per second (m/s)

Sensor type: Vaisala ultrasonic wind sentry

Model #: WS425 A1E2B	serial # B1540026

Range: 0-65 m/s

Accuracy: ± 0.135 m/s or 3% of the reading

Operating Temperature:-40 to 55 °C

Date of last calibration: 	04/13/2006

Dates began using data: 	07/05/2007 - current as of 12/31/2014

Parameter: Wind Direction

Units: degrees

Sensor type: Vaisala ultrasonic wind sentry

Model #: WS425 A1E2B	serial # B1540026

Range: 0-360°

Accuracy: +/- 2°

Operating Temperature: -40 to 55 °C

Date of last calibration: 	04/13/2006

Dates began using data: 	07/05/2007 - current as of 12/31/2014

Parameter: Photosynthetic Active Radiation (PAR)

Units: mmoles/m2 (total flux)

Sensor type: LiCor Quantum Sensor- High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SB

Light spectrum wavelength:  400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability:  < ±2% change over 1 yr

Operating Temperature:  -40 to 65°C; Humidity: 0 to 100%

Sensitivity:  typically 5 µA per 1000µmoles s-1 m-2

Date of last calibration: 	11/21/2011 (serial #Q42745), 12/12/2013 (serial #Q42698)

Dates deployed: 	01/03/2012 - 01/07/2014 (serial #Q42745), 01/07/2014 - current as of 12/31/2014 (serial #Q42698)

Multiplier: 	      1.10 on 01/03/2012 (serial #Q42745), 1.02 on 01/07/2014 (serial #Q42698)

Parameter: Precipitation

Units: millimeters (mm)

Sensor Type: Tipping Bucket Rain Gauge

Model # TR-525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Range: 0.1 mm

Accuracy: +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Date of last calibration: 	07/07/2014 (serial #21099-498), 07/08/2013 (serial #13772-594)

Dates of deployment: 	08/05/2014 - current as of 12/31/2014 (serial #21099-498), 07/09/2013 - 08/05/2014 (serial #13772-594)

Storage Module

CR1000	Serial # 4859

Operating range:  Temperature= -25o to 50oC

Baud rates:  300 bsp to 115.2 kbps

Power requirements:  9.6-16 VDC

Dates of installation: 05/13/2006 - 07/01/2014 (installed new), 08/05/2014 -current as of 12/31/2014

Date of last calibration:  	07/18/2014

The CR1000 has two MB Flash EEPROM that is used to store the Operating System. Another 128 K Flash is used to store configuration settings. A minimum of 2 MB SRAM is (4 MB optional) is available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

CR1000 Firmware Version (s):

21	12/27/2010 – 07/14/2014

27.04	07/14/2014 – 12/31/2014

CR1000 Program Version(s):

RKBUHMET_V6.0_031212.CR1	01/01/2014 – 01/07/2014

RKBUHMET_V6.0_010714.CR1	01/07/2014 – 07/07/2014

RKBUHMET_V6.0_070714.CR1	07/07/2014 – 09/02/2014

RKBUHMET_V6.0_090214.CR1	09/02/2014 – 10/06/2014

RKBUHMET_V6.0_100614.CR1	10/06/2014 – 12/31/2014

10) Coded variable indicator and variable code definitions:

Sampling station:			Sampling site code:		Station code:

Upper Henderson Creek			UH			rkbuhmet

11)  QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the station, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SDG 	Suspect due to sensor diagnostics

SIC 	Incorrect calibration constant, multiplier or offset

SIW 	Incorrect Wiring

SMT	Sensor Maintenance

SNV 	Negative value

SOC 	Out of calibration

SQR 	Data rejected due to QAQC checks

SSD	Sensor Drift

SSN 	Not a number/unknown value

SSM 	Sensor malfunction

SSR 	Sensor removed

Comments

CAF 	Acceptable calibration/accuracy error of sensor

CCU 	Cause unknown

CDF 	Data appear to fit conditions

CML 	Snow melt from previous snowfall event

CRE* 	Significant rain event

CSM* 	See metadata

CVT* 	Possible vandalism/tampering

CWE* 	Significant weather event

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

At night, small negative and positive PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

During 2017 the CDMO discovered an incorrect line in the CR1000 programming.  If RHumidity>100 And RHumidity<108 Then RHumidity=100.  A decision was made by the DMC during 2006 to discontinue correcting >100 RH values to 100. This change was never made in our program and has remained in each updated version until it was removed during 2017.  By correcting all values >100 during data collection we may have missed erroneous values that could have indicated a problem with the RH sensor.  CSM coding was added to all RH data from 2007 until the programming change in 2017.

CR1000 Storage Module:

All data were rejected 01/07/2014 16:15 due to a program reload following the PAR sensor swap.

Data are missing due to re-calibration of CR1000 from 07/01/2014 09:15 to 08/05/2014 11:00.  Data at 11:15 were rejected due to a program upload.

Data collected from 08/05/2014 11:30 to 09/02/2014 09:15 were recovered through the telemetered GOES table file.  The following parameters were divided by 10 or 100 to restore the proper decimal place:

ATemp (max and min, too)/100

RH/10

BP/10

WSpd (and MaxWSpd)/100

Wdir/10

SDWDir/10

TotPrcp(and CumPrcp)/1000

AvgVolt/100

All data are missing 09/02/2014 09:30 - 11:30 due to maintenance to replace the BP sensor.  Data were rejected 09/02/2014 11:45 due to a program reload.

All data were rejected 10/06/2014 11:15 and 15:45 due to a program reloads to change BP multipliers.

ATemp/RH sensor:

5/1/2014 10:15 ATemp/RH data were rejected due to sensor swap to a recently calibrated sensor.

Wind sensor:

On the following dates and times maximum wind speed and associated wind speed data were rejected due to potentially intermittent obstructions of the wind sensor or a program error resulting in a maximum wind speed reading of 447.0:

02/21/2014 13:15

02/23/2014 06:45

02/23/2014 07:00

03/19/2014 10:45

03/28/2014 08:15

04/05/2014 07:00

04/15/2014 16:30

05/02/2014 07:00

05/03/2014 12:15

05/14/2014 06:15

05/18/2014 07:15

05/25/2014 07:30

06/01/2014 15:45

06/04/2014 12:45

06/12/2014 04:45

06/14/2014 10:45

08/06/2014 18:30

08/09/2014 17:00

08/12/2014 06:45

08/14/2014 07:00

08/16/2014 10:00

08/16/2014 11:15

08/21/2014 06:00

08/21/2014 06:45

09/01/2014 15:15

09/01/2014 15:30

09/06/2014 18:30

09/08/2014 17:00

09/12/2014 09:15

09/12/2014 09:30

09/12/2014 11:30

09/12/2014 11:45

09/12/2014 12:00

09/12/2014 15:00

09/13/2014 15:15

09/14/2014 11:30

09/14/2014 11:45

09/15/2014 07:00

09/18/2014 18:30

09/18/2014 18:45

09/19/2014 14:00

09/22/2014 10:45

09/22/2014 14:30

09/22/2014 14:45

09/23/2014 08:45

09/24/2014 07:45

09/24/2014 08:45

09/28/2014 07:45

10/14/2014 12:30

10/26/2014 07:30

11/01/2014 17:45

11/03/2014 20:30

11/03/2014 20:45

11/04/2014 06:00

11/05/2014 16:00

11/09/2014 12:45

11/09/2014 13:00

11/09/2014 13:15

11/19/2014 08:00

11/26/2014 09:45

11/26/2014 10:00

11/27/2014 12:30

12/09/2014 10:30

Barometric sensor:

5/1/2014 10:15 BP data were rejected due to a drop in value during the ATemp/RH sensor swap.

At 08/09/2014 06:30 the cs105 sensor malfunctioned and data were rejected until 09/02/2014 09:15.  From 09/02/2014 12:00 to 10/29/2014 15:30 the previously installed (5/9/2012 - 8/9/2014) cs106 sensor (serial #H1230003) replaced the malfunctioning cs105 sensor.  The sensor was within calibration since its previously installation and the data are not considered suspect; however, users should be aware that this sensor was removed and then reinstalled.

10/6/2014 11:30 - 15:30 BP data are rejected due to an incorrect multiplier being used.  The multiplier was corrected with the 15:45 program upload.

10/29/201415:45-16:00 data were rejected due to maintenance.  CS106 (# H1230003) was replaced by a new cs106 (serial number K2330005) at 15:49

PAR:

There were noticeable changes in PAR values following the swap to a freshly calibrated sensor (assumed to be accurate) on 01/07/2014 at 16:15.  Apogee reported a -25.4%  (-8.1%/year) post cal drift for the sensor that was installed from  01/03/2012  to 01/07/2014 (Q42745).  Acceptable drift is +/- 2% for this sensor.  All PAR data 1 year prior the sensor swap, from 01/07/2013 to 01/07/2014* are flagged and coded as <1> SSD CSM.  PAR data for the remainder (*see note below) of this deployment are flagged and coded <0> CSM and users should note that drift for that period may have exceeded acceptable limits as well.  If users are comfortable assuming that drift was linear (in a real world environment it is unlikely to be entirely linear), these data may be ‘corrected’ for assumed linear drift at the user’s discretion using manufacturer’s instructions.

*Flagging and coding for these dates will be done as reviews are completed for those years.

Throughout the year there were times when nighttime PAR values were recorded.  Those data are considered suspect and flagged and coded as <1> CSM if they were below the maximum signal noise of 2.214 mmoles/m2.  Values greater than the maximum signal noise were rejected and flagged and coded as <-3> CSM.  From 1/1/2014 - 1/7/2014 any elevated nighttime values are flagged and coded as <1> SSD CSM due to the sensor drift mentioned above. Elevated nighttime readings are flagged and coded as <5> GIT CSM along with other telemetered data when they occurred during the dates that telemetry data was used 08/05/2014 11:30 to 09/02/2014 09:15.  Reasons for the elevated nighttime values are unknown but could be attributed to ambient light sources; however, some Reserves are experiencing elevated nighttime values due to moisture intrusion into their PAR sensors.  For flagging and coding purposes sunrise and sunset times were determined using Tides and Currents program for the Big Marco Pass station http://cms.nobeltec.com/cms/Products/NavigationData/TidesCurrents.aspx.

Storm events: No major storms this year

Acknowledgement: The data included with this document were collected by the staff of the Florida Department of Environmental Protection at the Rookery Bay National Estuarine Research Reserve with funding through NOAA’s Estuarine Research Division.  Any products derived from these data should clearly acknowledge this source (please use the attached logos).  This recognition is important for ensuring that this long-term monitoring program continues to receive the necessary political and financial support.