Sapelo Island (SAP) NERR Nutrient Metadata

January 2007 - December 2007

Latest Update: November 15, 2011

# I. Data Set and Research Descriptors

- Principal investigator(s) and contact persons

  - Reserve Contact

Dorset Hurley

P.O. Box 15

Sapelo Island, GA 31327

###### Phone: 912-485-2251

e-mail: dhurley@darientel.net

  - Laboratory Contact

Katy Austin Smith

715 Bay Street

##### Marine Extension Service Laboratory

University of Georgia

Brunswick, GA 31520

Phone: 912-262-3338

e-mail: klaustin@uga.edu

  - Other Contacts and Programs

none

- Research objectives

The nutrient monitoring program is designed upon spatial deployment across a wide variety of marsh types with differing fresh and marine water mixing. These differing dynamics allow scientists and researchers to select from both a wide variety of research sites as well as tailor research programs to specific tidal dynamics and utilize the Reserves SWMP data acquisitions to the maximum extent. Additionally, from a long-term trend perspective the variety of marsh types and hydrology being monitored will allow for a better understanding of the different effects of sea-level rise upon marsh type. Due to a lack of residential development and very low human activity within the watersheds of the sites, they serve as a proxy for reference conditions with  the various marsh and associated  hydrology types for the creeks and river stations. All of the sites selected have very little anthropogenicly driven nutrient influences. The following brief descriptions are associated with each nutrient monitoring site. For more detail please refer to the site descriptors located under section (4) of this document and/ or contact the Research Coordinator at the SAP NERR for detailed information of any/all sites.

Lower Duplin: Located at the mouth of the Duplin River with large, rapid and near-complete  hydraulic exchange with Doboy Sound within each diurnal cycle. Typical of a high salinity, well mixed estuary site.

Hunt Dock: Located on the upper Duplin with relatively high hydraulic retention requiring an estimated  6-7 diurnal events to complete a total hydraulic exchange. Rainfall  may drop salinity precipitously in the basin depending on tidal height, duration and volume of precipitation.

Cabretta Creek: Located on the eastern side of Sapelo Island with direct exchange with the Atlantic Ocean. Creek is typical of high salinity, high oceanic exchange and near complete hydraulic exchange with each diurnal event. Creek is extremely buffered from rainfall (event driven) fluctuations in salinity.

Dean Creek: Located on the Southern end of Sapelo is the primary drainage of the inter-dune

(located amid primary and secondary dune systems) meadow. This site is highly susceptible to very high salinity fluctuations associated with rainfall events on both seasonal and short –term, event driven scales. Tidal exchange is complete at each diurnal event and exchange water genesis is the Doboy Sound.

The Duplin River is a tidal basin with no freshwater influence within its headwaters apart from surficial aquifer weeping from the perched lens of water associated with Sapelo Island.  This nutrient monitoring effort is tied into the Georgia Coastal Ecosystems, Long-Term Ecological Research (GCE-LTER) initiative and the University of Georgia Marine Extension Service water quality database whose collection and analysis of the water samples facilitates the database. This long-term data set is being developed to provide information on estuarine water mixing within the well-studied Duplin River basin in addition to providing a long-term characterization of water quality as related to nutrient loading within the Duplin River.

  - Monthly Grab The monthly grab sampling program focuses on documentation of baseline reference nutrient trends within a wide array of local marsh systems with differing hydrology.

  - Diel Sampling Program The diel sampling program focuses on short-term temporal variability over a lunar tidal cycle.

- Research methods

  - Monthly Grab Sampling Program

Monthly grab samples were taken at four stations within the Duplin River estuary from January to December 2006.   Bottom water samples were taken at the Lower Duplin (LD), Hunt Dock (HD), Cabretta Creek (CA) and Dean’s Creek (DC) stations using a Niskin style sampling bottle.  All grab samples were taken sequentially in duplicate (triplicate for the month of January) beginning near the time the last diel sample was collected by the ISCO sampler (this time corresponds to low tide at the end of the tidal cycle).  Chronological collection times for each of the four sites varied as two teams of people were conducting the actual sampling.  A group of two remained onboard the research vessel to sample at the Hunt Dock and Lower Duplin sites, while the other group drove inland to Cabretta Creek and Dean’s Creek.  At the time of sample collection, latitude, longitude, time and depth were recorded.  All grab samples were collected from the Niskin bottle into an acid-washed (10% HCl) polypropylene beaker for filtering.  Two filter towers were set up, one acid-washed tower with a 0.45 um polycarbonate filter for nutrient filtering and one clean tower with a GF/F filter for chlorophyll filtering.  A small amount of sample was used to rinse the nutrient filter tower equipped with a filter and then the filtrate was discarded.  The tower was then filled to the 250-mL mark.  The chlorophyll tower with the GF/F filter was also filled to the 250-mL mark and the two towers were connected by a small piece of tubing.  The vacuum pump was turned on to pull the 250 mL through each filter and then the vacuum was released.  The nutrient sample tower was disconnected and an acid-washed 250-mL polypropylene bottle was rinsed and filled with the filtrate.  Space was left in the sample bottle for expansion during freezing at approximately –18 degC.  If the first 250 milliliters went through the chlorophyll filter easily, the filtrate was discarded and an additional 50, 100 or 250 milliliters was filtered, depending on suspended sediment load, to concentrate the sample onto the filter.  The chlorophyll filter was then removed with tweezers and placed face up in a petri dish, wrapped in aluminum foil and labeled with the volume filtered and sample information.  The chlorophyll filter towers were rinsed between replicate grabs with distilled water and the nutrient filter tower was acid-washed and DI water rinsed between samples.  Nutrient and chlorophyll filtering between grabs took approximately 10 minutes to complete.  At the Cabretta and Dean’s Creek sites, a vacuum hand pump was used rather than a mechanical pump, which is available only on the research vessel.  The depths at these two sites were estimated as sampling took place from a bridge.  Samples were immediately placed on ice, in the dark and returned to the laboratory within six hours.  Once in the laboratory, samples were frozen and processed within the specified times (unless flagged) for nutrient and chlorophyll-a concentrations.

  - Diel Sampling Program

WWW Tide and Current Predictor for Wolf Island, South End was used to estimate low tide.  As close to an early, low, neap tide as possible was selected each month for sampling.  The ISCO sampler was deployed at the Lower Duplin (LD) site on the day previous to the grab sample date chosen for that particular month with the sample line suction placed at 1.5 feet below the surface of the water.  The ISCO sampler collected the first diel sample two hours later than low tide on the following day and continued collecting samples every two hours for the next 22 hours, representing a full tidal cycle, a total of 12 samples, and ending at low tide when grab sampling began.  The ISCO was turned off and the 12 samples were secured with caps upon arriving at the site.  The samples were filter processed either in the field after completion of grab sampling at Lower Duplin or back in the laboratory, weather depending.  The filtration process for the diel samples follows the same process as for grab samples described above.  High-density polypropylene bottles were used to collect the samples after filtration.  Polypropylene bottles and filter towers were soaked in 10% HCl in preparation for the fieldwork, then triple rinsed with distilled water.  A squeeze bottle was used to acid wash (then rinse with distilled water) beakers and filter towers in the field between filtering of each sample.

- Site location and character

The Sapelo Island National Estuarine Research Reserve is located on the Southeastern Atlantic coast of the United States in McIntosh County, Georgia.  The study area encompasses the Duplin River estuary, a tidally flushed drainage system flowing into Doboy Sound from the north and two inland creeks, Cabretta and Dean’s Creek. The Duplin River watershed occupies most of the Reserve, which also contains various forest types, sand dunes, a section of ocean beach and minor developed areas.   The Duplin River estuary covers 3,300 acres between Sapelo Island and the mainland in McIntosh County.  It drains a tidal bay and an extensive network of salt marshes about 6 miles long, into which there is little upland run-off.  Diverse estuarine wetlands provide extensive and complex habitat types for fish and wildlife. The island contains several small, interior brackish and freshwater marshes fed by surficial aquifer expression (interdune meadow of Nannygoat beach: south end) and anthropogenic upland ditches and dikes produced in the early 19th century (north end). The upland forests are composed of several diverse habitats including long leaf pine/slash pine forests, climax maritime forests, small amounts of pond cypress bays and naturally regenerated loblolly pine forests which are timbered on a 70 year selectively cut harvest rotation.  There are no current studies on pollutants in this area. Sapelo Island is typically considered a pristine environment, with minimal pollutant input.

Locations-

Lower Duplin:  Lat:  31 25’ 4” N, Long:  81 17’ 46” W

Hunt Dock:  Lat:  31 28’ 43” N, Long:  81 16’ 23” W

Cabretta Creek:  Lat:  31 26 37.3” N, Long:  81 14 23.7” W

Dean Creek:  Lat:  31 23 22.5” N, Long:  81 16 44.2” W

Water Quality site descriptions-

Salinities at all Duplin River sites vary according to localized rainfall and associated runoff.  The upper Duplin River site (Hunt Dock) experiences slightly lower salinities associated with rainfall events (2 -3ppt) as compared to the lower Duplin River site.  Average salinities range from 15 ppt to 30 ppt depending on seasonal or event rainfall.  Average tidal range of diurnal tidal cycle is approximately 2.5 meters twice daily.  Due to high turbidity, all Duplin River sites are lacking any persistent submerged aquatic vegetation and have an unconsolidated sandy/mud bottom (soft sediment) typical of southeastern near-ocean estuaries. Marsh sediments are relatively pristine and free of pollutants based on sediment analysis conducted in 1996 by C. Alexander, Skidaway Institue of Oceanography.  Watershed is dominated by oceanic tidal influences associated with Doboy Sound.  Depths are as follows: Lower Duplin (LD) ranges from 1.5 meters to 6.0 meters depending on tide, and Hunt Dock’s maximum depth is 4.27 meters.

Cabretta Creek is fed directly from waters of the Atlantic Ocean.  Cabretta experiences a maximum tidal range of approximately 4.3 meters.  Average mean low water depth at the sample site is approximately 3.25 meters.  Salinity ranges, with exception to major, long-term precipitation events, from 15-36 ppt., seasonally.  The station is located on a small (one-lane), wooden, roadway bridge spanning Cabretta Creek, located on the island’s extreme eastern side. The benthos is composed primarily of sand substrate with small, intertidal oyster reef conglomerate communities.  Adjacent to the site is extensive, intertidal, bank stabilization (armoring) in the form of woven rip-rap fencing and granite rocks. This manipulation is slowly becoming stabilized via oyster reef community colonization. The adjacent marshes are dominated by Spartina alterniflora with occasional Juncus romerianus in the nearby fringe community habitat. The creek has very little adjacent uplands due to: 1) the low elevational gradient and 2) the areas geologically recent accretion genesis (Holocene) resulting in sandy soils; of which neither condition allows for extensive floral colonization or stabilization.

The Dean Creek site is located on a small wooden bridge spanning Dean Creek, in close proximity to the adjacent Nannygoat Beach causeway. Dean Creek is a small tidal basin fed from the waters of Doboy Sound, which is located on Sapelo Island’s south end. With exception to short duration local or long duration regional precipitation events, the creek’s salinity normally ranges between 20 and 30 ppt. The benthic community consists of a sandy-mud substrate with occasional, small, intertidal oyster reef community and mean tidal amplitude of approximately 8 feet. Average mean low water depth at the sample site is approximately 1 meter, but fluctuates due to bank erosion. The small creek feeds approximately 150 acres of Spartina alterniflora dominated salt marsh, which is interspersed with small 0.5-1 acre hammocks and salt pans. Fringe community components range from Loblolly pine forests with a sub-canopy of Yaupon holly to Wax myrtle and Sable Palm.

- Coded variable definitions

LD = Lower Duplin; HD = Hunt Dock; CA = Cabretta Creek; DC = Dean Creek.

Each individual sample is given a 3 part name code in addition to other codes. The 3 part name code, “sapldnut” for example, gives the reserve name (sap = Sapelo), station name (LD = Lower Duplin, etc), and SWMP program code (nut = nutrient monitoring program).

Sampling Site codes:

sapldnut –  Sapelo Island nutrient data for Lower Duplin

saphdnut – Sapelo Island nutrient data for Hunt Dock

sapcanut – Sapelo Island nutrient data for Cabretta Creek

sapdcnut – Sapelo Island nutrient data for Dean Creek

The monitoring codes are set as “1” to indicate grab samples and “2” to indicate diel samples. Replicates are also given specific codes. Grab samples in which duplicate field samples are taken utilize a “1” for the first sample, “2” for the second sample, and “3” for the third sample. Subsequent lab splits of each field rep are labeled with an “S”.  Diel samples are always labeled with a “1” for the first lab replicate and an “S” for the second lab replicate. Only one actual sample is taken at each interval with the ISCO sampler.

- Data collection period

Diel sampling for 2007 began at 15:02:00 on January 29, 2007.  Grab sampling commenced on January 30, 2007 at 12:08:00 at the Hunt Dock site.

Diel Sampling

Site	Start 		Start 	End 		End

Date		Time	Date		Time

LD 	01/29/2007	1502	01/30/2007	1302

LD	02/26/2007	1350	02/27/2007	1150

LD	03/26/2007	1326	03/27/2007	1136

LD	04/23/2007	1150	04/24/2007	0950

LD        05/21/2007           1019      05/22/2007           0819

LD        06/25/2007           1424      06/26/2007           1224

LD        07/23/2007           1248      07/24/2007           1048

LD        08/22/2007           1304      08/23/2007           1104

LD        09/17/2007           0934      09/18/2007           0734

LD        10/22/2007           1507      10/23/2007           1307

LD        11/05/2007           1428      11/06/2007           1248

LD        12/03/2007           1309      12/04/2007           1109

Grab Sampling

Site	Start 		Start 	End 		End

Date		Time	Date		Time

CA	01/30/2007	1355	01/30/2007	1412

CA	02/27/2007	1300	02/27/2007	1311

CA	03/27/2007	1318	03/27/2007	1330

CA	04/24/2007	1212	04/24/2007	1223

CA	05/22/2007	1029	05/22/2007	1039

CA	06/26/2007	1316	06/26/2007	1329

CA         07/24/2007           1200      07/24/2007           1210

CA         08/21/2007           1027      08/21/2007           1038

CA         09/18/2007           0835      09/18/2007           0845

CA         10/23/2007           1327      10/23/2007           1336

CA         11/06/2007           1337      11/6/2007             1355

CA         12/04/2007           1220      12/04/2007           1229

LD	01/30/2007	1331	01/30/2007	1345

LD	02/27/2007	1138	02/27/2007	1145

LD	03/27/2007	1146	03/27/2007	1151

LD	04/24/2007	1048	04/24/2007	1057

LD	05/22/2007	0936	05/22/2007	0941

LD	06/26/2007	1141	06/26/2007	1146

LD     	07/24/2007	1136  	07/24/2007	1143

LD         08/21/2007           0945    	08/21/2007           0949

LD         09/18/2007           0856    	09/18/2007          	0900

LD         10/23/2007           1300     	10/23/2007         	1309

LD         11/06/2007           1311     	11/06/2007          	1319

LD	12/04/2007           1103      12/04/2007         	1112

HD	01/30/2007	1208	01/30/2007	1223

HD	02/27/2007	1104	02/27/2007	1109

HD	03/27/2007	1110	03/27/2007	1116

HD	04/24/2007	1002	04/24/2007	1014

HD	05/22/2007	0845	05/22/2007	0852

HD         06/26/2007          1103       06/26/2007          1109

HD	07/24/2007	1100	07/24/2007	1106

HD	08/21/2007	0902	08/21/2007	0910

HD	09/18/2007	0820	09/18/2007	0824

HD	10/23/2007	1227	10/23/2007	1233

HD	11/06/2007	1143	11/06/2007	1153

HD	12/04/2007	1032	12/04/2007	1039

DC         01/30/2007           1444      01/30/2007	       1459

DC         02/27/2007           1347      02/27/2007	       1355

DC         03/27/2007           1405      03/27/2007	       1414

DC         04/24/2007           1302      04/24/2007	       1312

DC         05/22/2007           1117      05/22/2007	       1126

DC         06/26/2007           1418      06/26/2007	       1426

DC         07/24/2007           1242      07/24/2007	       1250

DC         08/21/2007           1120      08/21/2007	       1128

DC        09/18/2007           0920      09/18/2007          0930

DC         10/23/2007           1419	 10/23/2007	1432

DC        11/06/2007           1432      11/06/2007          1442

DC        12/04/2007           1308      12/04/2007          1317

- Associated researchers and projects

As part of the SWMP long-term monitoring program, SAP NERR also monitors Meteorological and Water Quality data which may be correlated with this Nutrient dataset.  These data are available from the Research Coordinator or online at http://cdmo.baruch.sc.edu/.

For a complete viewing of associated projects visit the following website and search the collaborators links:

http://gce-lter.marsci.uga.edu/lter/

http://www.uga.edu/marine_advisory/

8) Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format.

II. Physical Structure Descriptors

9) Entry verification

A Lachat QuikChem 8000 FIA+ is used to analyze nutrient concentrations.  The instrument is calibrated daily for each parameter to be tested using a series of working standards.  Once the calibration run is complete and satisfactory (r >/= 0.99900 up to 1.0000), the samples are set up for analysis.  A set of mid-range check standards is used before the sample run, after approximately every 10 samples and at the end of the run to ensure the instrument is in control.  The check standards must remain within + or – 10% of their original value during the entire run.  Also, a blank sample is run and then spiked with each analyte to a known concentration, which must come out within + or – 10% as well.  Once the run is complete, the raw data is reviewed on the computer attached to the Lachat QuikChem 8000 FIA+ instrument, and the timing is checked to ensure proper integration of sample peaks.  Once this is completed, the data is exported onto a floppy disk and transferred to another computer.  Here the raw text file is converted to an Excel file and calculations are performed to obtain the appropriate units (ie. uM to mg/L and ugL to mg/L  – see below).  The data file for each month is saved and the results are copied into a comprehensive file with all results.  A data quality management (DQM) report is filed with the results.

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and automatically flags and codes values below MDL; calculates parameters chosen by the user and automatically flags for component values below MDL and negative values; allows the user to apply QAQC flags and codes to the data; graphs selected parameters for review; append files; and export the resulting data files to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

The data was entered and reviewed by Katy Austin Smith, Research Professional II and Lab Manager at the University of Georgia Marine Extension Service.

Unit conversion equations:

NO23 μM * 0.014  mg/L as N

NO2 μM * 0.014  mg/L as N

PO4 μM * 0.031  mg/L as P

NH4 μg/L as N / 1000  mg/L as N

10) Parameter Titles and Variable Names by Data Category

Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisks “*”.

Data Category	Parameter				Variable Name	Units of Measure

    - Phosphorus:	*Orthophosphate			PO4F		mg/L as P

    - Nitrogen:	*Nitrite + Nitrate, Filtered		NO23F		mg/L as N

*Nitrite, Filtered			NO2F		mg/L as N

*Nitrate, Filtered			NO3F		mg/L as N

*Ammonium, Filtered			NH4F		mg/L as N

Dissolved Inorganic Nitrogen		DIN		mg/L as N

    - Other Lab Parameters:

*Chlorophyll a				CHLA_N	g/L

    - Field Parameters:

none

Notes:

1.  Time is coded based on a 2400 hour clock and is referenced to Eastern Standard Time (EST).

2.  Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

11) Measured and Calculated Laboratory Parameters

    - Variables Measured Directly

Nitrogen species:		NO2F, NO23F, NH4F

Phosphorus species:		PO4F

Other:				CHLA

    - Computed Variables

NO3:				NO23F-NO2F

DIN:				NO23F+NH4F

- Limits of Detection

Method Detection Limits (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect, have been established by the UGA Marine Extension Service Laboratory.  The MDL is determined as 3 times the standard deviation of a minimum of 7 replicates of a low concentration sample.  Table 1 presents the current MDLs; these values are reviewed and revised periodically.

Table 1.  Method Detection Limits (MDL) for measured water quality parameters.

| Parameter | Variable | Mean Conc. | Std. Dev. | MDL | Dates in use |
| --- | --- | --- | --- | --- | --- |
|  |  | mg/L as N or P |  | mg/L as N or P |  |
| Ammonium | NH4F | 0.047 | 0.001 | 0.003 | Dec.’01 – Dec.‘07 |
| Nitrite | NO2F | 0.126 | 0.001 | 0.004 | Dec.’01 – Dec.‘07 |
| Nitrite + Nitrate | NO23F | 0.126 | 0.001 | 0.004 | Dec.’01 – Dec.‘07 |
| Orthophosphate | PO4F | 0.087 | 0.001 | 0.002 | Dec.’01 – Dec.‘07 |
| Chl-a | CHLA |  |  | 0.0 | Dec.’01 – June‘07 |
| Chl-a | CHLA | 0.6849 | 0.0053 | 0.0168 | June ’07 – Dec. ‘07 |

13) Laboratory Methods

    - Parameter: NH4F

QuikChem Method:  31-107-06-1-E

Method Reference:  U.S. EPA 1983. USEPA-600/4-79-020. Method 350.1.

Standard Methods 4500-NH3 H.

Method Descriptor:  Samples were filtered with a 0.45 m membrane filter and subjected to hypochlorite, which in the presence of phenol, catalytic amounts of nitroprusside and excess hypochlorite, yields indophenol blue, which measured at 630 nm is proportional to the original ammonia concentration.

Preservation Method: Samples filtered and stored frozen (-18 degC).

Holding Time:  2-3 days

    - Parameter: NO23F

QuikChem Method:  31-107-04-1-C

Method Reference:  U.S. EPA 1974.  Method 353.2.

Standard Methods 4500-NO3 F.

Method Descriptor: Samples were filtered with 0.45 um polycarbonate filters. Filtered sample is subjected to cadmium reduction column to reduce nitrate to nitrite.  The sample nitrite is then determined by diatizing with sulfanilamide and coupling with N-(1-napthyl)-ethylenediamine dihydrochloride to form a highly colored azo dye which is measured at 520 nm and is proportional to the original nitrate + nitrite concentration.  The NO2F concentration (below) is subtracted from this result to give NO3F.

Preservation Method:  Samples filtered and stored frozen (-18 degC).

Holding Time:  2 weeks

iii) Parameter: NO2F

QuikChem Method:  31-107-04-1-C

Method Reference:  U.S. EPA 1974.  Method 353.2.

Standard Methods 4500-NO3 F.

Method Descriptor: Samples were filtered with 0.45 um polycarbonate filters.  Nitrite in a filtered sample is measured by closing off the cadmium reduction column so that the nitrate is not converted and the sample follows through the same chemistry as with NO3F to yield the original nitrite concentration.

Preservation Method:  Samples filtered and stored frozen (-18 degC).

Holding Time:  1-2 days

iv) Parameter: NO3F

QuikChem Method:  31-107-04-1-C

Method Reference:  U.S. EPA 1974.  Method 353.2.

Standard Methods 4500-NO3 F.

Method Descriptor:  Nitrate is calculated from NO23F minus NO2F results.

Preservation Method:  Samples filtered and stored frozen (-18 degC).

Holding Time:  2 weeks

v) Parameter: DIN

Method:  DIN is calculated by adding the NH4F and NO23F results together.

vi) Parameter: PO4F

QuikChem Method:  31-115-01-3-A

Method Reference:  U.S. EPA 1978.  Method 365.1.

Standard Methods 4500-P E.

Method Descriptor:  Samples were filtered with 0.45 um polycarbonate filters. Filtered sample is subjected to ammonium molybdate and antimony potassium tartrate under acidic conditions to form a yellow complex.  This complex is reduced with ascorbic acid to form a blue complex, which absorbs light at 880 nm.  The absorbance is proportional to the concentration of orthophosphate in the sample.

Preservation Method:  Samples filtered and stored frozen (-18 degC).

Holding Time:  30 days

vii) Parameter: CHLA

APHA Standard Methods:  10200 H.

Method Reference:

Method Descriptor:  Suspended sediment and other material in a water sample is concentrated onto a 47 mm GF/F filter under low vacuum.  The sample is stored in a petri dish wrapped in aluminum foil in an airtight plastic bag kept on ice while in the field.  The samples are then kept frozen and in the dark until analysis.  The acetone extraction method is used to extract the chlorophyll over 2-24 hours and a spectrophotometer is used to obtain readings, which are calculated into a final result.

Preservation Method:  Filters are stored frozen (-18 degC).

Holding Time:  28 days

14) Field and Laboratory QAQC programs – This section describes field variability, laboratory variability, the use of inter-organizational splits, sample spikes, standards, and cross calibration exercises.

  - Precision

    - Field Variability – Field replicates are successive grab samples.  These are done in triplicate through January 2007 and duplicate post January 2007.  Samples are filtered and placed on ice before the next sample is grabbed (usually about 10 minutes between grabs).

    - Laboratory Variability – Laboratory replicates are done in duplicate.

    - Inter-organizational splits –Samples were analyzed by one lab.

  - Accuracy

    - Sample Spikes – A blank sample is spiked with each set for each analyte to obtain a 100 % recovery + or – 10 %.  One or two sample unknowns are spiked with each set for each analyte to obtain a 100 % recovery + or – 20 percent under ideal conditions.

    - Standard Reference Material Analysis –April 2006

    - Cross Calibration Exercises - None.

15) QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range*

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

*The -4 Outside Low Sensor Range flag was added to the 2007 dataset in August of 2011.  See the Other Remarks section for more details.

16)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data rejected due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Value calculated with a value that is below the MDL

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N	from the north

NNE	from the north northeast

NE	from the northeast

ENE	from the east northeast

E	from the east

ESE	from the east southeast

SE	from the southeast

SSE	from the south southeast

S	from the south

SSW	from the south southwest

SW	from the southwest

WSW	from the west southwest

W	from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17)  Other remarks/notes

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

*The 2007 dataset was updated on August of 2011 to include the -4 Outside Low Sensor Range flag.  The 2007 data published prior to that time used the -3 Rejected data flag with the SBL and SCB QAQC codes to indicate that data were below the minimum detection limit.  These flag code combinations were all replaced with the -4 SBL or SCB update as mandated by the Data Management Committee.

Samples that have been diluted and rerun are coded CDR .  This happens frequently with PO4 results as those values above the upper limit of the linear range (upper limit 2.2 uM or 0.0682 mg P/L) are diluted, rerun and the appropriate dilution factor applied to the raw data, thus yielding a final result analyzed within the linear range.

One Chlorophyll-a data point was deleted from the July ’07 Diel sample set.  The raw readings for rep 2 on this sample were marked as outliers as there must have been a slug of filter paper in the cuvette when it was read on the spectrophotometer.