San Francisco Bay (SFB) NERR Meteorological Metadata

January 1, 2008 – December 31, 2008

Latest Update: 12/18/2012

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons

Matt Ferner, Ph.D., Research Coordinator (November 2008 – Present)

Email: mferner@sfsu.edu

Rachel Endicott, Monitoring Technician (November 2008 – Present)

Email: rale@sfsu.edu

Lara Martin, Monitoring Technician (2006-Present)

Email: laram@sfsu.edu

Drew M. Talley Ph.D., Research Coordinator (Ending July 2008)

Email:  dtalley@sfsu.edu

Nicole A. Christiansen, Monitoring Technician (July 2007 – July 2008)

Email: nicsen@sfsu.edu

Address:

San Francisco Bay NERR

3150 Paradise Drive

Tiburon, CA

94920

Phone: 415-338-3724

Fax: 415-435-7120

2)  Entry verification

Deployment data (.DAT files) are uploaded from the CR 1000 datalogger to a Personal Computer (IBM compatible).  Files are exported from LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

Outliers are no longer automatically flagged during secondary QAQC. Outliers, suspect and erroneous data are identified during secondary QAQC and flagged and notated appropriately based on review and comparisons with field readings during data downloads. Persons responsible for data verification are Rachel Endicott and Nicole Christiansen

Data is collected in the following formats for the CR1000:

3)  Research objectives

As part of the System Wide Monitoring Program, a meteorological station is used to supplement the water quality data.  Overall goals of the San Francisco Bay NERR/SWMP monitoring stations are to collect continuous information to address three primary topics: 1.) Short-term variability and long-term changes in atmospheric conditions within the regions of the estuary (i.e., localized impacts of seasonal storm events, interannual differences in rainfall, magnitude and influence of El Niño – La Niña events, spatial extent of oceanic and tidal forcing, and long-term changes associated with the Pacific Decadal Oscillation). 2.) Anthropogenic disturbance caused by land use changes including population migration, urbanization and proximity of livestock and grazing areas. 3.) Integration of these data to the broader Central and Northern California Ocean Observing System (CenCOOS) provides much needed inland and shallow water data to complement the extensive, deep-water and offshore data currently available from the CenCOOS system.  Together these data provide an ecosystem-based network for understanding the environmental conditions and ecological processes of the San Francisco Bay.

4)  Research methods

The Campbell Scientific weather station samples every 5 seconds continuously throughout the year to produce 15 minute data.  Averages for air temperature, relative humidity, barometric pressure, wind speed, and wind direction are reported for each time interval.  Maximum and minimum readings and the time they were recorded are included for air temperature (available from SFB NERR) and wind speed (maximum only).  15 minute totals are recorded for rainfall and PAR (LiCor).

The meteorological station is visited monthly to download the data to a laptop computer through direct connection via a nine pin serial cable.  At the time of download the current readings of the CR1000 are displayed and compared to readings from an external device (Kestrel 4000) to cross check the accuracy. The station is also maintained during these visits; the wind sensors are checked to make sure there are no obstructions and can move freely, PAR, temperature and humidity, sensors and the tipping rain bucket are periodically cleaned.  Sensors are recalibrated on a one to two year schedule by swapping the existing sensors with ones that have been freshly calibrated, and manually calibrating the precipitation gage.   All sensors were exchanged/ recalibrated on December 5, 2007 with the exception of the PAR sensor which was changed on December 14, 2007. During 2008 the following programs were in use: SANFRAN_V4.0_120507.CR1 (01/01/2008 00:00 – 05/08/2008 09:45 AM) and SANFRAN_V5_5_041708.CR1 (05/08/2008 10:00 AM – 12/31/2008 23:45). There is no documentation for the 05/08/2008 program upload but the data showed a record column reset to zero on 05/08/2008 at 10:00 with missing data.

Campbell Scientific data telemetry equipment was installed at the Rush Ranch Station on 8/16/2006 and transmits data to the NOAA GOES satellite, NESDIS ID # 3B016066 (Where 3B016066 is the GOES ID for that particular station.).  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts)

Maximum, Minimum, and their times from 5-second data:

Air Temperature (°C) (these data are not in the dataset but are available from SFB NERR)

Wind Speed, (m/s) (maximum only)

Wind Direction Standard Deviation (degrees) from 5-second data

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm) (began collecting as a standard parameter at SFB NERR on 5/8/2008)

- Site location and character

San Francisco Bay estuary is located at the confluence of the Sacramento and San Joaquin Rivers, and has the largest watershed on the west coast.  There are four main bays in this system; Central Bay (or San Francisco Bay), South Bay, and San Pablo Bay Suisun Bay.  The San Francisco Bay National Estuarine Research Reserve (SFBNERR) is a multi-component site totaling approximately 3,700 acres.  The reserve sites include tidal salt marsh at China Camp State Park (38°0'4.49"N, 122°27'37.28"W) in San Pablo Bay, and brackish tidal marsh at Rush Ranch Open Space Preserve (38°13'21.00"N, 122°01'33.60"W) in Suisun Bay. The two component sites include open water, tidal wetland, and adjacent upland buffer habitats and are representative of the diverse plant and animal communities that occupy the salinity gradient within the San Francisco Estuary.  Development within the watersheds of the sites ranges from predominantly urban/suburban to protected land/agricultural.

The meteorological station is located within the Rush Ranch Open Space Preserve in the transition zone from the brackish tidal marsh to the upland grassland habitat.  The station is placed in a flat grass area free from obstructions overhead or nearby, marsh vegetation begins approximately 10 meters away, and the station is just above mean higher high water. The latitude and longitude for the meteorological station are 38° 12’ 01.6 N, 122° 1' 35.3 W and its elevation is approximately 3.96 meters.

The tower itself is a Campbell Scientific 20 foot (6.096 m) tower, and the sensors are located at 7.5 m (wind, temperature and humidity, and PAR), 2.5 m (barometric pressure), and 1 m (rain gauge) above the substrate.  The rain gauge is a separate unit located about 3 m from the tower.  The upland habitat is undeveloped aside from a nature center and a historical barn, both of which are almost a kilometer away from the station.  The surrounding grassland is used intermittently for agricultural grazing of cattle.

There are two water quality monitoring stations at Rush Ranch Open Space Preserve. First Mallard station is located at the intersection of Cut Off Slough and First Mallard Creek (38° 11' 41.70 N, 122° 1' 58.02 W) and is approximately 3.13 km southwest of the weather station. Second Mallard station is located at the intersection of Cut Off Slough and Second Mallard Creek (38° 11' 02.8 N, 122° 0' 48.9 W) and is approximately 4.52 km southeast of the weather station.  Cut Off Slough is a major offshoot of the Suisun Slough and connects to Montezuma Slough, which connects to the Suisun Bay.  Each of the Mallard Creeks drains different regions of tidal marshes.  First Mallard Creek empties the Northwestern portion of Rush Ranch, while Second Mallard drains the southeastern areas.  Both stations are mounted on treated wood pilings placed approximately 10 meters within the entrance of the creeks and a third of the width across the creek.  Small platforms are constructed near the top of the piling to facilitate access to the logger mount.  The dataloggers are deployed approximately half a meter above the water-benthos interface over soft silt mud.  These stations represent brackish tidal marsh in the San Francisco Bay. The First Mallard site has a salinity range of 3.2 ppt to 10.3 ppt and the Second Mallard site has a salinity range of 3.1 ppt to 9.9 ppt.  Freshwater input at the Rush Ranch sites is influenced by the larger watershed including the Sacramento River as well as localized runoff from immediately adjacent uplands. Pollutants at the Rush Ranch sites include runoff from roads and the ranchlands. Tidal range at these sites, as referenced from the 2008 NOAA tide predictions, range from a low of -0.5 to a high of approximately 2 meters. Water depth at the Rush Ranch sites ranges from approximately 1 to 4 meters.

The weather station’s location is representative of the atmospheric conditions at the water quality stations at Rush Ranch.   Rush Ranch is located about 40 kilometers inland of the other reserve site, China Camp State Park.  At China Camp there are two water quality monitoring stations, and due to the distance from and distinct differences in climate at Rush Ranch, this meteorological station is not representative of the conditions at China Camp.

6)  Data collection period

Meteorological data has been collected continually at Rush Ranch since August 2006.  Data was collected for the entire year in 2008, from January 1 at 00:00 through December 31 at 23:45.

7)  Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR weather data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in comma separated format.

8)  Associated researchers and projects

The meteorological data is part of the System Wide Monitoring Program. SFBNERR monitors a total of four water quality stations where data is collected continuously. There are two stations located at the China Camp State Park location and two at the Rush Ranch location. Nutrient sampling is also conducted at the four sites at levels adjacent to sondes. We are also currently conducting biomonitoring programs in both subtidal habitats (via trapping) and intertidal habitats (via quadrat surveys), with plans to expand these efforts over the coming year.

II.  Physical Structure Descriptors

9)  Sensor specifications:

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module

Date CR1000 installed:  August 16, 2006

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #:  HMP45C Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Date of Last calibration: Installed in July 2006, installed recalibrated probe December 5, 2007.

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #:  HMP45C Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Date of Last calibration: Installed in July 2006, installed recalibrated probe December 5, 2007.

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Date of Last calibration: Installed in July 2006, installed recalibrated probe December 5, 2007.

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Date of last calibration: Installed in July 2006, installed recalibrated probe December 5, 2007.

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Model #: R.M. Young 05103 Wind Monitor

Range:  360° mechanical, 355° electrical (5° open)

Accuracy: +/- 3 degrees

Date of last calibration: Installed in July 2006, installed recalibrated probe December 5, 2007.

Parameter: Photosynthetically Active Radiation (PAR)

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SB

Light spectrum waveband:  400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity: 0 to 100%

Sensitivity:  typically 5 µA per 1000 µmoles s-1 m-2

Multiplier:  1.27

Multiplier changed on 12/05/2007 at 9:45.

Date of Last calibration: 6/7/2007 and installed 12/5/2007

Parameter: Precipitation

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Date of Last calibration: Installed in July 2006, recalibrated December 5, 2007.

10)  Coded variable definitions

Sampling Site Code 	Station Code		Sampling Station

RR			sfbrrmet		Rush Ranch Meteorological Station

11)  QAQC flag definitions – This section details the automated primary and secondary QAQC flag definitions.

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		 Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions – This section details the secondary QAQC Code definitions used in combination with the QAQC flags above.

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SIC	Incorrect Calibration Constant, Multiplier or Offset

SNV	Negative Value

SOC	Out of Calibration

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR	Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CDF	Data Appear to Fit Conditions

CRE	Significant Rain Event

CSM	See Metadata

CVT	Possible Vandalism/Tampering

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Cumulative precipitation data are recorded from 00:00 to 23:59 with the daily total recorded at the midnight mark (00:00). The midnight CumPrcp value is actually the total from the previous day.

From 09/01/2008 to 07/26/2011 SFBNERR has experienced periods where the nighttime PAR readings have been significantly above 0.  After substantial troubleshooting, LI-COR definitively declares this problem was due to water intrusion.  This diagnosis is considered a sensor malfunction.  In accordance with CDMO/DMC  policy, all SFBNERR PAR data that show signs of water intrusion (i.e. after the dry freshly calibrated PAR sensor begins recording elevated nighttime readings until the sensor is replaced) have been either flagged as suspect or rejected, this included day and nighttime readings

On 2/19/2008 the wind meter was adjusted to true north. Prior to this time it is believed the sensor was aligned to magnetic north.  Because the exact time of the alignment was not recorded, all wind parameter data from 08:00-13:00 are considered suspect.

Due to a wiring problem, the rain gauge stopped working on an unknown date. The last known accurate recording was on May 24, 2008 at 4:30. No significant rain events occurred between May 24, 2008 4:30 and July 19, 2008 23:45 so all total and cumulative data from this period is flagged as suspect (1) with the general error code, See Metadata (GSM) and the comment code, data appear to fit conditions (CDF). All data from July 20, 2008 00:00 – December 31, 2008 23:45 were rejected given the sensor malfunction code (SSM) and the comment code, see metadata (CSM).

There was no documentation of the program upload of SANFRAN_V5_5_041708.CR1 but it is believed that this program was uploaded on 05/08/08 at 10:00 AM as the program shows a column reset to zero at this time and there is data missing at this time. The 05/08/08 10:00AM reading is marked as missing (-2) due to maintenance (GMT, CSM). The reading immediately following this upload (5/8/8 10:15 AM) was rejected (-3) and flagged as maintenance (GPR, CSM) due to the incomplete sampling event.