San Francisco Bay (SFB) NERR Meteorological Metadata

January 1, 2009 – December 31, 2009

Latest Update: 01/17/2013

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons

Matt Ferner, Ph.D., Research Coordinator (November 2008 – Present)

San Francisco Bay NERR

3152 Paradise Drive

Tiburon, CA 94920

Ph: 415-338-3724

Email: mferner@sfsu.edu

Rachel Endicott, Monitoring Technician (November 2008 – January 2010)

San Francisco Bay NERR

3152 Paradise Drive

Tiburon, CA 94920

Ph: 415-338-3719

Email: rale@sfsu.edu

Lara Martin, Monitoring Technician (2006-Present)

San Francisco Bay NERR

3152 Paradise Drive

Tiburon, CA 94920

Ph: 415-338-3719

Email: laram@sfsu.edu.

Caitlin Coleman-Hulbert, Monitoring Technician (January 2010- Present)

San Francisco Bay NERR

3152 Paradise Drive

Tiburon, CA 94920

Ph: 415-338-3719

Email: cech@sfsu.edu

2)  Entry verification

Data (.DAT files) are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data, and graphs the data for review.  It allows the user to apply QAQC flags to outliers, suspect and erroneous data identified during secondary QAQC based on review and comparisons with field readings, code the data, append files and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

Persons responsible for data verification are the Monitoring Technicians (Rachel Endicott from November 2008 to January 2010 and Caitlin Coleman-Hulbert from January 2010 to present) and the Research Coordinator (Matt Ferner from November 2008 to present).

3)  Research objectives

As part of the System-wide Monitoring Program (SWMP), a meteorological station is used to supplement the water quality data.  The goal of the San Francisco Bay NERR/SWMP monitoring stations is to collect continuous information to address three primary topics: 1.) Short-term variability and long-term changes in atmospheric conditions within the estuary (i.e., localized impacts of seasonal storm events, interannual differences in rainfall, magnitude and influence of El Niño – La Niña events, spatial extent of oceanic and tidal forcing, and long-term changes associated with the Pacific Decadal Oscillation). 2.) Anthropogenic disturbance caused by land use changes including population migration, urbanization and proximity of livestock and grazing areas. 3.) Integration of these data into the broader Central and Northern California Ocean Observing System (CeNCOOS) to provide much needed inland and shallow water data to complement the extensive, deep-water and offshore data currently available from the CeNCOOS system.  Together these data provide an ecosystem-based network for understanding the environmental conditions and ecological processes of San Francisco Bay.

4)  Research methods

The Campbell Scientific weather station samples every 5 seconds continuously throughout the year and reports a datapoint every 15 minutes.  Averages for air temperature, relative humidity, barometric pressure, wind speed, and wind direction are reported for each time interval.  Maximum and minimum readings and the time they were recorded are included for air temperature (available from SFB NERR)  and wind speed (maximum only).  Fifteen minute averages are recorded for rainfall and photosynthetic radiation (LiCor).

The meteorological station is visited monthly to download the data to a laptop computer through direct connection via a nine pin serial cable.  At the time of download the current readings of the CR1000 are displayed and compared to readings from an external device (Kestrel 4000) to cross check the accuracy. The station is also maintained during these visits; the wind sensor is checked to make sure there are no obstructions and that it can move freely, PAR, temperature, and humidity sensors and the tipping rain bucket are cleaned as needed.  Sensors are recalibrated on a one to two year schedule by swapping the existing sensors with ones that have been freshly calibrated.  The rain gauge is manually calibrated annually. All sensors were exchanged/ recalibrated on December 5, 2007 and again on September 22, 2009 with the exception of the PAR sensor which was changed on December 14, 2007 and again October 30, 2009. During 2009 the following programs were in use: SANFRAN_V5_5_041708.CR1 (05/08/2008 10:00 – 09/22/2009 13:45) then SANFRAN_V6_5_041708.CR1 (09/22/2009 14:00 – 12/31/2009 23:45).

Campbell Scientific data telemetry equipment was installed at the Rush Ranch Station on 8/16/2006 and transmits data to the NOAA GOES satellite, NESDIS ID # 3B016066 which is the GOES ID for that particular station.  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute data are collected in the following formats for the  CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts)

Maximum, Minimum, and their times from 5-second data:

Air Temperature (°C) (these data are not in the dataset but are available from SFB NERR)

Wind Speed, (m/s) (maximum only)

Wind Direction Standard Deviation (degrees) from 5-second data

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm)

- Site location and character

The San Francisco Estuary is located at the confluence of the Sacramento and San Joaquin Rivers and has the largest watershed on the west coast.  There are four main bays in this system; Central Bay (or San Francisco Bay), South Bay, San Pablo Bay, and Suisun Bay.  The San Francisco Bay National Estuarine Research Reserve (SFBNERR) is a multi-component site totaling approximately 3,700 acres.  The reserve sites include tidal salt marsh at China Camp State Park (38°0'4.49"N, 122°27'37.28"W) in San Pablo Bay and brackish tidal marsh at Rush Ranch Open Space Preserve (38°13'21.00"N, 122°01'33.60"W) in Suisun Bay. The two component sites include open water, tidal wetland, and adjacent upland buffer habitats and are representative of the diverse plant and animal communities that occupy the salinity gradient within the San Francisco Estuary.  Development within the watersheds of the sites ranges from predominantly urban/suburban to protected land/agricultural.

The meteorological station is located within the Rush Ranch Open Space Preserve in the transition zone from brackish tidal marsh to upland grassland habitat.  The station is placed in a flat grass area free from obstructions overhead.  Marsh vegetation begins approximately 10 meters away and the station is just above mean higher high water. The latitude and longitude for the meteorological station are 38° 12’ 01.6 N, 122° 1' 35.3 W and its elevation is approximately 3.96 meters.

The tower itself is a Campbell Scientific 20 foot (6.1 m) tower, and the sensors are located at 7.5 m (wind, temperature and humidity, and PAR), 2.5 m (barometric pressure), and 1 m (rain gauge) above the substrate.  The rain gauge is a separate unit located about 3 m from the tower.  The surrounding upland habitat is undeveloped aside from a nature center, guest quarters, and historical barn, all of which are almost a kilometer away from the station.  The nearby grasslands are used intermittently for agricultural cattle grazing.

There are two water quality monitoring stations at Rush Ranch Open Space Preserve. The First Mallard station is located at the intersection of Cutoff Slough and First Mallard Creek (38° 11' 41.70 N, 122° 1' 58.02 W) and is approximately 3.13 km southwest of the weather station. The Second Mallard station is located at the intersection of Cutoff Slough and Second Mallard Creek (38° 11' 02.8 N, 122° 0' 48.9 W) and is approximately 4.52 km southeast of the weather station.  Cutoff Slough is a major offshoot of Suisun Slough and connects to Montezuma Slough which connects to Suisun Bay.  Each of the Mallard Creeks drains different regions of tidal marsh.  First Mallard Creek empties the northwestern portion of Rush Ranch while Second Mallard drains the southeastern portion.  Both stations are mounted on treated wood pilings placed approximately 10 meters within the entrance of the creeks and a third of the width across the creek.  Small platforms are constructed near the top of the piling to facilitate access to the logger mounts.  The data loggers are deployed approximately half a meter above the water-benthos interface over soft silt mud.  These stations represent brackish tidal marsh in San Francisco Bay. The First Mallard site has a salinity range of 3.2 ppt to 10.3 ppt and the Second Mallard site has a salinity range of 3.1 ppt to 9.9 ppt.  Freshwater input at the Rush Ranch sites is influenced by the larger watershed including the Sacramento River as well as localized runoff from immediately adjacent uplands. Pollutants at the Rush Ranch sites include runoff from roads, livestock corrals at the headquarters and the ranchlands. The tidal range at these sites, as referenced from the 2009 NOAA tide predictions, is approximately -0.5 to 2 meters. Water depth at the Rush Ranch sites ranges from approximately 1 to 4 meters.

The weather station’s central location allows it to experience the same atmospheric conditions as the water quality stations at Rush Ranch.   Rush Ranch is located about 40 kilometers inland of the other reserve site, China Camp State Park.  At China Camp there are two water quality monitoring stations but due to the distance from and the distinct climate differences at Rush Ranch, this meteorological station is not representative of the conditions at China Camp.

6)  Data collection period

Meteorological data has been collected continually at Rush Ranch since August 2006.  Data was collected for all of 2009, January 1 00:00 through December 31 23:45.

7)  Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR weather data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in comma separated format.

8)  Associated researchers and projects

The meteorological data is part of the System-wide Monitoring Program. SFBNERR monitors a total of four water quality stations where data is collected continuously with YSI data sondes. There are two stations located at China Camp State Park and two at Rush Ranch. Nutrient sampling is also conducted at the four sites.  Monthly diurnal and grab samples are collected adjacent to the sondes.  We are also currently conducting biomonitoring programs in both subtidal habitats (via trapping) and intertidal habitats (via quadrat surveys) with plans to expand these efforts over the coming year.

II.  Physical Structure Descriptors

9)  Sensor specifications:

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM (4 MB optional upgrade) is available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module

Date  CR1000installed:  August 16, 2006

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #:  HMP45C Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Date of last calibration:

2006: s/n A5250046 was installed (calibration date uknown)

12/05/2007: s/n C3610004 was installed (calibration date 09/03/2007)

09/22/2009: s/n A5250046 was installed (calibration date 08/09/2009)

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #:  HMP45C Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Date of last calibration:

2006: s/n A5250046 was installed (calibration date uknown)

12/05/2007: s/n C3610004 was installed (calibration date 09/03/2007)

09/22/2009: s/n A5250046 was installed (calibration date 08/09/2009)

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Date of last calibration:

12/05/2007: s/n C2430021 was installed (calibration date 06/15/2007, multiplier 0.240/500)

09/22/2009: s/n A0240014 was installed (calibration date 08/26/2009, multiplier 0.184/600)

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Date of last calibration:

12/05/2007: s/n 81058 was installed (Campbell check date unknown)

09/22/2009: s/n 65477 was installed (Campbell check date 08/26/2009)

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Model #: R.M. Young 05103 Wind Monitor

Range:  360° mechanical, 355° electrical (5° open)

Accuracy: +/- 3 degrees

Date of last calibration:

12/05/2007: s/n 81058 was installed (Campbell check date unknown)

09/22/2009: s/n 65477 was installed (Campbell check date 08/26/2009)

Parameter: Photosynthetically Active Radiation (PAR)

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SB

Light spectrum waveband:  400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity: 0 to 100%

Sensitivity:  typically 5 µA per 1000 µmoles s-1 m-2

Date of last calibration:

12/14/2007: s/n Q38103 was installed (calibration 06/12/2007, multiplier 1.27)

09/22/2009: s/n Q41616 was installed (calibration 09/12/2009, multiplier 1.2997)

Parameter: Precipitation

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Date of last calibration:

12/05/2007, 10/30/2009

10)  Coded variable definitions

Sampling Site Code 	Station Code		Sampling Station

RR			sfbrrmet		Rush Ranch Meteorological Station

11)  QAQC flag definitions –

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		 Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions –

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SIC	Incorrect Calibration Constant, Multiplier or Offset

SNV	Negative Value

SOC	Out of Calibration

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR	Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CDF	Data Appear to Fit Conditions

CRE	Significant Rain Event

CSM	See Metadata

CVT	Possible Vandalism/Tampering

13)  Other remarks/notes

MISSING DATA

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

PROGRAM RELOAD & CALIBRATION

09/22/2009 A new program was added along with newly calibrated instruments (except the rain gauge) and a few data points are missing.

10/30/2009 The rain gauge was calibrated.

PAR

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 data logger and are flagged <1> (CAF). The maximum signal noise error for the LiCor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

From 09/01/2008 to 07/26/2011 SFBNERR has experienced periods where the nighttime PAR readings have been significantly above 0.  After substantial troubleshooting, LI-COR definitively declares this problem was due to water intrusion.  This diagnosis is considered a sensor malfunction.  In accordance with CDMO/DMC policy, all SFBNERR PAR data that show signs of water intrusion (i.e. after the dry freshly calibrated PAR sensor begins recording elevated nighttime readings until the sensor is replaced) have been either flagged as suspect or rejected, this includes day and night time readings.

TEMPERATURE

12/18/2009 07:30 – 12/19/2009 04:00 The temperature data are off, unknown cause, and the data rejected.

RELATIVE HUMIDITY

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

12/18/2009 07:30 – 12/19/2009 04:00 Relative humidity data are rejected because the temperature data is incorrect.

RAIN GAUGE

Cumulative precipitation data are recorded from 00:00 to 23:59 with the daily total recorded at the midnight mark (00:00). The midnight CumPrcp value is actually the total from the previous day.

01/01/2009 00:00 -  10/29/2009 09:30 The rain gauge stopped working in July 2008 as best we can tell and was not fixed until 10/29/2009 09:30.  All total and cumulative precipitation data are rejected for this period.

10/30/2009 13:00-10/31/2009 00:00  The rain gauge was calibrated from 13:00-16:15.  Cumulative precipitation data were corrected to 0.0 following the calibration.

12/03/2009 – 01/13/2010 17:00 <0> [CSM] During the January 13th weather station check the screen from the rain bucket was found on the ground.  This likely did not affect data collected during this time.

BAROMETRIC PRESSURE

09/22/2009 14:00 – 15:30 <-3> [SIC] (CSM) These data were rejected when a newly calibrated barometric pressure sensor was installed.  The old multiplier was initially left in the program.  Once this was discovered the new one was added.

WIND METER

9/22/2009  14:15-10/29/2009 09:30 Wind direction and SDWDir are flagged as suspect.  During this time period the wind sensor was not aligned to True North.  There are no notes of how far off the sensor was when it was realigned.  It was probably aligned using line of sight.

10/29/2009 09:45 – 14:00 Wind meter was adjusted to true north.  During this period all wind data are rejected.