San Francisco Bay National Estuarine Research Reserve (SFBNERR) Nutrient Metadata

January 1, 2012 to December 31, 2012

Latest Update: 4/15/2013

Note: This is a provisional metadata document; it has not been authenticated as of its download date.  Contents of this document are subject to change throughout the QAQC process and it should not be considered a final record of data documentation until that process is complete.  Contact the CDMO (cdmosupport@belle.baruch.sc.edu) or Reserve with any additional questions.

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons

Matt Ferner, Ph.D., Research Coordinator (November 2008 – Present)

San Francisco Bay NERR

3150 Paradise Drive

Tiburon, CA	94920

Ph: 415-338-3724

Email: mferner@sfsu.edu

Lara Martin, Monitoring Technician (2006-Present)

San Francisco Bay NERR

3150 Paradise Drive

Tiburon, CA	94920

Ph: 415-338-3719

Email: laram@sfsu.edu

Caitlin Coleman-Hulbert, Monitoring Tech, data management (Jan 2010- Feb 2013)

San Francisco Bay NERR

3150 Paradise Drive

Tiburon, CA 94920

Ph: 415-338-3717

Email: cech@sfsu.edu

2)  Entry verification

Data are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.  Caitlin Coleman-Hulbert is responsible for 2012 data management.

3)  Research objectives

As part of the System Wide Monitoring Program (SWMP), a meteorological station is used to supplement the collection of water quality and biological data.  Overall goals of the San Francisco Bay NERR/SWMP meteorological monitoring effort are to collect weather data to: 1.) Document short-term variability and long-term changes in atmospheric conditions at the Reserve sites (e.g., localized impacts of seasonal storm events, interannual differences in rainfall, magnitude and influence of El Niño – La Niña events, etc.). 2.) Provide insight into patterns or occurrences in meteorological conditions that have direct relevance to estuarine communities and resource-management practices. 3.) Integrate these data into the broader Central and Northern California Ocean Observing System (CeNCOOS) that links to a wide variety of real-time data on water quality, currents and weather around the region.  Together these data provide an ecosystem-based network for understanding the environmental conditions and ecological processes of the San Francisco Bay.

4)  Research methods

Campbell Scientific data telemetry equipment was installed at the Rush Ranch Station on 8/16/2006 and transmits data to the NOAA GOES satellite, NESDIS ID # 3B016066. The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The Campbell Scientific weather station samples a variety of parameters every five seconds continuously throughout the year.  High-resolution data are averaged over 15-minute intervals and recorded for air temperature (°C), relative humidity (%), barometric pressure (mb), wind speed (m/s), wind direction (degrees), and battery voltage (volts).  Maximum wind speed (m/s) from the five-second data and standard deviation of wind direction (degrees) are also recorded.  Maximum and minimum readings and their times from 5-second data for air temperature and wind speed are available directly from the Reserve through the contacts provided above.  Fifteen minute totals are recorded for rainfall (mm), photosynthetically available radiation or PAR (millimoles/m2), and cumulative precipitation (mm).

The meteorological station is visited monthly to download the data to a laptop computer through direct connection via a nine-pin serial cable.  At the time of download, the current readings of the CR1000 are displayed and compared to readings from an external device (Kestrel 4000) to cross check sampling accuracy. The station is also maintained during these visits; the wind sensor is checked to make sure there are no obstructions and can move freely, and the PAR and temperature/relative humidity sensors and the tipping rain bucket are cleaned as needed.  Sensors are calibrated on a one to two year schedule by swapping the existing sensors with ones that have been freshly calibrated, and by manually calibrating the precipitation gauge.  The following is a list of which sensors were swapped in 2012 and when they were calibrated:

| Sensor | Date installed | Calibration Date |
| --- | --- | --- |
| PAR | 07/16/2012-current | 02/08/2012 |
| BP | 09/29/2011-current | 09/14/2011 |
| Temp/RH | 10/23/2012-current | 09/20/2012 |
| Wind | 10/23/2012-current | 08/16/2012 |
| Rain |  | 10/20/2012 |

During 2012, the following programs were in use:

Oct 6 – March 31 SANFRAN_V6.3_100611

5)  Site location and character

The San Francisco Estuary is the largest estuary on the west coast.  There are four main bays; Central Bay, South Bay, San Pablo Bay and Suisun Bay.  The San Francisco Bay National Estuarine Research Reserve (SFBNERR) is a multi-component site totaling approximately 3,700 acres.  Reserve sites include tidal salt marsh at China Camp State Park (38°0'4.49"N, 122°27'37.28"W) in San Pablo Bay and brackish tidal marsh at Rush Ranch Open Space Preserve (38°13'21.00"N, 122°01'33.60"W) in Suisun Marsh. The two component sites include open water, tidal wetland, and adjacent upland buffer habitats and are well-protected remnants of the diverse plant and animal communities that once occupied the salinity gradient within the estuary.  Development within the watersheds of the sites ranges from predominantly urban/suburban to protected land/agricultural.

The meteorological station is located within the Rush Ranch Open Space Preserve in the transition zone from the brackish tidal marsh to the upland grassland habitat.  The station is placed just above mean higher high water in a flat grass area approximately 10 meters from marsh vegetation and is free from obstructions overhead or nearby. The latitude and longitude of the meteorological station are 38° 12’ 01.6 N, 122° 1' 35.3 W and its elevation is approximately 3.96 meters. The upland habitat is undeveloped aside from a nature center, historical barn, and horse corrals, all of which are almost a kilometer away from the meteorological station.  The surrounding grassland is used intermittently for agricultural grazing of cattle.

The tower itself is a Campbell Scientific 20 foot (6.096 m) metal tower. The sensors are located at 7.5 m (wind, temperature and humidity, and PAR), 2.5 m (barometric pressure), and 1 m (rain gauge) above the ground.  The rain gauge is a separate unit located about 3 m from the tower.

There are two water quality monitoring stations at Rush Ranch Open Space Preserve. The First Mallard station is located at the intersection of Cutoff Slough and First Mallard Creek (38° 11' 41.70 N, 122° 1' 58.02 W) and is approximately 3.13 km southwest of the weather station. The Second Mallard station is located at the intersection of Cutoff Slough and Second Mallard Creek (38° 11' 02.8 N, 122° 0' 48.9 W) and is approximately 4.52 km southeast of the weather station.

The weather station’s location is representative of the atmospheric conditions at the water quality stations at Rush Ranch.   Rush Ranch is located about 40 kilometers inland of the other Reserve site, China Camp State Park.  At China Camp there are two water quality monitoring stations and due to the distance from and distinct differences in climate at Rush Ranch, this meteorological station is not representative of the conditions at China Camp.

6)  Data collection period

Meteorological data has been collected routinely at Rush Ranch since August 2006.  Data was collected for all of 2012, January 1 00:00 through December 31 23:45.

7)  Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR weather data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in comma separated format.

8)  Associated researchers and projects

These meteorological data are part of the NERR SWMP. SFBNERR monitors a total of four water quality stations where data is collected continuously with YSI data sondes. There are two stations located at the China Camp State Park location and two at the Rush Ranch Open Space Preserve location. Nutrient sampling is also conducted at the four sites at water depths adjacent to the sondes. The Reserve is currently monitoring benthic invertebrates in both subtidal habitats (via trapping) and intertidal habitats (via quadrat surveys) with plans to expand these efforts over the coming years.

II.  Physical Structure Descriptors

9)  Sensor specifications

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #:  HMP45C Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Date of Last calibration:

2006: s/n A5250046 was installed (calibration date unknown)

12/05/2007: s/n C3610004 was installed (calibration date 09/03/2007)

09/22/2009: s/n A5250046 was installed (calibration date 08/09/2009)

09/23/2010: s/n F4610016 was installed (calibration date 08/28/2010)

09/29/2011: s/n F4610016 was installed (calibration date 11/16/2010)

11/29/2011: s/n F5230141 was installed (calibration date 10/17/2011)

10/23/2012: s/n D0930093 was installed (calibration date 09/20/2012)

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #:  HMP45C Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Date of Last calibration:

2006: s/n A5250046 was installed (calibration date uknown)

12/05/2007: s/n C3610004 was installed (calibration date 09/03/2007)

09/22/2009: s/n A5250046 was installed (calibration date 08/09/2009)

09/23/2010: s/n F4610016 was installed (calibration date 08/28/2010)

09/29/2011: s/n F4610016 was installed (calibration date 11/16/2010)

11/29/2011: s/n F5230141 was installed (calibration date 10/17/2011)

10/23/2012: s/n D0930093 was installed (calibration date 09/20/2012)

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105 and/or CS-106

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Date of Last calibration:

12/05/2007: s/n C2430021 was installed (calibration date 06/15/2007, multiplier 0.240/500)

09/22/2009: s/n A0240014 was installed (calibration date 08/26/2009, multiplier 0.184/600)

09/29/2011: s/n C2430021 was installed (calibration date 09/14/2011, multiplier 0.240/500)

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Date of last calibration:

12/05/2007: s/n 81058 was installed (Campbell check date unknown)

09/22/2009: s/n 65477 was installed (Campbell check date 08/26/2009)

09/29/2011: s/n 81058 was installed (Campbell check date 09/02/2011)

10/23/2012: s/n 65477 was installed (Campbell check date 08/16/2012 )

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Model #: R.M. Young 05103 Wind Monitor

Range:  360° mechanical, 355° electrical (5° open)

Accuracy: +/- 3 degrees

Date of last calibration:

12/05/2007: s/n 81058 was installed (Campbell check date unknown)

09/22/2009: s/n 65477 was installed (Campbell check date 08/26/2009)

09/29/2011: s/n 81058 was installed (Campbell check date 09/02/2011)

10/23/2012: s/n 65477 was installed (Campbell check date 08/16/2012 )

Parameter: Photosynthetically Active Radiation (PAR)

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SB

Light spectrum waveband:  400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity: 0 to 100%

Sensitivity:  typically 5 µA per 1000 µmoles s-1 m-2

Date of last calibration:

12/14/2007: s/n Q38103 was installed (calibration 06/12/2007, multiplier 1.27)

09/22/2009: s/n Q41616 was installed (calibration 09/12/2009, multiplier 1.2997)

04/02/2010: s/n Q38103 was installed (calibration 03/10/2010, multiplier 1.3245)

10/18/2010: s/n Q39861 was installed (calibration 09/08/2010, multiplier 1.4809)

07/26/2011: s/n Q30334 was installed (calibration 06/01/2011, multiplier 1.1692)

09/29/2011: s/n Q45608 was installed (calibration 06/28/2011, multiplier -1.0 for our program, -200 from the cal sheet)

07/16/2012: s/n Q47150 was installed (calibration 02/08/2012, multiplier -1.0 for our program, -200 from the cal sheet)

Parameter: Precipitation (specify if heated rain gauge)

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Date of calibration:  12/05/2007, 10/30/2009, 10/18/2010, 09/29/2011, 10/20/2012

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Date CR1000 Installed: 05/09/2011 (borrowed from Campbell Scientific)

07/26/2011 (serial # 5479)

10)  Coded variable definitions

Sampling station:		Sampling site code:	Station code:

Rush Ranch			RR			sfbrrmet

11)  QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		 Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC		Incorrect Calibration Constant, Multiplier or Offset

SIW	Incorrect Wiring

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SQR	Data rejected due to QAQC checks

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR		Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CDF	Data Appear to Fit Conditions

CML	Snow melt from previous snowfall event

CRE*	Significant Rain Event

CSM*	See Metadata

CCU	Cause Unknown

CVT*	Possible Vandalism/Tampering

CWE*	Significant weather event

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.

RAIN BUCKET

Cumulative precipitation data are recorded from 00:00 to 23:59 with the daily total recorded at the midnight mark (00:00). The midnight CumPrcp value is actually the total from the previous day.

02/22/2012 12:30 The rain bucket was tipped during the met station check.  Total and cumulative precipitation recalculated to 0.

03/26/2012 10:45 and 11:00  The rain bucket was tipped during the met station check.  Total and cumulative precipitation recalculated to 0.

05/29/2012 10:15 The rain bucket was tipped during the met station check.  Total and cumulative precipitation recalculated to 0.

08/10/2012 09:45 The rain bucket was tipped during the met station check.  Total and cumulative precipitation recalculated to 0.

10/20/2012 07:15 – 09:45 The rain gauge was being calibrated.  Total and cumulative precipitation recalculated to 0.

11/15/2012 11:30 – 12/04/2012 16:30 <-3>[SSM] (CSM) A wire had come loose and the rain gauge wasn’t working.

12/04/2012 16:45 – 23:45  The rain gauge was fixed this day so all cumulative precipitation was recalculated to 0 for the reminder of the day.

12/11/2012 11:00 – 11:45  The rain bucket was tipped during the met station check.  Total and cumulative precipitation recalculated to 0.

PAR

Small negative PAR  values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

05/29/2012 10:00 <-3> [SMT] (CSM) PAR sensor was being cleaned.

06/19/2012 14:15 – 14:45 <-3> [SSR] (CSM) The PAR sensor wasn’t installed.

07/16/2012 11:00-11:45 <-3> [SMT] (CSM) The PAR sensor was being swapped during this time.

12/11/2012 10:15-11:15 <-3> [SMT] (CSM) The PAR sensor was being worked on during this time.

WIND

4/10/2012 14:30 – 15:15 <-3> [SMT] (CSM) The wind sensor was being worked on.

10/23/2012 10:00-10:45 <-3> [SMT] (CSM) All wind rejected due to maintenance.

TEMP/RH

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

05/29/2012 10:00 <-3> [SMT] (CSM) Sensor was being cleaned.

10/23/2012 10:00 <-3> [SMT] (CSM) Swapped sensor.