San Francisco Bay (SFB) NERR Nutrient Metadata

January 1, 2007 – December 31, 2007

Latest Update: November 15, 2011

I.  Data Set and Research Descriptors

- Principal investigator(s) and contact persons

Reserve Contact

Matt Ferner, Ph.D., Research Coordinator (November 2008 – Present)

Email: mferner@sfsu.edu

Rachel Endicott, Monitoring Technician (November 2008 – Present)

Email: rale@sfsu.edu

Lara Martin, Monitoring Technician (2006-Present)

Email: laram@rtc.sfsu.edu

Drew M. Talley Ph.D., Research Coordinator (Ended July 2008)

Nicole A. Christiansen, Monitoring Technician (July 2007 – July 2008)

Address:

San Francisco Bay NERR

3150 Paradise Drive

Tiburon, CA 	94920

Phone: 415-338-3724

Fax: 415-435-7120

Laboratory contact

William Cochlan, Ph.D. San Francisco State University P.I.

Email: cochlan@sfsu.edu

Address:

Romberg Tiburon Center

3150 Paradise Drive

Tiburon, CA

94920

Phone: 415-338-3541

2)  Research objectives

Four long-term monitoring stations have been or are in the process of being established along the salinity gradient of the San Francisco Bay estuary in order to collect essential baseline information and improve our understanding of tidal dynamics and watershed inputs.   In particular, the San Francisco Bay National Estuarine Research Reserve’s (NERR) System-wide Monitoring Program (SWMP) collects information to address three primary topics:

1.) Short-term variability and long-term changes in estuarine water parameters within different regions of the estuary (i.e., localized impacts of seasonal storm events, variability in water quality parameters in association with re-establishment of tidal circulation, interannual differences in rainfall, magnitude and influence of El Niño – La Niña events, spatial extent of oceanic and tidal forcing, and long-term changes associated with the Pacific Decadal Oscillation).

2.) Anthropogenic disturbance caused by land use changes including population migration, urbanization and proximity of livestock and grazing areas.

3.) Integration of these data into the broader Central and Northern California Ocean Observing System (CenCOOS) provides much needed inland and shallow water data to complement the extensive, deep-water and offshore data currently available from the CenCOOS system.  Together these data provide an ecosystem-based network for understanding the environmental conditions and ecological processes of the San Francisco Bay.

The San Francisco Bay estuary is located at the confluence of the Sacramento and San Joaquin Rivers and is the largest watershed on the west coast. There are four main bays in this system: Central Bay (or San Francisco Bay), San Pablo Bay, South Bay and Suisun Bay. Only two of these bays, Suisun and San Pablo Bay, are within reserve boundaries. In order to best capture the salinity gradient, long-term monitoring stations were sited on either end of the reserve boundary in China Camp located in San Pablo Bay.  Similar locations are planned for Suisun Bay.  The first monitoring station in China Camp, the China Camp pier station, was established in May of 2005 and is located closest to the Central Bay and the Golden Gate Bridge.  It is mounted on the end of a historic pier in a semi-pristine area with minimal upland development and serves as a reference site for the saltwater part of the estuary, lower in the watershed. The second station in San Pablo Bay, Gallinas Creek, is located in a small creek that meets San Pablo Bay at the edge of the reserve site. Gallinas Creek station is a more heavily impacted site with upstream wastewater discharge, development lining one edge of the creek and natural vegetation lining the other. This station is outside of the reserve boundary. Data collection at this site commenced in December 2005.  Data logger recording ceased from December 2006 through the end of December 2007 due to the destruction of the data logger piling and mount.  Nutrient data collection continued during this time at a nearby dock. Two additional monitoring stations are planned for Suisun Bay, at First and Second Mallard.

YSI 6600 EDS data loggers are deployed at the each of the monitoring stations. The loggers are programmed to record water quality parameter measurements every 15 minutes for approximately 30 day periods. Parameters measured include Depth, Temperature, Salinity, Specific Conductivity, pH, Dissolved Oxygen (DO) and Turbidity.

  - Monthly Grab

Grab samples are taken to determine spatial and temporal differences in water quality in ecologically distinct regions of the SFBNERR and to supplement the YSI water quality data which is collected continuously. Grab sites are at the China Camp Pier and Gallinas Creek water quality monitoring stations mentioned above. During 2007, samples were taken at the water’s surface.

  - Diel Sampling Program

The diel sampling objective is to determine the impact of tidal water exchange on water quality. Diel collection occurs over the course of a lunar day with the use of an automated sampler. The diel sampling site is the China Camp Pier data logger location with sampling occurring approximately at data logger depth.

- Research methods

- Monthly Grab Sampling Program

Monthly grab sampling occurs approximately every 30 days at the China Camp Pier and Gallinas Creek monitoring stations adjacent to the water quality data loggers.  We avoid significant precipitation events. Grab samples are taken over a short period of time, within five minutes.

It is NERR protocol to take samples at low tide. At each site, replicate grab samples are taken from the surface of the water.  At China Camp Pier, a bucket rinsed with ambient water is lowered into the water to assist with sample collection from the pier. At Gallinas Creek, samples are taken directly from the water. Orthophosphate, nitrite/nitrate, silicate, ammonium, and urea samples are filtered immediately in the field into Milli-q rinsed centrifuge tubes using 0.45 µm Whatman disposable filter devices attached to rinsed syringes.  Ammonium samples are fixed and stored at -20 C until processing.  Other samples are frozen at -20 C until analysis.  Note: Beginning in August 2007, ammonium grab samples were processed the same day as collection.

Chlorophyll a samples are collected using a clean, ambient water rinsed, amber wide-mouth bottle.  Upon returning to the lab, sample water is filtered using a 0.45 µm glass fiber filter. The filter is placed in glass culture tubes for acetone extraction and analysis using fluorometric techniques.

- Diel Sampling Program

Diel sampling occurs approximately every 30 days at the China Camp Pier site (sfbdknut).  Days (72 hours) around storm events are avoided.  Collection occurs with an ISCO 6712 auto sampler over the course of a lunar day.  Sample collection commences approximately 30 min before slack low water.  The sample bottles of the ISCO are acid washed (10%) and rinsed twice with DI and once with Milli-q water prior to deployment to avoid contamination.  The ISCO collects two one liter samples 11 times (total of 22 samples) at intervals of 2.5 hours which are stored in the dark chilled chamber of the sampler.  The core of the ISCO is periodically filled with ice to prevent sample alteration during the sampling event. From January through August 2007, the first sample bottle was used only for tube rinsing purposes.  During this period, only the second bottle of sample water was used for analysis and pseudo-replicates were created from this bottle. Beginning in September 2007, the tubing was rinsed using a function at the beginning of each sampling interval. Two samples were then taken allowing for true replicates.

At the end of a sampling event, the samples are processed and stored in a timely fashion according to established protocols. Ammonium, orthophosphate, nitrite/nitrate, silicate and urea samples are filtered in the lab with 0.45 µm Whatman glass fiber filters using acid washed/Milli-q rinsed filter funnels rigged to a vacuum pump.  Ammonium samples are fixed and stored at -20 C until processing.  Other samples are frozen at -20 C until analysis.  Note: Beginning in July 2007, diel ammonium samples were processed the same day as collection.

Chlorophyll a samples (20ml sample) are filtered using a 0.45 µm glass fiber filter. The filter is placed in glass culture tubes for acetone extraction and analysis using fluorometric techniques.

- Site location and character

The San Francisco Bay National Estuarine Research Reserve (SFBNERR) is a multi-component site totaling approximately 3,700 acres. The reserve includes the following two component sites:  China Camp State Park (38°0'4.49"N, 122°27'37.28"W) and Rush Ranch Open Space Preserve (38°13'21.00"N, 122°01'33.60"W).  The two component sites include open water, tidal wetland, and adjacent upland buffer habitats and are representative of the diverse plant and animal communities that occupy the salinity gradient within the San Francisco Estuary.  Development within the watersheds of the four component sites ranges from predominantly urban/suburban to protected land/agricultural.

One of the monitoring sites at China Camp State Park is located at the end of a historic pier in an historic fishing village (38° 0' 4.49 N, 122° 27' 37.28 W). The sediment is soft silt clay bottom type and the logger at this site occasionally undergoes out of water events during extremely low tides. The second monitoring site in China Camp is located marginally outside of the state park boundary in Gallinas Creek (38° 00' 57.3 N, 122° 30' 30.6 W) and is fixed to a piling. The bottom type at this location is also soft silt/clay. This arrangement captures both ends of China Camp (albeit with one logger marginally outside of the boundary) as well as some of the salinity gradient and non-point source inputs that are likely to occur here especially because the Gallinas Creek monitoring site borders a residential neighborhood whereas there is a single residence inside the park’s boundary.  Gallinas Creek is a likely source of sediment, nutrients and freshwater, while China Camp Pier site captures more of the “San Francisco Bay” signature.  The China Camp Pier site has a salinity range of 8.8 ppt to 30.0 ppt and the Gallinas Creek site has a salinity range of 3.4 ppt to 29.9 ppt. The China Camp sites receive freshwater input from local runoff and rainwater as well as wastewater at the Gallinas Creek site. Pollutants at the China Camp sites include road runoff and at the Gallinas Creek site, impacts from the wastewater plant. Water depth at the China Camp site ranges from approximately 1 meter to 3.5 meters. At Gallinas Creek, the depth range is approximately 0.5 meter to 4 meters.

There is a meteorological station located within the Suisun Bay site, Rush Ranch Open Space Preserve, in the transition zone from the brackish tidal marsh to the upland grassland habitat.  The station is placed in a flat grass area free from obstructions overhead or nearby.  Marsh vegetation begins approximately 10 meters away and the station is just above mean higher high water. The latitude and longitude for the meteorological station are 38° 12’ 01.6 N, 122° 1' 35.3 W and its elevation is approximately 3.96 meters.

5) Coded variable definitions

Station Code	Sampling Station

sfbccnut	China Camp Pier nutrients 	
sfbgcnut	Gallinas Creek nutrients

Monthly grab sample program = 1

Diel grab sample program = 2

The first sample is designated with a 1, the second, replicate sample, with a 2

If the sample was split, the second portion was designated with an “S”

6) Data collection period

Grab sampling at Gallinas Creek and China Camp Pier commenced on January 10, 2006 and January 19, 2006 respectively.  Diel sampling began on April 18, 2006.

Grab Samples

Each set (1A & 1B) of grab samples occur within 2 minutes of each other unless otherwise noted.

| Site | Type of Sampling Event | Date | Start Time |
| --- | --- | --- | --- |
| CC | Monthly Grab (1A) | 01/16/2007 | 17:06 |
| CC | Monthly Grab (1B) | 01/16/2007 |  |
| CC | Monthly Grab (1A) | 02/06/2007 | 08:47 |
| CC | Monthly Grab (1B) | 02/06/2007 |  |
| GC | Monthly Grab (1A) | 02/06/2007 | 09:20 |
| GC | Monthly Grab (1B) | 02/06/2007 |  |
| CC | Monthly Grab (1A) | 03/06/2007 | 14:00* |
| CC | Monthly Grab (1B) | 03/06/2007 |  |
| GC | Monthly Grab (1A) | 03/06/2007 | 14:40* |
| GC | Monthly Grab (1B) | 03/06/2007 |  |
| CC | Monthly Grab (1A) | 04/12/2007 | 16:35* |
| CC | Monthly Grab (1B) | 04/12/2007 |  |
| GC | Monthly Grab (1A) | 04/12/2007 | 17:00* |
| GC | Monthly Grab (1B) | 04/12/2007 |  |
| CC | Monthly Grab (1A) | 05/07/2007 | 10:30* |
| CC | Monthly Grab (1B) | 05/07/2007 |  |
| GC | Monthly Grab (1A) | 05/07/2007 | 10:00 |
| GC | Monthly Grab (1B) | 05/07/2007 |  |
| CC | Monthly Grab (1A) | 06/12/2007 | 12:00* |
| CC | Monthly Grab (1B) | 06/12/2007 |  |
| GC | Monthly Grab (1A) | 06/12/2007 | 12:30* |
| GC | Monthly Grab (1B) | 06/12/2007 |  |
| CC | Monthly Grab (1A) | 07/10/2007 | 14:10 |
| CC | Monthly Grab (1B) | 07/10/2007 |  |
| GC | Monthly Grab (1A) | 07/10/2007 | 14:45* |
| GC | Monthly Grab (1B) | 07/10/2007 |  |
| CC | Monthly Grab (1A) | 08/01/2007 | 08:46* |
| CC | Monthly Grab (1B) | 08/01/2007 |  |
| GC | Monthly Grab (1A) | 08/01/2007 | 09:15* |
| GC | Monthly Grab (1B) | 08/01/2007 |  |
| CC | Monthly Grab (1A) | 09/25/2007 | 06:00* |
| CC | Monthly Grab (1B) | 09/25/2007 |  |
| GC | Monthly Grab (1A) | 09/25/2007 | 06:30* |
| GC | Monthly Grab (1B) | 09/25/2007 |  |
| CC | Monthly Grab (1A) | 10/10/2007 | 06:00* |
| CC | Monthly Grab (1B) | 10/10/2007 |  |
| GC | Monthly Grab (1A) | 10/10/2007 | 06:30* |
| GC | Monthly Grab (1B) | 10/10/2007 |  |
| CC | Monthly Grab (1A) | 11/13/2007 | 07:45* |
| CC | Monthly Grab (1B) | 11/13/2007 |  |
| GC | Monthly Grab (1A) | 11/13/2007 | 08:15* |
| GC | Monthly Grab (1B) | 11/13/2007 |  |
| CC | Monthly Grab (1A) | 12/12/2007 | 07:00 |
| CC | Monthly Grab (1B) | 12/12/2007 |  |
| GC | Monthly Grab (1A) | 12/12/2007 | 07:30 |
| GC | Monthly Grab (1B) | 12/12/2007 |  |

* Collected outside of the two hour window before low tide (standard time).

Diel Samples

| Site | Sampling Method | Date of Deployment | Time of Deployment | Date of Retrieval | Time of Retrieval |
| --- | --- | --- | --- | --- | --- |
| sfbccnut | Diel | 01/16/2007 | 03:30* | 01/17/2007 | 07:00 |
| sfbccnut | Diel | 02/06/2007 | 09:00* | 02/07/2007 | 12:30 |
| sfbccnut | Diel | 03/06/2007 | 07:45** | 03/07/2007 | 11:15 |
| sfbccnut | Diel | 04/04/2007 | 07:00 | 04/05/2007 | 10:30 |
| sfbccnut | Diel | 05/07/2007 | 09:00 | 05/08/2007 | 12:30 |
| sfbccnut | Diel | 06/06/2007 | 10:00* | 06/07/2007 | 13:30 |
| sfbccnut | Diel | 07/11/2007 | 03:45* | 07/12/2007 | 07:15 |
| sfbccnut | Diel | 08/01/2007 | 08:45** | 08/02/2007 | 12:15 |
| sfbccnut | Diel | 09/25/2007 | 03:45 | 09/26/2007 | 07:15 |
| sfbccnut | Diel | 10/10/2007 | 05:15* | 10/11/2007 | 08:45 |
| sfbccnut | Diel | 11/13/2007 | 07:30* | 11/14/2007 | 11:00 |
| sfbccnut | Diel | 12/12/2007 | 07:15* | 12/13/2007 | 10:45 |

*Sampling began at low tide.

**Sampling began after low tide.

7) Associated researchers and projects

Nutrient sampling is part of a suite of monitoring programs that make up the System- wide Monitoring Program of the NERR system. SFBNERR monitors meteorological conditions continuously at Rush Ranch and has two water quality stations in China Camp State Park where data is collected continuously. Two additional water monitoring stations are planned at Rush Ranch.  Biomonitoring programs are conducted in both subtidal habitats (via trapping) and intertidal habitats (via quadrat surveys).

8) Distribution

NOAA Ocean and Coastal Resource Management Data Ownership and Dissemination Policy for the NERRS National Monitoring Program:

Data collected in conjunction with the National Estuarine Research Reserve System's (NERRS) National Monitoring Program is considered public information. The principle investigators (PIs) responsible for collecting data at each NERRS site must process the data using Estuarine Reserve Division (ERD) approved quality assurance and quality control (QA/QC) protocols. The schedule reflected below assumes an adequate and uninterrupted funding for the monitoring effort at each site, therefore the data processing and the transmission timetable may be adjusted accordingly.

Unpublished intellectual property owned and developed by the recipient with federal assistance funds, i.e., processed data, will not be officially disseminated to the public for a period of one (1) year from the date of data processing. After one (1) year the QA/QC processed data will be made officially available to the general public on the Internet by the CDMO.  However, if the data are requested pursuant to the Freedom of Information Act, the Government is required to release any data which are transferred to the Government by the recipient under the award.

An awardee may own a copyright on the publication of the processed data developed or bought under a Federal assistance award. The Federal awarding agency reserves a royalty-free, nonexclusive, and irrevocable license to reproduce, publish, or otherwise use, and to authorize others to use, for Federal government purposes, the copyright in any work developed under an award, or any rights of copyright purchased by an awardee with Federal assistance support (15 C.F.R. 24.34). Any such publication will include a notice identifying the award and recognizing the license rights of the government under this clause. This paragraph will have no force and effect for the processed data not published by the recipient.

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format.

II. Physical Structure Descriptors

9) Entry verification

Chlorophyll a analysis is run directly by NERR staff.  Data is entered into an Excel spreadsheet. Data was entered and verified by the technician.  Data is collected in the required units (ug/L) and is directly pasted into the final CDMO spreadsheet.

An onsite laboratory (Cochlan lab) processed all nutrients except chlorophyll. This data was sent in an Excel spreadsheet to the NERR.  Data was calculated and reported in µM. For purposes of consistency in the NERR System, SFBNERR calculates the concentrations as mg/L based on atomic weights of 14.01, 30.97, 28.09, and 12.01 for N, P, Si and C respectively. SFBNERR used an Excel spreadsheet (Unit Conversion spreadsheet) pre-populated with formulas to multiply the concentrations (in µM) by 0.01401, 0.03097, 0.02809, and 0.01201 to yield concentrations in mg/L as N, P, Si, and C respectively. Data from the Unit Conversion spreadsheet was pasted into the final CDMO spreadsheet and significant digits were rounded using the CDMO macro.

After data are pasted into the final CDMO spreadsheet, they are processed using the CDMO Nutrient Excel macro. The NutrientQAQC macro sets up the data, metadata, and method detection limit (MDL) worksheets, adds chosen parameters and facilitates data entry, allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules, allows the user to input MDL values and automatically flags/codes values below MDL, calculates parameters chosen by the user and automatically flags/codes for component values below MDL, negative calculated values, and missing data, allows the user to apply QAQC flags and codes to the data, and graphs selected parameters for review. The Macro then allows the user to create charts to look for outliers and cross check the data, insert comment and flag codes which are synchronized into metadata sheets, and exports the data to a comma delimitated (.csv) format to upload to the CDMO. The resulting data file is sent to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  Multiple Reserve personnel were responsible for these tasks.

10) Parameter titles and variable names by category

Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisks “*”.

Data Category	Parameter				Variable Name	Units of Measure

Phosphorus and Nitrogen:

*Orthophosphate				PO4F		mg/L as P

*Ammonium, Filtered				NH4F		mg/L as N

*Nitrite, Filtered				NO2F		mg/L as N

*Nitrate, Filtered				NO3F		mg/L as N

*Nitrite + Nitrate, Filtered			NO23F	mg/L as N

Urea						UREA_F	mg/L as N

Plant Pigments:

*Chlorophyll a				CHLA_N	µg/L

Other Lab Parameters:

Silicate, Filtered				SiO4F		mg/L as SI

Notes:

1.  Time is coded based on a 2400 clock and is referenced to Standard Time.

2.  Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

11) Measured or calculated laboratory parameters

- Parameters measured directly

Nitrogen species:		NH4F, NO2F, NO23F

Phosphorus species:		PO4F

Other:			CHLA_N, SiO4F, UREA_F

- Calculated parameters

NO3F			NO23-NO2

12) Limits of detection

1. January – December 2007, all parameters except Chlorophyll a

Cochlan Lab

Method Detection Limits (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect.  The MDL is determined as 3 times the standard deviation of a minimum of 7 replicates of a single low concentration sample.  Table 1 presents the current MDLs.  These values are reviewed and revised periodically.

Table 1.

| Parameter | Start Date | End Date | MDL µM/L | MDL mg/L |
| --- | --- | --- | --- | --- |
| NO23F | 01/01/07 | 12/31/07 | 0.02 | 0.0003 |
| NO2F | 01/01/07 | 12/31/07 | 0.01 | 0.0001 |
| NH4 | 01/01/07 | 07/10/07 | 0.05 | 0.0007 |
| NH4 | 07/11/07 | 12/31/07 | 0.01 | 0.0001 |
| Urea | 1/01/07 | 12/31/07 | .04 | .0005 |
| PO4F | 01/01/07 | 12/31/07 | 0.02 | 0.0006 |
| SiO2 | 01/01/07 | 12/31/07 | 0.13 | 0.0036 |

2. January – December 2007, Chlorophyll a

SFBNERR Lab

Method Detection Limits (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect.  The MDL is determined as 3 times the standard deviation of a minimum of 7 replicates of a single low concentration sample. Table 3 presents the current MDLs.  These values are reviewed and revised periodically.

Table 2.

| Parameter | Start Date | End Date | MDL µM/L |
| --- | --- | --- | --- |
| Chla a | 01/01/07 | 12/31/07 | 0.01 |

13) Laboratory methods

1. January – June 2007, all parameters except Chlorophyll a

Cochlan Lab

i) Parameter: NO3 and NO2

SFSU – Cochlan’s analysis lab Romberg Tiburon Center

Method Reference:  Grasshoff, K. Methods of Seawater Analysis, Verlag Chemie, Second Edition, 1976.

Method Descriptor: Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis. A filtered sample is passed through a column containing granulated copper-cadmium to reduce nitrate to nitrite. The nitrite (that was originally present plus reduced nitrate) is determined by diazotizing nitrite with sulfanilamide and coupling with N-(1-napthyl)-ethylenediamine dihydrochloride to form a highly colored azo dye which is measured colorimetrically. Separate, rather than nitrate + nitrite, values are obtained by carrying out the procedure first with, and then without, the Cadmium reduction step.

Preservation Method:  Samples can be maintained at -20C and held for up to two weeks.

ii) Parameter: NH4

SFSU – Cochlan’s analysis lab Romberg Tiburon Center

January – July 2007, Grab Samples & January – June, Diel Samples

Method Reference:  Solórzano, L. 1969. Determination of ammonia in natural waters by the phenol-hypochlorite method. Limnol. Oceanogr. 14, 799-801.

Degobbis, D. 1973. On the storage of sea water samples for ammonia determination. Limnol. Oceanogr. 18:146-150.

Method Descriptor:  Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis. Phenol solution, sodium nitroprusside solution, and oxidizing reagent are added to the sample in successive additions. The color is allowed to develop and the absorbance recorded in a spectrophotometer.

Preservation Method: If storage of sample is required, add 1mL of phenol reagent to 25mL sample as quickly as possible after collection to stabilize ammonium.  Refrigerate up to 2 weeks, freeze at -15 C for up to 2 weeks, or if employing flow injection analysis acidification to < pH2, storage at 4 C for 28 days is also allowable.

August – December 2007 Grab Samples and July – December 2007 Diel Samples

Method Reference:  Holmes, R.M., A. Aminot, R. Kerouel, B.A. Hooker and B.J. Peterson.  1999.  A simple and precise method for measuring ammonium in marine and freshwater ecosystems.  Can. J. Fish. Aquat. Sci.  56: 1801-1808.

Method Descriptor:  Sample is filtered with a 0.45 um glass fiber filter immediately after collection.  Ammonium is analyzed using a modification of the Holmes et al. (1999) fluorometric technique. A working reagent is added to the sample and a T=0 reading is taken to account for background fluorescence.  Samples sit for 4 hours in the dark and another reading is taken.  Matrix effects are assumed to be minimal and a Nanopure standard curve, created with each new reagent, is used to determine ammonium concentration.

Preservation Method: Samples are kept chilled and processed the same day.

iii) Parameter: Urea

SFSU – Cochlan’s analysis lab Romberg Tiburon Center

Method Reference:  Price, N.M. and P.J. Harrison. 1987. A comparison of methods for the measurement of dissolved urea concentration in seawater. Mar. Biol. 92, 307-319.

Method Descriptor: Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis. Samples were thawed at room temperature before manual analysis using the diacetyl monoxime thiosemicarbizide technique (Price and Harrison, 1987), modified to account for a longer (72 h) and lower digestion temperature (22˚C).

Preservation Method: Samples are kept frozen at -20° C. Although others have found that freezing may decrease urea concentrations (Mulvenna and Savidge, 1992), tests conducted by Cochlan and Bronk (2001) with seawater standards of known concentrations demonstrated no such losses, in agreement with Price et al., (1987).

iv) Parameter: SiO4

SFSU – Cochlan’s analysis lab Romberg Tiburon Center

Method Reference:  Parsons, T.R., Y. Maita, and C.M. Lalli. “A Manual of Chemical and Biological Methods for Seawater Analysis,” 1.7, Determination of Silicate, 25-27.

Grasshoff, K. “Methods of Seawater Analysis.” Verlag Chemie, Second Edition, 1976.

Lachat Instruments Method 31-114-27-1-C.  Written by P. Smith and K. Bogren on 8 January 2001.  Revised by K. Bogren on 14 January 2002.

Method Descriptor: Samples are immediately filtered with a 0.45 m membrane filter upon return to the laboratory from the field and stored at –20 C until analysis. Soluble silica species react with molybdate at 37°C and a pH of 1.2 to form a yellow silicamolybdate complex. This complex is subsequently reduced with stannous chloride to form a heteropoly blue complex which has an absorbance maximum at 820 nm. The absorbance is proportional to the concentration of “molybdate reactive” silica.

Preservation Method: Samples may be frozen at -20C but some loss of silicate may occur at high concentrations (e.g. deep samples). Ideally, analysis should commence within 24 hours of sample collection.

v) Parameter: PO4

SFSU – Cochlan’s analysis lab Romberg Tiburon Center

Method Reference:  Murphy J. and Riley, J.P., A Modified Single Solution Method for the Determination of Phosphate in Natural Waters, Anal. Chim. Acta, Vol. 27, 1962, p. 31-36.

Method Descriptor:  Glass bottles should be used. Ammonium molybdate and antimony potassium tartrate reacts in an acid medium with phosphate to form an antimony-phospho-molybdate complex. This complex is reduced to an intensely blue-colored complex by ascorbic acid. The color produced is proportional to the phosphate concentration in the sample.

Preservation Method: Frozen samples may be stored for up to three months.

2. January – December 2007, Chlorophyll a

SFBNERR Lab – Romberg Tiburon Center

i) Parameter: Chlorophyll a

SFBNERR Lab - Romberg Tiburon Center

Method Reference:  Parsons, T.R, Maita, Y., Lalli, C.M., 1989. A manual of chemical and biological methods for seawater analysis. Pergamon Press. Oxford, England, pp. 173.

Method Descriptor:  Sample filtered onto 0.45 m glass fiber filter. Filter is placed in 8 ml acetone and extracted in darkness at -20 C and analyzed with a fluorometer.  Two to three drops of HCl are added to account for ingested chlorophyll.

Preservation Method: Samples are immediately analyzed, therefore no preservation methods are taken.  However, filters can be stored in the dark at -20 C for 28 days.

14)  Field and Laboratory QAQC programs

  - Precision

    - Field variability – From January – August 2007, one pseudo-replicate was collected for each diel sample. From September – December 2007, true replicates were collected for each diel sample. True replicates were collected for all grab samples from January – December 2007.

    - Laboratory variability – From January – August 2007, one pseudo-replicate was collected for each diel sample. From September – December 2007, true replicates were collected for each diel sample. True replicates were collected for all grab samples from January – December 2007.

    - Inter-organizational splits – Samples were not split and analyzed by two different labs.

  - Accuracy

    - Sample spikes – Not applicable

    - Standard Reference Material Analysis – Below are results from the NOAA/NERR Nutrient Inter-comparison (April, 2006).

SFBay NERR

Analyzed by J. Herndon, Cochlan Lab, RTC-SFSU on 4/29/06

Table 1: mg/L (N, P)

|  | Nitrate | Nitrite | Nitrate+Nitrite | Orthophosphate |
| --- | --- | --- | --- | --- |
| Lab Results | 0.212 | 0.031 | 0.242 | 0.0297 |
| Actual concentrations |  | 0.03 | 0.233 | 0.034 |

    - Cross calibration exercises – Dr. Cochlan’s lab participates in cross calibration exercises with other NERR analytical laboratories.

15) QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range*

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

*The -4 Outside Low Sensor Range flag was added to the 2007 dataset in August of 2011.  See the Other Remarks section for more details.

16)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data suspect due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Value calculated with a value that is below the MDL

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N	from the north

NNE	from the north northeast

NE	from the northeast

ENE	from the east northeast

E	from the east

ESE	from the east southeast

SE	from the southeast

SSE	from the south southeast

S	from the south

SSW	from the south southwest

SW	from the southwest

WSW	from the west southwest

W	from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

- Other remarks/notes

Metadata was written up by the technician hired at the end of 2008 (Endicott) and reviewed by the technician hired at the end of 2006 (Martin). The 2007 data report was produced using the 2008 CDMO Macro and 2008 metadata template and every attempt was made to document events as they occurred in 2007.

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

*The 2007 dataset was updated on August of 2011 to include the -4 Outside Low Sensor Range flag.  The 2007 data published prior to that time used the -3 Rejected data flag with the SBL and SCB QAQC codes to indicate that data were below the minimum detection limit.  These flag code combinations were all replaced with the -4 SBL or SCB update as mandated by the Data Management Committee.

A significant rain event that may have impacted nutrient sampling occurred on the same day (02/07/07) as diel sampling at China Camp Pier station (CC). This data has either the comment code (CRE), significant rain event, or (CSM), see metadata.

The Gallinas Creek piling was destroyed in a boating accident in December 2006. Grab sample collection at this site did not take place in January 2007 but resumed in February 2007, at a nearby dock (within 2 meters). This data has the comment code (CSM), see metadata.

From January – August 2007, pseudo-replicates were used for Diel sampling. Pseudo-replicates were taken from the same container as the initial sample. Beginning in September 2007, true replicates were taken for all Diel sampling periods. The first sample for the 09/25/2007 Diel sampling period was given the comment code (CSM), see metadata.

Records of 2007 grab samples show conflicting times of collection for January – May 2007, and original documents do not indicate times of sampling. Times listed are therefore estimates from available documentation.

During 2007 all grab samples were taken from the surface of the water.

Ammonium

Ammonium was processed using the phenol method (see section 13) from January – July 2007 for grab samples, and from January – June 2007 for diel samples. Beginning with the July 11, 2007 Diel sampling period, the fluorometric method was used (see section 13). The first ammonium record for the July 11th Diel sampling period and all of the grab samples for the August sampling period have the comment code (CSM), see metadata.

The two grab samples from the China Camp 07/10/09 collection period were combined for one sample reading so only data for one grab sample is available. The first grab sample has the comment code (CSM), see metatdata and the second sample has the general error code [GDM], data missing or not collected, and the comment code (CSM), see metatdata.

Chlorophyll a

The 08/01/2007 grab samples from China Camp Pier and Gallinas Creek resulted in abnormal readings for chlorophyll a. The acetone used may have been contaminated and in addition, samples were read less than 24 hours after extraction. This data has been rejected <-3> and given the comment code (CSM), see metadata.

The wrong amount of acetone was added to the first pseudo-replicate sample of chlorophyll a taken at 06:15 during the Diel sampling session on 07/11/2007. This record has been flagged as suspect <1> and given the comment code (CSM), see metadata.

1 ml of extra acetone was used to dilute the Chlorophyll a sample on the following dates:

10/10/2007 15:15 Both Diel samples

10/10/2007 17:45 First Diel sample only

These data have the comment code (CSM), see metadata.

During the November 13th sampling session, the grab samples at China Camp Pier were taken close to the same time as one of the diel samples (The grab samples on were taken at 07:45 and 07:57. Diel samples were taken at 07:30). The grab samples were substantially different from the diel samples for unknown reasons and these data are flagged as suspect <1> and given the comment code (CSM), see metadata.

Silicate

Silicate samples were frozen to preserve them before analysis. Some loss of silicate may occur due to freezing, especially at high concentrations. All silicate data is flagged as suspect <1> with the comment code (CSM), see metadata.

During the following collection times an exact concentration was not available as value accuracy was compromised by a bubble. In these instances, the lab reported a range of values. The mean of the highest and lowest value was reported in the data. These data are flagged as suspect <1> and given the comment code (CSM), see metadata.

Nitrite

China Camp, 03/06/2007, 14:00, First Grab Sample

China Camp, 03/07/07, 01:15, ISCO/Diel Sample – both samples

During the following collection times an exact concentration was not available as value accuracy was compromised by a bubble. In these instances, the lab reported an estimated value. These data are flagged as suspect <1> and given the comment code (CSM), see metadata.

Orthophosphate

China Camp, 03/06/07, 15:15, ISCO/Diel Sample – second sample only

The following samples were held beyond recommended holding times. The data is either flagged with the comment code (CHB), sample held beyond specified holding times, or (CSM), see metadata.

            - All nitrite samples

            - All nitrate + nitrite samples

            - All ammonium samples from January through June and all July grab samples.

            - All orthophosphate samples except for the September ISCO/Diel sampling session

            - All urea samples

            - All silicate samples

During the following collection times, analysis resulted in values over the standard curve (not diluted/sufficiently diluted). These data are flagged as suspect <1> and have the comment code (CSM), see metadata.

NO23 (and calculated NO3 values)

China Camp, 05/07/07,16:30, ISCO/Diel Sample – both samples

China Camp, 05/08/07, 00:00, ISCO/Diel Sample – second sample only

China Camp, 05/08/07, 02:30, ISCO/Diel Sample – both samples

Orthophosphate

Gallinas Creek, 06/12/2007, 12:30, Grab Sample, first sample only

Silicate

China Camp, 02/06/07, 09:00, ISCO/Diel Sample – first sample only

China Camp, 04/05/07, 03:00, ISCO/Diel Sample – both samples

China Camp, 04/05/07, 05:30, ISCO/Diel Sample – first sample only

China Camp, 04/05/07, 08:00 & 10:30, ISCO/Diel Sample – both samples

China Camp, 05/07/07, 11:30, 14:00, 16:30, 19:00, 21:30, ISCO/Diel Sample – both samples

China Camp, 05/08/07, 00:00, ISCO/Diel Sample – first sample only

China Camp, 05/08/07, 05:00, 07:30, 10:00, 12:30, ISCO/Diel Sample – both samples

Gallinas Creek, 06/12/07, 12:30 & 12:32, Grab Samples – both samples

The following grab samples were collected outside of the two hour window before low tide (standard time). The data is flagged (CLE), sample collected later/earlier than expected, or (CSM), see metadata.

| Date | Location | Sampling Start | Low Tide (approximate) |
| --- | --- | --- | --- |
| 03/06/2007 | CC | 14:00 | 19:38 |
| 03/06/2007 | GC | 14:40 | 20:02 |
| 04/12/2007 | CC | 16:35 | 14:11 |
| 04/12/2007 | GC | 17:00 | 14:41 |
| 05/07/2007 | CC | 10:30 | 09:37 |
| 06/12/2007 | CC | 12:00 | 15:14 |
| 06/12/2007 | GC | 12:30 | 15:44 |
| 07/10/2007 | GC | 14:45 | 14:26 |
| 08/01/2007 | CC | 08:45 | 07:39 |
| 08/01/2007 | GC | 09:15 | 08:09 |
| 09/25/2007 | CC | 06:00 | 04:38 |
| 09/25/2007 | GC | 06:30 | 05:08 |
| 10/10/2007 | CC | 06:00 | 05:12 |
| 10/10/2007 | GC | 06:30 | 05:42 |
| 11/13/2007 | CC | 07:45 | 07:22 |
| 11/13/2007 | GC | 08:15 | 07:52 |

The following Diel sampling collections began after the 30 min period before slack low water (standard time). The data is flagged (CLE), sample collected later/earlier than expected, or (CSM), see metadata.

| Date | Sampling Start | 30 min prior to slack low water (approximate) |
| --- | --- | --- |
| 01/16/2007 | 03:30 | 03:01 |
| 02/06/2007 | 09:00 | 08:23 |
| 03/06/2007 | 07:45 | 07:08 |
| 06/06/2007 | 10:00 | 09:31 |
| 07/11/2007 | 03:45 | 03:14 |
| 08/01/2007 | 08:45 | 07:09 |
| 10/10/2007 | 05:15 | 04:42 |
| 11/13/2007 | 07:30 | 06:52 |
| 12/12/2007 | 07:15 | 06:41 |