South Slough (SOS) NERR Meteorological Metadata

October, 2016 – December, 2016

Latest Update October 3, 2017

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons

- Adam DeMarzo, Estuarine Monitoring Technician

63466 Boat Basin Rd, Charleston, OR 97420

(541) 888-8270

adam.demarzo@state.or.us

- Alicia Helms, Estuarine Monitoring Coordinator

63466 Boat Basin Rd, Charleston, OR 97420

(541) 888-8270

alicia.r.helms@state.or.us

- Bree Yednock, Research Coordinator

63466 Boat Basin Rd, Charleston, OR 97420

(541) 888-8270

bree.yednock@dsl.state.or.us

2)  Entry verification

Data are uploaded from the CR1000 data logger to a Personal Computer.  Files are exported from or LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.  Adam DeMarzo was responsible for all data management.

3)  Research objectives

This meteorological station is located in a tidal salt marsh in the Winchester arm of the South Slough Estuary.  This location is considered representative of current and future research and monitoring sites located in South Slough. Data will be used to characterize long and short term local meteorological changes, to supplement water quality monitoring, and to inform other scientific studies as needed.

4)  Research methods

Campbell Scientific data telemetry equipment was installed at the Tom’s Creek station on 10/18/16 and transmits data to the NOAA GOES satellite, NESDIS ID #3B0032E0 .  Prior to 10/18/2016 the Charleston Meteorological station was used.  It was taken out of service 05/04/2015.  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

Data are reported in Pacific Standard Time (PST) for the year.

Data collection information:

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees),  Battery Voltage (volts)

Maximum and Minimum Air Temperature (°C) and their times from 5-second data (these data are available from the Reserve)

Maximum Wind Speed (m/s) and time from 5-second data

Wind Direction Standard Deviation (degrees)

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm)

Calibration information:

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- yearly recalibration

- Rain Gauge- yearly recalibration

- Wind Speed/Direction- yearly or every 2 years (depending on the sensor)

- Barometric Pressure- every 2 years recalibration

- PAR- every 2 years recalibration

- CR1000-every 5 years (required beginning 2014, one year initial grace period)

5)  Site location and character

South Slough is characterized as a drowned river-mouth estuary.  Located 8.5 km southwest of Coos Bay, Oregon, South Slough is one of seven tidal inlets that collectively form the Coos estuary.  The South Slough National Estuarine Research Reserve is representative of the mid-Pacific portion of the Lower Columbian Bioregion, extending from Cape Mendocino, CA north to the Columbia River, OR.  The Reserve includes 4,700 acres with 600 acres of tidal marshes, mudflats and open water channels.

Within the South Slough, there are three primary sub-systems that provide much of the freshwater input to the area.  Only two sub-systems, the Sengstacken and Winchester arms, are within Reserve boundaries.  The Sengstacken arm drains a semi-pristine area with no current development.  The Winchester arm is similarly located in an area with development limited to one small farm (<10 cattle) high in the watershed.  Valino Island marks the northern boundary of the Reserve and is a marine-dominated area.

Prior to 10/14/2016 the SWMP meteorological station was located at the Charleston Meteorological Station:  [43 20’42.18’ N, 124 19’43.44 W].  On Dec 10, 2014, OIMB installed a 140 ft tall wind turbine (128.223.70.77) to power a nearby University outreach facility, the Charleston Marine Life Center.  The wind turbine is located approximately 30 meters north of the SWMP meteorological station.  Due to this structure and potential but unknown influence on the weather measurements at our station, the Reserve decided to move the station.  Data collection from this station ended 05/04/2015 13:45.

As of 10/18/2016:

The Tom’s Creek station is centered in a tidal salt marsh located approximately 200 meters away from the Winchester SWMP water quality monitoring station.  The 3.0m aluminum tower sits on a 1.5m x 1.5m  wood platform raised approximately 0.6m above the marsh surface.  The rain gage is located on a separate post approximately 1.5m above the marsh surface and 3.0m away from the tower.  There are tree covered hills immediately to the east, west and north that alter local winds and shade the morning and evening sunlight.  Distance to the nearest obstructions is approximately 61.0m.  Station coordinates:  N 43°16'44.86", W 124°19'06.01"

Sensor heights above ground:

Rain - 1.5m.

Relative Humidity/Air Temp - 2.7m.

PAR - 3.7m.

Wind -4.0m.

Barometric Pressure - 2.1m (located in CR1000 enclosure)

SWMP Station Timeline:

| Station Code | Station Name | SWMP Status | Location | Active Dates | Reason Decommissioned | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| SOSCMMET | Charleston Meteorological | P | 43° 20' 42.18 N, 124° 19' 43.44 W | 08/01/2001 - 05/04/2015 | Station location compromised | wind turbine installed near station |
| SOSTCMET | Tom’s Creek | P | 43° 16' 44.86 N, 124° 19' 06.01 W | 10/14/2016 - |  |  |

6)  Data collection period

Data was collected for all parameters at the station from 10/19/2016 15:00 pst  – 12/31/2016 23:45 pst

| Raw File Start Date and Time | Raw File End Date and Time |
| --- | --- |
| 10/19/2016 15:00 | 11/08/2016 14:45 |
| 11/08/2016 15:00 | 12/07/2016 14:30 |
| 12/07/2016 14:45 | 01/06/2017 15:45 |

7)  Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2016.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

8)  Associated researchers and projects

As part of the SWMP long-term monitoring program, SOS NERR also monitors 15-minute water quality along with monthly grab samples and diel sampling for nutrient data which may be correlated with this meteorological dataset.  These data are available at www.nerrsdata.org.

II.  Physical Structure Descriptors

9)  Sensor specifications

Parameter: Temperature

Units: Celsius

Model #:  HC2S3 Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Serial Number: 60837423

Date of Last Calibration: 10/01/2015

Dates of Sensor Use: 10/19/2016 - current as of 12/31/2016

Parameter: Relative Humidity

Units: Percent

Model #:  HC2S3 Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Serial Number:  60867423

Date of Last Calibration: 10/01/2015

Dates of Sensor Use: 10/19/2016 - current as of 12/31/2016

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Serial Number:  X3540012

Date of Last Calibration: 10/13/2015

Dates of Sensor Use: 10/19/2016 - current as of 12/31/2016

Parameter: Wind speed/Direction

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05106 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Serial Number: WM115744

Date of Last Calibration: 10/19/2015

Dates of Sensor Use: 10/19/2016 - current as of 12/31/2016

Parameter: Photosynthetically Active Radiation (PAR)

Units:  mmoles m-2 (total flux)

Sensor type:  anodized aluminum with cast acrylic diffuser

Model #SQ110 Apogee Quantum Sensor

Light spectrum waveband:  410 to 655 nm

Temperature dependence: 0.06+/-0.06% per °C

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 70°C; Humidity 0 to 100%

Cosine Response: 45° zenith angle: +/- 2%; 75° zenith angle: +/- 5%

Sensitivity:  0.2mV per µmol s-1 m-2

PAR Multiplier: 0.025 (multiplier never changes with this model)

Serial Number: 19422

Date of Last Calibration: 10/2015

Dates of Sensor Use: 10/19/2016 - current as of 12/31/2016

Parameter: Precipitation

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Serial Number: 41930-307

Date of Last Calibration: 10/15/2015

Dates of Sensor Use: 10/19/2016 - current as of 12/31/2016

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Serial Number: 5098

Date CR1000 Calibrated: 10/30/2015

Date CR1000 Installed: 10/19/2016 - current as of 12/31/2016

CR1000 Firmware Version (s):  Version(s) and dates changed: unknown

CR1000 Program Version:  SOSTCMET_V5.7_101216.CR1

10)  Coded variable definitions

Sampling station: 		Sampling site code:	Station code:

Tom’s Creek		TC			sostcmet

11)  QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument malfunction

GIT	Instrument recording error, recovered telemetry data

GMC	No instrument deployed due to maintenance/calibration

GMT 	Instrument maintenance

GPD	Power down

GPF	Power failure / Low battery

GPR	Program reload

GQR	Data rejected due to QA/QC checks

GSM	See metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC		Incorrect calibration constant, multiplier or offset

SIW	Incorrect wiring

SMT	Sensor maintenance

SNV	Negative value

SOC	Out of calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor drift

SSN	Not a number / unknown value

SSM	Sensor malfunction

SSR		Sensor removed

Comments

CAF	Acceptable calibration/accuracy error of sensor

CCU	Cause unknown

CDF	Data appear to fit conditions

CML	Snow melt from previous snowfall event

CRE*	Significant rain event

CSM*	See metadata

CVT*	Possible vandalism/tampering

CWE*	Significant weather event

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval. These values are automatically flagged and coded as <1> (CAF).

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3% and are flagged and coded as suspect, <1> (CAF).  Values greater than 103 are rejected <-3>.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

10/14/2016 15:00 -12/31/2016 23:45 Air Temperature and relative humidity data were rejected due to incorrect CR1000 programming and sensor failure.

10/28/2016 06:00 - 10/28/2016 12:30 Data are missing as a result of a power failure due to a solar power wiring mistake. All parameters 10/28/2016 12:45 were rejected since they may not be a full 15 minutes of 5-second data.