## South Slough (SOS) NERR Nutrient Metadata

## January - December 2007

Latest Update: November 15, 2011

##### I. Data Set & Research Descriptors

- Principle investigators and contact persons

  - Reserve contact

South Slough National Estuarine Research Reserve

PO Box 5417

Charleston, OR 97420

Phone: 541-888-5558

Fax: 541-888-5559

Dr. Steven Rumrill, Research Program Coordinator

Phone: 541-888-2581 ext. 302

E-mail: steve.rumrill@state.or.us

Alicia Helms, Estuarine Monitoring Coordinator

Phone: 541-888-2581 ext. 304

E-mail: alicia.r.helms@state.or.us

Adam DeMarzo, Estuarine Monitoring Assistant

Phone: 541-888-2581 ext. 307

E-mail: adam.demarzo@state.or.us

  - Laboratory contact

Kathy Krogslund, Lab Manager

Marine Chemistry Laboratory

School of Oceanography

Rm G Marine Sciences Bldg

University of Washington

Seattle, WA 98195-7940

Phone: 206-543-9235

E-mail: kkrog@u.washington.edu

  - Other Contacts

None

### Research objectives

The main objective is to determine whether nutrient concentrations within the estuarine water column are driven by tidal fluctuations, oceanic forcing, or watershed inputs along the estuarine gradient of the South Slough estuary. Understanding the seasonal and tidal dynamics of nitrogen and phosphorus concentrations is particularly important because high levels of these nutrients can lead to over-enrichment and problems associated with eutrophication. Since 1995, we have been monitoring abiotic water quality parameters at fixed sites within the South Slough, according to the System-Wide Monitoring Program (SWMP). Water quality data chronicles essential baseline information and improves our understanding of tidal dynamics and watershed inputs. Adding nutrients to this framework of water quality parameters provides a second tier of factors that will ultimately afford better capability to characterize how the South Slough estuarine system functions.

Previous studies have monitored nutrient concentrations in South Slough, although the sampling was not extensive throughout the estuary. Our current work complements the earlier studies, while providing new information. Previous studies include Browning (1980), who observed relatively low average concentrations of total phosphorus (0.064 mg L-1), ortho-phosphorus (0.047 mg L-1), nitrite (4.55 μg NO2), and nitrate (21.06 μg NO3) measured in surface waters at low tide. This result suggests that the South Slough estuary does not receive substantial nutrient runoff from the adjacent uplands. Fry et al., (2001) analyzed several biochemical indicators of nitrogen loading along the entire estuarine gradient of South Slough, including fully marine waters (Cape Arago) and freshwater creeks, as part of a study of four Pacific Coast National Estuarine Research Reserves (Tijuana River, CA; Elkhorn Slough, CA; South Slough, OR; Padilla Bay, WA). Nutrient concentrations in water samples were determined for NH4+, NO3-, NO2-, SiO32-, and PO43-. In addition, δ15N values were determined for sediments, water column particulates, attached macroalgae, snails, and barnacles. Average ammonium and nitrate concentrations within the South Slough estuary were approximately double those at the marine sites, with the exception of samples collected near a fish-processing plant in Charleston. Low average Dissolved Inorganic Nitrogen (DIN) concentrations decreased along the estuarine gradient from 36 μM at the mouth of the estuary to 18 μM in the riverine tidal channels. These low DIN concentrations indicate that nitrogen loading is not excessive in the South Slough, and that flooding ocean waters are the principal source of nitrogen input.

Total dissolved solids (TDS) have not been measured directly within the South Slough estuary. However, Wells and Baird (1990) used correlation analysis to estimate TDS levels from measurements of conductivity. Time-series estimates suggest that tidal flooding results in increased conductivity and elevated TDS levels, while ebb tides lower water conductivity and TDS levels. Consequently, estimates of TDS varied directly with tidal currents and ranged between 4,000 and 35,000 mg L-1 during Feb-Mar of 1990 (Wells and Baird, 1990).

Estuarine phytoplankton are a major source of autotrophic primary production in the open water habitats of the South Slough. Assemblages of estuarine phytoplankton are influenced seasonally and spatially by variations in ocean forcing, nutrient availability, solar energy, and riverine inputs. The typical succession pattern in Pacific Northwest estuaries begins with low densities of phytoplankton in late fall and winter (due to reduced light and high turbidity), followed by a bloom of small diatoms in late winter/early spring. The diatom bloom usually terminates in late spring when nitrogen sources are depleted; phytoplankton densities remain low in the summer months when nutrient availability is low and grazing pressure is high (Karentz and McIntire, 1977); Pequegnat and Butler, 1982; Nybakken, 1993). Relatively high concentrations of chlorophyll measured throughout the summer suggest that nutrient availability in the marine-dominated region of the estuary may be pulsed and tightly linked to seasonal upwelling of the nearshore ocean in summer (Cowlishaw, 2001).

### Monthly grab sampling

Results from these samples track nutrient and phytopigment concentrations along the estuarine gradient to determine whether nutrient inputs are delivered by the ocean or by the freshwater streams that flow into South Slough. Since samples are collected at or near sites equipped with Yellow Springs Instrument (YSI) dataloggers and at the same level in the water column, they are reflective of the water masses sampled by the datasondes. Any fluctuations in nutrient levels in response to meteorological events and/or seasonal ocean conditions will also be evident.

  - Diel sampling

Collecting samples through a diel cycle (lunar day- 24hour:48min) highlights any fluctuations in nutrient and phytopigment loading as a function of tidal forcing. The levels of nutrients and phytopigments at different tidal stages should determine the impact of ocean versus freshwater input. Diel sampling was also conducted at Winchester Arm (WI - diel site) during the January, March, June, and August 2004 sampling periods. These double sampling events were conducted to compare nutrient levels between marine-dominated and freshwater-dominated zones within South Slough estuary.

- Research methods

Detailed protocols for research methods are contained in “SSNERR-SWMP: Standard Operating Protocols: Collection and Filtration of SWMP Nutrients and Plant Pigments.”

  - Monthly grab sampling program

Monthly grab samples are taken adjacent to three of the four established water quality SWMP stations within the South Slough estuary (Winchester, Valino, and Charleston). A fourth sampling location (Boathouse), in the Coos Bay estuary next to the Oregon Institute of Marine Biology (OIMB) boathouse, was added in March 2003. The boathouse station was established (following approval by the DMC) because SSNERR’s fourth SWMP site (Sengstacken) is not accessible at low tide. The boathouse site is equipped with a YSI600XL CTD monitored by OIMB personnel. These data are provided monthly to the SWMP Coordinator. The Boathouse site is accessed by wading to a location adjacent to the CTD where the water is over a meter deep at low tide. Charleston is accessed via a wooden dock and samples are collected within 2 meters of the YSI station. Valino is accessed by land, followed by wading to within a few meters of the YSI station. Because the Winchester station is difficult to access at low tide, samples are collected from a bridge over the slough approximately 600 meters upstream. At all sites, field calibration data including water temperature, salinity, specific conductance, pH, and dissolved oxygen are recorded with a hand-held YSI 600 QS.

Although the SWMP SOP only requires sample collection at or near slack low tide, grab samples at SS NERR are collected at low tides and high tides, as close as possible to the hour and half-hour recording times of deployed YSI water quality datasondes. Charleston grab samples are always taken at the same time the Charleston ISCO is collecting a diel sample (see below).  All grab samples are taken on the same day not more than 1.5 hours before or after slack water. Efforts are made to sample during spring tides, although this is not always feasible due to the timing of tides and schedules of researchers.

For the Charleston grabs, three consecutive samples are collected using a 2-L Van Dorn bottle held 0.5 m above the channel bottom.  Boathouse low tide and Valino samples are also collected consecutively, but midway between the water surface and channel bottom due to shallow depth. Using the Van Dorn sampler lengthens the collection time because of the distance to the water from the dock and bridge and the time it takes to fill the sample bottle. Samples from the Van Dorn bottle are decanted into amber, wide-mouth, Nalgene bottles.  All sample collection bottles were acid washed (10% HCl) and rinsed three times with deionized water. Immediately prior to sample collection at each site, sample bottles are rinsed three times with ambient water in the field. Sample bottles are immediately capped, placed on ice in the dark, and returned to the laboratory where they are stored at 4oC until filtration.

  - Diel sampling program

### At the Charleston site (see above) an ISCO 6712 autosampler is programmed to collect 1-L samples every 2.5 hours over a 25-hour tidal cycle. All bottles were acid washed (10% HCl) and rinsed three times with deionized water prior to sampling. It is not possible to rinse them with ambient water in the field. The ISCO is situated on a limited-access dock within a few meters of the Charleston YSI datalogger and is chained to a piling for security. A 25-pound weight is lowered to the channel bottom and the suction head of the ISCO is secured to the line. The suction head is lowered to the bottom and then retracted 0.5 m in order to sample the water at the same level as the YSI datalogger. A second weight is secured to the line and lowered to just below the low-tide level in order to maintain the vertical positioning of the suction head during tidal exchanges. The suction line is secured to the dock to ensure a smooth, continuous rise to the pump head. During diel sampling, drift algae is removed from the lines, if necessary, to reduce drag. The central well of the ISCO is filled with ice to insure samples are preserved as close to 4oC as possible until collection. On warm days ice is replenished two or three times to ensure the integrity of the samples. After collecting the Charleston grab samples, ice is replenished and all filled ISCO bottles are collected and returned to the lab. When diel sampling is complete, the remaining bottles are capped, stored on ice in the dark and returned to the lab where they are refrigerated at 4oC until filtration.

  - Laboratory filtration of samples

Samples are stored at 4oC in the dark until they are filtered. All samples are filtered within 24-28 hours of collection.  Immediately prior to filtration, samples are gently agitated to ensure homogenization. Samples are filtered for Chlorophyll and phaeophytin analysis using a vacuum filtration apparatus and for inorganic nutrient analysis using a syringe and Acrodisc filters.  Filters and filtrates are stored at -20oC prior to mailing.  As close as possible to the overnight shipment pickup time, frozen samples are packed in a styrofoam container, surrounded with bags of ice, and sent to the University of Washington Marine Chemistry Laboratory for analysis.

  - Researchers responsible for sample collection

| Month | Researchers |
| --- | --- |
| January | Adam DeMarzo, Ali Helms |
| February | Adam DeMarzo, Ali Helms |
| March | Adam DeMarzo, Ali Helms |
| April | Adam DeMarzo, Ali Helms |
| May | Adam DeMarzo, Ali Helms |
| June | Adam DeMarzo, Ali Helms |
| July | Adam DeMarzo, Ali Helms |
| August | Adam DeMarzo, Ali Helms |
| September | Adam DeMarzo, Ali Helms |
| October | Adam DeMarzo, Ali Helms |
| November | Adam DeMarzo, Ali Helms |
| December | Adam DeMarzo, Ali Helms |

- Site location and character

South Slough is characterized as a drowned river-mouth estuary in the Lower Columbia Biogeographic Region.  The South Slough National Estuarine Research Reserve (NERR) is representative of the mid-Pacific portion of the Bioregion, extending from Cape Mendocino, CA north to the Columbia River, OR.  South Slough NERR was formally designated in 1974 as the founding member of the NERR System.  As the southernmost of several shallow tidal marine and freshwater systems that drain into the greater Coos estuary, the South Slough watershed and estuary contain the full gradient of marine, estuarine, freshwater, and terrestrial habitats encompassed by the geomorphic boundaries of the coastal drainage basin.  At its northern (seaward) side, the mouth of the South Slough estuary merges with the Coos estuary near its confluence with the Pacific Ocean.  Conversely, at its southern (landward) side, the riverine regions of the South Slough NERR give way to freshwater ponds, forested wetlands, and coastal forests that extend up valleys and steep hillsides to the tops of watershed ridges.  Consequently, the South Slough estuary is a land-margin ecosystem that contains representative habitats and communities along the entire environmental gradient from the marine-dominated channels and tideflats, through the mesohaline estuarine mixing zone, to a mixture of freshwater and riparian habitats at the riverine and palustrine region of the drainage system.

With approval from the NERR Data Management Committee, South Slough NERR collects SWMP nutrient samples from the following four stations: (1) Winchester Arm (Hinch Road Bridge) – riverine; (2) Valino Island – mesohaline; (3) Charleston Bridge – marine dominated; (4) Coos Bay (OIMB Boathouse) – marine.  These four stations are located along the estuarine gradient of the South Slough, and the nutrient samples are collected from different hydrographic areas.  Nutrient samples are not collected from the South Slough NERR Sengstacken SWMP station because it is located in a riverine hydrographic region (the same as the Winchester SWMP station), and access to the site is not possible during the same low tide series.  It was determined that a more complete sampling of the estuarine gradient would be gained by adding a more marine site.

- Winchester Arm (WI) (Winchester Tidelands Restoration Project site): [43 16.945’ N, 124 19.216’ W]

This station serves as a management-treatment site and is located within the riverine (southern) portion of the Winchester Arm.  It is approximately 7.1 km from the mouth of the South Slough estuary in a tidal channel that is 3 meters deep.  The site is adjacent to an experimental marsh restoration area, where dikes were removed from 5.1 ha in 1996 and 1998.  About 4.2 hectares of natural marsh surround the project site.  The tidal channel is immediately adjacent to intertidal marshes in habitats characterized by soft mud and eelgrass (Zostera marina).  Tidal amplitude is 2.0 m, monthly mean Salinities are between 4-20 psu, and mean Temperatures range from 8-18oC.  Freshwater inputs are between 1-23 cfs, depending on precipitation events.  The upper watershed is fairly pristine, primarily second growth coniferous forest, with development limited to one farm (<15 cattle).

- Valino Island (VA):  [43 19.034’ N, 124 19.298’ W]

This station serves as a reference area for the restoration site and is located immediately north of Valino Island, in the southern portion of the marine-dominated region of the estuary.  The tidal channel is 4 meters deep.  There are no known point source pollutants although high bacterial levels from unknown sources cause frequent regulatory problems for commercial oyster growers.  This area receives freshwater input directly from rainfall and from three primary creeks that drain adjacent watershed sub-basins.  Additional freshwater enters this region indirectly during ebb tides from the Winchester and Sengstacken arms of the estuary.  The Valino site is approximately 3.6 km from the mouth of South Slough estuary.  Situated in the primary tidal channel, the station is adjacent to large eelgrass beds, tidal mudflats, sandflats, and commercial oyster cultivation areas.  The tidal range is about 2.7 m and monthly mean Salinities fluctuate seasonally from 5-31 psu.

- Charleston Bridge (CH):  [43 20.262’ N, 124 19.232’ W]

This station is located immediately south of the Charleston Bridge in the northern end of the marine-dominated portion of the estuary, and is a second reference area.  The site is approximately 1.2 km from the mouth of South Slough estuary and 2.4 km north of the Valino site.  In this area, the tidal channel is 4.5 meters deep.  The adjacent Joe Ney Slough has a decade-long history of tributyltin pollution.  The area was voluntarily cleaned up and passed Oregon Department of Environmental Quality site review.  In addition, occasional high bacterial levels are noted.  Freshwater input is limited to seasonal rainwater and a small transient salinity/density signal from Joe Ney Slough.  The YSI station is attached to a log dock with controlled access by land for security purposes.  Situated in the primary tidal channel, the station is adjacent to extensive eelgrass beds, tidal mudflats, sandflats, and commercial oyster cultivation areas.  The tidal amplitude is 2.7 meters, monthly mean Temperatures range from 8-15oC, and the monthly mean salinities fluctuate seasonally from 20-33 psu.

- OIMB Boathouse (BH):  [43 21.0042’ N / 124 19.7146’ W]

This station is representative of the marine waters that flood into Coos Bay/South Slough directly from the Pacific Ocean. There are no known freshwater inputs.  It is located at the entrance to Coos Bay and the mouth of the South Slough estuary, approximately 1.5 km north of the Charleston (CH) site.  The shoreline environment in the immediate vicinity of the station includes a rip-rap jetty, rocky headlands, a rocky intertidal platform and high-energy pocket beaches.  Samples are collected at 0.5 meters above the bottom, depth at the sample site varies from approximately 0.8 to 1.5 meters at low tide and from approximately 2.0 to 3.0 meters at high tide.  There are no known sources of pollution.  Since 7/23/2002, OIMB has maintained a YSI600XL CTD at this site, fastened to a vertical dock piling in the lower intertidal zone.  The average tidal amplitude is 3.0 meters, monthly mean Temperatures range from 8-15oC, and the monthly mean salinities fluctuate seasonally from 20-33 psu.  Data from the station are downloaded by OIMB Research Assistant Aleta Carte.  Limited QA/QC was performed on the data prior to its transmission to South Slough NERR.

- Code variable definitions

Station Code Names

soswinut = South Slough Winchester Arm (WI) nutrients

sosvanut = South SloughValino Island (VA) nutrients

soschnut = South Slough Charleston Bridge (CH) nutrients

sosbhnut = South Slough OIMB Boathouse (BH) nutrients

Monitoring Programs

Monthly grab sample program (1)

Diel grab sample program (2)

- Data collection period

  - Monthly grab sampling

| Site | Start Date/Time | Stop Date/Time |
| --- | --- | --- |
| sosbhnut | 01/16/2007 16:12 | 01/16/2007 16:14 |
| sosbhnut | 01/16/2007 09:02 | 01/16/2007 09:07 |
| sosbhnut | 02/12/2007 13:58 | 02/12/2007 14:00 |
| sosbhnut | 02/12/2007 06:23 | 02/12/2007 06:27 |
| sosbhnut | 03/21/2007 07:21 | 03/21/2007 07:25 |
| sosbhnut | 03/21/2007 13:38 | 03/21/2007 13:43 |
| sosbhnut | 04/17/2007 05:40 | 04/17/2007 05:42 |
| sosbhnut | 04/17/2007 11:57 | 04/17/2007 12:05 |
| sosbhnut | 05/08/2007 10:02 | 05/08/2007 10:04 |
| sosbhnut | 05/08/2007 16:35 | 05/08/2007 16:40 |
| sosbhnut | 06/20/2007 09:27 | 06/20/2007 09:29 |
| sosbhnut | 06/20/2007 16:42 | 06/20/2007 16:48 |
| sosbhnut | 07/17/2007 07:57 | 07/17/2007 07:59 |
| sosbhnut | 07/17/2007 14:18 | 07/17/2007 14:24 |
| sosbhnut | 08/28/2007 06:13 | 08/28/2007 06:15 |
| sosbhnut | 08/28/2007 12:36 | 08/28/2007 12:40 |
| sosbhnut | 09/25/2007 04:33 | 09/25/2007 04:35 |
| sosbhnut | 09/25/2007 10:45 | 09/25/2007 10:48 |
| sosbhnut | 10/29/2007 07:40 | 10/29/2007 07:48 |
| sosbhnut | 10/29/2007 13:12 | 10/29/2007 13:20 |
| sosbhnut | 11/26/2007 06:23 | 11/26/2007 06:28 |
| sosbhnut | 11/26/2007 12:25 | 11/26/2007 12:30 |
| sosbhnut | 12/19/2007 13:45 | 12/19/2007 13:53 |
| sosbhnut | 12/19/2007 07:30 | 12/19/2007 07:37 |
| soschnut | 01/16/2007 16:35 | 01/16/2007 16:40 |
| soschnut | 01/16/2007 09:28 | 01/16/2007 09:32 |
| soschnut | 02/12/2007 14:13 | 02/12/2007 14:19 |
| soschnut | 02/12/2007 06:45 | 02/12/2007 06:47 |
| soschnut | 03/21/2007 07:48 | 03/21/2007 07:50 |
| soschnut | 03/21/2007 14:03 | 03/21/2007 14:08 |
| soschnut | 04/17/2007 05:56 | 04/17/2007 06:02 |
| soschnut | 04/17/2007 12:34 | 04/17/2007 12:40 |
| soschnut | 05/08/2007 10:20 | 05/08/2007 10:30 |
| soschnut | 05/08/2007 17:00 | 05/08/2007 17:07 |
| soschnut | 06/20/2007 09:57 | 06/20/2007 10:02 |
| soschnut | 06/20/2007 17:26 | 06/20/2007 17:35 |
| soschnut | 07/17/2007 08:14 | 07/17/2007 08:18 |
| soschnut | 07/17/2007 14:42 | 07/17/2007 14:47 |
| soschnut | 08/28/2007 06:43 | 08/28/2007 06:47 |
| soschnut | 08/28/2007 13:00 | 08/28/2007 13:05 |
| soschnut | 09/25/2007 04:59 | 09/25/2007 05:04 |
| soschnut | 09/25/2007 11:10 | 09/25/2007 11:15 |
| soschnut | 10/29/2007 08:05 | 10/29/2007 08:12 |
| soschnut | 10/29/2007 13:42 | 10/29/2007 13:46 |
| soschnut | 11/26/2007 06:42 | 11/26/2007 06:47 |
| soschnut | 11/26/2007 12:47 | 11/26/2007 12:52 |
| soschnut | 12/19/2007 14:01 | 12/19/2007 14:08 |
| soschnut | 12/19/2007 07:53 | 12/19/2007 07:58 |
| sosvanut | 01/16/2007 17:07 | 01/16/2007 17:10 |
| sosvanut | 01/16/2007 09:55 | 01/16/2007 09:57 |
| sosvanut | 02/12/2007 14:43 | 02/12/2007 14:45 |
| sosvanut | 02/12/2007 07:10 | 02/12/2007 07:12 |
| sosvanut | 03/21/2007 08:10 | 03/21/2007 08:12 |
| sosvanut | 03/21/2007 14:28 | 03/21/2007 14:30 |
| sosvanut | 04/17/2007 06:29 | 04/17/2007 06:31 |
| sosvanut | 04/17/2007 13:02 | 04/17/2007 13:04 |
| sosvanut | 05/08/2007 10:52 | 05/08/2007 10:54 |
| sosvanut | 05/08/2007 17:34 | 05/08/2007 17:36 |
| sosvanut | 06/20/2007 10:32 | 06/20/2007 10:35 |
| sosvanut | 06/20/2007 18:00 | 06/20/2007 18:02 |
| sosvanut | 07/17/2007 08:38 | 07/17/2007 08:40 |
| sosvanut | 07/17/2007 15:10 | 07/17/2007 15:12 |
| sosvanut | 08/28/2007 07:09 | 08/28/2007 07:11 |
| sosvanut | 08/28/2007 13:24 | 08/28/2007 13:26 |
| sosvanut | 09/25/2007 05:25 | 09/25/2007 05:27 |
| sosvanut | 09/25/2007 11:36 | 09/25/2007 11:38 |
| sosvanut | 10/29/2007 08:36 | 10/29/2007 08:38 |
| sosvanut | 10/29/2007 14:05 | 10/29/2007 14:07 |
| sosvanut | 11/26/2007 07:05 | 11/26/2007 07:07 |
| sosvanut | 11/26/2007 13:11 | 11/26/2007 13:13 |
| sosvanut | 12/19/2007 14:22 | 12/19/2007 14:24 |
| sosvanut | 12/19/2007 08:13 | 12/19/2007 08:18 |
| soswinut | 01/16/2007 17:45 | 01/16/2007 17:50 |
| soswinut | 01/16/2007 10:25 | 01/16/2007 10:28 |
| soswinut | 02/12/2007 15:14 | 02/12/2007 15:20 |
| soswinut | 02/12/2007 07:50 | 02/12/2007 07:54 |
| soswinut | 03/21/2007 08:48 | 03/21/2007 08:50 |
| soswinut | 03/21/2007 14:57 | 03/21/2007 15:01 |
| soswinut | 04/17/2007 07:02 | 04/17/2007 07:07 |
| soswinut | 04/17/2007 13:33 | 04/17/2007 13:40 |
| soswinut | 05/08/2007 11:54 | 05/08/2007 11:58 |
| soswinut | 05/08/2007 18:02 | 05/08/2007 18:06 |
| soswinut | 06/20/2007 11:11 | 06/20/2007 11:15 |
| soswinut | 06/20/2007 18:30 | 06/20/2007 18:36 |
| soswinut | 07/17/2007 09:13 | 07/17/2007 09:17 |
| soswinut | 07/17/2007 15:38 | 07/17/2007 15:44 |
| soswinut | 08/28/2007 07:42 | 08/28/2007 07:47 |
| soswinut | 08/28/2007 14:05 | 08/28/2007 14:08 |
| soswinut | 09/25/2007 06:02 | 09/25/2007 06:05 |
| soswinut | 09/25/2007 12:05 | 09/25/2007 12:07 |
| soswinut | 10/29/2007 09:10 | 10/29/2007 09:15 |
| soswinut | 10/29/2007 14:32 | 10/29/2007 14:37 |
| soswinut | 11/26/2007 07:37 | 11/26/2007 07:41 |
| soswinut | 11/26/2007 13:41 | 11/26/2007 13:45 |
| soswinut | 12/19/2007 14:47 | 12/19/2007 14:53 |
| soswinut | 12/19/2007 08:44 | 12/19/2007 08:48 |

  - Diel sampling

| Site | Start Date/Time | Stop Date/Time |
| --- | --- | --- |
| soschnut | 01/16/2007 09:00 | 01/17/2007 10:00 |
| soschnut | 02/12/2007 07:00 | 02/13/2007 08:00 |
| soschnut | 03/21/2007 07:30 | 03/22/2007 08:30 |
| soschnut | 04/17/2007 06:00 | 04/18/2007 07:00 |
| soschnut | 05/08/2007 10:00 | 05/09/2007 11:00 |
| soschnut | 06/20/2007 09:30 | 06/21/2007 10:30 |
| soschnut | 07/17/2007 08:00 | 07/18/2007 09:00 |
| soschnut | 08/28/2007 05:30 | 08/29/2007 06:30 |
| soschnut | 09/25/2007 04:30 | 09/25/2007 19:30 |
| soschnut | 10/29/2007 07:30 | 10/30/2007 08:30 |
| soschnut | 11/26/2007 06:30 | 11/27/2007 07:30 |
| soschnut | 12/19/2007 07:30 | 12/20/2007 08:30 |

Note: Time is based on a 24 hour clock and is referenced to Pacific Standard Time (PST).

- Associated researchers and projects

As part of the SWMP long-term monitoring program, SS NERR also monitors Meteorological and Water Quality data which may be correlated with this Nutrient dataset.  These data are available from the Research Coordinator or online at http://cdmo.baruch.sc.edu/.

The SSNERR water quality monitoring project has been incorporated into the State of Oregon Coastal Non-Point Pollution Control Program (S. 6217) developed by the Oregon Department of Land Conservation and Development and the Oregon Department of Environmental Quality in July 1995.

South Slough NERR is a partner in NANOOS, Northwest Association of Networked Ocean Observing Systems. SWMP data will be used to address critical environmental issues for sustainable development of the Pacific Northwest, including estuarine water quality, estuarine ecosystem management & restoration, coastal storms and erosion and search and rescue operations.  SSNERR SWMP staff are currently working to install and maintain three NANOOS water quality monitoring stations in the greater Coos estuary.

- Distribution

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The PI retains the right to be fully credited for having collected and processed the data.  Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  Manuscripts resulting from this NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu/.  Data are available in text tab-delimited format.

##### II. Physical Structure Descriptors

- Entry verification

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and automatically flags and codes values below MDL; calculates parameters chosen by the user and automatically flags for component values below MDL and negative values; allows the user to apply QAQC flags and codes to the data; graphs selected parameters for review; append files; and export the resulting data files to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

Analysis results were sent from the University of Washington Marine Chemistry Laboratory in Excel format.  Files consisted of sampling station ID, date, replicate number, and parameter values expressed in unit concentrations.  Most of the results are reported from the University of Washington (UW) Marine Chemistry Laboratory in µmol and must be converted to mg/L prior to CDMO database integration (see conversion table in Section 11).    Data are double-checked to insure correct data transfer.

Files received from UW Marine Chemistry Laboratory do not contain data with specified numbers of significant figures.  Rather, data values contain more decimal places than can be accurately measured (often >10), but the file format is altered to show only two decimals on screen, which is the correct data accuracy according to the lab manager.  When the data reported in µmol are converted to mg/L, the number of decimal places that can be used changes.  We reported our data with the number of decimal places that conserves the appropriate number of significant figures, i.e., four decimal places for all nutrients.  Data that does not require unit conversion (CHLA, PHEA) are reported to two decimals.  The UW laboratory manager concurs that this conserves the proper number of significant figures in each case.

Adam DeMarzo entered and double-checked 2007 sampling dates, locations, times, field parameters, and replicates from the original field data sheets.

- Parameter titles and variable names by data category

| Data Category |  | Parameter |  | Variable Name |  |  | Units of Measure |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phosphorus: |  |  |  |  |  |  |  |  |  |
|  |  | Orthophosphate, Filtered |  | PO4F |  |  | mg/L as P |  |  |
| Nitrogen: |  |  |  |  |  |  |  |  |  |
|  |  | Ammonium, Filtered |  | NH4F |  |  | mg/L as N |  |  |
|  |  | Nitrite, Filtered |  | NO2F |  |  | mg/L as N |  |  |
|  |  | Nitrate, Filtered |  | NO3F |  |  | mg/L as N |  |  |
|  |  | Nitrite + Nitrate, Filtered |  | NO23F |  |  | mg/L as N |  |  |
|  |  | Dissolved Inorganic Nitrogen |  | DIN |  |  | mg/L as N |  |  |
| Plant Pigments: |  |  |  |  |  |  |  |  |  |
|  |  | Chlorophyll a |  | CHLA_N |  |  | µg/L |  |  |
|  |  | Phaeophytin |  | PHEA |  |  | µg/L |  |  |
| Other Laboratory Parameters: |  |  |  |  |  |  |  |  |  |
|  |  | *Silicate, Filtered |  | SiO4F |  |  | mg/L as SI |  |  |
| Field Parameters: |  |  |  |  |  |  |  |  |  |
|  |  | Water Temperature |  | WTEM_N |  |  | oC |  |  |
|  |  | Specific Conductance |  | SCON_N |  |  | mS/cm |  |  |
|  |  | Salinity |  | SALT_N |  |  | ppt |  |  |
|  |  | Dissolved Oxygen |  | DO_N |  |  | mg/L |  |  |
|  |  | % Dissolved Oxygen Saturation |  | DO_S_N |  |  | % |  |  |
|  |  | pH |  | PH_N |  |  | units |  |  |
|  |  | **Tidal Stage |  |  |  |  |  |  |  |

*SiO4F is the same as Si(OH)4 which is the designation used by the UW Marine Chemistry Laboratory.

**Tidal stages are reported by code in the record comment column of the data file and defined as follows:  High (TSH) – one hour before and after the predicted high tide time; Low (TSL) – one hour before and after the predicted low tide time; Ebb (TSE) – after high tide but before the next low tide; Flood (TSF) – after low tide but before the next high tide.

Notes:

1.  Time is coded based on a 2400 clock and is referenced to Standard Time.

2.  Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.

- Measured and calculated laboratory parameters

  - Variables measured directly

Nitrogen species:		NH4F, NO2F, NO23F*

Phosphorus species:		PO4F

Plant Pigments:		CHLA_N, PHEA

Other:		SiO4F

*NO23F is measured directly by the lab and used to calculate NO3F, however only NO2F and NO3F are reported to the reserve.  Therefore NO23F values are “re-calculated” by adding the NO3F and NO2F values onsite in Excel.  For the purposes of this dataset NO23F is considered to be directly measured.

  - Computed variables

NO3:	NO23F-NO2F

DIN:	NO23F+NH4F

  - Unit conversions

PO4F (mg/L as P)	PO4 (µM)/32.29

NH4 (mg/ L as N)	NH4 (µM)/71.39

NO2 (mg/ L as N)	NO2 (µM)/71.39

NO3 (mg/ L as N)	NO3 (µM)/71.39

SiO4F (mg/L as SI)	SiO4F (µM)/35.6

- Limits of detection

Method Detection Limits (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect, have been established by the University of Washington Marine Chemistry Laboratory.  The MDLs are the same as those reported to South Slough in the 2004 and 2005 metadata because according to the laboratory manager, she has not rechecked their instrumentation since October 2002.  When we asked for updated MDLs the lab manager reported that the instrumentation does not drift enough to constitute the time and cost that goes into determining the MDLs.

Method detection limits for each laboratory parameter reported

| Parameter | Variable | Method Detection Limit | Dates In Use |
| --- | --- | --- | --- |
| Orthophosphate | PO4F | 0.0006 mg/L as P | 2007 |
| Ammonium | NH4F | 0.0007 mg/L as N | 2007 |
| Nitrite | NO2F | 0.0001 mg/L as N | 2007 |
| Nitrate | NO3F | 0.0021 mg/L as N | 2007 |
| Nitrite + Nitrate | NO23F | * |  |
| Dissolved Inorganic Nitrogen | DIN | ** |  |
| Chlorophyll a1 | CHLA_N | 0.02 µg/L | 2007 |
| Phaeophytin1 | PHEA | 0.02 µg/L | 2007 |
| Silicate | SiO4F | 0.0059 mg/L as SI | 2007 |

* There is no MDL listed for NO23F because the University of Washington Marine Chemistry Laboratory does not use an MDL for this parameter.  The lab manager has provided MDLs for only NO2F and NO3F, even though NO3F is a calculated value resulting from NO23F.

** There is no MDL for DIN because it is a calculated value resulting from the addition of NO23F and NH4F.

For all analyses requiring filtration, the MDL is variable as detection limit is lowered with more filtration.

- Laboratory methods

    - Parameter:  NH4F

Method Reference:  Slawyk, G. and MacIsaac, J.J. (1972) Comparison of two automated ammonium methods in a region of coastal upwelling. Deep Sea Research 19:521-524.

Method Descriptor:  A water sample is treated with phenol and alkaline hypochlorite in the presence of NH3 to form indophenol blue (Berthelot reaction).  Sodium nitroferricyanide is used as a catalyst in the reaction.  Precipitation of Ca and Mg hydroxides is eliminated by the addition of sodium citrate-complexing reagent. The sample stream is passed through a 55 oC heating bath, then through a 50 mm flowcell and absorbance is measured at 640 nm.on a Technicon Model AAII.

Preservation Method:  Sample is filtered through a 0.45 µm disposable disk filter and stored at –20oC until analyzed.

    - Parameter:  NO23F, NO3F and NO2F

Method Reference:  Armstrong, F. A., C. R. Stearns, and J. D. H. Strickland.  1967.  The measurement of upwelling and subsequent biological processes by means of the Technicon AutoAnalyzer and associated equipment.  Deep Sea Research 14:381-389.

Method Descriptor:  For nitrate+nitrite (NO23) analysis, a water sample is passed through a cadmium column where the nitrate (NO3) is reduced to nitrite (NO2).  This NO2 is then diazotized with sulfanilamide and coupled with N-(1-naphthyl)-ethylenediamine to form an azo dye.  The sample is then passed through a 15 mm flowcell and absorbance is measured at 540 nm on a Technicon Model AAII, giving NO23. A 50 mm flowcell is required for NO2.  The procedure is the same for the NO2 analysis less the cadmium column.  NO3 concentration equals the NO23 concentration minus the NO2 concentration.

Preservation Method:  Sample is filtered through a 0.45 µm disposable disk filter and stored at –20oC until analyzed.

    - Parameter:  SiO4F, [SI(OH)4]

Method Reference: Armstrong, F. A., C. R. Stearns, and J. D. H. Strickland.  1967.  The measurement of upwelling and subsequent biological processes by means of the Technicon AutoAnalyzer and associated equipment.  Deep Sea Research 14:381-389.

Method Descriptor:  Ammonium molybdate is added to a water sample to produce silicomolybdic acid, which is then reduced to silicomolybdous acid (a blue compound) following the addition of stannous chloride.  The sample is passed through a 15 mm flowcell and absorbance is measured at 820 nm on a Technicon Model AAII.

Preservation Method:  Sample is filtered through a 0.45 µm disposable disk filter and stored at –20oC until analyzed.

    - Parameter:  PO4F

Method Reference:  Bernhardt, H. and A. Wilhelms.  1967.  The continuous determination of low level iron, soluble phosphate, and total phosphate with the AutoAnalyzer.  Technicon Symposium 1:386.

Method Descriptor:  Ammonium molybdate is added to a water sample to produce phosphomolybdic acid, which is then reduced to phosphomolybdous acid (a blue compound) following the addition of dihydrazine (or hydrazine) sulfate.  The sample is passed through a 50 mm flowcell and absorbance is measured at 820 nm on a Technicon Model AAII.

Preservation Method:  Sample is filtered through a 0.45 µm disposable disk filter and stored at –20oC until analysis.

    - Parameter:  CHLA, PHEA

Method References:  EPA  1997.  In vitro determination of chlorophyll a and phaeophytin a in marine and freshwater algae by fluorescence.  Method 445.0.

UNESCO.  1994.  Protocols for the joint global ocean flux study (JGOFS) core measurements.  pp. 97-100.

Method Descriptor:  CHLA is extracted in 10 ml 90% acetone and fluorescence is measured and recorded (Fo) with a Turner model TD700 fluorometer using the multi-option raw fluorescence mode.  Several drops of 10% hydrochloric acid are added to convert the CHLA to PHEA.  The fluorescence is again measured and recorded (Fa).  The concentration (µg/L) of CHLA and PHEA are calculated using the Fo/Fa ratio.

Preservation Method:  A known volume of sample is filtered onto a 25 mm GF/F filter, folded in half and wrapped in aluminum foil.  Foil is stored at –20oC until analysis.

- QA/QC programs

  - Precision

    - Field variability.  Three successive grab samples are collected for the monthly grab samples.  Low tide samples at two sites (BH and VA) are collected successively by hand about one minute apart, high tide samples at these sites are collected via Van Dorn collection method.  At the other two sites (CH and WI) a Van Dorn sampler is used to collect high tide samples. Low tide samples at WI are collected by hand.  Van Dorn samples are taken within approximately two minutes of each other.  A 5-lb. weight attached to the sampler is suspended 0.5 m below the Van Dorn sample collection tube.  The sampler and weight are lowered carefully until the weight touches the channel bottom, ensuring that replicate samples are collected at the same depth.

    - Laboratory variability.  The University of Washington Marine Chemistry Laboratory runs test standards and checks samples in duplicate or triplicate. Three blanks for CHN analysis are run monthly.

    - Inter-organizational splits.  None.

  - Accuracy

    - Sample spikes.  None.

    - Standard reference material analysis.

In order to ensure that quality control measures are being performed by the analytical laboratory, and to determine the quality of analysis, the NERRS quality assessment program conducted a laboratory comparison study.  In August 2003, a commercially-available, certified seawater reference sample for nutrients was purchased.  A sealed, 50-ml aliquot was mailed to the University of Washington laboratory for analysis.  Staff were asked to report nitrite, nitrate + nitrite, and orthophosphate concentrations expressed as mg/L as N or P.  Results were (reference material values with overall uncertainty values in parentheses) orthophosphate 0.053 mg/L (0.048 +/-0.002 mg/L, a +10.4% deviation), nitrite 0.046 mg/L (0.043 +/- 0.002 mg/L, a +7%deviation) and nitrate + nitrite 0.342 mg/L (0.332 +/- 0.013 mg/L, a +3% deviation).

    - Cross calibration exercises.

The University of Washington Marine Chemistry Laboratory runs two commercially available performance evaluation samples per year as part of its accreditation with the State of Washington. It also informally cross-checks results with peer laboratories.  This lab has participated in the U.S. Environmental Protection Agency Performance Evaluation program since 1990, but South Slough has been unable to attain copies of any of these results.

15) QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range*

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

*The -4 Outside Low Sensor Range flag was added to the 2007 dataset in August of 2011.  See the Other Remarks section for more details.

16)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data rejected due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Value calculated with a value that is below the MDL

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N	from the north

NNE	from the north northeast

NE	from the northeast

ENE	from the east northeast

E	from the east

ESE	from the east southeast

SE	from the southeast

SSE	from the south southeast

S	from the south

SSW	from the south southwest

SW	from the southwest

WSW	from the west southwest

W	from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17)  Other remarks

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

*The 2007 dataset was updated on August of 2011 to include the -4 Outside Low Sensor Range flag.  The 2007 data published prior to that time used the -3 Rejected data flag with the SBL and SCB QAQC codes to indicate that data were below the minimum detection limit.  These flag code combinations were all replaced with the -4 SBL or SCB update as mandated by the Data Management Committee.

References

Browning, N. 1980. Determination of nutrient concentration in the South slough estuary.  South Slough National Estuarine Sanctuary Technical Report.  28 pp.

Cowlishaw, R. 2001.Microzooplankton trophic interactions and their impact on phytoplankton production and community structure in the South Slough arm of Coos Bay, Oregon.  Ph.D. thesis progress report. University of Oregon, Oregon Institute of Marine Biology. 14 pp.

Fry, B., A. Grace, and J.W. McClelland. 2001. Chemical indicators of anthropogenic nitrogen loading in west coast NERR estuaries. CICEET Project Report 99-296. 80 pp.

Karentz, D. and C.D. McIntire. 1977. Distribution of diatoms – the plankton of Yaquina estuary, Oregon.  Journal of Phycology 13:379-388.

Nybakken, J.W. Marine Biology: An Ecological Approach.  3rd edition. New York, NY: HarperCollins College Publishers. 462 pp.

Pequegnat, J.E., and J.H. Butler. 1982.  The biological oceanography of Humboldt Bay.  In Proceedings of the Humboldt Bay Symposium (C. Toole and C. Diebel, eds.) pp. 39-51. Center for Community Development, Humboldt State University, Arcata, CA.

South Slough National Estuarine Research Reserve.  2004.  SWMP: Standard operating protocol: collection and filtration of SWMP nutrients and plant pigments.

Wells, S.A. and B. Baird. 1992.  Field survey data summaries, South Slough, Oregon (July 1989- September 1990). NOAA, Marine and Estuarine Management Division NA89AA-D-CZ047. Technical Report EWR-007-92, Department of Civil Engineering, Portland State University, Portland, OR. 117 pp.