Waquoit Bay (WQB) National Estuarine Research Reserve Meteorological Metadata

January 2017 – June 2017

Last updated: July 26, 2017

Note: This is a provisional metadata document; it has not been authenticated as of its download date.  Contents of this document are subject to change throughout the QAQC process and it should not be considered a final record of data documentation until that process is complete.  Contact the CDMO (cdmosupport@belle.baruch.sc.edu) or Reserve with any additional questions.

I.  Data Set and Research Descriptors

- Principal investigator(s) and contact persons –

Jordan Mora, Research Associate, Jordan.mora@state.ma.us, 508-457-0495 x 128

Megan Tyrrell, Research Coordinator, Megan.tyrrell@state.ma.us, 508-457-0495 x 105

Address:

Waquoit Bay NERR

131 Waquoit Highway

PO Box 3092

Waquoit, MA 02536

Phone: (508)-457-0495

Homepage: http://www.waquoitbayreserve.org

- Entry verification –

System I: CR1000, CR10X

The initial weather station and CR10X program was installed in late October 2003.  The old program (ner30.csi) was revised (NERR_4.CSI) to standardize the program for all sites.  The revision was necessary to meet new data reporting requirements of CDMO to eliminate instantaneous data sample reporting, add cumulative daily rainfall and additional sensors.

Starting July 12, 2006 at 17:45, WQBNERR weather station was changed from the CR10X Datalogger to a CR1000 Datalogger and associated software program. The sensors for Air Temperature (C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed, Wind Direction, Total Precipitation (mm), Total Photosynthetically Available Radiation (PAR), and Total Solar Radiation (SoRAD) remain the same. See section 9: Sensor specifications, operating range, accuracy, date of last calibration for sensor specifications.

On February 10th, 2005, a new EPLAB© Black and White Pyranometer was installed. It was mounted on the same structure as the PAR instrument approximately within one meters distance. This Pyranometer is an Eppley 10-and 50-junction 180O pyrheliometer originally introduced by Kimball and Hobbs in 1923. The detector is a differential thermopile with the hot-junction receivers blackened and the cold-junction receivers whitened. The element is of radial wire-wound-plated construction. Built in temperature compensation with thermistor circuitry is incorporated to free the instrument from the effects of ambient temperature. A precision ground optical glass hemisphere of Schott glass WG295 uniformly transmits energy from 285 to 2,800 millimicrons. This hemispherical envelope seals the instrument from the weather, but is readily removable for instrument repair. The cast aluminum case carries a circular spirit level and adjustable leveling screws. Also supplied is a desiccator, which can be inspected readily. See section 9 for most recent calibration dates.

Although the Eppley pyranometer was installed in 2005 and collected data until 2014, in October 2014 research staff at Waquoit Bay realized the sensor was collecting total solar radiation data in Watt-hours per meter-squared (a measure of maximums within a 15 minute period).  CDMO (Central Database Management Office) protocols require the total solar radiation data to be displayed in Watts per meter-squared (a measure of 5-second averages over a 15 minute period). Because of the discrepancy, these data have been removed from the national database but are available by request. Please contact the WBNERR research staff for total solar radiation data series (contact information on page 1 of this document).

The meteorological information is sampled every 5 seconds from each instrument on the weather station and stored on a Campbell Scientific CR1000 data logger. Data are output to a file in three file formats: CR1000_A5Min.dat stores 5 minute data; CR1000_GOESout.dat stores 15 minute averages that are transmitted hourly for Satellite upload; and CR1000_SWMP.dat files stores the 15 minute average data that is submitted to CDMO on a quarterly basis for primary QAQC review. The CDMO Data Logger (NERR_4.CSI) was loaded into the CR1000 and controls the sensors.

Data are uploaded from the CR1000 data logger to an IBM-compatible computer.  Files are exported from or LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

Jordan Mora, Research Associate, error checked and compiled the meteorological data in 2017.

- Research objectives –

The principal objectives are to record meteorological information for the Waquoit Bay NERR’s site that can be used: 1) as a vital reference of atmospheric data for various research projects at the reserve - an integral part of our general NERR mission is to provide a platform for estuarine research; 2) to give meteorological context (atmospheric-forcing) for our fifteen minute SWMP water quality data, and other long-term environmental monitoring programs at the Reserve (including nutrients and shoreline change); 3) to observe and characterize important events, such as storms, heat and cold waves, droughts and heavy rainfall; and 4) to detect trends and characterize climate variability over the long term.

- Research methods –

The Campbell Scientific weather station samples every 5 seconds continuously throughout the year.  These data are used by the CR1000 to produce 15 minute, hourly and daily averages of those measurements of air temperature, relative humidity, barometric pressure, wind speed, and wind direction. Precipitation and PAR are recorded as totals for each interval. As mentioned above, we generally upload data from the CR1000 storage module about once a month. CR1000 raw data are currently stored on one data storage module capable of storing about 3 months of data. The CR1000 is also cabled directly to a desktop PC where the instantaneous 5 sec data are displayed (in a LoggerNet window) and can be viewed at any time. During quarterly review, the error/anomaly reports and all monthly parameter graphs are printed and reviewed.  Any error/anomaly messages are further investigated and the data is either corrected/deleted (if necessary) or commented on and left unchanged.

Sensors on the weather station are inspected monthly for damage or debris. See section 9 for most recent calibration dates. Also, once a month at the time of uploading, we obtain ancillary weather data from a handheld device, Kestrel 5500, to check against the Campbell Scientific Inc. sensors. We also compare the current sensor readings to the weather information available online for the Hyannis airport. All data are recorded with monthly maintenance report.

Real-time Data Methods:

In July 2006, our meteorological station was linked to a NOAA GOES © satellite system, which allowed near real-time access to our meteorological data over the internet. Campbell Scientific data telemetry equipment is used at this station to transmit to the NOAA GOES satellite, NESDIS ID #3B022462. The transmissions are scheduled hourly and contain four (4) datasets reflecting the fifteen minute data sampling interval. Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above. The “real time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database. Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15-minute data are collected in the following formats for the CR1000:

Air Temperature (°C)

- 15 minute average (averages from 5-second data over previous 15 minutes)

- Maximum (over previous 15 minutes)*

- Minimum (over previous 15 minutes)*

- Time Maximum (from 5-second data)*

- Time Minimum (from 5-second data)*

Relative Humidity (%)

- 15 minute average (averages from 5-second data over previous 15 minutes)

Barometric Pressure (mb)

- 15 minute average (averages from 5-second data over previous 15 minutes)

Wind Speed (m/s)

- 15 minute average (averages from 5-second data over previous 15 minutes)

- Maximum (over previous 15 minutes)

- Time Maximum (over previous 15 minutes)

Wind Direction

- 15 minute average (averages from 5-second data over previous 15 minutes)

- Standard Deviation (over previous 15 minute period)

Total Precipitation (mm)

- previous 15 minute total

Cumulative precipitation (mm)

- running total of daily precipitation sum of 15 minute totals over a 24 hour period

Total Photosynthetically Available Radiation (PAR)

- previous 15 minute total (millimoles/m2)

*Available from the Waquoit Bay Reserve (see contact information on page 1). Not a standard CDMO parameter.

Recommended calibration frequency for the MET station sensors:

Sensor					Calibration Frequency

Temperature/Humidity		yearly

Rain Gauge-				yearly

Wind Speed/Direction			every 2 years

Barometric Pressure			every 2 years

PAR					every 2 years

CR1000				every 5 years

5)  Site location and character –

The weather station is located beside the Carriage House, an historic building which is part of the Waquoit Bay Reserve Headquarters, a 24-acre parcel of Reserve land. Wind (speed and direction), temperature and relative humidity sensors are mounted on a 10-m aluminum tower next to the Carriage House, which houses our education classroom and research laboratory. As of April 18th, 2016, the temperature, relative humidity, and wind sensors are mounted approximately 28ft above ground level. Before April 18th, the wind sensor was mounted at the top of the tower (30ft); however, it was lowered to a cross-arm in order to accommodate a lightening rod installation needed to better protect the weather equipment (see Figure 1).

Prior to August 11th, 2015, the weather tower, which supports the wind, temperature, and relative humidity sensors, was located at 41° 34’54.12 N, 70° 31’ 30.36 W. Starting on November 16th, 2015, the tower location changed to 41° 34’54.09” N, 70° 31’30.65”W. The move occurred in response to a renovation project on the Carriage House; a new ADA-compliant (Americans with Disabilities Act of 1990) ramp was built on the side of the building where the tower was previously installed (see Figure 1). The current location is roughly 6.5 meters northwest of the past location, and instead of being on the south side of the Carriage House, the tower is now on the west side (see Figure 2). The top of the tower exceeds the height of the building; its attached probes stand approximately 2.5 m above the roof peak of the adjacent building and are separated from any trees by 10 m. A crushed shell parking area (bleach white in color) is located to the south (20m) and west (8m) of the current tower.  The tower base is 10.39 m above sea level (NGVD), approximately 100 m north from Waquoit Bay’s northern shoreline. The location is most exposed to winds from the west and south.

- 2002

b) 2017

Figure 1: Photos of before (a) and after (b) the 2015 Carriage House renovation and the 2016 tower upgrade. The tower upgrade in 2016 involved a new lightening rod, grounding wire, cross bar, and upgraded temperature/relative humidity and photosynthetically active radiation sensors.

Before April 18th, 2016, the LiCor Quantum Sensor (for photosynthetically active radiation readings) was mounted about 10 m east from the weather tower, where the temperature/relative humidity and wind sensors are installed. The PAR sensor pole included an extended aluminum arm at a height of 3 m (~ 10 ft) above the ground level. On April 18th, 2016, a new PAR sensor (see sensor details in Section 9) was installed on the weather tower at a height of approximately 4m (~14 ft) above ground level. Thus, current PAR readings are taken roughly 16m westward and roughly 1m higher in relation to readings taken before April 18th, 2016. However, photon scatter related to the roofline and ground should be similar between pre- and post-sensor location change. Due to shading interference in the summer months from a nearby tree, the PAR sensor was moved 10 ft higher (~24ft above ground level) on October 25th, 2016.

The air pressure sensor, which is mounted next to the CR1000 in the laboratory, is approximately 1.5meters in height above ground level. The rain gauge is located in an open field away from trees about 55 m northwest of the laboratory and tower and 11.2 m above sea level (NGVD). The top of the gauge is 1 meter above ground.

Carriage House

Visitor Center

Figure 2: The image above shows the Waquoit Bay NERR Headquarters. The red circle marks the location of the current 30-foot tower location (November 2015 – present). The yellow circle marks the past location of the wind/temperature/relative humidity tower (2006 – August 2015). The white circle marks the location of the photosynthetically active radiation sensor prior to April 18th, 2016, when the sensor was moved to the 30-foot tower (red dot). The larger blue circle indicates the location of the precipitation gauge in the open field. The barometric pressure sensor is located inside the Carriage House in the laboratory along with the CR1000 logger. Aerial provided by Google Earth (spring 2015).

As for its general setting, the Waquoit Bay National Estuarine Research Reserve (WQBNERR) is located in the northeastern United States on the southern coast of Cape Cod, Massachusetts.  Climatically, this region is considered temperate maritime, and experiences relatively mild winters and cool summers relative to the rest of New England because of its exposed oceanic location. Typical of the mid-latitudes (41 N), prevailing winds are from the southwest, while storm winds tend to be from the east.

The area is adjacent to one of the world’s most active regions for cyclogenesis (extra-tropical cyclone formation) off the East coast of North America. These generally winter season storms are most frequent (almost weekly) from late October until late April and are locally called Nor’easters because of the NE wind direction typical to the area during the period of peak wind speeds.  These storms generally develop rapidly as secondary lows off the mid-Atlantic coast (Carolinas to New Jersey) and track northeastward passing Cape Cod either directly overhead, or to the southeast or northwest. These winter season storms are important agents of coastal erosion and shoreline alteration in the region, particularly for easterly facing coasts.

Hurricanes are also important phenomena in the region. Most years, during the period from July to November, the Cape experiences some interaction with a passing tropical storm. About once every decade the area experiences a nearby landfall, with winds exceeding hurricane threshold (>33 m/s), usually from the southerly quarter. Hurricanes are particularly important agents of change for the Cape’s southern coastal areas, and can have profound effects on local estuaries, including Waquoit Bay. Typically, barrier beach over-wash (with salt marsh burial) and breaching (with new tidal inlet formation) occur during these extreme events.

Meteorological data from Waquoit Bay NERR can be compared to that from other nearby meteorological stations. These stations are located at Otis Air Force Base (10 km to the north), Falmouth Water Department-Long Pond (8 km to the west), Woods Hole Oceanographic Institution–Quisset Campus (13 km to the southwest), Hyannis Airport (23 km to the northeast), and Buzzards Bay Texas Tower (41 km to the southwest) – this latter station being a particularly valuable reference site because it is offshore and southwest with at least 15 km of unobstructed open water around it and it also records other useful sea surface parameters (wave height and direction, and ocean temperatures).

Other stations which are also used for reference include the Menauhant Yacht Club weather station operated and maintained by Dr. Richard Taylor as a Local National Weather Service Reporting Station (Data Garrison https://datagarrison.com/). Since December 2002, the National Weather Service Reporting Station has been recording daily observations at 0700 and 1900 for the following weather parameters:  Temperature (oF; minimum and maximum), Precipitation (inches; rain and melted snow, snow fall, initial and endpoint times of events), Sky Conditions and Wind Direction (mph; gusts).

6)  Data collection period –

Weather data has been collected at the Waquoit Bay NERR Carriage House since December 2001.

Program Reloads in 2017 (EST):

Program Reload on 06/08/2017 at approximately 14:15

Weather data was downloaded from the station for the following periods 2017 (EST):

START			END

Time	 Date			Time	 Date

10:30	01/25/2017		11:30 	02/22/2017

11:45 	02/22/2017		14:00 	03/28/2017

14:15 	03/28/2017		14:15 	04/27/2017

14:30 	04/27/2017 		13:45	05/23/2017

14:00 	05/23/2017		14:00 	06/08/2017

7)  Distribution –

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2016.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

8)  Associated researchers and projects –

The Waquoit Bay Meteorological Station located at the Carriage House satisfies the weather data collection requirement of a broader nationwide program, the System-Wide Monitoring Program (SWMP). Within the SWMP, additional required parameters include water quality and nutrient monitoring.

- Water quality monitoring involves continuous measurements of water temperature, specific conductivity, salinity, dissolved oxygen (percent saturation and concentration in mg/L), depth, pH, turbidity, and chlorophyll fluorescence. These measurements are taken every 15 minutes and stored on a submersible YSI multi-parameter sonde (i.e., data logger). Sondes are rotated every 2-3 weeks for cleaning and calibration.

- To meet the nutrient sampling requirements of the SWMP, monthly grab samples, in tandem with a 24-hour ISCO water sampler, are collected and processed for various nutrient compounds. The chemical analyses mainly focus on levels of nitrogen, phosphorous, and carbon in the water column.

The water quality monitoring and nutrient sampling occur at four water quality stations located throughout the Waquoit Bay Estuary: Childs River (tidal riverine system with high nitrogen load), Menauhant Yacht Club (closest proximity to ocean influence from Vineyard Sound), Sage Lot Pond (tidal pond surrounded by polyhaline salt marshes), and Metoxit Point (open water location inside Waquoit Bay). These data are available at www.nerrsdata.org.

II.  Physical Structure Descriptors

9)  Sensor specifications –

Temperature

January 1, 2015 – December 4, 2015 (WBNERR unit)

Units: Celsius

Sensor Type: Platinum Resistance Thermometer (PRT), Thermistor (100kΩ @ 25°C)

Model #: HMP35C

Operating Temperature:  -35 to +60°C

Temperature Measurement Range:  -35 to +55°C

Temperature Accuracy:  ±0.4°C

S/N: A4940008

Date of last calibration: 6/27/2012

Dates of Sensor Use: 6/27/2012 – 12/04/2015

Starting December 4, 2015 (Great Bay NERR unit on loan)

Units: Celsius

Sensor Type: Platinum Resistance Thermometer (PRT) 1000 Ω, IEC 751 1/3 Class B Model #: HMP45AC

Operating Temperature:  -40 to +60°C

Temperature Measurement Range:  -40 to +60°C

Temperature Accuracy:  ±0.4°C

S/N: Z2220075

Date of last calibration: 02/05/2014

Dates of Sensor Use: 12/04/2015 – 04/18/2016

Starting April 18th, 2016

Units: Celsius

Sensor type: PT100 RTD, IEC 751 1/3 Class B, with calibrated signal conditioning

Model #:  HC2-S3 Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.1 °C @ 23°C

S/N: 0020052725

Date of last calibration: 08/17/2015

Dates of sensor use: 04/18/2016 – 06/08/2017

S/N: 0020072974

Date of last calibration: 11/28/2015 (new, received 05/16/2017)

Dates of Sensor Use: 06/08/2017 – current as of 07/24/2017

Relative Humidity (RH)

June 27, 2012 – December 4, 2015 (WBNERR unit)

Units: Percent

Sensor Type: Sensor Type: Vaisala capacitive polymer H chip

Model #: HMP35C

Relative Humidity Measurement Range:  0 to 100%

RH Accuracy:

±2% RH (0 to 90%)

±3% RH (90-100%)

Stability: 1% per year

Uncertainty of calibration:  0.3% RH

S/N: A4940008

Date of last calibration: 6/27/2012

Dates of Sensor Use: 6/27/2012 – 12/04/2015

Starting December 4, 2015 (Great Bay NERR unit on loan)

Units: Percent

Sensor Type: HUMICAP® 180

Model #: HMP45AC

Relative Humidity Measurement Range:  0 to 100% non-condensing

RH Accuracy:

±2% RH (0 to 90%)

±3% RH (90-100%)

Stability: 1% per year

Uncertainty of calibration:  0.3% RH

S/N: Z2220075

Date of last calibration: 02/05/2014

Dates of Sensor Use: 12/04/2015 – 04/18/2016

Starting April 18th, 2016

Units: Percent

Sensor type: ROTRONIC® Hygromer IN-1

Model #: HC2-S3 Temperature and Relative Humidity Probe

Range: 0-100% non-condensing

Accuracy at 23°C:  +/- 0.8% RH with standard configuration settings

Temperature dependence of RH measurement +/- 3% (-40 to 60C)

S/N: 0020052725

Date of last calibration: 08/17/2015

Dates of Sensor Use: 04/18/2016 – 06/08/2017

S/N: 0020072974

Date of last calibration: 11/28/2015 (new, received 05/16/2017)

Dates of Sensor Use: 06/08/2017 – current as of 07/24/2017

Photosynthetically Active Radiation (PAR)

Starting September 9, 2011

Units: millimoles/m2 (total flux)

Sensor Type: LiCor Quantum Sensor (PAR)

Model #: LI-L1190SZ

Stability: < ±2% change over a 1 year period

Operating Range

Temperature: -40°C to 65°C

Humidity: 0 to 100%

Sensitivity:  typically 5µA per 1000 µmoles s-1m-2

Light spectrum wavelength:  400 to 700 nm

S/N: Q45963

Date of last calibration: 09/09/2011

Multiplier: -264.77 (millivolt adapter)

Dates of Sensor Use: 9/9/2011 – 04/18/2016

Starting April 18th, 2016

Units:  mmoles m2 (total flux)

Sensor type:  anodized aluminum with cast acrylic diffuser

Model #SQ110 Apogee Quantum Sensor

Light spectrum waveband:  410 to 655 nm

Temperature dependence: 0.06+/-0.06% per °C

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 70°C; Humidity 0 to 100%

Cosine Response: 45° zenith angle: +/- 2%; 75° zenith angle: +/- 5%

Sensitivity:  0.2mV per µmol s-1 m-2

S/N: 20507

Date of last calibration: 03/31/2016

Dates of Sensor Use: 04/18/2016 – current as of 07/24/2017

Wind Speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model: RMY 05103

Range:

0-60 m/s (134 mph)

gust survival 100 m/s (220 mph)

Direction Range: 360° Threshold Sensitivity: 0.5 m/s (1.1 kts) at 10° displacement

Accuracy: +/- 0.3 m/s

S/N: 09834 (WBNERR unit)

Date of last calibration: 06/29/2016

Dates of Sensor Use: 	6/27/2012–12/04/2015, 10/25/2016 – current as of 7/24/17

S/N: 05178A (Great Bay NERR unit on loan)

Date of last calibration: New in 2015

Dates of Sensor Use:  12/04/2015 – 10/25/16

Wind Direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Model: RMY 05103

Direction Range: 360° Threshold Sensitivity: 0.5 m/s (1.1 kts) at 10° displacement

S/N: 09834 (WBNERR unit)

Date of last calibration: 06/29/2016

Dates of Sensor Use: 	6/27/2012–12/04/2015,

10/25/2016– current as of 07/24/2017

S/N: 05178A (Great Bay NERR unit on loan)

Date of last calibration: New in 2015

Dates of Sensor Use:  12/04/2015 – 10/25/2016

Barometric Pressure

Units: millibars (mb)

Sensor Type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS 105 Vaisala PTB101B

Operating Range:

Pressure: 600-1060 mb

Temperature: -40 to +60°C

Humidity: non-condensing

Accuracy: ±0.5 mb @ +20°C

Stability: ± 0.1 mb per year

S/N: P5240006 (WBNERR unit)

Date of last calibration: 6/27/2012

Dates of Sensor Use: 6/27/2012 – 06/07/2016

S/N: Z3720039 (loan from Campbell Scientific)

Date of last calibration: May 2016

Dates of Sensor Use: 06/07/2016 – 06/08/2017

Starting 06/08/2017

Units: millibars (mb)

Sensor Type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS 106 Vaisala PTB101B

Operating Range:

Pressure: 500-1100 mb

Temperature: -40 to +60°C

Humidity: non-condensing

Accuracy:

±0.3 mb (@ +20°C)

±0.6 mb (@ 0° to 40°C)

±1.0 mb (@ -20° to +45°C)

±1.5 mb (@ -40° to +60°C)

Stability: ± 0.1 mb per year

S/N: N0220822

Date of last calibration: 01/11/2017 (new, received 05/16/2017)

Dates of Sensor Use: 06/08/2017 – current as of 07/24/2017

Precipitation

Units: millimeters (mm)

Sensor Type: Heated Tipping Bucket Rain Gauge

Model #: 385L

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:

±0.5% @ < 0.5” (1.25 cm)/hr rate

±2.0% @ < 3.0” (7.50 cm)/hr rate

S/N: C1521

Date of last calibration:  06/08/2017

Dates of Sensor Use: 9/9/2011 - current as of 07/24/2017

The CR1000 has two MB Flash EEPROM that are used to store the Operating System. Another 128 K Flash is used to store configuration settings. A minimum of 2 MB SRAM is (4 MB optional) is available for program storage (16K), operating system use, and data storage. Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module. This station was installed on 7/12/2006. Additional CR1000 specification can be found at: http://www.campbellsci.com/documents/lit/s_cr1000.pdf.

S/N: 5246

Manufacture Year: 2006

Dates CR1000 Installed: 07/12/2006 – 06/07/2016, 10/25/2016 – current as of 07/24/17

Date CR1000 Calibrated: 06/21/2016

CR1000 Firmware Version: CR1000.Std.29 (date: 04/18/2016)

CR1000 Program Version: wqbchmet_6.3.3_052517.cr1

Campbell Charger for CR1000: Model # CH 100, 12 V

06/07/2016 – 10/25/2016

S/N: 55704 (Campbell Scientific loan while WBNERR’s is being recalibrated)

Date CR1000 Installed: 06/07/2016

Date CR1000 Calibrated: May 2016

CR1000 Firmware Version: CR1000.Std.29

CR1000 Program Version: wqbchmet_6.3_060316.cr1

Campbell Charger for CR1000: Model # CH 100, 12 V

10)  Coded variable definitions -

Sampling station:		Sampling site code:		Station code:

Carriage House		CH				wqbchmet

11)  QAQC flag definitions –

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions –

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument malfunction

GIT	Instrument recording error, recovered telemetry data

GMC	No instrument deployed due to maintenance/calibration

GMT 	Instrument maintenance

GPD	Power down

GPF	Power failure / Low battery

GPR	Program reload

GQR	Data rejected due to QA/QC checks

GSM	See metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC		Incorrect calibration constant, multiplier or offset

SIW	Incorrect wiring

SMT	Sensor maintenance

SNV	Negative value

SOC	Out of calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor drift

SSN	Not a number / unknown value

SSM	Sensor malfunction

SSR		Sensor removed

Comments

CAF	Acceptable calibration/accuracy error of sensor

CCU	Cause unknown

CDF	Data appear to fit conditions

CML	Snow melt from previous snowfall event

CRE*	Significant rain event

CSM*	See metadata

CVT*	Possible vandalism/tampering

CWE*	Significant weather event

13)  Other remarks/notes –

- General Comments:

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval. These values are automatically flagged and coded as <1> (CAF).

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3% and are flagged and coded as suspect, <1> (CAF).  Values greater than 103 are rejected <-3>.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

- Program Reloads (GPR):

- 06/08/2017, 14:26 – new program uploaded due to new barometric pressure sensor (changed from CS105 to CS106).

- Specific 2017 Metadata Notes:

General:

- 04/27/2017, 13:10 – unplugged the heating cable.

Temperature and Relative Humidity (RH):

- 04/19/2017 to 06/08/2017, <1>[SOC](CSM) – Temperature and relative humidity are flagged as suspect because the sensor (SN: 0020052725) had surpassed the CDMO recommended calibration standards. It’s important to note that while these sensors do not meet recommended calibration standards, the data appear to fall within acceptable ranges and are comparable to local weather reports. Please contact Jordan Mora (contact information on page 1) for more information and maintenance log reports.

- 06/08/2017, 13:15 to 13:45 <-3>[SMT](CSM) – The weather tower was down to replace the temperature and relative humidity sensor for the yearly calibration rotation. The gill shield was cleaned as well. Replacement serial number: 0020072974. Before the sensor rotation, temperature = 23.26 °C and relative humidity = 49.97%. After the sensor rotation, temperature = 21.14 °C and relative humidity = 57.75%.

Wind Speed and Direction (Wspd, Wdir and SDWDir):

- 06/08/2017, 13:15 to 13:45 <-3>[SMT](CSM) – The weather tower was down to replace the temperature and relative humidity sensor. All wind data is rejected while the tower was down.

PAR (Photosynthetically Active Radiation)

- 06/08/2017, 13:15 to 13:45 <-3>[SMT](CSM) – The weather tower was down to replace the temperature and relative humidity sensor. While the tower was down, the PAR sensor was cleaned.

Barometric Pressure

- 06/08/2017, 14:10 to 15:04, <-3>[SMT](CSM) – Removing old barometric sensor (CS 105; on loan from Campbell Scientific) and replaced with new sensor (CS106; SN: N0220822). Took a while to discover the correct location for the jumper, so an hour of data is discarded.

Precipitation

- 02/22/2017, 10:45 <5>[SMT](CSM) – Tipping gauge was accidentally tipped during monthly maintenance, but data have been corrected for error. Gauge tipped twice during maintenance, so 0.5mm have been removed from the Total Precipitation at 10:45 and from the Cumulative Precipitation from 10:45 to 2/23/17 00:00.

- 03/28/17, 12:45 <5>[SMT](CSM) - Tipping gauge was accidentally tipped during monthly maintenance, but data have been corrected for error. Gauge tipped once during maintenance, so 0.3mm have been removed from the Total Precipitation at 12:45 on 3/28/17 and from the Cumulative Precipitation from 12:45 on 3/28/17 to 00:00 on 3/29/17.

- 06/07/2017 to 06/08/2017, <1>[SOC](CSM) – Total Precipitation and Cumulative Precipitation are flagged as suspect because the gauge had surpassed the CDMO recommended calibration standards. It’s important to note that while these sensors do not meet recommended calibration standards, the data appear to fall within acceptable ranges and are comparable to local weather reports. Please contact Jordan Mora (contact information on page 1) for more information and maintenance log reports.

- 06/08/2017, 09:13 to 10:46, <-3>[SMT](CSM) - Calibrating precipitation gauge. All data rejected during the calibration process.

- 06/08/2017, 11:00 to 23:45, <5>[SMT](CSM) - Cumulative precipitation adjusted to correct for calibration interference. Removed 20.1mm from the incremental total.