Mission Aransas (MAR) NERR Meteorological Metadata

January – June 2017

Latest Update: 06/30/2017

Note: This is a provisional metadata document; it has not been authenticated as of its download date.  Contents of this document are subject to change throughout the QAQC process and it should not be considered a final record of data documentation until that process is complete.  Contact the CDMO (cdmosupport@belle.baruch.sc.edu) or Reserve with any additional questions.

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons

Address: 	Mission Aransas National Estuarine Research Reserve

750 Channelview Drive

Port Aransas, TX 78373

Phone:  (361)749-6711

Fax:	 (361)749-6777

Contact Persons:

Dr. Edward Buskey, Research Program Coordinator

E-mail: ed.buskey@utexas.edu; (361)749-3102

Jace Tunnell, Manager

E-mail: jace.tunnell@austin.utexas.edu; (361)749-3046

Cammie Hyatt, SWMP Data Manager & Technician

E-mail: cammie.hyatt@utexas.edu; (361)749-3093

Collin Croulet, SWMP Technician, Boat Operations

E-mail: collin.croulet@austin.utexas.edu; (361)749-3101

2)  Entry verification –

Data are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from or LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12. Cammie Hyatt is responsible for data management

3)  Research objectives

Five long-term monitoring stations have been established along the Texas Coast in the Mission-Aransas estuary to collect baseline data on environmental conditions within the bay systems. These System Wide Monitoring Program Stations (SWMPs) provide continual monitoring of temperature, salinity, oxygen and other parameters that characterize the water quality of the bays. In addition to the water quality stations, one meteorological station has been installed in Copano Bay (East), as well as a secondary meteorological station at the Ship Channel, to collect data on air temperature, relative humidity, barometric pressure, wind speed, wind direction, precipitation, and solar radiation (PAR). The goal of the SWMP meteorological station is to develop quantitative measurements of short-term variability and long-term changes in the weather within the Mission Aransas NERR. This data will help us determine climactic and hydrological patterns that influence freshwater inflow in the Mission- Aransas Estuary and will be used to help coastal managers make informed decisions on local and regional issues, such as “no discharge” zones for boats and measuring the success of restoration projects. To the best of my knowledge, this is the only water-based weather station within the NERR.

4)  Research methods

Campbell Scientific data telemetry equipment was installed at the MARCEMET station on 11/04/06 and transmits data to the NOAA GOES satellite, NESDIS ID #3B039516.  (Where 3B039516 is the GOES ID for that particular station.)  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees),  Battery Voltage (volts)

Maximum and Minimum Air Temperature (°C) and their times from 5-second data (these data are available from the Reserve)

Maximum Wind Speed (m/s) and time from 5-second data

Wind Direction Standard Deviation (degrees)

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm)

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- yearly recalibration

- Rain Gauge- yearly recalibration

- Wind Speed/Direction- yearly or every 2 years (depending on the sensor)

- Barometric Pressure- every 2 years recalibration

- PAR- every 2 years recalibration

- CR1000-every 5 years (required beginning 2014, one year initial grace period)

-Total Solar Radiation (this is an optional sensor, only include if you are collecting TSR data)-

Eppley 8-48 - every 5 years, but annual recalibration encouraged

LI-COR LI-200S - every 2 years, with recalibrations performed during the spring and summer

5)  Site location and character

The MANERR meteorological station (MARCEMET) is located in open water in east Copano Bay near the Copano Causeway at 28º 7’ 56.28 N, 97º 2’ 3.84 W. The weather station is located on a platform that is 3.5 meters above the water (at the high water mark). Wind (speed and direction), temperature, relative humidity, and PAR sensors are mounted onto a 3 meter aluminum tower so that the sensors stand approximately 4 meters off the platform. The PAR sensor is located on one side of the 1.2 meter sensor crossarm, with the wind monitor on the other side. The rain gauge is located on the arm rail of the platform which is approximately 1.5 meters off the platform. The barometric pressure sensor is located inside the Campbell Scientific enclosure, which is 1.1 m off the platform, and has an intake valve that runs through a hole in the bottom of the enclosure that allows pressure equalization. To the best of my knowledge, all the weather instrumentation and associated sensors are set up according to standard CDMO protocols. There are no obstructions around the platform; therefore, the tower does not receive shade and it is not blocked from the wind. This site also has a SWMP water quality monitoring station installed on the lower level of the platform.

Major climactic influences within the MANERR are temperature, precipitation, evaporation, wind, tropical storms, and hurricanes. The lowest air temperature recorded at MARCEMET in the second quarter of 2017 was 11.5 ºC and the highest temperature was 23.1 ºC, with an average temperature of 23.6 ºC. Minimum wind speed recorded was 0.0 meters per second (m/s) and the maximum was 14.8 m/s, with an average of 4.6 m/s. The lowest reading of relative humidity was 26% and the highest was 97%, with an average of 77% for the quarter. Total rainfall for the quarter measured 17.37 cm. Note that these values were taken from data that were flagged as 0, as passed initial QAQC checks.

A secondary SWMP meteorological station (MARSCMET) began monitoring on 05/08/2015 on the Ship Channel pier located at the University of Texas Marine Science Institute [27º 50’ 17.23 N, 93º 3’ 0.79 W]. This weather station had been previously set up and monitored by Anthony Amos for several decades; however, the equipment and data had not been maintained following CDMO protocols. Since the Mission Aransas NERR was already monitoring weather in Copano East, it was decided that the Ship Channel MET station be added to the MANERR SWMP and new equipment was purchased. The weather station is located on the top of the pier instrument room at the end of the pier and is set up following standard CDMO protocols. The wind sensor (speed and direction) is mounted at the top of a pole so that the sensor is approximately 9 meters off the water. The PAR sensor is located on a side bar about 0.35 m below the wind sensor and positioned so that no shadows cover the sensor. The temperature, relative humidity, and rain gage sensors are positioned halfway down the pole so that they are just over 7.5 m off the water. The barometric pressure sensor is located inside the Campbell Scientific enclosure approximately 4 m off of the water, which is inside the pier instrument room, and has an intake valve that runs through a hole in the bottom of the enclosure that allows pressure equalization. This site also has a SWMP water quality monitoring station installed on the lower level of the pier.

The lowest air temperature recorded at MARSCMET in the first quarter of 2017 was -2.7 ºC and the highest temperature was 26.9 ºC, with an average temperature of 19.0 ºC. Minimum wind speed recorded was 0.0 meters per second (m/s) and the maximum was 16.0 m/s, with an average of 5.5 m/s. The lowest quarterly reading of relative humidity was 21% and the highest was 100%, with an average of 84% for the quarter. Total rainfall for the quarter measured 10.63 cm. Note that these values were taken from data that were flagged as 0, as passed initial QAQC checks.

6)  Data collection period - Included in annual metadata document

7)  Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2016.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

8)  Associated researchers and projects

As part of the SWMP long-term monitoring program, MAR NERR also monitors 15-minute water quality along with monthly grab samples and diel sampling for nutrient data which may be correlated with this meteorological dataset.  These data are available at www.nerrsdata.org.

The following researchers are currently using water quality data recorded at the SWMP stations in addition to collecting water and/or samples for their own projects:

Lisa Campbell, a research scientist from Texas A & M College Station, monitors an in-situ flowcytometer (FlowCytoBot) to study the phytoplankton at the Ship Channel site in order to detect potential harmful algal blooms (HABs). This work is being conducted in conjunction with Robert Olsen of Woods Hole Oceanographic Institute.

Lindsay Scheef, a post-doc at UTMSI, is using tilt meters to study the currents and water exchange within the bays of the Mission Aransas NERR.

II.  Physical Structure Descriptors

9)  Sensor specifications

Parameter: Temperature

Units: Celsius

Sensor type: PT100 RTD, IEC 751 1/3 Class B, with calibrated signal conditioning

Model #:  HC2-S3 Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.1 °C @ 23°C

Serial number, dates of calibration and use:

Copano East

Serial Number: 61407321

Date of Last Calibration: 06/10/2014

Dates of Sensor Use: 06/10/2015 – 07/22/2015

Serial Number: 20018168

Date of Last Calibration: 04/21/2015

Dates of Sensor Use: 07/22/2015 – 05/25/2016

Serial Number: 20021951

Date of Last Calibration: 05/11/2015

Dates of Sensor Use: 05/25/2016 – 05/31/17

Serial Number: 61247942

Date of Last Calibration: 05/04/2017

Dates of Sensor Use: 05/31/2017 – current as of 06/30/2017

Ship Channel

Serial Number: 61247942

Date of Last Calibration: 02/25/2014

Dates of Sensor Use: 05/08/2015 – 08/23/2016

Serial Number: 20078580

Date of Last Calibration: 11/23/2015

Dates of Sensor Use: 08/23/2016 – current as of 06/30/2017

Parameter: Relative Humidity

Units: Percent

Sensor type: ROTRONIC® Hygromer IN-1

Model #: HC2-S3 Temperature and Relative Humidity Probe

Range: 0-100% non-condensing

Accuracy at 23°C:  +/- 0.8% RH with standard configuration settings

Temperature dependence of RH measurement +/- 3% (-40 to 60C)

Serial number, dates of calibration and use:

Copano East

Serial Number: 61407321

Date of Last Calibration: 06/10/2014

Dates of Sensor Use: 06/10/2015 – 07/22/2015

Serial Number: 20018168

Date of Last Calibration: 04/21/2015

Dates of Sensor Use: 07/22/2015 – 05/25/2016

Serial Number: 20021951

Date of Last Calibration: 05/11/2015

Dates of Sensor Use: 05/25/2016 – 05/31/2017

Serial Number: 61247942

Date of Last Calibration: 05/04/2017

Dates of Sensor Use: 05/31/2017 – current as of 06/30/2017

Ship Channel

Serial Number: 61247942

Date of Last Calibration: 02/25/2014

Dates of Sensor Use: 05/08/2015 – 08/23/2016

Serial Number: 20078580

Date of Last Calibration: 11/23/2015

Dates of Sensor Use: 08/23/2016 – current as of 06/30/2017

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Serial number, dates of calibration and use:

Copano East

Serial Number: A4830003 (CS-105)

Date of Last Calibration: 11/30/2005

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: D1440029 (CS-106)

Date of Last Calibration: 04/04/2008

Dates of Sensor Use: 07/29/2008 – 08/31/2010

Serial Number: A4830003 (CS-105)

Date of Last Calibration: 06/29/2010

Dates of Sensor Use: 08/31/2010 – 10/30/2012

Serial Number: D1440029 (CS-106)

Date of Last Calibration: 09/25/2012

Dates of Sensor Use: 10/30/2012 – 01/06/2015

Serial Number: A4830003 (CS-105)

Date of Last Calibration: 12/09/2014

Dates of Sensor Use: 01/06/2015 – 05/31/2017

Serial Number: D1440029 (CS-106)

Date of Last Calibration: 05/08/2017

Dates of Sensor Use: 05/31/2017 – 06/29/17

Serial Number: A4830003 (CS-105)

Date of Last Calibration: 12/09/2014

Dates of Sensor Use: 06/29/17 – current as of 06/30/2017

Ship Channel

Serial Number: K3840002 (CS-106)

Date of Last Calibration: 09/18/2014

Dates of Sensor Use: 05/08/2015 – current as of 06/30/2017

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Serial number, dates of calibration and use:

Copano East

Serial Number: WM73868

Date of Last Calibration: 07/20/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: WM84419

Date of Last Calibration: 02/21/2008

Dates of Sensor Use: 07/29/2008 – 08/31/2010

Serial Number: WM73868

Date of Last Calibration: 06/14/2010

Dates of Sensor Use: 08/31/2010 – 10/30/2012

Serial Number: WM84419

Date of Last Calibration: 09/19/2012

Dates of Sensor Use: 10/30/2012 – 01/06/2015

Serial Number: WM73868

Date of Last Calibration: 12/12/2014

Dates of Sensor Use: 01/06/2015 – 05/31/2017

Serial Number: WM84419

Date of Last Calibration: 05/05/2017

Dates of Sensor Use: 05/31/2017 – current as of 06/30/2017

Ship Channel

Serial Number: WM138025

Date of Last Calibration: 12/03/2014

Dates of Sensor Use: 05/08/2015 – current as of 06/30/2017

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Serial number, dates of calibration and use:

Copano East

Serial Number: WM73868

Date of Last Calibration: 07/20/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: WM84419

Date of Last Calibration: 02/21/2008

Dates of Sensor Use: 07/29/2008 – 08/31/2010

Serial Number: WM73868

Date of Last Calibration: 06/14/2010

Dates of Sensor Use: 08/31/2010 – 10/30/2012

Serial Number: WM84419

Date of Last Calibration: 09/19/2012

Dates of Sensor Use: 10/30/2012 – 01/06/2015

Serial Number: WM73868

Date of Last Calibration: 12/12/2014

Dates of Sensor Use: 01/06/2015 – 05/31/2017

Serial Number: WM84419

Date of Last Calibration: 05/05/2017

Dates of Sensor Use: 05/31/2017 – current as of 06/30/2017

Ship Channel

Serial Number: WM138025

Date of Last Calibration: 12/03/2014

Dates of Sensor Use: 05/08/2015 – current as of 06/30/2017

Parameter: Photosynthetically Active Radiation (PAR)

Units:  mmoles m-2 (total flux)

Sensor type:  anodized aluminum with cast acrylic diffuser

Model #SQ110 Apogee Quantum Sensor

Light spectrum waveband:  410 to 655 nm

Temperature dependence: 0.06+/-0.06% per °C

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 70°C; Humidity 0 to 100%

Cosine Response: 45° zenith angle: +/- 2%; 75° zenith angle: +/- 5%

Sensitivity:  0.2mV per µmol s-1 m-2

Multiplier: 0.025 (even though this does not change it should still be included)

Serial number, dates of calibration and use:

Copano East

Serial Number: 17015

Date of Last Calibration: 11/01/14

Dates of Sensor Use: 06/10/2015 – 05/31/2017

Serial Number: 23188

Date of Last Calibration: 05/10/2017

Dates of Sensor Use: 05/31/2017 – current as of 06/30/2017

Ship Channel

Serial Number: 17007

Date of Last Calibration: 10/01/2014

Dates of Sensor Use: 05/08/2015 – current as of 06/30/2017

Parameter: Precipitation

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Serial number, dates of calibration and use:

Copano East

Serial Number: 39832-806

Date of Last Calibration: 05/05/2005

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: 45841-208

Date of Last Calibration: 02/26/2008

Dates of Sensor Use: 07/29/2008 – 06/16/2009

Serial Number: 39832-806

Date of Last Calibration: 06/12/2009

Dates of Sensor Use: 06/16/2009 – 08/31/2010

Serial Number: 45841-208

Date of Last Calibration: 08/12/2010

Dates of Sensor Use: 08/31/2010 – 01/19/2012

Serial Number: 39832-806

Date of Last Calibration: 11/11/2011

Dates of Sensor Use: 01/19/2012 – 05/02/2014

Serial Number: 45841-208

Date of Last Calibration: 05/01/2014

Dates of Sensor Use: 05/02/2014 – 10/20/2015

Serial Number: 39832-806

Date of Last Calibration: 10/17/2015

Dates of Sensor Use: 10/20/2015 – 05/31/2017

Serial Number: 61945-1014

Date of Last Calibration: 05/30/2017

Dates of Sensor Use: 05/31/2017 – current as of 06/30/2017

Ship Channel

Serial Number: 61945-1014

Date of Last Calibration: 10/08/2014

Dates of Sensor Use: 05/08/2015 – 08/23/2016

Serial Number: 45841-208

Date of Last Calibration: 08/23/2016

Dates of Sensor Use: 08/23/2016 – current as of 06/30/2017

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Date CR1000 Installed: 11/14/2006

Date CR1000 Calibrated: 07/22/2014

CR1000 Firmware Version (s):  Std.27.04, updated on 08/06/2014

CR1000 Program Version(s):  marcemet_5.6_A_10_19_15.cr1

marcemet_5.6.2_053117.cr1

marcemet_5.6.3_062817.cr1

Ship Channel

Date CR1000 Installed: 05/08/2015

Date CR1000 Calibrated: 12/17/2014

CR1000 Firmware Version (s):  Std.27.04

CR1000 Program Version(s): marscmet_5.5A_52815.cr1

10)  Coded variable definitions

Sampling station:		Sampling site code:	Station code:

Copano East			CE			marcemet

Ship Channel			SC			marscmet

11)  QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument malfunction

GIT	Instrument recording error, recovered telemetry data

GMC	No instrument deployed due to maintenance/calibration

GMT 	Instrument maintenance

GPD	Power down

GPF	Power failure / Low battery

GPR	Program reload

GQR	Data rejected due to QA/QC checks

GSM	See metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC	Incorrect calibration constant, multiplier or offset

SIW	Incorrect wiring

SMT	Sensor maintenance

SNV	Negative value

SOC	Out of calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor drift

SSN	Not a number / unknown value

SSM	Sensor malfunction

SSR	Sensor removed

Comments

CAF	Acceptable calibration/accuracy error of sensor

CCU	Cause unknown

CDF	Data appear to fit conditions

CML	Snow melt from previous snowfall event

CRE*	Significant rain event

CSM*	See metadata

CVT*	Possible vandalism/tampering

CWE*	Significant weather event

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval. These values are automatically flagged and coded as <1> (CAF).

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3% and are flagged and coded as suspect, <1> (CAF).  Values greater than 103 are rejected <-3>.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

Copano East

01/01/17 0:00 – 01/02/17 13:00		Data are missing during this time due to an issue with power which has been an ongoing problem with the WQ data, but has just started to cause MET data loss. We are still trying to determine the cause.

01/01/17 0:00 – 05/12/17 07:30		Directional placement of wind monitor is off due to set screw being loose. WDir, WSpd, SDWDir, and MaxWSpd all flagged suspect. On 5/12/17, the direction of the junction box was at 120° rather than 180°. The junction box was moved back to 180° and the screw was cleaned of corrosion and tightened once again.

01/01/17 0:00 – 05/31/17 12:45		Programming for RH is incorrect. During

program upload in Oct 2015, the following line of code was added to the program

in error:

If RHumidity > 100 and RHumidity < 108 Then RHumidity = 100

01/02/17 13:15 – 01/12/17 14:15		Data had to be retrieved via telemetry and is intermittent due to power loss. Gaps of data are missing during this time as well as all non-standard parameters and CUMPRCP.

02/08/17 11:45				Data were rejected due to missing 5-second data during a power down of the CR1000. Cumulative Precipitation data from 02/08/17 12:00 – 02/09/17 00:00 were not rejected as no precipitation was recorded prior to the power down since the value remained zero.

02/14/17 08:15 – 02/16/17 11:15		Temp/RH sensor started acting wacky. Erroneous data were rejected whereas data that appeared only suspect were flagged as such.

05/31/17 12:15-12:45				Temp/RH flagged suspect since Temp started increasing for no apparent reason.

05/31/17 13:00-13:45				Data are missing during this time due to a power down for maintenance. The following MET equipment were swapped: Temp/RH (s/n 61247942), Apogee PAR (s/n 23188), Wind (s/n WM84419), Rain gage (s/n 61945-1014), BP (CS-106, s/n D1440029). Data at 14:00 were rejected due to missing 5-second data during program upload. Cumulative Precipitation data from 05/31/17 14:15 – 06/01/17 00:00 were not rejected as no precipitation was recorded prior to the power down since the value remained zero.

05/31/17 14:00 – 06/09/17 09:45		BP was wired incorrectly during this time; therefore, readings were incorrect and had to be rejected. In addition, the Temp/RH started rising and had intermittent issues throughout the deployment. Erroneous Temp/RH data were rejected and all other Temp/RH data which appeared normal were flagged suspect.

06/09/17 10:00-10:15				Station powered down to try and determine the cause of the issue with the BP sensor. Data at 10:30 were rejected due to missing 5-second data during program upload. Cumulative Precipitation data from 06/09/17 14:15 – 06/10/17 00:00 were not rejected as no precipitation was recorded prior to the power down since the value remained zero.

06/09/17 10:30 – 06/22/17 10:00		BP sensor still not working correctly so those data were rejected. Temp/RH data had intermittent high readings which were rejected; all other Temp/RH data were flagged suspect after the malfunction began.