Mission Aransas (MAR) NERR Meteorological Metadata

January – December 2014

Latest Update:  12/05/2016

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons:

Address: 	Mission Aransas National Estuarine Research Reserve

750 Channelview Drive

Port Aransas, TX 78373

Phone:  (361)749-6711

Fax:	 (361)749-6777

Contact Persons:

Dr. Edward Buskey, Research Program Coordinator

E-mail: ed.buskey@utexas.edu; (361)749-3102

Sally Morehead, Manager

E-mail: sally.morehead@utexas.edu; (361)749-3046

Cammie Hyatt, SWMP Data Manager & Technician

E-mail: cammie.hyatt@utexas.edu; (361)749-3093

Rae Mooney, SWMP Technician, Boat Operations

E-mail: rmooney@utexas.edu; (361)749-3101

2)  Entry verification:

Prior to 04/17/07, data were uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12.

On 04/17/07, a Campbell CFM100 was installed on the CR1000 and code was uploaded via a laptop to direct the CR1000 to start sending data to the compact flash card within the CFM100. Once a month, the compact flash card is swapped out with a blank card and the full card is taken back to the lab where the file on it is converted using Loggernet. The resulting comma-delimited format (.CSV) file is then uploaded to the CDMO and handled in the same manner as listed above. Cammie Hyatt is responsible for data management.

3)  Research objectives:

Five long-term monitoring stations have been established along the Texas Coast in the Mission-Aransas estuary to collect baseline data on environmental conditions within the bay systems. These System Wide Monitoring Program Stations (SWMPs) provide continual monitoring of temperature, salinity, oxygen and other parameters that characterize the water quality of the bays. In addition to the water quality stations, one meteorological station has been installed in Copano Bay (east) to collect data on air temperature, relative humidity, barometric pressure, wind speed, wind direction, precipitation, and solar radiation (PAR). The goal of the SWMP meteorological station is to develop quantitative measurements of short-term variability and long-term changes in the weather within the Mission Aransas NERR. This data will help us determine climactic and hydrological patterns that influence freshwater inflow in the Mission- Aransas Estuary and will be used to help coastal managers make informed decisions on local and regional issues, such as “no discharge” zones for boats and measuring the success of restoration projects. To the best of my knowledge, this is the only water-based weather station within the NERR.

4)  Research methods:

Campbell Scientific data telemetry equipment was installed at the (MARCEMET) station on 11/04/06 and transmits data to the NOAA GOES satellite, NESDIS ID #3B039516.  (Where 3B039516 is the GOES ID for that particular station.)  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts)

Maximum and Minimum Air Temperature (°C) and their times from

5-second data (these data are available from the Reserve)

Maximum Wind Speed (m/s) and time from 5-second data

Wind Direction Standard Deviation (degrees)

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm)

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- yearly recalibration

- Rain Gauge- yearly recalibration

- Wind Speed/Direction- yearly or every 2 years (depending on the sensor)

- Barometric Pressure- every 2 years recalibration

- PAR- every 2 years recalibration

- CR1000-every 5 years (required beginning 2014, one year initial grace period)

5)  Site location and character:

The MANERR meteorological station (MARCEMET) is located in open water in east Copano Bay near the Copano Causeway at 28º 7’ 56.28 N, 97º 2’ 3.84 W. The weather station is located on a platform that is 3.5 meters above the water (at the high water mark). Wind (speed and direction), temperature, relative humidity, and PAR sensors are mounted onto a 3 meter aluminum tower so that the sensors stand approximately 4 meters off the platform. The PAR sensor is located on one side of the 1.2 meter sensor crossarm, with the wind monitor on the other side. The rain gauge is located on the arm rail of the platform which is approximately 1.5 meters off the platform. The barometric pressure sensor is located inside the Campbell Scientific enclosure, which is 1.1 m off the platform, and has an intake valve that runs through a hole in the bottom of the enclosure that allows pressure equalization. To the best of my knowledge, all the weather instrumentation and associated sensors are set up according to standard CDMO protocols. There are no obstructions around the platform; therefore, the tower does not receive shade and it is not blocked from the wind. This site also has a SWMP water quality monitoring station installed on the lower level of the platform.

Major climactic influences within the MANERR are temperature, precipitation, evaporation, wind, tropical storms, and hurricanes. The lowest yearly air temperature recorded at MARCEMET was -0.9 ºC and the highest temperature was 32.4 ºC, with an average temperature of 20.6 ºC. Minimum wind speed recorded was zero meters per second (m/s) and the maximum was 17.4 m/s, with an average of 5.9 m/s. The lowest yearly reading of relative humidity was 23% and the highest was 100%, with an average of 79% for the year. Total rainfall for the year measured 56.9 cm.

6)  Data collection period:

Data collection at the MARCEMET began in November 2006. Data was initially recorded onto the CR1000 and was downloaded via laptop as indicated in Section 2.

Once the CFM100 was installed, the flash card holding the data was swapped out with a clean card approximately once a month. If time and weather permitted, card was swapped more frequently. The starting and ending times of those flash card files are listed as follows:

12/16/13 12:30 - 01/10/14 10:15

01/10/14 10:30 – 02/20/14 12:00

02/20/14 12:15 – 03/05/14 11:30

03/05/14 11:45 – 03/19/14 10:30

03/19/14 10:45 – 04/01/14 11:00

04/01/14 11:15 – 04/16/14 10:00

04/16/14 10:15 – 05/02/14 10:00

05/02/14 11:30 – 05/15/14 10:30

05/15/14 10:45 – 05/29/14 10:00

05/29/14 10:15 – 06/11/14 10:00

06/11/14 10:15 – 06/24/14 10:30

06/24/14 10:45 – 07/08/14 10:30

07/08/14 10:45 – 07/15/14 11:30

07/15/14 12:00 – 08/06/14 10:15

08/06/14 11:00 – 08/19/14 09:30

08/19/14 09:45 – 09/02/14 10:45

09/02/14 11:00 – 09/17/14 09:45

09/17/14 10:00 – 10/01/14 09:30

10/01/14 09:45 – 10/15/14 09:30

10/15/14 09:45 – 10/28/14 09:15

10/28/14 09:30 – 11/19/14 10:45

11/19/14 11:00 – 12/09/14 10:45

12/09/14 11:00 – 01/06/15 10:30

7)  Distribution:

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2012.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

8)  Associated researchers and projects:

The MANERR water quality monitoring project has been incorporated into the Texas Coastal Oceanic Observation Network (TCOON).

In addition to the SWMP Meteorological data that are collected, MANERR also has five SWMP Water Quality stations that collect Water Temperature, Specific Conductivity, Salinity, Dissolved Oxygen (% and mg/L), Depth (or Level, depending on the site), pH, Turbidity, and in situ Chlorophyll. Water is also collected at each of the SWMP sites for nutrient analysis to determine the amount of chlorophyll a, orthophosphate, ammonium, and nitrogen (nitrite, nitrate, or nitrite + nitrate) that is present. Whole water samples are collected on each sampling trip and fixed with Lugol’s iodine for enumeration of microzooplankton and mesozooplankton samples are collected once per month and preserved with 5% formaldehyde for enumeration of plankton abundance.

Lisa Campbell, a research scientist from Texas A & M College Station, has installed an in-situ flowcytometer (FlowCytoBot) to study the phytoplankton at the Ship Channel site in order to detect potential harmful algal blooms (HABs). This work is being conducted in conjunction with Robert Olsen of Woods Hole Oceanographic Institute.

The following researchers are currently using water quality data recorded at the SWMP stations in addition to collecting water and/or samples for their own projects:

Rodrigo Almeda, a post-doc at UTMSI, used zooplankton collected within the reserve to conduct experiments that will help us to better understand the effects of oil on zooplankton.

Colleen McCue, the Volunteer Coordinator for MANERR, is supervising a project to enumerate blue crab megalope abundance coming into the estuary. This project relies on volunteers to collect samples daily at Horace Caldwell pier and the Ship Channel, in addition to a site in Rockport and Aransas Pass.

Lindsay Scheef, a post-doc at UTMSI, is using tilt meters to study the currents and water exchange within the bays.

Summer REU (Research Experience for Undergraduates) Students collected samples within the Mission Aransas NERR for independent research projects:

Michele Mei, Tufts University, Biology & Computer Science, studied the effects of crude oil ingestion on copepod escape behavior.

Claire Weirich,  Texas Lutheran University, Environmental Biology & Math, studied currents and tides in the Ship Channel and how they affect settlement of blue crab larvae.

II.  Physical Structure Descriptors

9)  Sensor specifications:

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #:  HMP45C Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Serial number, dates of calibration and use:

Serial Number: B3230004

Date of Calibration: 08/09/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: D2020113

Date of Last Calibration: 05/27/2008

Dates of Sensor Use: 07/29/2008 – 06/16/2009

Serial Number: B3230004

Date of Calibration: 06/12/2009

Dates of Sensor Use: 06/16/2009 – 08/31/2010

Serial Number: D2020113

Date of Last Calibration: 06/21/2010

Dates of Sensor Use: 08/31/2010 – 01/19/2012

Serial Number: B3230004

Date of Calibration: 08/25/2011

Dates of Sensor Use: 01/19/2012 – 10/30/2012

Serial Number: D2020113

Date of Last Calibration: 09/14/2012

Dates of Sensor Use: 10/30/2012 – 05/02/2014

Serial Number: B3230004

Date of Calibration: 04/17/2014

Dates of Sensor Use: 05/02/2014 – 11/19/2014

Serial Number: D2020113

Date of Last Calibration: 11/07/2014

Dates of Sensor Use: 11/19/2014 – current as of 12/31/2014

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #:  HMP45C Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Serial number, dates of calibration and use:

Serial Number: B3230004

Date of Calibration: 08/09/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: D2020113

Date of Last Calibration: 05/27/2008

Dates of Sensor Use: 07/29/2008 – 06/16/2009

Serial Number: B3230004

Date of Calibration: 06/12/2009

Dates of Sensor Use: 06/16/2009 – 08/31/2010

Serial Number: D2020113

Date of Last Calibration: 06/21/2010

Dates of Sensor Use: 08/31/2010 – 01/19/2012

Serial Number: B3230004

Date of Calibration: 08/25/2011

Dates of Sensor Use: 01/19/2012 – 10/30/2012

Serial Number: D2020113

Date of Last Calibration: 09/14/2012

Dates of Sensor Use: 10/30/2012 – 05/02/2014

Serial Number: B3230004

Date of Calibration: 04/17/2014

Dates of Sensor Use: 05/02/2014 – 11/19/2014

Serial Number: D2020113

Date of Last Calibration: 11/07/2014

Dates of Sensor Use: 11/19/2014 – current as of 12/31/2014

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105, CS-106

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Serial number, dates of calibration and use:

Serial Number: A4830003 (CS-105)

Date of Last Calibration: 11/30/2005

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: D1440029 (CS-106)

Date of Last Calibration: 04/04/2008

Dates of Sensor Use: 07/29/2008 – 08/31/2010

Serial Number: A4830003 (CS-105)

Date of Last Calibration: 06/29/2010

Dates of Sensor Use: 08/31/2010 – 10/30/2012

Serial Number: D1440029 (CS-106)

Date of Last Calibration: 09/25/2012

Dates of Sensor Use: 10/30/2012 – current as of 12/31/2014

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Serial number, dates of calibration and use:

Serial Number: WM73868

Date of Last Calibration: 07/20/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: WM84419

Date of Last Calibration: 02/21/2008

Dates of Sensor Use: 07/29/2008 – 08/31/2010

Serial Number: WM73868

Date of Last Calibration: 06/14/2010

Dates of Sensor Use: 08/31/2010 – 10/30/2012

Serial Number: WM84419

Date of Last Calibration: 09/19/2012

Dates of Sensor Use: 10/30/2012 – current as of 12/31/2014

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Serial number, dates of calibration and use:

Serial Number: WM73868

Date of Last Calibration: 07/20/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: WM84419

Date of Last Calibration: 02/21/2008

Dates of Sensor Use: 07/29/2008 – 08/31/2010

Serial Number: WM73868

Date of Last Calibration: 06/14/2010

Dates of Sensor Use: 08/31/2010 – 10/30/2012

Serial Number: WM84419

Date of Last Calibration: 09/19/2012

Dates of Sensor Use: 10/30/2012 – current as of 12/31/2014

Parameter: Photosynthetically Active Radiation(PAR)

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SB

Light spectrum waveband:  400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity: 0 to 100%

Sensitivity:  typically 5 µA per 1000 µmoles s-1 m-2

Multiplier:

1.29 changed 11/04/2005

1.26 changed 07/29/2008

1.25 changed 06/16/2009

1.47 changed 08/31/2010

1.22 changed 05/04/2011

1.27 changed 07/26/2011

1.23 changed 01/19/2012

1.11 changed 07/08/2014

Serial number, dates of calibration and use:

Serial Number: Q35626

Date of Last Calibration: 11/04/2005

Dates of Sensor Use: 11/04/2006 – 07/29/2008

Serial Number: Q38890

Date of Last Calibration: 11/20/2007

Dates of Sensor Use: 07/29/2008 – 06/16/2009

Serial Number: Q35626

Date of Last Calibration: 05/22/2009

Dates of Sensor Use: 06/16/2009 – 08/31/2010

Serial Number: Q38890

Date of Last Calibration: 06/17/2010

Dates of Sensor Use: 08/31/2010 – 05/04/2011

Serial Number: Q35626

Date of Last Calibration:

Dates of Sensor Use: 05/04/2011 – 07/26/2011

Serial Number: Q45313

Date of Last Calibration: 04/26/2011

Dates of Sensor Use: 07/26/2011 – 01/19/2012

Serial Number: Q43906

Date of Last Calibration: 08/24/2011

Dates of Sensor Use: 01/19/2012 – 05/02/2014

Serial Number: Q45313

Date of Last Calibration: 04/18/2014

Dates of Sensor Use: 05/02/2014 - current as of 12/31/2014

Parameter: Precipitation (not heated)

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Serial number, dates of calibration and use:

Serial Number: 39832-806

Date of Last Calibration: 05/05/2005

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: 45841-208

Date of Last Calibration: 02/26/2008

Dates of Sensor Use: 07/29/2008 – 06/16/2009

Serial Number: 39832-806

Date of Last Calibration: 06/12/2009

Dates of Sensor Use: 06/16/2009 – 08/31/2010

Serial Number: 45841-208

Date of Last Calibration: 08/12/2010

Dates of Sensor Use: 08/31/2010 – 01/19/2012

Serial Number: 39832-806

Date of Last Calibration: 11/11/2011

Dates of Sensor Use: 01/19/2012 – 05/02/2014

Serial Number: 45841-208

Date of Last Calibration: 05/01/2014

Dates of Sensor Use: 05/02/2014 - current as of 12/31/2014

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Date CR1000 Installed: 11/14/2006 (initially), 08/06/2014 (after calibration)

Date CR1000 Calibrated: 07/22/2014

CR1000 Firmware Version (s):  Std.27.04, updated on 08/06/2014

CR1000 Program Version(s):  marcemet_5.6_102912.cr1, marcemet_5.6_071414.cr1

10)  Coded variable definitions - List the sampling station, sampling site code, and station code used in the data.

Sampling station:		Sampling site code:	Station code:

Copano East			CE				marcemet

11)  QAQC flag definitions:

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported Parameter

0	Passed Initial QAQC Checks

1	Suspect Data

2	Open - reserved for later flag

3	Open - reserved for later flag

4	Historical Data:  Pre-Auto QAQC

5	Corrected Data

12)  QAQC code definitions:

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument Malfunction

GIT	Instrument Recording Error, Recovered Telemetry Data

GMC	No Instrument Deployed due to Maintenance/Calibration

GMT 	Instrument Maintenance

GPD	Power Down

GPF	Power Failure / Low Battery

GPR	Program Reload

GQR	Data Rejected Due to QA/QC Checks

GSM	See Metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC		Incorrect Calibration Constant, Multiplier or Offset

SIW	Incorrect Wiring

SMT	Sensor Maintenance

SNV	Negative Value

SOC	Out of Calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor Drift

SSN	Not a Number / Unknown Value

SSM	Sensor Malfunction

SSR		Sensor Removed

Comments

CAF	Acceptable Calibration/Accuracy Error of Sensor

CCU	Cause Unknown

CDF	Data Appear to Fit Conditions

CML	Snow melt from previous snowfall event

CRE*	Significant Rain Event

CSM*	See Metadata

CVT*	Possible Vandalism/Tampering

CWE*	Significant weather event

13)  Other remarks/notes:

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

There were many nights during the year in which the nighttime PAR readings exceeded the noise error of the LICOR (+/- 2.214) and were rejected. I am unsure as to the reason for the higher readings.  The occurrence of high nighttime readings has been an issue with our sensors for several years and I feel that this issue has to do with moisture permeating the underside of the sensor where the screws join the sensor; however, we installed a new PAR sensor on 01/19/12 that had silicone sealant on the bottom screws of the sensor to prevent water inundation and we are still seeing nighttime readings and have been unable to determine the cause. See below for specifics with PAR data flagging.

- All PAR data are coded with CSM, unless otherwise coded, so that users are more aware of PAR issues occurring during 2016.

- Elevated nighttime readings <2.214mmolsm-2 are flagged and coded as suspect, <1> CSM.

- Higher than expected daytime PAR values are considered suspect and flagged and coded as <1> CSM.

- Elevated nighttime readings >2.214mmolesm-2 are flagged and coded as rejected, <-3>CSM.

- On days where the previous nighttime readings were > 30 mmoles m-2, the daytime PAR readings have been marked as suspect.

- Negative PAR readings beyond the noise error of the LICOR are flagged and coded <-3> CSM. This may be due to water inundation since it appears more prevalent during rain events or days when it is especially foggy.

- Beginning on 10/29/2014 and lasting through 11/19/2014 much higher elevated nighttime readings than are normal and NAN readings were recorded at this site.  These values are flagged and coded as <-3> CSM.

- 05/02/14 11:30 – 07/15/14 11:30 PAR data were corrected during this time period by taking the readings and dividing by the old PAR multiplier (1.23004) and then multiplying the results of that by the new multiplier (1.11265).  Elevated nighttime suspect and slightly negative suspect data are included in the <5> flagging since corrected data are inherently suspect.

All RH data for 2014 are coded with CSM so that the user will be aware that there were sensor problems during the year.  There were many occasions during 2014 when RH data exceeded the maximum range of the sensor or NAN was recorded.  Values >103 or NAN were rejected as <-3> (CSM).  While values within the range of the sensor but >100 were flagged as suspect <-1> CSM.  In some instances, this coincided with periods of time when it was either raining or foggy (compared to local weather station); however, it got to where it was reading NANs quite a bit prior to the sensor swap on 05/02/14. Perhaps there was some moisture trapped in the tip that wasn’t detected during the monthly maintenance checks. Once the newly calibrated sensor was in place readings were more stable until the ATemp/RH sensor began having even more issues between 09/12/14 12:15 – 11/19/14 11:00 (see below).

01/19/14 00:00 - 05/02/2014 10:00 Total and cumulative precipitation are flagged and coded as <1> SOC CSM due to the precipitation gauge being out of calibration.

05/02/14 10:15 – 11:30			Swapped Temp/RH, PAR, and Rain gauge sensors; however, we didn’t change the PAR multiplier in the program because there was some confusion about the calibration of the sensor with respect to the mV adapter. Data from 10:15 - 11:15 are missing due to the station being powered down.  Data at 11:30 are rejected since they are more than likely not a full 15 minutes of 5-second data following the power down.

07/15/14 11:45 - 13:15				MET station re-grounded; swapped CR1000 borrowed from Campbell Scientific (s/n 62514) and uploaded program with correct PAR sensor multiplier. First readings right after the swap were rejected.

07/23/14 10:00				Rain gauge clogged with bird poop; rinsed funnel and tipped buckets to clean. Precipitation and CumPrcp (through the end of the day) readings corrected to 0.

08/06/14 10:30-11:00 				Swapped our newly-calibrated CR1000 for the Campbell loaner that was installed on 07/15/14; uploaded the program.

09/12/14 12:15 – 11/19/14 11:00		Temp/RH sensor having intermittent issues. By 10/30/14 the sensor started reading low negative air temperature numbers intermittently.  Data flagged suspect or rejected based on data comparison with weather from airport near station. If data were 10% lower they were flagged suspect; readings >10% were rejected. NAN readings were also rejected.  First data point from newly calibrated sensor was rejected following standard protocol. Contact the Reserve for more information about the data that was used for comparisons.

09/17/14 10:00				Rain gauge clogged with bird poop; rinsed funnel and tipped buckets to clean. Precipitation and CumPrcp (corrected through the end of the day to reflect actual precipitation for the day).

11/19/14 10:50				Swapped out Temp/RH sensor. Data were rejected at 11:00.

F_Record {CRE} precipitation events (>1inch). :

05/13/2014 02:45 - 11:30

06/26/2014 06:30 - 11:00

08/02/2014 02:15 - 04:15