Mission Aransas (MAR) NERR Meteorological Metadata

January – December 2015

Latest Update: 08/08/2017

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons:

Address: 	Mission Aransas National Estuarine Research Reserve

750 Channelview Drive

Port Aransas, TX 78373

Phone:  (361)749-6711

Fax:	 (361)749-6777

Contact Persons:

Dr. Edward Buskey, Research Program Coordinator

E-mail: ed.buskey@utexas.edu; (361)749-3102

Jace Tunnell, Manager

E-mail: jace.tunnell@austin.utexas.edu; (361)749-3046

Cammie Hyatt, SWMP Data Manager & Technician

E-mail: cammie.hyatt@utexas.edu; (361)749-3093

Collin Croulet, SWMP Technician, Boat Operations

E-mail: collin.croulet@austin.utexas.edu; (361)749-3101

2)  Entry verification

Data are uploaded from the CR1000 data logger to a Personal Computer (IBM compatible).  Files are exported from or LoggerNet in a comma-delimited format and uploaded to the CDMO where they undergo automated primary QAQC and become part of the CDMO’s online provisional database.  During primary QAQC, data are flagged if they are missing or out of sensor range.  The edited file is then returned to the Reserve where it is opened in Microsoft Excel and processed using the CDMO’s NERRQAQC Excel macro.  The macro inserts station codes, creates metadata worksheets for flagged data and summary statistics, and graphs the data for review.  It allows the user to apply QAQC flags and codes to the data, append files, and export the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.  For more information on QAQC flags and QAQC codes, see Sections 11 and 12. Cammie Hyatt is responsible for data management

3)  Research objectives

Five long-term monitoring stations have been established along the Texas Coast in the Mission-Aransas estuary to collect baseline data on environmental conditions within the bay systems. These System Wide Monitoring Program Stations (SWMPs) provide continual monitoring of temperature, salinity, oxygen and other parameters that characterize the water quality of the bays. In addition to the water quality stations, one meteorological station has been installed in Copano Bay (east) as well as a secondary SWMP meteorological station at Ship Channel to collect data on air temperature, relative humidity, barometric pressure, wind speed, wind direction, precipitation, and solar radiation (PAR). The goal of the SWMP meteorological station is to develop quantitative measurements of short-term variability and long-term changes in the weather within the Mission Aransas NERR. This data will help us determine climactic and hydrological patterns that influence freshwater inflow in the Mission- Aransas Estuary and will be used to help coastal managers make informed decisions on local and regional issues, such as “no discharge” zones for boats and measuring the success of restoration projects. To the best of my knowledge, this is the only water-based weather station within the NERR.

4)  Research methods

Campbell Scientific data telemetry equipment was installed at the (MARCEMET) station on 11/04/06 and transmits data to the NOAA GOES satellite, NESDIS ID #3B039516.  The NESDIS ID for our Ship Channel station (MARSCMET) is #3B05A042.  The transmissions are scheduled hourly and contain four (4) data sets reflecting fifteen minute data sampling intervals.  Upon receipt by the CDMO, the data undergoes the same automated primary QAQC process detailed in Section 2 above.  The “real-time” telemetry data become part of the provisional dataset until undergoing secondary and tertiary QAQC and assimilation in the CDMO’s authoritative online database. The CDMO does not authenticate the data from the Ship Channel station since it is a secondary site in the NERRS SWMP. Only primary SWMP sites go through tertiary review by the CDMO.  Provisional and authoritative data are available at http://cdmo.baruch.sc.edu.

The 15 minute Data are collected in the following formats for the CR1000:

Averages from 5-second data:

Air Temperature (°C), Relative Humidity (%), Barometric Pressure (mb), Wind Speed (m/s), Wind Direction (degrees), Battery Voltage (volts)

Maximum and Minimum Air Temperature (°C) and their times from 5-second data (these data are available from the Reserve)

Maximum Wind Speed (m/s) and time from 5-second data

Wind Direction Standard Deviation (degrees)

Totals:

Precipitation (mm), PAR (millimoles/m2), and Cumulative Precipitation (mm)

Recommended calibration frequency for the MET station sensors:

- Temperature/Humidity- yearly recalibration

- Rain Gauge- yearly recalibration

- Wind Speed/Direction- yearly or every 2 years (depending on the sensor)

- Barometric Pressure- every 2 years recalibration

- PAR- every 2 years recalibration

- CR1000-every 5 years (required beginning 2014, one year initial grace period)

5)  Site location and character

The MANERR meteorological station (MARCEMET) is located in open water in east Copano Bay near the Copano Causeway at 28º 7’ 56.28 N, 97º 2’ 3.84 W. The weather station is located on a platform that is 3.5 meters above the water (at the high water mark). Wind (speed and direction), temperature, relative humidity, and PAR sensors are mounted onto a 3 meter aluminum tower so that the sensors stand approximately 4 meters off the platform. The PAR sensor is located on one side of the 1.2 meter sensor crossarm, with the wind monitor on the other side. The rain gauge is located on the arm rail of the platform which is approximately 1.5 meters off the platform. The barometric pressure sensor is located inside the Campbell Scientific enclosure, which is 1.1 m off the platform, and has an intake valve that runs through a hole in the bottom of the enclosure that allows pressure equalization. To the best of my knowledge, all the weather instrumentation and associated sensors are set up according to standard CDMO protocols. There are no obstructions around the platform; therefore, the tower does not receive shade and it is not blocked from the wind. This site also has a SWMP water quality monitoring station installed on the lower level of the platform.

Major climactic influences within the MANERR are temperature, precipitation, evaporation, wind, tropical storms, and hurricanes. The lowest air temperature recorded at MARCEMET in 2015 was 0.2 ºC and the highest temperature was 33.6 ºC, with an average temperature of 21.3 ºC. Minimum wind speed recorded was zero meters per second (m/s) and the maximum was 17.2 m/s, with an average of 4.2 m/s. The lowest reading of relative humidity was 32% and the highest was 100%, with an average of 80% for the year. Total rainfall for 2015 measured 103.15 cm.  Note that these values were taken from data that were flagged as 0, as passed initial QAQC checks.

A secondary SWMP meteorological station (MARSCMET) began monitoring on 05/08/2015 on the Ship Channel pier located at the University of Texas Marine Science Institute [27º 50’ 17” N, 93º 3’ 1” W]. This weather station had been previously set up and monitored by Anthony Amos for several decades; however, the equipment and data had not been maintained following CDMO protocols. Since the Mission Aransas NERR was already monitoring weather in Copano East, it was decided that the Ship Channel MET station be added to the MANERR SWMP and new equipment was purchased. The weather station is located on the top of the pier instrument room at the end of the pier and is set up following standard CDMO protocols. The wind sensor (speed and direction) is mounted at the top of a pole so that the sensor is approximately 9 meters off the water. The PAR sensor is located on a side bar about 0.35 m below the wind sensor and positioned so that no shadows cover the sensor. The temperature, relative humidity, and rain gage sensors are positioned halfway down the pole so that they are just over 7.5 m off the water. The barometric pressure sensor is located inside the Campbell Scientific enclosure approximately 4m off of the water, which is inside the pier instrument room, and has an intake valve that runs through a hole in the bottom of the enclosure that allows pressure equalization. This site also has a SWMP water quality monitoring station installed on the lower level of the pier.

As of 05/08/2015, the lowest air temperature recorded at MARSCMET in 2015 was 6.6 ºC and the highest temperature was 31.0 ºC, with an average temperature of 25.4 ºC. Minimum wind speed recorded was zero meters per second (m/s) and the maximum was 17.8 m/s, with an average of 5.7 m/s. The lowest yearly reading of relative humidity was 29% and the highest was 100%, with an average of 82% for the year. Total rainfall for 2015 measured 55.61 cm. Note that these values were taken from data that were flagged as 0, as passed initial QAQC checks.

6)  Data collection period

Data collection at the MARCEMET began in November 2006. In 2015, data was initially recorded onto a compact flash card inserted into the CFM 100. The flash card holding the data was swapped with a newly formatted card approximately once per month. If time and weather permitted, card was swapped more frequently. In July, issues with the CFM 100 were encountered and the files from 07/22/15 – 09/14/15 04:30 had to be retrieved via telemetry since files on the flash card were deemed “corrupted.” On 10/15/15, the CR1000 program was changed to allow download of data onto the laptop; however, the record table was missing from the program and data were lost. The correct program was uploaded on 10/20/15. At the next data retrieval, a large file was recovered that contained all of the original data from 07/15/15 – 10/15/15. The starting and ending times of all data files are listed as follows:

Copano East

12/09/14 11:00 - 01/06/15 10:30

01/06/15 11:30 – 02/04/15 13:30

02/04/15 13:45 – 03/12/15 08:45

03/12/15 09:00 – 03/25/15 10:15

03/25/15 10:30 – 04/10/15 08:15

04/10/15 10:30 – 04/30/15 08:15

04/30/15 10:30 – 05/20/15 10:15

05/20/15 10:30 – 06/04/15 09:15

06/04/15 09:15 – 06/10/15 09:00

06/10/15 10:45 – 06/23/15 09:45

06/23/15 10:00 – 07/09/15 10:00

07/09/15 10:15 – 07/22/15 10:15

07/15/15 15:45 – 10/15/15 09:45

10/20/15 11:45 – 10/29/15 10:00

10/29/15 10:15 – 11/18/15 11:00

11/18/15 11:15 – 12/14/15 11:00

12/14/15 11:15 – 01/14/16 10:45

Data collection for MARSCMET began on 05/08/15.

Ship Channel

05/08/15 10:00 – 05/18/15 10:00

05/18/15 10:15 – 05/29/15 10:30

05/29/15 14:30 – 06/01/15 12:45

06/01/15 13:00 – 06/02/15 12:45

06/02/15 13:00 – 06/16/15 14:30

06/16/15 14:45 – 07/06/15 12:45

07/07/15 09:45 – 07/14/15 09:00

07/14/15 09:15 – 07/14/15 09:30

07/14/15 09:45 – 07/24/15 08:15

07/24/15 08:30 – 08/13/15 10:00

08/13/15 10:15 – 08/26/15 10:00

08/26/15 10:15 – 09/01/15 09:45

09/01/15 10:00 – 09/21/15 09:15

09/21/15 09:30 – 10/05/15 12:45

10/05/15 13:00 – 10/26/15 09:30

10/26/15 09:45 – 11/10/15 13:00

11/10/15 13:15 – 11/11/15 13:45

11/13/15 14:15 – 12/01/15 11:15

12/01/15 13:00 – 01/05/15 13:45

7)  Distribution

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement.  The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

NOAA National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: http://www.nerrsdata.org/; accessed 12 October 2012.

NERR meteorological data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal Investigators and Contact Persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma delimited format.

8)  Associated researchers and projects

In addition to the SWMP Meteorological data and secondary SWMP meteorological data that are collected, MANERR also has five SWMP Water Quality stations that collect Water Temperature, Specific Conductivity, Salinity, Dissolved Oxygen (% and mg/L), Depth (or Level, depending on the site), pH, Turbidity, and in situ Chlorophyll. Water is also collected at each of the SWMP sites for nutrient analysis to determine the amount of chlorophyll a, orthophosphate, ammonium, and nitrogen (nitrite, nitrate, or nitrite + nitrate) that is present. Whole water samples are collected on each sampling trip and fixed with Lugol’s iodine for enumeration of microzooplankton and mesozooplankton samples are collected once per month and preserved with 5% formaldehyde for enumeration of plankton abundance.

Lisa Campbell, a research scientist from Texas A & M College Station, has an in-situ flowcytometer (FlowCytoBot) installed on the end of the UTMSI pier to study the phytoplankton at this site in order to detect potential harmful algal blooms (HABs). This work is being conducted in conjunction with Robert Olsen of Woods Hole Oceanographic Institute.

The following researchers are currently using water quality data recorded at the SWMP stations in addition to collecting water and/or samples for their own projects:

Nicole Pringle, the Volunteer Coordinator for MANERR, is wrapping up a Citizen Science project designed to enumerate blue crab megalope abundance coming into the estuary. This project relies on volunteers to collect samples daily at the Ship Channel.

Lindsay Scheef, a post-doc at UTMSI, is using tilt meters to study the currents and water exchange within the bays.

Chris Payne, an REU (Research Experience for Undergraduates) student collected samples within the Mission Aransas NERR to study the effects of oil and dispersant on plankton communities.

II.  Physical Structure Descriptors

9)  Sensor specifications

Parameter: Temperature

Units: Celsius

Sensor type: Platinum resistance temperature detector (PRT)

Model #:  HMP45C Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.2 °C @ 20°C

Serial number, dates of calibration and use:

Copano East

Serial Number: B3230004

Date of Calibration: 08/09/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: D2020113

Date of Last Calibration: 05/27/2008

Dates of Sensor Use: 07/29/2008 – 06/16/2009

Serial Number: B3230004

Date of Calibration: 06/12/2009

Dates of Sensor Use: 06/16/2009 – 08/31/2010

Serial Number: D2020113

Date of Last Calibration: 06/21/2010

Dates of Sensor Use: 08/31/2010 – 01/19/2012

Serial Number: B3230004

Date of Calibration: 08/25/2011

Dates of Sensor Use: 01/19/2012 – 10/30/2012

Serial Number: D2020113

Date of Last Calibration: 09/14/2012

Dates of Sensor Use: 10/30/2012 – 05/02/2014

Serial Number: B3230004

Date of Calibration: 04/17/2014

Dates of Sensor Use: 05/02/2014 – 11/19/2014

Serial Number: D2020113

Date of Last Calibration: 11/07/2014

Dates of Sensor Use: 11/19/2014 – 06/10/2015

Parameter: Relative Humidity

Units: Percent

Sensor type: Vaisala HUMICAP© 180 capacitive relative humidity sensor

Model #:  HMP45C Temperature and Relative Humidity Probe

Range:  0-100% non-condensing

Accuracy at 20°C:  +/- 2% RH (0-90%) and +/- 3% (90-100%)

Temperature dependence of RH measurement: +/- 0.05% RH/°C

Serial number, dates of calibration and use:

Copano East

Serial Number: B3230004

Date of Calibration: 08/09/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: D2020113

Date of Last Calibration: 05/27/2008

Dates of Sensor Use: 07/29/2008 – 06/16/2009

Serial Number: B3230004

Date of Calibration: 06/12/2009

Dates of Sensor Use: 06/16/2009 – 08/31/2010

Serial Number: D2020113

Date of Last Calibration: 06/21/2010

Dates of Sensor Use: 08/31/2010 – 01/19/2012

Serial Number: B3230004

Date of Calibration: 08/25/2011

Dates of Sensor Use: 01/19/2012 – 10/30/2012

Serial Number: D2020113

Date of Last Calibration: 09/14/2012

Dates of Sensor Use: 10/30/2012 – 05/02/2014

Serial Number: B3230004

Date of Calibration: 04/17/2014

Dates of Sensor Use: 05/02/2014 – 11/19/2014

Serial Number: D2020113

Date of Last Calibration: 11/07/2014

Dates of Sensor Use: 11/19/2014 – 06/10/2015

Parameter: Temperature

Units: Celsius

Sensor type: PT100 RTD, IEC 751 1/3 Class B, with calibrated signal conditioning

Model #:  HC2-S3 Temperature and Relative Humidity Probe

Operating Temperature:  -40°C to +60°C

Range: -40°C to +60°C

Accuracy: ± 0.1 °C @ 23°C

Serial number, dates of calibration and use:

Copano East

Serial Number: 61407321

Date of Last Calibration: 10/06/2014

Dates of Sensor Use: 06/10/2015 – 07/22/2015

Serial Number: 20018168

Date of Last Calibration: 04/21/2015

Dates of Sensor Use: 07/22/2015 – current as of 12/31/2015

Ship Channel

Serial Number: 61247942

Date of Last Calibration: 02/25/2014

Dates of Sensor Use: 05/08/2015 – current as of 12/31/2015

Parameter: Relative Humidity

Units: Percent

Sensor type: ROTRONIC® Hygromer IN-1

Model #: HC2-S3 Temperature and Relative Humidity Probe

Range: 0-100% non-condensing

Accuracy at 23°C:  +/- 0.8% RH with standard configuration settings

Temperature dependence of RH measurement +/- 3% (-40 to 60C)

Serial number, dates of calibration and use:

Copano East

Serial Number: 61407321

Date of Last Calibration: 10/06/2014

Dates of Sensor Use: 06/10/2015 – 07/22/2015

Serial Number: 20018168

Date of Last Calibration: 04/23/2015

Dates of Sensor Use: 07/22/2015 – current as of 12/31/2015

Ship Channel

Serial Number: 61247942

Date of Last Calibration: 02/25/2014

Dates of Sensor Use: 05/08/2015 – current as of 12/31/2015

Parameter: Barometric Pressure

Units: millibars (mb)

Sensor type: Vaisala Barocap © silicon capacitive pressure sensor

Model #: CS-105, CS-106

Operating Range:	 Pressure: 600 to 1060 mb; Temperature:  -40°C to +60°C;

Humidity:  non-condensing

Accuracy: ± 0.5 mb @ 20°C; +/- 2 mb @ 0°C to 40°C; +/- 4 mb @ -20°C to 45°C; +/- 6 mb @ -40°C to 60°C

Stability: ± 0.1 mb per year

Serial number, dates of calibration and use:

Copano East

Serial Number: A4830003 (CS-105)

Date of Last Calibration: 11/30/2005

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: D1440029 (CS-106)

Date of Last Calibration: 04/04/2008

Dates of Sensor Use: 07/29/2008 – 08/31/2010

Serial Number: A4830003 (CS-105)

Date of Last Calibration: 06/29/2010

Dates of Sensor Use: 08/31/2010 – 10/30/2012

Serial Number: D1440029 (CS-106)

Date of Last Calibration: 09/25/2012

Dates of Sensor Use: 10/30/2012 – 01/06/2015

Serial Number: A4830003 (CS-105)

Date of Last Calibration: 12/09/2014

Dates of Sensor Use: 01/06/2015 – current as of 12/31/2015

Ship Channel

Serial Number: K3840002 (CS-106)

Date of Last Calibration: 09/18/2014

Dates of Sensor Use: 05/08/2015 – current as of 12/31/2015

Parameter: Wind speed

Units: meter per second (m/s)

Sensor type: 18 cm diameter 4-blade helicoids propeller molded of polypropylene

Model #: R.M. Young 05103 Wind Monitor

Range:  0-60 m/s (134 mph); gust survival 100 m/s (220 mph)

Accuracy: +/- 0.3 m/s

Serial number, dates of calibration and use:

Copano East

Serial Number: WM73868

Date of Last Calibration: 07/20/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: WM84419

Date of Last Calibration: 02/21/2008

Dates of Sensor Use: 07/29/2008 – 08/31/2010

Serial Number: WM73868

Date of Last Calibration: 06/14/2010

Dates of Sensor Use: 08/31/2010 – 10/30/2012

Serial Number: WM84419

Date of Last Calibration: 09/19/2012

Dates of Sensor Use: 10/30/2012 – 01/06/2015

Serial Number: WM73868

Date of Last Calibration: 12/12/2014

Dates of Sensor Use: 01/06/2015 – current as of 12/31/2015

Ship Channel

Serial Number: WM138025

Date of Last Calibration: 12/03/2014

Dates of Sensor Use: 05/08/2015 – current as of 12/31/2015

Parameter: Wind direction

Units: degrees

Sensor type: balanced vane, 38 cm turning radius

Serial number, dates of calibration and use:

Copano East

Serial Number: WM73868

Date of Last Calibration: 07/20/2006

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: WM84419

Date of Last Calibration: 02/21/2008

Dates of Sensor Use: 07/29/2008 – 08/31/2010

Serial Number: WM73868

Date of Last Calibration: 06/14/2010

Dates of Sensor Use: 08/31/2010 – 10/30/2012

Serial Number: WM84419

Date of Last Calibration: 09/19/2012

Dates of Sensor Use: 10/30/2012 – 01/06/2015

Serial Number: WM73868

Date of Last Calibration: 12/12/2014

Dates of Sensor Use: 01/06/2015 – current as of 12/31/2015

Ship Channel

Serial Number: WM138025

Date of Last Calibration: 12/03/2014

Dates of Sensor Use: 05/08/2015 – current as of 12/31/2015

Parameter: Photosynthetically Active Radiation(PAR)

Units: mmoles m-2 (total flux)

Sensor type: High stability silicon photovoltaic detector (blue enhanced)

Model #: LI190SB

Light spectrum waveband:  400 to 700 nm

Temperature dependence: 0.15% per °C maximum

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 65°C; Humidity: 0 to 100%

Sensitivity:  typically 5 µA per 1000 µmoles s-1 m-2

Multiplier:

1.29 changed 11/04/2005

1.26 changed 07/29/2008

1.25 changed 06/16/2009

1.47 changed 08/31/2010

1.22 changed 05/04/2011

1.27 changed 07/26/2011

1.23 changed 01/19/2012

1.11 changed 07/08/2014

Serial number, dates of calibration and use:

Serial Number: Q35626

Date of Last Calibration: 11/04/2005

Dates of Sensor Use: 11/04/2006 – 07/29/2008

Serial Number: Q38890

Date of Last Calibration: 11/20/2007

Dates of Sensor Use: 07/29/2008 – 06/16/2009

Serial Number: Q35626

Date of Last Calibration: 05/22/2009

Dates of Sensor Use: 06/16/2009 – 08/31/2010

Serial Number: Q38890

Date of Last Calibration: 06/17/2010

Dates of Sensor Use: 08/31/2010 – 05/04/2011

Serial Number: Q35626

Date of Last Calibration:

Dates of Sensor Use: 05/04/2011 – 07/26/2011

Serial Number: Q45313

Date of Last Calibration: 04/26/2011

Dates of Sensor Use: 07/26/2011 – 01/19/2012

Serial Number: Q43906

Date of Last Calibration: 08/24/2011

Dates of Sensor Use: 01/19/2012 – 05/02/2014

Serial Number: Q45313

Date of Last Calibration: 04/18/2014

Dates of Sensor Use: 05/02/2014 – 06/10/2015

Parameter: Photosynthetically Active Radiation (PAR)

Units:  mmoles m-2 (total flux)

Sensor type:  anodized aluminum with cast acrylic diffuser

Model #SQ110 Apogee Quantum Sensor

Light spectrum waveband:  410 to 655 nm

Temperature dependence: 0.06+/-0.06% per °C

Stability:  <±2% change over 1 yr

Operating Temperature:  -40°C to 70°C; Humidity 0 to 100%

Cosine Response: 45° zenith angle: +/- 2%; 75° zenith angle: +/- 5%

Sensitivity:  0.2mV per µmol s-1 m-2

Multiplier: 0.025

Serial number, dates of calibration and use:

Copano East

Serial Number: 17015

Date of Last Calibration: 11/1/2014

Dates of Sensor Use: 06/10/2015 – current as of 12/31/2015

Ship Channel

Serial Number: 17007

Date of Last Calibration: 10/1/2014

Dates of Sensor Use: 05/08/2015 – current as of 12/31/2015

Parameter: Precipitation (specify if heated rain gauge)

Units: millimeters (mm)

Sensor type: Tipping Bucket Rain Gauge

Model #:  TE525

Rainfall per tip: 0.01 inch

Operating range: Temperature: 0° to 50°C; Humidity: 0 to 100%

Accuracy:  +/- 1.0% up to 1 in./hr; +0, -3% from 1 to 2 in./hr; +0, -5% from 2 to 3 in./hr

Serial number, dates of calibration and use:

Copano East

Serial Number: 39832-806

Date of Last Calibration: 05/05/2005

Dates of Sensor Use: 11/14/2006 – 07/29/2008

Serial Number: 45841-208

Date of Last Calibration: 02/26/2008

Dates of Sensor Use: 07/29/2008 – 06/16/2009

Serial Number: 39832-806

Date of Last Calibration: 06/12/2009

Dates of Sensor Use: 06/16/2009 – 08/31/2010

Serial Number: 45841-208

Date of Last Calibration: 08/12/2010

Dates of Sensor Use: 08/31/2010 – 01/19/2012

Serial Number: 39832-806

Date of Last Calibration: 11/11/2011

Dates of Sensor Use: 01/19/2012 – 05/02/2014

Serial Number: 45841-208

Date of Last Calibration: 05/01/2014

Dates of Sensor Use: 05/02/2014 – 10/20/2015

Serial Number: 39832-806

Date of Last Calibration: 10/17/2015

Dates of Sensor Use: 10/20/2015 - current as of 12/31/2015

Ship Channel

Serial Number: 61945-1014

Date of Last Calibration: 10/08/2014

Dates of Sensor Use: 05/08/2015 – current as of 12/31/2015

The CR1000 has 2 MB of Flash EEPROM that is used to store the Operating System.  Another 128 K Flash is used to store configuration settings.  A minimum of 2 MB SRAM is (4 MB optional upgrade) available for program storage (16K), operating system use, and data storage.  Additional storage is available by using a compact flash card in the optional CFM100 Compact Flash Module.

Copano East

Serial Number: 5968

Date CR1000 Installed: 11/14/2006

Date CR1000 Calibrated: 07/22/2014

CR1000 Firmware Version (s):  Std.27.04, updated on 08/06/2014

CR1000 Program Version(s): marcemet_5.6._A_10_19_15.cr1

Ship Channel

Serial Number: 69572

Date CR1000 Installed: 05/08/2015

Date CR1000 Calibrated: 12/17/2014

CR1000 Firmware Version (s):  Std.27.04

CR1000 Program Version(s): marscmet_5.5A_52815.cr1

10)  Coded variable definitions

Sampling station:		Sampling site code:	Station code:

Copano East			CE				marcemet

Ship Channel			SC				marscmet

11)  QAQC flag definitions

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   During primary automated QAQC (performed by the CDMO), -5, -4, and -2 flags are applied automatically to indicate data that is above or below sensor range, or missing.  All remaining data are then flagged 0, as passing initial QAQC checks.   During secondary and tertiary QAQC 1, -3, and 5 flags may be used to note data as suspect, rejected due to QAQC, or corrected.

-5	Outside High Sensor Range

-4	Outside Low Sensor Range

-3	Data Rejected due to QAQC

-2	Missing Data

-1	Optional SWMP supported parameter

0		Passed Initial QAQC Checks

1		Suspect Data

2		Open - reserved for later flag

3		Open - reserved for later flag

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

12)  QAQC code definitions

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the CR1000, sensor errors are sensor specific, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point, but some comment codes (marked with an * below)  can be applied to the entire record in the F_Record column.

General Errors

GIM	Instrument malfunction

GIT	Instrument recording error, recovered telemetry data

GMC	No instrument deployed due to maintenance/calibration

GMT 	Instrument maintenance

GPD	Power down

GPF	Power failure / Low battery

GPR	Program reload

GQR	Data rejected due to QA/QC checks

GSM	See metadata

Sensor Errors

SDG	Suspect due to sensor diagnostics

SIC	Incorrect calibration constant, multiplier or offset

SIW	Incorrect wiring

SMT	Sensor maintenance

SNV	Negative value

SOC	Out of calibration

SQR	Data rejected due to QAQC checks

SSD	Sensor drift

SSN	Not a number / unknown value

SSM	Sensor malfunction

SSR	Sensor removed

Comments

CAF	Acceptable calibration/accuracy error of sensor

CCU	Cause unknown

CDF	Data appear to fit conditions

CML	Snow melt from previous snowfall event

CRE*	Significant rain event

CSM*	See metadata

CVT*	Possible vandalism/tampering

CWE*	Significant weather event

13)  Other remarks/notes

Data are missing due to equipment or associated specific sensors not being deployed, equipment failure, time of maintenance or calibration of equipment, or repair/replacement of a sampling station platform.  Any NANs in the dataset stand for “not a number” and are the result of low power, disconnected wires, or out of range readings.  If additional information on missing data is needed, contact the Research Coordinator at the reserve submitting the data.

Small negative PAR values are within range of the sensor and are due to normal errors in the sensor and the CR1000 Datalogger. The Maximum signal noise error for the Licor sensor is +/- 2.214 mmoles/m2 over a 15 minute interval.

Relative Humidity data greater than 100 are within range of the sensor accuracy of +/-3%.

Data recorded for all parameters (with the exception of cumulative precipitation) at the midnight timestamp (00:00) are the 15 minute averages and totals for the 23:45-23:59 time period of the previous day.  Cumulative precipitation data at the midnight timestamp (00:00) are the sum of raw (unrounded) precipitation data from 00:00 to 23:59 of the previous day.  Summing each individual 15-minute total precipitation value from the same period will result in small differences from cumulative precipitation due to rounding.  It is especially important to note how data at the midnight timestamp are recorded when using January 1st and December 31st data.  Note:  Cumulative precipitation is no longer available via export from the CDMO.  Please contact the Reserve or the CDMO for more information or to obtain these data.

Copano East

During 2017 the CDMO discovered an incorrect line in the CR1000 programming for the program uploaded on 06/10/2015.  If RHumidity>100 And RHumidity<108 Then RHumidity=100.  A decision was made by the DMC during 2006 to discontinue correcting >100 RH values to 100. By correcting all values >100 during data collection we may have missed erroneous values that could have indicated a problem with the RH sensor.  CSM coding was added to all RH data from 06/10/2015 until the programming change in 2017.

There were many nights during the year in which the nighttime PAR readings exceeded the noise error of the LICOR (+/- 2.214) and were rejected. I am unsure as to the reason for the higher readings.

- Elevated nighttime readings <2.214mmolsm-2 are flagged and coded as <1> CSM.

- Elevated nighttime readings >2.214mmolesm-2 are flagged and coded as <-3>CSM.

- Negative PAR readings beyond the noise error of the LICOR are flagged and coded <-3> CSM. This may be due to water inundation since it appears more prevalent during rain events or days when it is especially foggy.

- On days where the previous nighttime readings were > 30 mmoles m-2, the daytime PAR readings have been marked as suspect.

From 1/1/15 00:00 through the sensor swap on 06/10/15 09:00 RH data are flagged and coded as either rejected or suspect.  Data 101-103 are flagged as suspect and data >103 are flagged as rejected.  The newly installed RH sensor continued to have problems with high values from 06/23/15 10:00 – 7/22/15 10:15 Temperature and RH data were flagged suspect since readings appeared off compared to SC MET and data checks with the Kestrel; highly erroneous RH data were rejected and the sensor was replaced again on 07/22/2015.

01/06/15 10:45-11:15			Swapped wind monitor (WM78368) and BP sensor (A4830003); uploaded new program with CS-105 BP sensor enabled. All data are missing due to power down for maintenance. Data were rejected at 11:30 due to missing 5-second data as a result of program reload. Cumulative precipitation data from 11:45 - 01/07/15 00:00 were not rejected as no precipitation was recorded prior to the upload since the value remained zero.

01/06/15 11:45 – 03/12/15 09:15		Wind Dir and SDWindDir rejected since QAQC checks indicated that the wind monitor was off. It is believed that the nut clamping the monitor to the pole was not tightened enough and high winds moved it off course.

03/12/15 09:15				Adjusted the direction of the junction box on the wind monitor so that it was facing due south and the nose cone was pointed true north.

03/12/15 09:30 - 12/31/15 23:45           WDir, SDWDir, WSpd, and MaxWSpd flagged suspect since wind monitor was installed ~ 60º off from where it had previously been positioned.

04/10/15 08:30-10:15			Due some unknown reason, there was a gap in the data upon retrieval of the flash card.

04/10/15 10:30-04/11/15 00:00 		Cumulative precipitation data were rejected.  Data were missing from the flash card so we do not know how accurate cumulative totals are for the day.

03/26/15 06:45 – 03/29/15 19:00		Daytime PAR readings flagged suspect

04/03/15 06:45 – 04/10/15 08:15		since they are much lower than normal and although the weather was overcast, the readings don’t follow the normal pattern of an overcast day where there are periods of time when the sun comes out.

04/10/1510:30 - 06/10/15 09:00 PAR data rejected.  PAR sensor not working properly even though it has only been just over a year since calibration.  A new sensor was installed and began recording data 06/10/15 11:00.

04/30/15 08:30-10:15			Encountered trouble retrieving file from flashcard and there was a gap in the data when we repaired the file.

05/24/15 01:30				Precipitation data flagged suspect since the reading was higher than the accuracy of the sensor; however, when compared to data collected at the airport closest to the site, the readings were very close to what was recorded there. Cumulative precipitation data were flagged as suspect through 05/25/15 00:00

06/04/15 09:15 – 06/10/15 09:00		Data recovered via telemetry (thus no CumPrcp) due to CFM100 not writing onto card (malfunction). Max wind speed time was missing for the 09:15 reading.  All data not rejected or missing are considered suspect.

06/10/15 09:15-10:45			Swapped PAR (replaced w/ Apogee), Temp/RH sensor (HC2S3 Rotronic Hygroclip 2), back-up battery, navigation lamp, and uploaded new CR1000 program. Data are missing 09:15 - 10:30 due to power down for maintenance. Data were rejected at 10:45 due to missing 5-second data as a result of program reload. Cumulative precipitation data were not rejected for the remainder of the day as no precipitation was recorded prior to the upload since the value remained zero.

06/10/15 10:45 – 12/31/16 23:45          CSM was added to RH coding. Programming for RH is incorrect. During program upload, the following line of code was added:

If RHumidity > 100 and RHumidity < 108 then RHumidity = 100

06/23/15 10:00 – 7/22/15 10:15		Temperature and RH data were flagged suspect since readings appeared off compared to SC MET and data checks with the Kestrel; highly erroneous RH data were rejected.

07/09/15 10:15				Data were rejected due to missing 5-second data during power down to check telemetry and conduct maintenance. Cumulative precipitation data were not rejected as no precipitation was recorded prior to the upload since the value remained zero.

07/22/15 10:30-10:45				Swapped Temperature/RH sensor since RH was maxing out and then dropping to 0.3%. This was the new probe installed on 06/10/15. We also replaced the gill shield in case water had gotten up under it. Data at 10:30 are missing due to a power down. Data were rejected due to missing 5-second data as a result of the power down. Cumulative precipitation data for the remainder of the day were not rejected as no precipitation was recorded prior to the power down since the value remained zero.

10/15/15 10:00 – 10/20/15 11:30		Program was uploaded to allow us to download onto the laptop rather than to the CFM 100 (which was having issues) but we forgot to include the record table in the program, so data were lost.

10/20/15 11:45  				Data were rejected due to missing 5-second data during power down to swap Rain Gage and reload new program. TotPrcp data at 12:00 and CumPrcp data 12:00 through the end of the day were corrected since there were Prcp readings from the swap that did not reflect real data, but rather water added to tipping bucket to ensure that it was working correctly.

12/10/15 19:30 – 12/14/15 10:45		Grounding wire to the solar panel came out during this time which resulted in the panels not being charged and the battery was drained. Unfortunately, the telemetry is also tied into this battery so data were lost.

12/14/15 11:00				All parameters were rejected due to not being a full 15 minutes of 5-second data following the power failure.  CumPrcp 11:15 through 12/15/15 00:00 were also rejected due to not having a value (NAN).

Ship Channel (Secondary SWMP MET Station)

The CDMO does not authenticate the data from the Ship Channel station since it is a secondary site in the NERRS SWMP. Only primary SWMP sites go through tertiary review by the CDMO.

01/01/2015 00:00 - 05/08/2015 10:00 	No data were collected at the Ship Channel MET station.  All data were flagged as missing because the station was not installed until 05/08/2015.

05/08/15 10:00 -10:45			Station was being installed and there were some issues with the ATemp and RH during this time frame, so that Collin had to mess around with the sensor in order for it to operate properly. This resulted in the rejection of those data for ATemp and RH. Data for all parameters were rejected at 10:00 since this was the first reading and there may not have been a full 15 minutes of 5-second data.

05/08/15 19:00 – 05/29/15 10:30	Corrected PAR data by dividing by 5 (multiplier incorrectly used in the program) and then multiplied by 0.025 to change CR1000 readings to mmoles/m2.

05/08/15 13:15				Data were rejected due to missing 5-second data during power down for maintenance. Cumulative precipitation data from 05/08/15 13:30 – 05/09/15 00:00 were not rejected as no precipitation was recorded prior to the upload since the value remained zero.

05/29/15 10:45-14:30			Data are missing due to a power down for maintenance. Data at 14:30 were rejected due to missing 5-second data as a result of uploading CR1000 program with the corrected PAR multiplier. Cum Prcp data have been corrected since the program reload reset the Cum Prcp.

06/01/2015 Data rejected due to a power down.

06/02/15 10:15-10:30			BP data rejected as it dropped very low for no apparent reason. Data were compared with data from CE MET as well as the weather station at the local airport and the weird anomaly was not present in those data.

06/12/15 14:15				Data were rejected due to missing 5-second data during a power down to work on telemetry. Cumulative precipitation data from 06/12/15 14:30 – 06/13/15 00:00 were not rejected as no precipitation was recorded prior to the upload since the value remained zero.

06/17/15 11:30				Precipitation was flagged suspect since rain gage is reading slightly higher than the accuracy of the sensor; however, the streets flooded within a 15-min time period, so the data is reflecting a real event.  Cumulative data through 6/18/15 00:00 were flagged as suspect.

07/07/15 09:45				Data were rejected due to missing 5-second data as a result of power down for maintenance. Cumulative precipitation data from 07/07/15 10:00 – 07/08/15 00:00 were not rejected as no precipitation was recorded prior to the upload since the value remained zero.

08/26/15 11:15				Data were rejected due to missing 5-second data during brief power down for maintenance. Cumulative precipitation data from 08/26/15 11:30 – 08/27/15 00:00 were not rejected as no precipitation was recorded prior to the upload since the value remained zero.

09/14/15 11:15				Data were rejected due to missing 5-second data during power down for maintenance. Cumulative precipitation data from 09/14/15 11:30 – 09/15/15 00:00 were not rejected as no precipitation was recorded prior to the upload since the value remained zero.

09/21/15 09:30-09:45			Prcp and Cum Prcp data were corrected due to cleaning rain gage during routine monthly maintenance check. Cumulative data were corrected through 09/22/15 00:00.

10/26/15 10:45				Data were rejected due to missing 5-second data during power down for maintenance; working on troubleshooting telemetry issue. Cumulative precipitation data from 10/26/15 11:00 – 10/27/15 00:00 were not rejected as no precipitation was recorded prior to the upload since the value remained zero.

11/10/15 14:00				Data were rejected due to missing 5-second data during a power down of the CR1000. Cumulative precipitation data from 11/10/15 14:15 – 11/11/15 00:00 were not rejected as no precipitation was recorded prior to the upload since the value remained zero.

11/11/15 14:00 – 11/13/15 14:00		For some unknown reason, there was a gap of missing data for 2 days…it doesn’t appear to have been an issue with power, so we’re not sure why it stopped recording. Unfortunately, the telemetry was still out at this time so we couldn’t recover those data.

12/01/15 13:00				Data were rejected due to missing 5-second data during power down for maintenance; still trying to figure out why telemetry isn’t working. Cumulative precipitation data other than first value after power up were not rejected since data collection occurred prior to power down and there wasn’t a program reload.