Mission Aransas (MAR) NERR Nutrient Metadata

January – December 2014

Latest Update: November 23, 2016

I.  Data Set and Research Descriptors

1)  Principal investigator(s) and contact persons:

Address: 	Mission Aransas National Estuarine Research Reserve

750 Channelview Drive

Port Aransas, TX 78373

Phone:  (361)749-6711

Fax:	 (361)749-6777

Contact Persons:

Dr. Edward Buskey, Research Program Coordinator

E-mail: ed.buskey@utexas.edu; (361)749-3102

Sally Morehead, Manager

E-mail: sally.morehead@utexas.edu; (361)749-3046

Cammie Hyatt, SWMP Data Manager & Technician (chlorophyll sample analysis)

E-mail: cammie.hyatt@utexas.edu; (361)749-3093

Rae Mooney, SWMP Technician, Boat Operations (nutrient sample analysis)

E-mail: rmooney@utexas.edu; (361)749-3101

2)  Research objectives:

Five long-term monitoring stations have been established along the Texas Coast in the Mission-Aransas estuary to collect baseline data on environmental conditions within the bay systems. These System Wide Monitoring Program Stations (SWMPs) provide continual monitoring of temperature, salinity, oxygen and other parameters that characterize the water quality of the bays. In addition to the above monitoring program, water samples are collected monthly to quantify the amounts of nutrients (orthophosphate, ammonium, nitrate/nitrite, silicate, and chlorophyll a) present in the water. There are two components to the nutrient monitoring program in place within the Mission-Aransas NERR: a) Monthly Grab Sampling Program, and b) Diel Sampling Program.

- Monthly Grab Sampling Program:

The monthly grab samples are collected at each of the five monitoring stations which are situated along a salinity gradient that is highest at the Aransas Ship Channel and lowest in Copano West. The main goal of the Monthly Grab Sampling Program is to provide baseline information on inorganic nutrients and chlorophyll a within the reserve. Nutrient concentrations are important in determining pollution that enters the system from agricultural and urban sources via runoff.

- Diel Sampling Program:

Monthly diel samples are collected over a 24-hr period. The main goal of this sampling program is to provide baseline information on inorganic nutrients and chlorophyll a.  By collecting these samples throughout a lunar tidal cycle, a more focused view of these nutrients can be achieved than just a single grab sample will allow. The site selected for this monitoring program (Aransas Ship Channel) was based primarily on the extreme tidal exchange that occurs in this area. On the incoming tide, higher salinity offshore water is drawn into the bays and on the outgoing tide, the lower salinity water from the bays is drawn out.

3) Research methods:

- Monthly Grab Sampling Program:

Monthly grab samples are taken at each of the MANERR SWMP stations where datasondes are deployed. All grab samples are taken on the same day; however, due to the large size of the reserve and the fact that all but one of the sites are only accessible by boat, the stipulation that samples be taken at low/slack tide is usually not possible. Efforts are made to allow for an antecedent dry period of 72 hours prior to sampling but that is not always possible due to long periods of inclement weather.

Samples are collected at approximately 30 day intervals, weather permitting. Two replicate samples are taken using a Van Dorn alpha bottle deployed at the depth of the sondes (0.5 m off the bottom of the bay except at the SC site which is 1m). A weight attached to the bottom of the bottle allows the collector to determine the proper depth. Once the weight has settled on the bottom of the bay, the sand and/or mud is allowed to settle for a minute or so before the messenger is sent to seal the bottle. The amber collection bottles are rinsed (3X) before filling with sample. Prior to 9/14/10, samples were then placed on ice until processing back in the laboratory; however, now the nutrient samples are filtered in the field using the same protocol described below.

While in the field, a spare sonde is used to collect ambient readings of Temperature, Specific Conductivity, Salinity, Dissolved Oxygen, Turbidity, and Chlorophyll (chlorophyll and dissolved oxygen available by contacting the Reserve directly). These readings are performed at the same approximate depth as the nutrient collection with the exception of the Ship Channel site that is collected at approximately 1.5 m due to the length of the field cable. Secchi readings are taken at each site, although the strong current at the SC site sometimes makes secchi readings there impossible.  The cloud cover code and wave height codes are also noted; any available cloud cover and wave height observations may be obtained by contacting the Reserve directly.

Prior to filtering nutrients, a 60 ml syringe is rinsed with the sample water before attaching a disposable 0.45 um MCE syringe filter. For each replicate sample, approximately 10 mls of sample is pushed through the filter to rinse it, and then two sample tubes are filled, capped, and placed on ice until all samples are processed. (Note: Two sub-samples are taken from each replicate sample as a means of checking the auto-analyzer that is used to process our nutrient samples.) Gloves are worn while filtering the nutrients to prevent contamination. The nutrient samples are placed on ice until return to the laboratory where they are delivered to the in-house nutrient specialist. They are then placed in the freezer and held at -20°C until analysis. While the nutrient samples are being filtered, the amber bottles containing the remainder of each sample are also placed on ice until being filtered for chlorophyll back in the laboratory.

The chlorophyll a processing includes mixing the sample and then measuring an aliquot for filtration. To prevent clogging, the Secchi reading is used to determine how much water to filter. (This is based on an arbitrary convention developed by Paul Montagna’s Lab to avoid filter “clogging”) Maximum and minimum volumes are 50 ml (any Secchi reading ≥ 1m) and 15 ml (any Secchi reading ≤ 0.3m), respectively. Filtered volumes between the upper and lower limits are determined by Secchi reading as a percent of 100 cm multiplied by the maximum volume of 50 mls. (e.g. a Secchi reading of 80 cm would require a volume of 40 ml, a Secchi reading of 50 cm would require a volume of 25 ml, a Secchi reading of 120 cm would require a Secchi reading of 50 ml, and a Secchi reading of 20 cm would require a volume of 15 ml). Once the appropriate volume has been determined and measured into a graduated cylinder, the samples are filtered onto 25mm Whatman GF/F filters using a Gast vacuum pump attached to a filtration manifold. The filters are folded in half and placed into glass scintillation vials to which 10 mls 90% acetone is added. All filtering is done in semi-darkness and the vials are then placed in a box inside the freezer to extract for two days. At the end of the extraction period, the vials are removed from the freezer and placed in the dark at room temperature for approximately 30 minutes. After the samples have defrosted for 30 minutes, the solid secondary standard is read to ensure that the calibration of the fluorometer (Turner Designs Trilogy) is still good. A blank of 90% acetone is also read prior to reading the chlorophyll samples. All lab glassware is washed with Liquinox cleaning solution, rinsed (3X) with tap water, soaked in 5% HCl overnight and then rinsed (3X) with de-ionized water. Note: We began running size-fractionated (5.0 um, and 20 um) chlorophylls on 07/14/09 in addition to the GF/F’s that we are already required to run. Starting on 11/8/10, we switched from 20 um Nylon filters to pre-filtering the water through a 20 um mesh onto a GF/F filter to collect the < 20 um fraction. The size-fractionated chlorophyll protocol was changed again on 08/23/11. The >5<20 um fraction was obtained by pouring the sample through a 20 um sieve onto the 5 um Nylon filter and the 0.7-20 um fraction was obtained by pouring the sample through a 20 um sieve onto a GF/F filter. The 0.7 um-∞ fraction (GF/F) was collected as it has always been. Data for size fractionated chlorophyll samples are located in the Chlorophylls2014 Excel file and may be obtained by contacting the CDMO or the Research Coordinator of the Mission Aransas NERR.

- Diel Sampling Program:

Diel samples are collected once a month at the University of Texas Marine Science Institute pier which is located on the Aransas Ship Channel. This location is designated as one of the MANERR SWMP stations and has a datasonde deployed there. An ISCO Avalanche refrigerated auto-sampler takes 0.9 L samples at 2-hour intervals over a 24-hour cycle, resulting in 13 samples per event. (In the event of equipment repair on the refrigerated model, we have a non-refrigerated model as back-up.) Note: The CDMO determined that we needed to change this protocol slightly to include the full 24 hour period, so we began collecting 13 samples, beginning in September 2012. Diel sampling usually takes place between +/- 3 hours of slack-low tide but due to tidal conditions that may not have been possible. Efforts are made to allow for an antecedent dry period of 72 hours prior to sampling; however, this was not always possible due to long periods of inclement weather. Sampling events take place at approximately 30 day intervals, weather permitting. Samples are collected at an approximate depth of 5 m which is slightly less than the depth of the datasonde. All samples are collected in ISCO sampling bottles that have been previously cleaned in the same manner as the lab glassware described above. As soon as the first sample is taken, the auto-sampler refrigeration lowers the temperature in the chamber down to 3 ºC and keeps it at that temperature until the unit is unplugged and taken back into the laboratory. Back in the laboratory, it is plugged back in to keep the samples cold during processing. Samples for nutrients and chlorophyll are handled in the same manner as previously described in the “Monthly Grab Sampling Program” portion of this section. Note: Size-fractionated Chlorophyll samples are not run for the ISCO samples.

4)  Site location and character:

Mission-Aransas is an estuarine system composed of tertiary, secondary, and primary bays. Mission Bay is the only tertiary bay, and Copano, Port and St. Charles Bay are secondary bays. Mesquite, Aransas and Redfish Bay are primary bays because they are adjacent to the oceanic outlets. There is a salinity gradient which is highest at the Aransas Ship Channel and decreases into the upper bays. The five sites within Mission-Aransas that are being monitored with YSI data loggers are:

- Aransas Bay (AB): [27º 58’ 47.28 N, 97º 1’ 43.32 W].  This station is located in open water in the middle of Aransas Bay. Tidal range is 1.0 meter, and yearly salinity ranges between 25-41 ppt. Yearly temperatures range from 7-32 ºC. Aransas Bay is influenced by freshwater inflow from both San Antonio Bay (which has inflow from the San Antonio River and the Guadalupe River) and Copano Bay. The average water depth is 3.0 meters and the bottom is composed of soft sediment. The Texas Commission of Environmental Quality (TCEQ) has determined that there are no exceedences for pollutants in Aransas Bay. This station is primarily influenced by the watershed sub-basin in Aransas County with a hydrologic unit code (HUC) of 12100405.

- Copano East (CE): [28º 7’ 56.28 N, 97º 2’ 3.84 W]. This station is located in open water on the east side of Copano Bay near the Copano Causeway. Tidal range is 1.0 meter, and yearly salinity ranges between 24-42 ppt. Yearly temperatures range from 6-32 ºC. Copano East is influenced by freshwater inflow from the Aransas River, Mission River, and Copano Creek. The average water depth is 2.5 meters and the bottom is composed of soft sediment. The TCEQ has determined that there are no exceedences for pollutants in Copano East. This station is influenced by three watershed sub-basins in several counties with HUC’s of 12100407, 12100406, and 12100405.

- Copano West (CW): [28º 5’ 2.76 N, 97º 12’ 3.24 W]. This station is located in open water on the west side of Copano Bay near Bayside. Tidal range is 1.0 meter, and yearly salinity ranges between 23-42 ppt. Yearly temperatures range from 7-32 ºC. Copano West is primarily influenced by freshwater inflow from the Aransas River (See Table 1 in Section 14). The average water depth is 1.5 meters and the bottom is composed of soft sediment. The TCEQ has determined that there is an exceedence for enterococci bacteria in Copano West. This station is primarily influenced by the watershed sub-basin in San Patricio and Bee County with a HUC of 12100407 (see Figure 1 in Section 14).

- Ship Channel (SC): [27º 50’ 17.88 N, 97º 3’ 0.0 W]. This station is located on the end of the UTMSI pier. Tidal range is 1.0 meter, and yearly salinity ranges between 27-42 ppt. Yearly temperatures range from 8-31 ºC. The Aransas Ship Channel is located on the Aransas Pass, which is the primary outlet to the Gulf of Mexico. This pass receives freshwater inflow from several bay systems, including Copano Bay, Aransas Bay, San Antonio Bay, and Corpus Christi Bay. The average water depth is 6.2 meters and the bottom is composed of soft sediment. The TCEQ currently has no exceedences for pollutants in the Aransas Ship Channel. This site is primarily influenced by the watershed sub-basin in Aransas County with a HUC of 12100405.

- Mesquite Bay (MB): [28° 8’ 18.24 N, 96° 49’ 42.60 W]. This station is located in open water in Mesquite Bay which is within the Aransas National Wildlife Refuge. Tidal range is 1.0 meter, and yearly salinity ranges between 19-41 ppt. Yearly temperatures range from 5-33 ºC. Mesquite Bay is primarily influenced by freshwater inflow from San Antonio Bay, which has inflow from the San Antonio River and the Guadalupe River. The average water depth is 1.2 m and the bottom is composed of soft sediment. The TCEQ currently has no exceedences for pollutants in Mesquite Bay. The Mesquite SWMP station is influenced by the watershed sub-basin in Refugio and Calhoun County with a HUC of 12100404.

Note: The yearly ranges for Temperature and Salinity are for the time frame of this document. These ranges change slightly from year to year, depending on whether we have a wet or dry year as well as the mildness of our winter.

5) Coded variable definitions:

marabnut = Mission-Aransas Aransas Bay nutrients

marcenut = Mission-Aransas Copano East nutrients

marcwnut = Mission-Aransas Copano West nutrients

marscnut = Mission-Aransas Ship Channel nutrients

marmbnut = Mission-Aransas Mesquite Bay nutrients

monthly grab sample program = 1

diel grab sample program = 2

replicates = 1, 2, or s (indicates laboratory split)

6) Data collection period:

Monthly Grab Sampling Program

Nutrient monitoring for this program began at each station on the following dates: AB, 06/15/07; CE, 06/15/07; CW, 07/24/07; SC, 09/20/07; MB, 01/10/08.

Site			Date			Rep 1		Rep 2

Aransas Bay		01/10/2014		08:30		08:33

AB			02/04/2014		09:15		09:18

03/19/2014		08:19		08:22

04/16/2014		07:15		07:18

05/29/2014		07:20		07:23

06/24/2014		07:40		07:43

07/23/2014		07:24		07:27

08/19/2014		07:02		07:05

09/17/2014		07:24		07:27

10/15/2014		07:10		07:13

11/19/2014		08:45		08:48

12/09/2014		08:45		08:48

Site			Date			Rep 1		Rep 2

Copano East		01/10/2014		10:10		10:13

CE			02/20/2014		12:07		12:10

03/19/2014		10:40		10:43

04/16/2014		10:00		10:03

05/29/2014		09:55		09:58

06/24/2014		10:30		10:33

07/23/2014		09:42		09:45

08/19/2014		09:42		09:45

09/17/2014		09:52		09:55

10/15/2014		09:25		09:28

11/19/2014		10:45		10:48

12/09/2014		10:50		10:53

Site			Date			Rep 1		Rep 2

Copano West		01/10/2014		12:00		12:03

CW			02/20/2014		10:55		10:58

03/19/2014		09:58		10:01

04/16/2014		08:59		09:02

05/29/2014		09:12		09:15

06/24/2014		09:40		09:43

07/23/2014		08:55		08:58

08/19/2014		08:45		08:48

09/17/2014		08:48		08:51

10/15/2014		08:50		08:53

11/19/2014		09:50		09:53

12/09/2014	           10:00	     10:03

Site			Date			Rep 1		Rep 2

Ship Channel		01/22/2014		11:40		11:50

SC			02/04/2014		13:35		13:38

03/19/2014		11:50		11:52

04/16/2014		12:20		12:23

05/29/2014		12:05		12:08

06/24/2014		13:05		13:08

07/23/2014		12:10		12:13

08/19/2014		11:58		12:02

09/17/2014		14:03		14:06

10/15/2014		13:56		13:59

11/19/2014		13:40		13:43

12/09/2014		13:14		13:17

Site			Date			Rep 1		Rep 2

Mesquite Bay		01/22/2014		10:10		10:13

MB			02/20/2014		13:45		13:48

03/21/2014		09:37		09:40

04/16/2014		10:55		10:58

05/29/2014		10:48		10:51

06/24/2014		11:38		11:41

07/23/2014		10:50		10:53

08/19/2014		10:37		10:40

09/17/2014		10:45		10:48

10/15/2014		10:30		10:33

11/19/2014		12:05		12:08

12/09/2014		12:05		12:08

Diel Sampling Program

Nutrient monitoring for this program began 09/19/07.

Site			First Sample			Last Sample

Ship Channel		01/16/2014 13:49		11/17/2015 13:49

SC			02/24/2014 14:44		02/25/2014 14:44

03/25/2014 15:20		03/26/2014 15:20

04/21/2014 13:20		04/22/2014 13:20

05/27/2014 12:13		05/28/2014 12:13

06/26/2014 13:28		06/27/2014 13:28

07/21/2014 14:44		07/22/2014 14:44

08/28/2014 09:23		08/29/2014 09:23

09/29/2014 14:38		09/30/2014 14:38

10/28/2014 14:20		10/29/2014 14:20

11/25/2014 10:11		11/26/2014 10:11

12/11/2014 10:23		12/12/2014 10:23

7) Associated researchers and projects:

The MANERR water quality monitoring project has been incorporated into the Texas Coastal Oceanic Observation Network (TCOON).

In addition to the SWMP Nutrient data that are collected, MANERR also has a SWMP Meteorological station (MARCEMET) that collects air temperature, humidity, barometric pressure, wind speed, wind direction, precipitation, and photosynthetically active radiation (PAR). Water Quality data are also collected using a multi-parameter sonde that measures water temperature, specific conductivity, salinity, dissolved oxygen (% and mg/L), depth (or level, depending upon site), pH, turbidity, and in situ chlorophyll a. Whole water samples are collected on each sampling trip and fixed with Lugol’s iodine for enumeration of microzooplankton and mesozooplankton samples are collected monthly.

Lisa Campbell, a research scientist from Texas A & M College Station, has installed an in-situ flowcytometer (FlowCytoBot) to study the phytoplankton at this site in order to detect potential harmful algal blooms (HABs). This work is being conducted in conjunction with Robert Olsen of Woods Hole Oceanographic Institute.

The following researchers are currently using water quality data recorded at the SWMP stations in addition to collecting water and/or samples for their own projects:

Rodrigo Almeda, a post-doc at UTMSI, used zooplankton collected within the reserve to conduct experiments that will help us to better understand the effects of oil on zooplankton.

Colleen McCue, the Volunteer Coordinator for MANERR, is supervising a project to enumerate blue crab megalope abundance coming into the estuary. This project relies on volunteers to collect samples daily at Horace Caldwell pier and the Ship Channel, in addition to a site in Rockport and Aransas Pass.

Lindsay Scheef, a post-doc at UTMSI, is using tilt meters to study the currents and water exchange within the bays.

Summer REU (Research Experience for Undergraduates) Students collected samples within the Mission Aransas NERR for independent research projects:

Michele Mei, Tufts University, Biology & Computer Science, studied the effects of crude oil ingestion on copepod escape behavior.

Claire Weirich, Texas Lutheran University, Environmental Biology & Math, studied currents and tides in the Ship Channel and how they affect settlement of blue crab larvae.

8) Distribution:

NOAA retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data.  The NERRS retains the right to be fully credited for having collected and process the data.  Following academic courtesy standards, the NERR site where the data were collected should be contacted and fully acknowledged in any subsequent publications in which any part of the data are used.  The data set enclosed within this package/transmission is only as good as the quality assurance and quality control responsibility for its subsequent use/misuse in any further analyses or comparisons.  The Federal government does not assume liability to the Recipient or third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

Requested citation format:

National Estuarine Research Reserve System (NERRS). System-wide Monitoring Program. Data accessed from the NOAA NERRS Centralized Data Management Office website: www.nerrsdata.org; accessed 12 October 2012.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under the general information link on the CDMO home page) and online at the CDMO home page www.nerrsdata.org.  Data are available in comma separated version format.

II. Physical Structure Descriptors

9) Entry verification:

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro.  The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; adds chosen parameters and facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and then automatically flags/codes measured values below MDL and inserts the MDL; calculates parameters chosen by the user and automatically flags/codes for component values below MDL, negative calculated values, and missing data; allows the user to apply QAQC flags and codes to the data; produces summary statistics; graphs selected parameters for review; and exports the resulting data file to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

Cammie Hyatt is responsible for data processing and QAQC, Collin Croulet (SWMP Technician) is responsible for secondary validation.

The Buskey Laboratory at the University of Texas Marine Science Institute calculates and reports results in µM/L.  For purposes of consistency in the NERR System, Mission Aransas NERR calculates the concentrations as mg/ l-1 based on atomic weights of 14.01, 30.97, 28.09, and 12.01 for N, P, Si, and C respectively.  Therefore, Mission Aransas NERR staff multiplies the concentrations reported by the Buskey Laboratory at the University of Texas Marine Science Institute by 0.01401, 0.03097, 0.02809, and 0.01201 to yield concentrations in mg/L as N, P, Si, and C respectively.

10) Parameter titles and variable names by category:

Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisks “*”.

Data Category	  Parameter				Variable Name	Units of Measure

Phosphorus and Nitrogen:

*Orthophosphate			PO4F			mg/L as P

*Ammonium, Filtered			NH4F			mg/L as N

*Nitrite + Nitrate, Filtered		NO23F		mg/L as N

Dissolved Inorganic Nitrogen		DIN			mg/L as N

Plant Pigments:

*Chlorophyll a			CHLA_N		µg/L

Other Lab Parameters:

Silicate, Filtered			SiO4F			mg/L as SI

Field Parameters:

Water Temperature			WTEM_N		ºC

Specific Conductivity			SCON_N		mS

Salinity				SALT_N		ppt

Turbidity 				TURB_N		NTU

Secchi					SECCHI		m

Notes:

1.  Time is coded based on a 2400 clock and is referenced to Standard Time.

2.  Reserves have the option of measuring either NO2 and NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3. MAR is very nitrogen limited; therefore, we report it as NO23.

11) Measured or calculated laboratory parameters:

- Parameters measured directly

Nitrogen species:		NH4F, NO23F

Phosphorus species:		PO4F

Other:			CHLA_N, SiO4F

- Calculated parameters

DIN				NO23F + NH4F

12) Limits of detection:

Method Detection Limits (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect, have been established by the VIMS Nutrient Analytical Laboratory.  The MDL is determined as 3 times the standard deviation of a minimum of 7 replicates of a single low concentration sample.  These values are reviewed and revised periodically.

| Parameter | Start Date | End Date | MDL |
| --- | --- | --- | --- |
| PO4F | 1/1/14 | 12/31/14 | 0.0003 |
| NH4F | 1/1/14 | 12/31/14 | 0.0003 |
| NO23F | 1/1/14 | 12/31/14 | 0.0004 |
| SiO4F | 1/1/14 | 12/31/14 | 0.0084 |
| CHLA_N | 1/1/14 | 7/3/14 | 0.87 |
| CHLA_N | 7/4/14 | 12/31/14 | 0.35 |

13) Laboratory methods:

- Parameter: NH4F

UTMSI Laboratory Method: Seal QuAAtro Q-069-05 Rev. 4.

EPA or other reference method: 349.0

Method Reference: U.S. EPA Methods of Chemical Analysis of Water and Wastes, EPA-600/4-79-020, revised March 1983, Method 365.3

Method Descriptor: This method is based on the Berthelot reaction. Ammonia within the filtered seawater reacts in alkaline solution with hypochlorite to form monochloramina which, in the presence of phenol, catalytic amounts of nitroprusside (nitroferricyanide) and excess hypochlorite, gives indophenol blue. The formation of monochloramone requires a pH between 8 and 11.5. At pH greater than 9.6, some precipitation of calcium and magnesium as hydroxides and carbonates occurs in seawater, but these ions may be held in solution by complexing them with EDTA. The indophenol blue measured at 630 nm is proportional to the original ammonia concentration.

Preservation method: Samples filtered and stored frozen at -20 ºC for 2-3 days.

b)   Parameter: NO23F

UTMSI Laboratory Method: Seal QuAAtro Q-068-05 Rev. 4.

EPA or other reference method: 353.4

Method Reference: Zimmerman, C.F. and C.W. Keefe. EPA Method 353.4, Determination of Nitrate + Nitrite in Estuarine and Coastal Waters by Automated Colormetric Analysis in An Interim Manual of Methods for the Determination of Nutrients in Estuarine and Coastal Waters, Revision 1.1, June 1991.

Method Descriptor: Filtered sample is passed through a copperized cadmium column. Nitrite (reduced nitrate plus original nitrate) is then determined by diazotization with sulfanilamide under acidic conditions to form a diazonium ion. The resulting diazonium ion is coupled with N-(1-naphthyl) ethylenediamine dihydrochloride. The resulting pink dye absorbs at 520 nm.

Preservation method: Samples filtered and stored frozen at -20 ºC up to 14 days.

- Parameter: PO4F

UTMSI Laboratory Method: Seal QuAAtro Q-064-05 Rev. 4.

EPA or other reference method:

Method Reference: Diamond, D. 1994. Lachat Instruments Inc., QuikChem Method 31-115-01-1-A.

Method Descriptor: Filtered sample is treated with ammonium molybdate and antimony potassium tartrate under acidic conditions forming a yellow complex. This complex is reduced with ascorbic acid to form a blue complex which absorbs light at 880 nm. The ascorbic acid and molybdate reagents are merged on the chemistry manifold and the reagent stream is then merged with the carrier stream. The absorption is proportional to the concentration of orthophosphate in the sample

Preservation method: Samples filtered and stored frozen at -20 ºC.

- Parameter: SiO4F

UTMSI Laboratory Method: Seal QuAAtro Q-066-05 Rev. 4.

EPA or other reference method: 366

Method Reference: Parsons, T.R., Y. Maita and C.M. Lilli. A Manual of Chemical and Biological Methods for Seawater Analysis, 1.7, Determination of Silicate, p. 25-27.

Method Descriptor: Filtered sample is allowed to react with molybdate at 37 ºC and a pH of 1.2 to form a yellow silicamolybdate complex. This complex is reduced with stannous chloride to form a heteropoly blue complex which has an absorbance maximum of 820 nm. The intensity of the color is proportional to the concentration of molybdate reactive silica.

Preservation method: Samples filtered and stored frozen at -20 ºC.

e)	Parameter: CHLA

UTMSI Laboratory Method: Buskey Chl SOP v3_082311

EPA or other reference method:

Method Reference: Welschmeyer, N.A. 1994. Fluorometric analysis of chlorophyll a in the presence of chlorophyll b and pheopigments. Limnol. Oceanogr. 39(8): 1985-1992.

Method Descriptor: Samples are filtered onto GF/F filters and placed into scintillation vials with 10 mls 90% acetone and allowed to extract in the freezer for 2 days.

Preservation method: Samples filtered and stored frozen at -20 ºC for 2 days.

14)  Field and Laboratory QAQC programs:

- Precision

  - Field variability:

Two separate field replicates are collected using a van Dorn alpha collection bottle deployed ~ 0.5 m off the bottom of the bay.

  - Laboratory variability:

100% of samples are split and analyzed as 2 subsamples/replicates in the lab.

  - Inter-organizational splits:

Samples were not split and analyzed by two different labs.

- Accuracy

  - Sample spikes:

% Recovery of field and laboratory samples is approximately 85%.

  - Standard reference material analysis:

To the best of my knowledge, the UTMSI nutrient lab has not received or processed any samples for the EPA.

  - Cross calibration exercises

MANERR does not participate in cross calibration exercises at this time.

15) QAQC flag definitions:

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

16)  QAQC code definitions:

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data suspect due to QA/QC checks

GSM	See metadata

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Calculated value could not be determined due to a below MDL 				component

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N		from the north

NNE	from the north northeast

NE		from the northeast

ENE	from the east northeast

E		from the east

ESE	from the east southeast

SE		from the southeast

SSE	from the south southeast

S		from the south

SSW	from the south southwest

SW	from the southwest

WSW	from the west southwest

W		from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17)  Other remarks/notes:

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

Quality control for the data set, specifically looking at differences between replicates, is conducted so that data are flagged suspect (or rejected, depending on how far off) if they are outside the range of +/- 2 standard deviations. According to one nutrient specialist, although some of the subsamples may appear to be different from one another, since we have such small amounts of nutrients in our system, there is typically not a significant difference between them.

05/29/14 12:05 and 12:08	Ship Channel field readings for Turbidity rejected since they are a negative value and outside the calibration accuracy for that sensor.

08/28/14 21:23		Diel sample missing upon retrieval due to crack in ISCO bottle which caused the sample to leak out.

11/26/14 10:11		Nutrient sample missing due to loss during analysis.

Samples held beyond protocol (CHB code).  NERRS protocol allows up to 5 days for processing/shipping samples and then hold times of up to 28 days at -20˚C before analysis.

February through April nutrient samples were analyzed on 5/20/14, meaning that all but the 4/21-4/22 diel samples were held longer than allowed by protocol.

May through July nutrient samples were analyzed on 8/29/14, all samples were held longer than allowed by protocol.

August diel and September nutrient samples were analyzed on 11/5/14, samples were held longer than allowed by protocol.

Table 1. The mean daily discharge for 2014

| River | Location | USGS gauge | Discharge (ft3 s-1) |
| --- | --- | --- | --- |
| Aransas River | Skidmore, TX | 08189700 | 3.36 |
| Mission River | Refugio, TX | 08189500 | 1.52 |
| Copano Creek | Refugio, TX | 08189200 | 5.07 |
| San Antonio River | Goliad, TX | 08188500 | 355.8 |
| Guadalupe River | Victoria, TX | 08176500 | 675.9 |