Mission Aransas (MAR) NERR Nutrient Metadata

June – December 2007

Latest Update:  November 15, 2011

I. Data Set & Research Descriptors

- Principal investigator & contact persons:

Address: 	Mission Aransas National Estuarine Research Reserve

750 Channelview Drive

Port Aransas, TX 78373

Phone:  (361)749-6711

Fax:	 (361)749-6777

Contact Persons:

Dr. Edward Buskey, Research Program Coordinator

E-mail: ed.buskey@mail.utexas.edu; (361)749-6794

Sally Morehead, Manager

E-mail: sally.morehead@mail.utexas.edu; (361)749-6771

Cammie Hyatt, SWMP Data Manager & Technician

E-mail: cammie.hyatt@mail.utexas.edu; (361)749-6774

Brad Gemmell, SWMP Field Technician

E-mail: bgemmell@mail.utexas.edu; (361)749-6702

Colt Cook, SWMP Technician

E-mail : coltcook@mail.utexas.edu; (361)749-6843

- Research objectives:

Four long-term monitoring stations have been established along the Texas Coast in the Mission-Aransas estuary to collect baseline data on environmental conditions within the bay systems. These System Wide Monitoring Program Stations (SWMPs) provide continual monitoring of temperature, salinity, oxygen and other parameters that characterize the water quality of the bays. In addition to the above monitoring program, water samples are collected monthly to quantify the amounts of nutrients (orthophosphate, ammonium, nitrate/nitrite, silicate, and chlorophyll a) present in the water. There are two nutrient monitoring programs in place within the Mission-Aransas NERR: a) Monthly Grab Sampling Program, and b) Diel Sampling Program.

The monthly grab samples are collected at each of the four monitoring stations which are situated along a salinity gradient that is highest at the Aransas Ship Channel and lowest in Copano West. The main goal of the Monthly Grab Sampling Program is to provide baseline information on inorganic nutrients and chlorophyll a within the reserve. Nutrient concentrations are important in determining pollution that enters the system from agricultural and urban sources via runoff.

Monthly diel samples are collected over a 24-hr period. The main goal of this sampling program is to provide baseline information on inorganic nutrients and chlorophyll a.  By collecting these samples throughout a lunar tidal cycle, a more focused view of these nutrients can be achieved than just a single grab sample will allow. The site selected for this monitoring program (Aransas Ship Channel) was based primarily on the extreme tidal exchange that occurs in this area. On the incoming tide, higher salinity offshore water is drawn into the bays and on the outgoing tide, the lower salinity water from the bays is drawn out.

- Research Methods:

Monthly Grab Sampling Program:

Monthly grab samples are taken at each of the MANERR SWMP stations where datasondes are deployed. All grab samples are taken on the same day; however, due to the large size of the reserve and the fact that all but one of the sites are only accessible by boat, the stipulation that samples be taken at low/slack tide is usually not possible. Efforts are made to allow for an antecedent dry period of 72 hours prior to sampling but that is not always possible due to long periods of inclement weather.

Samples are collected at approximately 30 day intervals, weather permitting. Two replicate samples are taken using a Van Dorn alpha bottle deployed at the depth of the sondes (0.5 m off the bottom of the bay). A weight attached to the bottom of the bottle allows the collector to determine the proper depth. Once the weight has settled on the bottom of the bay, the sand and/or mud is allowed to settle for a minute or so before the messenger is sent to seal the bottle. The amber collection bottles are rinsed (3X) before filling with sample. A 60 ml syringe is rinsed with the sample water before attaching a syringe filter holder containing a 0.4 um Millipore polycarbonate filter. Approximately 10 mls of sample is pushed through the filter to rinse it, and then the two respective sample tubes are filled, capped, and placed on ice until return to the laboratory. If the water is particularly turbid, a GF/F pre-filter in a separate filter holder is used to prevent clogging. The amber bottle containing the chlorophyll sample is also placed on ice until return to the laboratory for filtration. Gloves are worn while filtering the nutrients to prevent contamination.

While in the field, a hand-held YSI 30 is used to record Specific Conductivity, Salinity and Temperature of the water being sampled. These readings are performed at the same approximate depth as the nutrient collection. A Turner Designs AquaFluor hand-held fluorometer is also used at each site to obtain field readings of Turbidity and in situ Chlorophyll (in situ chlorophyll data are available by contacting the Reserve directly). Secchi readings are taken at each site and the cloud cover code and wave height code are also noted.

Once in the MANERR laboratory, the nutrient samples are delivered to the in-house nutrient specialist for analysis. Two subsamples of each replicate are taken for both nutrient and chlorophyll processing (denoted by replicate “S” in the data file).

The chlorophyll a processing includes mixing the sample and then measuring an aliquot for filtration. To prevent clogging, the Secchi reading is used to determine how much water to filter. (This is based on an arbitrary convention developed by Paul Montagna’s Lab to avoid filter “clogging”) Maximum and minimum volumes are 50 ml (any Secchi reading ≥ 1m) and 15 ml (any Secchi reading ≤ 0.3m), respectively. Filtered volumes between the upper and lower limits are determined by Secchi reading as a percent of 100 cm multiplied by the maximum volume of 50 mls. (e.g. a Secchi reading of 80 cm would require a volume of 40 ml, a Secchi reading of 50 cm would require a volume of 25 ml, a Secchi reading of 120 cm would require a Secchi reading of 50 ml, and a Secchi reading of 20 cm would require a volume of 15 ml). Once the appropriate volume has been determined and measured into a graduated cylinder, the samples are filtered onto 25mm Whatman GF/F filters using a Gast vacuum pump attached to a filtration manifold. The filters are folded in half and placed into glass scintillation vials on ice. Once all samples are filtered, 10 mls 90% acetone is added to the vials. All filtering is done in semi-darkness and the vials are then placed in a box inside the freezer to extract for two days. At the end of the extraction period, the vials are removed from the freezer and placed in the dark at room temperature for approximately 30 minutes. During this time, the fluorometer (Turner Designs TD-700) is allowed to warm-up. Once the fluorometer is ready, the solid secondary standards (high and low) are read to ensure that the calibration of the fluorometer is still good. A blank of 90% acetone is also read prior to reading the chlorophyll samples. All lab glassware is washed with Liquinox cleaning solution and after rinsing (3X), rinsed with 5% HCl and then rinsed again (3X) with r.o. (reverse osmosis) water.

Diel Sampling Program:

Diel samples are collected once a month at the University of Texas Marine Science Institute pier which is located on the Aransas Ship Channel. This location is designated as one of the MANERR SWMP stations and has a datasonde deployed there. An ISCO Avalanche refrigerated auto-sampler takes 0.9 L samples at 2-hour intervals over a 24-hour cycle, resulting in 12 samples per event. Diel sampling usually begins between +/- 3 hours of slack-low tide but due to tidal conditions that may not have been possible. Efforts are made to allow for an antecedent dry period of 72 hours prior to sampling; however, this was not always possible due to long periods of inclement weather. Sampling events take place at approximately 30 day intervals, weather permitting. Samples are collected at an approximate depth of 5 m which is slightly less than the depth of the datasonde. All samples are collected in ISCO sampling bottles that have been previously cleaned in the same manner as the lab glassware described above. As soon as the first sample is taken, the auto-sampler refrigeration lowers the temperature in the chamber down to 3 ºC and keeps it at that temperature until the unit is unplugged and taken back into the laboratory. Samples for nutrients and chlorophyll are handled in the same manner as previously described in the “Monthly Grab Sampling Program” portion of this section. The only difference is that the nutrients are filtered in the lab rather than in the field.

4) Site location and character:

Mission-Aransas is an estuarine system composed of tertiary, secondary, and primary bays. Mission Bay is the only tertiary bay, and Copano, Port and St. Charles Bay are secondary bays. Mesquite, Aransas and Redfish Bay are primary bays because they are adjacent to the oceanic outlets. There is a salinity gradient which is highest at the Aransas Ship Channel and decreases into the upper bays. The four sites within Mission-Aransas that are being monitored for nutrients are:

- Aransas Bay (AB): [27º 58.791’N, 97º 1.727’W].  This station is located in open water in the middle of Aransas Bay. Tidal range is 1.0 meter, and yearly salinity ranges between 4-33 ppt. Yearly temperatures range from 13-32 ºC. Aransas Bay is influenced by freshwater inflow from both San Antonio Bay (which has inflow from the San Antonio River and the Guadalupe River) and Copano Bay. The average water depth is 3.1 meters and the bottom is composed of soft sediment. The Texas Commission of Environmental Quality (TCEQ) has determined that there are no exceedences for pollutants in Aransas Bay. This station is primarily influenced by the watershed sub-basin in Aransas County with a hydrologic unit code (HUC) of 12100405.

- Copano East (CE): [28º 7.941’N, 97º 2.067’W]. This station is located in open water on the east side of Copano Bay near the Copano Causeway. Tidal range is 1.0 meter, and yearly salinity ranges between 1-21 ppt. Yearly temperatures range from 12-33 ºC. Copano East is influenced by freshwater inflow from the Aransas River, Mission River, and Copano Creek. The average water depth is 2.6 meters and the bottom is composed of soft sediment. The TCEQ has determined that there are no exceedences for pollutants in Copano East. This station is influenced by three watershed sub-basins in several counties with HUC’s of 12100407, 12100406, and 12100405.

- Copano West (CW): [28º 5.043’N, 97º 12.056’W]. This station is located in open water on the west side of Copano Bay near Bayside. Tidal range is 1.0 meter, and yearly salinity ranges between 0-9 ppt. Yearly temperatures range from 10-33 ºC. Copano West is primarily influenced by freshwater inflow from the Aransas River (See Table 1 in Section 14). The average water depth is 1.9 meters and the bottom is composed of soft sediment. The TCEQ has determined that there is an exceedence for enterococci bacteria in Copano West. This station is primarily influenced by the watershed sub-basin in San Patricio and Bee County with a HUC of 12100407 (see Figure 1 in Section 14).

- Ship Channel (SC): [27º 50.296’N, 97º 3.017’W]. This station is located on the end of the UTMSI pier. Tidal range is 1.0 meter, and yearly salinity ranges between 13-35 ppt. Yearly temperatures range from 14-32 ºC. The Aransas Ship Channel is located on the Aransas Pass, which is the primary outlet to the Gulf of Mexico. This pass receives freshwater inflow from several bay systems, including Copano Bay, Aransas Bay, San Antonio Bay, and Corpus Christi Bay. The average water depth is 6.5 meters and the bottom is composed of soft sediment. The TCEQ currently has no exceedences for pollutants in the Aransas Ship Channel. This site is primarily influenced by the watershed sub-basin in Aransas County with a HUC of 12100405.

5) Code variable definitions:

marabnut = Mission-Aransas Aransas Bay nutrients

marcenut = Mission-Aransas Copano East nutrients

marcwnut = Mission-Aransas Copano West nutrients

marscnut = Mission-Aransas Ship Channel nutrients

monthly grab sample program = 1

diel grab sample program = 2

6) Data collection period:

Monthly Grab Sampling Program:

Site			Date			Rep 1		Rep 2

Aransas Bay		06/15/07*		07:50		08:02

AB			07/24/07		08:07		08:19

08/21/07		07:53		08:05

09/20/07		07:45		07:56

10/16/07		08:01		08:12

11/20/07		08:30		08:45

12/18/07		08:36		08:41

Site			Date			Rep 1		Rep 2

Copano East		06/15/07*		09:20		09:32

CE			07/24/07		08:55		09:07

08/21/07		08:55		09:08

09/20/07		08:34		08:47

10/16/07		08:47		08:54

11/20/07		09:15		09:27

12/18/07		09:15		09:23

Site			Date			Rep 1		Rep 2

Copano West		07/24/07*		09:45		09:57

CW			08/21/07		09:43		09:55

09/20/07		09:15		09:28

10/16/07		09:31		09:44

11/20/07		10:07		10:20

12/18/07		10:00		10:12

Site			Date			Rep 1		Rep 2

Ship Channel		09/20/07*		11:48		12:02

SC			10/16/07		11:44		11:56

11/20/07		12:50		12:59

12/18/07		14:03		14:29

* Denotes the date that SWMP nutrient monitoring began at each site.

Diel Grab Sample Program:

Site			First Sample			Last Sample

Ship Channel		09/19/07 14:57*		09/20/07 12:57

SC			10/16/07 13:49		10/17/07 11:49

11/19/07 11:53		11/20/07 09:53

12/17/07 14:47		12/18/07 12:47

* Denotes the date that SWMP diel nutrient monitoring began.

7) Associated researchers and projects:

In addition to the MANERR nutrient monitoring project, SWMP Water Quality data are collected using YSI datasondes that continually record the water temperature, specific conductivity, salinity, dissolved oxygen, depth, pH, and turbidity at each of the sites. MANERR also has a SWMP Meteorological station (MARCEMET) that collects air temperature, humidity, barometric pressure, wind speed, wind direction, precipitation, and photosynthetically active radiation (PAR).

Two researchers are currently conducting collaborative projects at the Ship Channel station:

Dong-Ha Min, a research scientist at the University of Texas Marine Science Institute (UTMSI), is using a vertical profiling system at the MSI pier to study the physical and chemical composition of the water column and water exchange processes of the Aransas Pass tidal inlet.

Lisa Campbell, a research scientist from Texas A & M College Station, has installed an in-situ flowcytometer (FlowCytoBot) to study the phytoplankton at this site in order to detect potential harmful algal blooms (HABs). This work is being conducted in conjunction with Robert Olsen of Woods Hole Oceanographic Institute.

Several researchers are currently using water quality data recorded at the Copano East and Copano West stations in addition to collecting water for their own projects:

Rae Mooney, a Ph.D. candidate at UTMSI, is collecting water in Copano Bay for nutrient analysis. She also calculates nutrient flux from the Aransas and Mission Rivers into Copano Bay and studies storm events. A NERR Graduate Research Fellowship supports her work.

Christopher Shank, a research scientist at UTMSI, is investigating sediment suspension and the release of organic material from the sediments. He is also interested in the interaction of solar radiation with the sediments.

8) Distribution:

NOAA/ERD retains the right to analyze, synthesize and publish summaries of the NERRS System-wide Monitoring Program data. The PI retains the right to be fully credited for having collected and processed the data. Following academic courtesy standards, the PI and NERR site where the data were collected will be contacted and fully acknowledged in any subsequent publications in which any part of the data are used. Manuscripts resulting from the NOAA/OCRM supported research that are produced for publication in open literature, including refereed scientific journals, will acknowledge that the research was conducted under an award from the Estuarine Reserves Division, Office of Ocean and Coastal Resource Management, National Ocean Service, National Oceanic and Atmospheric Administration. The data set enclosed within this package/transmission is only as good as the quality assurance and quality control procedures outlined by the enclosed metadata reporting statement. The user bears all responsibility for its subsequent use/misuse in any further analyses or comparisons. The Federal government does not assume liability to the Recipient of third persons, nor will the Federal government reimburse or indemnify the Recipient for its liability due to any losses resulting in any way from the use of this data.

NERR nutrient data and metadata can be obtained from the Research Coordinator at the individual NERR site (please see Principal investigators and contact persons), from the Data Manager at the Centralized Data Management Office (please see personnel directory under general information link on CDMO home page) and online at the CDMO home page http://cdmo.baruch.sc.edu. Data are available in text tab-delimited format.

# II. Physical Structure Descriptors

9) Entry verification:

The Villareal Lab at the University of Texas Marine Science Institute calculates and reports nutrient data in μM. For purposes of consistency in the NERR system, Mission-Aransas NERR calculates the concentrations as mg/L-1 based on atomic weights of 14.01, 30.97, 28.09, and 12.01 for N, P, Si, and C respectively. Therefore, Mission-Aransas NERR staff multiplies the concentrations reported by the Villareal Lab by 0.01401, 0.03097, 0.02809, and 0.01201 to yield concentrations in mg/L as N, P, Si, and C respectively. Once the data has been converted into mg/L, it is then copied and pasted into an Excel worksheet that has been generated by the NutrientQAQC macro. Prior to processing, the data is validated by the person who entered it into the worksheet and then further validated by a second person. Cammie Hyatt is responsible for the initial entry and validation. Colt Cook is responsible for secondary validation.

Nutrient data are entered into a Microsoft Excel worksheet and processed using the NutrientQAQC Excel macro. The NutrientQAQC macro sets up the data worksheet, metadata worksheets, and MDL worksheet; facilitates data entry; allows the user to set the number of significant figures to be reported for each parameter and rounds using banker’s rounding rules; allows the user to input MDL values and automatically flags and codes values below MDL; calculates parameters chosen by the user and automatically flags for component values below MDL and negative values; allows the user to apply QAQC flags and codes to the data; graphs selected parameters for review; append files; and export the resulting data files to the CDMO for tertiary QAQC and assimilation into the CDMO’s authoritative online database.

10) Parameter titles and variable names by category:

Required NOAA/NERRS System-wide Monitoring Program nutrient parameters are denoted by an asterisks “*”.

Data Category	Parameter	Variable Name	Units of Measure

Phosphorus and Nitrogen:

*Orthophosphate	PO4F			mg/L as P

*Ammonium, Filtered	NH4F			mg/L as N

*Nitrite + Nitrate, Filtered	NO23F		mg/L as N

Dissolved Inorganic Nitrogen	DIN			mg/L as N

Plant Pigments:

*Chlorophyll a	CHLA_N		µg/L

Other Lab Parameters:

Silicate, Filtered	SiO4F			mg/L as Si

Field Parameters:	Water Temperature	WTEM_N		ºC

Specific Conductivity	SCON_N		mS

Salinity	SALT_N		ppt

Turbidity	TURB_N		NTU

Secchi	SECCHI		m

Notes:

1. Time is coded based on a 2400 clock and is referenced to local Standard Time.

2. Reserves have the option of measuring either NO2 or NO3 or they may substitute NO23 for individual analyses if they can show that NO2 is a minor component relative to NO3.  At MAR NERR, nitrogen levels are so low that only NO23 reporting is feasible.

11) Measured or calculated laboratory parameters:

- Parameters measured directly:

Nitrogen species:			NH4, NO23

Phosphorus species:			PO4F

Other:	CHLA, SiO4, WTEM, SCON, SALT, TURB, SECCHI

- Calculated parameters:

DIN					NO23 + NH4

12) Limits of detection:

Method Detection Limits (MDL), the lowest concentration of a parameter that an analytical procedure can reliably detect, have been established by the Villareal Lab at the University of Texas Marine Science Institute. The MDL is determined as 3 times the standard deviation of a minimum of 7 replicates of a single low concentration sample. These values are reviewed and revised periodically.

Parameter	Start Date	End Date	MDL

PO4F		06/15/07	12/31/07	0.0009

NH4F		06/15/07	12/31/07	0.0007

NO23F	06/15/07	12/31/07	0.0004

SiO4F		06/15/07	12/31/07	0.0003

CHLA_N	06/15/07	12/31/07	0.1

13) Laboratory methods:

- Parameter: NH4F

UTMSI Laboratory Method: Lachat Quikchem 31-107-06-1-A

EPA or other reference method: 349.0

Method Reference: U.S. EPA Methods of Chemical Analysis of Water and Wastes, EPA-600/4-79-020, revised March 1983, Method 365.3

Method Descriptor: This method is based on the Berthelot reaction. Ammonia within the filtered seawater reacts in alkaline solution with hypochlorite to form monochloramina which, in the presence of phenol, catalytic amounts of nitroprusside (nitroferricyanide) and excess hypochlorite, gives indophenol blue. The formation of monochloramone requires a pH between 8 and 11.5. At pH greater than 9.6, some precipitation of calcium and magnesium as hydroxides and carbonates occurs in seawater, but these ions may be held in solution by complexing them with EDTA. The indophenol blue measured at 630 nm is proportional to the original ammonia concentration.

Preservation method: Samples filtered and stored frozen at -20 ºC for 2-3 days.

b)   Parameter: NO23F

UTMSI Laboratory Method: Lachat Quikchem 31-107-04-1-A

EPA or other reference method: 353.4

Method Reference: Zimmerman, C.F. and C.W. Keefe. EPA Method 353.4, Determination of Nitrate + Nitrite in Estuarine and Coastal Waters by Automated Colormetric Analysis in An Interim Manual of Methods for the Determination of Nutrients in Estuarine and Coastal Waters, Revision 1.1, June 1991.

Method Descriptor: Filtered sample is passed through a copperized cadmium column. Nitrite (reduced nitrate plus original nitrate) is then determined by diazotization with sulfanilamide under acidic conditions to form a diazonium ion. The resulting diazonium ion is coupled with N-(1-naphthyl) ethylenediamine dihydrochloride. The resulting pink dye absorbs at 520 nm.

Preservation method: Samples filtered and stored frozen at -20 ºC up to 14 days.

- Parameter: PO4F

UTMSI Laboratory Method: Lachat Quikchem 31-115-01-3-A

EPA or other reference method:

Method Reference: Diamond, D. 1994. Lachat Instruments Inc., QuikChem Method 31-115-01-1-A.

Method Descriptor: Filtered sample is treated with ammonium molybdate and antimony potassium tartrate under acidic conditions forming a yellow complex. This complex is reduced with ascorbic acid to form a blue complex which absorbs light at 880 nm. The ascorbic acid and molybdate reagents are merged on the chemistry manifold and the reagent stream is then merged with the carrier stream. The absorption is proportional to the concentration of orthophosphate in the sample

Preservation method: Samples filtered and stored frozen at -20 ºC.

- Parameter: SiO4F

UTMSI Laboratory Method: Lachat Quikchem 31-114-27-1-B

EPA or other reference method: 366

Method Reference: Parsons, T.R., Y. Maita and C.M. Lilli. A Manual of Chemical and Biological Methods for Seawater Analysis, 1.7, Determination of Silicate, p. 25-27.

Method Descriptor: Filtered sample is allowed to react with molybdate at 37 ºC and a pH of 1.2 to form a yellow silicamolybdate complex. This complex is reduced with stannous chloride to form a heteropoly blue complex which has an absorbance maximum of 820 nm. The intensity of the color is proportional to the concentration of molybdate reactive silica.

Preservation method: Samples filtered and stored frozen at -20 ºC.

e)	Parameter: CHLA

UTMSI Laboratory Method: Buskey Chl SOP v1 (6/15/07-9/20/07), v2 (10/16/07-12/31/07)

EPA or other reference method:

Method Reference: Welschmeyer, N.A. 1994. Fluorometric analysis of chlorophyll a in the presence of chlorophyll b and pheopigments. Limnol. Oceanogr. 39(8): 1985-1992.

Method Descriptor: V1. Samples are filtered onto GF/F filters and ground thoroughly using 90% acetone and allowed to extract in the freezer overnight. V2. Samples are filtered onto GF/F filters and placed into scintillation vials with 10 mls 90% acetone and allowed to extract in the freezer for 2 days.

Preservation method: Samples filtered and stored frozen at -20 ºC for 2 days.

14) Field and Laboratory QAQC programs:

- Precision

  - Field variability:

Two separate field replicates are collected using a van Dorn alpha collection bottle deployed ~ 0.5 m off the bottom of the bay.

  - Laboratory variability:

100% of samples are split and analyzed as 2 subsamples/replicates in the lab.

  - Inter-organizational splits:

Samples were not split and analyzed by two different labs.

- Accuracy

  - Sample spikes:

% Recovery of field and laboratory samples is approximately 85%.

  - Standard reference material analysis:

To the best of my knowledge, the UTMSI nutrient lab has not received or processed any samples for the EPA.

  - Cross calibration exercises

MANERR does not participate in cross calibration exercises at this time.

15) QAQC flag definitions:

QAQC flags provide documentation of the data and are applied to individual data points by insertion into the parameter’s associated flag column (header preceded by an F_).   QAQC flags are applied to the nutrient data during secondary QAQC to indicate data that are out of sensor range low (-4), rejected due to QAQC checks (-3), missing (-2), optional and were not collected (-1), suspect (1), and that have been corrected (5).  All remaining data are flagged as having passed initial QAQC checks (0) when the data are uploaded and assimilated into the CDMO ODIS as provisional plus data.  The historical data flag (4) is used to indicate data that were submitted to the CDMO prior to the initiation of secondary QAQC flags and codes (and the use of the automated primary QAQC system for WQ and MET data).  This flag is only present in historical data that are exported from the CDMO ODIS.

-4		Outside Low Sensor Range*

-3		Data Rejected due to QAQC

-2		Missing Data

-1		Optional SWMP Supported Parameter

0		Data Passed Initial QAQC Checks

1		Suspect Data

4		Historical Data:  Pre-Auto QAQC

5		Corrected Data

*The -4 Outside Low Sensor Range flag was added to the 2007 dataset in August of 2011.  See the Other Remarks section for more details.

16) QAQC code definitions:

QAQC codes are used in conjunction with QAQC flags to provide further documentation of the data and are also applied by insertion into the associated flag column.  There are three (3) different code categories, general, sensor, and comment.  General errors document general problems with the sample or sample collection, sensor errors document common sensor or parameter specific problems, and comment codes are used to further document conditions or a problem with the data.  Only one general or sensor error and one comment code can be applied to a particular data point.  However, a record flag column (F_Record) in the nutrient data allows multiple comment codes to be applied to the entire data record.

General errors

GCM	Calculated value could not be determined due to missing data

GCR	Calculated value could not be determined due to rejected data

GDM	Data missing or sample never collected

GQD	Data rejected due to QA/QC checks

GQS	Data rejected due to QA/QC checks

Sensor errors

SBL	Value below minimum limit of method detection

SCB	Value calculated with a value that is below the MDL

SCC	Calculation with this component resulted in a negative value

SNV	Calculated value is negative

SRD	Replicate values differ substantially

SUL	Value above upper limit of method detection

Parameter Comments

CAB	Algal bloom

CDR	Sample diluted and rerun

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Record comments

CAB	Algal bloom

CHB	Sample held beyond specified holding time

CIP	Ice present in sample vicinity

CIF	Flotsam present in sample vicinity

CLE	Sample collected later/earlier than scheduled

CRE	Significant rain event

CSM	See metadata

CUS	Lab analysis from unpreserved sample

Cloud cover

CCL	clear (0-10%)

CSP	scattered to partly cloudy (10-50%)

CPB	partly to broken (50-90%)

COC	overcast (>90%)

CFY	foggy

CHY	hazy

CCC	cloud (no percentage)

Precipitation

PNP	none

PDR	drizzle

PLR	light rain

PHR	heavy rain

PSQ	squally

PFQ	frozen precipitation (sleet/snow/freezing rain)

PSR	mixed rain and snow

Tide stage

TSE	ebb tide

TSF	flood tide

TSH	high tide

TSL	low tide

Wave height

WH0	0 to <0.1 meters

WH1	0.1 to 0.3 meters

WH2	0.3 to 0.6 meters

WH3	0.6 to > 1.0 meters

WH4	1.0 to 1.3 meters

WH5	1.3 or greater meters

Wind direction

N	from the north

NNE	from the north northeast

NE	from the northeast

ENE	from the east northeast

E	from the east

ESE	from the east southeast

SE	from the southeast

SSE	from the south southeast

S	from the south

SSW	from the south southwest

SW	from the southwest

WSW	from the west southwest

W	from the west

WNW	from the west northwest

NW	from the northwest

NNW	from the north northwest

Wind speed

WS0	0 to 1 knot

WS1	> 1 to 10 knots

WS2	> 10 to 20 knots

WS3	> 20 to 30 knots

WS4	> 30 to 40 knots

WS5	> 40 knots

17) Other remarks/notes:

Data may be missing due to problems with sample collection or processing.  Laboratories in the NERRS System submit data that are censored at a lower detection rate limit, called the Method Detection Limit or MDL.  MDLs for specific parameters are listed in the Laboratory Methods and Detection Limits Section (Section II, Part 12) of this document.  Concentrations that are less than this limit are censored with the use of a QAQC flag and code, and the reported value is the method detection limit itself rather than a measured value.  For example, if the measured concentration of NO23F was 0.0005 mg/l as N (MDL=0.0008), the reported value would be 0.0008 and would be flagged as out of sensor range low (-4) and coded SBL.  In addition, if any of the components used to calculate a variable are below the MDL, the calculated variable is removed and flagged/coded -4 SCB.  If a calculated value is negative, it is rejected and all measured components are marked suspect.  If additional information on MDL’s or missing, suspect, or rejected data is needed, contact the Research Coordinator at the Reserve submitting the data.

Note: The way below MDL values are handled in the NERRS SWMP dataset was changed in November of 2011.  Previously, below MDL data from 2007-2010 were also flagged/coded, but either reported as the measured value or a blank cell.  Any 2007-2011 nutrient/pigment data downloaded from the CDMO prior to November of 2011 will reflect this difference.

*The 2007 dataset was updated on August of 2011 to include the -4 Outside Low Sensor Range flag.  The 2007 data published prior to that time used the -3 Rejected data flag with the SBL and SCB QAQC codes to indicate that data were below the minimum detection limit.  These flag code combinations were all replaced with the -4 SBL or SCB update as mandated by the Data Management Committee.

The following comments are referred to in the flag columns within the data set with the CSM (see metadata) flag.

Grab Samples

Site AB

06/15/07		No field readings taken for turbidity.

06/15/07 - 09/20/07 	Acetone volumes for chlorophyll analysis corrected due to displacement caused by grinding filters.

07/24/07	Forgot to read turbidity of the second sample.

09/20/07	Field readings of specific conductivity and salinity taken with YSI 30 were off >10% compared to newly calibrated sonde.

12/18/07	Field readings of temperature, specific conductivity, and salinity taken with YSI 30 were off >10% compared to newly calibrated sonde.

Site CE

06/15/07		No field readings taken for turbidity.

06/15/07 - 09/20/07 	Acetone volumes for chlorophyll analysis corrected due to displacement caused by grinding filters.

07/24/07	Field readings of specific conductivity and salinity taken with YSI 30 were off >10% compared to newly calibrated sonde. Field reading of rep 2 turbidity taken with Aquafluor was >10% off compared with newly calibrated sonde.

08/21/07	Field readings of turbidity taken with Aquafluor were off >10% compared with newly calibrated sonde.

10/16/07 08:54	Field reading of turbidity taken with Aquafluor was off >10% compared with newly calibrated sonde.

Site CW

07/24/07	Batteries went dead in Aquafluor before the turbidity of the second sample could be read.

07/24/07 - 09/20/07 	Acetone volumes for chlorophyll analysis corrected due to displacement caused by grinding filters.

08/21/07	Field readings of turbidity taken with Aquafluor were off >10% compared with newly calibrated sonde.

09/20/07 09:28	Field reading of turbidity taken with Aquafluor was off >10% compared with newly calibrated sonde.

11/20/07 10:20	Field reading of turbidity taken with Aquafluor was off >10% compared with newly calibrated sonde.

Site SC

09/20/07 	Acetone volumes for chlorophyll analysis corrected due to displacement caused by grinding filters.

09/20/07 11:48	Slightly negative value for the turbidity field reading is within acceptable calibration error.

10/16/07 11:44	Field reading of turbidity taken with Aquafluor was off >10% compared with newly calibrated sonde.

11/20/07	Field readings of turbidity taken with Aquafluor were off >10% compared with newly calibrated sonde.

Diel Sampling

Site SC

09/19/07 – 09/20/07	Acetone volumes for chlorophyll analysis corrected due to displacement caused by grinding filters.

Table 1. The mean daily discharge for 2007.

| River | Location | USGS gauge | Discharge (ft3 s-1) |
| --- | --- | --- | --- |
| Aransas River | Skidmore, TX | 08189700 | 89.0 |
| Mission River | Refugio, TX | 08189500 | 331.4 |
| Copano Creek | Refugio, TX | 08189200 | 136.4 |
| San Antonio River | Goliad, TX | 08188500 | 2,066.0 |
| Guadalupe River | Victoria, TX | 08176500 | 3,864.0 |

Figure 1. Watershed sub-basins that drain into the Mission-Aransas NERR.